HPE A5120EI-CMW520-R2222P11 Release Notes

Software Feature Changes

The information in this document is subject to change without notice. © Copyright 2018 Hewlett Packard Enterprise Development LP

Contents

A5120EI-CMW520-R2222P11	1

Modified feature: RADIUS Calling-Station-ID attribute value for the login service	1

Feature change description	1

Command changes	1

Modified feature: Port security need to know feature	1

Feature change description	1

Command changes	1

Modified command: port-security ntk-mode	1

A5120EI-CMW520-R2222P09	3

Modified feature: RADIUS Calling-Station-ID attribute value for the login service	3

Feature change description	3

Command changes	3

Modified feature: Port security need to know feature	3

Feature change description	3

Command changes	3

Modified command: port-security ntk-mode	3

A5120EI-CMW520-R2222P08	0

A5120EI-CMW520-R2222P07	1

New feature: Configuring the action a port takes after it receives an Ethernet OAM event from the remote end	1

Configuring the action a port takes after it receives an Ethernet OAM event from the remote end	1

Command reference	1

oam remote-failure action	1

A5120EI-CMW520-R2222P05	3

A5120EI-CMW520-R2222P02	4

A5120EI-CMW520-R2222P01	5

Removed feature: User profile	5

Feature change description	5

Removed commands	5

A5120EI-CMW520-R2222	6

Modified feature: Support for forwarding PTP multicast packets of Layer 2 multicast	6

Feature change description	6

Modified feature: Uploading IPv6 addresses for 802.1X and MAC authentication users	6

Feature change description	6

A5120EI-CMW520-R2221P30	7

Modified feature: Specifying file name for storing DHCP snooping entries on a remote server	7

Feature change description	7

Command changes	7

Modified command: dhcp-snooping binding database filename	7

A5120EI-CMW520-R2221P29	8

New feature: Including user IP addresses in realtime accounting packets for MAC authentication users with dynamic IP addresses	8

New feature: Ignoring the ingress ports of ARP packets during user validity check	8

Configuring ARP detection to ignore the ingress ports of ARP packets during user validity check	8

Command reference	9

arp detection port-match-ignore	9

A5120EI-CMW520-R2221P27	10

New feature: Authorization VLAN auto-tagging for MAC authentication	10

Enabling authorization VLAN auto-tagging for MAC authentication	10

Command reference	10

mac-authentication auto-tag	10

Modified feature: Confining RADIUS Vendor-Specific extended attributes to a specific vendor	11

Feature change description	11

Command changes	11

Modified command: service-type	11

A5120EI-CMW520-R2221P25	13

New feature: Configuring periodic MAC re-authentication	13

Configuring periodic MAC re-authentication	13

Command reference	14

New command: mac-authentication timer reauth-period (system view)	14

New command: mac-authentication re-authenticate	14

New command: mac-authentication timer reauth-period (interface view)	15

Modified command: display mac-authentication	16

A5120EI-CMW520-R2221P22	18

New feature: Enabling sending of ICMPv6 redirect messages	18

Enabling sending of ICMPv6 redirect messages	18

Command reference	18

New command: ipv6 redirects enable	18

Modified feature: Disabling advertising prefix information in RA messages	19

Feature change description	19

Command changes	19

Modified command: ipv6 nd ra prefix	19

A5120EI-CMW520-R2221P20	20

New feature: MAC authentication voice VLAN	20

Configuring a MAC authentication voice VLAN	20

Configuration prerequisites	20

Configuration guidelines	20

Configuration procedure	20

Example of Configuring 802.1X or MAC authentication for IP phones	21

Network requirements	21

Requirements analysis	21

Configuration restrictions and guidelines	21

Configuration procedure	22

Command reference	24

mac-authentication voice vlan	24

A5120EI-CMW520-R2221P18	25

New feature: Sending EAP-Success packets to 802.1X users in critical VLAN	25

Configuring the device to send EAP-Success packets to 802.1X users in critical VLAN	25

Command reference	26

dot1x critical eapol	26

A5120EI-CMW520-R2221P15	27

A5120EI-CMW520-R2221P12	28

New feature: Login delay	28

Enabling the login delay	28

Command reference	28

attack-defense login reauthentication-delay	28

Modified feature: IPv6 address with a 127-bit prefix length	29

Feature change description	29

Command changes	29

A5120EI-CMW520-R2221P10	30

New feature: Support for NTP configuration in IPv6 networks	30

Configuring NTP in IPv6 networks	30

Command reference	30

New command: display ntp-service ipv6 sessions	30

New command: ntp-service ipv6 access	32

New command: ntp-service ipv6 dscp	33

New command: ntp-service ipv6 in-interface disable	33

New command: ntp-service ipv6 multicast-client	34

New command: ntp-service ipv6 multicast-server	34

New command: ntp-service ipv6 source-interface	35

New command: ntp-service ipv6 unicast-peer	36

New command: ntp-service ipv6 unicast-server	36

A5120EI-CMW520-R2221P08	38

New feature: Applicable scope of packet filtering on a VLAN interface	38

Configuring the applicable scope of packet filtering on a VLAN interface	38

Command reference	38

packet-filter filter	38

New feature: SNMP notifications for PVST topology changes	39

Enabling SNMP notifications for PVST topology changes	39

Command reference	39

snmp trap enable stp	39

New feature: Disabling SSL 3.0	40

Disabling SSL 3.0	40

Command reference	40

ssl version ssl3.0 disable	40

A5120EI-CMW520-R2221P07	42

New feature: 802.1X MAC address binding	42

Configuring 802.1X MAC address binding	42

Command reference	43

dot1x binding-mac enable	43

dot1x binding-mac	43

New feature: Automatic PI reset	44

Enabling automatic PI reset	44

Command reference	44

poe reset enable	44

A5120EI-CMW520-R2221P06	46

A5120EI-CMW520-R2221P05	47

New feature: Telnet/SSH user connection control	47

Configuring Telnet/SSH user connection control	47

Configuration prerequisites	47

Configuration procedure	47

Command reference	48

ssh server acl	48

ssh server ipv6 acl ipv6	49

telnet server acl	49

telnet server ipv6 acl ipv6	50

Modified feature: Including time zone information in the timestamp of system information sent to a log host	51

Feature change description	51

Command changes	51

Modified command: info-center timestamp loghost	51

Modified feature: Configuring physical state change suppression on an Ethernet interface	52

Feature change description	52

Command changes	52

Modified command: link-delay	52

Modified feature: Configuring a tag and description for an IPv6 static route	53

Feature change description	53

Command changes	53

Modified command: ipv6 route-static	53

A5120EI-CMW520-R2221P04	54

New feature: 802.1X voice VLAN	54

Configuring an 802.1X voice VLAN	54

Configuration guidelines	54

Configuration prerequisites	54

Configuration procedure	55

Command reference	55

New command: dot1x voice vlan	55

New feature: TCP fragment attack protection	56

Enabling TCP fragment attack protection	56

Command reference	56

attack-defense tcp fragment enable	56

Modified feature: Username request timeout timer for 802.1X authentication	57

Feature change description	57

Command changes	57

Modified command: dot1x timer	57

A5120EI-CMW520-R2221P02	58

New feature: Support for BPDU guard configuration in interface or port group view	58

Configuring BPDU guard for an interface or port group	58

Enabling BPDU guard for an interface or port group when BPDU guard is globally disabled	58

Disabling BPDU guard for an interface or port group when BPDU guard is globally enabled	59

Command reference	59

New command: stp port bpdu-protection	59

New feature: MAC re-authentication timer for users in guest VLAN	60

Configuring MAC re-authentication timer for users in guest VLAN	60

Command reference	61

mac-authentication timer guest-vlan-reauth	61

New feature: MAC and port uniqueness check by the DHCP snooping device	62

Enabling MAC and port uniqueness check on the DHCP snooping device	62

Command reference	62

dhcp-snooping check mac-port	62

Modified feature: Auto status transition of dynamic secure MAC addresses	63

Feature change description	63

Command changes	63

Modified feature: The maximum number of gateways supported in MFF automatic mode	63

Feature change description	63

Command changes	63

A5120EI-CMW520-R2221P01	64

New feature: Discarding IPv6 packets that contain extension headers	64

Enabling a device to discard IPv6 packets that contain extension headers	64

Command reference	64

New command: ipv6 option drop enable	64

A5120EI-CMW520-R2221	66

New feature: SSL server policy association with the FTP service	66

Configuration procedure	66

Command reference	66

ftp server ssl-server-policy	66

New feature: MFF	67

Overview	67

Basic concepts	68

Operation modes	68

Working mechanism	69

Protocols and standards	69

Configuring MFF	69

Configuration prerequisites	69

Enabling MFF	69

Configuring a network port	70

Enabling periodic gateway probe	70

Specifying the IP addresses of servers	70

Displaying and maintaining MFF	71

MFF configuration examples	71

Auto-mode MFF configuration example in a tree network	71

Auto-mode MFF configuration example in a ring network	72

Manual-mode MFF configuration example in a tree network	74

Manual-mode MFF configuration example in a ring network	76

Command reference	77

display mac-forced-forwarding interface	77

display mac-forced-forwarding vlan	78

mac-forced-forwarding	79

mac-forced-forwarding gateway probe	79

mac-forced-forwarding network-port	80

mac-forced-forwarding server	81

Modified feature: Setting the device name	82

Feature change description	82

Command changes	82

Modified command: sysname	82

Modified feature: Displaying brief interface information	82

Feature change description	82

Command changes	82

Modified command: display interface	82

Modified feature: Displaying brief IP configuration for Layer 3 interfaces	83

Feature change description	83

Command changes	83

Modified command: display ip interface brief	83

Modified feature: Configuring static multicast MAC address entries	83

Feature change description	83

Command changes	84

Modified command: mac-address multicast	84

Modified command: display mac-address multicast	84

Modified feature: Specifying the username and password to log in to the SCP server	85

Feature change description	85

Command changes	85

Modified command: SCP	85

Modified feature: Disabling an untrusted port from recording clients' IP-to-MAC bindings	86

Feature change description	86

Command changes	86

Modified command: dhcp-snooping trust	86

New command: dhcp-snooping no-user-binding	86

Modified feature: Customizing DHCP options	87

Feature change description	87

Command changes	87

Modified command: option	87

A5120EI-CMW520-R2220P11	88

Modified feature: Specifying multiple public keys for an SSH user	88

Feature change description	88

Command changes	88

Modified command: ssh user	88

Modified feature: ACL-based packet filtering on a VLAN interface	89

Feature change description	89

Command changes	89

Modified command: packet-filter	89

A5120EI-CMW520-R2220P09	91

New feature: 802.1X-based dynamic IPv4 source guard binding entries	91

Overview	91

Configuration procedure	91

Configuration task list	91

Enabling the 802.1X IP freezing function	92

Enabling a port to generate 802.1X-based dynamic IPv4 source guard binding entries	92

Command reference	92

dot1x user-ip freeze	92

ip verify source dot1x	93

New feature: Multicast ND	93

Configuring multicast ND	93

Command reference	94

New feature: Configuring packet capture	94

Overview	94

Configuring the packet capture function	94

Displaying and maintaining packet capture	95

Packet capture configuration example	96

Command reference	97

display packet capture buffer	97

display packet capture status	98

packet capture	99

packet capture buffer save	100

packet capture schedule	101

packet capture start	101

packet capture stop	103

reset packet capture buffer	103

New feature: Enabling MAC authentication multi-VLAN mode	104

Overview	104

Configuration procedure	104

Command reference	104

mac-authentication host-mode multi-vlan	104

New feature: Binding IP, MAC, and port on Web	105

Overview	105

Command reference	105

New feature: Configuring the ARP detection logging function	105

Configuring the ARP detection logging function	105

Command reference	106

arp detection log enable	106

Modified feature: Configuring system information for the SNMP agent	106

Feature change description	106

Command changes	106

Modified command: snmp-agent sys-info	106

Modified feature: Specifying multiple secondary HWTACACS servers	107

Feature change description	107

Command changes	107

Modified command: primary accounting	107

Modified command: primary authentication	108

Modified command: primary authorization	108

Modified command: secondary accounting	109

Modified command: secondary authentication	109

Modified command: secondary authorization	110

A5120EI-CMW520-R2220P02	112

Modified feature: Enabling/disabling FIPS mode	112

Feature change description	112

Command changes	112

Modified feature: Setting the IRF link down report delay	112

Feature change description	112

Command changes	112

Modified command: irf link-delay	112

Modified feature: Setting the minimum password length	113

Feature change description	113

Command changes	113

Modified command: password-control length	113

Modified feature: Switching the user privilege level	113

Feature change description	113

Command changes	113

Modified command: super	113

Modified feature: Implementing ACL-based IPsec	114

Feature change description	114

Command changes	114

Modified feature: Cluster management	114

A5120EI-CMW520-R2220P01	115

A5120EI-CMW520-R2220	116

New feature: Disabling password recovery capacity	116

Disabling password recovery capacity	116

Command reference	116

password-recovery enable	116

New feature: Configuring a port to forward 802.1X EAPOL packets untagged	117

Configuring a port to forward 802.1X EAPOL packets untagged	117

Command reference	117

dot1x eapol untag	117

New feature: Enabling source IP conflict prompt	118

Enabling source IP conflict prompt	118

Command reference	118

arp ip-conflict prompt	118

New feature: Delaying the MAC authentication	119

Configuring the MAC authentication delay	119

Command reference	119

mac-authentication timer auth-delay	119

New feature: Disabling MAC entry aging timer refresh based on destination MAC address	120

Disabling MAC entry aging timer refresh based on destination MAC address	120

Application example	120

Command reference	121

mac-address destination-hit disable	121

New feature: Setting the deletion delay time for SAVI	121

Setting the deletion delay time for SAVI	121

Command reference	122

ipv6 savi down-delay	122

Modified feature: Default configuration	122

Feature change description	122

Command changes	123

A5120EI-CMW520-F2218	124

New feature: Supporting using a self-signed certificate for HTTPS	124

New feature: Setting the maximum number of 802.1X authentication attempts for MAC authentication users	124

Setting the maximum number of 802.1X authentication attempts for MAC authentication users	124

Command reference	125

dot1x attempts max-fail	125

New feature: Support of 802.1X for issuing VLAN groups	125

Support of 802.1X for issuing VLAN groups	125

Configuring a VLAN group	126

Command reference	126

vlan-group	126

vlan-list	127

New feature: Enabling MAC address migration log notifying	127

Enabling MAC address migration log notifying	127

Command reference	128

mac-flapping notification enable	128

Modified feature: Cluster management	129

Feature change description	129

Command changes	129

Modified command: cluster enable	129

Modified command: ndp enable	129

Modified command: ntdp enable	129

Removed feature: WiNet	130

Feature change description	130

Removed commands	130

A5120EI-CMW520-F2217	131

New feature: Automatic configuration file backup for software downgrading	131

Configuring automatic configuration file backup for software downgrading	131

Command reference	132

New feature: FIPS	132

Overview	132

FIPS self-tests	132

Power-up self-test	132

Conditional self-tests	132

Triggering a self-test	132

Configuring FIPS	133

Enabling the FIPS mode	133

Triggering a self-test	133

Displaying and maintaining FIPS	134

FIPS configuration example	134

Command reference	135

fips mode enable	135

display fips status	136

fips self-test	136

New feature: IPsec	137

IPsec overview	137

Basic concepts	137

Protocols and standards	139

Configuring IPsec	140

Implementing ACL-based IPsec	140

Feature restrictions and guidelines	140

ACL-based IPsec configuration task list	140

Configuring ACLs	141

Configuring an IPsec proposal	142

Configuring an IPsec policy	143

Applying an IPsec policy group to an interface	145

Configuring the IPsec session idle timeout	146

Enabling ACL checking of de-encapsulated IPsec packets	146

Configuring the IPsec anti-replay function	147

Configuring packet information pre-extraction	147

Displaying and maintaining IPsec	148

IKE-based IPsec tunnel for IPv4 packets configuration example	148

Command reference	150

ah authentication-algorithm	150

connection-name	151

display ipsec policy	151

display ipsec proposal	154

display ipsec sa	155

display ipsec session	158

display ipsec statistics	160

display ipsec tunnel	161

encapsulation-mode	163

esp authentication-algorithm	163

esp encryption-algorithm	164

ike-peer (IPsec policy view)	165

ipsec anti-replay check	165

ipsec anti-replay window	166

ipsec decrypt check	166

ipsec policy (interface view)	167

ipsec policy (system view)	167

ipsec proposal	168

ipsec sa global-duration	169

ipsec session idle-time	170

pfs	170

policy enable	171

proposal (IPsec policy view)	171

qos pre-classify	172

reset ipsec sa	173

reset ipsec session	174

reset ipsec statistics	174

sa authentication-hex	175

sa duration	176

sa encryption-hex	176

sa spi	177

security acl	178

transform	179

tunnel local	180

tunnel remote	180

New feature: IKE	181

IKE overview	181

IKE security mechanism	181

IKE operation	182

IKE functions	182

Relationship between IKE and IPsec	183

Protocols and standards	183

IKE configuration task list	183

Configuring a name for the local security gateway	184

Configuring an IKE proposal	184

Configuring an IKE peer	185

Setting keepalive timers	187

Setting the NAT keepalive timer	187

Configuring a DPD detector	188

Disabling next payload field checking	188

Displaying and maintaining IKE	188

IKE configuration example	189

Troubleshooting IKE	192

Invalid user ID	192

Proposal mismatch	192

Failing to establish an IPsec tunnel	192

ACL configuration error	193

Command reference	193

authentication-algorithm	193

authentication-method	194

certificate domain	194

dh	195

display ike dpd	196

display ike peer	197

display ike proposal	198

display ike sa	199

dpd	202

encryption-algorithm	203

exchange-mode	203

id-type	204

ike dpd	205

ike local-name	205

ike next-payload check disabled	206

ike peer (system view)	207

ike proposal	207

ike sa keepalive-timer interval	208

ike sa keepalive-timer timeout	208

ike sa nat-keepalive-timer interval	209

interval-time	209

local	210

local-address	211

local-name	211

nat traversal	212

peer	212

pre-shared-key	213

Syntax	213

proposal (IKE peer view)	214

remote-address	214

remote-name	215

reset ike sa	216

sa duration	217

time-out	218

New feature: Verifying the correctness and integrity of the file	218

Verifying the correctness and integrity of the file	218

Command reference	218

crypto-digest	218

Modified feature: Configuring a password for the local user	219

Feature change description	219

Command changes	219

Modified command: password (local user view)	219

Modified feature: Clearing all users from the password control blacklist	220

Feature change description	220

Command changes	220

Modified feature: 802.1X critical VLAN	220

Feature change description	220

Command changes	220

Modified feature: MAC authentication critical VLAN	220

Feature change description	220

Command changes	221

Modified feature: Modifying CLI configuration commands executed in FIPS mode for CC evaluation	221

Feature change description	221

Modified command: super password	221

Modified feature: Modifying login management commands executed in FIPS mode for CC evaluation	222

Feature change description	222

Command changes	222

Modified command: authentication-mode	222

Modified command: protocol inbound	223

Modified command: set authentication password	224

Modified Feature: Modifying software upgrade commands executed in FIPS mode for CC evaluation	225

Feature change description	225

Command changes	225

Modified Feature: Modifying configuration file management commands executed in FIPS mode for CC evaluation	225

Feature change description	225

Command changes	225

Modified Feature: Modifying security commands executed in FIPS mode for CC evaluation	225

Feature change description	225

Command changes	225

Modified command: key (HWTACACS scheme view)	225

Modified command: key (RADIUS scheme view)	226

Modified command: password	226

Modified command: primary accounting (RADIUS scheme view)	227

Modified command: primary authentication (RADIUS scheme view)	227

Modified command: secondary accounting (RADIUS scheme view)	228

Modified command: secondary authentication (RADIUS scheme view)	228

Modified command: password-control composition	228

Modified command: password-control length	229

Modified command: password-control super composition	229

Modified command: password-control super length	230

Modified command: public-key local create	230

Modified command: scp	230

Modified command: ssh user	231

Modified command: ssh2	232

Modified command: sftp	233

Modified command: ciphersuite	234

Modified command: prefer-cipher	235

Modified command: certificate request mode	236

Modified feature: Modifying SNMP commands executed in FIPS mode for CC evaluation	236

Feature change description	236

Command changes	236

Modified command: display snmp-agent community	236

Modified command: snmp-agent community	236

Modified command: snmp-agent group	237

Modified command: snmp-agent usm-user { v1 | v2c }	237

Modified command: snmp-agent calculate-password	237

Modified command: snmp-agent sys-info	238

Modified command: snmp-agent target-host	238

Modified command: snmp-agent usm-user v3	239

A5120EI-CMW520-R2215	240

New feature: Changing the brand name	240

Changing the brand name	240

Configuration preparation	240

Configuration procedure	241

Command reference	241

brand	241

display brand	242

New feature: Configuring the maximum number of Selected ports allowed for an aggregation group	243

Configuration guidelines	243

Configuration procedure	243

Command reference	243

link-aggregation selected-port maximum	243

New feature: Bulk configuring interfaces	244

Configuration guidelines	244

Configuration procedure	244

Command reference	245

interface range	245

interface range name	246

Modified feature: Displaying the remaining power of the IRF fabric	247

Feature change description	247

Command changes	247

Modified command: display poe interface	247

Modified command: display poe interface power	248

Modified feature: NTP	249

Feature change description	249

Command changes	249

Modified command: ntp-service broadcast-server	249

Modified command: ntp-service multicast-server	249

Modified command: ntp-service unicast-peer	250

Modified command: ntp-service unicast-server	250

Modified feature: Setting the IRF link down report delay	250

Feature change description	250

Command changes	250

Modified command: irf link-delay	250

A5120EI-CMW520-F2212P02	252

New feature: Configuring LLDP to advertise a specific voice VLAN	252

Configuration guidelines	252

Configuration procedure	253

Dynamically advertising server-assigned VLANs through LLDP	253

Command reference	253

lldp voice-vlan	253

A5120EI-CMW520-F2212	255

New feature: Setting the DSCP value for multiple types of protocol packets	255

Setting the DSCP value for DHCPv6 protocol packets	255

Setting the DSCP value for DHCP protocol packets	255

Setting the DSCP value for DNS protocol packets	256

Setting the DSCP value for FTP and TFTP protocol packets	256

Setting the DSCP value for HTTP protocol packets	257

Setting the DSCP value for IGMP protocol packets sent by IGMP snooping	257

Setting the DSCP value for IPv6 DNS protocol packets	257

Setting the DSCP value for MLD protocol packets sent by MLD snooping	258

Setting the ToS value for packets sent by the TCP listening service on the NQA server	258

Setting the ToS value for packets sent by the UDP listening service on the NQA server	258

Setting the ToS value for NQA probe packets	259

Setting the DSCP value for NTP protocol packets	259

Setting the DSCP value for RADIUS protocol packets	259

Setting the DSCP value for SNMP trap packets	260

Setting the DSCP value for SNMP response packets	260

Setting the DSCP value for SSH protocol packets	260

Setting the DSCP value for Telnet protocol packets	261

Setting the DSCP value for the protocol packets sent to the log host	261

Added commands	262

dhcp client dscp	262

dhcp dscp	262

dns dscp	263

dns ipv6 dscp	263

dscp (IGMP-Snooping view)	264

dscp (MLD-Snooping view)	264

ftp client dscp	265

ftp client ipv6 dscp	265

ftp server dscp	266

ip http dscp	266

ipv6 dhcp client dscp	267

ipv6 dhcp dscp	267

ipv6 http dscp	268

nqa server tcp-connect tos	268

nqa server udp-echo tos	269

ntp-service dscp	269

radius dscp	270

radius ipv6 dscp	270

sftp client dscp	271

sftp client ipv6 dscp	271

snmp-agent packet response dscp	272

ssh client dscp	272

ssh client ipv6 dscp	273

ssh server dscp	273

ssh server ipv6 dscp	274

telnet client dscp	274

telnet client ipv6 dscp	275

telnet server dscp	275

telnet server ipv6 dscp	276

tftp client dscp	276

tftp client ipv6 dscp	277

tos (DHCP operation type view)	277

Modified commands	278

Modified command: info-center loghost	278

Modified command: ping ipv6	278

Modified command: snmp-agent target-host	278

Modified command: tracert	279

Modified command: tracert ipv6	279

Modified feature: Password configuration and display	279

Feature change description	279

Command changes	280

Modified command: bims-server	280

Modified command: certificate request mode	280

Modified command: cluster-local-user	281

Modified command: cluster-snmp-agent usm-user v3	281

Modified command: dldp authentication-mode	282

Modified command: ftp-server	282

Modified command: key (HWTACACS scheme view)	283

Modified command: key (RADIUS scheme view)	283

Modified command: mac-authentication user-name-format	284

Modified command: ntp-service authentication-keyid	284

Modified command: password (FTP operation type view)	285

Modified command: password (local user view)	285

Modified command: password (RADIUS-server user view)	286

Modified command: primary accounting (RADIUS scheme view)	286

Modified command: primary authentication (RADIUS scheme view)	287

Modified command: radius-server client-ip	287

Modified command: secondary accounting (RADIUS scheme view)	288

Modified command: secondary authentication (RADIUS scheme view)	288

Modified command: set authentication password	289

Modified command: snmp-agent usm-user v3	289

Modified command: super password	291

Modified feature: Task ID for IPv6 socket display	291

Feature change description	291

Command changes	291

Modified command: display ipv6 socket	291

Removed feature: Local user password display	292

Feature change description	292

Removed commands	292

local-user password-display-mode	292

A5120EI-CMW520-F2211	293

New feature: Critical VLAN	293

Overview	293

Configuring a 802.1X critical VLAN	295

Configuration guidelines	295

Configuration prerequisites	295

Configuration procedure	295

Configuring a MAC authentication critical VLAN	296

Configuration prerequisites	296

Configuration procedure	296

Configuring a RADIUS server probe	297

Command reference	297

dot1x critical vlan	297

dot1x critical recovery-action	298

mac-authentication critical vlan	299

primary authentication probe	300

secondary authentication probe	301

New feature: SCP	302

Overview	302

Configuring the switch as an SCP server	302

Configuring the switch as the SCP client	302

SCP client configuration example	303

SCP server configuration example	304

Command reference	305

scp	305

ssh user	306

New feature: Specifying the source interface for DNS packets	308

Specifying the source interface for DNS packets	308

Command reference	308

dns source-interface	308

New feature: MVRP	309

Overview	309

MRP	309

MVRP registration modes	311

Protocols and standards	312

MVRP configuration task list	312

Configuration prerequisites	312

Enabling MVRP	312

Configuration restrictions and guidelines	312

Configuration procedure	312

Configuring the MVRP registration mode	313

Configuring MRP timers	313

Enabling GVRP compatibility	314

Displaying and maintaining MVRP	315

Configuration example for MVRP in normal registration mode	315

Network requirements	315

Configuration procedure	316

Command reference	323

display mvrp running-status	323

display mvrp state	325

display mvrp statistics	326

display mvrp vlan-operation	329

mrp timer join	329

mrp timer leave	330

mrp timer leaveall	331

mrp timer periodic	332

mvrp global enable	332

mvrp enable	333

mvrp gvrp-compliance	334

mvrp registration	334

reset mvrp statistics	335

New feature: Enabling LLDP to automatically discover IP phones	335

Overview	335

Configuration prerequisites	336

Configuration procedure	336

Command reference	336

voice vlan track lldp	336

A5120EI-CMW520-R2210	337

New feature: Displaying information about the patch package	338

Displaying information about the patch package	338

Command reference	338

New feature: Displaying alarm information	338

Displaying alarm information	338

Command reference	339

New feature: Configuring jumbo frame support on an Ethernet interface	339

Configuring jumbo frame support on an Ethernet interface	339

Command reference	339

New feature: Restoring the default settings for the interface	339

Restoring the default settings for the interface	339

Command reference	339

New feature: Enabling MAC address roaming	339

Enabling MAC address roaming	339

Command reference	340

New feature: Assigning the port an aggregation priority	340

Assigning the port an aggregation priority	340

Command reference	340

New feature: Setting the minimum number of Selected ports for an aggregation group	340

Setting the minimum number of Selected ports for an aggregation group	340

Command reference	340

New feature: PVST	340

Configuring PVST	340

Command reference	341

New feature: Configuring TC snooping	341

Configuring TC snooping	341

Command reference	341

New feature: Setting the MTU for the VLAN interface	341

Setting the MTU for the VLAN interface	341

Command reference	342

New feature: Restoring the default operating mode of CDP-compatible LLDP	342

Restoring the default operating mode of CDP-compatible LLDP	342

Command reference	342

New feature: PoE power negotiation through Power Via MDI TLV	342

Configuring PoE power negotiation through Power Via MDI TLV	342

Command reference	342

Modified command: display lldp local-information	343

Modified command: display lldp neighbor-information	343

New feature: Configuring multicast ARP	345

Configuring multicast ARP	345

Command reference	345

New feature: Specifying the IP address range for the DHCP clients of a specified vendor	345

Specifying the IP address range for the DHCP clients of a specified vendor	345

Command reference	345

New feature: Specifying a server’s IP address for the DHCP client	345

Specifying a server’s IP address for the DHCP client	345

Command reference	345

New feature: Configuring DHCP packet rate limit	346

Configuring DHCP packet rate limit	346

Command reference	346

New feature: Configuring DHCP snooping support for sub-option 9 in Option 82	346

Configuring DHCP snooping support for sub-option 9 in Option 82	346

Command reference	346

New feature: Configuring TCP path MTU discovery	346

Configuring TCP path MTU discovery	346

Command reference	346

New feature: local ND proxy	347

Configuring local ND proxy	347

Command reference	347

New feature: Specifying the AFTR address	347

Specifying the AFTR address	347

Command reference	347

New feature: Displaying detailed information about neighbors	347

Displaying detailed information about neighbors	347

Command reference	347

New feature: Configuring the interface as an uplink interface and disabling it from learning ND snooping entries	348

Configuring the interface as an uplink interface and disabling it from learning ND snooping entries	348

Command reference	348

New feature: Configuring permanent static route	348

Configuring permanent static route	348

Command reference	348

New feature: Enabling the IGMP snooping & MLD snooping host tracking function	348

Enabling the IGMP snooping & MLD snooping host tracking function	348

Command reference	348

New feature: PIM snooping & IPv6 PIM snooping	349

Configuring PIM snooping & IPv6 PIM snooping	349

Command reference	349

New feature: Configuring rule range remark	349

Configuring rule range remark	349

Command reference	349

New feature: Configuring routing header type for an IPv6 ACL rule	349

Configuring routing header type for an IPv6 ACL	349

Command reference	349

New feature: Configuring byte-count or packet-based WFQ queuing	350

Configuring byte-count or packet-based WFQ queuing	350

Command reference	350

New feature: Configuring SP+WFQ queuing	350

Configuring SP+WFQ queuing	350

Command reference	350

New feature: Setting the validity time of the local user	350

Setting the validity time of the local user	350

Command reference	350

New feature: Specifying the local user as a guest or guest manager	351

Specifying the local user as a guest or guest manager	351

Command reference	351

New feature: Setting the guest attribute for a user group	351

Setting the guest attribute for a user group	351

Command reference	351

New feature: Authorizing a local user to use the Web service	351

Authorizing a local user to use the Web service	351

Command reference	351

New feature: Specifying ciphertext shared keys for RADIUS/HWTACACS servers	352

Specifying ciphertext shared keys for RADIUS/HWTACACS servers	352

Command reference	352

New feature: Specifying supported domain name delimiters	352

Specifying supported domain name delimiters	352

Command reference	352

New feature: Enabling inactivity aging	352

Enabling inactivity aging	352

Command reference	352

New feature: Enabling the dynamic secure MAC function	353

Enabling the dynamic secure MAC function	353

Command reference	353

New feature: Enabling SSL client weak authentication	353

Enabling SSL client weak authentication	353

Command reference	353

New feature: Setting the maximum number of IPv4/IPv6 source guard binding entries	353

Setting the maximum number of IPv4/IPv6 binding entries	353

Command reference	353

New feature: SAVI	353

Configuring SAVI	353

Command reference	354

New feature: Blacklist	354

Configuring blacklist	354

Command reference	354

New feature: Enabling Ethernet OAM remote loopback in user view and system view	354

Enabling Ethernet OAM remote loopback in user view and system view	354

Command reference	354

New feature: Restoring the default Ethernet OAM connection mode	354

Restoring the default Ethernet OAM connection mode	354

Command reference	354

New feature: Configuring the collaboration between Smart Link and CC of CFD	355

Configuring the collaboration between Smart Link and CC of CFD	355

Command reference	355

Modified feature: Improving the isolate-user-VLAN usability	355

Feature change description	355

Command changes	355

Modified command: port isolate-user-vlan promiscuous	355

Modified feature: Installing patches in one step	356

Feature change description	356

Command changes	356

Modified command: patch install	356

Modified feature: Displaying file or directory information	356

Feature change description	356

Command changes	356

Modified command: dir	356

Modified feature: Loopback interface numbering	357

Feature change description	357

Command changes	357

Modified command: interface loopback	357

Modified feature: Enabling address check	357

Feature change description	357

Command changes	357

Modified command: dhcp relay address-check enable	357

Modified feature: Enabling ND proxy	358

Feature change description	358

Command changes	358

Modified command: proxy-nd enable	358

Modified feature: ND snooping	358

Feature change description	358

Command changes	358

New command: ipv6 nd snooping enable global	358

New command: ipv6 nd snooping enable link-local	359

Modified feature: Displaying IPv6 FIB entries	359

Feature change description	359

Command changes	360

Modified command: display ipv6 fib	360

Modified feature: Displaying the IPv6 information of an interface	360

Feature change description	360

Command changes	360

Modified command: display ipv6 interface	360

Modified feature: Routing policy	361

Feature change description	361

Command changes	361

Modified command: route-policy	361

Modified feature: CFD	361

Feature change description	361

Command changes	361

Modified feature: Configuring the protected VLANs for the RRPP domain	362

Feature change description	362

Command changes	362

Modified command: protected-vlan	362

Modified feature: Configuring the protected VLANs for a smart link group	362

Feature change description	362

Command changes	362

Modified command: protected-vlan	362

Modified feature: Enabling traps globally	363

Feature change description	363

Command changes	363

Modified command: snmp-agent trap enable	363

Modified feature: Configuring IP source guard	363

Feature change description	363

Command changes	363

Modified command: display ip source binding	363

Modified command: display ipv6 source binding	364

Modified command: ip source binding (system view)	364

Modified command: ipv6 source binding (system view)	364

Modified command: ip source binding (interface view)	365

Modified command: ipv6 source binding (interface view)	365

Modified command: ip verify source	365

Modified command: ipv6 verify source	366

Removed command: display user-bind	366

Removed command: user-bind uplink	366

A5120EI-CMW520-R2208	367

New feature: Setting the age timer for ND entries in stale state	367

Setting the age timer for ND entries in stale state	367

Command reference	367

S5120EI-CMW520-R2208	368

A5120EI-CMW520-R2222P11

This release has no feature changes.

# A5120EI-CMW520-R2222P09

This release has the following changes:

- Modified feature: RADIUS Calling-Station-ID attribute value for the login service

- Modified feature: Port security need to know feature

  - Modified feature: RADIUS Calling-Station-ID attribute value for the login service

    - Feature change description

Before modification, the device fills 0 in the RADIUS Calling-Station-ID attribute for the login service.

After modification, the device fills the IP address of each login user in the RADIUS Calling-Station-ID attribute for the login service.

    - Command changes

None

  - Modified feature: Port security need to know feature

Feature change description

In this version, the ntkauto mode was added to the need to know (NTK) feature.

Command changes

      - Modified command: port-security ntk-mode

Old syntax

port-security ntk-mode { ntk-withbroadcasts | ntk-withmulticasts | ntkonly }

undo port-security ntk-mode

New syntax

port-security ntk-mode { ntk-withbroadcasts | ntk-withmulticasts | ntkauto | ntkonly }

undo port-security ntk-mode

Views

Ethernet interface view

Change description

The ntkauto keyword was added. If you specify this keyword, the device can send the following packets out of a port security-enabled port only when the port has online users:

Broadcast packets.

Multicast packets.

Unicast packets whose destination MAC addresses have passed authentication.

# A5120EI-CMW520-R2222P08

This release has no feature changes.

# A5120EI-CMW520-R2222P07

This release has the following changes:

- New feature: Configuring the action a port takes after it receives an Ethernet OAM event from the remote end

- New feature: Configuring the action a port takes after it receives an Ethernet OAM event from the remote end

    - Configuring the action a port takes after it receives an Ethernet OAM event from the remote end

This feature enables a port to log events and automatically terminate the OAM connection and set the link state to down.

To configure the action the port takes after it receives an Ethernet OAM event from the remote end:

| Step | Command | Remarks |
| --- | --- | --- |
| ##### Enter system view. | system-view | N/A |
| ##### Enter Layer 2 Ethernet port view. | interface interface-type interface-number | N/A |
| ##### Configure the action the port takes after it receives an Ethernet OAM event from the remote end. | oam remote-failure { connection-expired ¦ critical-event ¦ dying-gasp ¦ link-fault } action error-link-down | By default, the port only logs the Ethernet OAM event it receives from the remote end. |

    - Command reference

    - oam remote-failure action

Use oam remote-failure action to configure the action the port takes after it receives an Ethernet OAM event from the remote end.

Use undo oam remote-failure action to remove the configuration.

Syntax

oam remote-failure { connection-expired | critical-event | dying-gasp | link-fault } action error-link-down

undo oam remote-failure { connection-expired | critical-event | dying-gasp | link-fault } action error-link-down

Default

The port only logs the Ethernet OAM event it receives from the remote end.

Views

Layer 2 Ethernet port view

Default command level

2: System level

Parameters

connection-expired: Specifies a connection timeout fault.

critical-event: Specifies a critical fault.

dying-gasp: Specifies a fatal fault.

link-fault: Specifies a link fault.

error-link-down: Terminates the OAM connection and sets the link state of the port to down.

Examples

# Configure Gigabitethernet 1/0/1 to terminate the OAM connection after it receives a critical event from the remote end, and set the link state of the interface to down.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname- Gigabitethernet1/0/1] oam remote-failure critical-event action error-link-down

# A5120EI-CMW520-R2222P05

This release has no feature changes.

# A5120EI-CMW520-R2222P02

This release has no feature changes.

# A5120EI-CMW520-R2222P01

This release has the following changes:

Removed feature: User profile

Removed feature: User profile

Feature change description

Removed the user profile feature. For information about this feature, see Security Configuration Guide of HP 5120 EI Switch Series Configuration Guides-Release 2220.

Removed commands

For information about the removed commands, see Security Command Reference of HP 5120 EI Switch Series Command References-Release 2220.

# A5120EI-CMW520-R2222

This release has the following changes:

Modified feature: Support for forwarding PTP multicast packets of Layer 2 multicast

Modified feature: Uploading IPv6 addresses for 802.1X and MAC authentication users

Modified feature: Support for forwarding PTP multicast packets of Layer 2 multicast

Feature change description

Before modification: Layer 2 multicast does not support forwarding PTP multicast packets.

After modification: Layer 2 multicast supports forwarding PTP multicast packets.

Modified feature: Uploading IPv6 addresses for 802.1X and MAC authentication users

Feature change description

Before modification: The device does not support uploading the IPv6 addresses of 802.1X and MAC authentication users to the RADIUS server.

After modification: After a user passes 802.1X or MAC authentication, the device obtains the IPv6 address of the user through DHCPv6 snooping or ND snooping entries. Then, the device uploads the IPv6 address to the RADIUS server through the Framed-IPv6-Address RADIUS attribute in realtime accounting packets.

# A5120EI-CMW520-R2221P30

This release has the following changes:

Modified feature: Specifying file name for storing DHCP snooping entries on a remote server

Modified feature: Specifying file name for storing DHCP snooping entries on a remote server

Feature change description

In earlier releases, the DHCP snooping entries are saved to a local file in the flash memory. Frequent refreshing might damage this file.

In this release, the device supports storing the DHCP snooping entries on a remote server. You can configure the login username and password when you specify the remote file name.

If you want to back up the entries, save the DHCP snooping entries on a remote server as a best practice.

Command changes

Modified command: dhcp-snooping binding database filename

Old syntax

dhcp-snooping binding database filename filename

New syntax

dhcp-snooping binding database filename { filename | url url [ username username [ password { cipher | simple } string ] ] }

Views

System view

Change description

After modification, the device supports configuring the URL of a remote file and configuring the username and password for login to the remote server.

# A5120EI-CMW520-R2221P29

This release has the following changes:

New feature: Including user IP addresses in realtime accounting packets for MAC authentication users with dynamic IP addresses

New feature: Ignoring the ingress ports of ARP packets during user validity check

  - New feature: Including user IP addresses in realtime accounting packets for MAC authentication users with dynamic IP addresses

This feature enables the device to add user IP addresses to realtime accounting packets for the MAC authentication users who obtain IP addresses dynamically from a DHCP server. For this feature to work, you must configure MAC authentication and DHCP snooping.

The device sends a DHCP-REQUEST packet to the DHCP server after an MAC authentication users passes authentication. Upon receiving a DHCP-ACK packet from the DHCP server, the device generates a DHCP snooping entry and an IPSG binding entry for the user. The MAC authentication module obtains the user IP address from the IPSG binding entry. The device includes the obtained IP address in the realtime accounting packets for the user.

New feature: Ignoring the ingress ports of ARP packets during user validity check

Configuring ARP detection to ignore the ingress ports of ARP packets during user validity check

ARP detection performs user validity check on ARP packets from ARP untrusted interfaces. User validity check compares the sender IP and sender MAC in the received ARP packet with static IP source guard bindings, DHCP snooping entries, and 802.1X security entries. In addition, user validity check also compares the ingress port of the ARP packet with the port in the entries. If no matching port is found, the ARP packet is discarded.

You can enable this feature to ignore the ingress ports of ARP packets during user validity check.

To ignore the ingress ports of ARP packets during user validity check:

| Step | Command | Remarks |
| --- | --- | --- |
| ##### Enter system view. | system-view | N/A |
| - Ignore the ingress ports of ARP packets during user validity check. | arp detection port-match-ignore | Required. By default, the ingress ports of ARP packets are not ignored during user validity check. |

Command reference

arp detection port-match-ignore

Use arp detection port-match-ignore to ignore the ingress ports of ARP packets during user validity check.

Use undo arp detection port-match-ignore to remove the configuration.

Syntax

arp detection port-match-ignore

undo arp detection port-match-ignore

Default

The ingress ports of ARP packets are not ignored during user validity check.

Views

System view

Default command level

2: System view

Examples

# Ignore the ingress ports of ARP packets during user validity check.

<Sysname> system-view

[Sysname] arp detection port-match-ignore

# A5120EI-CMW520-R2221P27

This release has the following changes:

New feature: Authorization VLAN auto-tagging for MAC authentication

Modified feature: Confining RADIUS Vendor-Specific extended attributes to a specific vendor

New feature: Authorization VLAN auto-tagging for MAC authentication

Enabling authorization VLAN auto-tagging for MAC authentication

This feature adds a port to the authorization VLAN as a tagged or untagged member based on the tagged status of packets that triggered MAC authentication on the port.

This feature takes effect only on hybrid ports with MAC-based VLAN enabled.

The VLAN tag configuration set by this feature has higher priority than the server setting of whether to assign a tagged VLAN or not. However, if the server assigns a PVID, this feature does not take effect. Whether the port will add to the PVID as a tagged or untagged member depends on the server setting.

To enable authorization VLAN auto-tagging for MAC authentication:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Ethernet interface view. | interface interface-type interface-number | N/A |
| - Enable authorization VLAN auto-tagging for MAC authentication. | mac-authentication auto-tag [ ignore-config ] | By default, authorization VLAN auto-tagging for MAC authentication is disabled. If you do not specify the ignore-config keyword, this command does not take effect if the authorization VLAN is specified by the port hybrid vlan command. |

Command reference

mac-authentication auto-tag

Syntax

mac-authentication auto-tag [ ignore-config ]

undo mac-authentication auto-tag

Views

Ethernet interface view

Default command level

2: System level

Parameters

ignore-config: Ignores VLAN tag configuration on a port. If you do not specify this keyword, this command does not take effect if the authorization VLAN is specified by the port hybrid vlan command. Whether the port adds to the authorization VLAN as a tagged or untagged member depends on the port configuration.

Usage guidelines

Use mac-authentication auto-tag to enable authorization VLAN auto-tagging for MAC authentication.

Use undo mac-authentication auto-tag to disable authorization VLAN auto-tagging for MAC authentication.

By default, authorization VLAN auto-tagging for MAC authentication is disabled.

This command enables the device to add a port to the authorization VLAN as a tagged or untagged member based on the tagged status of packets that triggered MAC authentication.

This command takes effect only on hybrid ports with MAC-based VLAN enabled.

The VLAN tag configuration set by this command has higher priority than the server setting of whether to assign a tagged VLAN. However, if the server assigns a PVID, this command does not take effect. Whether the port will add to the PVID as a tagged or untagged member depends on the server setting.

Examples

# Enable authorization VLAN auto-tagging for MAC authentication.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication auto-tag ignore-config

Modified feature: Confining RADIUS Vendor-Specific extended attributes to a specific vendor

Feature change description

Support for confining RADIUS Vendor-Specific extended attributes to a specific vendor was added. With this feature, the device supports only a specific vendor for the Vendor-Specific attribute field to be sent to a RADIUS server.

Command changes

Modified command: service-type

Old syntax

server-type { extended | standard }

undo server-type

New syntax

server-type { extended [ vendor vendor-id ] | standard }

undo server-type

Views

RADIUS scheme view

Change description

The vendor vendor-id option was added to this command. This option confines Vendor-Specific extended attributes to a specific vendor. The value range for the vendor-id argument is 1 to 65535. The device supports the vendor IDs of 9, 43, 311, 2011, and 25506 in the current software version. If you do not specify a vendor ID with the extended keyword, the device can send Vendor-Specific extended attributes of all supported vendors to a RADIUS server.

# A5120EI-CMW520-R2221P25

This release has the following changes:

New feature: Configuring periodic MAC re-authentication

- New feature: Configuring periodic MAC re-authentication

  - Configuring periodic MAC re-authentication

The device re-authenticates online MAC authentication users on a port at the periodic re-authentication interval if the port is enabled with periodic MAC re-authentication. Periodic MAC re-authentication tracks the connection status of online users and updates the authorization attributes assigned by the server, such as the ACL and VLAN.

You can set the periodic re-authentication interval either in system view or in interface view by using the mac-authentication timer reauth-period command. A change to the periodic re-authentication timer applies to online users only after the old timer expires.

The device selects a periodic re-authentication timer for MAC re-authentication in the following order:

            - Server-assigned re-authentication timer.

            - Port-specific re-authentication timer.

            - Global re-authentication timer.

            - Default re-authentication timer.

To configure periodic MAC re-authentication:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Set the global periodic re-authentication timer. | mac-authentication timer reauth-period reauth-period-value | Optional. The default is 3600 seconds. |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| - Enable periodic MAC re-authentication. | mac-authentication re-authenticate | Required. By default, periodic MAC re-authentication is disabled on a port. |
| - Set the periodic re-authentication timer on the port. | mac-authentication timer reauth-period reauth-period-value | Optional. By default, no periodic re-authentication timer is set on a port. |

  - Command reference

    - New command: mac-authentication timer reauth-period (system view)

Use mac-authentication timer reauth-period to set the global periodic MAC re-authentication timer.

Use undo mac-authentication timer reauth-period to restore the default.

Syntax

mac-authentication timer reauth-period reauth-period-value

undo mac-authentication timer reauth-period

Default

The global periodic MAC re-authentication timer is 3600 seconds.

Views

System view

Default command level

2: System view

Parameters

reauth-period-value: Specifies the global periodic MAC re-authentication timer in seconds. The value range is 60 to 7200.

Usage guidelines

The device re-authenticates online MAC authentication users on a port at the specified periodic re-authentication interval if the port is enabled with periodic MAC re-authentication. To enable periodic MAC re-authentication on a port, use the mac-authentication re-authenticate command.

A change to the global periodic re-authentication timer applies to online users only after the old timer expires.

The device selects a periodic re-authentication timer for MAC re-authentication in the following order:

            - Server-assigned re-authentication timer.

            - Port-specific re-authentication timer.

            - Global re-authentication timer.

            - Default re-authentication timer.

Examples

# Set the global periodic MAC re-authentication timer to 150 seconds.

<Sysname> system-view

[Sysname] mac-authentication timer reauth-period 150

Related commands

- display mac-authentication

- mac-authentication re-authenticate

    - New command: mac-authentication re-authenticate

Use mac-authentication re-authenticate to enable the periodic MAC re-authentication feature on a port.

Use undo mac-authentication re-authenticate to disable the periodic MAC re-authentication feature on a port.

Syntax

mac-authentication re-authenticate

undo mac-authentication re-authenticate

Default

The periodic MAC re-authentication feature is disabled on a port.

Views

Layer 2 Ethernet interface view

Default command level

2: System view

Usage guidelines

Periodic MAC re-authentication enables the access device to periodically authenticate online MAC authentication users on a port. This feature tracks the connection status of online users and updates the authorization attributes assigned by the server, such as the ACL and VLAN.

To set the periodic re-authentication interval, use the mac-authentication timer reauth-period command in system view or Layer 2 Ethernet interface view.

Examples

# Enable the periodic MAC re-authentication feature on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication re-authenticate

Related commands

- display mac-authentication

- mac-authentication timer

    - New command: mac-authentication timer reauth-period (interface view)

Use mac-authentication timer reauth-period to set the port-specific periodic MAC re-authentication timer.

Use undo mac-authentication timer reauth-period to restore the default.

Syntax

mac-authentication timer reauth-period reauth-period-value

undo mac-authentication timer reauth-period

Default

No port-specific periodic MAC re-authentication timer is set for MAC re-authentication.

Views

Layer 2 Ethernet interface view

Default command level

2: System view

Parameters

reauth-period-value: Specifies the port-specific periodic MAC re-authentication timer in seconds. The value range is 60 to 7200.

Usage guidelines

The device re-authenticates online MAC authentication users on a port at the specified periodic re-authentication interval if the port is enabled with periodic MAC re-authentication. To enable periodic MAC re-authentication on a port, use the mac-authentication re-authenticate command.

A change to the port-specific periodic re-authentication timer applies to online users only after the old timer expires.

The device selects a periodic re-authentication timer for MAC re-authentication in the following order:

            - Server-assigned re-authentication timer.

            - Port-specific re-authentication timer.

            - Global re-authentication timer.

            - Default re-authentication timer.

Examples

# Set the periodic MAC re-authentication timer to 90 seconds on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication timer reauth-period 90

Related commands

- display mac-authentication

- mac-authentication re-authenticate

    - Modified command: display mac-authentication

Syntax

display mac-authentication [ interface interface-list ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

New fields were added to the command output for the periodic MAC re-authentication feature.

Examples

# Display MAC authentication information.

<Sysname> display mac-authentication

MAC address authentication is enabled.

User name format is fixed account

Fixed username: aaa

Fixed password: ******

Offline detect period is 300s

Quiet period is 60s

Server response timeout value is 100s

Reauthentication period is 3600s

Guest vlan reauthentication timeout value is 30s

The max allowed user number is 1024 per slot

Current user number amounts to 0

Current domain is bbb

Silent MAC User info:

MAC Addr         From Port                    Port Index

GigabitEthernet1/0/1 is link-up

MAC address authentication is disabled

Periodic reauthentication is enabled

Reauthentication period is not configured

Authenticate success: 0, failed: 0

Max number of on-line users is 0

Current online user number is 0

MAC Addr         Authenticate State           Auth Index

GigabitEthernet1/0/2 is link-down

MAC address authentication is disabled

Periodic reauthentication is disabled

Authenticate success: 0, failed: 0

Max number of on-line users is 0

Current online user number is 0

MAC Addr         Authenticate State           Auth Index

Command output

| Field | Description |
| --- | --- |
| Periodic reauthentication is enabled/disabled | Status of the MAC re-authentication feature on a port. |
| Reauthentication period | MAC re-authentication interval. |

# A5120EI-CMW520-R2221P22

This release has the following changes:

New feature: Enabling sending of ICMPv6 redirect messages

Modified feature: Disabling advertising prefix information in RA messages

New feature: Enabling sending of ICMPv6 redirect messages

Enabling sending of ICMPv6 redirect messages

When a device receives a large number of attack packets that require the device to send ICMPv6 redirect packets, the device's performance is degraded for processing these packets. To protect the device from such attacks, you can use the undo form of the following command to disable sending of ICMPV6 redirect packets.

To enable sending of ICMPv6 redirect messages:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view | system-view | N/A |
| - Enable sending of ICMPv6 redirect messages | ipv6 redirects enable | Optional. By default, this function is disabled. |

    - Command reference

      - New command: ipv6 redirects enable

Use ipv6 redirects enable to enable sending of ICMPv6 redirect packets.

Use undo ipv6 redirects to disable sending of ICMPv6 redirect packets.

Syntax

ipv6 redirects enable

undo ipv6 redirects

Default

Sending of ICMPv6 redirect packets is disabled.

Views

System view

Default command level

System level

Examples

# Enable sending of ICMPv6 redirect packets.

<Sysname> system-view

[Sysname] ipv6 redirects enable

Modified feature: Disabling advertising prefix information in RA messages

    - Feature change description

The no-advertise keyword was added to disable the device from advertising the prefix specified in the ipv6 nd ra prefix command.

    - Command changes

    - Modified command: ipv6 nd ra prefix

Old syntax

ipv6 nd ra prefix { ipv6-prefix prefix-length | ipv6-prefix/prefix-length } valid-lifetime preferred-lifetime [ no-autoconfig | off-link ] *

undo ipv6 nd ra prefix { ipv6-prefix | ipv6-prefix/prefix-length }

New syntax

ipv6 nd ra prefix { ipv6-prefix prefix-length | ipv6-prefix/prefix-length } { valid-lifetime preferred-lifetime [ no-autoconfig | off-link ] * | no-advertise }

undo ipv6 nd ra prefix { ipv6-prefix | ipv6-prefix/prefix-length }

Views

Interface view

Change description

Before modification: The device advertises the prefix specified in the ipv6 nd ra prefix command.

After modification: If the no-advertise keyword is specified, the device does not advertise the prefix specified in this command.

# A5120EI-CMW520-R2221P20

This release has the following changes:

New feature: MAC authentication voice VLAN

  - New feature: MAC authentication voice VLAN

    - Configuring a MAC authentication voice VLAN

You can configure a MAC authentication voice VLAN on a MAC authentication-enabled port that connects to voice terminals. The MAC authentication voice VLAN feature applies only to voice terminals that support VLAN-tagged packets.

The MAC authentication voice VLAN feature works with a remote authentication server. The device uses the following process to implement this feature:

Identifies a terminal as a voice device from the packet sent by the authentication server when the terminal passes MAC authentication. The authentication server identifies the terminal as a voice device by its OUI information, and then sends the terminal type information to the device.

Assigns the port to the configured voice VLAN as a tagged member and sends the voice VLAN information through an LLDP or CDP packet to the terminal.

      - Configuration prerequisites

Before you configure this feature, complete the following tasks:

- Enable MAC authentication on the port.

- Set the port type to hybrid or trunk, because the port is assigned to the MAC authentication voice VLAN as a tagged member. For information about port types, see VLAN in Layer 2—LAN Switching Configuration Guide.

- Configure LLDP or CDP compatibility on the device. For information about the LLDP and CDP compatibility features, see LLDP in Layer 2—LAN Switching Configuration Guide.

      - Configuration guidelines

When you configure a MAC authentication voice VLAN, follow these restrictions and guidelines:

- You can configure only one MAC authentication voice VLAN on a port. The MAC authentication voice VLANs on different ports can be different.

- A server-assigned authorization VLAN for a voice terminal takes precedence over the MAC authentication voice VLAN. The port will be assigned to the authorization VLAN if both VLANs coexist. For information about MAC authentication VLAN assignment, see MAC authentication in Security Configuration Guide.

- The MAC authentication voice VLAN feature cannot work with the RADIUS server provided by IMC.

      - Configuration procedure

To configure a MAC authentication voice VLAN on a port:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Ethernet interface view. | interface interface-type interface-number | N/A |
| - Configure a MAC authentication voice VLAN on the port. | mac-authentication voice vlan vlan-id | By default, no MAC authentication voice VLAN exists on a port. |

Example of Configuring 802.1X or MAC authentication for IP phones

Network requirements

As shown in Figure 1:

- Configure the switch to perform 802.1X and MAC authentication for the host and the IP phone.

- Configure the authentication server to assign authorization VLAN 100 to the host without tags and authorization VLAN 2 to the IP phone with tags, respectively.

After the host passes authentication, data packets from the host will be forwarded within VLAN 100 without tags. After the IP phone passes authentication, voice packets from the IP phone will be forwarded within VLAN 2 with tags.

          - Network diagram

Requirements analysis

To meet the network requirements, complete the following tasks:

- For users to use 802.1X and MAC authentication flexibly, enable port security on the switch, and set the port security mode to macAddressOrUserLoginSecureExt.

- For voice packets to pass through GigabitEthernet 1/0/1 with tags and data packets to pass through GigabitEthernet 1/0/1 without tags, configure the port as a hybrid port.

- To ensure a correct exchange of 802.1X EAPOL packets, configure the switch to send 802.1X EAPOL packets without tags on the port that connects to the IP phone.

- For the switch to notify the IP phone of the specified voice VLAN information through LLDP or CDP packets, configure LLDP or CDP compatibility on the device.

Configuration restrictions and guidelines

When you configure 802.1X and MAC authentication for the IP phone, follow these restrictions and guidelines:

- An authentication server such as ACS is required for this configuration example. The authentication server cannot be the RADIUS server provided by IMC.

- If the IP phone can automatically record voice VLANs, you must use the port hybrid vlan vlan-id tagged command on GigabitEthernet 1/0/1. As a result, the port will be assigned to the specified voice VLANs that are automatically recorded by the IP phone and can send the tagged packets of these voice VLANs.

- By default, the switch encapsulates LLDP packets in Ethernet II format. To ensure correct communication with the IP phone, make sure the switch encapsulates LLDP packets on the IP phone-connected port in the same encapsulation format with the IP phone. For example, if a Cisco IP phone uses SNAP format, you must set the encapsulation format for LLDP packets to SNAP by using the lldp encapsulation snap command.

Configuration procedure

            - Assign IP addresses to the host, the IP phone, the switch, and the authentication server. Make sure they can reach one another. (Details not shown.)

            - Perform the following tasks on the authentication server:

  - Configure the authentication server and add user accounts for 802.1X users and MAC authentication users. (Details not shown.)

  - Configure the server to support the device-traffic-class=voice attribute. (Details not shown.)

  - Configure the server to assign authorization VLAN 100 to the host without tags and authorization VLAN 2 to the IP phone with tags, respectively. (Details not shown.)

Authentication server configuration varies by server type. For more information, see related server configuration guideline.

            - Configure VLAN settings on the switch:

# Create VLAN 2 and VLAN 100.

<Switch> system-view

[Switch] vlan 2

[Switch-vlan2] quit

[Switch] vlan 100

[Switch-vlan100] quit

# Disable the voice VLAN security mode.

[Switch] undo voice vlan security enable

# Configure GigabitEthernet 1/0/1 as a hybrid port.

[Switch] interface gigabitethernet 1/0/1

[Switch-GigabitEthernet1/0/1] port link-type hybrid

# Assign GigabitEthernet 1/0/1 to VLAN 2 as a tagged member and to VLAN 100 and VLAN 1 as an untagged member.

[Switch-GigabitEthernet1/0/1] port hybrid vlan 2 tagged

[Switch-GigabitEthernet1/0/1] port hybrid vlan 100 untagged

[Switch-GigabitEthernet1/0/1] port hybrid vlan 1 untagged

[Switch-GigabitEthernet1/0/1] quit

          - Configure an authentication scheme and domain on the switch:

# Create a RADIUS scheme named radius1 and enter its view.

[Switch] radius scheme radius1

# Specify the IP addresses of the primary authentication and accounting servers.

[Switch-radius-radius1] primary authentication 10.1.1.1

[Switch-radius-radius1] primary accounting 10.1.1.1

# Specify the shared key between the switch and the authentication server.

[Switch-radius-radius1] key authentication key

# Specify the shared key between the switch and the accounting server.

[Switch-radius-radius1] key accounting key

# Exclude the ISP domain names from the usernames sent to the RADIUS servers.

[Switch-radius-radius1] user-name-format without-domain

[Switch-radius-radius1] quit

# Create an ISP domain named acs, and apply the RADIUS scheme radius1 to the ISP domain for authentication, authorization, and accounting.

[Switch] domain acs

[Switch-isp-acs] authentication lan-access radius-scheme radius1

[Switch-isp-acs] authorization lan-access radius-scheme radius1

[Switch-isp-acs] accounting lan-access radius-scheme radius1

# Enable the accounting optional feature for users in the domain acs.

[Switch-isp-acs] accounting optional

[Switch-isp-acs] quit

Configure authentication globally on the switch:

# Enable port security.

[Switch] port-security enable

# Specify the domain acs as the authentication domain for MAC authentication.

[Switch] mac-authentication domain acs

# Configure MAC authentication to use MAC-based accounts. Each MAC address is in the hexadecimal notation with hyphens, and letters are in lower case.

[Switch] mac-authentication user-name-format mac-address with-hyphen lowercase

Configure authentication on GigabitEthernet 1/0/1 of the switch:

# Set the port security mode to macAddressOrUserLoginSecureExt.

[Switch] interface gigabitethernet 1/0/1

[Switch-GigabitEthernet1/0/1] port-security port-mode userlogin-secure-or-mac-ext

# Specify the domain acs as the mandatory 802.1X authentication domain.

[Switch-GigabitEthernet1/0/1] dot1x mandatory-domain acs

# Disable 802.1X online user handshake.

[Switch-GigabitEthernet1/0/1] undo dot1x handshake

# Disable the 802.1X multicast trigger feature, and enable the 802.1X unicast trigger feature.

[Switch-GigabitEthernet1/0/1] undo dot1x multicast-trigger

[Switch-GigabitEthernet1/0/1] dot1x unicast-trigger

# Send 802.1X EAPOL packets without tags.

[Switch-GigabitEthernet1/0/1] dot1x eapol untag

# Configure VLAN 2 as the 802.1X voice VLAN.

[Switch-GigabitEthernet1/0/1] dot1x voice vlan 2

# Configure VLAN 2 as the MAC authentication voice VLAN.

[Switch-GigabitEthernet1/0/1] mac-authentication voice vlan 2

[Switch-GigabitEthernet1/0/1] quit

Configure LLDP:

# Enable LLDP globally.

[Switch] lldp enable

# Enable global CDP compatibility.

[Switch] lldp compliance cdp

# Configure CDP-compatible LLDP to operate in TxRx mode on GigabitEthernet 1/0/1.

[Switch] interface gigabitethernet 1/0/1

[Switch-GigabitEthernet1/0/1] lldp compliance admin-status cdp txrx

[Switch-GigabitEthernet1/0/1] quit

    - Command reference

      - mac-authentication voice vlan

Use mac-authentication voice vlan to configure a MAC authentication voice VLAN on a port.

Use undo mac-authentication voice vlan to restore the default.

Syntax

mac-authentication voice vlan vlan-id

undo mac-authentication voice vlan

Default

No MAC authentication voice VLAN exists on a port.

Views

Ethernet interface view

Default command level

2: System level

Parameters

vlan-id: Specifies a voice VLAN by its ID in the range of 1 to 4094. The VLAN must have been created.

Examples

# Configure VLAN 20 as the MAC authentication voice VLAN on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication voice vlan 20

# A5120EI-CMW520-R2221P18

This release has the following changes:

New feature: Sending EAP-Success packets to 802.1X users in critical VLAN

New feature: Sending EAP-Success packets to 802.1X users in critical VLAN

Configuring the device to send EAP-Success packets to 802.1X users in critical VLAN

This feature allows specific 802.1X users in the critical VLAN to pass re-authentication directly when the device detects a reachable server. The device sends EAP-Success packets to the 802.1X clients that cannot respond to the EAP-Request packets of the device (for example, the Windows built-in 802.1X client).

The feature takes effect only after the dot1x critical recovery-action reinitialize command is configured on the port.

To configure the device to send EAP-Success packets to users in the 802.1X critical VLAN:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter Ethernet interface view. | interface interface-type interface-number | N/A |
| ##### Configure the 802.1X critical VLAN on the port. | dot1x critical vlan vlan-id | Required. By default, no 802.1X critical VLAN is configured. Different ports can be configured with different critical VLANs, and one port can only be configured with a maximum of one critical VLAN. |
| ##### Configure the port to trigger 802.1X re-authentication on detection of an active authentication server for users in the critical VLAN. | dot1x critical recovery-action reinitialize | Required. By default, when a reachable server is detected, the system removes the port or 802.1X users from the critical VLAN without triggering authentication. |
| ##### Configure the device to send EAP-Success packets to 802.1X users in the critical VLAN on the port. | dot1X critical eapol | Required. By default, the device does not send EAP-Success packets to 802.1X users in the critical VLAN. |

Command reference

dot1x critical eapol

Use dot1x critical eapol to configure the device to send EAP-Success packets to 802.1X users in the critical VLAN.

Use undo dot1x critical eapol to restore the default.

Syntax

dot1x critical eapol

undo dot1x critical eapol

Default

The device does not send EAP-Success packets to 802.1X users in the critical VLAN.

Views

Layer 2 Ethernet interface view

Default command level

2: System level

Examples

# Configure GigabitEthernet 1/0/1 to send EAP-Success packets to 802.1X users in the critical VLAN.

<Sysname> system-view

[Sysname] interface gigabitethernet1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x critical eapol

# A5120EI-CMW520-R2221P15

This release has no feature changes.

# A5120EI-CMW520-R2221P12

This release has the following changes:

- New feature: Login delay

- Modified feature: IPv6 address with a 127-bit prefix length

New feature: Login delay

  - Enabling the login delay

The login delay feature delays the device to accept a login request from a user after the user fails a login attempt. This feature can slow down login dictionary attacks.

To enable the login delay:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable the login delay feature. | attack-defense login reauthentication-delay seconds | By default, the login delay feature is disabled. |

Command reference

    - attack-defense login reauthentication-delay

Syntax

attack-defense login reauthentication-delay seconds

undo attack-defense login reauthentication-delay

Views

System view

Default command level

2: System level

Parameters

seconds: Sets the delay period in seconds, in the range of 4 to 60.

Description

Use attack-defense login reauthentication-delay to enable the login delay feature.

Use undo attack-defense login reauthentication-delay to restore the default.

By default, the login delay feature is disabled. The device does not delay accepting a login request from a user who has failed a login attempt.

Examples

# Enable the login delay feature and set the delay period to 5 seconds.

<Sysname> system-view

[Sysname] attack-defense login reauthentication-delay 5

Modified feature: IPv6 address with a 127-bit prefix length

Feature change description

Before modification, you cannot execute the ipv6 address command to configure an IPv6 global unicast address in the form of XXX::2/127. The system identifies IPv6 address in this form as an anycast address.

After modification:

- You can use the ipv6 address command to configure an IPv6 global unicast address in the form of XXX::2/127.

- The system does not support any IPv6 anycast address with the 127-bit prefix length.

Command changes

None.

# A5120EI-CMW520-R2221P10

This release has the following changes:

New feature: Support for NTP configuration in IPv6 networks

New feature: Support for NTP configuration in IPv6 networks

Configuring NTP in IPv6 networks

You can configure NTP in IPv6 networks.

Command reference

New command: display ntp-service ipv6 sessions

Syntax

display ntp-service ipv6 sessions [ verbose ] [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

verbose: Displays detailed information about all IPv6 NTP sessions. If you do not specify this keyword, the command only displays brief information about the IPv6 NTP sessions.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use display ntp-service ipv6 sessions to display information about all IPv6 NTP sessions.

Examples

# Display brief information about all IPv6 NTP sessions.

<Sysname> display ntp-service ipv6 sessions

Notes: 1 source(master), 2 source(peer), 3 selected, 4 candidate, 5 configured.

Source:   [125]3000::32

Reference: 127.127.1.0           Clock stratum: 2

Reachabilities: 1                Poll interval: 64

Last receive time: 6             Offset: -0.0

Roundtrip delay: 0.0             Dispersion: 0.0

Total sessions: 1

Command output

| Field | Description |
| --- | --- |
| [12345] | - 1—Clock source selected by the system (the current reference source). It has a system clock stratum level less than or equal to 15. - 2—The stratum level of the clock source is less than or equal to 15. - 3—The clock source has survived the clock selection algorithm. - 4—The clock source is a candidate clock source. - 5—The clock source was created by a command. |
| Source | IPv6 address of the NTP server. If this field displays ::, the IPv6 address of the NTP server has not been resolved successfully. |
| Reference | Reference clock ID of the NTP server: - If the reference clock is the local clock, the value of this field is related to the value of the Clock stratum field: - When the value of the Clock stratum field is 0 or 1, this field displays Local. - When the Clock stratum field has another value, this field displays the MD5 digest value of the first 32 bits of the IPv6 address. The MD5 digest value is in dotted decimal format. - If the reference clock is the clock of another device on the network, this field displays the MD5 digest value of the first 32 bits of the IPv6 address. The MD5 digest value is in dotted decimal format. If this field displays INIT, the local device has not established a connection with the NTP server. |
| Clock stratum | Stratum level of the NTP server, which determines the clock accuracy. The value is in the range of 1 to 16. A lower stratum level represents higher clock accuracy. A stratum 16 clock is not synchronized and cannot be used as a reference clock. |
| Reachabilities | Reachability count of the NTP server. 0 indicates that the NTP server is unreachable. |
| Poll interval | Polling interval in seconds. It is the maximum interval between successive NTP messages. |
| Last receive time | Length of time from when the last NTP message was received or when the local clock was last updated to the current time. Time is in seconds by default. - If the time length is greater than 2048 seconds, it is displayed in minutes. - If the time length is greater than 300 minutes, it is displayed in hours. - If the time length is greater than 96 hours, it is displayed in days. - If the time length is greater than 999 days, it is displayed in years. If the time when the most recent NTP message was received or when the local clock was updated most recently is behind the current time, a hyphen (-) is displayed. |
| Offset | Offset of the system clock relative to the reference clock, in milliseconds. |
| Roundtrip delay | Roundtrip delay from the local device to the clock source, in milliseconds. |
| Dispersion | Maximum error of the system clock relative to the reference source. |
| Total sessions | Total number of associations. |

New command: ntp-service ipv6 access

Syntax

ntp-service ipv6 access { peer | query | server | synchronization } acl-number

undo ntp-service ipv6 access { peer | query | server | synchronization }

View

System view

Default level

3: Manage level

Parameters

peer: Permits full access. This level of right permits the peer devices to perform synchronization and control query to the local device and also permits the local device to synchronize its clock to that of a peer device. Control query refers to query of NTP status information, such as alarm information, authentication status, and clock source information.

query: Permits control query. This level of right permits the peer devices to perform control query to the NTP service on the local device but does not permit a peer device to synchronize its clock to that of the local device.

server: Permits server access and query. This level of right permits the peer devices to perform synchronization and control query to the local device but does not permit the local device to synchronize its clock to that of a peer device.

synchronization: Permits server access only. This level of right permits a peer device to synchronize its clock to that of the local device but does not permit the peer devices to perform control query.

acl-number: Specifies a basic ACL number in the range of 2000 to 2999.

Description

Use ntp-service ipv6 access to configure the access-control right for the peer devices to access the IPv6 NTP services of the local device.

Use undo ntp-service ipv6 access to remove the configured IPv6 NTP service access-control right to the local device.

By default, the access-control right for the peer devices to access the IPv6 NTP services of the local device is set to peer.

From the highest NTP service access-control right to the lowest one are peer, server, synchronization, and query. When a device receives an NTP request, it matches against the access-control right in this order and uses the first matched right. If no matched right is found, the device drops the NTP request.

The ntp-service ipv6 access command provides only a minimum degree of security protection. A more secure method is identity authentication. The related command is ntp-service authentication enable.

Before specifying an ACL number in the ntp-service ipv6 access command, make sure you have already created and configured this ACL.

Examples

# Configure the peer devices on subnet 2001::1 to have full access to the local device.

<Sysname> system-view

[Sysname] acl ipv6 number 2001

[Sysname-acl6-basic-2001] rule permit source 2001::1 64

[Sysname-acl6-basic-2001] quit

[Sysname] ntp-service ipv6 peer acl 2001

New command: ntp-service ipv6 dscp

Syntax

ntp-service ipv6 dscp dscp-value

undo ntp-service ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: Specifies the Differentiated Services Code Point (DSCP) value for IPv6 NTP messages, in the range of 0 to 63.

Description

Use the ntp-service ipv6 dscp command to set the DSCP value for IPv6 NTP messages.

Use the undo ntp-service ipv6 dscp command to restore the default.

By default, the DSCP value for IPv6 NTP messages is 56.

Examples

# Set the DSCP value to 30 for IPv6 NTP messages.

<Sysname> system-view

[Sysname] ntp-service ipv6 dscp 30

New command: ntp-service ipv6 in-interface disable

Syntax

ntp-service ipv6 in-interface disable

undo ntp-service ipv6 in-interface disable

View

VLAN interface view

Default level

3: Manage level

Parameters

None

Description

Use ntp-service ipv6 in-interface disable to disable an interface from receiving IPv6 NTP messages.

Use undo ntp-service ipv6 in-interface disable to restore the default.

By default, all interfaces are enabled to receive IPv6 NTP messages.

Examples

# Disable VLAN-interface 1 from receiving IPv6 NTP messages.

<Sysname> system-view

[Sysname] interface vlan-interface 1

[Sysname-Vlan-interface1] ntp-service ipv6 in-interface disable

New command: ntp-service ipv6 multicast-client

Syntax

ntp-service ipv6 multicast-client ipv6-address

undo ntp-service ipv6 multicast-client ipv6-address

View

VLAN interface view

Default level

3: Manage level

Parameters

ipv6-address: Specifies an IPv6 multicast IP address. An IPv6 broadcast client and an IPv6 broadcast server must be configured with the same multicast address.

Description

Use ntp-service ipv6 multicast-client to configure the device to operate in IPv6 NTP multicast client mode and use the current interface to receive IPv6 NTP multicast packets.

Use undo ntp-service ipv6 multicast-client to remove the configuration.

By default, the device does not operate in any NTP operation mode.

Examples

# Configure the device to operate in IPv6 multicast client mode and receive IPv6 NTP multicast messages with the destination FF21::1 on VLAN-interface 1.

<Sysname> system-view

[Sysname] interface vlan-interface 1

[Sysname-Vlan-interface1] ntp-service ipv6 multicast-client ff21::1

New command: ntp-service ipv6 multicast-server

Syntax

ntp-service ipv6 multicast-server [ ipv6-address ] [ authentication-keyid keyid | ttl ttl-number ] *

undo ntp-service ipv6 multicast-server [ ipv6-address ]

View

VLAN interface view

Default level

3: Manage level

Parameters

ipv6-address: Specifies an IPv6 multicast IP address. An IPv6 multicast client and server must be configured with the same multicast address.

authentication-keyid keyid: Specifies the key ID to be used for sending multicast messages to multicast clients, where keyid is in the range of 1 to 4294967295. This parameter is not meaningful if authentication is not required.

ttl ttl-number: Specifies the TTL of NTP multicast messages. The value range for the ttl-number argument is 1 to 255, and the default is 16.

Description

Use ntp-service ipv6 multicast-server to configure the device to operate in IPv6 NTP multicast server mode and use the current interface to send IPv6 NTP multicast packets.

Use undo ntp-service ipv6 multicast-server to remove the configuration.

By default, the device does not operate in any NTP operation mode.

Examples

# Configure the device to operate in IPv6 multicast server mode and send IPv6 NTP multicast messages on VLAN-interface 1 to the multicast address FF21::1, using key 4 for encryption.

<Sysname> system-view

[Sysname] interface vlan-interface 1

[Sysname-Vlan-interface1] ntp-service ipv6 multicast-server ff21::1 authentication-keyid 4

New command: ntp-service ipv6 source-interface

Syntax

ntp-service ipv6 source-interface interface-type interface-number

undo ntp-service ipv6 source-interface

View

System view

Default level

3: Manage level

Parameters

interface-type interface-number: Specifies an interface by its type and number.

Description

Use ntp-service ipv6 source-interface to specify the source interface for IPv6 NTP messages.

Use undo ntp-service ipv6 source-interface to restore the default.

By default, no source interface is specified for IPv6 NTP messages, and the system uses the IP address of the interface determined by the matched route as the source IP address of IPv6 NTP messages.

If you do not want the IP address of a certain interface on the local device to become the destination address of response messages, use this command to specify the source interface for IPv6 NTP messages so that the source IP address in IPv6 NTP messages is the primary IP address of this interface.

If the specified source interface goes down, NTP searches the routing table for the outgoing interface, and uses the primary IP address of the outgoing interface as the source IP address.

Examples

# Specify the source interface of IPv6 NTP messages as VLAN-interface 1.

<Sysname> system-view

[Sysname] ntp-service ipv6 source-interface vlan-interface 1

New command: ntp-service ipv6 unicast-peer

Syntax

ntp-service ipv6 unicast-peer  { ipv6-address | peer-name } [ authentication-keyid keyid | priority | source-interface interface-type interface-number ] *

undo ntp-service ipv6 unicast-peer { ipv6-address | peer-name }

View

System view

Default level

3: Manage level

Parameters

peer-name: Specifies a host name of the symmetric-passive peer, a string of 1 to 20 characters.

authentication-keyid keyid: Specifies the key ID to be used for sending NTP messages to the peer, where keyid is in the range of 1 to 4294967295.

priority: Specifies the peer designated by ip-address or peer-name as the first choice under the same condition.

source-interface interface-type interface-number: Specifies the source interface for NTP messages. In an NTP message that the local device sends to its peer, the source IP address is the primary IP address of this interface.

Description

Use ntp-service ipv6 unicast-peer to designate an IPv6 symmetric-passive peer for the device.

Use undo ntp-service ipv6 unicast-peer to remove the IPv6 symmetric-passive peer designated for the device.

By default, no IPv6 symmetric-passive peer is designated for the device.

Examples

# Designate the device with the IPv6 address of 2001::1 as the symmetric-passive peer of the device, configure the device to run IPv6 NTP version 4, and specify the source interface of IPv6 NTP messages as VLAN-interface 1.

<Sysname> system-view

[Sysname] ntp-service ipv6 unicast-peer 2001::1 source-interface vlan-interface 1

New command: ntp-service ipv6 unicast-server

Syntax

ntp-service ipv6 unicast-server { ipv6-address | server-name } [ authentication-keyid keyid | priority | source-interface interface-type interface-number ] *

undo ntp-service ipv6 unicast-server { ipv6-address | server-name }

View

System view

Default level

3: Manage level

Parameters

server-name: Specifies a host name of the NTP server, a string of 1 to 20 characters.

authentication-keyid keyid: Specifies the key ID to be used for sending NTP messages to the NTP server, where keyid is in the range of 1 to 4294967295.

priority: Specifies this NTP server as the first choice under the same condition.

source-interface interface-type interface-number: Specifies the source interface for IPv6 NTP messages. In an IPv6 NTP message that the local device sends to the NTP server, the source IPv6 address is the primary IP address of this interface.

Description

Use ntp-service ipv6 unicast-server to designate an IPv6 NTP server for the device.

Use undo ntp-service ipv6 unicast-server to remove an IPv6 NTP server designated for the device.

By default, no IPv6 NTP server is designated for the device.

Examples

# Designate NTP server 2001::1 for the device, and configure the device to run NTP version 4.

<Sysname> system-view

[Sysname] ntp-service ipv6 unicast-server 2001::1 version 4

# A5120EI-CMW520-R2221P08

This release has the following changes:

New feature: Applicable scope of packet filtering on a VLAN interface

New feature: SNMP notifications for PVST topology changes

New feature: Disabling SSL 3.0

New feature: Applicable scope of packet filtering on a VLAN interface

Configuring the applicable scope of packet filtering on a VLAN interface

You can configure the packet filtering on a VLAN interface to filter the following packets:

- Packets forwarded at Layer 3 by the VLAN interface.

- All packets, including packets forwarded at Layer 3 by the VLAN interface and packets forwarded at Layer 2 by the physical ports associated with the VLAN interface.

To configure the applicable scope of packet filtering on a VLAN interface:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Create a VLAN interface and enter its view. | interface vlan-interface vlan-interface-id | If the VLAN interface already exists, you directly enter its view. By default, no VLAN interface exists. |
| ##### Specify the applicable scope of packet filtering on the VLAN interface. | packet-filter filter { route ¦ all } | By default, the packet filtering filters all packets. |

Command reference

packet-filter filter

Use packet-filter filter to specify the applicable scope of packet filtering on a VLAN interface.

Use undo packet-filter filter to restore the default.

Syntax

packet-filter filter { route | all }

undo packet-filter filter

Default

The packet filtering filters all packets.

Views

VLAN interface view

Predefined user roles

network-admin

Parameters

route: Filters packets forwarded at Layer 3 by the VLAN interface.

all: Filters all packets, including packets forwarded at Layer 3 by the VLAN interface and packets forwarded at Layer 2 by the physical ports associated with the VLAN interface.

Examples

# Configure the packet filtering on VLAN-interface 2 to filter packets forwarded at Layer 3.

<Sysname> system-view

[Sysname] interface vlan-interface 2

[Sysname-Vlan-interface2] packet-filter filter route

New feature: SNMP notifications for PVST topology changes

Enabling SNMP notifications for PVST topology changes

This feature enables the device to generate logs and report PVST topology change events to an NMS when the device detects or receives a TC BPDU. For the SNMP notifications to be sent correctly, you must also configure SNMP as described in Network Management and Monitoring Configuration Guide.

To enable SNMP notifications for PVST topology changes:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable SNMP notifications for PVST topology changes. | snmp-agent trap enable stp [ tc ] | By default, SNMP notifications are disabled for PVST topology changes on all VLANs. |

Command reference

snmp trap enable stp

Use snmp-agent trap enable stp to enable SNMP notifications for PVST topology changes.

Use undo snmp-agent trap enable stp to disable SNMP notifications for PVST topology changes.

Syntax

snmp-agent trap enable stp [ tc ]

undo snmp-agent trap enable stp [ tc ]

Default

SNMP notifications are disabled for PVST topology changes on all VLANs.

Views

System view

Predefined user roles

3: Manage level

Parameters

tc: Specifies SNMP notifications for PVST topology changes.

Usage guidelines

This command configures SNMP notifications only for PVST topology changes whether you specify the tc keyword or not.

Examples

# Enable SNMP notifications for PVST topology changes.

<Sysname> system-view

[Sysname] snmp-agent trap enable stp tc

  - New feature: Disabling SSL 3.0

Disabling SSL 3.0

This feature allows you to disable SSL 3.0 on a device to enhance system security.

- An SSL server supports only TLS 1.0 after SSL 3.0 is disabled.

- An SSL client always uses SSL 3.0 if SSL 3.0 is specified for the client policy, whether you disable SSL 3.0 or not.

To ensure successful establishment of an SSL connection, do not disable SSL 3.0 on a device when the peer device only supports SSL 3.0. HP recommends upgrading the peer device to support TLS 1.0 to improve security.

To disable SSL 3.0 on a device:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Disable SSL 3.0 on the device. | ssl version ssl3.0 disable | By default, the device supports SSL 3.0. |

Command reference

ssl version ssl3.0 disable

Syntax

ssl version ssl3.0 disable

undo ssl version ssl3.0 disable

Views

System view

Parameters

None

Description

Use ssl version ssl3.0 disable to disable SSL 3.0 on the device. Use undo ssl version ssl3.0 disable restore the default.

By default, the device supports SSL 3.0.

Examples

# Disable SSL 3.0 on the device.

<Sysname> system-view

[Sysname] ssl version ssl3.0 disable

# A5120EI-CMW520-R2221P07

This release has the following changes:

New feature: 802.1X MAC address binding

New feature: Automatic PI reset

New feature: 802.1X MAC address binding

Configuring 802.1X MAC address binding

This feature can automatically bind MAC addresses of authenticated 802.1X users to the users' access port and generate 802.1X MAC address binding entries. You can also use the dot1x binding-mac mac-address command to manually configure 802.1X MAC address binding entries.

802.1X MAC address binding entries never age out. They can survive a user logoff or a device reboot. To delete an entry, you must use the undo dot1x binding-mac mac-address command.

After the number of 802.1X MAC address binding entries reaches the upper limit of concurrent 802.1X users, the following restrictions exist:

Users not in the binding entries will fail authentication even after users in the binding entries go offline.

New 802.1X MAC address binding entries are not allowed.

When you configure the 802.1X MAC address binding feature on a port, follow these restrictions and guidelines:

The 802.1X MAC address binding feature takes effect only when the port performs MAC-based access control.

Manually configured MAC address binding entries take effect only when the 802.1X MAC address binding feature takes effect.

An 802.1X MAC address binding entry cannot be deleted when the user in the entry is online.

To configure the 802.1X MAC address binding feature on a port:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| - Enable the 802.1X MAC address binding feature. | dot1x binding-mac enable | By default, the feature is disabled. |
| - (Optional.) Manually configure 802.1X MAC address binding entries. | dot1x binding-mac mac-address | By default, no 802.1X MAC address binding entries are configured on the port. |

Command reference

dot1x binding-mac enable

Use dot1x binding-mac enable to enable the 802.1X MAC address binding feature.

Use undo dot1x binding-mac enable to restore the default.

Syntax

dot1x binding-mac enable

undo dot1x binding-mac enable

Default

The 802.1X MAC address binding feature is disabled.

Views

Layer 2 Ethernet interface view

Default command level

2: System level

Usage guidelines

This command takes effect on a port only when the port performs MAC-based access control.

The 802.1X MAC address binding feature automatically binds MAC addresses of authenticated 802.1X users to the users' access port and generates 802.1X MAC address binding entries.

802.1X MAC address binding entries, both automatically generated and manually configured, never age out. They can survive a user logoff or a device reboot. To delete an entry, you must use the undo dot1x binding-mac mac-address command. An 802.1X MAC address binding entry cannot be deleted when the user in the entry is online.

After the number of 802.1X MAC address binding entries reaches the upper limit of concurrent 802.1X users (set by using the dot1x max-user command), the following restrictions exist:

Users not in the binding entries will fail authentication even after users in the binding entries go offline.

New 802.1X MAC address binding entries are not allowed.

Examples

# Enable 802.1X MAC address binding on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x binding-mac enable

dot1x binding-mac

Use dot1x binding-mac to configure an 802.1X MAC address binding entry.

Use undo dot1x binding-mac to delete an 802.1X MAC address binding entry.

Syntax

dot1x binding-mac mac-address

undo dot1x binding-mac mac-address

Default

No 802.1X MAC address binding entries are configured on a port.

Views

Layer 2 Ethernet interface view

Default command level

2: System level

Parameters

mac-address: Specifies a MAC address, in the format of H-H-H, excluding broadcast, multicast, and all-zero MAC addresses.

Usage guidelines

This command takes effect only the 802.1X MAC address binding feature takes effect.

802.1X MAC address binding entries, both manually configured and automatically generated, never age out. They can survive a user logoff or a device reboot. To delete an entry, you must use the undo dot1x binding-mac mac-address command. An 802.1X MAC address binding entry cannot be deleted when the user in the entry is online.

After the number of 802.1X MAC address binding entries reaches the upper limit of concurrent 802.1X users (set by using the dot1x max-user command), the following restrictions exist:

Users not in the binding entries will fail authentication even after users in the binding entries go offline.

New 802.1X MAC address binding entries are not allowed.

Examples

# Configure an 802.1X MAC address binding entry on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x binding-mac 000a-eb29-75f1

New feature: Automatic PI reset

Enabling automatic PI reset

This feature enables PIs to reset automatically after you reboot the device by using the reboot command. After the reset, the PIs resume data and power supply services.

To enable automatic PI reset:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enable automatic PI reset. | poe reset enable | By default, automatic PI reset is disabled. |

Command reference

    - poe reset enable

Use poe reset enable to enable automatic PI reset.

Use undo poe reset enable to disable automatic PI reset.

Syntax

poe reset enable

undo poe reset enable

Default

Automatic PI reset is disabled.

Views

System view

Default command level

2: System level

Examples

# Enable automatic PI reset.

<Sysname> system-view

[Sysname] poe reset enable

# A5120EI-CMW520-R2221P06

This release has no feature changes.

# A5120EI-CMW520-R2221P05

This release has the following changes:

- New feature: Telnet/SSH user connection control

- Modified feature: Including time zone information in the timestamp of system information sent to a log host

- Modified feature: Configuring physical state change suppression on an Ethernet interface

- Modified feature: Configuring a tag and description for an IPv6 static route

New feature: Telnet/SSH user connection control

Configuring Telnet/SSH user connection control

This feature allows you to control Telnet/SSH user connections to the device based on the referenced ACL. Only the Telnet/SSH users that the referenced ACL permits can initiate Telnet/SSH connections to the device.

All Telnet/SSH users can initiate Telnet/SSH connections to the device when any one of the following conditions exits:

- You do not specify any ACLs.

- The specified ACL does not exist.

- The specified ACL does not have any rules.

Configuration prerequisites

Before you configure Telnet/SSH user connection control, configure the ACL as required.

Configuration procedure

To configure Telnet user connection control:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Configure Telnet user connection control. | - Configure IPv4 Telnet user connection control: telnet server acl acl-number - Configure IPv6 Telnet user connection control: telnet server ipv6 acl ipv6 acl-number | By default, all Telnet users can initiate Telnet connections to the device. |

To configure SSH user connection control:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Configure SSH user connection control. | - Configure IPv4 SSH user connection control: ssh server acl acl-number - Configure IPv6 SSH user connection control: ssh server ipv6 acl ipv6 acl-number | By default, all SSH users can initiate SSH connections to the device. |

Command reference

    - ssh server acl

Use ssh server acl to specify an ACL to control IPv4 SSH user connections.

Use undo ssh server acl to restore the default.

Syntax

ssh server acl acl-number

undo ssh server acl

Default

No ACLs are specified and all IPv4 SSH users can initiate SSH connections to the device.

Views

System view

Default command level

3: Manage level

Parameters

acl-number: Specifies an IPv4 ACL by its number in the range of 2000 to 3999.

Usage guidelines

The specified ACL filters IPv4 SSH users' connection requests. Only the IPv4 SSH users that the ACL permits can initiate SSH connections to the device.

All IPv4 SSH users can initiate SSH connections to the device when any one of the following conditions exists:

- You do not specify any ACLs.

- The specified ACL does not exist.

- The specified ACL does not have any rules.

The ACL takes effect only on SSH connections that are initiated after the ACL configuration.

If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Configure ACL 2001 and permit only the users at 1.1.1.1 to initiate SSH connections to the device.

<Sysname> system-view

[Sysname] acl number 2001

[Sysname-acl-basic-2001] rule permit source 1.1.1.1 0

[Sysname-acl-basic-2001] quit

[Sysname] ssh server acl 2001

    - ssh server ipv6 acl ipv6

Use ssh server ipv6 acl ipv6 to specify an ACL to control IPv6 SSH user connections.

Use undo ssh server ipv6 acl to restore the default.

Syntax

ssh server ipv6 acl ipv6 acl-number

undo ssh server ipv6 acl

Default

No ACLs are specified and all IPv6 SSH users can initiate SSH connections to the device.

Views

System view

Default command level

3: Manage level

Parameters

acl-number: Specifies an IPv6 ACL by its number in the range of 2000 to 3999.

Usage guidelines

The specified ACL filters IPv6 SSH users' connection requests. Only the IPv6 SSH users that the ACL permits can initiate SSH connections to the device.

All IPv6 SSH users can initiate SSH connections to the device when any one of the following conditions exists:

- You do not specify any ACLs.

- The specified ACL does not exist.

- The specified ACL does not have any rules.

The ACL takes effect only on SSH connections that are initiated after the ACL configuration.

If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Configure ACL 2001 and permit only the users on the subnet 1::1/64 to initiate SSH connections to the device.

<Sysname> system-view

[Sysname] acl ipv6 number 2001

[Sysname-acl6-basic-2001] rule permit source 1::1 64

[Sysname-acl6-basic-2001] quit

[Sysname] ssh server ipv6 acl ipv6 2001

    - telnet server acl

Use telnet server acl to specify an ACL to control IPv4 Telnet user connections.

Use undo telnet server acl to restore the default.

Syntax

telnet server acl acl-number

undo telnet server acl

Default

No ACLs are specified and all IPv4 Telnet users can initiate Telnet connections to the device.

Views

System view

Default command level

3: Manage level

Parameters

acl-number: Specifies an IPv4 ACL by its number in the range of 2000 to 3999.

Usage guidelines

This command is not supported in FIPS mode.

The specified ACL filters IPv4 Telnet users' connection requests. Only the IPv4 Telnet users that the ACL permits can initiate Telnet connections to the device.

All IPv4 Telnet users can initiate Telnet connections to the device when any one of the following conditions exists:

- You do not specify any ACLs.

- The specified ACL does not exist.

- The specified ACL does not have any rules.

The ACL takes effect only on Telnet connections that are initiated after the ACL configuration.

If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Configure ACL 2001 and permit only the users at 1.1.1.1 to initiate Telnet connections to the device.

<Sysname> system-view

[Sysname] acl number 2001

[Sysname-acl-basic-2001] rule permit source 1.1.1.1 0

[Sysname-acl-basic-2001] quit

[Sysname] telnet server acl 2001

    - telnet server ipv6 acl ipv6

Use telnet server ipv6 acl ipv6 to specify an ACL to control IPv6 Telnet user connections.

Use undo telnet server ipv6 acl to restore the default.

Syntax

telnet server ipv6 acl ipv6 acl-number

undo telnet server ipv6 acl

Default

No ACLs are specified and all IPv6 Telnet users can initiate Telnet connections to the device.

Views

System view

Default command level

3: Manage level

Parameters

acl-number: Specifies an IPv6 ACL by its number in the range of 2000 to 3999.

Usage guidelines

This command is not supported in FIPS mode.

The specified ACL filters IPv6 Telnet users' connection requests. Only the IPv6 Telnet users that the ACL permits can initiate Telnet connections to the device.

All IPv6 Telnet users can initiate Telnet connections to the device when any one of the following conditions exists:

- You do not specify any ACLs.

- The specified ACL does not exist.

- The specified ACL does not have any rules.

The ACL takes effect only on Telnet connections that are initiated after the ACL configuration.

If you execute this command multiple times, the most recent configuration takes effect.

Examples

# Configure ACL 2001 and permit only the users at 2000::1 to initiate Telnet connections to the device.

<Sysname> system-view

[Sysname] acl ipv6 number 2001

[Sysname-acl6-basic-2001] rule permit source 2000::1 128

[Sysname-acl6-basic-2001] quit

[Sysname] telnet server ipv6 acl ipv6 2001

  - Modified feature: Including time zone information in the timestamp of system information sent to a log host

    - Feature change description

Added support for including time zone information in the timestamp of system information sent to a log host.

    - Command changes

      - Modified command: info-center timestamp loghost

Old syntax

info-center timestamp loghost { date | iso | no-year-date | none }

New syntax

info-center timestamp loghost { date | iso [ with-timezone ] | no-year-date | none }

Views

System view

Change description

The following parameter was added:

with-timezone: Includes time zone information in the timestamp of system information sent to a log host.

Modified feature: Configuring physical state change suppression on an Ethernet interface

Feature change description

Before modification:

- The system can suppress only link-down or only link-up events. For example, if you configure the link-delay delay-time [ mode up ] command and then configure the link-delay delay-time command, the system suppresses only link-down events.

- When you disable physical state change suppression on an interface, suppression for both link-up and link-down events are disabled.

After modification, you can perform the following operations:

- Enable the physical state change suppression time to be accurate to milliseconds by specifying the msec keyword.

- Enable the system to suppress both link-down and link-up events by specifying the updown keyword.

- Configure different suppression intervals for link-up and link-down events. For example, if you configure the link-delay [ msec ] delay-time [ mode up ] command and then configure the link-delay [ msec ] delay-time command, both commands take effect.

- Disable suppression for only link-up events, only link-down events, or both. For example, when both link-up and link-down events are suppressed on an interface and you configure the undo link-delay delay-time mode up command, only suppression for link-up events is disabled.

Command changes

Modified command: link-delay

Old syntax

link-delay delay-time [ mode up ]

undo link-delay

New syntax

link-delay [ msec ] delay-time [ mode { up | updown } ]

undo link-delay [ [ msec delay-time ] [ mode { up | updown } ] ]

Views

Ethernet interface view

Change description

Before modification:

- The value range for the delay-time argument is 2 to 10 seconds.

- When you configure the undo link-delay command on an interface, suppression for both link-up and link-down events are disabled.

After modification:

- The msec and updown keywords were added to the link-delay delay-time [ mode up ] command.

  - If you specify the msec keyword, the value range for the delay-time argument is 500 to 10000 milliseconds, and the value must be an integer multiple of 100. If you do not specify the msec keyword, the value range for the delay-time argument is 2 to 10 seconds.

  - If you specify the updown keyword, the link-down or link-up event is not reported to the CPU unless the interface is still down or up when the suppression interval (delay-time) expires.

- The undo link-delay command was changed to undo link-delay [ [ msec delay-time ] [ mode { up | updown } ] ].

You can disable suppression for only link-up events, only link-down events, or both. For example, when both link-up and link-down events are suppressed on an interface and you configure the undo link-delay delay-time mode up command, only suppression for link-up events is disabled.

Modified feature: Configuring a tag and description for an IPv6 static route

Feature change description

The tag tag-value and description description-text options were added to the ipv6 route-static command. The tag tag-value option configures a tag for an IPv6 static route, and the description description-text option configures a description for an IPv6 static route.

Command changes

Modified command: ipv6 route-static

Old syntax

ipv6 route-static ipv6-address prefix-length { interface-type interface-number [ next-hop-address ] | next-hop-address } [ preference preference-value ]

New syntax

ipv6 route-static ipv6-address prefix-length { interface-type interface-number [ next-hop-address ] | next-hop-address } [ preference preference-value ] [ tag tag-value ] [ description description-text ]

Views

System view

Change description

The tag tag-value and description description-text options were added.

- tag tag-value: Configures a tag for an IPv6 static route, in the range of 1 to 4294967295. The default is 0. Tags of routes are used for route control in routing policies.

- description description-text: Configures a description for an IPv6 static route. The description is a string of 1 to 60 characters, including special characters such as the space, but excluding the question mark (?).

# A5120EI-CMW520-R2221P04

This release has the following changes:

- New feature: 802.1X voice VLAN

- New feature: TCP fragment attack protection

- Modified feature: Username request timeout timer for 802.1X authentication

New feature: 802.1X voice VLAN

Configuring an 802.1X voice VLAN

You can configure an 802.1X voice VLAN on an 802.1X-enabled port that connects to a voice terminal. The 802.1X voice VLAN feature is effective only on voice terminals that support VLAN-tagged packets.

The 802.1X voice VLAN feature works with a remote authentication server. The device uses the following process to implement this feature:

Identifies a voice terminal from the packet sent by the authentication server when the terminal passes 802.1X authentication. The authentication server identifies the terminal type by information such as its OUI and user account, and then sends the terminal type information to the device.

Assigns the port to the configured voice VLAN as a tagged member and sends the voice VLAN information through an LLDP or CDP packet to the terminal.

A voice terminal is not associated with a specific voice VLAN. The voice VLAN assigned to the voice terminal depends on the voice VLAN configuration on the access port.

Configuration guidelines

When you configure an 802.1X voice VLAN, follow these guidelines:

- You can configure only one 802.1X voice VLAN on a port. The 802.1X voice VLANs on different ports can be different.

- To ensure a correct exchange of 802.1X EAPOL packets, you must configure the dot1x eapol untag command. For information about how to configure this command, see HP 5120 EI Switch Series Security Configuration Guide-Release 2220.

- A server-assigned authorization VLAN for a voice terminal takes precedence over the 802.1X voice VLAN. The port will be assigned to the authorization VLAN if both VLANs coexist. For information about 802.1 X VLAN manipulations, see HP 5120 EI Switch Series Security Configuration Guide-Release 2220.

- This feature cannot work with the RADIUS server provided by IMC.

Configuration prerequisites

Before you configure this feature, complete the following tasks:

- Enable 802.1X on the port.

- Set the port type to hybrid or trunk, because the port is assigned to the 802.1X voice VLAN as a tagged member. For information about port types, see HP 5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2220.

- Configure LLDP or CDP compatibility on the device. For information about the LLDP and CDP compatibility features, see HP 5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2220.

Configuration procedure

To configure an 802.1X voice VLAN on a port:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Ethernet interface view. | interface interface-type interface-number | N/A |
| - Configure an 802.1X voice VLAN on the port. | dot1x voice vlan vlan-id | By default, no 802.1X voice VLAN is configured on a port. |

    - Command reference

      - New command: dot1x voice vlan

Use dot1x voice vlan to configure an 802.1X voice VLAN on a port.

Use undo dot1x voice vlan to remove the 802.1X voice VLAN on a port.

Syntax

dot1x voice vlan vlan-id

undo dot1x voice vlan

Default

No 802.1X voice VLAN is configured on a port.

Views

Ethernet interface view

Default command level

2: System level

Parameters

vlan-id: Specifies a voice VLAN by its ID in the range of 1 to 4094. The VLAN must have been created.

Usage guidelines

This command must function with a remote authentication server (for example, FreeRADIUS). It cannot work with the RADIUS server provided by IMC.

To ensure a correct exchange of 802.1X EAPOL packets, you must configure the dot1x eapol untag command. For information about how to configure this command, see HP 5120 EI Switch Series Security Configuration Guide-Release 2220.

The server-assigned authorization VLAN takes precedence over the 802.1X voice VLAN on a port. The port will be assigned to the authorization VLAN if both VLANs coexist. For information about 802.1 X VLAN manipulations, see HP 5120 EI Switch Series Security Configuration Guide-Release 2220.

Before you configure an 802.1X voice VLAN on a port, perform the following tasks:

- Enable 802.1X on the port.

- Set the port type to hybrid or trunk, because the port is assigned to the 802.1X voice VLAN as a tagged member. For information about port types, see HP 5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2220.

- Configure LLDP or CDP compatibility on the device. For information about the LLDP and CDP compatibility features, see HP 5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2220.

Examples

# Configure VLAN 20 as the 802.1X voice VLAN on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x voice vlan 20

New feature: TCP fragment attack protection

  - Enabling TCP fragment attack protection

The TCP fragment attack protection function enables the device to drop attack TCP fragments to prevent TCP fragment attacks. As defined in RFC 1858, attack TCP fragments refer to the following TCP fragments:

- First fragments in which the TCP header is smaller than 20 bytes.

- Non-first fragments with a fragment offset of 8 bytes (FO=1).

Traditional packet filter on the device detects the source and destination IP addresses, source and destination ports, and transport layer protocol of the first fragment of a TCP packet. If the first fragment passes the detection, all subsequent fragments of the TCP packet are allowed to pass through. An attacker can launch TCP fragment attacks through either of the following ways:

- Make the first fragment small enough to force some TCP header fields into the second fragment and set TCP flags illegally in the second fragment.

- Fabricate a non-first fragment in which the fragment offset is set to 8 bytes and the TCP flags are set differently and illegally from those in the first fragment. When the receiving host reassembles the fragments, the illegal TCP flags in the non-first fragment overwrite the legal TCP flags in the first fragment.

Because the first fragment does not hit any match in the packet filter, the subsequent fragments can all pass through. After the receiving host reassembles the fragments, a TCP fragment attack occurs.

To enable TCP fragment attack protection:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable TCP fragment attack protection. | attack-defense tcp fragment enable | By default, TCP fragment attack protection is enabled. |

Command reference

attack-defense tcp fragment enable

Use attack-defense tcp fragment enable to enable TCP fragment attack protection.

Use undo attack-defense tcp fragment enable to disable TCP fragment attack protection.

Syntax

attack-defense tcp fragment enable

undo attack-defense tcp fragment enable

Default

TCP fragment attack protection is enabled.

Views

System view

Default command level

2: System level

Usage guidelines

This command enables the device to drop attack TCP fragments to prevent TCP fragment attacks.

Examples

# Enable TCP fragment attack protection.

<Sysname> System-view

[Sysname] attack-defense tcp fragment enable

Modified feature: Username request timeout timer for 802.1X authentication

Feature change description

The minimum value for the 802.1X username request timeout timer was changed from 10 seconds to 1 second. This modification allows the device to send EAP-Request/Identity packets to initiate 802.1X authentication at a shorter interval.

Command changes

Modified command: dot1x timer

Syntax

dot1x timer tx-period tx-period-value

Views

System view

Change description

Before modification: The value range for the tx-period-value argument is 10 to 120 seconds.

After modification: The value range for the tx-period-value argument is 1 to 120 seconds.

# A5120EI-CMW520-R2221P02

This release has the following changes:

- New feature: Support for BPDU guard configuration in interface or port group view

- New feature: MAC re-authentication timer for users in guest VLAN

- New feature: MAC and port uniqueness check by the DHCP snooping device

- Modified feature: Auto status transition of dynamic secure MAC addresses

- Modified feature: The maximum number of gateways supported in MFF automatic mode

- New feature: Support for BPDU guard configuration in interface or port group view

Configuring BPDU guard for an interface or port group

Before this feature was introduced, the device supported only global BPDU guard configuration (stp bpdu-protection). Global BPDU guard configuration takes effect on all edge ports. Edge ports are configured by using the stp edged-port enable command.

This feature allows you to perform the following tasks:

- Enable BPDU guard for an interface or port group when BPDU guard is globally disabled.

- Disable BPDU guard for an interface or port group when BPDU guard is globally enabled.

You must enable BPDU guard on a port that directly connects to a user terminal rather than another device or shared LAN segment.

Enabling BPDU guard for an interface or port group when BPDU guard is globally disabled

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter interface view or port group view. | - Enter Layer 2 Ethernet interface view or Layer 2 aggregate interface view: interface interface-type interface-number - Enter port group view: port-group manual port-group-name | Use one of the commands. |
| ##### Enable BPDU guard. | stp port bpdu-protection enable | BPDU guard is disabled on all interfaces if it is globally disabled. By default, BPDU guard is globally disabled. |

Disabling BPDU guard for an interface or port group when BPDU guard is globally enabled

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable BPDU guard globally. | stp bpdu-protection | By default, BPDU guard is globally disabled. |
| ##### Enter interface view or port group view. | - Enter Layer 2 Ethernet interface view or Layer 2 aggregate interface view: interface interface-type interface-number - Enter port group view: port-group manual port-group-name | Use one of the commands. |
| ##### Disable BPDU guard. | stp port bpdu-protection disable | By default, BPDU guard is enabled on all edge ports if it is globally enabled. |

Command reference

New command: stp port bpdu-protection

Use stp port bpdu-protection to configure BPDU guard on an interface.

Use undo stp port bpdu-protection to restore the default.

Syntax

stp port bpdu-protection { enable | disable }

undo stp port bpdu-protection

Default

BPDU guard is not configured on an interface. For an edge port, BPDU guard is enabled on the port if the function is globally enabled. BPDU guard is disabled on the port if the function is disabled globally.

Views

Layer 2 Ethernet interface view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Parameters

enable: Enables BPDU guard on the interface.

disable: Disables BPDU guard on the interface.

Usage guidelines

When the setting is configured in Layer 2 Ethernet interface view, it takes effect on only that interface.

When the setting is configured in Layer 2 aggregate interface view, it takes effect on only the aggregate interface.

When the setting is configured in port group view, it takes effect on all ports in the port group.

When the setting is configured on a member port in an aggregation group, it takes effect only after the port leaves the aggregation group.

Examples

# Enable BPDU guard on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] stp port bpdu-protection enable

Related commands

- stp bpdu-protection

- stp edged-port

For more information about these commands, see spanning tree commands in Layer 2—LAN Switching Command Reference.

New feature: MAC re-authentication timer for users in guest VLAN

Configuring MAC re-authentication timer for users in guest VLAN

The MAC re-authentication timer sets the interval that the device must wait before it can re-authenticate a user in the MAC authentication guest VLAN.

The device handles VLANs for users in the MAC authentication guest VLAN based on the following rules:

| Authentication status | VLAN manipulation |
| --- | --- |
| A user fails MAC re-authentication because of unreachable servers. | - If a MAC authentication critical VLAN is available, the device assigns the user to the critical VLAN. - If no MAC authentication critical VLAN is configured, the user is still in the MAC authentication guest VLAN. The MAC re-authentication timer restarts for the user. |
| A user fails MAC re-authentication for any other reasons except for unreachable servers. | The user is still in the MAC authentication guest VLAN. The MAC re-authentication timer restarts for the user. |
| A user passes MAC re-authentication. | - The device removes the user from the MAC authentication guest VLAN and assigns the user to the authorization VLAN. - If the authentication server does not authorize a VLAN, the user is assigned to the initial VLAN. The initial VLAN refers to the VLAN to which the user belongs before it was assigned to the MAC authentication guest VLAN. |

To configure the MAC re-authentication timer for users in the MAC authentication guest VLAN:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter Ethernet interface view. | interface interface-type interface-number | N/A |
| ##### Configure the MAC authentication guest VLAN on the port. | mac-authentication guest-vlan vlan-id | By default, no guest VLAN is configured on the port. |
| ##### Return to system view. | quit | N/A |
| ##### Configure the MAC re-authentication timer for users in the guest VLAN. | mac-authentication timer guest-vlan-reauth interval | The default timer is 30 seconds. |

Command reference

mac-authentication timer guest-vlan-reauth

Use mac-authentication timer guest-vlan-reauth to set the MAC re-authentication timer for users in the MAC authentication guest VLAN.

Use undo mac-authentication timer guest-vlan-reauth to restore the default.

Syntax

mac-authentication timer guest-vlan-reauth interval

undo mac-authentication timer guest-vlan-reauth

Default

The MAC re-authentication timer is 30 seconds for users in the MAC authentication guest VLAN.

Views

System view

Default command level

2: System view

Parameters

interval: Set the MAC re-authentication timer for users in the MAC authentication guest VLAN. The value range for this argument is 1 to 3600, in seconds.

Usage guidelines

When the MAC re-authentication timer expires, the device re-authenticates the users in the MAC authentication guest VLAN.

The device handles VLANs for users in the MAC authentication guest VLAN based on the following rules:

| Authentication status | VLAN manipulation |
| --- | --- |
| A user fails MAC re-authentication because of unreachable servers. | - If a MAC authentication critical VLAN is available, the device assigns the user to the critical VLAN. - If no MAC authentication critical VLAN is configured, the user is still in the MAC authentication guest VLAN. The MAC re-authentication timer restarts for the user. |
| A user fails MAC re-authentication for any other reasons except for unreachable servers. | The user is still in the MAC authentication guest VLAN. The MAC re-authentication timer restarts for the user. |
| A user passes MAC re-authentication. | - The device removes the user from the MAC authentication guest VLAN and assigns the user to the authorization VLAN. - If the authentication server does not authorize a VLAN, the user is assigned to the initial VLAN. The initial VLAN refers to the VLAN to which the user belongs before it was assigned to the MAC authentication guest VLAN. |

Examples

# Set the MAC re-authentication timer to 60 seconds for users in the MAC authentication guest VLAN.

<Sysname> system-view

[Sysname] mac-authentication timer guest-vlan-reauth 60

New feature: MAC and port uniqueness check by the DHCP snooping device

Enabling MAC and port uniqueness check on the DHCP snooping device

This function allows the DHCP snooping device to maintain only one DHCP snooping entry for the same client's MAC address in one VLAN.

When receiving a DHCP REQUEST, the DHCP snooping device checks for a DHCP snooping entry that matches the client's MAC address (the chaddr field in the request). If an entry exists with the same MAC address and VLAN but different receiving port, the device updates the entry. When DHCP snooping entries are used by security modules, such as IP source guard, this function prevents clients from using the same MAC address to apply for multiple IP addresses.

To enable MAC and port uniqueness check on the DHCP snooping device:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable MAC and port uniqueness check on the DHCP snooping device. | dhcp-snooping check mac-port | By default, MAC and port uniqueness check is disabled on the DHCP snooping device. |

Command reference

dhcp-snooping check mac-port

Use dhcp-snooping check mac-port to enable MAC and port uniqueness check on the DHCP snooping device.

Use undo dhcp-snooping check mac-port to disable MAC and port uniqueness check on the DHCP snooping device.

Syntax

dhcp-snooping check mac-port

undo dhcp-snooping check mac-port

Default

MAC and port uniqueness check is disabled on the DHCP snooping device.

Views

System view

Default command level

2: System level

Examples

# Enable MAC and port uniqueness check on the DHCP snooping device.

<Sysname> system-view

[Sysname] dhcp-snooping check mac-port

Modified feature: Auto status transition of dynamic secure MAC addresses

Feature change description

Before modification: A dynamic secure MAC address entry will not be deleted if the port for the entry goes down.

After modification: The status of dynamic secure MAC address entries transits automatically based on the port status. The device deletes a dynamic secure MAC address entry if the port for the entry goes down. This MAC address is reported as an unknown source MAC address if it is detected on another port.

Command changes

None.

- Modified feature: The maximum number of gateways supported in MFF automatic mode

Feature change description

In MFF automatic mode, the maximum number of gateways that can be learned in a VLAN was changed from 20 to 64. No more gateways can be learned when the limit is reached.

Command changes

None.

# A5120EI-CMW520-R2221P01

This release has the following changes:

- New feature: Discarding IPv6 packets that contain extension headers

- New feature: Discarding IPv6 packets that contain extension headers

Enabling a device to discard IPv6 packets that contain extension headers

This feature enables a device to discard a received IPv6 packet if the first extension header of the packet is Hop-by-Hop Options.

To enable a device to discard IPv6 packets that contain extension headers:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable the device to discard IPv6 packets that contain extension headers. | ipv6 option drop enable | By default, the device does not discard IPv6 packets that contain extension headers. |

Command reference

New command: ipv6 option drop enable

Use ipv6 option drop enable to enable the device to discard IPv6 packets that contain extension headers.

Use undo ipv6 option drop to disable the device from discarding IPv6 packets that contain extension headers.

Syntax

ipv6 option drop enable

undo ipv6 option drop

Default

A device does not discard IPv6 packets that contain extension headers.

Views

System view

Default command level

2: System level

Usage guidelines

This feature enables a device to discard a received IPv6 packet if the first extension header of the packet is Hop-by-Hop Options.

Examples

# Enable the device to discard IPv6 packets that contain extension headers.

<Sysname> system-view

[Sysname] ipv6 option drop enable

# A5120EI-CMW520-R2221

This release has the following changes:

New feature: SSL server policy association with the FTP service

New feature: MFF

Modified feature: Setting the device name

Modified feature: Displaying brief interface information

Modified feature: Displaying brief IP configuration for Layer 3 interfaces

Modified feature: Configuring static multicast MAC address entries

Modified feature: Specifying the username and password to log in to the SCP server

Modified feature: Disabling an untrusted port from recording clients' IP-to-MAC bindings

Modified feature: Customizing DHCP options

New feature: SSL server policy association with the FTP service

Configuration procedure

For two devices that support secure FTP, you can associate an SSL server policy with the FTP service on the FTP server. Then, the FTP connection will be established over an SSL connection.

Before associating an SSL server policy with the FTP service, you must create the policy and disable FTP server.

To associate an SSL server policy with the FTP service:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Associate an SSL server policy with the FTP server to ensure data security. | ftp server ssl-server-policy policy-name | By default, no SSL server policy is associated with the FTP server. |

Command reference

    - ftp server ssl-server-policy

Syntax

ftp server ssl-server-policy policy-name

undo ftp server ssl-server-policy

Views

System view

Default level

2: System level

Parameters

policy-name: Specifies an SSL server policy by its name, a string of 1 to 16 characters.

Description

Use ftp server ssl-server-policy to associate an SSL server policy with the FTP server.

Use undo ftp server ssl-server-policy to remove the association.

By default ,no SSL server policy is associated with the FTP server.

Examples

# Associate SSL server policy myssl with the FTP server.

<Sysname> system-view

[Sysname] ftp server ssl-server-policy myssl

  - New feature: MFF

  - Overview

Traditional Ethernet networking solutions use the VLAN technology to isolate users at Layer 2 and to allow them to communicate at Layer 3. However, when a large number of hosts need to be isolated at Layer 2, many VLAN resources are occupied, and many IP addresses are used because you have to assign a network segment to each VLAN and an IP address to each VLAN interface for Layer 3 communication.

MAC-forced forwarding (MFF) provides a solution for Layer 2 isolation and Layer 3 communication between hosts in the same broadcast domain.

An MFF enabled device intercepts an ARP request and then returns the MAC address of a gateway (or server) to the sender. In this way, the sender is forced to send packets to the gateway for traffic monitoring and attack prevention.

Network diagram for MFF

As shown in Figure 1, hosts are connected to Switch C (aggregation node) through Switch A and Switch B (Ethernet access nodes, or EANs). The MFF enabled EANs forward packets from the hosts to the gateway for further forwarding. Thus, the hosts, isolated at Layer 2, can communicate at Layer 3 without knowing the MAC address of each other.

MFF is often used in cooperation with the DHCP snooping, ARP snooping, IP source guard, ARP detection, and VLAN mapping features to enhance network security by implementing traffic filtering, Layer 2 isolation, and Layer 3 communication on the access switches.

|  | NOTE: An MFF-enabled device and a host cannot ping each other. |
| --- | --- |

    - Basic concepts

A device with MFF enabled provides two types of ports: user port and network port.

If you enable MFF for a VLAN, each port in the VLAN must be an MFF network or user port.

Link aggregation is supported by network ports in an MFF-enabled VLAN, but is not supported by user ports in the VLAN. You can add network ports to link aggregation groups, but cannot add user ports to link aggregation groups. For more information about link aggregation, see Layer 2—LAN Switching Configuration Guide.

      - User port

An MFF user port is directly connected to a host and processes the following packets differently:

- Allows DHCP packets and multicast packets to pass.

- Delivers ARP packets to the CPU.

- After learning gateways’ MAC addresses, a user port allows only the unicast packets with the gateways’ MAC addresses as the destination MAC addresses to pass. If no gateways’ MAC addresses are learned, a user port discards all received unicast packets.

      - Network port

An MFF network port is connected to a networking device, such as an access switch, a distribution switch or a gateway. A network port processes the following packets differently:

- Allows multicast packets and DHCP packets to pass.

- Delivers ARP packets to the CPU.

- Denies broadcast packets.

You need to configure the following ports as network ports:

- Upstream ports connected to a gateway.

- Ports connected to the downstream MFF devices in a cascaded network (a network with multiple MFF devices connected to one another).

- Ports between devices in a ring network.

|  | NOTE: A network port is not always an upstream port. |
| --- | --- |

    - Operation modes

      - Manual mode

The manual mode applies to the case where IP addresses are statically assigned to the hosts, and the hosts cannot obtain the gateway information through DHCP. A VLAN maintains only the MAC address of the default gateway.

In manual mode, after receiving an ARP request for a host’s MAC address from the gateway, the MFF device directly replies the host’s MAC address to the gateway according to the ARP snooping entries. The MFF device also forges ARP requests to get the gateway’s MAC address based on ARP snooping entries.

After learning the gateway’s MAC address and then receiving an ARP packet with a different source MAC address from the default gateway, the MFF device will replace the old MAC address with the new one.

      - Automatic mode

The automatic mode applies to the situation where hosts use DHCP to obtain IP addresses.

In MFF automatic mode, a VLAN can learn and maintain up to 20 gateways. The gateway IP addresses will not be updated, and the gateway information does not age out unless MFF is disabled.

With MFF automatic mode enabled, a DHCP snooping device, upon receiving a DHCP ACK message, resolves Option 3 in the message (Router IP option) to obtain a gateway for the client’s IP-MAC snooping entry. If the DHCP ACK message contains multiple gateway addresses, only the first one is recorded for the entry. If the message contains no gateway IP address, the first gateway recorded by the current VLAN is used.

|  | NOTE: If the source MAC address of an incoming ARP packet from a gateway is different from that of the gateway, the MFF device uses the new MAC to replace the old one. |
| --- | --- |

    - Working mechanism

Hosts connecting to an MFF device use the ARP fast-reply mechanism for Layer 3 communication. This mechanism helps reduce the number of broadcast messages.

The MFF device processes ARP packets in the following steps:

- After receiving an ARP request from a host, the MFF device sends the MAC address of the corresponding gateway to the host. In this way, hosts in the network have to communicate at Layer 3 through a gateway.

- After receiving an ARP request from a gateway, the MFF device sends the requested host’s MAC address to the gateway if the corresponding entry is available; if the entry is not available, the MFF device will forward the ARP request.

- The MFF device forwards ARP replies between hosts and gateways.

- If the source MAC addresses of ARP requests from gateways are different from those recorded, the MFF device updates and broadcasts the IP and MAC addresses of the gateways.

    - Protocols and standards

RFC 4562, MAC-Forced Forwarding

  - Configuring MFF

    - Configuration prerequisites

- In MFF automatic mode, enable DHCP snooping on the device and configure DHCP snooping trusted ports.

- In MFF manual mode, enable ARP snooping on the device.

    - Enabling MFF

To enable MFF and specify an MFF operating mode:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter VLAN view. | vlan vlan-id | N/A |
| - Enable MFF and specify an MFF operating mode. | mac-forced-forwarding { auto ¦ default-gateway gateway-ip } | Disabled by default. |

    - Configuring a network port

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter port view. | interface interface-type interface-number | N/A |
| - Configure the port as a network port. | mac-forced-forwarding network-port | By default, the port is a user port. |

    - Enabling periodic gateway probe

You can configure the MFF device to detect gateways periodically for the change of MAC addresses. This feature is supported by MFF manual mode and MFF automatic mode.

The time interval for sending gateway probes is 30 seconds. To get a gateway’s MAC address, MFF in automatic mode uses the IP and MAC addresses of the first DHCP snooping entry corresponding to the gateway as the sender IP and MAC addresses of an ARP request, and sends the ARP request to the gateway. (In manual mode, MFF uses the IP and MAC addresses of the ARP snooping entry corresponding to the gateway.) After that, MFF will always use this entry to detect the gateway’s MAC address, unless the entry is removed. If the entry is removed, MFF will look for another entry; if no other entry is found for the gateway, information about the gateway is removed.

To enable periodic gateway probe:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter VLAN view. | vlan vlan-id | N/A |
| - Enable periodic gateway probe. | mac-forced-forwarding gateway probe | Disabled by default. |

    - Specifying the IP addresses of servers

You need to maintain a server list on the MFF device. The list contains the IP addresses of servers in the network to ensure communication between the servers and clients.

You can specify a server’s IP address in either manual or automatic MFF mode. The server can be a DHCP server, a server providing some other service, or the real IP address of a VRRP standby group. After you specify a server’s IP address and then an ARP request from the server is received, the MFF device will search the IP-to-MAC address entries it has stored, and reply with the corresponding MAC address to the server. As a result, packets from a host to a server are forwarded by the gateway, but packets from a server to a host are not forwarded by the gateway.

To specify the IP addresses of servers:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter VLAN view. | vlan vlan-id | N/A |
| - Specify the IP addresses of servers. | mac-forced-forwarding server server-ip&<1-10> | No server IP address is specified by default. |

  - Displaying and maintaining MFF

| Task | Command | Remarks |
| --- | --- | --- |
| Display MFF port configuration information. | display mac-forced-forwarding interface [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display the MFF configuration information of a specified VLAN. | display mac-forced-forwarding vlan vlan-id [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |

  - MFF configuration examples

    - Auto-mode MFF configuration example in a tree network

      - Network requirements

As shown in Figure 2, all the devices are in VLAN 100. Host A, Host B, and Host C obtain IP addresses from the DHCP server. They are isolated at Layer 2, and can communicate with each other through the gateway. MFF automatic mode is enabled on Switch A and Switch B.

Network diagram

      - Configuration procedure

            - Configure the IP address of VLAN-interface 1 on the gateway.

<Gateway> system-view

[Gateway] interface Vlan-interface 1

[Gateway-Vlan-interface1] ip address 10.1.1.100 24

            - Configure the DHCP server:

# Enable DHCP, and configure a DHCP address pool.

<Device> system-view

[Device] dhcp enable

[Device] dhcp server ip-pool 1

[Device-dhcp-pool-1] network 10.1.1.0 mask 255.255.255.0

# Add the gateway’s IP address into DHCP address pool 1.

[Device-dhcp-pool-1] gateway-list 10.1.1.100

[Device-dhcp-pool-1] quit

# Configure the IP address of VLAN-interface 1.

[Device] interface Vlan-interface 1

[Device-Vlan-interface1] ip address 10.1.1.50 24

            - Configure Switch A:

# Enable DHCP snooping.

<SwitchA> system-view

[SwitchA] dhcp-snooping

# Enable MFF in automatic mode.

[SwitchA] vlan 100

[SwitchA-vlan-100] mac-forced-forwarding auto

[SwitchA-vlan-100] quit

# Configure GigabitEthernet 1/0/2 as a network port.

[SwitchA] interface gigabitethernet 1/0/2

[SwitchA-GigabitEthernet1/0/2] mac-forced-forwarding network-port

# Configure GigabitEthernet 1/0/2 as a DHCP snooping trusted port.

[SwitchA-GigabitEthernet1/0/2] dhcp-snooping trust

            - Configure Switch B:

# Enable DHCP snooping.

<SwitchB> system-view

[SwitchB] dhcp-snooping

# Enable MFF in automatic mode.

[SwitchB] vlan 100

[SwitchB-vlan-100] mac-forced-forwarding auto

[SwitchB-vlan-100] quit

# Configure GigabitEthernet 1/0/6 as a network port.

[SwitchB] interface gigabitethernet 1/0/6

[SwitchB-GigabitEthernet1/0/6] mac-forced-forwarding network-port

# Configure GigabitEthernet 1/0/6 as a DHCP snooping trusted port.

[SwitchB-GigabitEthernet1/0/6] dhcp-snooping trust

    - Auto-mode MFF configuration example in a ring network

      - Network requirements

As shown in Figure 3, all the devices are in VLAN 100, and the switches form a ring. Host A, Host B, and Host C obtain IP addresses from the DHCP server. They are isolated at Layer 2, and can communicate with each other through the gateway. MFF automatic mode is enabled on Switch A and Switch B.

Network diagram

      - Configuration procedure

            - Configure the IP address of VLAN-interface 1 on the gateway.

<Gateway> system-view

[Gateway] interface Vlan-interface 1

[Gateway-Vlan-interface1] ip address 10.1.1.100 24

            - Configure the DHCP server:

# Enable DHCP and configure an address pool.

<Device> system-view

[Device] dhcp enable

[Device] dhcp server ip-pool 1

[Device-dhcp-pool-1] network 10.1.1.0 mask 255.255.255.0

# Add gateway’s IP address into DHCP address pool 1.

[Device-dhcp-pool-1] gateway-list 10.1.1.100

[Device-dhcp-pool-1] quit

# Configure the IP address of VLAN-interface 1.

[Device] interface Vlan-interface 1

[Device-Vlan-interface1] ip address 10.1.1.50 24

            - Configure Switch A:

# Enable DHCP snooping.

<SwitchA> system-view

[SwitchA] dhcp-snooping

# Enable STP.

[SwitchA] stp enable

# Enable MFF in automatic mode.

[SwitchA] vlan 100

[SwitchA-vlan-100] mac-forced-forwarding auto

[SwitchA-vlan-100] quit

# Configure GigabitEthernet 1/0/2 as a network port.

[SwitchA] interface gigabitethernet 1/0/2

[SwitchA-GigabitEthernet1/0/2] mac-forced-forwarding network-port

# Configure GigabitEthernet 1/0/2 as a DHCP snooping trusted port.

[SwitchA-GigabitEthernet1/0/2] dhcp-snooping trust

[SwitchA-GigabitEthernet1/0/2] quit

# Configure GigabitEthernet 1/0/3 as a network port.

[SwitchA] interface gigabitethernet 1/0/3

[SwitchA-GigabitEthernet1/0/3] mac-forced-forwarding network-port

# Configure GigabitEthernet 1/0/3 as a DHCP snooping trusted port.

[SwitchA-GigabitEthernet1/0/3] dhcp-snooping trust no-user-binding

            - Configure Switch B:

# Enable DHCP snooping.

<SwitchB> system-view

[SwitchB] dhcp-snooping

# Enable STP.

[SwitchB] stp enable

# Enable MFF in automatic mode.

[SwitchB] vlan 100

[SwitchB-vlan-100] mac-forced-forwarding auto

[SwitchB-vlan-100] quit

# Configure GigabitEthernet 1/0/4 as a network port.

[SwitchB] interface gigabitethernet 1/0/4

[SwitchB-GigabitEthernet1/0/4] mac-forced-forwarding network-port

# Configure GigabitEthernet 1/0/4 as a DHCP snooping trusted port.

[SwitchB-GigabitEthernet1/0/4] dhcp-snooping trust no-user-binding

[SwitchB-GigabitEthernet1/0/4] quit

# Configure GigabitEthernet 1/0/6 as a network port.

[SwitchB] interface gigabitethernet 1/0/6

[SwitchB-GigabitEthernet1/0/6] mac-forced-forwarding network-port

# Configure GigabitEthernet 1/0/6 as a DHCP snooping trusted port.

[SwitchB-GigabitEthernet1/0/6] dhcp-snooping trust

            - Enable STP on Switch C.

<SwitchC> system-view

[SwitchC] stp enable

    - Manual-mode MFF configuration example in a tree network

      - Network requirements

As shown in Figure 4, all the devices are in VLAN 100. Host A, Host B, and Host C are configured with IP addresses manually. They are isolated at Layer 2, and can communicate with each other through the gateway. To ensure communication between hosts and the server, the IP address of the server is specified on the MFF devices manually.

Network diagram

      - Configuration procedure

            - Configure IP addresses of the hosts, as shown in Figure 4.

            - Configure the IP address of VLAN-interface 1 on the gateway.

<Gateway> system-view

[Gateway] interface Vlan-interface 1

[Gateway-Vlan-interface1] ip address 10.1.1.100 24

            - Configure Switch A:

# Configure manual-mode MFF.

[SwitchA] vlan 100

[SwitchA-vlan-100] mac-forced-forwarding default-gateway 10.1.1.100

# Specify the IP address of the server.

[SwitchA-vlan-100] mac-forced-forwarding server 10.1.1.200

# Enable ARP snooping.

[SwitchA-vlan-100] arp-snooping enable

[SwitchA-vlan-100] quit

# Configure GigabitEthernet 1/0/2 as a network port.

[SwitchA] interface gigabitethernet 1/0/2

[SwitchA-GigabitEthernet1/0/2] mac-forced-forwarding network-port

            - Configure Switch B:

# Configure manual-mode MFF.

[SwitchB] vlan 100

[SwitchB-vlan-100] mac-forced-forwarding default-gateway 10.1.1.100

# Specify the IP address of the server.

[SwitchB-vlan-100] mac-forced-forwarding server 10.1.1.200

# Enable ARP snooping.

[SwitchB-vlan-100] arp-snooping enable

[SwitchB-vlan-100] quit

# Configure GigabitEthernet 1/0/6 as a network port.

[SwitchB] interface gigabitethernet 1/0/6

[SwitchB-GigabitEthernet1/0/6] mac-forced-forwarding network-port

    - Manual-mode MFF configuration example in a ring network

      - Network requirements

As shown in Figure 5, all the devices are in VLAN 100, and the switches form a ring. Host A, Host B, and Host C are configured with IP addresses manually. They are isolated at Layer 2, and can communicate with each other through the gateway. To ensure communication between hosts and the server, the IP address of the server is specified on the MFF devices manually.

Network diagram

      - Configuration procedure

            - Configure IP addresses of the hosts, as in shown in Figure 5.

            - Configure the IP address of VLAN-interface 1 on the gateway.

<Gateway> system-view

[Gateway] interface Vlan-interface 1

[Gateway-Vlan-interface1] ip address 10.1.1.100 24

            - Configure Switch A:

# Enable STP.

[SwitchA] stp enable

# Configure manual-mode MFF.

[SwitchA] vlan 100

[SwitchA-vlan-100] mac-forced-forwarding default-gateway 10.1.1.100

# Specify the IP address of the server.

[SwitchA-vlan-100] mac-forced-forwarding server 10.1.1.200

# Enable ARP snooping.

[SwitchA-vlan-100] arp-snooping enable

[SwitchA-vlan-100] quit

# Configure GigabitEthernet 1/0/2 and GigabitEthernet 1/0/3 as network ports.

[SwitchA] interface gigabitethernet 1/0/2

[SwitchA-GigabitEthernet1/0/2] mac-forced-forwarding network-port

[SwitchA-GigabitEthernet1/0/2] quit

[SwitchA] interface gigabitethernet 1/0/3

[SwitchA-GigabitEthernet1/0/3] mac-forced-forwarding network-port

            - Configure Switch B:

# Enable STP.

[SwitchB] stp enable

# Configure manual-mode MFF.

[SwitchB] vlan 100

[SwitchB-vlan-100] mac-forced-forwarding default-gateway 10.1.1.100

# Specify the IP address of the server.

[SwitchB-vlan-100] mac-forced-forwarding server 10.1.1.200

# Enable ARP snooping.

[SwitchB-vlan-100] arp-snooping enable

[SwitchB-vlan-100] quit

# Configure GigabitEthernet 1/0/4 and GigabitEthernet 1/0/6 as network ports.

[SwitchB] interface gigabitethernet 1/0/4

[SwitchB-GigabitEthernet1/0/4] mac-forced-forwarding network-port

[SwitchB- GigabitEthernet1/0/4] quit

[SwitchB] interface gigabitethernet 1/0/6

[SwitchB-GigabitEthernet1/0/6] mac-forced-forwarding network-port

            - Enable STP on Switch C.

<SwitchC> system-view

[SwitchC] stp enable

Command reference

    - display mac-forced-forwarding interface

Syntax

display mac-forced-forwarding interface [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use display mac-forced-forwarding interface to display MFF port configuration information.

Related commands: mac-forced-forwarding network-port.

Examples

# Display MFF port configuration information.

<Sysname> display mac-forced-forwarding interface

Network Port:

GE1/0/1                    GE1/0/2                    GE1/0/3

User Port:

GE1/0/4                    GE1/0/5                    GE1/0/6

        - Command output

| Field | Description |
| --- | --- |
| Network Port | List of network ports. |
| User Port | List of user ports. |

    - display mac-forced-forwarding vlan

Syntax

display mac-forced-forwarding vlan vlan-id [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

vlan-id: Specifies a VLAN by its number.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use display mac-forced-forwarding vlan to display the MFF configuration information of a specified VLAN.

Related commands: mac-forced-forwarding and mac-forced-forwarding server.

Examples

# Display the MFF configuration information of VLAN 1.

<Sysname> display mac-forced-forwarding vlan 1

VLAN 1

Mode: Auto/Single

Gateway:

--------------------------------------------------------------------------

192.168.1.42     (000f-e200-8046)

Server:

--------------------------------------------------------------------------

192.168.1.48         192.168.1.49

        - Command output

| Field | Description |
| --- | --- |
| VLAN 1 | ID of the VLAN to which the gateways belong. |
| Mode | MFF operating mode, which can be automatic (Auto), manual (Manual), and single-gateway (Single). |
| Gateway | IP and MAC addresses of gateways. If no information is learned, N/A is displayed. |
| Server | Server IP addresses. |

    - mac-forced-forwarding

Syntax

mac-forced-forwarding { auto | default-gateway gateway-ip }

undo mac-forced-forwarding

View

VLAN view

Default level

2: System level

Parameters

auto: Specifies the automatic mode.

default-gateway gateway-ip: Specifies the IP address of the default gateway in the manual mode.

Description

Use mac-forced-forwarding to enable MFF and specify an MFF operating mode. To enable the manual mode, you need to specify a default gateway.

Use undo mac-forced-forwarding to disable MFF.

By default, MFF is disabled.

If you execute this command repeatedly, the last configuration takes effect.

If the automatic mode is specified, make sure that DHCP snooping works normally; if the manual mode is configured, make sure that ARP snooping works normally.

For a network (or VLAN) with IP addresses manually configured, the gateway IP address should be manually configured with the mac-forced-forwarding default-gateway gateway-ip command.

For a network (or VLAN) running DHCP, the gateway IP address can be manually configured with the mac-forced-forwarding default-gateway gateway-ip command, or can be resolved from the Option field in the DHCP messages.

Examples

# Enable MFF in the automatic mode for VLAN 1.

<Sysname> system-view

[Sysname] vlan 1

[Sysname-vlan1] mac-forced-forwarding auto

    - mac-forced-forwarding gateway probe

Syntax

mac-forced-forwarding gateway probe

undo mac-forced-forwarding gateway probe

View

VLAN view

Default level

2: System level

Parameters

None

Description

Use mac-forced-forwarding gateway probe to enable periodic gateway MAC address probe. The probe interval is 30 seconds, and the probe mode can be manual or automatic.

Use undo mac-forced-forwarding gateway probe to restore the default.

By default, periodic gateway MAC address probe is disabled.

Make sure you have enabled MFF before enabling periodic gateway MAC address probe.

Examples

# Enable periodic gateway MAC address probe.

<Sysname> system-view

[Sysname] vlan 1

[Sysname-vlan1] mac-forced-forwarding gateway probe

    - mac-forced-forwarding network-port

Syntax

mac-forced-forwarding network-port

undo mac-forced-forwarding network-port

View

Layer 2 Ethernet interface view

Default level

2: System level

Parameters

None

Description

Use mac-forced-forwarding network-port to configure the Ethernet port as a network port.

Use undo mac-forced-forwarding network-port to restore the default.

By default, the port is a user port.

The upstream ports connecting to a gateway or the ports between devices in a ring network should be configured as network ports. You can configure multiple ports as network ports.

You can configure a port as a network port regardless of whether MFF is enabled for the VLAN of the port; however, the configuration takes effect only after MFF is enabled.

Link aggregation is supported by network ports in an MFF-enabled VLAN, but is not supported by user ports in the VLAN. If a network port is added to a link aggregation group belonging to an MFF-enabled VLAN, you need to remove the network port from the link aggregation group before you can cancel the network port configuration. For more information about link aggregation, see Layer 2—LAN Switching Configuration Guide.

Examples

# Configure GigabitEthernet 1/0/1 as a network port.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-forced-forwarding network-port

    - mac-forced-forwarding server

Syntax

mac-forced-forwarding server server-ip&<1-10>

undo mac-forced-forwarding server [ server-ip&<1-10> ]

View

VLAN view

Default level

2: System level

Parameters

server-ip&<1-10>: Specifies the IP address of a server in the network. &<1-10> means you can specify up to ten server IP addresses in one command line.

Description

Use mac-forced-forwarding server to specify the IP addresses of servers.

Use undo mac-forced-forwarding server to remove the specified or all server IP addresses.

By default, no server IP address is specified.

You can use this command (in either manual or automatic MFF operating mode) to specify the IP address of a DHCP server, the IP address of a server providing some other service, or the real IP address of a VRRP group.

If the MFF device receives an ARP request from a server, it will search the IP-to-MAC address entries it has stored, and reply the corresponding MAC address to the server. In this way, packets from a server to a host are not forwarded by the gateway, but packets from a host to a server are forwarded by the gateway.

MFF does not check whether the IP address of a server is on the same network segment as that of a gateway, but it checks whether the IP address of a server is all-zero or all-one. An all-zero or all-one server IP address is invalid.

If no server IP address is specified using this command, clients cannot communicate with any server.

Check that MFF is enabled before executing the mac-forced-forwarding server command.

If no IP address is specified in the undo mac-forced-forwarding server command, all specified server IP addresses are removed.

Examples

# Specify the server at 192.168.1.100.

<Sysname> system-view

[Sysname] vlan 1

[Sysname-vlan1] mac-forced-forwarding server 192.168.1.100

Modified feature: Setting the device name

Feature change description

The allowed maximum device name length has changed.

Command changes

Modified command: sysname

Syntax

sysname sysname

Views

System view

Change description

Before modification: The device name can have 1 to 30 characters.

After modification: The device name can have 1 to 64 characters.

Modified feature: Displaying brief interface information

Feature change description

The description keyword was added to the display interface command.

If the interface description includes more than 27 characters and the brief keyword is specified for the display interface command, you can use the description keyword to display the full interface description for interfaces. The display interface command displays information about interfaces, such as Ethernet interfaces, aggregate interfaces, VLAN interfaces, loopback interfaces, and the null interface.

Command changes

Modified command: display interface

Old syntax

display interface [ interface-type ] [ brief [ down ] ] [ | { begin | exclude | include } regular-expression ]

display interface interface-type interface-number [ brief ] [ | { begin | exclude | include } regular-expression ]

New syntax

display interface [ interface-type ] [ brief [ down | description ] ] [ | { begin | exclude | include } regular-expression ]

display interface interface-type interface-number [ brief [ description ] ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: The display interface command with the brief keyword specified displays at most the first 27 characters of an interface description.

After modification: If the interface description includes more than 27 characters and the brief keyword is specified for the display interface command, you must specify the description keyword to display the full description. Without the description keyword, the command displays only the first 27 characters.

Modified feature: Displaying brief IP configuration for Layer 3 interfaces

Feature change description

The description keyword was added to the display interface brief command.

If the interface description includes more than 12 characters, you can use this keyword to display the full interface description for Layer 3 interfaces.

Command changes

Modified command: display ip interface brief

Old syntax

display ip interface [ interface-type [ interface-number ] ] brief [ | { begin | exclude | include } regular-expression ]

New syntax

display ip interface [ interface-type [ interface-number ] ] brief [ description ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: If the interface description includes more than 12 characters, only the first 9 characters of an interface description are displayed, followed by an ellipsis (…).

After modification: If the interface description includes more than 12 characters, you must specify the description keyword to display the full description. Without the description keyword, only the first 9 characters are displayed, followed by an ellipsis (...).

Modified feature: Configuring static multicast MAC address entries

Feature change description

In this release, you can configure a multicast MAC address in the value range of 0100-5Exx-xxxx and 3333-xxxx-xxxx in a static multicast MAC address entry. The x octet represents an arbitrary hexadecimal number from 0 to F.

The multicast MAC addresses used in protocol packets are in this multicast MAC adress range. If the multicast MAC address of a protocol packet matches a configured satic multicast MAC address entry on the device, one of the following occurs:

- If the protocol packet needs to be processed by the CPU, the configuration of the static multicast MAC address entry does not take effect.

- If the protocol packet needs to be transparently transmitted by the device, the device forwards the packet through the outgoing port in the matching static multicast MAC address entry.

Command changes

Modified command: mac-address multicast

Syntax

In system view:

mac-address multicast mac-address interface interface-list vlan vlan-id

undo mac-address [ multicast ] [ [ mac-address [ interface interface-list ] ] vlan vlan-id ]

In Ethernet interface view or Layer 2 aggregate interface view:

mac-address multicast mac-address vlan vlan-id

undo mac-address [ multicast ] mac-address vlan vlan-id

In port group view:

mac-address multicast mac-address vlan vlan-id

undo mac-address multicast mac-address vlan vlan-id

Views

System view, Ethernet interface view, Layer 2 aggregate interface view, port group view

Change description

Before modification: The value of the mac-address argument is any legal multicast MAC address except 0100-5Exx-xxxx and 3333-xxxx-xxxx. A multicast MAC address is the MAC address in which the least signification bit of the most significant octet is 1.

After modification: The value of the mac-address argument is any legal multicast MAC address. A multicast MAC address is the MAC address in which the least signification bit of the most significant octet is 1.

Modified command: display mac-address multicast

Syntax

display mac-address [ mac-address [ vlan vlan-id ] | [ multicast ] [ vlan vlan-id ] [ count ] ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: The value of the mac-address argument is any legal multicast MAC address except 0100-5Exx-xxxx and 3333-xxxx-xxxx. A multicast MAC address is the MAC address in which the least signification bit of the most significant octet is 1.

After modification: The value of the mac-address argument is any legal multicast MAC address. A multicast MAC address is the MAC address in which the least signification bit of the most significant octet is 1.

Modified feature: Specifying the username and password to log in to the SCP server

Feature change description

Before you transfer files through SCP, you can log in to the SCP server by using one of the following methods for password, password-publickey, or any authentication:

- Entering the username and password as prompted

- Specifying the username and password in the scp command

Command changes

Modified command: SCP

Old syntax

In non-FIPS mode:

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

In FIPS mode:

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key rsa | prefer-ctos-cipher { aes128 | aes256 } | prefer-ctos-hmac { sha1 | sha1-96 } | prefer-kex dh-group14 | prefer-stoc-cipher { aes128 | aes256 } | prefer-stoc-hmac { sha1 | sha1-96 } ] *

New syntax

In non-FIPS mode:

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } | username username password password ] *

In FIPS mode:

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key rsa | prefer-ctos-cipher { aes128 | aes256 } | prefer-ctos-hmac { sha1 | sha1-96 } | prefer-kex dh-group14 | prefer-stoc-cipher { aes128 | aes256 } | prefer-stoc-hmac { sha1 | sha1-96 } | username username password password ] *

Views

User view

Change description

Before modification: The username username password password option is not supported. You can enter the username and password only as prompted.

After modification: The username username password password option is supported. In addition to entering the username and password as prompted, you can also specify the username and password in the scp command.

The username argument specifies the username. It is a case-sensitive string of 1 to 80 characters.

The password argument specifies the password in plain text. It is a string of 1 to 63 characters.

Modified feature: Disabling an untrusted port from recording clients' IP-to-MAC bindings

Feature change description

In previous releases, you can disable only trusted ports from recording clients' IP-to-MAC bindings. In this release, both trusted and untrusted ports can be disabled from recording clients' IP-to-MAC bindings.

Command changes

Modified command: dhcp-snooping trust

Old syntax

dhcp-snooping trust [ no-user-binding ]

undo dhcp-snooping trust

New syntax

dhcp-snooping trust

undo dhcp-snooping trust

Views

Layer 2 Ethernet interface view, Layer 2 aggregate interface view

Change description

Before modification: You can use the dhcp-snooping trust command to configure a port as a trusted port, and specify the no-user-binding keyword to disable the trusted port from recording clients' IP-to-MAC bindings. Untrusted ports on the DHCP snooping device always record clients' IP-to-MAC bindings, and this function cannot be disabled.

After modification: The no-user-binding keyword is removed from the dhcp-snooping trust command. You can use the new command dhcp-snooping no-user-binding to disable a port from recording clients' IP-to-MAC bindings. The port can be either a trusted port or an untrusted port.

New command: dhcp-snooping no-user-binding

Use dhcp-snooping no-user-binding to disable a port (either trusted or untrusted) from recording clients' IP-to-MAC bindings.

Use undo dhcp-snooping no-user-binding to restore the default.

Syntax

dhcp-snooping no-user-binding

undo dhcp-snooping no-user-binding

Default

With DHCP snooping enabled, all ports record clients' IP-to-MAC bindings.

Views

Layer 2 Ethernet interface view, Layer 2 aggregate interface view

Default command level

2: System level

Examples

# Disable GigabitEthernet 1/0/1 from recording clients' IP-to-MAC bindings.

<Sysname> system-view

[Sysname] interface GigabitEthernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dhcp-snooping no-user-binding

Modified feature: Customizing DHCP options

Feature change description

Changed the value range for the code argument.

Command changes

Modified command: option

Syntax

option code { ascii ascii-string | hex hex-string&<1-16> | ip-address ip-address&<1-8> }

undo option code

Views

DHCP address pool view

Change description

Before modification: The value range for the code argument is 2 to 254, excluding 12, 50 through 55, 57 through 61, and 82.

After modification: The vaule range for the code argument is 2 to 254, excluding 50 through 54, 58, 59, 61, and 82.

# A5120EI-CMW520-R2220P11

This release has the following changes:

Modified feature: Specifying multiple public keys for an SSH user

Modified feature: ACL-based packet filtering on a VLAN interface

Modified feature: Specifying multiple public keys for an SSH user

Feature change description

When the SSH server uses the digital signature to authentication an SSH user, up to six public keys can be assigned to the user. The SSH server authenticates the user through the first matching public key.

Command changes

Modified command: ssh user

Old syntax

In non-FIPS mode:

ssh user username service-type stelnet authentication-type { password | { any | password-publickey | publickey } assign publickey keyname }

ssh user username service-type { all | scp | sftp } authentication-type { password | { any | password-publickey | publickey } assign publickey keyname work-directory directory-name }

undo ssh user username

In FIPS mode:

ssh user username service-type stelnet authentication-type { password | password-publickey assign publickey keyname }

ssh user username service-type { all | scp | sftp } authentication-type { password | password-publickey assign publickey keyname work-directory directory-name }

undo ssh user username

New syntax

In non-FIPS mode:

ssh user username service-type stelnet authentication-type { password | { any | password-publickey | publickey } assign publickey keyname&<1-6> }

ssh user username service-type { all | scp | sftp } authentication-type { password | { any | password-publickey | publickey } assign publickey keyname&<1-6> work-directory directory-name }

undo ssh user username

In FIPS mode:

ssh user username service-type stelnet authentication-type { password | password-publickey assign publickey keyname&<1-6> }

ssh user username service-type { all | scp | sftp } authentication-type { password | password-publickey assign publickey keyname&<1-6> work-directory directory-name }

undo ssh user username

Views

System view

Change description

Before modification: You can assign only one public key to an SSH user. The assign publickey keyname option is used to specify this public key.

After modification: You can assign multiple public keys to an SSH user. The assign publickey keyname&<1-6> option is used to specify these public keys, and &<1-6> indicates that up to six public keys can be specified. When multiple public keys are used, the SSH server authenticates the user through the first matching public key.

Modified feature: ACL-based packet filtering on a VLAN interface

Feature change description

In versions prior to Release 2220P11, the ACL applied to a VLAN interface filters packets forwarded at Layer 3. In Release 2220P11 and later versions, the ACL applied to a VLAN interface filters packets forwarded at Layer 3 and packets forwarded at Layer 2.

Command changes

Modified command: packet-filter

Syntax

packet-filter { acl-number | name acl-name } { inbound | outbound }

Views

Interface view

Change description

Before modification, the ACL applied to a VLAN interface filters packets forwarded at Layer 3.

After modification, the ACL applied to a VLAN interface filters packets forwarded at Layer 3 and packets forwarded at Layer 2.

Examples

As shown in Figure 1, configure packet filtering on Switch A to meet the following requirements:

Allow only packets from PC A to Switch B to pass through.

Allow PC A and PC B to communicate at Layer 2.

Network diagram

In versions before Release 2220P11, the configuration on Switch A is as follows:

<SwitchA>system-view

System View: return to User View with Ctrl+Z.

[SwitchA]acl number 3000

[SwitchA-acl-adv-3000]rule permit ip source 192.168.0.1 0 destination 10.1.1.1 0.0.0.255

[SwitchA-acl-adv-3000]rule deny ip

[SwitchA-acl-adv-3000]quit

[SwitchA]interface Vlan-interface 20

[SwitchA-Vlan-interface20]packet-filter 3000 inbound

Because the ACL does not take effect on packets forwarded at Layer 2, you just need to configure two rules in the following order:

A permit rule that permits packets from PC A to Switch B.

A deny rule that denies all packets.

In Release 2220P11 and later versions, the configuration on Switch A is as follows:

<SwitchA>system-view

System View: return to User View with Ctrl+Z.

[SwitchA]acl number 3000

[SwitchA-acl-adv-3000]rule permit ip source 192.168.0.1 0 destination 10.1.1.1 0.0.0.255

[SwitchA-acl-adv-3000]rule permit ip source 192.168.0.1 0.0.0.255 destination 192.168.0.10 0.0.0.255

[SwitchA-acl-adv-3000]rule deny ip

[SwitchA-acl-adv-3000]quit

[SwitchA]interface Vlan-interface 20

[SwitchA-Vlan-interface20]packet-filter 3000 inbound

Because the ACL takes effect on packets forwarded at Layer 3 and packets forwarded at Layer 2, you need to configure one more permit rule to permit packets from PC A to PC B. Configure the rules in the following order:

                - A permit rule that permits packets from PC A to Switch B.

A permit rule that permits packets from PC A to PC B.

A deny rule that denies all packets.

# A5120EI-CMW520-R2220P09

This release has the following changes:

New feature: 802.1X-based dynamic IPv4 source guard binding entries

New feature: Multicast ND

New feature: Configuring packet capture

New feature: Enabling MAC authentication multi-VLAN mode

New feature: Binding IP, MAC, and port on Web

New feature: Configuring the ARP detection logging function

Modified feature: Configuring system information for the SNMP agent

Modified feature: Specifying multiple secondary HWTACACS servers

  - New feature: 802.1X-based dynamic IPv4 source guard binding entries

    - Overview

To protect 802.1X users from IP attacking, you can enable 802.1X to cooperate with IP source guard. This IP source guard feature generates dynamic IPv4 source guard binding entries based on 802.1X secure entries. It can filter out IPv4 packets from unauthenticated 802.1X users.

To deny any online authenticated 802.1X users to change their IP addresses, you can enable the 802.1X IP freezing function on the authentication port. The port saves the IP addresses of 802.1X users when they get online, and it does not update these IP addresses even if the IP addresses of these users have changed. If an online authenticated 802.1X user changes its IP address, the port denies the user to access the network, because the user's IP address does not match any IPv4 source guard binding entries.

Configuration procedure

Configuration task list

| Task | Remarks |
| --- | --- |
| Enabling 802.1X | For more information about 802.1X, see Security Configuration Guide. |
| Enabling the 802.1X IP freezing function | Optional. |
| Enabling a port to generate 802.1X-based dynamic IPv4 source guard binding entries | N/A |
| Enabling the IPv4 source guard function on an interface | See the ip verify source { ip-address ¦ ip-address mac-address ¦ mac-address } command. For more information about IP source guard, see Security Configuration Guide. |

Enabling the 802.1X IP freezing function

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| - Enable the 802.1X IP freezing function. | dot1x user-ip freeze | By default, the port saves the IP address received from an 802.1X user and updates the IP address when it receives a different IP address from the same user. |

Enabling a port to generate 802.1X-based dynamic IPv4 source guard binding entries

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| - Enable the port to generate 802.1X-based dynamic IPv4 source guard binding entries. | ip verify source dot1x | By default, this function is disabled. |

|  | NOTE: If the 802.1X client does not upload users' IP addresses to the device, you must enable DHCP snooping or ARP snooping on the device. Then 802.1X can obtain the IP addresses of 802.1X users for the device to generate 802.1X-based dynamic IP source guard binding entries. |
| --- | --- |

Command reference

      - dot1x user-ip freeze

Syntax

dot1x user-ip freeze

undo dot1x user-ip freeze

View

Layer 2 Ethernet interface view

Default level

2: System level

Description

Use dot1x user-ip freeze to enable the 802.1X IP freezing function.

Use undo dot1x user-ip freeze to restore the default.

By default, a port saves the IP address received from an 802.1X user and updates the IP address when it receives a different IP address from the same user.

Examples

# Enable 802.1X IP freezing on port GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x user-ip freeze

      - ip verify source dot1x

Syntax

ip verify source dot1x

undo ip verify source dot1x

View

Layer 2 Ethernet interface view

Default level

2: System level

Description

Use ip verify source dot1x to enable a port to generate 802.1X-based dynamic IPv4 source guard binding entries.

Use undo ip verify source dot1x to remove the 802.1X-based dynamic IPv4 source guard binding entries.

By default, a Layer 2 Ethernet port generates dynamic IPv4 source guard binding entries based on DHCP snooping.

Executing the undo ip verify source dot1x command or disabling 802.1X on the port will remove all 802.1X-based dynamic IPv4 source guard binding entries on the port.

The port will remove the dynamic IPv4 source guard binding entry of an 802.1X user after the user gets offline.

Example

# Enable port GigabitEthernet 1/0/1 to generate 802.1X-based dynamic IPv4 source guard binding entries.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] ip verify source dot1x

New feature: Multicast ND

Configuring multicast ND

Microsoft NLB is a load balancing technology for server clustering developed on Windows Server.

NLB supports load sharing and redundancy among servers within a cluster. To implement fast failover, NLB requires that the switch forwards network traffic to all servers or specified servers in the cluster, and each server filters out unexpected traffic. In a medium or small data center that uses the Windows Server operating system, the proper cooperation of the switch and NLB is very important. For more information about NLB, see the related documents for Windows Server.

Microsoft NLB provides the following packet sending modes to make the switch forward network traffic to all servers or specified servers:

- Unicast mode—NLB assigns each cluster member a common MAC address, which is the cluster MAC address, and changes the source MAC address of each sent packet. The switch cannot add the cluster MAC address to its MAC table. In addition, because the cluster MAC address is unknown to the switch, packets destined to it are forwarded on all ports of the switch.

- Multicast mode—NLB uses a multicast MAC address that is a virtual MAC address for network communication (for example 0300-5e11-1111).

- Internet Group Management Protocol (IGMP) multicast mode—The switch sends packets only out of the ports that connect to the cluster members rather than all ports.

|  | NOTE: Multicast ND is applicable to only multicast-mode NLB. |
| --- | --- |

To configure multicast ND:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Configure a static neighbor entry. | ipv6 neighbor ipv6-address mac-address vlan-id port-type port-number | Optional. |
| - Configure a static multicast MAC address entry. | mac-address multicast mac-address interface interface-list vlan vlan-id | No static multicast MAC address entries exist by default. |

Command reference

For more information about the mac-address multicast command, see "IGMP Snooping Commands" in HP A5120 EI Switch Series IP Multicast Command Reference-R2208.

For more information about the ipv6 neighbor command, See "IPv6 basics configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-R2208.

New feature: Configuring packet capture

Overview

The packet capture feature facilitates network problem identification. Packets captured are stored in the packet capture buffer on the device. You can display the packets at the CLI, or export them to a .pcap file and analyze them by using packet analysis software such as Ethereal or Wireshark.

Configuring the packet capture function

When you configure this function, follow these guidelines:

- After you enable packet capture which uses an ACL, you cannot modify the ACL rules, including adding, deleting, and modifying rules.

- When you enable packet capture which uses an ACL, the actions in the ACL are ignored, and the ACL is only used for traffic classification.

- To release system resources after finishing packet capture, use the undo packet capture command to disable this function.

To configure the packet capture function:

| Step | Command | Remarks |
| --- | --- | --- |
| - Set packet capture parameters. | packet capture { acl { acl-number ¦ ipv6 acl6-number } ¦ buffer-size size ¦ length capture-length ¦ mode { circular ¦ linear } }* | Optional. |
| - Enable packet capture. | - (Approach 1) Start packet capture immediately: packet capture start [ acl { acl-number ¦ ipv6 acl6-number } ¦ buffer-size size ¦ length capture-length ¦ mode { circular ¦ linear } ¦ [ packets packet-number ¦ seconds second-number ] ]* - (Approach 2) Configure a packet capture schedule: packet capture schedule datetime time date | Use either approach. You can set packet capture parameters at the same time when you use approach 1. By default, packet capture is disabled, and no packet capture schedule is configured. If you use approach 1, the existing packet capture schedule is invalid. |
| - Stop packet capture. | packet capture stop | Optional. Stop packet capture before you display, save, or clear the buffered contents. The device automatically stops packet capture when: - The packet capture function operates in linear mode, and the packet capture buffer is full. - The number of packets captured exceeds the upper limit. - The duration of the packet capture process exceeds the upper limit. |
| - Save the contents in the packet capture buffer. | packet capture buffer save [ filename ] | Optional. Save the file with a filename in .pcap format. |

Displaying and maintaining packet capture

| Task | Command | Remarks |
| --- | --- | --- |
| Display the current packet capture status. | display packet capture status | Available in any view. |
| Display the buffered contents. | display packet capture buffer [ start-index [ end-index ] ] [ length display-length ] | Available in any view. |
| Clear the buffered contents. | reset packet capture buffer | Available in user view. |

Packet capture configuration example

      - Network requirements

As shown in Figure 1, the switch captures the packets from 192.168.1.0/24, and saves the result in a .pcap file so that the PC can download the file for packet analysis.

Network diagram

      - Configuration procedure

            - Enable the packet capture function on the switch:

# Create an ACL rule for IPv4 basic ACL 2000 to permit packets with a source address in 192.168.1.0/24.

<Switch> system-view

[Switch] acl number 2000

[Switch-acl-basic-2000] rule permit source 192.168.1.0 0.0.0.255

[Switch-acl-basic-2000] quit

[Switch] quit

# Configure the switch to capture packets based on ACL 2000, and start packet capture immediately.

<Switch> packet capture start acl 2000

# Display the packet capture status.

<Switch> display packet capture status

Current status :          In process

Mode :                    Linear

Buffer size :             2097152 (bytes)

Buffer used :             1880 (bytes)

Max capture length :      68 (bytes)

ACL information :         Basic or advanced IPv4 ACL 2000

Schedule datetime:        Unspecified

Upper limit of duration : Unspecified (seconds)

Duration :                13 (seconds)

Upper limit of packets :  Unspecified

Packets count :           10

The output shows that packet capture is ongoing.

            - Save the packet capture result:

# Stop packet capture.

<Switch> packet capture stop

# Save the contents in the packet capture buffer to file test.pcap.

<Switch> packet capture buffer save test.pcap

# Display the contents and file information in the current directory.

<Switch> dir

Directory of flash:/

0     -rw-      1860  Sep 21 2012 12:52:58   test.pcap

1     drw-         -  Apr 26 2012 12:00:38   seclog

2     -rw-  10479398  Apr 26 2012 12:26:39   logfile.log

The output shows that the buffered contents are successfully saved.

# Stop packet capture, and release system resources after packet capture is completed.

<Switch> undo packet capture

The PC can access the switch through FTP or TFTP, save file test.pcap, and analyze the packets through packet analysis software such as Wireshark.

Command reference

display packet capture buffer

Syntax

display packet capture buffer [ start-index [ end-index ] ] [ length display-length ]

View

Any view

Default level

1: Monitor level

Parameters

start-index: Specifies a start packet record by its index in the packet capture buffer. If you do not specify this argument, the earliest packet record is displayed the first in the packet capture buffer by default.

end-index: Specifies an end packet record by its index in the packet capture buffer. If you do not specify this argument, the latest packet record is displayed the last in the packet capture buffer by default.

length display-length: Specifies the maximum length of data that can be displayed for a single packet record, in the range of 14 to 256 bytes. The default value is 68.

Description

Use display packet capture buffer to display the contents in the packet capture buffer.

- If you do not specify any option, the command displays all packet records in the packet capture buffer.

- This command limits the length of data that can be displayed for a single packet record. To display complete packet records, use the packet capture buffer save command to save the contents in a .pcap file, and display the contents by using the corresponding software.

- Do not use this command during the packet capturing process.

Related commands: packet capture start and packet capture buffer save.

Examples

# Display all contents in the packet capture buffer.

<Sysname> display packet capture buffer

2012-07-26 12:03:15:318  Index 1  GE1/0/2  64 (original 64) Bytes captured

01 80 c2 00 00 03 1c bd b9 e3 b5 02 81 00 00 01

88 8e 01 01 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

2012-07-26 12:03:25:749  Index 2  GE1/0/2  68 (original 90) Bytes captured

33 33 00 00 00 12 00 00 5E 00 02 50 86 DD 6E 00

00 00 00 20 70 FF FE 80 00 00 00 00 00 00 00 00

00 00 00 00 00 81 FF 02 00 00 00 00 00 00 00 00

00 00 00 00 00 12 31 50 64 01 02 58 6A AE FE 80

00 00 00 00

display packet capture status

Syntax

display packet capture status

View

Any view

Default level

1: Monitor level

Parameters

None

Description

Use display packet capture status to display the current packet capture status.

Examples

# Display the current packet capture status.

<Sysname> display packet capture status

Current status :          In process

Mode :                    Linear

Buffer size :             2097152 (bytes)

Buffer used :             0 (bytes)

Max capture length :      68 (bytes)

ACL information :         Ethernet frame header ACL 4200

Schedule datetime:        Unspecified

Upper limit of duration : Unspecified (seconds)

Duration :                60 (seconds)

Upper limit of packets :  Unspecified

Packets count :           0

        - Command output

| Field | Description |
| --- | --- |
| Current status | Packet capture status: - In process—The packet capturing process is ongoing. - Scheduled—The packet capture schedule is configured, but does not start. - Paused—Packet capture is stopped temporarily, and you can display, save, and clear the contents in the packet capture buffer. |
| Mode | Packet capture mode: - Linear. - Circular. |
| Buffer size | Packet capture buffer size. |
| Buffer used | Packet capture buffer size in use. One packet record comprises a packet header that records the incoming port, capture time, length of the captured packet and the actual length of the packet, and the data, so it occupies more buffer memory than the maximum captured data. |
| Max capture length | Maximum length of data that can be captured for a packet. |
| ACL information | ACL type and number for packet capture. |
| Schedule datetime | Start time of the packet capture schedule. |
| Upper limit of duration | Upper limit of the packet capture duration. |
| Duration | Packet capture duration. |
| Upper limit of packets | Maximum number of packets that can be captured. |
| Packets count | Number of packets that has been captured. |

packet capture

Syntax

packet capture { acl { acl-number | ipv6 acl6-number } | buffer-size size | length capture-length | mode { circular | linear } }*

undo packet capture [ acl |buffer-size | length | mode ]

View

User view

Default level

1: Monitor level

Parameters

acl: Specifies an ACL for packet capture. If you do not specify this keyword, this command captures all packets that the device receives.

acl-number: Specifies the number of an IPv4 ACL:

- 2000 to 2999 for IPv4 basic ACLs

- 3000 to 3999 for IPv4 advanced ACLs

- 4000 to 4999 for Ethernet frame header ACLs

acl6-number: Specifies the number of an IPv6 ACL:

- 2000 to 2999 for IPv6 basic ACLs

- 3000 to 3999 for IPv6 advanced ACLs

buffer-size size: Specifies the packet capture buffer size in the range of 32 to 65535 KB. The default value is 2048.

length capture-length: Specifies the maximum length of the data that can be captured for a packet, calculated from the first byte of the packet, in the range of 16 to 4000 bytes. The default value is 68. The data out of the range of the maximum length is not recorded.

circular: Specifies the circular packet capture mode. In this mode, packet capture continues even if the buffer is full, and the newly captured packet overwrites the previous records, starting from the earliest one.

linear: Specifies the linear packet capture mode. In this mode, packet capture pauses when the buffer is full. The default mode is linear mode.

Description

Use packet capture to set packet capture parameters.

Use undo packet capture to restore the default settings, and disable the packet capture function.

- Do not change packet capture parameters during the packet capturing process.

- After you enable packet capture which uses an ACL, you cannot modify the ACL rules, including adding, deleting, and modifying rules.

- When you enable packet capture which uses an ACL, the actions in the ACL are ignored, and the ACL is only used for traffic classification.

- If you specify a keyword for the undo packet capture command, the command restores the default setting for the specified keyword. If you do not specify any keyword, the command restores the default settings for all keywords, and disables the packet capture function.

Related commands: packet capture start.

Examples

# Set the size of the packet capture buffer to 4096 KB, the source address of packets to be captured to 192.168.1.0/24, and start packet capture immediately.

<Sysname> system-view

[Sysname] acl number 2000

[Sysname-acl-basic-2000] rule permit source 192.168.1.0 0.0.0.255

[Sysname-acl-basic-2000] quit

[Sysname] quit

<Sysname> packet capture buffer-size 4096

<Sysname> packet capture acl 2000

<Sysname> packet capture start

# Restore the default settings for packet capture parameters, and disable packet capture.

<Sysname> undo packet capture

packet capture buffer save

Syntax

packet capture buffer save [ filename ]

View

User view

Default level

1: Monitor level

Parameters

filename: Specifies the name of the file to be saved. The filename cannot contain special characters such as backslash (\), slash (/), colon (:), asterisk (*), quotation marks (" "), single quotes (' '), less-than sign (<), greater-than sign (>), and vertical bar (|). If you do not specify this argument, the command saves the file in the default filename pcapbuffer.pcap.

Description

Use packet capture buffer save to save the contents in the packet capture buffer.

- Save the file with a filename in the .pcap format.

- Do not use this command during the packet capturing process.

Related commands: packet capture.

Examples

# Save the contents in the packet capture buffer to file example.pcap.

<Sysname> packet capture buffer save example.pcap

packet capture schedule

Syntax

packet capture schedule datetime time date

undo packet capture schedule

View

User view

Default level

1: Monitor level

Parameters

time: Sets the time in the format of HH:MM:SS. HH takes a value range of 0 to 23, and MM and SS take a value range of 0 to 59.

date: Sets the date in the format of MM/DD/YYYY or YYYY/MM/DD. MM takes a value range of 1 to 12, YYYY takes a value range of 2000 to 2035, and the value range of DD depends on which month the day is in.

Description

Use packet capture schedule to configure a packet capture schedule.

Use undo packet capture schedule to invalidate the configured packet capture schedule.

By default, no packet capture schedule is configured.

- You can use the packet capture start command to enable packet capture as in this command.

- You can use the packet capture command to change packet capture parameters before the packet capture schedule starts, or use the packet capture start command to start packet capture immediately, and the existing packet capture schedule is invalidated.

- To disable packet capture and invalidate the configured packet capture schedule, execute the undo packet capture start command or the undo packet capture command without any keyword.

Related commands: packet capture.

Examples

# Configure a packet capture schedule.

<Sysname> packet capture schedule datetime 12:00:00 2012/12/25

packet capture start

Syntax

packet capture start [ acl { acl-number | ipv6 acl6-number } | buffer-size size | length capture-length | mode { circular | linear } | [ packets packet-number | seconds second-number ] ]*

undo packet capture start

View

User view

Default level

1: Monitor level

Parameters

acl: Specifies an ACL for packet capture. If you do not specify this keyword, this command captures all packets that the device receives.

acl-number: Specifies the number of an IPv4 ACL:

- 2000 to 2999 for IPv4 basic ACLs

- 3000 to 3999 for IPv4 advanced ACLs

- 4000 to 4999 for Ethernet frame header ACLs

acl6-number: Specifies the number of an IPv6 ACL:

- 2000 to 2999 for IPv6 basic ACLs

- 3000 to 3999 for IPv6 advanced ACLs

buffer-size size: Specifies the packet capture buffer size in the range of 32 to 65535 KB. The default value is 2048.

length capture-length: Specifies the maximum length of the data that can be captured for a packet, calculated from the first byte of the packet, in the range of 16 to 4000 bytes. The default value is 68. The data out of the range of the maximum length is not recorded.

circular: Specifies the circular packet capture mode. In this mode, packet capture continues even if the buffer is full, and the newly captured packet overwrites the previous records, starting from the earliest one.

linear: Specifies the linear packet capture mode. In this mode, packet capture pauses when the buffer is full. The default mode is linear mode.

packets packet-number: Sets the upper limit of packets that can be captured, in the range of 1 to 4294967295. The default value is 4294967295. Packet capture pauses when the number of captured packets reaches the upper limit.

seconds second-number: Sets the upper limit for packet capture duration, in the range of 1 to 4294967295 seconds. The default value is 4294967295 seconds. Packet capture pauses when the packet capture duration reaches the upper limit.

Description

Use packet capture start to start packet capture, and set packet capture parameters at the same time.

Use undo packet capture start to disable packet capture.

By default, packet capture is disabled.

- Do not start packet capture again or change parameters, or use the display packet capture buffer, reset packet capture buffer and packet capture buffer save commands during the packet capturing process. To do so, use the packet capture stop command to temporarily stop packet capture.

- If packet capture is enabled and an ACL number is specified, but the specified ACL does not exist, no packet is captured. If you modify the ACL rule for the specified ACL, the result of packet capture is not affected. The modified ACL rule takes effect after the packet capture start command is successfully executed.

- The undo packet capture start command stops packet capture, but the packet capture parameters configured are still effective, and you do no need to reconfigure them when you start packet capture again.

Related commands: packet capture stop, display packet capture status, and display packet capture buffer.

Examples

# Set the maximum length of the packet captured as 256 bytes, and start packet capture.

<Sysname> packet capture length 256 start

packet capture stop

Syntax

packet capture stop

View

User view

Default level

1: Monitor level

Parameters

None

Description

Use packet capture stop to temporarily stop packet capture.

- After packet capture is stopped, if you use the packet capture command to change packet capture parameters, the contents in the capture buffer are cleared.

- This command does not take effect if packet capture is not started.

- After packet capture is stopped, you can use the display packet capture buffer, reset packet capture buffer, or packet capture buffer save command to display or perform operations on the contents in the packet capture buffer, and use the packet capture start command to start packet capture again.

Related commands: packet capture, packet capture start, display packet capture buffer, reset packet capture buffer, and packet capture buffer save.

Examples

# Stop packet capture.

<Sysname> packet capture stop

reset packet capture buffer

Syntax

reset packet capture buffer

View

User view

Default level

1: Monitor level

Parameters

None

Description

Use reset packet capture buffer to clear the contents in the packet capture buffer.

Do not use this command during the packet capturing process.

Related commands: packet capture start.

Examples

# Clear the contents in the packet capture buffer.

<Sysname> reset packet capture buffer

New feature: Enabling MAC authentication multi-VLAN mode

Overview

By default, a MAC authentication-enabled port forwards packets for an authenticated user only in the VLAN where the user is authenticated. If the user forwards packets in a different VLAN, the port must re-authenticate the user. After the user passes re-authentication, the port will update the MAC and VLAN mapping of the user. For a user that sends various types of traffic (for example, data, video, and audio) in multiple VLANs, frequent MAC re-authentication can downgrade the system performance and affect data transmission quality.

The MAC authentication multi-VLAN mode enables a MAC authentication-enabled port to forward packets for an authenticated user in up to five VLANs without re-authentication.

For example, an IP phone can send tagged and untagged frames, the IP phone is connected to a MAC authentication-enabled port, and the port receives tagged frames in VLAN 2 and untagged frames in VLAN 1. Before you enable the multi-VLAN mode on the port, the port must re-authenticate the IP phone repeatedly, because it sends tagged frames and untagged frames alternately in different VLANs. After you enable the multi-VLAN mode, the port can receive tagged and untagged frames alternately from the IP phone without triggering a MAC re-authentication. The multi-VLAN mode improves the transmission quality of data that is vulnerable to delay and interference.

Configuration procedure

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| - Enable MAC authentication multi-VLAN mode. | mac-authentication host-mode multi-vlan | By default, A MAC-authenticated user only can forward packets in the VLAN where it was authenticated. |

Command reference

mac-authentication host-mode multi-vlan

Syntax

mac-authentication host-mode multi-vlan

undo mac-authentication host-mode multi-vlan

View

Layer 2 Ethernet interface view

Default level

2: System level

Description

Use mac-authentication host-mode multi-vlan to enable MAC authentication multi-VLAN mode on a port.

Use undo mac-authentication host-mode multi-vlan to restore the default.

By default, the MAC authentication multi-VLAN mode is disabled on a port.

The multi-VLAN mode enables a MAC-authenticated user to forward packets in multiple VLANs on the port without re-authentication. The device supports a maximum of four such VLANs on a port.

Examples

# Enable MAC authentication multi-VLAN mode on port GigabitEthernet 1/0/2.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/2

[Sysname-GigabitEthernet1/0/2] mac-authentication host-mode multi-vlan

New feature: Binding IP, MAC, and port on Web

Overview

None.

Command reference

None.

  - New feature: Configuring the ARP detection logging function

The ARP detection logging function enables a device to generate ARP detection log messages when ARP packet attacks are detected. An ARP detection log message can include the following information:

Receiving interface of the ARP packets.

Sender IP address.

Total number of ARP packets dropped.

The following is an example of an ARP detection log message:

Detected an inspection occurred on interface GigabitEthernet 1/0/1 with IP address 172.18.48.55 (Totally 10 packets dropped).

    - Configuring the ARP detection logging function

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - (Optional) Enable the ARP detection logging function. | arp detection log enable | By default, the ARP detection logging function is enabled. |

    - Command reference

      - arp detection log enable

Use arp detection log enable to enable logging for ARP detection.

Use undo detection log enable to disable the logging function for ARP detection

Syntax

arp detection log enable

undo arp detection log enable

Default

Logging is enabled for ARP detection

View

System view

Default level

3: Manage level

Examples

# Enable logging for ARP detection.

<Sysname> system-view

[Sysname] arp detection enable

Modified feature: Configuring system information for the SNMP agent

Feature change description

Modify the maximum string length of the sys-contact and sys-location arguments.

Command changes

Modified command: snmp-agent sys-info

Syntax

snmp-agent sys-info { contact sys-contact | location sys-location | version { all | { v1 | v2c | v3 }* } }

Views

System view

Change description

Before modification: Both the sys-contact and sys-location arguments specify a string of 1 to 200 characters.

After modification: Both the sys-contact and sys-location arguments specify a string of 1 to 255 characters.

Modified feature: Specifying multiple secondary HWTACACS servers

Feature change description

In this release, you can specify one primary HWTACACS server and up to 16 secondary HWTACACS servers in the same HWTACACS scheme. When the primary HWTACACS server is unreachable, the device uses a secondary HWTACACS server to process AAA requests.

You can configure a shared key for each HWTACACS server, primary or secondary. The device uses the shared keys to ensure secure communication with HWTACACS servers.

Command changes

Modified command: primary accounting

Old syntax

primary accounting ip-address [ port-number ]

undo primary accounting

New syntax

primary accounting ip-address [ port-number | key [ cipher | simple ] key ] *

undo primary accounting

Views

HWTACACS scheme view

Change description

The key [ cipher | simple ] key part is added to the primary accounting command. You can specify a shared key for secure communication between the device and the primary HWTACACS accounting server. Make sure the shared key configured on the device is the same as the one configured on the server.

- cipher key: Sets a ciphertext shared key. The key argument is case sensitive.

  - In non-FIPS mode, the key is a string of 1 to 373 characters.

  - In FIPS mode, the key is a string of 8 to 373 characters.

- simple key: Sets a plaintext shared key. The key argument is case sensitive.

  - In non-FIPS mode, the key is a string of 1 to 255 characters.

  - In FIPS mode, the key is a string of 8 to 255 characters and must contain digits, uppercase letters, lowercase letters, and special characters.

|  | NOTE: If you specify neither the cipher keyword nor the simple keyword, the shared key is set in plain text. |
| --- | --- |

Modified command: primary authentication

Old syntax

primary authentication ip-address [ port-number ]

undo primary authentication

New syntax

primary authentication ip-address [ port-number | key [ cipher | simple ] key ] *

undo primary authentication

Views

HWTACACS scheme view

Change description

The key [ cipher | simple ] key part is added to the primary authentication command. You can specify a shared key for secure communication between the device and the primary HWTACACS authentication server. Make sure the shared key configured on the device is the same as the one configured on the server.

- cipher key: Sets a ciphertext shared key. The key argument is case sensitive.

  - In non-FIPS mode, the key is a string of 1 to 373 characters.

  - In FIPS mode, the key is a string of 8 to 373 characters.

- simple key: Sets a plaintext shared key. The key argument is case sensitive.

  - In non-FIPS mode, the key is a string of 1 to 255 characters.

  - In FIPS mode, the key is a string of 8 to 255 characters and must contain digits, uppercase letters, lowercase letters, and special characters.

|  | NOTE: If you specify neither the cipher keyword nor the simple keyword, the shared key is set in plain text. |
| --- | --- |

Modified command: primary authorization

Old syntax

primary authorization ip-address [ port-number ]

undo primary authorization

New syntax

primary authorization ip-address [ port-number | key [ cipher | simple ] key ] *

undo primary authorization

Views

HWTACACS scheme view

Change description

The key [ cipher | simple ] key part is added to the primary authorization command. You can specify a shared key for secure communication between the device and the primary HWTACACS authorization server. Make sure the shared key configured on the device is the same as the one configured on the server.

- cipher key: Sets a ciphertext shared key. The key argument is case sensitive.

  - In non-FIPS mode, the key is a string of 1 to 373 characters.

  - In FIPS mode, the key is a string of 8 to 373 characters.

- simple key: Sets a plaintext shared key. The key argument is case sensitive.

  - In non-FIPS mode, the key is a string of 1 to 255 characters.

  - In FIPS mode, the key is a string of 8 to 255 characters and must contain digits, uppercase letters, lowercase letters, and special characters.

|  | NOTE: If you specify neither the cipher keyword nor the simple keyword, the shared key is set in plain text. |
| --- | --- |

Modified command: secondary accounting

Old syntax

secondary accounting ip-address [ port-number ]

undo secondary accounting

New syntax

secondary accounting ip-address [ port-number | key [ cipher | simple ] key ] *

undo secondary accounting [ ip-address ]

Views

HWTACACS scheme view

Change description

This command has the following modifications:

- The key [ cipher | simple ] key part is added to the secondary accounting command. You can use this command to specify a shared key for secure communication between the device and a secondary HWTACACS accounting server. Make sure the shared key configured on the device is the same as the one configured on that server.

  - cipher key: Sets a ciphertext shared key. The key argument is case sensitive.

    - In non-FIPS mode, the key is a string of 1 to 373 characters.

    - In FIPS mode, the key is a string of 8 to 373 characters.

  - simple key: Sets a plaintext shared key. The key argument is case sensitive.

    - In non-FIPS mode, the key is a string of 1 to 255 characters.

    - In FIPS mode, the key is a string of 8 to 255 characters and must contain digits, uppercase letters, lowercase letters, and special characters.

|  | NOTE: If you specify neither the cipher keyword nor the simple keyword, the shared key is set in plain text. |
| --- | --- |

- The ip-address argument is added to the undo secondary accounting command. You can remove a secondary HWTACACS accounting server with this command by specifying its IP address.

Modified command: secondary authentication

Old syntax

secondary authentication ip-address [ port-number ]

undo secondary authentication

New syntax

secondary authentication ip-address [ port-number | key [ cipher | simple ] key ] *

undo secondary authentication [ ip-address ]

Views

HWTACACS scheme view

Change description

This command has the following modifications:

- The key [ cipher | simple ] key part is added to the secondary authentication command. You can specify a shared key for secure communication between the device and a secondary HWTACACS authentication server. Make sure the shared key configured on the device is the same as the one configured on the server.

  - cipher key: Sets a ciphertext shared key. The key argument is case sensitive.

    - In non-FIPS mode, the key is a string of 1 to 373 characters.

    - In FIPS mode, the key is a string of 8 to 373 characters.

  - simple key: Sets a plaintext shared key. The key argument is case sensitive.

    - In non-FIPS mode, the key is a string of 1 to 255 characters.

    - In FIPS mode, the key is a string of 8 to 255 characters and must contain digits, uppercase letters, lowercase letters, and special characters.

|  | NOTE: If you specify neither the cipher keyword nor the simple keyword, the shared key is set in plain text. |
| --- | --- |

- The ip-address argument is added to the undo secondary authentication command. You can remove a secondary HWTACACS authentication server with this command by specifying its IP address.

Modified command: secondary authorization

Old syntax

secondary authorization ip-address [ port-number ]

undo secondary authorization

New syntax

secondary authorization ip-address [ port-number | key [ cipher | simple ] key ] *

undo secondary authorization [ ip-address ]

Views

HWTACACS scheme view

Change description

This command has the following modifications:

- The key [ cipher | simple ] key part is added to the secondary authorization command. You can specify a shared key for secure communication between the device and a secondary HWTACACS authorization server. Make sure the shared key configured on the device is the same as the one configured on the server.

  - cipher key: Sets a ciphertext shared key. The key argument is case sensitive.

    - In non-FIPS mode, the key is a string of 1 to 373 characters.

    - In FIPS mode, the key is a string of 8 to 373 characters.

  - simple key: Sets a plaintext shared key. The key argument is case sensitive.

    - In non-FIPS mode, the key is a string of 1 to 255 characters.

    - In FIPS mode, the key is a string of 8 to 255 characters and must contain digits, uppercase letters, lowercase letters, and special characters.

|  | NOTE: If you specify neither the cipher keyword nor the simple keyword, the shared key is set in plain text. |
| --- | --- |

- The ip-address argument is added to the undo secondary authorization command. You can remove a secondary HWTACACS authorization server with this command by specifying its IP address.

# A5120EI-CMW520-R2220P02

This chapter includes following contents:

- Modified feature: Enabling/disabling FIPS mode

- Modified feature: Setting the IRF link down report delay

- Modified feature: Setting the minimum password length

- Modified feature: Switching the user privilege level

- Modified feature: Implementing ACL-based IPsec

- Modified feature: Cluster management

Modified feature: Enabling/disabling FIPS mode

Feature change description

Added prompt information for the fips mode enable and undo fips mode enable commands:

[Sysname] fips mode enable

FIPS mode change requires a device reboot. Continue?[Y/N]:y

Change the configuration to meet FIPS mode requirements, save the configuration to the next-startup configuration file, and then reboot to enter FIPS mode.

[Sysname] undo fips mode enable

FIPS mode change requires a device reboot. Continue?[Y/N]:y

Change the configuration to meet non-FIPS mode requirements, save the configuration to the next-startup configuration file, and then reboot to enter non-FIPS mode.

    - Command changes

None

Modified feature: Setting the IRF link down report delay

Feature change description

Changed the value range of the interval argument.

Command changes

Modified command: irf link-delay

Syntax

irf link-delay interval

Views

System view

Change description

Before modification: The value range (in milliseconds) for the interval argument is 0 to 3000.

After modification: The value range (in milliseconds) for the interval argument is 0 to 10000.

Modified feature: Setting the minimum password length

Feature change description

Changed the value range of the minimum password length.

Command changes

Modified command: password-control length

Syntax

password-control length length

undo password-control length

Views

System view, user group view, local user view

Change description

Before modification: The value range for the length argument is 8 to 32.

After modification: The value range for the length argument is 8 to 32 in FIPS mode and 4 to 32 in non-FIPS mode.

Modified feature: Switching the user privilege level

Feature change description

Changed the user privilege level switching control mechanism.

Command changes

Modified command: super

Syntax

super [ level ]

Views

User view

Change description

Before modification: If a scheme authentication user fails to provide the correct password for the higher privilege level during 3 consecutive attempts, the system does not lock the switching function.

After modification: If a scheme authentication user fails to provide the correct password for the higher privilege level during 5 consecutive attempts, the system locks the switching function, and the user must wait 15 minutes before trying again. Trying again before the 15-minute period elapses restores the wait timer to 15 minutes and restarts the timer.

Modified feature: Implementing ACL-based IPsec

Feature change description

ACL-based IPsec can protect only traffic that is generated by the device and traffic that is destined for the device. You cannot use an ACL-based IPsec tunnel to protect user traffic. In the ACL that is used to identify IPsec protected traffic, ACL rules that match traffic forwarded through the device do not take effect. For example, an ACL-based IPsec tunnel can protect log messages the device sends to a log server, but it cannot protect traffic that is forwarded by the device for two hosts, even if the host-to-host traffic matches an ACL permit rule.

Command changes

None

Modified feature: Cluster management

Cluster management is not supported in FIPS mode.

# A5120EI-CMW520-R2220P01

This release has no feature changes.

None

# A5120EI-CMW520-R2220

This chapter includes following contents:

New feature: Disabling password recovery capacity

New feature: Configuring a port to forward 802.1X EAPOL packets untagged

New feature: Enabling source IP conflict prompt

New feature: Delaying the MAC authentication

New feature: Disabling MAC entry aging timer refresh based on destination MAC address

New feature: Setting the deletion delay time for SAVI

Modified feature: Default configuration

New feature: Disabling password recovery capacity

  - Disabling password recovery capacity

Password recovery capability controls console user access to the device configuration and NVRAM from BootWare menus.

If password recovery capability is enabled, a console user can access the device configuration without authentication and reconfigure the console login password and user privilege level passwords.

If password recovery capability is disabled, a console user must restore the factory-default configuration before configuring new passwords. Restoring the factory-default configuration deletes the next-startup configuration files.

To enhance system security:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Disable password recovery capacity. | undo password-recovery enable | By default, password recovery capability is enabled. |

For more information about BootWare menus and password recovery capacity, see appendix B in HP A5120EI-CMW520-R2220 Release Notes Release Notes.s

  - Command reference

password-recovery enable

Syntax

password-recovery enable

undo password-recovery enable

View

System view

Default level

3: Manage level

Description

Use password-recovery enable to enable password recovery capability.

Use undo password-recovery enable to disable password recovery capability.

By default, password recovery capability is enabled.

To enhance system security, disable password recovery capability.

Examples

# Disable password recovery capability.

<Sysname> system-view

[Sysname] undo password-recovery enable

New feature: Configuring a port to forward 802.1X EAPOL packets untagged

Configuring a port to forward 802.1X EAPOL packets untagged

After an 802.1X user passes authenticaiton, the 802.1X server assigns authorization attributes to the access device. If the port is assgined to a VLAN as a tagged member, the port that connects the clients forwards packets tagged. 802.1X defines EAP over LAN (EAPOL) for passing EAP packets between the client and the network access device over a wired or wireless LAN. An EAPOL-format 802.1X packet cannot carry any VLAN tag in its header. To ensure the communication between the client and the network access device, you can configure the port that connects the client and the network access device to forward 802.1X EAPOL packets after removing the tag.

To configure a port to forward 802.1X EAPOL packets untagged:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| ##### Configure the port to forward 802.1X EAPOL packets untagged. | dot1x eapol untag | Optional. By default, whether the port forwards 802.1X EAPOL packets with the VLAN tag depends on the port configuration and the server-assigned VLAN setting. |

|  | NOTE: An access port cannot be a tagged member of any VLAN. The device does not change the PVID of a hybrid or trunk port when the port is assigned to the VLAN as a tagged member. |
| --- | --- |

  - Command reference

    - dot1x eapol untag

Syntax

dot1x eapol untag

undo dot1x eapol untag

View

Layer 2 Ethernet interface view

Default level

3: Manage level

Description

Use dot1x eapol untag to configure a port to forward 802.1X EAPOL packets untagged.

By default, whether the port forwards 802.1X EAPOL packets with the VLAN tag depends on the port configuration and the server-assigned VLAN setting.

Examples

# Configure GigabitEthernet 1/0/1 to forward 802.1X EAPOL packets untagged.

<Sysname> system-view

[Sysname]interface gigabitethernet1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x eapol untag

New feature: Enabling source IP conflict prompt

Enabling source IP conflict prompt

When the sender IP address in a gratuitous ARP packet is the same as the IP address of the receiving switch, the switch operates as follows:

- If the source IP conflict prompt is enabled, the receiving switch immediately displays a message telling that IP address conflict occurs.

- If the source IP conflict prompt is disabled, the receiving switch sends a gratuitous ARP packet. After the switch is informed of the conflict by an ARP reply, it displays a message telling that IP address conflict occurs.

To enable source IP conflict prompt:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enable source IP conflict prompt. | arp ip-conflict prompt | Optional. By default, the function is disabled. |

Command reference

arp ip-conflict prompt

Use arp ip-conflict prompt to enable source IP conflict prompt.

Use undo arp ip-conflict prompt to restore the default.

Syntax

arp ip-conflict prompt

undo arp ip-conflict prompt

Default

The source IP conflict prompt function is disabled.

Views

System view

Default command level

2: System level

Parameters

None

Examples

# Enable source IP conflict prompt.

<Sysname> system-view

[Sysname] arp ip-conflict prompt

New feature: Delaying the MAC authentication

When both 802.1X authentication and MAC authentication are enabled on a port, you can delay the MAC authentication, so that 802.1X authentication is preferentially triggered. Configure the function as needed according to the network conditions.

Configuring the MAC authentication delay

To configure the MAC authentication delay:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| - Configure the MAC authentication delay. | mac-authentication timer auth-delay time | By default, MAC authentication is not delayed. |

Command reference

mac-authentication timer auth-delay

Syntax

mac-authentication timer auth-delay time

undo mac-authentication timer auth-delay

Views

Layer 2 Ethernet port view

Default command level

2: System level

Parameters

time: Specifies the MAC authentication delay, which ranges from 1 to 180 seconds.

Description

Use mac-authentication timer auth-delay to configure the MAC authentication delay.

Use undo mac-authentication timer auth-delay to restore the default.

By default, MAC authentication is not delayed.

Examples

# Set the MAC authentication delay to 30 seconds on port GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitEthernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication timer auth-delay 30

New feature: Disabling MAC entry aging timer refresh based on destination MAC address

Disabling MAC entry aging timer refresh based on destination MAC address

To accommodate network changes, the MAC address table keeps updating. Each dynamic MAC address entry has an aging timer. When the device receives a packet with the source or destination MAC address matching a dynamic MAC address entry, it restarts the aging timer for the entry.

If you want the device to restart the aging timer of dynamic entries for only matching source MAC addresses, disable MAC entry aging timer refresh based on destination MAC address.

To disable MAC entry aging timer refresh based on destination MAC address:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Disable MAC entry aging timer refresh based on destination MAC address. | mac-address destination-hit disable | By default, MAC entry aging timer refresh based on destination MAC address is enabled. |

Application example

Microsoft Network Load Balancing (NLB) is a load balancing technology for server clustering developed on Windows Server.

NLB cluster

NLB supports load sharing and redundancy among servers within a cluster. To implement fast failover, NLB requires that the switch forwards network traffic to all servers or specified servers in the cluster, and each server filters out unexpected traffic.

In NLB unicast mode, when a server joins the cluster or a failover occurs, a packet with a virtual source MAC address is sent. The switch then adds the virtual MAC address to its MAC address table, and packets destined for the server use the virtual MAC address (although not used by the server) as their destination address. If the virtual MAC address never ages out, the switch forwards packets only through the port associated with the virtual MAC address rather than all ports connected to the servers within the cluster.

To address this issue, disable MAC entry aging timer refresh based on destination MAC address to age out the virtual MAC address, so that the switch can forward packets to all servers within the cluster.

Command reference

mac-address destination-hit disable

Use mac-address destination-hit disable to disable MAC entry aging timer refresh based on destination MAC address.

Use undo mac-address destination-hit disable to restore the default.

Syntax

mac-address destination-hit disable

undo mac-address destination-hit disable

Default

MAC entry aging timer refresh based on destination MAC address is enabled.

View

System view

Default command level:

2: System level

Examples

# Disable MAC entry aging timer refresh based on destination MAC address.

<Sysname> system-view

[Sysname] mac-address destination-hit disable

New feature: Setting the deletion delay time for SAVI

Setting the deletion delay time for SAVI

The SAVI feature enables the access switch to check the validity of the source addresses of DHCPv6 protocol packets, ND protocol packets, and IPv6 data packets against the ND snooping entries, DHCPv6 snooping entries, and IP source guard bindings.

After a port is down, the switch can wait for a period of delay time before deleting the DHCPv6 snooping entries and ND snooping entries for that port. The deletion delay time is configurable. This delay ensures a valid IPv6 user to access the port for the event that a port goes down and resumes during that period.

        - Setting the deletion delay time for SAVI

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable SAVI. | ipv6 savi strict | By default, SAVI is disabled. |
| ##### Setting the deletion delay time for SAVI. | ipv6 savi down-delay time | The default setting is 30 seconds. |

Command reference

ipv6 savi down-delay

Use ipv6 savi down-delay to set the deletion delay time for SAVI.

Use undo ipv6 savi down-delay to restore the default.

Syntax

ipv6 savi down-delay time

undo ipv6 savi down-delay

Default

The deletion delay time is 30 seconds.

Views

System view

Default command level

2: System level

Parameters

time: Specifies the delay time in the range of 0 to 86400 seconds.

Usage guidelines

If a port is down for a period of time that exceeds the deletion delay time, the switch deletes the DHCPv6 snooping entries and ND snooping entries for that port.

Examples

# Set the deletion delay time for SAVI to 360 seconds.

<Sysname> system-view

[Sysname] ipv6 savi down-delay 360

Modified feature: Default configuration

Feature change description

The following changes are made to the default configuration in this release:

The telnet server enable command is deleted and Telnet service is disabled.

The interface vlan-interface1 command is deleted and VLAN-interface 1 does not exist.

The ip address dhcp-alloc client-identifier mac Vlan-interface1 command is deleted and VLAN-interface 1 does not apply for an IP address.

The undo ip http enable command is added and HTTP service is disabled.

The undo cwmp enable command is added and CWMP service is disabled.

Deleted the default RADIUS scheme system, which included the following commands: radius scheme system, server-type extended, primary authentication 127.0.0.1 1645, primary accounting 127.0.0.1 1646, and user-name-format without-domain.

The default configuration takes effect only when the switch starts up with no specific configuration file. Once you specify a specific startup configuration file for the switch, the switch uses the specific configuration file instead of the default configuration.

Command changes

None

# A5120EI-CMW520-F2218

This release has the following changes:

New feature: Supporting using a self-signed certificate for HTTPS

New feature: Setting the maximum number of 802.1X authentication attempts for MAC authentication users

New feature: Support of 802.1X for issuing VLAN groups

New feature: Enabling MAC address migration log notifying

Modified feature: Cluster management

Removed feature: WiNet

New feature: Supporting using a self-signed certificate for HTTPS

The switch supports simplified HTTPS login. To make the switch operate in this mode, you only need to enable HTTPS service on the switch. The switch will use a self-signed certificate (a certificate that is generated and signed by the switch itself, rather than a CA). If you specify an SSL server policy for the HTTPS service before enabling HTTPS service but do not specify the PKI domain for the SSH server, the switch still uses self-signed certificate. After you specify a PKI domain for the SSH server, the switch uses the PKI domain to obtain a certificate for the SSH server from the CA and uses the obtained certificate.

New feature: Setting the maximum number of 802.1X authentication attempts for MAC authentication users

Setting the maximum number of 802.1X authentication attempts for MAC authentication users

When both MAC authentication and 802.1X authentication are enabled on a port, if a MAC-authenticated user sends an EAP packet to the device for 802.1X authentication, the device performs 802.1X authentication for the user by default. If the user passes 802.1X authentication, the user goes online as an 802.1X user. If the user fails 802.1X authentication, the user might to try the authentication multiple times, depending on the configuration on the client. If you do not want such users to try 802.1X authentication for too many times, you can perform the following task on the device to limit the number of authentication failures.

To set the maximum number of 802.1X authentication attempts for MAC authentication users:

| Step | Command |
| --- | --- |
| - Enter system view. | system-view |
| - Enter Layer 2 Ethernet interface view. | interface interface-type interface-number |
| - Set the maximum number of 802.1X authentication attempts for MAC authentication users. | dot1x attempts max-fail unsuccessful-attempts |

Command reference

dot1x attempts max-fail

Use dot1x attempts max-fail to set the maximum number of 802.1X authentication attempts that a MAC-authenticated user can try.

Use undo dot1x attempts max-fail to restore the default.

Syntax

dot1x attempts max-fail unsuccessful-attempts

undo dot1x attempts max-fail

Default

The device allows a user that have passed MAC authentication to perform 802.1X authentication, and the maximum number of 802.1X authentication attempts that the user can try is determined by the configuration on the authentication client.

Views

Layer 2 Ethernet interface view

Default command level:

2: System level

Parameters

unsuccessful-attempts: Sets the maximum number of 802.1X authentication attempts that a MAC-authenticated user can try. The value range for this argument is 1 to 50.

Examples

# On interface GigabitEthernet 1/0/1, set the maximum number of 802.1X authentication attempts that a MAC-authenticated user can try to 3.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x attempts max-fail 3

New feature: Support of 802.1X for issuing VLAN groups

Support of 802.1X for issuing VLAN groups

After an 802.1X user passes the authentication on the server, the server delivers the authorization information to the device. If the server has specified the VLAN which is to be assigned to the user, the server contains the VLAN information in the authorization information to be delivered to device. Then, the device assigns the port through which the user performs authentication and logs in to the server-assigned VLAN.

The authentication server running the earlier releases issues a VLAN ID or VLAN name, and supports issuing only the specified VLAN. In this release or later, you can configure a VLAN group on the device, and the authentication server issues a VLAN group name. After the authentication server issues a VLAN group name, the access device selects a VLAN ID in the VLAN group and assigns the VLAN ID to a user.

The access device selects a VLAN ID from the VLAN group following these rules:

Select a VLAN with the least users.

Select the first queried VLAN if multiple VLANs have the same number of users.

For example, a VLAN group contains VLAN 2 and VLAN 3, VLAN 3 has been assigned to three users who have passed the authentication, and VLAN 2 has been assigned to two users who have passed the authentication. When a user passes the authentication, VLAN 2 is assigned to the user.

By issuing a VLAN group, you can balance the number of users in each VLAN, reduce the broadcasts in each VLAN, and improve the efficiency.

Configuring a VLAN group

You can create a VLAN group and add multiple VLAN IDs to a VLAN group.

To configure a VLAN group:

| Step | Command |
| --- | --- |
| - Enter system view. | system-view |
| - Create a VLAN group and enter VLAN group view. | vlan-group group-name |
| - Assign the specified VLANs to the VLAN group. | vlan-list vlan-list |

|  | NOTE: If a super VLAN is added to a VLAN group, the device ignores the super VLAN when selecting a server-assigned VLAN for a user passing the authentication. |
| --- | --- |

Command reference

vlan-group

Use vlan-group to create a VLAN group and enter VLAN group view.

Use undo vlan-group to delete the specified VLAN group.

Syntax

vlan-group group-name

undo vlan-group group-name

Default

No VLAN group exists.

Views

System view

Default command level

3: Manage level

Parameters

group-name: VLAN group name, which is a case-insensitive string of 1 to 31 characters and must start with a letter.

Usage guidelines

You can configure up to 100 VLAN groups.

Examples

# Create a VLAN group named test, and enter VLAN group view.

<Sysname> system-view

[Sysname] vlan-group test

vlan-list

Use vlan-list to configure member VLANs for the VLAN group.

Use undo vlan-list to delete member VLANs from the VLAN group.

Syntax

vlan-list vlan-list

undo vlan-list vlan-list

Views

VLAN group view

Default command level

3: Manage level

Parameters

vlan-list: Specifies a VLAN list in the form of vlan-list = { vlan-id1 [ to vlan-id2 ] }&<1-10>, where vlan-id1 and vlan-id2 each range from 1 to 4094 and vlan-id1 cannot be greater than vlan-id2. &<1-10> indicates that you can specify up to ten { vlan-id1 [ to vlan-id2 ] } parameters.

Usage guidelines

You can add VLANs that have not been created to a VLAN group.

You can add a VLAN to multiple VLAN groups.

Repeat this command to configure multiple member VLANs for a VLAN group.

If a super VLAN is added to a VLAN group, the device ignores the super VLAN when selecting a server-assigned VLAN for a user passing the authentication.

Examples

# Add VLANs 6, 7, and 8 to the VLAN group named test.

<Sysname> system-view

[Sysname] vlan-group test

[Sysname-vlan-group-test] vlan-list 6 7 8

New feature: Enabling MAC address migration log notifying

Enabling MAC address migration log notifying

This feature records and notifies MAC address migration information, including MAC addresses that migrate, IDs of VLANs to which MAC addresses belong, source interfaces from which MAC addresses migrate, and current interfaces with which MAC addresses associate, last migration time, and migration times in the last one minute.

MAC address migration refers to this process: a device learns a MAC address from an interface, Port A for example, and the device later learns the MAC address from another interface, Port B for example. If Port A and Port B belong to the same VLAN, the outgoing interface in the entry for the MAC address is changed to Port B from Port A, which means that the MAC address migrates from Port A to Port B.

To enable MAC address migration log notifying:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Enable MAC address migration log notifying. | mac-flapping notification enable | By default, MAC address migration log notifying is disabled. |

The MAC address migration logs of the last one minute are displayed once every one minute.

Command reference

mac-flapping notification enable

Use mac-flapping notification enable to enable MAC address migration log notifying.

Use undo mac-flapping notification enable to disable the MAC address migration notifying.

Syntax

mac-flapping notification enable

undo mac-flapping notification enable

Default

MAC address migration log notifying is disabled.

Views

System view

Default command level:

2: System level

Usage guidelines

A MAC address migration log contains a MAC address, ID of the VLAN to which the MAC address belongs, source interface from which the MAC address migrates, and the current interface with which the MAC address associates.

After enabling MAC address migration log notifying, the MAC address migration log of the last 1 minute are displayed once every 1 minute.

Up to 10 logs can be saved on each card in 1 minute.

Examples

# Enable MAC address migration log notifying.

<Sysname> system-view

[Sysname] mac-flapping notification enable

[Sysname]

%Sep 21 14:09:22:420 2012 HP MAC/5/MAC_FLAPPING: MAC address 0000-0012-0034 in vlan 500 has flapped from port GigabitEthernet1/0/16 to port GigabitEthernet1/0/1 1 time(s).

The output shows that the MAC address 0000-0012-0034 belongs to VLAN 500, the source interface from which the MAC address migrates from is GE1/0/16, the current interface with which the MAC address associates is GE1/0/1, and the MAC address migrates one time in the last one minute.

Modified feature: Cluster management

  - Feature change description

Changed the default state of the Cluster function, NDP, and NTDP from enabled to disabled.

  - Command changes

    - Modified command: cluster enable

Syntax

cluster enable

undo cluster enable

Views

System view

Change description

Before modification: By default, the cluster function is enabled.

After modification: By default, the cluster function is disabled.

    - Modified command: ndp enable

Syntax

In Layer 2 Ethernet port view or Layer 2 aggregate interface view:

ndp enable

undo ndp enable

In system view:

ndp enable [ interface interface-list ]

undo ndp enable [ interface interface-list ]

Views

System view, Layer 2 Ethernet port view, Layer 2 aggregate interface view

Change description

Before modification: By default, NDP is enabled globally and also on all ports.

After modification: By default, NDP is disabled globally and also on all ports.

    - Modified command: ntdp enable

Syntax

ntdp enable

undo ntdp enable

Views

System view, Layer 2 Ethernet port view, Layer 2 aggregate interface view

Change description

Before modification: By default, NTDP is enabled globally and also on all ports.

After modification: By default, NTDP is disabled globally and also on all ports.

Removed feature: WiNet

Feature change description

Removed the WiNet feature.

Removed commands

None

# A5120EI-CMW520-F2217

This release has the following changes:

New feature: Automatic configuration file backup for software downgrading

New feature: FIPS

New feature: IPsec

New feature: IKE

New feature: Verifying the correctness and integrity of the file

Modified feature: Configuring a password for the local user

Modified feature: Clearing all users from the password control blacklist

Modified feature: 802.1X critical VLAN

Modified feature: MAC authentication critical VLAN

Modified feature: Modifying CLI configuration commands executed in FIPS mode for CC evaluation

Modified feature: Modifying login management commands executed in FIPS mode for CC evaluation

Modified Feature: Modifying software upgrade commands executed in FIPS mode for CC evaluation

Modified Feature: Modifying configuration file management commands executed in FIPS mode for CC evaluation

Modified Feature: Modifying security commands executed in FIPS mode for CC evaluation

Modified feature: Modifying SNMP commands executed in FIPS mode for CC evaluation

New feature: Automatic configuration file backup for software downgrading

Configuring automatic configuration file backup for software downgrading

After a software upgrade, the first time you use the save [ safely ] [ backup | main ] [ force ] command to save configuration to a configuration file that was created before the upgrade, the system verifies the compatibility of the configuration file with the software version.

If any incompatibility is found, the system uses the running configuration to overwrite the configuration file after backing up the file to the Flash memory on each member device for future rollback. The backup file is named in the old-filename_bak.cfg format. For example, if the old configuration file is named config.cfg, the backup file is named config_bak.cfg.

If the backup attempt fails on an IRF member device, choose one of the following failure handling actions at prompt:

- Give up saving the configuration—In this approach, the system does not save the configuration on any member device.

- Overwrite the configuration file—In this approach, the system uses the running configuration to overwrite the configuration file on the member device without backing up the file. You can copy the backup configuration file from the master device to this member device for future rollback.

To load the backup configuration file after a software downgrade, specify the file as the next-startup configuration file before performing the downgrade.

Command reference

None.

New feature: FIPS

Overview

Federal Information Processing Standards (FIPS), developed by the National Institute of Standard and Technology (NIST) of the United States, specify the requirements for cryptography modules. FIPS 140-2 defines four levels of security, simply named "Level 1" to "Level 4" from low to high. Currently, the switch supports Level 2.

Unless otherwise noted, FIPS in the document refers to FIPS 140-2.

FIPS self-tests

When the device operates in FIPS mode, it has self-test mechanisms, including the power-up self-test and conditional self-tests, to ensure the normal operation of cryptography modules. You can also trigger a self-test. If a self-test fails, the device restarts.

|  | CAUTION: If the switch reboots repeatedly, it might be caused by software failures or hardware damages. Contact technical support engineers to upgrade the software or repair the damaged hardware. |
| --- | --- |

    - Power-up self-test

The power-up self-test, also called "known-answer test", examines the availability of FIPS-allowed cryptographic algorithms. A cryptographic algorithm is run on data for which the correct output is already known. The calculated output is compared with the known answer. If they are not identical, the known-answer test fails.

    - Conditional self-tests

A conditional self-test runs when an asymmetrical cryptographic module or a random number generator module is invoked. Conditional self-tests include the following types:

Pair-wise consistency test—This test is run when a DSA/RSA asymmetrical key-pair is generated. It uses the public key to encrypt a plain text, and uses the private key to decrypt the encrypted text. If the decryption is successful, the test succeeds. Otherwise, the test fails.

Continuous random number generator test—This test is run when a random number is generated in FIPS mode. If two consecutive random numbers are different, the test succeeds. Otherwise, the test fails.

    - Triggering a self-test

To examine whether the cryptography modules operate normally, you can use a command to trigger a self-test on the cryptographic algorithms. The triggered self-test is the same as the power-up self-test.

If the self-test fails, the device automatically reboots.

Configuring FIPS

To configure FIPS, complete the following tasks:

Remove the existing key pairs and certificates.

Enable the FIPS mode.

Enable the password control function.

Configure local user attributes (including local username, service type, password, and so on) on the switch.

Save the configuration.

After you finish the above configurations, reboot the switch. The switch works in FIPS mode that complies with the FIPS 140-2 standard after it starts up. For Common Criteria (CC) evaluation in FIPS mode, the switch also works in a operating mode that complies with the CC standard.

The switch does not support an upgrade from a FIPS-incompatible version to a FIPS-compatible version.

Enabling the FIPS mode

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable the FIPS mode. | fips mode enable | Disabled by default. |

After you enable the FIPS mode and reboot the switch, the switch works in FIPS mode after it starts up and the following changes occur.

FTP/TFTP is disabled.

Telnet is disabled.

The HTTP server is disabled.

SNMPv1 and SNMPv2c are disabled. Only SNMPv3 is available.

The SSL server only supports TLS1.0.

The SSH server does not support SSHv1 clients

SSH only supports RSA.

The generated RSA key pairs must have a modulus length of 2048 bits. The generated DSA key pair must have a modulus of at least 1024 bits.

SSH, SNMPv3, IPsec and SSL do not support DES, 3DES, RC4, or MD5.

Triggering a self-test

To examine whether the cryptography modules operate normally, you can use a command to trigger a self-test on the cryptographic algorithms. The triggered self-test is the same as the power-up self-test.

If the self-test fails, the device automatically reboots.

To trigger a self-test:

| Task | Command |
| --- | --- |
| - Enter system view. | system-view |
| ##### Trigger a self-test. | fips self-test |

Displaying and maintaining FIPS

| Task | Command | Remarks |
| --- | --- | --- |
| Display FIPS mode state | display fips status | Available in any view. |

FIPS configuration example

Network requirements

PC connects to Switch through a console port. Configure Switch to operate in FIPS mode and create a local user for PC so that PC can log in to the switch.

Network diagram

Configuration procedure

Configure Switch:

# Enable the FIPS mode.

<Sysname> system-view

[Sysname] fips mode enable

# Enable the password control function.

[Sysname] password-control enable

# Create a local user named test, and set its service type as terminal, privilege level as 3, and password as AAbbcc1234%. The password is a string of at least 10 characters by default and must contain both uppercase and lowercase letters, digits, and special characters.

[Sysname] local-user test

[Sysname-luser-test] service-type terminal

[Sysname-luser-test] authorization-attribute level 3

[Sysname-luser-test] password

Password:***********

Confirm :***********

Updating user(s) information, please wait...........

[Sysname-luser-test] quit

# Save the configuration.

[Sysname] save

The current configuration will be written to the device. Are you sure? [Y/N]:y

Please input the file name(*.cfg)[flash:/startup.cfg]

(To leave the existing filename unchanged, press the enter key):

flash:/startup.cfg exists, overwrite? [Y/N]:y

Validating file. Please wait..........................

Saved the current configuration to mainboard device successfully.

Configuration is saved to device successfully.

[Sysname] quit

# Reboot the switch.

<Sysname> reboot

|  | CAUTION: After you enable the FIPS mode, be sure to create a local user and its password before you reboot the switch. Otherwise, you cannot log in to the switch. If you cannot log in to the switch, reboot the switch without the configuration file (by ignoring or removing the configuration file) so that the switch works in non-FIPS mode, and then make correct configurations. |
| --- | --- |

Verify the configuration:

After the switch reboots, enter the username (test) and password (AAbbcc1234%). The system prompts that your first login is successful, and asks you to enter a new password. Enter a new password which has at least four characters different than the previous one and confirm the password. Then, the system displays the <Sysname> prompt.

User interface aux0 is available.

Please press ENTER.

Login authentication

Username:test

Password:

Info: First logged in. For security reasons you will need to change your password.

Please enter your new password.

Password:**********

Confirm :**********

Updating user(s) information, please wait...........

<Sysname>

# Display the current FIPS mode. You can see that the FIPS mode is enabled.

<Sysname> display fips status

FIPS mode is enabled

Command reference

fips mode enable

Use fips mode enable to enable the FIPS mode.

Use undo fips mode enable to disable the FIPS mode.

Syntax

fips mode enable

undo fips mode enable

Default

The FIPS mode is disabled.

Views

System view

Default command level

2: System level

Parameters

None

Usage guidelines

After you enable the FIPS mode, reboot the switch to make your configuration effective. After the switch starts up, the switch works in FIPS mode. The FIPS mode complies with the FIPS 140-2 standard.

Examples

# Enable the FIPS mode.

<Sysname> system-view

[Sysname] fips mode enable

Related commands

display fips status

display fips status

Use display fips status to display the current FIPS mode.

Syntax

display fips status

Views

Any view

Default command level

1: Monitor level

Examples

# Display the current FIPS mode.

<Sysname> display fips status

FIPS mode is enabled

Related commands

fips mode enable

fips self-test

Use fips self-test to trigger a self-test on the password algorithms.

Syntax

fips self-test

Views

System view

Default command level

3: Manage level

Usage guidelines

To examine whether the cryptography modules operate normally, you can use a command to trigger a self-test on the cryptographic algorithms. The triggered self-test is the same as the power-up self-test.

If the self-test fails, the device automatically reboots.

Examples

# Trigger a self-test on the cryptographic algorithms.

<Sysname> system-view

[Sysname] fips self-test

Self-tests are running. Please wait...

Self-tests succeeded.

New feature: IPsec

|  | NOTE: - The term router in this document refers to both routers and switches. - IPsec configuration is available for only the switches in FIPS mode. For information about the FIPS mode, see New feature: FIPS. - A switch in IRF mode does not support IPsec automatic negotiation. |
| --- | --- |

  - IPsec overview

IP Security (IPsec) is a security framework defined by the Internet Engineering Task Force (IETF) for securing IP communications.

IPsec provides the following security services at the IP layer for two communication parties:

- Confidentiality—The sender encrypts packets before transmitting them over the Internet, protecting the packets from being eavesdropped en route.

- Data integrity—The receiver verifies the packets received from the sender to make sure they are not tampered with during transmission.

- Data origin authentication—The receiver verifies the authenticity of the sender.

- Anti-replay—The receiver examines packets and drops outdated and duplicate packets.

IPsec delivers these benefits:

- Reduced key negotiation overheads and simplified maintenance by supporting the Internet Key Exchange (IKE) protocol. IKE provides automatic key negotiation and automatic IPsec security association (SA) setup and maintenance.

- Good compatibility. You can apply IPsec to all IP-based application systems and services without modifying them.

- Encryption on a per-packet rather than per-flow basis. Per-packet encryption allows for flexibility and greatly enhances IP security.

IPsec comprises a set of protocols, including Authentication Header (AH), Encapsulating Security Payload (ESP), Internet Key Exchange (IKE), and algorithms for authentication and encryption. AH and ESP provides security services and IKE performs automatic key exchange.

IPsec can protect both IPv4 and IPv6 packets.

    - Basic concepts

      - Security protocols

IPsec comes with two security protocols:

- AH (protocol 51)—Provides data origin authentication, data integrity, and anti-replay services by adding an AH header to each IP packet. AH is suitable only for transmitting non-critical data because it cannot prevent eavesdropping, although it can prevent data tampering. AH supports authentication algorithms such as Message Digest (MD5) and Secure Hash Algorithm (SHA-1).

- ESP (protocol 50)—Provides data encryption as well as data origin authentication, data integrity, and anti-replay services by inserting an ESP header and an ESP trailer in IP packets. Unlike AH, ESP encrypts data before encapsulating the data to guarantee data confidentiality. ESP supports encryption algorithms such as Data Encryption Standard (DES), 3DES, and Advanced Encryption Standard (AES), and authentication algorithms such as MD5 and SHA-1. The authentication function is optional to ESP.

Both AH and ESP provide authentication services, but the authentication service provided by AH is stronger. In practice, you can choose either or both security protocols. When both AH and ESP are used, an IP packet is encapsulated first by ESP and then by AH. Figure 2 shows the format of IPsec packets.

      - Security association

A security association (SA) is an agreement negotiated between two communicating parties called IPsec peers. It comprises a set of parameters for data protection, including security protocols, encapsulation mode, authentication and encryption algorithms, and shared keys and their lifetime. SAs can be set up manually or through IKE.

An SA is unidirectional. At least two SAs are needed to protect data flows in a bidirectional communication. If two peers want to use both AH and ESP to protect data flows between them, they construct an independent SA for each protocol.

An SA is uniquely identified by a triplet, which consists of the security parameter index (SPI), destination IP address, and security protocol identifier (AH or ESP).

An SPI is a 32-bit number for uniquely identifying an SA. It is transmitted in the AH/ESP header. A manually configured SA requires an SPI to be specified manually for it; an IKE created SA will have an SPI generated at random.

A manually configured SA never ages out. An IKE created SA has a specified period of lifetime, which comes in two types:

- Time-based lifetime, which defines how long the SA can be valid after it is created.

- Traffic-based lifetime, which defines the maximum traffic that the SA can process.

The SA becomes invalid when either of the lifetime timers expires. Before the SA expires, IKE negotiates a new SA, which takes over immediately after its creation.

      - Encapsulation modes

IPsec supports the following IP packet encapsulation modes:

- Tunnel mode—IPsec protects the entire IP packet, including both the IP header and the payload. It uses the entire IP packet to calculate an AH or ESP header, and then encapsulates the original IP packet and the AH or ESP header with a new IP header. If you use ESP, an ESP trailer is also encapsulated. Tunnel mode is typically used for protecting gateway-to-gateway communications.

- Transport mode—IPsec protects only the IP payload. It uses only the IP payload to calculate the AH or ESP header, and inserts the calculated header between the original IP header and payload. If you use ESP, an ESP trailer is also encapsulated. The transport mode is typically used for protecting host-to-host or host-to-gateway communications.

Figure 2 shows how the security protocols encapsulate an IP packet in different encapsulation modes. Data represents the transport layer data.

Encapsulation by security protocols in different modes

      - Authentication algorithms and encryption algorithms

            - Authentication algorithms

IPsec uses hash algorithms to perform authentication. A hash algorithm produces a fixed-length digest for an arbitrary-length message. IPsec peers respectively calculate message digests for each packet. If the resulting digests are identical, the packet is considered intact.

IPsec supports the following hash algorithms for authentication:

- MD5, which takes a message of arbitrary length as input and produces a 128-bit message digest.

- SHA-1, which takes a message of a maximum length less than the 64th power of 2 in bits as input and produces a 160-bit message digest.

Compared with SHA-1, MD5 is faster but less secure.

            - Encryption algorithms

IPsec mainly uses symmetric encryption algorithms, which encrypt and decrypt data by using the same keys. The following encryption algorithms are available for IPsec on the device:

- Data Encryption Standard (DES), which encrypts a 64-bit plain text block with a 56-bit key. DES is the least secure but the fastest algorithm. It is sufficient for general security requirements.

- Triple DES (3DES), which encrypts plain text data with three 56-bit DES keys. The key length totals up to 168 bits. It provides moderate security strength and is slower than DES.

- Advanced Encryption Standard (AES), which encrypts plain text data with a 128-bit, 192-bit, or 256-bit key. AES provides the highest security strength and is slower than 3DES.

      - IPsec SA setup modes

There are two IPsec SA setup modes:

- Manual mode. In this mode, you manually configure and maintain all SA settings. Advanced features like periodical key update are not available. However, this mode implements IPsec independently of IKE.

- ISAKMP mode. In this mode, IKE automatically negotiates and maintains IPsec SAs for IPsec.

If the number of IPsec tunnels in your network is small, use the manual mode. If the number of IPsec tunnels is large, use the ISAKMP mode.

      - IPsec tunnel

An IPsec tunnel is a bidirectional channel created between two peers. An IPsec tunnel comprises one or more pairs of SAs.

    - Protocols and standards

Protocols and standards relevant to IPsec are as follows:

- RFC 2401, Security Architecture for the Internet Protocol

- RFC 2402, IP Authentication Header

- RFC 2406, IP Encapsulating Security Payload

- RFC 4552, Authentication/Confidentiality for OSPFv3

  - Configuring IPsec

IPsec can be implemented based on only ACLs. ACL-based IPsec uses ACLs to identify the data flows to be protected. To implement ACL-based IPsec, configure IPsec policies, reference ACLs in the policies, and apply the policies to physical interfaces. By using ACLs, you can customize IPsec policies as needed, implementing IPsec flexibly.

  - Implementing ACL-based IPsec

    - Feature restrictions and guidelines

ACL-based IPsec is designed to protect only traffic that is generated by the device and traffic that is destined for the device. Providing IPsec protection for user traffic will severely decrease performance. To avoid this issue, HP recommends that you not include any rules in the security ACL to match traffic forwarded through the device. For example, you can configure an ACL-based IPsec tunnel to protect log messages the device sends to a log server. However, do not use an IPsec tunnel to protect traffic that is forwarded by the device for two hosts. For more information about configuring an ACL for IPsec, see "Configuring ACLs."

Typically, IKE uses UDP port 500 for communication, and AH and ESP use the protocol numbers 51 and 50, respectively. Make sure flows of these protocols are not denied on the interfaces with IKE or IPsec configured.

ACL-based IPsec configuration task list

The following is the generic configuration procedure for implementing ACL-based IPsec:

Configure ACLs for identifying data flows to be protected.

Configure IPsec proposals to specify the security protocols, authentication and encryption algorithms, and encapsulation mode.

Configure IPsec policies to associate data flows with IPsec proposals and specify the SA negotiation mode, the peer IP addresses (the start and end points of the IPsec tunnel), the required keys, and the SA lifetime.

Apply the IPsec policies to interfaces to finish IPsec configuration.

To configure ACL-based IPsec:

| Task | Remarks |
| --- | --- |
| Configuring ACLs | Required. Basic IPsec configuration. |
| Configuring an IPsec proposal |  |
| Configuring an IPsec policy |  |
| Applying an IPsec policy group to an interface |  |
| Configuring the IPsec session idle timeout | Optional. |
| Enabling ACL checking of de-encapsulated IPsec packets | Optional. |
| Configuring the IPsec anti-replay function | Optional. |
| Configuring packet information pre-extraction | Optional. |

Configuring ACLs

ACLs can be used to identify traffic. They are widely used in scenarios where traffic identification is desired, such as QoS and IPsec.

Keywords in ACL rules

IPsec uses ACLs to identify data flows. An ACL is a collection of ACL rules. Each ACL rule is a deny or permit statement. A permit statement identifies a data flow protected by IPsec, and a deny statement identifies a data flow that is not protected by IPsec. With IPsec, a packet is matched against the referenced ACL rules and processed according to the first rule that it matches:

- Each ACL rule matches both the outbound traffic and the returned inbound traffic. Suppose there is a rule rule 0 permit ip source 1.1.1.0 0.0.0.255 destination 2.2.2.0 0.0.0.255. This rule matches both traffic from 1.1.1.0 to 2.2.2.0 and traffic from 2.2.2.0 to 1.1.1.0.

- In the outbound direction, if a permit statement is matched, IPsec considers that the packet requires protection and continues to process it. If a deny statement is matched or no match is found, IPsec considers that the packet does not require protection and delivers it to the next function module.

- In the inbound direction:

  - Normal IP packets that match a permit statement are dropped.

  - IPsec packets that match a permit statement and are destined for the device itself are de-encapsulated and matched against the rule again. Only those that match a permit statement are processed by IPsec.

When you configure an ACL for IPsec, follow these guidelines:

- Permit only data flows that need to be protected and use the any keyword with caution. With the any keyword specified in a permit statement, all outbound traffic matching the permit statement will be protected by IPsec and all inbound IPsec packets matching the permit statement will be received and processed, but all inbound non-IPsec packets will be dropped. This will cause the inbound traffic that does not need IPsec protection to be all dropped.

- Avoid statement conflicts in the scope of IPsec policy groups. When creating a deny statement, be careful with its matching scope and matching order relative to permit statements. The policies in an IPsec policy group have different match priorities. ACL rule conflicts between them are prone to cause mistreatment of packets. For example, when configuring a permit statement for an IPsec policy to protect an outbound traffic flow, you must avoid the situation that the traffic flow matches a deny statement in a higher priority IPsec policy. Otherwise, the packets will be sent out as normal packets; if they match a permit statement at the receiving end, they will be dropped by IPsec.

- An ACL can be specified for only one IPsec policy. ACLs referenced by IPsec policies cannot be used by other services.

- You must create a mirror image ACL rule at the remote end for each ACL rule created at the local end. Otherwise, IPsec may protect traffic in only one direction.

Mirror image ACLs

To make sure SAs can be set up and the traffic protected by IPsec can be processed correctly at the remote peer, on the remote peer, create a mirror image ACL rule for each ACL rule created at the local peer.

If the ACL rules on peers do not form mirror images of each other, SAs can be set up only when both of the following requirements are met:

- The range specified by an ACL rule on one peer is covered by its counterpart ACL rule on the other peer.

- The peer with the narrower rule initiates SA negotiation. If a wider ACL rule is used by the SA initiator, the negotiation request may be rejected because the matching traffic is beyond the scope of the responder.

Protection modes

The switch supports IPsec for data flows in standard mode. In standard mode, one tunnel protects one data flow. The data flow permitted by an ACL rule is protected by one tunnel that is established solely for it.

For more information about ACL configuration, see ACL and QoS Configuration Guide.

|  | NOTE: To use IPsec in combination with QoS, make sure IPsec's ACL classification rules match the QoS classification rules. If the rules do not match, QoS may classify the packets of one IPsec SA to different queues, causing packets to be sent out of order. When the anti-replay function is enabled, IPsec will discard the packets beyond the anti-replay window in the inbound direction, resulting in packet loss. For more information about QoS classification rules, see ACL and QoS Configuration Guide. |
| --- | --- |

Configuring an IPsec proposal

An IPsec proposal, part of an IPsec policy or an IPsec profile, defines the security parameters for IPsec SA negotiation, including the security protocol, the encryption and authentication algorithms, and the encapsulation mode.

To configure an IPsec proposal:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view | system-view | N/A |
| ##### Create an IPsec proposal and enter its view | ipsec proposal proposal-name | By default, no IPsec proposal exists. |
| ##### Specify the security protocol for the proposal | transform { ah ¦ ah-esp ¦ esp } | Optional. ESP by default. |
| ##### Specify the security algorithms | - Specify the encryption algorithm for ESP: - esp encryption-algorithm aes [ key-length ] - Specify the authentication algorithm for ESP: - esp authentication-algorithm sha1 - Specify the authentication algorithm for AH: - ah authentication-algorithm sha1 | Optional. For ESP, the default encryption algorithm is AES-128. For ESP and AH, the default authentication algorithm is SHA1. |
| ##### Specify the IP packet encapsulation mode for the IPsec proposal | encapsulation-mode { transport ¦ tunnel } | Optional. Tunnel mode by default. Transport mode applies only when the source and destination IP addresses of data flows match those of the IPsec tunnel. |

|  | NOTE: - Changes to an IPsec proposal affect only SAs negotiated after the changes. To apply the changes to existing SAs, execute the reset ipsec sa command to clear the SAs so that they can be set up using the updated parameters. - Only when a security protocol is selected, can you configure security algorithms for it. For example, you can specify the ESP-specific security algorithms only when you select ESP as the security protocol. ESP supports both encryption and authentication for IP packets. In FIPS mode, you must use both ESP encryption and authentication. |
| --- | --- |

Configuring an IPsec policy

IPsec policies define which IPsec proposals should be used to protect which data flows. An IPsec policy is uniquely identified by its name and sequence number.

IPsec policies fall into two categories:

- Manual IPsec policy—The parameters are configured manually, such as the keys, the SPIs, and the IP addresses of the two ends in tunnel mode.

- IPsec policy that uses IKE—The parameters are automatically negotiated through IKE.

Configuring a manual IPsec policy

To guarantee successful SA negotiations, follow these guidelines when configuring manual IPsec policies at the two ends of an IPsec tunnel:

- The IPsec policies at the two ends must have IPsec proposals that use the same security protocols, security algorithms, and encapsulation mode.

- The remote IP address configured on the local end must be the same as the IP address of the remote end.

- At each end, configure parameters for both the inbound SA and the outbound SA and make sure that different SAs use different SPIs.

- The local inbound SA must use the same SPI and keys as the remote outbound SA. The same is true of the local outbound SA and remote inbound SA.

- The keys for the local and remote inbound and outbound SAs must be in the same format. For example, if the local inbound SA uses a key in characters, the local outbound SA and remote inbound and outbound SAs must use keys in characters.

When you configure a manual IPsec policy, follow these guidelines:

- An IPsec policy can reference only one ACL. If you apply multiple ACLs to an IPsec policy, only the last one takes effect.

- A manual IPsec policy can reference only one IPsec proposal. To change an IPsec proposal for an IPsec policy, you must remove the proposal reference first.

- At each end, configure parameters for both the inbound and the outbound SAs, and make sure different SAs use different SPIs.

- If you configure a key in two modes: string and hexadecimal, the last configured one is used.

- You cannot change the creation mode of an IPsec policy from manual to through IKE, or vice versa. To create an IPsec policy that uses IKE, delete the manual IPsec policy, and then use IKE to configure an IPsec policy.

To configure a manual IPsec policy:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Create a manual IPsec policy and enter its view. | ipsec policy policy-name seq-number manual | By default, no IPsec policy exists. |
| ##### Assign an ACL to the IPsec policy. | security acl acl-number | By default, an IPsec policy references no ACL. |
| ##### Assign an IPsec proposal to the IPsec policy. | proposal proposal-name | By default, an IPsec policy references no IPsec proposal. |
| ##### Configure the two ends of the IPsec tunnel. | - Configure the local address of the tunnel: tunnel local ip-address - Configure the remote address of the tunnel: tunnel remote ip-address | Configuring the remote address of the tunnel is required. Both the local and remote addresses are not configured by default. |
| ##### Configure the SPIs for the SAs. | sa spi { inbound ¦ outbound } { ah ¦ esp } spi-number | By default, SPIs for the SAs do not exist. |
| ##### Configure keys for the SAs. | - Configure an authentication key in hexadecimal: sa authentication-hex { inbound ¦ outbound } { ah ¦ esp } [ cipher ¦ simple ] hex-key - Configure an encryption key in hexadecimal for ESP: sa encryption-hex { inbound ¦ outbound } esp [ cipher ¦ simple ] hex-key | Use either command. For ESP, if you configure an authentication key, the system automatically generates an authentication key and an encryption key. |

Configuring an IPsec policy that uses IKE

To configure an IPsec policy that uses IKE, directly configure it by configuring the parameters in IPsec policy view.

Before you configure an IPsec policy that uses IKE, configure the ACLs and the IKE peer for the IPsec policy. For more information about IKE configuration, see the chapter "IKE configuration."

The parameters for the local and remote ends must match.

When you configure an IPsec policy that uses IKE, follow these guidelines:

- An IPsec policy can reference only one ACL. If you apply multiple ACLs to an IPsec policy, only the last one takes effect.

- With SAs to be established through IKE negotiation, an IPsec policy can reference up to six IPsec proposals. During negotiation, IKE searches for a fully matched IPsec proposal at the two ends of the expected IPsec tunnel. If no match is found, no SA can be set up and the packets expecting to be protected will be dropped.

- During IKE negotiation for an IPsec policy with PFS enabled, an additional key exchange is performed. If the local end uses PFS, the remote end must also use PFS for negotiation and both ends must use the same Diffie-Hellman (DH) group; otherwise, the negotiation will fail.

- An SA uses the global lifetime settings when it is not configured with lifetime settings in IPsec policy view. When negotiating to set up SAs, IKE uses the local lifetime settings or those proposed by the peer, whichever are smaller.

- You cannot change the creation mode of an IPsec policy directly. To create an IPsec policy in another creation mode, delete the current one and then configure a new IPsec policy.

To directly configure an IPsec policy that uses IKE:

| Step | Command | Remark |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Create an IPsec policy that uses IKE and enter its view. | ipsec policy policy-name seq-number isakmp | By default, no IPsec policy exists. |
| ##### Configure an IPsec connection name. | connection-name name | Optional. By default, no IPsec connection name is configured. |
| ##### Assign an ACL to the IPsec policy. | security acl acl-number | By default, an IPsec policy references no ACL. |
| ##### Assign IPsec proposals to the IPsec policy. | proposal proposal-name&<1-6> | By default, an IPsec policy references no IPsec proposal. |
| ##### Specify an IKE peer for the IPsec policy. | ike-peer peer-name | An IPsec policy cannot reference any IKE peer that is already referenced by an IPsec profile, and vice versa. |
| ##### Enable and configure the perfect forward secrecy feature for the IPsec policy. | pfs { dh-group2 ¦ dh-group5 ¦ dh-group14 } | Optional. By default, the PFS feature is not used for negotiation. For more information about PFS, see the chapter "IKE configuration." |
| ##### Set the SA lifetime. | sa duration { time-based seconds ¦ traffic-based kilobytes } | Optional. By default, the global SA lifetime is used. |
| ##### Enable the IPsec policy. | policy enable | Optional. Enabled by default. |
| ##### Return to system view. | quit | N/A |
| ##### Set the global SA lifetime. | ipsec sa global-duration { time-based seconds ¦ traffic-based kilobytes } | Optional. 3600 seconds for time-based SA lifetime by default. 1843200 kilobytes for traffic-based SA lifetime by default. |

Applying an IPsec policy group to an interface

An IPsec policy group is a collection of IPsec policies with the same name but different sequence numbers. In an IPsec policy group, an IPsec policy with a smaller sequence number has a higher priority.

You can apply an IPsec policy group to a logical or physical interface to protect certain data flows. To cancel the IPsec protection, remove the application of the IPsec policy group.

For each packet to be sent out an IPsec protected interface, the system looks through the IPsec policies in the IPsec policy group in ascending order of sequence numbers. If an IPsec policy matches the packet, the system uses the IPsec policy to protect the packet. If no match is found, the system sends the packet out without IPsec protection.

To apply an IPsec policy group to an interface:

| Step | Command |
| --- | --- |
| - Enter system view. | system-view |
| ##### Enter interface view. | interface interface-type interface-number |
| ##### Apply an IPsec policy group to the interface. | ipsec policy policy-name |

|  | NOTE: - IPsec policies can be applied only to VLAN interfaces on the switch. - An interface can reference only one IPsec policy group. An IPsec policy can be applied to only one interface. |
| --- | --- |

Configuring the IPsec session idle timeout

An IPsec session is created when the first packet matching an IPsec policy arrives. Also created is an IPsec session entry, which records the quintuplet (source IP address, destination IP address, protocol number, source port, and destination port) and the matched IPsec tunnel.

An IPsec session is automatically deleted after the idle timeout expires.

Subsequent data flows search the session entries according to the quintuplet to find a matched item. If found, the data flows are processed according to the tunnel information; otherwise, they are processed according to the original IPsec process: search the policy group or policy at the interface, and then the matched tunnel.

The session processing mechanism of IPsec saves intermediate matching procedures, improving the IPsec forwarding efficiency.

To set the IPsec session idle timeout:

| Step | Command | Remark |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the IPsec session idle timeout. | ipsec session idle-time seconds | Optional. 300 seconds by default. |

Enabling ACL checking of de-encapsulated IPsec packets

In tunnel mode, the IP packet that was encapsulated in an inbound IPsec packet may not be an object that is specified by an ACL to be protected. For example, a forged packet is not an object to be protected. If you enable ACL checking of de-encapsulated IPsec packets, all packets failing the checking will be discarded, improving the network security.

To enable ACL checking of de-encapsulated IPsec packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable ACL checking of de-encapsulated IPsec packets. | ipsec decrypt check | Optional. Enabled by default. |

Configuring the IPsec anti-replay function

The IPsec anti-replay function protects networks against anti-replay attacks by using a sliding window mechanism called anti-replay window. This function checks the sequence number of each received IPsec packet against the current IPsec packet sequence number range of the sliding window. If the sequence number is not in the current sequence number range, the packet is considered a replayed packet and is discarded.

IPsec packet de-encapsulation involves complicated calculation. De-encapsulation of replayed packets not only makes no sense, but also consumes large amounts of resources and degrades performance, resulting in DoS. IPsec anti-replay checking, when enabled, is performed before the de-encapsulation process, reducing resource waste.

In some cases, however, the sequence numbers of some normal service data packets may be out of the current sequence number range, and the IPsec anti-replay function may drop them as well, affecting the normal communications. If this happens, disable IPsec anti-replay checking or adjust the size of the anti-replay window as required.

To configure IPsec anti-replay checking:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable IPsec anti-replay checking. | ipsec anti-replay check | Optional. Enabled by default. |
| ##### Set the size of the IPsec anti-replay window. | ipsec anti-replay window width | Optional. 32 by default. |

|  | CAUTION: - IPsec anti-replay checking is enabled by default. Do not disable it unless it needs to be disabled. - A wider anti-replay window results in higher resource cost and more system performance degradation, which is against the original intention of the IPsec anti-replay function. Specify an anti-replay window size that is as small as possible. |
| --- | --- |

|  | NOTE: IPsec anti-replay checking does not affect manually created IPsec SAs. According to the IPsec protocol, only IPsec SAs negotiated by IKE support anti-replay checking. |
| --- | --- |

Configuring packet information pre-extraction

If you apply both an IPsec policy and QoS policy to an interface, by default, the interface first uses IPsec and then QoS to process IP packets, and QoS classifies packets by the headers of IPsec-encapsulated packets. If you want QoS to classify packets by the headers of the original IP packets, enable the packet information pre-extraction feature.

For more information about QoS policy and classification, see ACL and QoS Configuration Guide.

To configure packet information pre-extraction:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter IPsec policy view. | ipsec policy policy-name seq-number [ isakmp ¦ manual ] | Configure either command. |
| ##### Enable packet information pre-extraction. | qos pre-classify | Disabled by default. |

Displaying and maintaining IPsec

| To do… | Use the command… | Remarks |
| --- | --- | --- |
| Display IPsec policy information | display ipsec policy [ brief ¦ name policy-name [ seq-number ] ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display IPsec proposal information | display ipsec proposal [ proposal-name ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display IPsec SA information | display ipsec sa [ brief ¦ policy policy-name [ seq-number ] ¦ remote ip-address ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display IPsec session information | display ipsec session [ tunnel-id integer ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display IPsec packet statistics | display ipsec statistics [ tunnel-id integer ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display IPsec tunnel information | display ipsec tunnel [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Clear SAs | reset ipsec sa [ parameters dest-address protocol spi ¦ policy policy-name [ seq-number ] ¦ remote ip-address ] | Available in user view |
| Clear IPsec sessions | reset ipsec session [ tunnel-id integer ] | Available in user view |
| Clear IPsec statistics | reset ipsec statistics | Available in user view |

IKE-based IPsec tunnel for IPv4 packets configuration example

      - Network requirements

As shown in Figure 3, configure an IPsec tunnel between Switch A and Switch B to protect data flows between Switch A and Switch B. Configure the tunnel to use the security protocol ESP, the encryption algorithm AES-CBC-128, and the authentication algorithm HMAC-SHA1-96.

Network diagram

      - Configuration procedure

            - Configure Switch A:

# Assign an IP address to VLAN-interface 1.

<SwitchA> system-view

[SwitchA] interface vlan-interface 1

[SwitchA-Vlan-interface1] ip address 2.2.2.1 255.255.255.0

[SwitchA-Vlan-interface1] quit

# Define an ACL to identify data flows from Switch A to Switch B.

[SwitchA] acl number 3101

[SwitchA-acl-adv-3101] rule 0 permit ip source 2.2.2.1 0 destination 2.2.3.1 0

[SwitchA-acl-adv-3101] rule 5 permit ip source 2.2.3.1 0 destination 2.2.2.1 0

[SwitchA-acl-adv-3101] quit

# Create an IPsec proposal named tran1.

[SwitchA] ipsec proposal tran1

# Specify the encapsulation mode as tunnel.

[SwitchA-ipsec-proposal-tran1] encapsulation-mode tunnel

# Specify the security protocol as ESP.

[SwitchA-ipsec-proposal-tran1] transform esp

# Specify the algorithms for the proposal.

[SwitchA-ipsec-proposal-tran1] esp encryption-algorithm aes 128

[SwitchA-ipsec-proposal-tran1] esp authentication-algorithm sha1

[SwitchA-ipsec-proposal-tran1] quit

# Configure the IKE peer.

[SwitchA] ike peer peer

[SwitchA-ike-peer-peer] pre-shared-key Ab12<><>

[SwitchA-ike-peer-peer] remote-address 2.2.3.1

[SwitchA-ike-peer-peer] quit

# Create an IPsec policy that uses IKE for IPsec SA negotiation.

[SwitchA] ipsec policy map1 10 isakmp

# Apply the IPsec proposal.

[SwitchA-ipsec-policy-isakmp-map1-10] proposal tran1

# Apply the ACL.

[SwitchA-ipsec-policy-isakmp-map1-10] security acl 3101

# Apply the IKE peer.

[SwitchA-ipsec-policy-isakmp-map1-10] ike-peer peer

[SwitchA-ipsec-policy-isakmp-map1-10] quit

# Apply the IPsec policy group to VLAN-interface 1.

[SwitchA] interface vlan-interface 1

[SwitchA-Vlan-interface1] ipsec policy map1

            - Configure Switch B:

# Assign an IP address to VLAN-interface 1.

<SwitchB> system-view

[SwitchB] interface vlan-interface 1

[SwitchB-Vlan-interface1] ip address 2.2.3.1 255.255.255.0

[SwitchB-Vlan-interface1] quit

# Define an ACL to identify data flows from Switch B to Switch A.

[SwitchB] acl number 3101

[SwitchB-acl-adv-3101] rule 0 permit ip source 2.2.3.1 0 destination 2.2.2.1 0

[SwitchB-acl-adv-3101] rule 5 permit ip source 2.2.2.1 0 destination 2.2.3.1 0

[SwitchB-acl-adv-3101] quit

# Create an IPsec proposal named tran1.

[SwitchB] ipsec proposal tran1

# Specify the encapsulation mode as tunnel.

[SwitchB-ipsec-proposal-tran1] encapsulation-mode tunnel

# Specify the security protocol as ESP.

[SwitchB-ipsec-proposal-tran1] transform esp

# Specify the algorithms for the proposal.

[SwitchB-ipsec-proposal-tran1] esp encryption-algorithm aes 128

[SwitchB-ipsec-proposal-tran1] esp authentication-algorithm sha1

[SwitchB-ipsec-proposal-tran1] quit

# Configure the IKE peer.

[SwitchB] ike peer peer

[SwitchB-ike-peer-peer] pre-shared-key Ab12<><>

[SwitchB-ike-peer-peer] remote-address 2.2.2.1

[SwitchB-ike-peer-peer] quit

# Create an IPsec policy that uses IKE for IPsec SA negotiation.

[SwitchB] ipsec policy use1 10 isakmp

# Apply the ACL.

[SwitchB-ipsec-policy-isakmp-use1-10] security acl 3101

# Apply the IPsec proposal.

[SwitchB-ipsec-policy-isakmp-use1-10] proposal tran1

# Apply the IKE peer.

[SwitchB-ipsec-policy-isakmp-use1-10] ike-peer peer

[SwitchB-ipsec-policy-isakmp-use1-10] quit

# Apply the IPsec policy group to VLAN-interface 1.

[SwitchB] interface vlan-interface 1

[SwitchB-Vlan-interface1] ipsec policy use1

            - Verifying the configuration

After the previous configuration, send traffic from Switch B to Switch A. Switch A starts IKE negotiation with Switch B when receiving the first packet. If IKE negotiation is successful and SAs are set up, the traffic between the two switches will be IPsec protected.

Command reference

    - ah authentication-algorithm

Syntax

ah authentication-algorithm sha1

undo ah authentication-algorithm

View

IPsec proposal view

Default level

2: System level

Parameters

sha1: Uses SHA1.

Description

Use the ah authentication-algorithm command to specify an authentication algorithm for the authentication header (AH) protocol.

Use the undo ah authentication-algorithm command to restore the default.

By default, SHA-1 is used.

Before specifying the authentication algorithm for AH, be sure to use the transform command to specify the security protocol as AH or both AH and ESP.

Related commands: ipsec proposal and transform.

Examples

# Configure IPsec proposal prop1 to use AH and SHA1.

<Sysname> system-view

[Sysname] ipsec proposal prop1

[Sysname-ipsec-proposal-prop1] transform ah

[Sysname-ipsec-proposal-prop1] ah authentication-algorithm sha1

    - connection-name

Syntax

connection-name name

undo connection-name

View

IPsec policy view

Default level

2: System level

Parameters

name: IPsec connection name, a case-insensitive string of 1 to 32 characters.

Description

Use the connection-name command to configure an IPsec connection name. This name functions only as a description of the IPsec policy.

Use the undo connection-name command to restore the default.

By default, no IPsec connection name is configured.

Example

# Set IPsec connection name to aaa.

<Sysname> system-view

[Sysname] ipsec policy policy1 1 isakmp

[Sysname-ipsec-policy-isakmp-policy1-1] connection-name aaa

    - display ipsec policy

Syntax

display ipsec policy [ brief | name policy-name [ seq-number ] ] [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

brief: Displays brief information about all IPsec policies.

name: Displays detailed information about a specified IPsec policy or IPsec policy group.

policy-name: Name of the IPsec policy, a string of 1 to 15 characters.

seq-number: Sequence number of the IPsec policy, in the range 1 to 65535.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use the display ipsec policy command to display information about IPsec policies.

If you do not specify any parameters, the command displays detailed information about all IPsec policies.

If you specify the name policy-name option but leave the seq-number argument, the command displays detailed information about the specified IPsec policy group.

Related commands: ipsec policy (system view).

Examples

# Display brief information about all IPsec policies.

<Sysname> display ipsec policy brief

IPsec-Policy-Name     Mode    acl    ike-peer name    Mapped Template

------------------------------------------------------------------------

aaa-100               manual

policy1-1             isakmp

IPsec-Policy-Name     Mode    acl          Local-Address  Remote-Address

------------------------------------------------------------------------

aaa-100               manual

        - Output description

| Field | Description |
| --- | --- |
| IPsec-Policy-Name | Name and sequence number of the IPsec policy separated by hyphen |
| Mode | Negotiation mode of the IPsec policy, which can be: - manual: Manual mode - isakmp: IKE negotiation mode |
| acl | Access control list (ACL) referenced by the IPsec policy |
| ike-peer name | IKE peer name |
| Local-Address | IP address of the local end |
| Remote-Address | IP address of the remote end |

# Display detailed information about all IPsec policies.

<Sysname> display ipsec policy

===========================================

IPsec Policy Group: "aaa"

Interface:

===========================================

-----------------------------

IPsec policy name: "aaa"

sequence number: 100

mode: manual

-----------------------------

security data flow :

tunnel local  address:

tunnel remote address:

proposal name:

inbound AH setting:

AH spi:

AH string-key:

AH authentication hex key:

inbound ESP setting:

ESP spi:

ESP string-key:

ESP encryption hex key:

ESP authentication hex key:

outbound AH setting:

AH spi:

AH string-key:

AH authentication hex key:

outbound ESP setting:

ESP spi:

ESP string-key:

ESP encryption hex key:

ESP authentication hex key:

===========================================

IPsec Policy Group: "policy1"

Interface:

===========================================

-----------------------------

IPsec policy name: "policy1"

sequence number: 1

mode: isakmp

-----------------------------

security data flow :

selector mode: standard

tunnel remote address:

perfect forward secrecy:

proposal name:

IPsec sa local duration(time based): 3600 seconds

IPsec sa local duration(traffic based): 1843200 kilobytes

policy enable: True

        - Output description

| Field | Description |
| --- | --- |
| security data flow | ACL referenced by the IPsec policy. |
| Interface | Interface to which the IPsec policy is applied. |
| sequence number | Sequence number of the IPsec policy. |
| mode | Negotiation mode of the IPsec policy, which can be: - manual: Manual mode - isakmp: IKE negotiation mode |
| selector mode | Data flow protection mode of the IPsec policy. |
| ike-peer name | IKE peer referenced by the IPsec policy. |
| tunnel local address | Local IP address of the tunnel. |
| tunnel remote address | Remote IP address of the tunnel. |
| perfect forward secrecy | Whether PFS is enabled. |
| proposal name | Proposal referenced by the IPsec policy. |
| policy enable | Whether the IPsec policy is enabled or not. |
| inbound/outbound AH/ESP setting | AH/ESP settings in the inbound/outbound direction, including the SPI and keys. |

    - display ipsec proposal

Syntax

display ipsec proposal [ proposal-name ] [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

proposal-name: Name of a proposal, a string of 1 to 32 characters.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use the display ipsec proposal command to display information about IPsec proposals.

If you do not specify any parameters, the command displays information about all IPsec proposals.

Related commands: ipsec proposal.

Examples

# Display information about all IPsec proposals.

<Sysname> display ipsec proposal

IPsec proposal name: aaa

encapsulation mode: tunnel

transform: ah-new

AH protocol: authentication sha1-hmac-96

        - Output description

| Field | Description |
| --- | --- |
| IPsec proposal name | Name of the IPsec proposal |
| encapsulation mode | Encapsulation mode used by the IPsec proposal, transport or tunnel |
| transform | Security protocol(s) used by the IPsec proposal: AH, ESP, or both. If both protocols are configured, IPsec uses ESP before AH. |
| AH protocol | Authentication algorithm used by AH |
| ESP protocol | Authentication algorithm and encryption algorithm used by ESP |

    - display ipsec sa

Syntax

display ipsec sa [ brief | policy policy-name [ seq-number ] | remote ip-address ] [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

brief: Displays brief information about all IPsec SAs.

policy: Displays detailed information about IPsec SAs created by using a specified IPsec policy.

policy-name: Name of the IPsec policy, a string 1 to 15 characters.

seq-number: Sequence number of the IPsec policy, in the range 1 to 65535.

remote ip-address: Displays detailed information about the IPsec SA with a specified remote address.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use the display ipsec sa command to display information about IPsec SAs.

If you do not specify any parameters, the command displays information about all IPsec SAs.

Related commands: reset ipsec sa and ipsec sa global-duration.

Examples

# Display brief information about all IPsec SAs.

<Sysname> display ipsec sa brief

Src Address  Dst Address  SPI    Protocol     Algorithm

--------------------------------------------------------

10.1.1.1     10.1.1.2     300    ESP          E:AES-192;

A:HMAC-SHA1-96

10.1.1.2     10.1.1.1     400    ESP          E:AES-192;

A:HMAC-SHA1-96

        - Output description

| Field | Description |
| --- | --- |
| Src Address | Local IP address |
| Dst Address | Remote IP address |
| SPI | Security parameter index |
| Protocol | Security protocol used by IPsec |
| Algorithm | Authentication algorithm and encryption algorithm used by the security protocol, where E indicates the encryption algorithm and A indicates the authentication algorithm. A value of NULL means that type of algorithm is not specified. |

# Display detailed information about all IPsec SAs.

<Sysname> display ipsec sa

===============================

Interface: Vlan-interface 1

path MTU: 1500

===============================

-----------------------------

IPsec policy name: "r2"

sequence number: 1

mode: isakmp

-----------------------------

connection id: 3

encapsulation mode: tunnel

perfect forward secrecy:

tunnel:

local  address: 2.2.2.2

remote address: 1.1.1.2

flow:

sour addr: 192.168.2.0/255.255.255.0  port: 0  protocol: IP

dest addr: 192.168.1.0/255.255.255.0  port: 0  protocol: IP

[inbound ESP SAs]

spi: 3564837569 (0xd47b1ac1)

proposal: ESP-ENCRYPT-DES ESP-AUTH-MD5

sa duration (kilobytes/sec): 4294967295/604800

sa remaining duration (kilobytes/sec): 1843200/2686

max received sequence-number: 5

anti-replay check enable: Y

anti-replay window size: 32

udp encapsulation used for nat traversal: N

[outbound ESP SAs]

spi: 801701189 (0x2fc8fd45)

proposal: ESP-ENCRYPT-DES ESP-AUTH-MD5

sa duration (kilobytes/sec): 4294967295/604800

sa remaining duration (kilobytes/sec): 1843200/2686

max sent sequence-number: 6

udp encapsulation used for nat traversal: N

===============================

Protocol: OSPFv3

===============================

-----------------------------

IPsec policy name: "manual"

sequence number: 1

mode: manual

-----------------------------

connection id: 2

encapsulation mode: transport

perfect forward secrecy:

tunnel:

flow :

[inbound AH SAs]

spi: 1234563 (0x12d683)

proposal: AH-SHA1HMAC96

No duration limit for this sa

[outbound AH SAs]

spi: 1234563 (0x12d683)

proposal: AH-SHA1HMAC96

No duration limit for this sa

        - Output description

| Field | Description |
| --- | --- |
| Interface | Interface referencing the IPsec policy. |
| path MTU | Maximum IP packet length supported by the interface. |
| Protocol | Name of the protocol to which the IPsec policy is applied. Support for this field depends on the device model. |
| IPsec policy name | Name of IPsec policy used. |
| sequence number | Sequence number of the IPsec policy. |
| mode | IPsec negotiation mode. |
| connection id | IPsec tunnel identifier. |
| encapsulation mode | Encapsulation mode, transport or tunnel. |
| perfect forward secrecy | Whether the perfect forward secrecy feature is enabled. |
| tunnel | IPsec tunnel. |
| local address | Local IP address of the IPsec tunnel. |
| remote address | Remote IP address of the IPsec tunnel. |
| flow | Data flow. |
| sour addr | Source IP address of the data flow. |
| dest addr | Destination IP address of the data flow. |
| port | Port number. |
| protocol | Protocol type. |
| inbound | Information of the inbound SA. |
| spi | Security parameter index. |
| proposal | Security protocol and algorithms used by the IPsec proposal. |
| sa duration | Lifetime of the IPsec SA. |
| sa remaining key duration | Remaining lifetime of the SA. |
| max received sequence-number | Maximum sequence number of the received packets (relevant to the anti-replay function provided by the security protocol). |
| udp encapsulation used for nat traversal | Whether NAT traversal is enabled for the SA. |
| outbound | Information of the outbound SA. |
| max sent sequence-number | Maximum sequence number of the sent packets (relevant to the anti-replay function provided by the security protocol). |
| anti-replay check enable | Whether IPsec anti-replay checking is enabled. |
| anti-replay window size | Size of the anti-replay window. |

    - display ipsec session

Syntax

display ipsec session [ tunnel-id integer ] [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

integer: ID of the IPsec tunnel, in the range 1 to 2000000000.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use the display ipsec session command to display information about IPsec sessions.

If you do not specify any parameters, the command displays information about all IPsec sessions.

IPsec can find matched tunnels directly by session, reducing the intermediate matching procedures and improving the forwarding efficiency. A session is identified by the quintuplet of protocol, source IP address, source port, destination IP address, and destination port.

Related commands: reset ipsec session.

Examples

# Display information about all IPsec sessions.

<Sysname> display ipsec session

------------------------------------------------------------

total sessions : 2

------------------------------------------------------------

tunnel-id : 3

session idle time/total duration (sec) : 36/300

session flow :      (8 times matched)

Sour Addr : 15.15.15.1             Sour Port:    0  Protocol : 1

Dest Addr : 15.15.15.2             Dest Port:    0  Protocol : 1

------------------------------------------------------------

tunnel-id : 4

session idle duration/total duration (sec) : 7/300

session flow :      (3 times matched)

Sour Addr : 12.12.12.1             Sour Port:    0  Protocol : 1

Dest Addr : 13.13.13.1             Dest Port:    0  Protocol : 1

# Display information about the session with an IPsec tunnel ID of 5.

<Sysname> display ipsec session tunnel-id 5

------------------------------------------------------------

total sessions : 1

------------------------------------------------------------

tunnel-id : 5

session idle time/total duration (sec) : 30/300

session flow :      (4 times matched)

Sour Addr : 12.12.12.2             Sour Port:    0  Protocol : 1

Dest Addr : 13.13.13.2             Dest Port:    0  Protocol : 1

        - Output description

| Field | Description |
| --- | --- |
| total sessions | Total number of IPsec sessions |
| tunnel-id | IPsec tunnel ID, same as the connection-id of the IPsec SA |
| session idle time | Idle duration of the IPsec session in seconds |
| total duration | Lifetime of the IPsec session in seconds, defaulted to 300 seconds |
| session flow | Flow information of the IPsec session |
| times matched | Total number of packets matching the IPsec session |
| Sour Addr | Source IP address of the IPsec session |
| Dest Addr | Destination IP address of the IPsec session |
| Sour Port | Source port number of the IPsec session |
| Dest Port | Destination port number of the IPsec session |
| Protocol | Protocol number of the IPsec protected data flow, for example, 1 for ICMP |

    - display ipsec statistics

Syntax

display ipsec statistics [ tunnel-id integer ] [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

tunnel-id integer: Specifies an IPsec tunnel by its ID, which is in the range 1 to 2000000000.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use the display ipsec statistics command to display IPsec packet statistics.

If you do not specify any parameters, the command displays the statistics for all IPsec packets.

Related commands: reset ipsec statistics.

Examples

# Display statistics on all IPsec packets.

<Sysname> display ipsec statistics

the security packet statistics:

input/output security packets: 47/62

input/output security bytes: 3948/5208

input/output dropped security packets: 0/45

dropped security packet detail:

not enough memory: 0

can't find SA: 45

queue is full: 0

authentication has failed: 0

wrong length: 0

replay packet: 0

packet too long: 0

wrong SA: 0

        - Output description

| Field | Description |
| --- | --- |
| Connection ID | ID of the tunnel |
| input/output security packets | Counts of inbound and outbound IPsec protected packets |
| input/output security bytes | Counts of inbound and outbound IPsec protected bytes |
| input/output dropped security packets | Counts of inbound and outbound IPsec protected packets that are discarded by the device |
| dropped security packet detail | Detailed information about inbound/outbound packets that get dropped |
| not enough memory | Number of packets dropped due to lack of memory |
| can't find SA | Number of packets dropped due to finding no security association |
| queue is full | Number of packets dropped due to full queues |
| authentication has failed | Number of packets dropped due to authentication failure |
| wrong length | Number of packets dropped due to wrong packet length |
| replay packet | Number of packets replayed |
| packet too long | Number of packets dropped due to excessive packet length |
| wrong SA | Number of packets dropped due to improper SA |

    - display ipsec tunnel

Syntax

display ipsec tunnel [ | { begin | exclude | include } regular-expression ]

View

Any view

Default level

1: Monitor level

Parameters

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use the display ipsec tunnel command to display information about IPsec tunnels.

Examples

# Display information about IPsec tunnels.

<Sysname> display ipsec tunnel

total tunnel : 2

------------------------------------------------

connection id: 3

perfect forward secrecy:

SA's SPI:

inbound:  187199087 (0xb286e6f) [ESP]

outbound: 3562274487 (0xd453feb7) [ESP]

tunnel:

local  address:  44.44.44.44

remote address : 44.44.44.55

flow:

sour addr : 44.44.44.0/255.255.255.0  port: 0  protocol : IP

dest addr : 44.44.44.0/255.255.255.0  port: 0  protocol : IP

current Encrypt-card: None

------------------------------------------------

connection id: 5

perfect forward secrecy:

SA's SPI:

inbound:  12345 (0x3039) [ESP]

outbound: 12345 (0x3039) [ESP]

tunnel:

flow:

current Encrypt-card:

        - Output description

| Field | Description |
| --- | --- |
| connection id | Connection ID, used to uniquely identify an IPsec Tunnel |
| perfect forward secrecy | Perfect forward secrecy, indicating which DH group is to be used for fast negotiation mode in IKE phase 2 |
| SA's SPI | SPIs of the inbound and outbound SAs |
| tunnel | Local and remote addresses of the tunnel |
| flow | Data flow protected by the IPsec tunnel, including source IP address, destination IP address, source port, destination port and protocol |
| as defined in acl 3001 | The IPsec tunnel protects all data flows defined by ACL 3001 |
| current Encrypt-card | Encryption card interface used by the current tunnel |

    - encapsulation-mode

Syntax

encapsulation-mode { transport | tunnel }

undo encapsulation-mode

View

IPsec proposal view

Default level

2: System level

Parameters

transport: Uses transport mode.

tunnel: Uses tunnel mode.

Description

Use the encapsulation-mode command to set the encapsulation mode that the security protocol uses to encapsulate IP packets.

Use the undo encapsulation-mode command to restore the default.

By default, a security protocol encapsulates IP packets in tunnel mode.

Related commands: ipsec proposal.

Examples

# Configure IPsec proposal prop2 to encapsulate IP packets in transport mode.

<Sysname> system-view

[Sysname] ipsec proposal prop2

[Sysname-ipsec-proposal-prop2] encapsulation-mode transport

    - esp authentication-algorithm

Syntax

esp authentication-algorithm sha1

undo esp authentication-algorithm

View

IPsec proposal view

Default level

2: System level

Parameters

sha1: Uses the SHA1 algorithm, which uses a 160-bit key.

Description

Use the esp authentication-algorithm command to specify an authentication algorithm for ESP.

Use the undo esp authentication-algorithm command to configure ESP not to perform authentication on packets.

By default, SHA-1 is used.

You must use both ESP authentication and encryption.

For ESP, you must specify an encryption algorithm, an authentication algorithm, or both. The undo esp authentication-algorithm command takes effect only if one encryption algorithm is specified for ESP.

Related commands: ipsec proposal, esp encryption-algorithm, proposal, and transform.

Examples

# Configure IPsec proposal prop1 to use ESP and specify SHA1 as the authentication algorithm for ESP.

<Sysname> system-view

[Sysname] ipsec proposal prop1

[Sysname-ipsec-proposal-prop1] transform esp

[Sysname-ipsec-proposal-prop1] esp authentication-algorithm sha1

    - esp encryption-algorithm

Syntax

esp encryption-algorithm aes [ key-length ]

undo esp encryption-algorithm

View

IPsec proposal view

Default level

2: System level

Parameters

aes: Uses the Advanced Encryption Standard (AES) in CBC mode as the encryption algorithm. The AES algorithm uses a 128- bit, 192-bit, or 256-bit key for encryption.

key-length: Key length for the AES algorithm, which can be 128, 192, and 256 and defaults to 128. This argument is for AES only.

Description

Use the esp encryption-algorithm command to specify an encryption algorithm for ESP.

Use the undo esp encryption-algorithm command to configure ESP not to encrypt packets.

By default, AES-128 is used.

You must use both ESP authentication and encryption.

For ESP, you must specify an encryption algorithm, an authentication algorithm, or both. The undo esp encryption-algorithm command takes effect only if one authentication algorithm is specified for ESP.

Related commands: ipsec proposal, esp authentication-algorithm, proposal, and transform.

Examples

# Configure IPsec proposal prop1 to use ESP and specify AES as the encryption algorithm for ESP.

<Sysname> system-view

[Sysname] ipsec proposal prop1

[Sysname-ipsec-proposal-prop1] transform esp

[Sysname-ipsec-proposal-prop1] esp encryption-algorithm aes

    - ike-peer (IPsec policy view)

Syntax

ike-peer peer-name

undo ike-peer peer-name

View

IPsec policy view

Default level

2: System level

Parameters

peer-name: IKE peer name, a string of 1 to 32 characters.

Description

Use the ike-peer command to reference an IKE peer in an IPsec policy configured through IKE negotiation.

Use the undo ike peer command to remove the reference.

This command applies to only IKE negotiation mode.

Related commands: ipsec policy.

Examples

# Configure a reference to an IKE peer in an IPsec policy.

<Sysname> system-view

[Sysname] ipsec policy policy1 10 isakmp

[Sysname-ipsec-policy-isakmp-policy1-10] ike-peer peer1

    - ipsec anti-replay check

Syntax

ipsec anti-replay check

undo ipsec anti-replay check

View

System view

Default level

2: System level

Parameters

None

Description

Use the ipsec anti-replay check command to enable IPsec anti-replay checking.

Use the undo ipsec anti-replay check command to disable IPsec anti-replay checking.

By default, IPsec anti-replay checking is enabled.

Examples

# Enable IPsec anti-replay checking.

<Sysname> system-view

[Sysname] ipsec anti-replay check

    - ipsec anti-replay window

Syntax

ipsec anti-replay window width

undo ipsec anti-replay window

View

System view

Default level

2: System level

Parameters

width: Size of the anti-replay window. It can be 32, 64, 128, 256, 512, or 1024.

Description

Use the ipsec anti-replay window command to set the size of the anti-replay window.

Use the undo ipsec anti-replay window command to restore the default.

By default, the size of the anti-replay window is 32.

Your configuration affects only IPsec SAs negotiated later.

Examples

# Set the size of the anti-replay window to 64.

<Sysname> system-view

[Sysname] ipsec anti-replay window 64

    - ipsec decrypt check

Syntax

ipsec decrypt check

undo ipsec decrypt check

View

System view

Default level

2: System level

Parameters

None

Description

Use the ipsec decrypt check command to enable ACL checking of de-encapsulated IPsec packets.

Use the undo ipsec decrypt check command to disable ACL checking of de-encapsulated IPsec packets.

By default, ACL checking of de-encapsulated IPsec packets is enabled.

Examples

# Enable ACL checking of de-encapsulated IPsec packets.

<Sysname> system-view

[Sysname] ipsec decrypt check

    - ipsec policy (interface view)

Syntax

ipsec policy policy-name

undo ipsec policy [ policy-name ]

View

Interface view

Default level

2: System level

Parameters

policy-name: Name of the existing IPsec policy group to be applied to the interface, a string of 1 to 15 characters.

Description

Use the ipsec policy command to apply an IPsec policy group to an interface.

Use the undo ipsec policy command to remove the application.

IPsec policies can be applied only to VLAN interfaces and Layer 3 Ethernet interfaces on the switch.

Only one IPsec policy group can be applied to an interface. To apply another IPsec policy group to the interface, remove the original application first. An IPsec policy can be applied to only one interface.

With an IPsec policy group applied to an interface, the system uses each IPsec policy in the group to protect certain data flows.

For each packet to be sent out an IPsec protected interface, the system checks the IPsec policies of the IPsec policy group in the ascending order of sequence numbers. If it finds an IPsec policy whose ACL matches the packet, it uses the IPsec policy to protect the packet. If it finds no ACL of the IPsec policies matches the packet, it does not provide IPsec protection for the packet and sends the packet out directly.

Related commands: ipsec policy (system view).

Examples

# Apply IPsec policy group pg1 to interface VLAN-interface 1.

<Sysname> system-view

[Sysname] interface vlan-interface 1

[Sysname-Vlan-interface1] ipsec policy pg1

    - ipsec policy (system view)

Syntax

ipsec policy policy-name seq-number [ isakmp | manual ]

undo ipsec policy policy-name [ seq-number ]

View

System view

Default level

2: System level

Parameters

policy-name: Name for the IPsec policy, a case-insensitive string of 1 to 15 characters, including letters and digits. No minus sign (-) can be included.

seq-number: Sequence number for the IPsec policy, in the range of 1 to 65535.

isakmp: Sets up SAs through IKE negotiation.

manual: Sets up SAs manually.

Description

Use the ipsec policy command to create an IPsec policy and enter its view.

Use the undo ipsec policy command to delete the specified IPsec policies.

By default, no IPsec policy exists.

When creating an IPsec policy, you must specify the generation mode.

You cannot change the generation mode of an existing IPsec policy; you can only delete the policy and then re-create it with the new mode.

IPsec policies with the same name constitute an IPsec policy group. An IPsec policy is identified uniquely by its name and sequence number. In an IPsec policy group, an IPsec policy with a smaller sequence number has a higher priority.

The undo ipsec policy command without the seq-number argument deletes an IPsec policy group.

Related commands: ipsec policy (interface view) and display ipsec policy.

Examples

# Create an IPsec policy with the name policy1 and sequence number 100, and specify to set up SAs through IKE negotiation.

<Sysname> system-view

[Sysname] ipsec policy policy1 100 isakmp

[Sysname-ipsec-policy-isakmp-policy1-100]

# Create an IPsec policy with the name policy1 and specify the manual mode for it.

<Sysname> system-view

[Sysname] ipsec policy policy1 101 manual

[Sysname-ipsec-policy-manual-policy1-101]

    - ipsec proposal

Syntax

ipsec proposal proposal-name

undo ipsec proposal proposal-name

View

System view

Default level

2: System level

Parameters

proposal-name: Name for the proposal, a case-insensitive string of 1 to 32 characters .

Description

Use the ipsec proposal command to create an IPsec proposal and enter its view.

Use the undo ipsec proposal command to delete an IPsec proposal.

By default, no IPsec proposal exists.

An IPsec proposal created by using the ipsec proposal command takes the security protocol of ESP, the encryption algorithm of AES-128, and the authentication algorithm of SHA1 by default.

Related commands: display ipsec proposal.

Examples

# Create an IPsec proposal named newprop1.

<Sysname> system-view

[Sysname] ipsec proposal newprop1

    - ipsec sa global-duration

Syntax

ipsec sa global-duration { time-based seconds | traffic-based kilobytes }

undo ipsec sa global-duration { time-based | traffic-based }

View

System view

Default level

2: System level

Parameters

seconds: Time-based global SA lifetime in seconds, in the range 180 to 604800.

kilobytes: Traffic-based global SA lifetime in kilobytes, in the range 2560 to 4294967295.

Description

Use the ipsec sa global-duration command to configure the global SA lifetime.

Use the undo ipsec sa global-duration command to restore the default.

By default, the time-based global SA lifetime is 3,600 seconds, and the traffic-based global SA lifetime is 1843200 kilobytes.

When negotiating to set up an SA, IKE prefers the lifetime of the IPsec policy that it uses. If the IPsec policy is not configured with its own lifetime, IKE uses the global SA lifetime.

When negotiating to set up an SA, IKE prefers the shorter one of the local lifetime and that proposed by the remote.

The SA lifetime applies to only IKE negotiated SAs. It is not effective for manually configured SAs.

In FIPS mode, if IPsec uses IKE automatic negotiation, when IPsec SAs reach the traffic-based lifetime, IPsec notifies IKE to re-perform phase 1 and phase 2 negotiations.

Related commands: sa duration.

Examples

# Set the time-based global SA lifetime to 7200 seconds (2 hours).

<Sysname> system-view

[Sysname] ipsec sa global-duration time-based 7200

# Set the traffic-based global SA lifetime to 10240 kilobytes (10 Mbytes).

[Sysname] ipsec sa global-duration traffic-based 10240

    - ipsec session idle-time

Syntax

ipsec session idle-time seconds

undo ipsec session idle-time

View

System view

Default level

2: System level

Parameters

Seconds: IPsec session idle timeout in seconds, in the range of 60 to 3,600.

Description

Use the ipsec session idle-time command to set the idle timeout for IPsec sessions.

Use the undo ipsec session idle-time command to restore the default.

By default, the IPsec session idle timeout is 300 seconds.

Examples

# Set the IPsec session idle timeout to 600 seconds.

<Sysname> system-view

[Sysname] ipsec session idle-time 600

    - pfs

Syntax

pfs { dh-group2 | dh-group5 | dh-group14 }

undo pfs

View

IPsec policy view

Default level

2: System level

Parameters

dh-group2: Uses 1024-bit Diffie-Hellman group.

dh-group5: Uses 1536-bit Diffie-Hellman group.

dh-group14: Uses 2048-bit Diffie-Hellman group.

Description

Use the pfs command to enable and configure the perfect forward secrecy (PFS) feature so that the system uses the feature when employing the IPsec policy to initiate a negotiation.

Use the undo pfs command to remove the configuration.

By default, the PFS feature is not used for negotiation.

In terms of security and necessary calculation time, the following four groups are in the descending order: 2048-bit Diffie-Hellman group (dh-group14), 1536-bit Diffie-Hellman group (dh-group5), and 1024-bit Diffie-Hellman group (dh-group2).

This command allows IPsec to perform an additional key exchange process during the negotiation phase 2, providing an additional level of security.

The local Diffie-Hellman group must be the same as that of the peer.

Related commands: ipsec policy (system view).

Examples

# Enable and configure PFS for IPsec policy policy1.

<Sysname> system-view

[Sysname] ipsec policy policy1 200 isakmp

[Sysname-ipsec-policy-isakmp-policy1-200] pfs dh-group2

    - policy enable

Syntax

policy enable

undo policy enable

View

IPsec policy view

Default level

2: System level

Parameters

None

Description

Used the policy enable command to enable the IPsec policy.

Use the undo policy enable command to disable the IPsec policy.

By default, the IPsec policy is enabled.

If the IPsec policy is not enabled for the IKE peer, the peer cannot take part in the IKE negotiation.

Related commands: ipsec policy (system view).

Examples

# Enable the IPsec policy with the name policy1 and sequence number 100.

<Sysname> system-view

[Sysname] ipsec policy policy1 100 isakmp

[Sysname-ipsec-policy-isakmp-policy1-100] policy enable

    - proposal (IPsec policy view)

Syntax

proposal proposal-name&<1-6>

undo proposal [ proposal-name ]

View

IPsec policy view

Default level

2: System level

Parameters

proposal-name&<1-6>: Name of the IPsec proposal, a string of 1 to 32 characters. &<1-6> means that you can specify the proposal-name argument for up to six times.

Description

Use the proposal command to specify an IPsec proposal for the IPsec policy to reference.

Use the undo proposal command to remove an IPsec proposal reference by the IPsec policy .

By default, an IPsec policy references no IPsec proposal.

The IPsec proposals must already exist.

A manual IPsec policy can reference only one IPsec proposal. To replace a referenced IPsec proposal, use the undo proposal command to remove the original proposal binding and then use the proposal command to reconfigure one.

An IKE negotiated IPsec policy can reference up to six IPsec proposals. The IKE negotiation process will search for and use the exactly matched proposal.

Related commands: ipsec proposal, ipsec policy (system view).

Examples

# Configure IPsec policy policy1 to reference IPsec proposal prop1.

<Sysname> system-view

[Sysname] ipsec proposal prop1

[Sysname-ipsec-proposal-prop1] quit

[Sysname] ipsec policy policy1 100 manual

[Sysname-ipsec-policy-manual-policy1-100] proposal prop1

    - qos pre-classify

Syntax

qos pre-classify

undo qos pre-classify

View

IPsec policy view

Default level

2: System level

Parameters

None

Description

Use the qos pre-classify command to enable packet information pre-extraction.

Use the undo qos pre-classify command to restore the default.

By default, packet information pre-extraction is disabled.

With the packet information pre-extraction feature enabled, QoS classifies a packet based on the header of the original IP packet—the header of the IP packet that has not been encapsulated by IPsec.

Related commands: ipsec policy (system view).

Examples

# Enable packet information pre-extraction.

<Sysname> system-view

[Sysname] ipsec policy policy1 100 isakmp

[Sysname-ipsec-policy-isakmp-policy1-100] qos pre-classify

    - reset ipsec sa

Syntax

reset ipsec sa [ parameters dest-address protocol spi | policy policy-name [ seq-number ] | remote ip-address ]

View

User view

Default level

2: System level

Parameters

parameters: Specifies IPsec SAs that use the specified destination IP address, security protocol, and SPI .

dest-address: Destination address, in dotted decimal notation.

protocol: Security protocol, which can be keyword ah or esp, case insensitive.

spi: Security parameter index, in the range 256 to 4294967295.

policy: Specifies IPsec SAs that use an IPsec policy.

policy-name: Name of the IPsec policy , a case-insensitive string of 1 to 15 characters, including letters and digits.

seq-number: Sequence number of the IPsec policy, in the range 1 to 65535. If no seq-number is specified, all the policies in the IPsec policy group named policy-name are specified.

remote: Specifies SAs to or from a remote address, in dotted decimal notation.

Description

Use the reset ipsec sa command to clear IPsec SAs.

Immediately after a manually set up SA is cleared, the system automatically sets up a new SA based on the parameters of the IPsec policy. After IKE negotiated SAs are cleared, the system sets up new SAs only when IKE negotiation is triggered by interesting packets.

IPsec SAs appear in pairs. If you specify the parameters keyword to clear an IPsec SA, the IPsec SA in the other direction is also automatically cleared.

If you do not specify any parameter, the command clears all IPsec SAs.

Related commands: display ipsec sa.

Examples

# Clear all IPsec SAs.

<Sysname> reset ipsec sa

# Clear the IPsec SA with a remote IP address of 10.1.1.2.

<Sysname> reset ipsec sa remote 10.1.1.2

# Clear the IPsec SA of the IPsec policy with the name of policy1 and sequence number of 10.

<Sysname> reset ipsec sa policy policy1 10

# Clear the IPsec SA with a remote IP address of 10.1.1.2, security protocol of AH, and SPI of 10000.

<Sysname> reset ipsec sa parameters 10.1.1.2 ah 10000

    - reset ipsec session

Syntax

reset ipsec session [ tunnel-id integer ]

View

User view

Default level

2: System level

Parameters

integer: ID of the IPsec tunnel, in the range 1 to 2000000000.

Description

Use the reset ipsec session command to clear the sessions of a specified IPsec tunnel or all IPsec tunnels.

Related commands: display ipsec session.

Examples

# Clear all IPsec sessions.

<Sysname> reset ipsec session

# Clear the sessions of IPsec tunnel 5.

<Sysname> reset ipsec session tunnel-id 5

    - reset ipsec statistics

Syntax

reset ipsec statistics

View

User view

Default level

2: System level

Parameters

None

Description

Use the reset ipsec statistics command to clear IPsec packet statistics.

Related commands: display ipsec statistics.

Examples

# Clear IPsec packet statistics.

<Sysname> reset ipsec statistics

    - sa authentication-hex

Syntax

sa authentication-hex { inbound | outbound } { ah | esp } [ cipher | simple ] hex-key

undo sa authentication-hex { inbound | outbound } { ah | esp }

View

IPsec policy view

Default level

2: System level

Parameters

inbound: Specifies the inbound SA through which IPsec processes the received packets.

outbound: Specifies the outbound SA through which IPsec processes the packets to be sent.

ah: Uses AH.

esp: Uses ESP.

cipher: Sets a ciphertext authentication key.

simple: Sets a plaintext authentication key.

hex-key: Authentication key for the SA. The hex-key argument is a case-sensitive ciphertext string of 8 to 85 characters when the cipher keyword is specified, or a case-insensitive plaintext hexadecimal string when the simple keyword is specified. The plaintext string must be a 20-byte hexadecimal string for SHA1. If neither cipher nor simple is specified, you set a plaintext authentication key string.

Description

Use the sa authentication-hex command to configure an authentication key for an SA.

Use the undo sa authentication-hex command to remove the configuration.

When configuring a manual IPsec policy, you must set the parameters of both the inbound and outbound SAs.

The authentication key for the inbound SA at the local end must be the same as that for the outbound SA at the remote end, and the authentication key for the outbound SA at the local end must be the same as that for the inbound SA at the remote end.

At both ends of an IPsec tunnel, the keys for the inbound and outbound SAs must be in the same format.

Related commands: ipsec policy (system view).

Examples

# Configure the authentication keys of the inbound and outbound SAs that use AH as 0x112233445566778899aabbccddeeff00 and 0xaabbccddeeff001100aabbccddeeff00 in simple text, respectively.

<Sysname> system-view

[Sysname] ipsec policy policy1 100 manual

[Sysname-ipsec-policy-manual-policy1-100] sa authentication-hex inbound ah simple 112233445566778899aabbccddeeff00

[Sysname-ipsec-policy-manual-policy1-100] sa authentication-hex outbound ah simple aabbccddeeff001100aabbccddeeff00

    - sa duration

Syntax

sa duration { time-based seconds | traffic-based kilobytes }

undo sa duration { time-based | traffic-based }

View

IPsec policy view

Default level

2: System level

Parameters

seconds: Time-based SA lifetime in seconds, in the range 180 to 604800.

kilobytes: Traffic-based SA lifetime in kilobytes, in the range 2560 to 4294967295.

Description

Use the sa duration command to set an SA lifetime for the IPsec policy.

Use the undo sa duration command to restore the default.

By default, the SA lifetime of an IPsec policy equals the current global SA lifetime.

By default, the time-based global SA lifetime is 3600 seconds, and traffic-based SA lifetime is 1843200 kilobytes.

When negotiating to set up an SA, IKE prefers the lifetime settings of the IPsec policy that it uses. If the IPsec policy or IPsec proposal is not configured with its own lifetime settings, IKE uses the global SA lifetime settings, which are configured with the ipsec sa global-duration command.

When negotiating to set up an SA, IKE prefers the shorter ones of the local lifetime settings and those proposed by the remote.

The SA lifetime applies to only IKE negotiated SAs. It is not effective for manually configured SAs.

In FIPS mode, if IPsec uses IKE automatic negotiation, when IPsec SAs reach the traffic-based lifetime, the system notifies IKE to re-perform phase 1 and phase 2 negotiations.

Related commands: ipsec sa global-duration, ipsec policy (system view).

Examples

# Set the SA lifetime for IPsec policy1 to 7200 seconds (two hours).

<Sysname> system-view

[Sysname] ipsec policy policy1 100 isakmp

[Sysname-ipsec-policy-isakmp-policy1-100] sa duration time-based 7200

# Set the SA lifetime for IPsec policy policy1 to 20480 kilobytes (20 Mbytes).

<Sysname> system-view

[Sysname] ipsec policy policy1 100 isakmp

[Sysname-ipsec-policy-isakmp-policy1-100] sa duration traffic-based 20480

    - sa encryption-hex

Syntax

sa encryption-hex { inbound | outbound } esp [ cipher | simple ] hex-key

undo sa encryption-hex { inbound | outbound } esp

View

IPsec policy view

Default level

2: System level

Parameters

inbound: Specifies the inbound SA through which IPsec processes the received packets.

outbound: Specifies the outbound SA through which IPsec processes the packets to be sent.

esp: Uses ESP.

cipher string-key: Sets a ciphertext encryption key.

simple hex-key: Sets a plaintext encryption key.

hex-key: Encryption key for the SA. The hex-key argument is a case-sensitive ciphertext string of 8 to 117 characters when the cipher keyword is specified, or a case-insensitive plaintext hexadecimal string when the simple keyword is specified. The plaintext string must be a 16-byte hexadecimal string for AES128-CBC, a 24-byte hexadecimal string for AES192-CBC, or a 32-byte hexadecimal string for AES256-CBC. If neither cipher nor simple is specified, you set a plaintext encryption key string.

Description

Use the sa encryption-hex command to configure an encryption key for an SA.

Use the undo sa encryption-hex command to remove the configuration.

When configuring a manual IPsec policy, you must set the parameters of both the inbound and outbound SAs.

The encryption key for the inbound SA at the local end must be the same as that for the outbound SA at the remote end, and the encryption key for the outbound SA at the local end must be the same as that for the inbound SA at the remote end.

At both ends of an IPsec tunnel, the keys for the inbound and outbound SAs must be in the same format.

Related commands: ipsec policy (system view).

Examples

# Configure the encryption keys for the inbound and outbound SAs that use ESP as 0x1234567890abcdef and 0xabcdefabcdef1234 in simple text, respectively.

<Sysname> system-view

[Sysname] ipsec policy policy1 100 manual

[Sysname-ipsec-policy-manual-policy1-100] sa encryption-hex inbound esp simple 1234567890abcdef

[Sysname-ipsec-policy-manual-policy1-100] sa encryption-hex outbound esp simple abcdefabcdef1234

    - sa spi

Syntax

sa spi { inbound | outbound } { ah | esp } spi-number

undo sa spi { inbound | outbound } { ah | esp }

View

IPsec policy view

Default level

2: System level

Parameters

inbound: Specifies the inbound SA through which IPsec processes the received packets.

outbound: Specifies the outbound SA through which IPsec processes the packets to be sent.

ah: Uses AH.

esp: Uses ESP.

spi-number: Security parameters index (SPI) in the SA triplet, in the range 256 to 4294967295.

Description

Use the sa spi command to configure an SPI for an SA.

Use the undo sa spi command to remove the configuration.

When configuring a manual IPsec policy, you must configure parameters for both inbound and outbound SAs, and make sure that you specify different SPIs for different SAs.

The local inbound SA must use the same SPI and keys as the remote outbound SA. The same is true of the local outbound SA and remote inbound SA.

Related commands: ipsec policy (system view).

Examples

# Set the SPI for the inbound SA to 10000 and that for the outbound SA to 20000 in a manual IPsec policy.

<Sysname> system-view

[Sysname] ipsec policy policy1 100 manual

[Sysname-ipsec-policy-manual-policy1-100] sa spi inbound ah 10000

[Sysname-ipsec-policy-manual-policy1-100] sa spi outbound ah 20000

    - security acl

Syntax

security acl acl-number

undo security acl

View

IPsec policy view

Default level

2: System level

Parameters

acl-number: Number of the ACL for the IPsec policy to reference, in the range 3000 to 3999.

Description

Use the security acl command to specify the ACL for the IPsec policy to reference.

Use the undo security acl command to remove the configuration.

By default, an IPsec policy references no ACL.

With an IKE-dependent IPsec policy configured, data flows can be protected in standard mode. In standard mode, one tunnel protects one data flow. The data flow permitted by each ACL rule is protected by one tunnel that is established separately for it.

When you specify an ACL for an IPsec policy, note these guidelines:

- You must create a mirror image ACL rule at the remote end for each ACL rule created at the local end. Otherwise, IPsec may protect traffic in only one direction.

- The ACL cannot be deployed to an aggregate interface or a tunnel interface.

- You cannot specify multiple ACLs for one IPsec policy or one ACL for multiple IPsec policies. To configure ACL rules you want to deploy for an IPsec policy, you must configure all of them in one ACL and specify the ACL for the IPsec policy.

- You can specify only one ACL for an IPsec policy. To deploy multiple ACL rules, configure the ACL rules in one ACL, and then reference the ACL in an IPsec policy.

- ACLs referenced by IPsec cannot be used by other services.

Related commands: ipsec policy (system view).

Examples

# Configure IPsec policy policy1 to reference ACL 3001.

<Sysname> system-view

[Sysname] acl number 3001

[Sysname-acl-adv-3001] rule permit tcp source 10.1.1.0 0.0.0.255 destination 10.1.2.0 0.0.0.255

[Sysname-acl-adv-3001] quit

[Sysname] ipsec policy policy1 100 manual

[Sysname-ipsec-policy-manual-policy1-100] security acl 3001

    - transform

Syntax

transform { ah | ah-esp | esp }

undo transform

View

IPsec proposal view

Default level

2: System level

Parameters

ah: Uses the AH protocol.

ah-esp: Uses ESP first and then AH.

esp: Uses the ESP protocol.

Description

Use the transform command to specify a security protocol for an IPsec proposal.

Use the undo transform command to restore the default.

By default, the ESP protocol is used.

- If AH is used, the default authentication algorithm is SHA1.

- If ESP is used, the default encryption and authentication algorithms are AES-128 and SHA1, respectively.

- If both AH and ESP are used, AH uses the SHA1 authentication algorithm by default, and ESP uses the AES-128 encryption algorithm and the SHA1 authentication algorithm by default.

The IPsec proposals at the two ends of an IPsec tunnel must use the same security protocol.

Related commands: ipsec proposal.

Examples

# Configure IPsec proposal prop1 to use AH.

<Sysname> system-view

[Sysname] ipsec proposal prop1

[Sysname-ipsec-proposal-prop1] transform ah

    - tunnel local

Syntax

tunnel local ip-address

undo tunnel local

View

IPsec policy view

Default level

2: System level

Parameters

ip-address: Local address for the IPsec tunnel.

Description

Use the tunnel local command to configure the local address of an IPsec tunnel.

Use the undo tunnel local command to remove the configuration.

By default, no local address is configured for an IPsec tunnel.

The local address, if not configured, will be the address of the interface to which the IPsec policy is applied.

Related commands: ipsec policy (system view).

Examples

# Set the local address of the IPsec tunnel to the address of Loopback 0, 10.0.0.1.

<Sysname> system-view

[Sysname] interface loopback 0

[Sysname-LoopBack0] ip address 10.0.0.1 32

[Sysname-LoopBack0] quit

[Sysname] ipsec policy policy1 100 manual

[Sysname-ipsec-policy-manual-policy1-100] tunnel local 10.0.0.1

    - tunnel remote

Syntax

tunnel remote ip-address

undo tunnel remote [ ip-address ]

View

IPsec policy view

Default level

2: System level

Parameters

ip-address: Remote address for the IPsec tunnel.

Description

Use the tunnel remote command to configure the remote address of an IPsec tunnel.

Use the undo tunnel remote command to remove the configuration.

By default, no remote address is configured for the IPsec tunnel.

If you configure the remote address repeatedly, the last one takes effect.

An IPsec tunnel is established between the local and remote ends. The remote IP address of the local end must be the same as that of the local IP address of the remote end.

Related commands: ipsec policy (system view).

Examples

# Set the remote address of the IPsec tunnel to 10.1.1.2.

<Sysname> system-view

[Sysname] ipsec policy policy1 10 manual

[Sysname-ipsec-policy-policy1-10] tunnel remote 10.1.1.2

New feature: IKE

This feature is applicable only to the switches in FIPS mode. For more information about FIPS mode, see "New feature: FIPS"

IKE overview

Built on a framework defined by the Internet Security Association and Key Management Protocol (ISAKMP), Internet Key Exchange (IKE) provides automatic key negotiation and SA establishment services for IPsec, simplifying the application, management, configuration and maintenance of IPsec dramatically.

Instead of transmitting keys directly across a network, IKE peers transmit keying materials between them, and calculate shared keys respectively. Even if a third party captures all exchanged data for calculating the keys, it cannot calculate the keys.

IKE security mechanism

IKE has a series of self-protection mechanisms and supports secure identity authentication, key distribution, and IPsec SA establishment on insecure networks.

Data authentication

Data authentication involves two concepts:

- Identity authentication—Mutual identity authentication between peers. Two authentication methods are available: pre-shared key authentication and PKI-based digital signature authentication (RSA signature).

- Identity protection—Encrypts the identity information with the generated keys before sending the information.

DH

The Diffie-Hellman (DH) algorithm is a public key algorithm. With this algorithm, two peers can exchange keying material and then use the material to calculate the shared keys. Due to the decryption complexity, a third party cannot decrypt the keys even after intercepting all keying materials.

PFS

The Perfect Forward Secrecy (PFS) feature is a security feature based on the DH algorithm. By making sure keys have no derivative relations, it guarantees a broken key brings no threats to other keys. For IPsec, PFS is implemented by adding an additional key exchange at IKE negotiation phase 2.

IKE operation

IKE negotiates keys and establishes SAs for IPsec in two phases:

Phase 1—The two peers establish an ISAKMP SA, a secure, authenticated channel for communication.

Phase 2—Using the ISAKMP SA established in phase 1, the two peers negotiate to establish IPsec SAs.

IKE exchange process in main mode

As shown in Figure 4, the main mode of IKE negotiation in phase 1 involves three pairs of messages:

- SA exchange, used for negotiating the security policy.

- Key exchange, used for exchanging the Diffie-Hellman public value and other values like the random number. Key data is generated in this stage.

- ID and authentication data exchange, used for identity authentication and authentication of data exchanged in phase 1.

IKE functions

IKE provides the following functions for IPsec:

- Automatically negotiates IPsec parameters such as the keys.

- Performs DH exchange when establishing an SA, making sure that each SA has a key independent of other keys.

- Automatically negotiates SAs when the sequence number in the AH or ESP header overflows, making sure that IPsec provides the anti-replay service normally by using the sequence number.

- Provides end-to-end dynamic authentication.

- Identity authentication and management of peers influence IPsec deployment. A large-scale IPsec deployment needs the support of certificate authorities (CAs) or other institutes which manage identity data centrally.

Relationship between IKE and IPsec

Relationship between IKE and IPsec

Figure 5 illustrates the relationship between IKE and IPsec:

- IKE is an application layer protocol using UDP and functions as the signaling protocol of IPsec.

- IKE negotiates SAs for IPsec and delivers negotiated parameters and generated keys to IPsec.

- IPsec uses the SAs set up through IKE negotiation for encryption and authentication of IP packets.

Protocols and standards

These protocols and standards are relevant to IKE:

- RFC 2408, Internet Security Association and Key Management Protocol (ISAKMP)

- RFC 2409, The Internet Key Exchange (IKE)

- RFC 2412, The OAKLEY Key Determination Protocol

IKE configuration task list

Prior to IKE configuration, you must determine the following parameters:

- The strength of the algorithms for IKE negotiation (the security protection level), including the identity authentication method, encryption algorithm, authentication algorithm, and DH group. Different algorithms provide different levels of protection. A stronger algorithm means more resistant to decryption of protected data but requires more resources. Generally, the longer the key, the stronger the algorithm.

- The pre-shared key or the PKI domain the certificate belongs to. For more information about PKI configuration, see the chapter "PKI configuration."

To configure IKE:

| Task | Remarks |
| --- | --- |
| Configuring a name for the local security gateway | Optional. |
| Configuring an IKE proposal | Optional. Required if you want to specify an IKE proposal for an IKE peer to reference. |
| Configuring an IKE peer | Required. |
| Setting keepalive timers | Optional. |
| Setting the NAT keepalive timer | Optional. |
| Configuring a DPD detector | Optional. |
| Disabling next payload field checking | Optional. |

Configuring a name for the local security gateway

If the IKE negotiation peer uses the security gateway name as its ID to initiate IKE negotiation (the id-type name or id-type user-fqdn command is configured on the initiator), configure the ike local-name command in system view or the local-name command in IKE peer view on the local device. If you configure both commands, the name configured in IKE peer view is used.

To configure a name for the local security gateway:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Configure a name for the local security gateway. | ike local-name name | Optional. By default, the device name is used as the name of the local security gateway. |

Configuring an IKE proposal

An IKE proposal defines a set of attributes describing how IKE negotiation should take place. You may create multiple IKE proposals with different preferences. The preference of an IKE proposal is represented by its sequence number, and the lower the sequence number, the higher the preference.

Two peers must have at least one matching IKE proposal for successful IKE negotiation. During IKE negotiation, the initiator sends its IKE proposals to the peer, and the peer searches its own IKE proposals for a match. The search starts from the one with the lowest sequence number and proceeds in the ascending order of sequence number until a match is found or all the IKE proposals are found mismatching. The matching IKE proposals will be used to establish the secure tunnel.

Two matching IKE proposals have the same encryption algorithm, authentication method, authentication algorithm, and DH group. The SA lifetime will take the smaller one of the settings on the two sides.

By default, there is an IKE proposal, which has the lowest preference and uses the default encryption algorithm, authentication method, authentication algorithm, DH group, and ISAKMP SA lifetime.

To configure an IKE proposal:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Create an IKE proposal and enter its view. | ike proposal proposal-number | N/A |
| ##### Specify an encryption algorithm for the IKE proposal. | encryption-algorithm aes-cbc [ key-length ] | Optional. The default is AES-CBC-128. |
| ##### Specify an authentication method for the IKE proposal. | authentication-method { pre-share ¦ rsa-signature } | Optional. Pre-shared key by default. |
| - Specify an authentication algorithm for the IKE proposal. | authentication-algorithm sha | Optional. SHA1 by default. |
| ##### Specify a DH group for key negotiation in phase 1. | dh { group2 ¦ group5 ¦ group14 } | Optional. group2 (the 1024-bit DH group) by default. |
| ##### Set the ISAKMP SA lifetime for the IKE proposal. | sa duration seconds | Optional. 86400 seconds by default. |

|  | NOTE: Before an ISAKMP SA expires, IKE negotiates a new SA to replace it. DH calculation in IKE negotiation takes time, especially on low-end devices. To prevent SA updates from influencing normal communication, set the lifetime greater than 10 minutes. |
| --- | --- |

Configuring an IKE peer

For an IPsec policy that uses IKE, you must configure an IKE peer by performing the following tasks:

- Specify the IKE negotiation mode (main mode) for the local end to use in IKE negotiation phase 1. When acting as the IKE negotiation responder, the local end uses the IKE negotiation mode of the remote end.

- Specify the IKE proposals for the local end to use when acting as the IKE negotiation initiator. When acting as the responder, the local end uses the IKE proposals configured in system view for negotiation.

- Configure a pre-shared key for pre-shared key authentication or a PKI domain for digital signature authentication.

- Specify the ID type for the local end to use in IKE negotiation phase 1. With pre-shared key authentication, the ID type must be IP address for main mode IKE negotiation.

- Specify the name or IP address of the local security gateway. You perform this task only when you want to specify a special address, for example, a loopback interface address, as the local security gateway address.

- Specify the name or IP address of the remote security gateway. For the local end to initiate IKE negotiation, you must specify the name or IP address of the remote security gateway on the local end so the local end can find the remote end.

- Enable NAT traversal. If there is NAT gateway on the path for tunneling, you must configure NAT traversal at the two ends of the IPsec tunnel, because one end may use a public address while the other end uses a private address.

- Specify the dead peer detection (DPD) detector for the IKE peer.

To configure an IKE peer:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| - Create an IKE peer and enter IKE peer view. | ike peer peer-name | N/A |
| - Specify the IKE negotiation mode for phase 1. | exchange-mode main | Optional. The default is main. |
| - Specify the IKE proposals for the IKE peer to reference. | proposal proposal-number&<1-6> | Optional. By default, an IKE peer references no IKE proposals, and, when initiating IKE negotiation, it uses the IKE proposals configured in system view. |
| - Configure the pre-shared key for pre-shared key authentication. | pre-shared-key [ cipher ¦ simple ] key | Configure either command according to the authentication method for the IKE proposal. |
| - Configure the PKI domain for digital signature authentication. | certificate domain domain-name |  |
| - Select the ID type for IKE negotiation phase 1. | id-type { ip ¦ name ¦ user-fqdn } | Optional. ip by default. |
| - Configure the names of the two ends. | - Specify a name for the local security gateway: local-name name - Configure the name of the remote security gateway: remote-name name | Optional. By default, no name is configured for the local security gateway in IKE peer view, and the security gateway name configured by using the ike local-name command is used. The remote gateway name configured with remote-name command on the local gateway must be identical to the local name configured with the local-name command on the peer. |
| - Configure the IP addresses of the two ends. | - Specify an IP address for the local gateway: local-address ip-address - Configure the IP addresses of the remote gateway: remote-address { hostname [ dynamic ] ¦ low-ip-address [ high-ip-address ] } | Optional. By default, it is the primary IP address of the interface referencing the security policy. The remote IP address configured with the remote-address command on the local gateway must be identical to the local IP address configured with the local-address command on the peer. |
| - Enable the NAT traversal function for IPsec/IKE. | nat traversal | Optional. Disabled by default. |
| - Apply a DPD detector to the IKE peer. | dpd dpd-name | Optional. No DPD detector is applied to an IKE peer by default. For more information about DPD configuration, see "Configuring a DPD detector." |

|  | NOTE: After modifying the configuration of an IPsec IKE peer, execute the reset ipsec sa and reset ike sa commands to clear existing IPsec and IKE SAs. Otherwise, SA re-negotiation will fail. |
| --- | --- |

Setting keepalive timers

IKE maintains the link status of an ISAKMP SA by keepalive packets. Generally, if the peer is configured with the keepalive timeout, you must configure the keepalive packet transmission interval on the local end. If the peer receives no keepalive packet during the timeout interval, the ISAKMP SA will be tagged with the TIMEOUT tag (if it does not have the tag), or be deleted along with the IPsec SAs it negotiated (when it has the tag already).

To set the keepalive timers:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the ISAKMP SA keepalive interval. | ike sa keepalive-timer interval seconds | No keepalive packet is sent by default. |
| ##### Set the ISAKMP SA keepalive timeout. | ike sa keepalive-timer timeout seconds | No keepalive packet is sent by default. |

|  | NOTE: The keepalive timeout configured at the local end must be longer than the keepalive interval configured at the remote end. Since it seldom occurs that more than three consecutive packets are lost on a network, the keepalive timeout can be configured to be three times of the keepalive interval. |
| --- | --- |

Setting the NAT keepalive timer

If IPsec traffic needs to pass through NAT security gateways, you must configure the NAT traversal function. If no packet travels across an IPsec tunnel in a certain period of time, the NAT mapping may get aged and be deleted, disabling the tunnel beyond the NAT gateway from transmitting data to the intended end. To prevent NAT mappings from being aged, an ISAKMP SA behind the NAT security gateway sends NAT keepalive packets to its peer at a certain interval to keep the NAT session alive.

To set the NAT keepalive timer:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the NAT keepalive interval. | ike sa nat-keepalive-timer interval seconds | 20 seconds by default. |

Configuring a DPD detector

Dead peer detection (DPD) irregularly detects dead IKE peers. It works as follows:

            - When the local end sends an IPsec packet, it checks the time the last IPsec packet was received from the peer.

            - If the time interval exceeds the DPD interval, it sends a DPD hello to the peer.

            - If the local end receives no DPD acknowledgement within the DPD packet retransmission interval, it retransmits the DPD hello.

            - If the local end still receives no DPD acknowledgement after having made the maximum number of retransmission attempts (two by default), it considers the peer already dead, and clears the IKE SA and the IPsec SAs based on the IKE SA.

DPD enables an IKE entity to check the liveliness of its peer only when necessary. It generates less traffic than the keepalive mechanism, which exchanges messages periodically.

To configure a DPD detector:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Create a DPD detector and enter its view. | ike dpd dpd-name | N/A |
| ##### Set the DPD interval. | interval-time interval-time | Optional. 10 seconds by default. |
| ##### Set the DPD packet retransmission interval. | time-out time-out | Optional. 5 seconds by default. |

Disabling next payload field checking

The Next payload field is in the generic payload header of the last payload of the IKE negotiation message (the message comprises multiple payloads). According to the protocol, this field must be 0 if the payload is the last payload of the packet. However, it may be set to other values on some brands of devices. For interoperability, disable the checking of this field.

To disable Next payload field checking:

| Step | Command | Remark |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Disable Next payload field checking. | ike next-payload check disabled | Enabled by default. |

Displaying and maintaining IKE

| Task | Command | Remarks |
| --- | --- | --- |
| Display IKE DPD information | display ike dpd [ dpd-name ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view. |
| Display IKE peer information | display ike peer [ peer-name ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view. |
| Display IKE SA information | display ike sa [ verbose [ connection-id connection-id ¦ remote-address remote-address ] ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view. |
| Display IKE proposal information | display ike proposal [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view. |
| Clear SAs established by IKE | reset ike sa [ connection-id ] | Available in user view. |

IKE configuration example

      - Network requirements

As shown in Figure 6, configure an IPsec tunnel that uses IKE negotiation between gateways Switch A and Switch B to secure the communication between the two switches.

For Switch A, configure an IKE proposal that uses the sequence number 10 and the authentication algorithm SHA1. Configure Switch B to use the default IKE proposal.

Configure the two routers to use the pre-shared key authentication method.

Network diagram

      - Configuration procedure

            - Make sure Switch A and Switch B can reach each other.

            - Configure Switch A:

# Assign an IP address to VLAN-interface 1.

<SwitchA> system-view

[SwitchA] interface vlan-interface 1

[SwitchA-vlan-interface1] ip address 1.1.1.1 255.255.255.0

[SwitchA-Vlan-interface1] quit

# Configure ACL 3101 to identify traffic from Switch A to Switch B..

[SwitchA] acl number 3101

[SwitchA-acl-adv-3101] rule 0 permit ip source 1.1.1.1 0 destination 2.2.2.2 0

[SwitchA-acl-adv-3101] rule 1 permit ip source 2.2.2.2 0 destination 1.1.1.1 0

[SwitchA-acl-adv-3101] quit

# Create IPsec proposal tran1.

[SwitchA] ipsec proposal tran1

# Set the packet encapsulation mode to tunnel.

[SwitchA-ipsec-proposal-tran1] encapsulation-mode tunnel

# Use security protocol ESP.

[Switch-ipsec-proposal-tran1] transform esp

# Specify encryption and authentication algorithms.

[SwitchA-ipsec-proposal-tran1] esp encryption-algorithm aes 128

[SwitchA-ipsec-proposal-tran1] esp authentication-algorithm sha1

[SwitchA-ipsec-proposal-tran1] quit

# Create an IKE proposal numbered 10.

[SwitchA] ike proposal 10

# Set the authentication algorithm to SHA1.

[SwitchA-ike-proposal-10] authentication-algorithm sha

# Configure the authentication method as pre-shared key.

[SwitchA-ike-proposal-10] authentication-method pre-share

# Set the ISAKMP SA lifetime to 5000 seconds.

[SwitchA-ike-proposal-10] sa duration 5000

[SwitchA-ike-proposal-10] quit

# Create IKE peer peer.

[SwitchA] ike peer peer

# Configure the IKE peer to reference IKE proposal 10.

[SwitchA-ike-peer-peer]proposal 10

# Set the pre-shared key.

[SwitchA-ike-peer-peer] pre-shared-key Ab12<><>

# Specify the IP address of the peer security gateway.

[SwitchA-ike-peer-peer] remote-address 2.2.2.2

[SwitchA-ike-peer-peer] quit

# Create an IPsec policy that uses IKE negotiation.

[SwitchA] ipsec policy map1 10 isakmp

# Reference IPsec proposal tran1.

[SwitchA-ipsec-policy-isakmp-map1-10] proposal tran1

# Reference ACL 3101 to identify the protected traffic.

[SwitchA-ipsec-policy-isakmp-map1-10] security acl 3101

# Reference IKE peer peer.

[SwitchA-ipsec-policy-isakmp-map1-10] ike-peer peer

[SwitchA-ipsec-policy-isakmp-map1-10] quit

# Apply the IPsec policy to VLAN-interface 1.

[SwitchA] interface vlan-interface 1

[SwitchA-Vlan-interface1] ipsec policy map1

            - Configure Switch B:

# Assign an IP address to VLAN-interface 1.

<SwitchB> system-view

[SwitchB] interface Vlan-interface1

[SwitchB-Vlan-interface1] ip address 2.2.2.2 255.255.255.0

[SwitchB-Vlan-interface1] quit

# Configure ACL 3101 to identify traffic from Switch B to Switch A.

[SwitchB] acl number 3101

[SwitchB-acl-adv-3101] rule 0 permit ip source 2.2.2.2 0 destination 1.1.1.0 0

[SwitchB-acl-adv-3101] rule 1 permit ip source 1.1.1.1 0 destination 2.2.2.2 0

[SwitchB-acl-adv-3101] quit

# Create IPsec proposal tran1.

[SwitchB] ipsec proposal tran1

# Set the packet encapsulation mode to tunnel.

[SwitchB-ipsec-proposal-tran1] encapsulation-mode tunnel

# Use security protocol ESP.

[SwitchB-ipsec-proposal-tran1] transform esp

# Specify encryption and authentication algorithms.

[SwitchB-ipsec-proposal-tran1] esp encryption-algorithm aes 128

[SwitchB-ipsec-proposal-tran1] esp authentication-algorithm sha1

[SwitchB-ipsec-proposal-tran1] quit

# Create an IKE proposal numbered 10.

[SwitchB] ike proposal 10

# Set the authentication algorithm to SHA1.

[SwitchB-ike-proposal-10] authentication-algorithm sha

# Configure the authentication method as pre-shared key.

[SwitchB-ike-proposal-10] authentication-method pre-share

# Set the ISAKMP SA lifetime to 5000 seconds.

[SwitchB-ike-proposal-10] sa duration 5000

[SwitchB-ike-proposal-10] quit

# Create IKE peer peer.

[SwitchB] ike peer peer

# Configure the IKE peer to reference IKE proposal 10.

[SwitchB-ike-peer-peer]proposal 10

# Set the pre-shared key.

[SwitchB-ike-peer-peer] pre-shared-key Ab12<><>

# Specify the IP address of the peer security gateway.

[SwitchB-ike-peer-peer] remote-address 1.1.1.1

[SwitchB-ike-peer-peer] quit

# Create an IPsec policy that uses IKE negotiation.

[SwitchB] ipsec policy use1 10 isakmp

# Reference IPsec proposal tran1.

[SwitchB-ipsec-policy-isakmp-use1-10] proposal tran1

# Reference ACL 3101 to identify the protected traffic.

[SwitchB-ipsec-policy-isakmp-use1-10] security acl 3101

# Reference IKE peer peer.

[SwitchB-ipsec-policy-isakmp-use1-10] ike-peer peer

[SwitchB-ipsec-policy-isakmp-use1-10] quit

# Apply the IPsec policy to VLAN-interface 1.

[SwitchB-Vlan-interface1] ipsec policy use1

      - Verifying the configuration

After the above configuration, send traffic from Switch B to Switch A. Switch A starts IKE negotiation with Switch B when receiving the first packet. IKE proposal matching starts with the one having the highest priority. During the matching process, lifetime is not involved but it is determined by the IKE negotiation parties.

Troubleshooting IKE

When you configure parameters to establish an IPsec tunnel, enable IKE error debugging to locate configuration problems:

<Switch> debugging ike error

Invalid user ID

Symptom

Invalid user ID.

Analysis

In IPsec, user IDs are used to identify data flows and to set up different IPsec tunnels for different data flows. Now, the IP address and username are used as the user ID.

The following is the debugging information:

got NOTIFY of type INVALID_ID_INFORMATION

Or

drop message from A.B.C.D due to notification type INVALID_ID_INFORMATION

Solution

Check that the ACLs in the IPsec policies configured on the interfaces at both ends are compatible. Configure the ACLs to mirror each other. For more information about ACL mirroring, see the chapter "IPsec configuration."

Proposal mismatch

Symptom

The proposals mismatch.

Analysis

The following is the debugging information:

got NOTIFY of type NO_PROPOSAL_CHOSEN

Or

drop message from A.B.C.D due to notification type NO_PROPOSAL_CHOSEN

The two parties in the negotiation have no matched proposals.

Solution

For the negotiation in phase 1, look up the IKE proposals for a match. For the negotiation in phase 2, check whether the parameters of the IPsec policies applied on the interfaces are matched, and whether the referred IPsec proposals have a match in protocol, encryption and authentication algorithms.

Failing to establish an IPsec tunnel

Symptom

The expected IPsec tunnel cannot be established.

Analysis

Sometimes this may happen that an IPsec tunnel cannot be established or there is no way to communicate in the presence of an IPsec tunnel in an unstable network. According to examination results, however, ACLs of both parties are configured correctly, and proposals are also matched.

In this case, the problem is usually caused by the reboot of one router after the IPsec tunnel is established.

Solution

- Use the display ike sa command to check whether both parties have established an SA in phase 1.

- Use the display ipsec sa policy command to check whether the IPsec policy on the interface has established IPsec SA.

- If the two commands show that one party has an SA but the other does not, use the reset ipsec sa command to clear the IPsec SA that has no corresponding SA, use the reset ike sa command to clear the IKE SA that has no corresponding IKE SA, and trigger SA re-negotiation.

ACL configuration error

Symptom

ACL configuration error results in data flow blockage.

Analysis

When multiple devices create different IPsec tunnels early or late, a device may have multiple peers. If the device is not configured with ACL rule, the peers send packets to it to set up different IPsec tunnels in different protection granularity respectively. As the priorities of IPsec tunnels are determined by the order they are established, a device cannot interoperate with other peers in fine granularity when its outbound packets are first matched with an IPsec tunnel in coarse granularity.

Solution

When a device has multiple peers, configure ACLs on the device to distinguish different data flows and try to avoid configuring overlapping ACL rules for different peers. If it is unavoidable, the subrules in fine granularity should be configured with higher preferences.

Command reference

authentication-algorithm

Use authentication-algorithm to specify an authentication algorithm for an IKE proposal.

Use undo authentication-algorithm to restore the default.

Syntax

authentication-algorithm sha

undo authentication-algorithm

Default

An IKE proposal uses the SHA1 authentication algorithm.

Views

IKE proposal view

Default command level

2: System level

Parameters

sha: Uses HMAC-SHA1.

Examples

# Set SHA1 as the authentication algorithm for IKE proposal 10.

<Sysname> system-view

[Sysname] ike proposal 10

[Sysname-ike-proposal-10] authentication-algorithm sha

Related commands

- display ike proposal

- ike proposal

authentication-method

Use authentication-method to specify an authentication method for an IKE proposal.

Use undo authentication-method to restore the default.

Syntax

authentication-method { pre-share | rsa-signature }

undo authentication-method

Default

An IKE proposal uses the pre-shared key authentication method.

Views

IKE proposal view

Default command level

2: System level

Parameters

pre-share: Uses the pre-shared key method.

rsa-signature: Uses the RSA digital signature method.

Examples

# Specify that IKE proposal 10 uses the pre-shared key authentication method.

<Sysname> system-view

[Sysname] ike proposal 10

[Sysname-ike-proposal-10] authentication-method pre-share

Related commands

- display ike proposal

- ike proposal

certificate domain

Use certificate domain to configure the PKI domain of the certificate when IKE uses digital signature as the authentication mode.

Use undo certificate domain to remove the configuration.

Syntax

certificate domain domain-name

undo certificate domain

Views

IKE peer view

Default command level

2: System level

Parameters

domain-name: Name of the PKI domain, a string of 1 to 15 characters.

Examples

# Configure the PKI domain as abcde for IKE negotiation.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] certificate domain abcde

Related commands

- authentication-method

- pki domain

dh

Use dh to specify the DH group to be used in key negotiation phase 1 for an IKE proposal.

Use undo dh to restore the default.

Syntax

dh { group2 | group5 | group14 }

undo dh

Default

Group2, the 1024-bit Diffie-Hellman group, is used.

Views

IKE proposal view

Default command level

2: System level

Parameters

group2: Uses the 1024-bit Diffie-Hellman group for key negotiation in phase 1.

group5: Uses the 1536-bit Diffie-Hellman group for key negotiation in phase 1.

group14: Uses the 2048-bit Diffie-Hellman group for key negotiation in phase 1.

Examples

# Specify 1536-bit Diffie-Hellman for IKE proposal 10.

<Sysname> system-view

[Sysname] ike proposal 10

[Sysname-ike-proposal-10] dh group5

Related commands

- display ike proposal

- ike proposal

display ike dpd

Use display ike dpd to display information about Dead Peer Detection (DPD) detectors.

Syntax

display ike dpd [ dpd-name ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

1: Monitor level

Parameters

dpd-name: DPD name, a string of 1 to 32 characters.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Usage guidelines

If you do not specify any parameters, the command displays information about all DPD detectors.

Examples

# Display information about all DPD detectors.

<Sysname> display ike dpd

---------------------------

IKE dpd: dpd1

references: 1

interval-time: 10

time_out: 5

---------------------------

Command output

| Field | Description |
| --- | --- |
| references | Number of IKE peers that use the DPD detector. |
| Interval-time | DPD query trigging interval in seconds. |
| time_out | DPD packet retransmission interval in seconds. |

Related commands

ike dpd

display ike peer

Use display ike peer to display information about IKE peers.

Syntax

display ike peer [ peer-name ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

1: Monitor level

Parameters

peer-name: Name of the IKE peer, a string of 1 to 32 characters.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Usage guidelines

If you do not specify any parameters, the command displays information about all IKE peers.

Examples

# Display information about all IKE peers.

<Sysname> display ike peer

---------------------------

IKE Peer: aaa

exchange mode: main on phase 1

peer id type: ip

peer ip address: 0.0.0.0 ~ 255.255.255.255

local ip address:

peer name:

nat traversal: disable

dpd:

---------------------------

Command output

| Field | Description |
| --- | --- |
| exchange mode | IKE negotiation mode in phase 1. |
| pre-shared-key | Pre-shared key used in phase 1. |
| peer id type | ID type used in phase 1. |
| peer ip address | IP address of the remote security gateway. |
| local ip address | IP address of the local security gateway. |
| peer name | Name of the remote security gateway. |
| nat traversal | Whether NAT traversal is enabled. |
| dpd | Name of the peer DPD detector. |

Related commands

ike peer

display ike proposal

Use display ike proposal to view the settings of all IKE proposals.

Syntax

display ike proposal [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

1: Monitor level

Parameters

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Usage guidelines

This command displays the configuration information of all IKE proposals in the descending order of proposal priorities.

Examples

# Display the settings of all IKE proposals.

<Sysname> display ike proposal

priority authentication authentication encryption Diffie-Hellman duration

method       algorithm    algorithm     group       (seconds)

---------------------------------------------------------------------------

11       PRE_SHARED     SHA         AES_CBC_128     MODP_1024      86400

default  PRE_SHARED     SHA         AES_CBC_128     MODP_1024      86400

Command output

| Field | Description |
| --- | --- |
| priority | Priority of the IKE proposal. |
| authentication method | Authentication method used by the IKE proposal. |
| authentication algorithm | Authentication algorithm used by the IKE proposal. |
| encryption algorithm | Encryption algorithm used by the IKE proposal. |
| Diffie-Hellman group | DH group used in IKE negotiation phase 1. |
| duration (seconds) | ISAKMP SA lifetime of the IKE proposal in seconds. |

Related commands

- authentication-algorithm

- authentication-method

- dh

- encryption-algorithm

- ike proposal

- sa duration

display ike sa

Use display ike sa to display information about the current IKE SAs.

Syntax

display ike sa [ verbose [ connection-id connection-id | remote-address remote-address ] ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

1: Monitor level

Parameters

verbose: Displays detailed information.

connection-id connection-id: Displays detailed information about IKE SAs by connection ID, in the range 1 to 2000000000.

remote: Displays detailed information about IKE SAs with a specified remote address.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Usage guidelines

If you do not specify any parameters or keywords, the command displays brief information about the current IKE SAs.

Examples

# Display brief information about the current IKE SAs.

<Sysname> display ike sa

total phase-1 SAs:  1

connection-id  peer            flag        phase   doi

----------------------------------------------------------

1            202.38.0.2      RD|ST        1      IPSEC

2            202.38.0.2      RD|ST        2      IPSEC

flag meaning

RD--READY ST--STAYALIVE RL--REPLACED FD—FADING TO—TIMEOUT

Command output

| Field | Description |
| --- | --- |
| total phase-1 SAs | Total number of SAs for phase 1. |
| connection-id | Identifier of the ISAKMP SA. |
| peer | Remote IP address of the SA. |
| flag | Status of the SA: - RD (READY)—The SA has been established. - ST (STAYALIVE)—This end is the initiator of the tunnel negotiation. - RL (REPLACED)—The tunnel has been replaced by a new one and will be deleted later. - FD (FADING)—The soft lifetime is over but the tunnel is still in use. The tunnel will be deleted when the hard lifetime is over. - TO (TIMEOUT)—The SA has received no keepalive packets after the last keepalive timeout. If no keepalive packets are received before the next keepalive timeout, the SA will be deleted. |
| phase | The phase the SA belongs to: - Phase 1—The phase for establishing the ISAKMP SA. - Phase 2—The phase for negotiating the security service. IPsec SAs are established in this phase. |
| doi | Interpretation domain the SA belongs to. |

# Display detailed information about the current IKE SAs.

<Sysname> display ike sa verbose

---------------------------------------------

connection id: 2

transmitting entity: initiator

---------------------------------------------

local ip: 4.4.4.4

local id type: IPV4_ADDR

local id: 4.4.4.4

remote ip: 4.4.4.5

remote id type: IPV4_ADDR

remote id: 4.4.4.5

authentication-method: PRE-SHARED-KEY

authentication-algorithm: HASH-SHA1

encryption-algorithm: AES-CBC

life duration(sec): 86400

remaining key duration(sec): 86379

exchange-mode: MAIN

diffie-hellman group: GROUP1

nat traversal: NO

# Display detailed information about the IKE SA with the connection ID of 2.

<Sysname> display ike sa verbose connection-id 2

---------------------------------------------

connection id: 2

transmitting entity: initiator

---------------------------------------------

local ip: 4.4.4.4

local id type: IPV4_ADDR

local id: 4.4.4.4

remote ip: 4.4.4.5

remote id type: IPV4_ADDR

remote id: 4.4.4.5

authentication-method: PRE-SHARED-KEY

authentication-algorithm: HASH-SHA1

encryption-algorithm: AES-CBC

life duration(sec): 86400

remaining key duration(sec): 82480

exchange-mode: MAIN

diffie-hellman group: GROUP14

nat traversal: NO

# Display detailed information about the IKE SA with the remote address of 4.4.4.5.

<Sysname> display ike sa verbose remote-address 4.4.4.5

---------------------------------------------

connection id: 2

transmitting entity: initiator

---------------------------------------------

local ip: 4.4.4.4

local id type: IPV4_ADDR

local id: 4.4.4.4

remote ip: 4.4.4.5

remote id type: IPV4_ADDR

remote id: 4.4.4.5

authentication-method: PRE-SHARED-KEY

authentication-algorithm: HASH-SHA1

encryption-algorithm: DES-CBC

life duration(sec): 86400

remaining key duration(sec): 82236

exchange-mode: MAIN

diffie-hellman group: GROUP1

nat traversal: NO

Command output

| Field | Description |
| --- | --- |
| connection id | Identifier of the ISAKMP SA. |
| transmitting entity | Entity in the IKE negotiation. |
| local ip | IP address of the local gateway. |
| local id type | Identifier type of the local gateway. |
| local id | Identifier of the local gateway. |
| remote ip | IP address of the remote gateway. |
| remote id type | Identifier type of the remote gateway. |
| remote id | Identifier of the remote security gateway. |
| authentication-method | Authentication method used by the IKE proposal. |
| authentication-algorithm | Authentication algorithm used by the IKE proposal. |
| encryption-algorithm | Encryption algorithm used by the IKE proposal. |
| life duration(sec) | Lifetime of the ISAKMP SA in seconds. |
| remaining key duration(sec) | Remaining lifetime of the ISAKMP SA in seconds. |
| exchange-mode | IKE negotiation mode in phase 1. |
| diffie-hellman group | DH group used for key negotiation in IKE phase 1. |
| nat traversal | Whether NAT traversal is enabled. |

Related commands

- ike peer

- ike proposal

dpd

Use dpd to apply a DPD detector to an IKE peer.

Use undo dpd to remove the application.

Syntax

dpd dpd-name

undo dpd

Default

No DPD detector is applied to an IKE peer.

Views

IKE peer view

Default command level

2: System level

Parameters

dpd-name: DPD detector name, a string of 1 to 32 characters.

Examples

# Apply dpd1 to IKE peer peer1.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] dpd dpd1

encryption-algorithm

Use encryption-algorithm to specify an encryption algorithm for an IKE proposal.

Use undo encryption-algorithm to restore the default.

Syntax

encryption-algorithm aes-cbc [ key-length ]

undo encryption-algorithm

Default

The encryption algorithm for an IKE proposal is AES-128.

Views

IKE proposal view

Default command level

2: System level

Parameters

aes-cbc: Uses the AES algorithm in CBC mode as the encryption algorithm. The AES algorithm uses 128-bit, 192-bit, or 256-bit keys for encryption.

key-length: Key length for the AES algorithm, which can be 128, 192 or 256 bits and is defaulted to 128 bits.

Examples

# Use 128-bit AES in CBC mode as the encryption algorithm for IKE proposal 10.

<Sysname> system-view

[Sysname] ike proposal 10

[Sysname-ike-proposal-10] encryption-algorithm aes 128

Related commands

- display ike proposal

- ike proposal

exchange-mode

Use exchange-mode to select an IKE negotiation mode.

Use undo exchange-mode to restore the default.

Syntax

exchange-mode main

undo exchange-mode

Default

Main mode is used.

Views

IKE peer view

Default command level

2: System level

Parameters

main: Main mode.

Examples

# Specify that IKE negotiation works in main mode.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] exchange-mode main

Related commands

id-type

id-type

Use id-type to select the type of the ID for IKE negotiation.

Use undo id-type to restore the default.

Syntax

id-type { ip | name | user-fqdn }

undo id-type

Default

The ID type is IP address.

Views

IKE peer view

Default command level

2: System level

Parameters

ip: Uses an IP address as the ID during IKE negotiation.

name: Uses a FQDN name as the ID during IKE negotiation.

user-fqdn: Uses a user FQDN name as the ID during IKE negotiation.

Usage guidelines

In main mode, only the ID type of IP address can be used in IKE negotiation and SA creation.

Examples

# Use the ID type of name during IKE negotiation.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] id-type name

Related commands

- exchange-mode

- ike local-name

- local-address

- local-name

- remote-address

- remote-name

ike dpd

Use ike dpd to create a DPD detector and enter IKE DPD view.

Use undo ike dpd to remove a DPD detector.

Syntax

ike dpd dpd-name

undo ike dpd dpd-name

Views

System view

Default command level

2: System level

Parameters

dpd-name: Name for the dead peer detection (DPD) detector, a string of 1 to 32 characters.

Usage guidelines

Dead peer detection (DPD) irregularly detects dead IKE peers. It works as follows:

When the local end sends an IPsec packet, it checks the time the last IPsec packet was received from the peer.

If the time interval exceeds the DPD interval, it sends a DPD hello to the peer.

If the local end receives no DPD acknowledgement within the DPD packet retransmission interval, it retransmits the DPD hello.

If the local end still receives no DPD acknowledgement after having made the maximum number of retransmission attempts (two by default), it considers the peer already dead, and clears the IKE SA and the IPsec SAs based on the IKE SA.

DPD enables an IKE entity to check the liveliness of its peer only when necessary. It generates less traffic than the keepalive mechanism, which exchanges messages periodically.

Examples

# Create a DPD detector named dpd2.

<Sysname> system-view

[Sysname] ike dpd dpd2

Related commands

- display ike dpd

- interval-time

- time-out

ike local-name

Use ike local-name to configure a name for the local security gateway.

Use undo ike local-name to restore the default.

Syntax

ike local-name name

undo ike local-name

Default

The device name is used as the name of the local security gateway.

Views

System view

Default command level

2: System level

Parameters

name: Name of the local security gateway for IKE negotiation, a case-sensitive string of 1 to 32 characters.

Usage guidelines

If you configure the id-type name or id-type user-fqdn command on the initiator, the IKE negotiation peer uses the security gateway name as its ID to initiate IKE negotiation, and you must configure the ike local-name command in system view or the local-name command in IKE peer view on the local device. If you configure both the ike local-name command and the local-name command, the name configured by the local-name command is used.

The IKE negotiation initiator sends its security gateway name as its ID to the peer, and the peer uses the security gateway name configured with the remote-name command to authenticate the initiator. Make sure the local gateway name matches the remote gateway name configured on the peer.

Examples

# Configure the local security gateway name as app.

<Sysname> system-view

[Sysname] ike local-name app

Related commands

id-type

remote-name

ike next-payload check disabled

Use ike next-payload check disabled to disable the checking of the Next payload field in the last payload of an IKE message during IKE negotiation, gaining interoperation with products assigning the field a value other than zero.

Use undo ike next-payload check disabled to restore the default.

Syntax

ike next-payload check disabled

undo ike next-payload check disabled

Default

The Next payload field is checked.

Views

System view

Default command level

2: System level

Examples

# Disable Next payload field checking for the last payload of an IKE message.

<Sysname> system-view

[Sysname] ike next-payload check disabled

ike peer (system view)

Use ike peer to create an IKE peer and enter IKE peer view.

Use undo ike peer to delete an IKE peer.

Syntax

ike peer peer-name

undo ike peer peer-name

Views

System view

Default command level

2: System level

Parameters

peer-name: IKE peer name, a string of 1 to 32 characters.

Examples

# Create an IKE peer named peer1 and enter IKE peer view.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1]

ike proposal

Use ike proposal to create an IKE proposal and enter IKE proposal view.

Use undo ike proposal to delete an IKE proposal.

Syntax

ike proposal proposal-number

undo ike proposal proposal-number

Views

System view

Default command level

2: System level

Parameters

proposal-number: IKE proposal number, in the range 1 to 65535. The lower the number, the higher the priority of the IKE proposal. During IKE negotiation, a high priority IKE proposal is matched before a low priority IKE proposal.

Usage guidelines

The system provides a default IKE proposal, which has the lowest priority and uses these settings:

- Encryption algorithm AES-128.

- Authentication algorithm HMAC-SHA1.

- Authentication method Pre-shared key.

- DH group MODP_1024.

- SA lifetime 86400 seconds.

Examples

# Create IKE proposal 10 and enter IKE proposal view.

<Sysname> system-view

[Sysname] ike proposal 10

[Sysname-ike-proposal-10]

Related commands

display ike proposal

ike sa keepalive-timer interval

Use ike sa keepalive-timer interval to set the ISAKMP SA keepalive interval.

Use undo ike sa keepalive-timer interval to disable the ISAKMP SA keepalive transmission function.

Syntax

ike sa keepalive-timer interval seconds

undo ike sa keepalive-timer interval

Default

No keepalive packet is sent.

Views

System view

Default command level

2: System level

Parameters

seconds: Transmission interval of ISAKMP SA keepalives in seconds, in the range 20 to 28,800.

Usage guidelines

The keepalive interval configured at the local end must be shorter than the keepalive timeout configured at the remote end.

Examples

# Set the keepalive interval to 200 seconds.

<Sysname> system-view

[Sysname] ike sa keepalive-timer interval 200

Related commands

ike sa keepalive-timer timeout

ike sa keepalive-timer timeout

Use ike sa keepalive-timer timeout to set the ISAKMP SA keepalive timeout.

Use undo ike sa keepalive-timer timeout to disable the function.

Syntax

ike sa keepalive-timer timeout seconds

undo ike sa keepalive-timer timeout

Default

No keepalive packet is sent.

Views

System view

Default command level

2: System level

Parameters

seconds: ISAKMP SA keepalive timeout in seconds, in the range 20 to 28800.

Usage guidelines

The keepalive timeout configured at the local end must be longer than the keepalive interval configured at the remote end. Since it seldom occurs that more than three consecutive packets are lost on a network, the keepalive timeout can be configured to be three times of the keepalive interval.

Examples

# Set the keepalive timeout to 20 seconds.

<Sysname> system-view

[Sysname] ike sa keepalive-timer timeout 20

Related commands

ike sa keepalive-timer interval

ike sa nat-keepalive-timer interval

Use ike sa nat-keepalive-timer interval to set the NAT keepalive interval.

Use undo ike sa nat-keepalive-timer interval to disable the function.

Syntax

ike sa nat-keepalive-timer interval seconds

undo ike sa nat-keepalive-timer interval

Default

The NAT keepalive interval is 20 seconds.

Views

System view

Default command level

2: System level

Parameters

seconds: NAT keepalive interval in seconds, in the range 5 to 300.

Examples

# Set the NAT keepalive interval to 5 seconds.

<Sysname> system-view

[Sysname] ike sa nat-keepalive-timer interval 5

interval-time

Use interval-time to set the DPD query triggering interval for a DPD detector.

Use undo interval-time to restore the default.

Syntax

interval-time interval-time

undo interval-time

Default

The DPD interval is 10 seconds.

Views

IKE DPD view

Default command level

2: System level

Parameters

interval-time: Sets DPD interval in seconds, in the range of 1 to 300 seconds. When the local end sends an IPsec packet, it checks the time the last IPsec packet was received from the peer. If the time interval exceeds the DPD interval, it sends a DPD hello to the peer.

Examples

# Set the DPD interval to 1 second for dpd2.

<Sysname> system-view

[Sysname] ike dpd dpd2

[Sysname-ike-dpd-dpd2] interval-time 1

local

Use local to set the subnet type of the local security gateway for IKE negotiation.

Use undo local to restore the default.

Syntax

local { multi-subnet | single-subnet }

undo local

Default

The subnet is a single one.

Views

IKE peer view

Default command level

2: System level

Parameters

multi-subnet: Sets the subnet type to multiple.

single-subnet: Sets the subnet type to single.

Usage guidelines

Use this command to enable interoperability with a NetScreen device.

Examples

# Set the subnet type of the local security gateway to multiple.

<Sysname> system-view

[Sysname] ike peer xhy

[Sysname-ike-peer-xhy] local multi-subnet

local-address

Use local-address to configure the IP address of the local security gateway in IKE negotiation.

Use undo local-address to remove the configuration.

Syntax

local-address ip-address

undo local-address

Default

The primary address of the interface referencing the IPsec policy is used as the local security gateway IP address for IKE negotiation.

Views

IKE peer view

Default command level

2: System level

Parameters

ip-address: IP address of the local security gateway to be used in IKE negotiation.

Usage guidelines

Use this command if you want to specify a different address for the local security gateway.

Examples

# Set the IP address of the local security gateway to 1.1.1.1.

<Sysname> system-view

[Sysname] ike peer xhy

[Sysname-ike-peer-xhy] local-address 1.1.1.1

local-name

Use local-name to configure a name for the local security gateway to be used in IKE negation.

Use undo local-name to restore the default.

Syntax

local-name name

undo local-name

Default

The device name is used as the name of the local security gateway view.

Views

IKE peer view

Default command level

2: System level

Parameters

name: Name for the local security gateway to be used in IKE negotiation, a case-sensitive string of 1 to 32 characters.

Usage guidelines

If you configure the id-type name or id-type user-fqdn command on the initiator, the IKE negotiation peer uses the security gateway name as its ID to initiate IKE negotiation, and you must configure the ike local-name command in system view or the local-name command in IKE peer view on the local device. If you configure both the ike local-name command and the local-name command, the name configured by the local-name command is used.

The IKE negotiation initiator sends its security gateway name as its ID to the peer, and the peer uses the security gateway name configured with the remote-name command to authenticate the initiator. Make sure the local gateway name matches the remote gateway name configured on the peer.

Examples

# Set the name of the local security gateway to localgw in IKE peer view of peer1.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] local-name localgw

Relate commands

- id-type

- remote-name

nat traversal

Use nat traversal to enable the NAT traversal function of IKE/IPsec.

Use undo nat traversal to disable the NAT traversal function of IKE/IPsec.

Syntax

nat traversal

undo nat traversal

Default

The NAT traversal function is disabled.

Views

IKE peer view

Default command level

2: System level

Examples

# Enable the NAT traversal function for IKE peer peer1.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] nat traversal

peer

Use peer to set the subnet type of the peer security gateway for IKE negotiation.

Use undo peer to restore the default.

Syntax

peer { multi-subnet | single-subnet }

undo peer

Default

The subnet is a single one.

Views

IKE peer view

Default command level

2: System level

Parameters

multi-subnet: Sets the subnet type to multiple.

single-subnet: Sets the subnet type to single.

Usage guidelines

Use this command to enable interoperability with a NetScreen device.

Examples

# Set the subnet type of the peer security gateway to multiple.

<Sysname> system-view

[Sysname] ike peer xhy

[Sysname-ike-peer-xhy] peer multi-subnet

pre-shared-key

Use pre-shared-key to configure the pre-shared key to be used in IKE negotiation.

Use undo pre-shared-key to remove the configuration.

Syntax

pre-shared-key [ cipher | simple ] key

undo pre-shared-key

Views

IKE peer view

Default command level

2: System level

Parameters

key: Plaintext pre-shared key to be displayed in cipher text, a case-sensitive string of 8 to 128 characters.

cipher key: Specifies the ciphertext pre-shared key to be displayed in cipher text, a case-sensitive string of 8 to 201 characters.

simple key: Specifies the plaintext pre-shared key to be displayed in plain text, a case-sensitive string of 8 to 128 characters.

Examples

# Set the pre-shared key used in IKE negotiation to AAbbcc1234%.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] pre-shared-key AAbbcc1234%

Related commands

authentication-method

proposal (IKE peer view)

Use proposal to specify the IKE proposals for the IKE peer to reference.

Use undo proposal to remove one or all IKE proposals referenced by the IKE peer.

Syntax

proposal proposal-number&<1-6>

undo proposal [ proposal-number ]

Default

An IKE peer references no IKE proposals and, when initiating IKE negotiation, it uses the IKE proposals configured in system view.

Views

IKE peer view

Default command level

2: System level

Parameters

proposal-number&<1-6>: Sequence number of the IKE proposal for the IKE peer to reference, in the range 1 to 65535. &<1-6> means that you can specify the proposal-number argument for up to six times. An IKE proposal with a smaller sequence number has a higher priority.

Usage guidelines

In the IKE negotiation phase 1, the local peer uses the IKE proposals specified for it, if any.

An IKE peer can reference up to six IKE proposals.

The responder uses the IKE proposals configured in system view for negotiation.

Examples

# Configure IKE peer peer1 to reference IKE proposal 10.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] proposal 10

Related commands

- ike proposal

- ike peer (system view)

remote-address

Use remote-address to configure the IP address of the IPsec remote security gateway.

Use undo remote-address to remove the configuration.

Syntax

remote-address { hostname [ dynamic ] | low-ip-address [ high-ip-address ] }

undo remote-address

Views

IKE peer view

Default command level

2: System level

Parameters

hostname: Host name of the IPsec remote security gateway, a case-insensitive string of 1 to 255 characters. The host name uniquely identifies the remote IPsec peer and can be resolved to an IP address by the DNS server.

dynamic: Specifies to use dynamic address resolution for the IPsec remote peer name. If you do not provide this keyword, the local peer has the remote host name resolved only once after you configure the remote host name.

low-ip-address: IP address of the IPsec remote security gateway. It is the lowest address in the address range if you want to specify a range of addresses.

high-ip-address: Highest address in the address range if you want to specify a range of addresses.

Usage guidelines

The IP address configured with the remote-address command must match the local security gateway IP address that the remote security gateway uses for IKE negotiation, which is the IP address configured with the local-address command or, if the local-address command is not configured, the primary IP address of the interface to which the policy is applied.

The local peer can be the initiator of IKE negotiation if the remote address is a host IP address or a host name. The local end can only be the responder of IKE negotiation if the remote address is an address range that the local peer can respond to.

If the IP address of the remote address changes frequently, configure the host name of the remote gateway with the dynamic keyword so that the local peer can use the up-to-date remote IP address to initiate IKE negotiation.

Examples

# Configure the IP address of the remote security gateway as 10.0.0.1.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] remote-address 10.0.0.1

# Configure the host name of the remote gateway as test.com, and specify the local peer to dynamically update the remote IP address.

<Sysname> system-view

[Sysname] ike peer peer2

[Sysname-ike-peer-peer2] remote-address test.com dynamic

Related commands

- id-type ip

- local-address

remote-name

Use remote-name to configure the name of the remote gateway.

Use undo remote-name to remove the configuration.

Syntax

remote-name name

undo remote-name

Views

IKE peer view

Default command level

2: System level

Parameters

name: Name of the peer security gateway for IKE negotiation, a string of 1 to 32 characters.

Usage guidelines

If you configure the id-type name or id-type user-fqdn command on the initiator, the IKE negotiation initiator sends its security gateway name as its ID for IKE negotiation, and the peer uses the security gateway name configured with the remote-name command to authenticate the initiator. Make sure the local gateway name matches the remote gateway name configured on the peer.

Examples

# Configure the remote security gateway name as apple for IKE peer peer1.

<Sysname> system-view

[Sysname] ike peer peer1

[Sysname-ike-peer-peer1] remote-name apple

Related commands

- id-type

- ike local-name

- local-name

reset ike sa

Use reset ike sa to clear IKE SAs.

Syntax

reset ike sa [ connection-id ]

Views

User view

Default command level

2: System level

Parameters

connection-id: Connection ID of the IKE SA to be cleared, in the range 1 to 2000000000.

Usage guidelines

If you do not specify a connection ID, the command clears all ISAKMP SAs.

When you clear a local IPsec SA, its ISAKMP SA can transmit the Delete message to notify the remote end to delete the paired IPsec SA. If the ISAKMP SA has been cleared, the local end cannot notify the remote end to clear the paired IPsec SA, and you must manually clear the remote IPsec SA.

Examples

# Clear an IPsec tunnel to 202.38.0.2.

<Sysname> display ike sa

total phase-1 SAs:  1

connection-id  peer            flag        phase   doi

----------------------------------------------------------

1            202.38.0.2      RD|ST       1       IPSEC

2            202.38.0.2      RD|ST       2       IPSEC

flag meaning

RD--READY ST--STAYALIVE RL--REPLACED FD—FADING TO--TIMEOUT

<Sysname> reset ike sa 2

<Sysname> display ike sa

total phase-1 SAs:  1

connection-id  peer            flag        phase   doi

----------------------------------------------------------

1            202.38.0.2      RD|ST       1       IPSEC

flag meaning

RD--READY ST--STAYALIVE RL--REPLACED FD—FADING TO—TIMEOUT

Related commands

display ike sa

sa duration

Use sa duration to set the ISAKMP SA lifetime for an IKE proposal.

Use undo sa duration to restore the default.

Syntax

sa duration seconds

undo sa duration

Default

The ISAKMP SA lifetime is 86400 seconds.

Views

IKE proposal view

Default command level

2: System level

Parameters

Seconds: Specifies the ISAKMP SA lifetime in seconds, in the range 60 to 604800.

Usage guidelines

Before an SA expires, IKE negotiates a new SA. The new SA takes effect immediately after being set up, and the old one will be cleared automatically when it expires.

Examples

# Specify the ISAKMP SA lifetime for IKE proposal 10 as 600 seconds (10 minutes).

<Sysname> system-view

[Sysname] ike proposal 10

[Sysname-ike-proposal-10] sa duration 600

Related commands

- display ike proposal

- ike proposal

time-out

Use time-out to set the DPD packet retransmission interval for a DPD detector.

Use undo time-out to restore the default.

Syntax

time-out time-out

undo time-out

Default

The DPD packet retransmission interval is 5 seconds.

Views

IKE DPD view

Default command level

2: System level

Parameters

time-out: DPD packet retransmission interval in seconds, in the range 1 to 60.

Examples

# Set the DPD packet retransmission interval to 1 second for dpd2.

<Sysname> system-view

[Sysname] ike dpd dpd2

[Sysname-ike-dpd-dpd2] time-out 1

New feature: Verifying the correctness and integrity of the file

Verifying the correctness and integrity of the file

| Task | Command | Remarks |
| --- | --- | --- |
| Verify the correctness and integrity of the file. | crypto-digest sha256 file file-url | Available in user view. |

Command reference

    - crypto-digest

Use crypto-digest to calculate the digest value of a specific file.

Syntax

crypto-digest sha256 file file-url

Views

User view

Default command level

2: System level

Parameters

sha256: Specifies the digest algorithm SHA-256.

file file-url: Specifies a filename.

Usage guidelines

The digest value of a file is used to verify the correctness and integrity of the file. For example, you can use this command to calculate the digest value of a software package on your switch and compare it with the digest value issued by HP for the software package. If the two values are identical, it means that the package on your switch is the correct one.

Examples

# Use SHA-256 to calculate the digest value of the file 1.bin.

<Sysname> crypto-digest sha256 1.bin

Computing digest...

SHA256 digest(1.bin)=7bcb92458222f91f9a09a807c4c4567efd4d5dc4e4abc06c2a741df7045433eb

Modified feature: Configuring a password for the local user

Feature change description

Supported password to be saved by hash encryption algorithm and displayed as hash value.

Command changes

Modified command: password (local user view)

Old syntax

password [ { cipher | simple } password ]

New syntax

password [ [hash] { cipher | simple } password ]

Views

Local user view

Change description

Before modification:

- Both ciphertext and plaintext passwords are supported.

- A plaintext password is a string of 1 to 63 characters. A ciphertext password is a string of 1 to 117 characters.

After modification:

- Both ciphertext and plaintext passwords are supported. The password is saved by hash encryption algorithm and displayed as hash value.

- If you do not specify hash encryption algorithm, a plaintext password is a string of 1 to 63 characters and a ciphertext password is a string of 1 to 117 characters.

- If you specify hash encryption algorithm, a plaintext password is a string of 1 to 63 characters and a ciphertext password is a string of 1 to 110 characters.

Modified feature: Clearing all users from the password control blacklist

Feature change description

Changed the command to clear all users from the password control blacklist.

Command changes

Modified command: reset password-control blacklist

Old syntax

reset password-control blacklist [ user-name name ]

New syntax

reset password-control blacklist { all | user-name name }

Views

User view

Change description

Before modification: The reset password-control blacklist command without the user-name name option specified clears all users from the password control blacklist.

After modification: The reset password-control blacklist all command clears all users from the password control blacklist.

Modified feature: 802.1X critical VLAN

Feature change description

The events that trigger an 802.1X user to be removed from the 802.1X critical VLAN change in this release. Any of the following events reflects that a RADIUS authentication server is reachable:

- An authentication server is added.

- A response from a RADIUS authentication server is received.

- The RADIUS server probing function detects that a RADIUS authentication server is reachable.

Command changes

None.

Modified feature: MAC authentication critical VLAN

Feature change description

The events that trigger an user to be removed from the MAC authentication critical VLAN change in this release. Any of the following events reflects that a RADIUS authentication server is reachable:

- An authentication server is added.

- A response from a RADIUS authentication server is received.

- The RADIUS server probing function detects that a RADIUS authentication server is reachable.

Command changes

None.

Modified feature: Modifying CLI configuration commands executed in FIPS mode for CC evaluation

Feature change description

Changed CLI configuration command keywords and value ranges when the device is operating in FIPS mode.

Modified command: super password

Old syntax

super password [ level user-level ] { cipher | simple } password

undo super password [ level user-level ]

New syntax

In non-FIPS mode:

super password [ level user-level ] [ hash ] { cipher | simple } password

undo super password [ level user-level ]

In FIPS mode:

super password [ level user-level ] { cipher | simple } password

undo super password [ level user-level ]

Views

2: System level

Parameters

level user-level: User privilege level, which ranges from 1 to 3 and defaults to 3.

Hash: Specifies hash encryption algorithm for generating password. (This keyword is not available for FIPS mode.)

cipher: Sets a ciphertext password for user privilege level switching.

simple: Sets a plaintext password for user privilege level switching.

password: Password string, case sensitive. Change description

In both FIPS and non-FIPS modes, the password must contain uppercase and lowercase letters, digits, and special characters.

In non-FIPS mode:

- If you specify the simple keyword, the password is a plaintext string 1 to 16 characters.

- If you specify the cipher and hash keywords, the password is a ciphertext string of 1 to 110 characters.

- If you specify the cipher keyword only, the password is a ciphertext string of 1 to 53 characters.

In FIPS mode:

- If you specify the simple keyword, the password is a plaintext string of 8 to 16 characters.

- If you specify the cipher keyword, the password is a ciphertext string of 8 to 53 characters.

Change description

After modification:

- In non-FIPS mode

  - The hash keyword was added to support hash encryption algorithm for generating passwords for user privilege level switching.

  - The length of the ciphertext password was changed. A ciphertext password can be a string of 1 to 53 characters, or 1 to 110 characters with the hash keyword specified.

- In FIPS mode

  - The length of a plaintext password was changed to be a string of 8 to 16 characters.

  - The length of a ciphertext password was changed to be a string of 8 to 53 characters.

Modified feature: Modifying login management commands executed in FIPS mode for CC evaluation

Feature change description

- Changed related command keywords and value ranges when the device is operating in FIPS mode.

- Added restrictions to related commands when the device is operating in FIPS mode: The commands lock, user privilege level, and set authentication password are not supported in FIPS mode.

Command changes

Modified command: authentication-mode

Use authentication-mode to set the authentication mode for the user interface.

Use undo authentication-mode to restore the default.

Old syntax

authentication-mode { none | password | scheme }

undo authentication-mode

New syntax

In non-FIPS mode:

authentication-mode { none | password | scheme }

undo authentication-mode

In FIPS mode:

authentication-mode scheme

undo authentication-mode

Default

In non-FIPS mode, the default authentication mode for VTY user interfaces is password, and for AUX user interfaces is none.

In FIPS mode, the default authentication mode is scheme.

Views

User interface view

Default command level

3: Manage level

Parameters

none: Performs no authentication. This keyword is not available for FIPS mode.

password: Performs local password authentication. This keyword is not available for FIPS mode.

scheme: Performs AAA authentication.

Change description

After modification: In FIPS mode, only the authentication mode scheme is supported and the keywords none and password are deleted.

Modified command: protocol inbound

Use protocol inbound to enable the current user interface to support either Telnet, SSH, or all of them. The configuration takes effect next time you log in.

Use undo protocol inbound to restore the default.

Old syntax

protocol inbound { all | ssh | telnet }

undo protocol inbound

New syntax

In non-FIPS mode:

protocol inbound { all | ssh | telnet }

undo protocol inbound

In FIPS mode:

protocol inbound { all | ssh }

undo protocol inbound

Default

All the three protocols are supported.

Views

VTY interface view

Default command level

3: Manage level

Parameters

all: Specifies both Telnet and SSH in non-FIPS mode, and only SSH in FIPS mode.

ssh: Specifies SSH only.

telnet: Specifies Telnet only. This keyword is not available for FIPS mode.

Change description

After modification: In FIPS mode, Telnet is not supported.

Modified command: set authentication password

In non-FIPS mode:

Use set authentication password to set an authentication password.

Use undo set authentication password to remove the local authentication password.

Old syntax

set authentication password { cipher | simple } password

undo set authentication password

New syntax

set authentication password [ hash ] { cipher | simple } password

undo set authentication password

Default

No local authentication password is set.

Views

User interface view

Default command level

3: Manage level

Parameters

Hash: Specifies hash encryption algorithm for generating password. (This keyword is not available for FIPS mode.)

cipher: Sets a ciphertext password for authentication.

simple: Sets a plaintext password for authentication.

- If you specify the simple keyword, the password is a plaintext string 1 to 16 characters.

- If you specify the cipher and hash keywords, the password is a ciphertext string of 1 to 110 characters.

- If you specify the cipher keyword only, the password is a ciphertext string of 1 to 53 characters.

Usage guidelines

For secrecy, all passwords, including passwords configured in plain text, are saved in cipher text.

This command is not supported in FIPS mode.

Change description

After modification: In non-FIPS mode,

- The hash keyword was added to support hash encryption algorithm for generating passwords for user privilege level switching.

- The length of the ciphertext password was changed. A ciphertext password can be a string of 1 to 53 characters, or 1 to 110 characters with the hash keyword specified.

Modified Feature: Modifying software upgrade commands executed in FIPS mode for CC evaluation

Feature change description

Added verification to the signatures of the system software image, Boot ROM image, and path files when the device is operating in FIPS mode.

- The system verifies the signature of the system software image after you execute the commands boot-loader and boot-loader update file. If the verification succeeds, the commands take effect.

- The system verifies the signature of the Boot ROM image after you execute the command bootrom. If the verification succeeds, the command takes effect.

- The system verifies the signatures of the path files after you execute the commands patch install and patch load. If the verification succeeds, the commands take effect.

Command changes

None.

Modified Feature: Modifying configuration file management commands executed in FIPS mode for CC evaluation

Feature change description

The backup startup-configuration and restore startup-configuration commands are not supported when the device is operating in FIPS mode.

Command changes

N/A

Modified Feature: Modifying security commands executed in FIPS mode for CC evaluation

Feature change description

Changed related security command keywords and value ranges when the device is operating in FIPS mode.

Command changes

Modified command: key (HWTACACS scheme view)

Syntax

key { accounting | authentication | authorization } [ cipher | simple ] key

undo key { accounting | authentication | authorization }

Views

HWTACACS scheme view

Default command level

2: System level

Change description

Before modification: The key argument specifies the plaintext or ciphertext key string and must contain at least 1 character.

After modification: In FIPS mode, the key argument specifies the plaintext or ciphertext key string and must contain at least 8 characters.

Modified command: key (RADIUS scheme view)

Syntax

key { accounting | authentication } [ cipher | simple ] key

undo key { accounting | authentication }

Views

RADIUS scheme view

Default command level

2: System level

Change description

Before modification: The key argument specifies the plaintext or ciphertext key string and must contain at least 1 character.

After modification: In FIPS mode, the key argument specifies the plaintext or ciphertext key string and must contain at least 8 characters.

Modified command: password

Old syntax

password [ [ hash ] { cipher | simple } password ]

undo password

New syntax

In non-FIPS mode:

password [ [ hash ] { cipher | simple } password ]

undo password

In FIPS mode:

password

undo password

Views

Local user view

Default command level

2: System level

Change description

In FIPS mode, parameters [ hash ] { cipher | simple } password are deleted.

The FIPS mode must operate with the password control feature. You always set the password in interactive mode. To use the interactive mode, enable the password control feature by the password-control enable command, and then do not specify any option for this command. For more information about password control commands, see the chapter "Password control configuration commands."

When password control is enabled, the password attributes, such as the password length and complexity, are under the restriction of the password control, and the local user password will not be displayed.

Modified command: primary accounting (RADIUS scheme view)

Syntax

primary accounting { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key ] *

undo primary accounting

Views

RADIUS scheme view

Default command level

2: System level

Change description

Before modification: The key argument specifies the plaintext or ciphertext key string and must contain at least 1 character.

After modification: In FIPS mode, the key argument specifies the plaintext or ciphertext key string and must contain at least 8 characters.

Modified command: primary authentication (RADIUS scheme view)

Syntax

primary authentication { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key | probe username name [ interval interval ] ] *

undo primary authentication

Views

RADIUS scheme view

Default command level

2: System level

Change description

Before modification: The key argument specifies the plaintext or ciphertext key string and must contain at least 1 character.

After modification: In FIPS mode, the key argument specifies the plaintext or ciphertext key string and must contain at least 8 characters.

Modified command: secondary accounting (RADIUS scheme view)

Syntax

secondary accounting { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key ] *

undo secondary accounting [ ipv4-address | ipv6 ipv6-address ]

Views

RADIUS scheme view

Default command level

2: System level

Change description

Before modification: The key argument specifies the plaintext or ciphertext key string and must contain at least 1 character.

After modification: In FIPS mode, the key argument specifies the plaintext or ciphertext key string and must contain at least 8 characters.

Modified command: secondary authentication (RADIUS scheme view)

Syntax

secondary authentication { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key| probe username name [ interval interval ] ] *

undo secondary authentication [ ipv4-address | ipv6 ipv6-address ]

Views

RADIUS scheme view

Default command level

2: System level

Change description

Before modification: The key argument specifies the plaintext or ciphertext key string and must contain at least 1 character.

After modification: In FIPS mode, the key argument specifies the plaintext or ciphertext key string and must contain at least 8 characters.

Modified command: password-control composition

Syntax

password-control composition type-number type-number [ type-length type-length ]

undo password-control composition

Views

System view, user group view, local user view

Default command level

2: System level

Change description

Before modification:

The value range for the type-number argument is 1 to 4.

The default global password composition policy is as follows: the minimum number of password composition types is 1 and the minimum number of characters of a password composition type is 1.

After modification:

In FIPS mode, the value of the type-number argument must be 4.

In FIPS mode, the default global password composition policy is as follows: the minimum number of password composition types is 4 and the minimum number of characters of a password composition type is 1.

Modified command: password-control length

Syntax

password-control length length

undo password-control length

Views

System view, user group view, local user view

Default command level

2: System level

Change description

Before modification: The length argument specifies the minimum password length in the range of 4 to 32.

After modification: The value range for the length argument is 8 to 32.

Modified command: password-control super composition

Syntax

password-control super composition type-number type-number [ type-length type-length ]

undo password-control super composition

Views

System view

Default command level

2: System level

Change description

Before modification:

The value range for the type-number argument is 1 to 4.

By default, the minimum number of composition types is 1 and the minimum number of characters of a composition type is 1 for super passwords.

After modification:

In FIPS mode, the value of the type-number argument must be 4.

By default, the minimum number of composition types is 4 and the minimum number of characters of a composition type is 1 for super passwords in FIPS mode.

Modified command: password-control super length

Syntax

password-control super length length

undo password-control super length

Views

System view

Default command level

2: System level

Change description

Before modification: The length argument specifies the minimum length of a super password, in the range of 4 to 16.

After modification: The value range for the length argument is 8 to 16.

Modified command: public-key local create

Syntax

public-key local create { dsa | rsa }

Views

System view

Default command level

2: System level

Change description

Before modification: The DSA or RSA key modulus length is in the range of 512 to 2048 bits, and the default is 1024 bits.

After modification: In FIPS mode, the DSA key modulus length is in the range of 1024 to 2048 bits, and defaults to 1024 bits; the RSA key modulus length is 2048 bits. If the type of key pair already exists, the system asks you whether you want to overwrite it.

Modified command: scp

Old syntax

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

New syntax

In non-FIPS mode:

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

In FIPS mode:

scp [ ipv6] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key rsa | prefer-ctos-cipher { aes128 | aes256 } | prefer-ctos-hmac { sha1 | sha1-96 } | prefer-kex dh-group14 | prefer-stoc-cipher { aes128 | aes256 } | prefer-stoc-hmac { sha1 | sha1-96 } ] *

Views

User view

Default command level

3: Manage level

Change description

After modification:

- In FIPS mode, the following parameters are added:

prefer-ctos-cipher aes256: Specifies aes256-cbc as the preferred encryption algorithm from client to server.

prefer-stoc-cipher aes256: Specifies aes256-cbc as the preferred encryption algorithm from server to client.

In FIPS mode, the following parameters are deleted:

identity-key dsa: Specifies dsa as the algorithm for public key authentication.

prefer-ctos-cipher 3des: Specifies 3des-cbc as the preferred encryption algorithm from client to server.

prefer-ctos-cipher des: Specifies des-cbc as the preferred encryption algorithm from client to server.

prefer-ctos-hmac md5: Specifies hmac-md5 as the preferred HMAC algorithm from client to server.

prefer-ctos-hmac md5-96: Specifies hmac-md5-96 as the preferred HMAC algorithm from client to server.

prefer-kex dh-group-exchange: Specifies diffie-hellman-group-exchange-sha1 as the preferred key exchange algorithm.

prefer-kex dh-group1: Specifies diffie-hellman-group1-sha1 as the preferred key exchange algorithm.

prefer-stoc-cipher 3des: Specifies 3des-cbc as the preferred encryption algorithm from server to client.

prefer-stoc-cipher des: Specifies des-cbc as the preferred encryption algorithm from server to client.

prefer-stoc-hmac md5: Specifies hmac-md5 as the preferred HMAC algorithm from server to client.

prefer-stoc-hmac md5-96: Specifies hmac-md5-96 as the preferred HMAC algorithm from server to client.

Modified command: ssh user

Old syntax

ssh user username service-type stelnet authentication-type { password | { any | password-publickey | publickey } assign publickey keyname }

ssh user username service-type { all | scp | sftp } authentication-type { password | { any | password-publickey | publickey } assign publickey keyname work-directory directory-name }

undo ssh user username

New syntax

In non-FIPS mode:

ssh user username service-type stelnet authentication-type { password | { any | password-publickey | publickey } assign publickey keyname }

ssh user username service-type { all | scp | sftp } authentication-type { password | { any | password-publickey | publickey } assign publickey keyname work-directory directory-name }

undo ssh user username

In FIPS mode:

ssh user username service-type stelnet authentication-type { password | password-publickey assign publickey keyname }

ssh user username service-type { all | scp | sftp } authentication-type { password | password-publickey assign publickey keyname work-directory directory-name }

undo ssh user username

Views

System view

Default command level

3: Manage level

Change description

After modification: In FIPS mode, the any authentication method and publickey authentication method are deleted.

Modified command: ssh2

Old syntax

ssh2 [ ipv6 server] [ port-number ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

New syntax

In non-FIPS mode:

ssh2 [ ipv6 server] [ port-number ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

In FIPS mode:

ssh2 [ ipv6 ] server [ port-number ] [ identity-key rsa | prefer-ctos-cipher { aes128 | aes256 } | prefer-ctos-hmac { sha1 | sha1-96 } | prefer-kex dh-group14 | prefer-stoc-cipher { aes128 | aes256 } | prefer-stoc-hmac { sha1 | sha1-96 } ] *

Views

User view

Default command level

0: Visit level

Change description

After modification:

- In FIPS mode, the following parameters are added:

prefer-ctos-cipher aes256: Specifies aes256-cbc as the preferred encryption algorithm from client to server.

prefer-stoc-cipher aes256: Specifies aes256-cbc as the preferred encryption algorithm from server to client.

In FIPS mode, the following parameters are deleted:

identity-key dsa: Specifies dsa as the algorithm for public key authentication.

prefer-ctos-cipher 3des: Specifies 3des-cbc as the preferred encryption algorithm from client to server.

prefer-ctos-cipher des: Specifies des-cbc as the preferred encryption algorithm from client to server.

prefer-ctos-hmac md5: Specifies hmac-md5 as the preferred HMAC algorithm from client to server.

prefer-ctos-hmac md5-96: Specifies hmac-md5-96 as the preferred HMAC algorithm from client to server.

prefer-kex dh-group-exchange: Specifies diffie-hellman-group-exchange-sha1 as the preferred key exchange algorithm.

prefer-kex dh-group1: Specifies diffie-hellman-group1-sha1 as the preferred key exchange algorithm.

prefer-stoc-cipher 3des: Specifies 3des-cbc as the preferred encryption algorithm from server to client.

prefer-stoc-cipher des: Specifies des-cbc as the preferred encryption algorithm from server to client.

prefer-stoc-hmac md5: Specifies hmac-md5 as the preferred HMAC algorithm from server to client.

prefer-stoc-hmac md5-96: Specifies hmac-md5-96 as the preferred HMAC algorithm from server to client.

Modified command: sftp

Old syntax

sftp [ ipv6 ] server [ port-number ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

New syntax

In non-FIPS mode:

sftp [ ipv6 ] server [ port-number ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

In FIPS mode:

sftp [ ipv6 ] server [ port-number ] [ identity-key rsa | prefer-ctos-cipher { aes128 | aes256 } | prefer-ctos-hmac { sha1 | sha1-96 } | prefer-kex dh-group14 | prefer-stoc-cipher { aes128 | aes256 } | prefer-stoc-hmac { sha1 | sha1-96 } ] *

Views

User view

Default command level

3: Manage level

Change description

After modification:

- In FIPS mode, the following parameters are added:

prefer-ctos-cipher aes256: Specifies aes256-cbc as the preferred encryption algorithm from client to server.

prefer-stoc-cipher aes256: Specifies aes256-cbc as the preferred encryption algorithm from server to client.

In FIPS mode, the following parameters are deleted:

identity-key dsa: Specifies dsa as the algorithm for public key authentication.

prefer-ctos-cipher 3des: Specifies 3des-cbc as the preferred encryption algorithm from client to server.

prefer-ctos-cipher des: Specifies des-cbc as the preferred encryption algorithm from client to server.

prefer-ctos-hmac md5: Specifies hmac-md5 as the preferred HMAC algorithm from client to server.

prefer-ctos-hmac md5-96: Specifies hmac-md5-96 as the preferred HMAC algorithm from client to server.

prefer-kex dh-group-exchange: Specifies diffie-hellman-group-exchange-sha1 as the preferred key exchange algorithm.

prefer-kex dh-group1: Specifies diffie-hellman-group1-sha1 as the preferred key exchange algorithm.

prefer-stoc-cipher 3des: Specifies 3des-cbc as the preferred encryption algorithm from server to client.

prefer-stoc-cipher des: Specifies des-cbc as the preferred encryption algorithm from server to client.

prefer-stoc-hmac md5: Specifies hmac-md5 as the preferred HMAC algorithm from server to client.

prefer-stoc-hmac md5-96: Specifies hmac-md5-96 as the preferred HMAC algorithm from server to client.

Modified command: ciphersuite

Old syntax

ciphersuite [ rsa_3des_ede_cbc_sha | rsa_aes_128_cbc_sha | rsa_aes_256_cbc_sha | rsa_des_cbc_sha | rsa_rc4_128_md5 | rsa_rc4_128_sha ] *

New syntax

In non-FIPS mode:

ciphersuite [ rsa_3des_ede_cbc_sha | rsa_aes_128_cbc_sha | rsa_aes_256_cbc_sha | rsa_des_cbc_sha | rsa_rc4_128_md5 | rsa_rc4_128_sha ] *

In FIPS mode:

ciphersuite [ dhe_rsa_aes_128_cbc_sha | dhe_rsa_aes_256_cbc_sha | rsa_aes_128_cbc_sha | rsa_aes_256_cbc_sha ] *

Views

SSL server policy view

Default command level

2: System level

Change description

After modification:

- In FIPS mode, the following parameters are added:

dhe_rsa_aes_128_cbc_sha: Specifies the key exchange algorithm of DH_RSA, the data encryption algorithm of 128-bit AES_CBC, and the MAC algorithm of SHA.

dhe_rsa_aes_256_cbc_sha: Specifies the key exchange algorithm of DH_RSA, the data encryption algorithm of 256-bit AES_CBC, and the MAC algorithm of SHA.

- In FIPS mode, the following parameters are deleted:

rsa_3des_ede_cbc_sha: Specifies the key exchange algorithm of RSA, the data encryption algorithm of 3DES_EDE_CBC, and the MAC algorithm of SHA.

rsa_des_cbc_sha: Specifies the key exchange algorithm of RSA, the data encryption algorithm of DES_CBC, and the MAC algorithm of SHA.

rsa_rc4_128_md5: Specifies the key exchange algorithm of RSA, the data encryption algorithm of 128-bit RC4, and the MAC algorithm of MD5.

rsa_rc4_128_sha: Specifies the key exchange algorithm of RSA, the data encryption algorithm of 128-bit RC4, and the MAC algorithm of SHA.

Modified command: prefer-cipher

Old syntax

prefer-cipher { rsa_3des_ede_cbc_sha | rsa_aes_128_cbc_sha | rsa_aes_256_cbc_sha | rsa_des_cbc_sha | rsa_rc4_128_md5 | rsa_rc4_128_sha }

undo prefer-cipher

New syntax

In non-FIPS mode:

prefer-cipher { rsa_3des_ede_cbc_sha | rsa_aes_128_cbc_sha | rsa_aes_256_cbc_sha | rsa_des_cbc_sha | rsa_rc4_128_md5 | rsa_rc4_128_sha }

undo prefer-cipher

In FIPS mode:

prefer-cipher { dhe_rsa_aes_128_cbc_sha | dhe_rsa_aes_256_cbc_sha | rsa_aes_128_cbc_sha | rsa_aes_256_cbc_sha }

undo prefer-cipher

Views

SSL client policy view

Default command level

2: System level

Change description

After modification:

In FIPS mode, the following parameters are added:

dhe_rsa_aes_128_cbc_sha: Specifies the key exchange algorithm of DH_RSA, the data encryption algorithm of 128-bit AES_CBC, and the MAC algorithm of SHA.

dhe_rsa_aes_256_cbc_sha: Specifies the key exchange algorithm of DH_RSA, the data encryption algorithm of 256-bit AES_CBC, and the MAC algorithm of SHA.

In FIPS mode, the following parameters are deleted:

rsa_3des_ede_cbc_sha: Specifies the key exchange algorithm of RSA, the data encryption algorithm of 3DES_EDE_CBC, and the MAC algorithm of SHA.

rsa_des_cbc_sha: Specifies the key exchange algorithm of RSA, the data encryption algorithm of DES_CBC, and the MAC algorithm of SHA.

rsa_rc4_128_md5: Specifies the key exchange algorithm of RSA, the data encryption algorithm of 128-bit RC4, and the MAC algorithm of MD5.

rsa_rc4_128_sha: Specifies the key exchange algorithm of RSA, the data encryption algorithm of 128-bit RC4, and the MAC algorithm of SHA.

Modified command: certificate request mode

Syntax

certificate request mode { auto [ key-length key-length | password { cipher | simple } password ] * | manual }

undo certificate request mode

Views

PKI domain view

Default command level

2: System level

Change description

Before modification: The value range of the key-length argument is 512 bits to 2048 bits, and the default value is 1024 bits.

After modification: In FIPS mode, the value of the key-length argument must be 2048 bits.

Modified feature: Modifying SNMP commands executed in FIPS mode for CC evaluation

Feature change description

Changed related SNMP command keywords and value ranges when the device is operating in FIPS mode.

Command changes

Modified command: display snmp-agent community

Syntax

display snmp-agent community [ read | write ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

This command is not supported in FIPS mode.

Modified command: snmp-agent community

Syntax

snmp-agent community { read | write } community-name [ acl acl-number | mib-view view-name ] *

undo snmp-agent community { read | write } community-name

Views

System view

Change description

This command is not supported in FIPS mode.

Modified command: snmp-agent group

Syntax

snmp-agent group { v1 | v2c } group-name [ read-view view-name ] [ write-view view-name ] [ notify-view view-name ] [ acl acl-number ]

undo snmp-agent group { v1 | v2c } group-name

Views

System view

Change description

This command is not supported in FIPS mode.

Modified command: snmp-agent usm-user { v1 | v2c }

Syntax

snmp-agent group { v1 | v2c } group-name [ read-view view-name ] [ write-view view-name ] [ notify-view view-name ] [ acl acl-number ]

undo snmp-agent group { v1 | v2c } group-name

Views

System view

Change description

This command is not supported in FIPS mode.

Modified command: snmp-agent calculate-password

Old syntax

snmp-agent calculate-password plain-password mode { 3desmd5 | 3dessha | md5 | sha } { local-engineid | specified-engineid engineid }

New syntax

In non-FIPS mode:

snmp-agent calculate-password plain-password mode { 3desmd5 | 3dessha | md5 | sha } { local-engineid | specified-engineid engineid }

In FIPS mode:

snmp-agent calculate-password plain-password mode sha { local-engineid | specified-engineid engineid }

Views

System view

Change description

After modification: In FIPS mode, the keywords 3desmd5, 3dessha, and md5 are deleted.

Modified command: snmp-agent sys-info

Old syntax

snmp-agent sys-info { contact sys-contact | location sys-location | version { all | { v1 | v2c | v3 }* } }

undo snmp-agent sys-info { contact | location | version { all | { v1 | v2c | v3 }* } }

New syntax

In non-FIPS mode:

snmp-agent sys-info { contact sys-contact | location sys-location | version { all | { v1 | v2c | v3 }* } }

undo snmp-agent sys-info { contact | location | version { all | { v1 | v2c | v3 }* } }

In FIPS mode:

snmp-agent sys-info { contact sys-contact | location sys-location | version v3 }

undo snmp-agent sys-info { contact | location | version v3 }

Views

System view

Change description

After modification: In FIPS mode, the keywords all, v1, and v2c are deleted.

Modified command: snmp-agent target-host

Old syntax

snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] params securityname security-string [ v1 | v2c | v3 [ authentication | privacy ] ]

undo snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } params securityname security-string

New syntax

In non-FIPS mode:

snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] params securityname security-string [ v1 | v2c | v3 [ authentication | privacy ] ]

undo snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } params securityname security-string

In FIPS mode:

snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] params securityname security-string v3 [ authentication | privacy ]

undo snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } params securityname security-string

Views

System view

Change description

After modification: In FIPS mode, the keywords v1 and v2c are deleted.

Modified command: snmp-agent usm-user v3

Old syntax

snmp-agent usm-user v3 user-name group-name [ cipher ] [ authentication-mode { md5 | sha } auth-password [ privacy-mode { 3des | aes128 | des56 } priv-password ] ] [ acl acl-number | acl ipv6 ipv6-acl-number ] *

undo snmp-agent usm-user v3 user-name group-name { local | engineid engineid-string }

New syntax

In non-FIPS mode:

snmp-agent usm-user v3 user-name group-name [ cipher ] [ authentication-mode { md5 | sha } auth-password [ privacy-mode { 3des | aes128 | des56 } priv-password ] ] [ acl acl-number | acl ipv6 ipv6-acl-number ] *

undo snmp-agent usm-user v3 user-name group-name { local | engineid engineid-string }

In FIPS mode:

snmp-agent usm-user v3 user-name group-name [ cipher ] [ authentication-mode sha auth-password [ privacy-mode aes128 priv-password ] ] [ acl acl-number | acl ipv6 ipv6-acl-number ] *

undo snmp-agent usm-user v3 user-name group-name { local | engineid engineid-string }

Views

System view

Change description

After modification: In FIPS mode, the keywords md5, 3des, and des56 are deleted.

# A5120EI-CMW520-R2215

This release has the following changes:

New feature: Changing the brand name

New feature: Configuring the maximum number of Selected ports allowed for an aggregation group

New feature: Bulk configuring interfaces

Modified feature: Displaying the remaining power of the IRF fabric

Modified feature: NTP

Modified feature: Setting the IRF link down report delay

New feature: Changing the brand name

  - Changing the brand name

Some HP and H3C switches (see Table 1) can form an IRF fabric, and their MPUs are interchangeable.

If different brand MPUs are used on your switch or IRF fabric, change the MPU names to be the same to prevent an active/standby MPU switchover or master re-election from causing network management problems, including device information (vendor name, device model, and device panels) changes and re-collection of information about MPUs and other hardware components.

HP and H3C switch model mappings

| HP switch model | H3C switch model |
| --- | --- |
| HP 5120-24G EI Switch with 2 Interface Slots (JE068A) | S5120-28C-EI |
| HP 5120-48G EI Switch with 2 Interface Slots (JE069A) | S5120-52C-EI |
| HP 5120-24G-PoE+ EI Switch with 2 Interface Slots (JG236A) | S5120-28C-PWR-EI |
| HP 5120-48G-PoE+ EI Switch with 2 Interface Slots (JG237A) | S5120-52C-PWR-EI |

Configuration preparation

Before you change the brand name for an HP and H3C switch, prepare the proper Boot ROM and system software image file according to the switch model mappings as listed in Table 1. The following describes the procedure for changing the brand name of an H3C switch to HP. The procedure is the same for changing the brand names among HP, H3C switches.

            - Load the proper HP Boot ROM to the flash memory of the H3C switch and use the HP Boot ROM to upgrade the Boot ROM of the switch.

            - Load the proper HP system software image file to the flash memory of the H3C switch, specify the file as the main system software image file, and reboot the switch.

            - Execute the brand command and reboot the switch.

|  | NOTE: For HP 5120 EI, use the bootrom update command to upgrade the Boot ROM. |
| --- | --- |

Configuration procedure

You can use the display brand command to display the brand names of the member switches. If any consistent brand names exist in the IRF fabric, change them to the same.

To change brand name for a member switch:

| Step | Command |
| --- | --- |
| - Change the brand name for a member switch. | brand { hp ¦ h3 } [ slot slot-number ] |
| - Reboot the member switch. | reboot slot slot-number |

After you change the brand name for a member switch, the switch can use the later software versions for the new brand.

|  | NOTE: The default settings vary with different brands. Changing the brand name might affect the running configuration. After you change the brand name of a member switch, verify the configuration and re-configure the switch if necessary. |
| --- | --- |

Command reference

    - brand

Syntax

brand { hp | h3c } [ slot slot-number ]

View

User view

Default level

2: System level

Parameters

slot slot-number: Specifies an IRF member switch. If this option is not specified, the command applies to all member switches in the IRF fabric.

Description

Use brand to change the brand name for an IRF member switch.

After you perform this command, use the display brand command to verify the new brand name and then reboot the member switch to make your change take effect.

Examples

# Display brand information.

<H3C>display brand

Current BRANDs:

Slot 1: HP.

Slot 3: H3C.

New BRANDs:

Slot 1: HP.

Slot 3: H3C.

<H3C>

# Change the brand name of member switch 3 to HP.

<HP>brand hp slot 3

Configuration will take effect after next reboot.

Do you want to continue? [Y/N]:y

Configuration is successful.

# Display brand information.

<H3C>display brand

Current BRANDs:

Slot 1: HP.

Slot 3: H3C.

New BRANDs:

Slot 1: HP.

Slot 3: HP.

<H3C>

The output shows that the brand name has been changed. After a reboot, member switch 3 becomes an HP member switch.

    - display brand

Syntax

display brand [ | { begin | exclude | include } regular-expression ]

View

User view

Default level

1: Monitor level

Parameters

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Description

Use display brand to display the brand name of a member switch.

Examples

# Display the brand name of the current member switch.

<H3C>display brand

Current BRANDs:

Slot 1: HP.

Slot 3: H3C.

New BRANDs:

Slot 1: HP.

Slot 3: H3C.

<H3C>

New feature: Configuring the maximum number of Selected ports allowed for an aggregation group

By default, the maximum number of Selected ports allowed in an aggregation group depends on the hardware capabilities of the member ports. After you manually configure the maximum number of Selected ports in an aggregation group, the maximum number of Selected ports allowed in the aggregation group is the lower value of the two upper limits.

You can configure redundancy between two ports using the following guideline: Assign two ports to an aggregation group, and configure the maximum number of Selected ports allowed in the aggregation group as 1. In this way, only one Selected port is allowed in the aggregation group at any point in time, while the Unselected port serves as a backup port.

Configuration guidelines

Follow these guidelines when you configure the port threshold settings:

- If you set a minimum threshold for a static aggregation group, also make the same setting for its peer aggregation group to guarantee correct aggregation.

- Make sure the two link aggregation ends have the same maximum numbers of selected ports.

Make sure you understand the following impacts of the port threshold settings:

- Configuring the maximum number of Selected ports in an aggregation group may cause some of the selected member ports in the aggregation group to become unselected.

Configuration procedure

To configure the maximum number of Selected ports allowed for an aggregation group:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter Layer 2 aggregate interface view. | - interface bridge-aggregation interface-number | Use either command. |
| ##### Configure the maximum number of Selected ports allowed for the aggregation group. | link-aggregation selected-port maximum number | By default, the maximum number of Selected ports allowed in an aggregation group depends on only the hardware capabilities of the member ports. |

Command reference

link-aggregation selected-port maximum

Syntax

link-aggregation selected-port maximum number

undo link-aggregation selected-port maximum

View

Layer 2 aggregate interface view

Default level

2: System level

Parameters

number: Specifies the maximum number of Selected ports allowed in an aggregation group. This argument ranges from 1 to 8.

Description

Use link-aggregation selected-port maximum to configure the maximum number of Selected ports allowed in the aggregation group.

Use undo link-aggregation selected-port maximum to restore the default setting.

By default, the maximum number of Selected ports allowed in an aggregation group is limited only by the hardware capabilities of the member ports.

Executing this command may cause some of the member ports in the aggregation group to become unselected.

The maximum numbers of Selected ports for the local and peer aggregation groups must be consistent.

Examples

# Configure the maximum number of Selected ports as 3 in the aggregation group corresponding to Layer 2 aggregate interface Bridge-Aggregation 1.

<Sysname> system-view

[Sysname] interface bridge-aggregation 1

[Sysname-Bridge-Aggregation1] link-aggregation selected-port maximum 3

New feature: Bulk configuring interfaces

You can enter interface range view to bulk configure multiple interfaces with the same feature instead of configuring them one by one. For example, you can perform the shutdown command in interface range view to shut down a range of interfaces.

Failure of applying a command on one member interface does not affect the application of the command on the other member interfaces. If applying a command on one member interface fails, the system displays an error message and continues with the next member interface.

Configuration guidelines

When you bulk configure interfaces in interface range view, follow these restrictions and guidelines:

- In interface range view, only the commands supported by the first interface are available.

- Do not assign an aggregate interface and any of its member interfaces to an interface range at the same time. Some commands, after being executed on both an aggregate interface and its member interfaces, can break up the aggregation.

- No limit is set on the maximum number of interfaces in an interface range. The more interfaces in an interface range, the longer the command execution time.

Configuration procedure

To bulk configure interfaces:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter interface range view. | Approach 1: interface range interface-list Approach 2: interface range name name [ interface interface-list ] | Use either approach. In approach 2, you assign a name to an interface range and can specify this name rather than the interface range to enter the interface range view. |
| ##### Display commands available for the first interface in the interface range. | Enter ? at the interface range prompt. | Optional. |
| ##### Perform available commands to configure the interfaces. | Available commands vary by interface. | N/A |
| ##### Verify the configuration. | display this | Optional. |

Command reference

interface range

Syntax

interface range interface-list

View

System view

Default level

2: System level

Parameters

interface-list: Interface list in the format of interface-list = { interface-type interface-number [ to interface-type interface-number ] }&<1-5>. The interface-type interface-number argument specifies an interface by its type and number. &<1-5> indicates that you can specify up to five interfaces or interface lists. When you specify the to keyword in interface-type interface-number1 to interface-type interface-number2, the interfaces before and after the to keyword must be on the same interface card or subcard, and the interface number before to must be no greater than the one after to.

Description

Use interface range to create an interface range and enter interface range view.

You can use this command to enter interface range view to bulk configure multiple interfaces with the same feature instead of configuring them one by one. For example, you can perform the shutdown command in interface range view to shut down a range of interfaces.

In interface range view, only the commands supported by the first interface are available. To view the commands supported by the first interface in the interface range, enter the interface range view and enter ? at the command line interface prompt.

To verify the configuration of the first interface in the interface range, execute the display this command in interface range view.

Failure of applying a command on one member interface does not affect the application of the command on the other member interfaces. If applying a command on one member interface fails, the system displays an error message and continues with the next member interface.

Examples

# Shut down interfaces GigabitEthernet 1/0/1 through GigabitEthernet 1/0/24, VLAN interface 2,

<Sysname> system-view

[Sysname] interface range gigabitethernet1/0/1 to gigabitethernet1/0/24 vlan-interface 2

[Sysname-if-range] shutdown

interface range name

Syntax

interface range name name [ interface interface-list ]

undo interface range name name

View

System view

Default level

2: System level

Parameters

name: Interface range name, a case-sensitive string of 1 to 32 characters.

interface-list: Interface list in the format of interface-list = { interface-type interface-number [ to interface-type interface-number ] }&<1-5>. The interface-type interface-number argument specifies an interface by its type and number. &<1-5> indicates that you can specify up to five interfaces or interface lists. When you specify the to keyword in interface-type interface-number1 to interface-type interface-number2, the interfaces before and after the to keyword must be on the same interface card or subcard, and the interface number before to must be no greater than the one after to.

Description

Use the interface range name name interface interface-list command to create an interface range, configure a name for the interface range, add interfaces to the interface range, and enter the interface range view.

Use the interface range name command without the interface keyword to enter the view of an interface range with the specified name.

Use undo interface range name to delete the interface range with the specified name.

You can use this command to assign a name to an interface range and can specify this name rather than the interface range to enter the interface range view.

You can use the display current-configuration | include interface range command to view the member interfaces of an interface range.

In interface range view, only the commands supported by the first interface are available. To view the commands supported by the first interface in the interface range, enter the interface range view and enter ? at the command line interface prompt.

To verify the configuration of the first interface in the interface range, execute the display this command in interface range view.

Failure of applying a command on one member interface does not affect the application of the command on the other member interfaces. If applying a command on one member interface fails, the system displays an error message and continues with the next member interface.

When you bulk configure interfaces, follow these guidelines:

- Do not assign an aggregate interface and any of its member interfaces to an interface range at the same time. Some commands, after being executed on both an aggregate interface and its member interfaces, can break up the aggregation.

- No limit is set on the maximum number of interfaces in an interface range. The more interfaces in an interface range, the longer the command execution time.

Examples

# Add GigabitEthernet 1/0/1 to GigabitEthernet 1/0/12 to interface range named myEthPort, and enter the interface range view.

<Sysname> system-view

[Sysname] interface range name myEthPort interface GigabitEthernet1/0/1 to GigabitEthernet1/0/12

[Sysname-if-range-myEthPort]

# Enter the view of interface range named myEthPort.

<Sysname> system-view

[Sysname] interface range name myEthPort

[Sysname-if-range-myEthPort]

Modified feature: Displaying the remaining power of the IRF fabric

Feature change description

Before modification: The system displays only the remaining power of the master device.

After modification: The system displays the remaining power of each IRF member device.

Command changes

    - Modified command: display poe interface

Syntax

display poe interface [ | { begin | exclude | include } regular-expression ]

View

Any view

Change description

Before modification: The PSE field is not displayed. The system displays only the remaining power of the master device.

After modification: The PSE field is displayed. The system displays the remaining power of each IRF member device.

Examples

# Display the power supplying state of all PoE interfaces.

<Sysname> display poe interface

Interface  Status   Priority  CurPower  Operating    IEEE   Detection

(W)        Status       Class  Status

PSE : 4

GE1/0/1      enabled  high     3.6      on           0      delivering-power

GE1/0/2      enabled  low      0.0      off          0      searching

GE1/0/3      enabled  low      0.0      off          0      searching

GE1/0/4      enabled  low      0.0      off          0      searching

GE1/0/5      enabled  low      0.0      off          0      searching

GE1/0/6      enabled  low      0.0      off          0      searching

GE1/0/7      enabled  low      0.0      off          0      searching

GE1/0/8      enabled  low      0.0      off          0      searching

……

GE1/0/23     enabled  low      0.0      off          0      searching

GE1/0/24     enabled  low      0.0      off          0      searching

---  1 port(s) on,    3.6 (W) consumed,   367.4 (W) remaining ---

PSE : 7

GE2/0/1      enabled  high     7.6      on           0      delivering-power

GE2/0/2      enabled  low      0.0      off          0      searching

GE2/0/3      enabled  low      0.0      off          0      searching

GE2/0/4      enabled  low      0.0      off          0      searching

GE2/0/5      enabled  low      0.0      off          0      searching

GE2/0/6      enabled  low      0.0      off          0      searching

GE2/0/7      enabled  low      0.0      off          0      searching

GE2/0/8      enabled  low      0.0      off          0      searching

……

GE2/0/23     enabled  low      0.0      off          0      searching

GE2/0/24     enabled  low      0.0      off          0      searching

---  1 port(s) on,    7.6 (W) consumed,   362.4 (W) remaining ---

    - Modified command: display poe interface power

Syntax

display poe interface power [ | { begin | exclude | include } regular-expression ]

View

Any view

Change description

Before modification: The PSE field is not displayed. The system displays only the remaining power of the master device.

After modification: The PSE field is displayed. The system displays the remaining power of each IRF member device.

Examples

# Display power information for all PoE interfaces.

<Sysname> display poe interface power

Interface   CurPower  PeakPower   MaxPower  PD Description

(W)       (W)         (W)

PSE : 4

GE1/0/1     0.0       0.0         15.4

GE1/0/2     0.0       0.0         15.4

GE1/0/3     0.0       0.0         15.4

GE1/0/4     0.0       0.0         15.4

GE1/0/5     0.0       0.0         15.4

GE1/0/6     0.0       0.0         15.4

GE1/0/7     0.0       0.0         15.4

GE1/0/8     0.0       0.0         15.4

……

GE1/0/23    0.0       0.0         15.4

GE1/0/24    0.0       0.0         15.4

---  0 port(s) on,    0.0 (W) consumed,   370.0 (W) remaining ---

PSE : 7

GE2/0/1     0.0       0.0         15.4

GE2/0/2     0.0       0.0         15.4

GE2/0/3     0.0       0.0         15.4

GE2/0/4     0.0       0.0         15.4

GE2/0/5     0.0       0.0         15.4

GE2/0/6     0.0       0.0         15.4

GE2/0/7     0.0       0.0         15.4

GE2/0/8     0.0       0.0         15.4

……

GE2/0/23    0.0       0.0         15.4

GE2/0/24    0.0       0.0         15.4

---  0 port(s) on,    0.0 (W) consumed,   370.0 (W) remaining ---

Modified feature: NTP

Feature change description

Added NTP version 4.

Command changes

Modified command: ntp-service broadcast-server

Syntax

ntp-service broadcast-server [ authentication-keyid keyid | version number ] *

View

Layer 3 Ethernet port view, VLAN interface view

Change description

Before modification: The version number option is in the range of 1 to 3.

After modification: The version number option is in the range of 1 to 4.

Modified command: ntp-service multicast-server

Syntax

ntp-service multicast-server [ ip-address ] [ authentication-keyid keyid | ttl ttl-number | version number ] *

View

Layer 3 Ethernet port view, VLAN interface view

Change description

Before modification: The version number option is in the range of 1 to 3.

After modification: The version number option is in the range of 1 to 4.

Modified command: ntp-service unicast-peer

Syntax

ntp-service unicast-peer { ip-address | peer-name } [ authentication-keyid keyid | priority | source-interface interface-type interface-number | version number ] *

View

System view

Change description

Before modification: The version number option is in the range of 1 to 3.

After modification: The version number option is in the range of 1 to 4.

Modified command: ntp-service unicast-server

Syntax

ntp-service unicast-server { ip-address | server-name } [ authentication-keyid keyid | priority | source-interface interface-type interface-number | version number ] *

View

System view

Change description

Before modification: The version number option is in the range of 1 to 3.

After modification: The version number option is in the range of 1 to 4.

Modified feature: Setting the IRF link down report delay

Feature change description

Changed the value range and the default value of the IRF link down report delay.

Command changes

Modified command: irf link-delay

Old syntax

irf link-delay interval

New syntax

irf link-delay interval

View

System view

Change description

Before modification: The value range (in milliseconds) for the interval argument is 200 to 2000. By default, IRF link down events are immediately reported to the upper layer.

After modification: The value range (in milliseconds) for the interval argument is 0 to 30000. By default, IRF link down events are reported 4 seconds later after their occurrence.

# A5120EI-CMW520-F2212P02

This release has the following changes:

New feature: Configuring LLDP to advertise a specific voice VLAN

New feature: Configuring LLDP to advertise a specific voice VLAN

Voice VLAN advertisement through LLDP is available only for LLDP-enabled IP phones. If CDP-compatibility is enabled, this feature is also available for CDP-enabled IP phones. For more information about LLDP, CDP compatibility, and voice VLANs, see Layer 2—LAN Switching Configuration Guide.

  - Configuration guidelines

Use this feature in one of the following scenarios:

Decrease the voice VLAN processing delay in an IRF fabric.

By default, if the voice VLAN feature is configured on an LLDP-enabled port, LLDP advertises this voice VLAN to the IP hone connected to the port. When a packet arrives on the port, the switch compares the source MAC address against its voice device OUI list. If a match is found, the switch learns the MAC address in the voice VLAN, and promotes the forwarding priority for the packet. Because this process is completed in software, in an IRF fabric, MAC address learning and synchronization of the learned MAC address entry to all member devices introduces an undesirable delay. Directly specifying the voice VLAN to be advertised by LLDP enables the IRF fabric to learn and synchronize MAC address entries faster in hardware.

Avoid configuring the voice VLAN function on a port.

Figure 1 shows the procedure of voice VLAN advertisement through LLDP.

Voice VLAN advertisement through LLDP

With the received voice VLAN information, the IP phone automatically completes the voice VLAN configuration, including the voice VLAN ID, tagging status, and priority. This voice VLAN can be the voice VLAN directly specified for LLDP advertisement, the voice VLAN configured on the port, or the voice VLAN assigned by a server, depending on your configuration.

To identify the voice VLAN advertised by LLDP, execute the display lldp local-information command, and examine the MED information fields in the command output.

  - Configuration procedure

To configure LLDP to advertise a specific voice VLAN:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter interface view or port group view. | Enter Layer 2 Ethernet interface view: interface interface-type interface-number Enter port group view: port-group manual port-group-name | Use one of the commands. |
| ##### Configure LLDP to advertise a specific voice VLAN. | lldp voice-vlan vlan-id | By default, LLDP advertises the voice VLAN configured on the port. |

  - Dynamically advertising server-assigned VLANs through LLDP

Dynamic advertisement of server-assigned VLANs through LLDP must work with 802.1X or MAC authentication, and is available only for LLDP-enabled IP phones. If 802.1X authentication is used, make sure the IP phones also support 802.1X authentication.

To implement this function for an IP phone, perform the following configuration tasks:

Enable LLDP globally and on the port connected to the IP phone.

Configure 802.1X or MAC authentication to make sure the IP phone can pass security authentication. For more information about 802.1X authentication, MAC authentication, and VLAN assignment by servers, see Security Configuration Guide.

Configure VLAN authorization for the IP phone on the authentication server.

After the IP phone passes authentication, LLDP advertises the server-assigned VLAN in the Network Policy TLV to the IP phone. The IP phone will send its traffic tagged with the assigned VLAN.

Command reference

    - lldp voice-vlan

Syntax

lldp voice-vlan vlan-id

undo lldp voice-vlan

View

Layer 2 Ethernet interface view, port group view

Default level

2: System level

Parameters

vlan-id: Specifies a voice VLAN by its ID, which ranges from 1 to 4094.

Description

Use lldp voice-vlan vlan-id to configure a port to advertise a specific voice VLAN ID to the connected IP phone through LLDP. If CDP compatibility is enabled, LLDP also includes the specified voice VLAN ID in the CDP packets sent to the IP phone.

Use undo lldp voice-vlan to restore the default.

By default, if a voice VLAN is configured on an LLDP-enabled port, LLDP advertises this voice VLAN to the IP phone connected to the port.

Examples

# Configure port GigabitEthernet 1/0/1 to advertise voice VLAN 4094.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] lldp voice-vlan 4094

# A5120EI-CMW520-F2212

This release has the following changes:

New feature: Setting the DSCP value for multiple types of protocol packets

Modified feature: Password configuration and display

Modified feature: Task ID for IPv6 socket display

Removed feature: Local user password display

New feature: Setting the DSCP value for multiple types of protocol packets

A field in an IPv4 or IPv6 header contains 8 bits and is used to identify the service type of an IP packet. In an IPv4 packet, this field is called “Type of Service (ToS).” In an IPv6 packet, this field is called “Traffic class.” According to RFC 2474, the ToS field is redefined as the differentiated services (DS) field, where a DSCP value is represented by the first six bits (0 to 5) and is in the range 0 to 63. The remaining two bits (6 and 7) are reserved. When a packet is being transmitted, the network devices can identify its DSCP value, and determines the transmission priority of the packet according to the DSCP value.

This release allows you to set the DSCP value for multiple types of protocol packets, including RADIUS, SSH, HTTP, Telnet, FTP, TFTP, NTP, NQA, SNMP, ICMP, IGMP Snooping, MLD Snooping, DHCP, DNS, IPv6 DNS, and DHCPv6.

When you configure the DSCP value for some types of protocol packets, you should specify the ToS field value rather than the DSCP value. Because the DSCP field is the first 6 bits of the ToS field, each four continuous ToS field values, starting from 0, correspond to one DSCP value. An easier way to convert the DSCP value to the ToS value is to multiply the expected DSCP value by four to get the ToS field value.

Setting the DSCP value for DHCPv6 protocol packets

To set the DSCP value for DHCPv6 protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for DHCPv6 protocol packets sent by the DHCPv6 servers and DHCPv6 relay agents. | ipv6 dhcp dscp dscp-value | Optional. By default, the DSCP value in DHCPv6 protocol packets sent by the DHCPv6 servers and DHCPv6 relay agents is 56. |
| ##### Set the DSCP value for DHCPv6 protocol packets sent by the DHCPv6 clients. | ipv6 dhcp client dscp dscp-value | Optional. By default, the DSCP value in DHCPv6 protocol packets sent by the DHCPv6 clients is 56. |

Setting the DSCP value for DHCP protocol packets

To set the DSCP value for DHCP protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for DHCP protocol packets sent by the DHCP servers and DHCP relay agents. | dhcp dscp dscp-value | Optional. By default, the DSCP value in DHCP protocol packets sent by the DHCP servers and DHCP relay agents is 56. |
| ##### Set the DSCP value for DHCP protocol packets sent by the DHCP clients. | dhcp client dscp dscp-value | Optional. By default, the DSCP value in DHCP protocol packets sent by the DHCP clients is 56. |

Setting the DSCP value for DNS protocol packets

To set the DSCP value for DNS protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for DNS protocol packets transmitted. | dns dscp dscp-value | Optional. By default, the DSCP value in DNS protocol packets transmitted is 0. |

Setting the DSCP value for FTP and TFTP protocol packets

To set the DSCP value for FTP and TFTP protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for protocol packets sent by the IPv4 FTP clients. | ftp client dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 FTP clients is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 FTP clients. | ftp client ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 FTP clients is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv4 FTP servers. | ftp server dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 FTP servers is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv4 TFTP clients. | tftp client dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 TFTP clients is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 TFTP clients. | tftp client ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 TFTP clients is 0. |

Setting the DSCP value for HTTP protocol packets

To set the DSCP value for HTTP protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for IPv4 HTTP protocol packets transmitted. | ip http dscp dscp-value | Optional. By default, the DSCP value in IPv4 HTTP protocol packets transmitted is 16. |
| ##### Set the DSCP value for IPv6 HTTP protocol packets transmitted. | ipv6 http dscp dscp-value | Optional. By default, the DSCP value in IPv6 HTTP protocol packets transmitted is 0. |

Setting the DSCP value for IGMP protocol packets sent by IGMP snooping

This configuration allows you to set the DSCP value for IGMP protocol packets sent by IGMP snooping.

To set the DSCP value for IGMP protocol packets transmitted:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter IGMP-snooping view. | igmp-snooping | N/A |
| ##### Set the DSCP value for IGMP protocol packets transmitted. | dscp dscp-value | Required. By default, the DSCP value in IGMP protocol packets transmitted is 48. |

|  | NOTE: This configuration applies to only the IGMP messages that the local switch generates rather than those forwarded ones. |
| --- | --- |

Setting the DSCP value for IPv6 DNS protocol packets

To set the DSCP value for IPv6 DNS protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for IPv6 DNS protocol packets transmitted. | dns ipv6 dscp dscp-value | Optional. By default, the DSCP value in IPv6 DNS protocol packets transmitted is 0. |

Setting the DSCP value for MLD protocol packets sent by MLD snooping

This configuration allows you to set the DSCP value for MLD protocol packets sent by MLD snooping.

To set the DSCP value for MLD protocol packets transmitted:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter MLD-snooping view. | mld-snooping | N/A |
| ##### Set the DSCP value for MLD protocol packets transmitted. | dscp dscp-value | Required. By default, the DSCP value in MLD protocol packets transmitted is 48. |

|  | NOTE: This configuration applies to only the MLD messages that the local switch generates rather than those forwarded ones. |
| --- | --- |

Setting the ToS value for packets sent by the TCP listening service on the NQA server

To set the ToS value for packets sent by the TCP listening service on the NQA server:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the ToS value for packets sent by the TCP listening service on the NQA server. | nqa server tcp-connect tos tos | Optional. By default, the ToS value in the packets sent by the TCP listening service on the NQA server is 0. |

Setting the ToS value for packets sent by the UDP listening service on the NQA server

To set the ToS value for packets sent by the UDP listening service on the NQA server:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the ToS value for packets sent by the UDP listening service on the NQA server. | nqa server udp-echo tos tos | Optional. By default, the ToS value in the packets sent by the UDP listening service on the NQA server is 0. |

Setting the ToS value for NQA probe packets

To set the ToS value for NQA probe packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter NQA operation view. | nqa entry admin-name operation-tag | N/A |
| ##### Specify the DHCP type and enter its view. | type dhcp | Required. |
| ##### Set the ToS value for NQA probe packets. | tos value | Optional. By default, the ToS value in NQA probe packets is 0. |

Setting the DSCP value for NTP protocol packets

To set the DSCP value for NTP protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for NTP protocol packets. | ntp-service dscp dscp-value | Optional. By default, the DSCP value in NTP protocol packets is 16. |

Setting the DSCP value for RADIUS protocol packets

To set the DSCP value for RADIUS protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for IPv4 RADIUS protocol packets. | radius dscp dscp-value | Optional. By default, the DSCP value in IPv4 RADIUS protocol packets is 0. |
| ##### Set the DSCP value for IPv6 RADIUS protocol packets. | radius ipv6 dscp dscp-value | Optional. By default, the DSCP value in IPv6 RADIUS protocol packets is 0. |

Setting the DSCP value for SNMP trap packets

To set the DSCP value for SNMP trap packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for SNMP trap packets. | snmp-agent target-host trap address udp-domain { ip-address ¦ ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] params securityname security-string [ v1 ¦ v2c ¦ v3 [ authentication ¦ privacy ] ] | Optional. By default, the DSCP value in SNMP trap packets is 0. |

Setting the DSCP value for SNMP response packets

To set the DSCP value for SNMP response packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for SNMP response packets. | snmp-agent packet response dscp dscp-value | Optional. By default, the DSCP value in SNMP response packets is 0. |

Setting the DSCP value for SSH protocol packets

To set the DSCP value for SSH protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for protocol packets sent by the IPv4 SSH servers. | ssh server dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 SSH servers is 16. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 SSH servers. | ssh server ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 SSH servers is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv4 SSH clients. | ssh client dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 SSH clients is 16. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 SSH clients. | ssh client ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 SSH clients is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv4 SFTP clients. | sftp client dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 SFTP clients is 16. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 SFTP clients. | sftp client ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 SFTP clients is 8. |

Setting the DSCP value for Telnet protocol packets

To set the DSCP value for Telnet protocol packets:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for protocol packets sent by the IPv4 Telnet clients. | telnet client dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 Telnet clients is 16. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 Telnet clients. | telnet client ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 Telnet clients is 0. |
| ##### Set the DSCP value for protocol packets sent by the IPv4 Telnet servers. | telnet server dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv4 Telnet servers is 48. |
| ##### Set the DSCP value for protocol packets sent by the IPv6 Telnet servers. | telnet server ipv6 dscp dscp-value | Optional. By default, the DSCP value in protocol packets sent by the IPv6 Telnet servers is 0. |

Setting the DSCP value for the protocol packets sent to the log host

To set the DSCP value for the protocol packets sent to the log host:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Set the DSCP value for the protocol packets sent to the log host. | info-center loghost { host-ipv4-address ¦ ipv6 host-ipv6-address } [ port port-number ] [ dscp dscp-value ] [ channel { channel-number ¦ channel-name } ¦ facility local-number ] * | Optional. By default, the DSCP value in the protocol packets sent to the log host is 0. |

Added commands

dhcp client dscp

Syntax

dhcp client dscp dscp-value

undo dhcp client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the DHCP protocol packets transmitted, which ranges from 0 to 63.

Description

Use the dhcp client dscp command to set the DSCP value for DHCP protocol packets sent by the DHCP clients.

Use the undo dhcp client dscp command to restore the default.

By default, the DSCP value in DHCP protocol packets sent by the DHCP clients is 56.

Examples

# Set the DSCP value to 30 for DHCP protocol packets sent by the DHCP clients.

<Sysname> system-view

[Sysname] dhcp client dscp 30

dhcp dscp

Syntax

dhcp dscp dscp-value

undo dhcp dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the DHCP protocol packets transmitted, which ranges from 0 to 63.

Description

Use the dhcp dscp command to set the DSCP value for DHCP protocol packets sent by the DHCP servers and DHCP relay agents.

Use the undo dhcp dscp command to restore the default.

By default, the DSCP value in DHCP protocol packets sent by the DHCP servers and DHCP relay agents is 56.

Examples

# Set the DSCP value to 30 for DHCP protocol packets transmitted.

<Sysname> system-view

[Sysname] dhcp dscp 30

dns dscp

Syntax

dns dscp dscp-value

undo dns dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the DNS protocol packets transmitted, which ranges from 0 to 63.

Description

Use the dns dscp command to set the DSCP value for DNS protocol packets transmitted.

Use the undo dns dscp command to restore the default.

By default, the DSCP value in DNS protocol packets transmitted is 0.

Examples

# Set the DSCP value to 30 for DNS protocol packets transmitted.

<Sysname> system-view

[Sysname] dns dscp 30

dns ipv6 dscp

Syntax

dns ipv6 dscp dscp-value

undo dns ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the IPv6 DNS protocol packets transmitted, which ranges from 0 to 63.

Description

Use the dns ipv6 dscp command to set the DSCP value for IPv6 DNS protocol packets transmitted.

Use the undo dns ipv6 dscp command to restore the default.

By default, the DSCP value in IPv6 DNS protocol packets transmitted is 0.

Examples

# Set the DSCP value to 30 for IPv6 DNS protocol packets transmitted.

<Sysname> system-view

[Sysname] dns ipv6 dscp 30

dscp (IGMP-Snooping view)

Syntax

dscp dscp-value

undo dscp

View

IGMP-snooping view

Default level

2: System level

Parameters

dscp-value: DSCP value in the IGMP protocol packets transmitted, which ranges from 0 to 63.

Description

Use the dscp command to set the DSCP value for IGMP protocol packets transmitted.

Use the undo dscp command to restore the default.

By default, the DSCP value in IGMP protocol packets transmitted is 48.

Examples

# Set the DSCP value to 63 for IGMP protocol packets transmitted.

<Sysname> system-view

[Sysname] igmp-snooping

[Sysname-igmp-snooping] dscp 63

dscp (MLD-Snooping view)

Syntax

dscp dscp-value

undo dscp

View

MLD-snooping view

Default level

2: System level

Parameters

dscp-value: DSCP value in the MLD protocol packets transmitted, which ranges from 0 to 63.

Description

Use the dscp command to set the DSCP value for MLD protocol packets transmitted.

Use the undo dscp command to restore the default.

By default, the DSCP value in MLD protocol packets transmitted is 48.

Examples

# Set the DSCP value to 63 for MLD protocol packets transmitted by MLD-snooping.

<Sysname> system-view

[Sysname] mld-snooping

[Sysname-mld-snooping] dscp 63

ftp client dscp

Syntax

ftp client dscp dscp-value

undo ftp client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ftp client dscp command to set the DSCP value for FTP protocol packets sent by the FTP clients.

Use the undo ftp client dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the FTP clients is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the FTP clients.

<Sysname> system-view

[Sysname] ftp client dscp 30

ftp client ipv6 dscp

Syntax

ftp client ipv6 dscp dscp-value

undo ftp client ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ftp client ipv6 dscp command to set the DSCP value for FTP protocol packets sent by the IPv6 FTP clients.

Use the undo ftp client ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 FTP clients is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv6 FTP clients.

<Sysname> system-view

[Sysname] ftp client ipv6 dscp 30

ftp server dscp

Syntax

ftp server dscp dscp-value

undo ftp server dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ftp server dscp command to set the DSCP value for FTP protocol packets sent by the FTP servers.

Use the undo ftp server dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the FTP servers is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the FTP servers.

<Sysname> system-view

[Sysname] ftp server dscp 30

ip http dscp

Syntax

ip http dscp dscp-value

undo ip http dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ip http dscp command to set the DSCP value for HTTP protocol packets transmitted.

Use the undo ip http dscp command to restore the default.

By default, the DSCP value in HTTP protocol packets transmitted is 16.

Examples

# Set the DSCP value to 30 for HTTP protocol packets transmitted.

<Sysname> system-view

[Sysname] ip http dscp 30

ipv6 dhcp client dscp

Syntax

ipv6 dhcp client dscp dscp-value

undo ipv6 dhcp client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the DHCPv6 protocol packets transmitted, which ranges from 0 to 63.

Description

Use the ipv6 dhcp client dscp command to set the DSCP value for DHCPv6 protocol packets sent by the DHCPv6 clients.

Use the undo ipv6 dhcp client dscp command to restore the default.

By default, the DSCP value in DHCPv6 protocol packets sent by the DHCPv6 clients is 56.

Examples

# Set the DSCP value to 30 for DHCPv6 protocol packets sent by the DHCPv6 clients.

<Sysname> system-view

[Sysname] ipv6 dhcp client dscp 30

ipv6 dhcp dscp

Syntax

ipv6 dhcp dscp dscp-value

undo ipv6 dhcp dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the DHCPv6 protocol packets transmitted, which ranges from 0 to 63.

Description

Use the ipv6 dhcp dscp command to set the DSCP value for DHCPv6 protocol packets sent by the DHCPv6 servers and DHCPv6 relay agents.

Use the undo ipv6 dhcp dscp command to restore the default.

By default, the DSCP value in DHCPv6 protocol packets sent by the DHCPv6 servers and DHCPv6 relay agents is 56.

Examples

# Set the DSCP value to 30 for the DHCPv6 protocol packets sent by the DHCPv6 servers and DHCPv6 relay agents.

<Sysname> system-view

[Sysname] ipv6 dhcp dscp 30

ipv6 http dscp

Syntax

ipv6 http dscp dscp-value

undo ipv6 http dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ipv6 http dscp command to set the DSCP value for IPv6 HTTP protocol packets transmitted.

Use the undo ipv6 http dscp command to restore the default.

By default, the DSCP value in IPv6 HTTP protocol packets transmitted is 0.

Examples

# Set the DSCP value to 30 for IPv6 HTTP protocol packets transmitted.

<Sysname> system-view

[Sysname] ipv6 http dscp 30

nqa server tcp-connect tos

Syntax

nqa server tcp-connect tos tos

undo nqa server tcp-connect tos

View

System view

Default level

2: System level

Parameters

tos: Type of Service (ToS) field value in the protocol packets sent by the TCP listening service on the NQA server. This argument ranges from 0 to 255.

Description

Use the nqa server tcp-connect tos command to set the ToS value for packets sent by the TCP listening service on the NQA server.

Use the undo nqa server tcp-connect tos command to restore the default.

By default, the ToS value in the packets sent by the TCP listening service on the NQA server is 0.

Examples

# Set the ToS value to 30 for packets sent by the TCP listening service on the NQA server.

<Sysname> system-view

[Sysname] nqa server tcp-connect tos 30

nqa server udp-echo tos

Syntax

nqa server udp-echo tos tos

undo nqa server udp-echo tos

View

System view

Default level

2: System level

Parameters

tos: Type of Service (ToS) field value in the protocol packets sent by the UDP listening service on the NQA server. This argument ranges from 0 to 255.

Description

Use the nqa server udp-echo tos command to set the ToS value for packets sent by the UDP listening service on the NQA server.

Use the undo nqa server udp-echo tos command to restore the default.

By default, the ToS value in the packets sent by the UDP listening service on the NQA server is 0.

Examples

# Set the ToS value to 30 for packets sent by the UDP listening service on the NQA server.

<Sysname> system-view

[Sysname] nqa server udp-echo tos 30

ntp-service dscp

Syntax

ntp-service dscp dscp-value

undo ntp-service dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ntp-service dscp command to set the DSCP value for NTP protocol packets.

Use the undo ntp-service dscp command to restore the default.

By default, the DSCP value in NTP protocol packets is 16.

Examples

# Set the DSCP value to 30 for NTP protocol packets.

<Sysname> system-view

[Sysname] ntp-service dscp 30

radius dscp

Syntax

radius dscp dscp-value

undo radius dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the radius dscp command to set the DSCP value for IPv4 RADIUS protocol packets.

Use the undo radius dscp command to restore the default.

By default, the DSCP value in IPv4 RADIUS protocol packets is 0.

Examples

# Set the DSCP value to 6 for IPv4 RADIUS protocol packets.

<Sysname> system-view

[Sysname] radius dscp 6

radius ipv6 dscp

Syntax

radius ipv6 dscp dscp-value

undo radius ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the radius ipv6 dscp command to set the DSCP value for IPv6 RADIUS protocol packets.

Use the undo radius ipv6 dscp command to restore the default.

By default, the DSCP value in IPv6 RADIUS protocol packets is 0.

Examples

# Set the DSCP value to 6 for IPv6 RADIUS protocol packets.

<Sysname> system-view

[Sysname] radius ipv6 dscp 6

sftp client dscp

Syntax

sftp client dscp dscp-value

undo sftp client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the sftp client dscp command to set the DSCP value for protocol packets sent by the IPv4 SFTP clients.

Use the undo sftp client dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv4 SFTP clients is 16.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv4 SFTP clients.

<Sysname> system-view

[Sysname] sftp client dscp 30

sftp client ipv6 dscp

Syntax

sftp client ipv6 dscp dscp-value

undo sftp client ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the sftp client ipv6 dscp command to set the DSCP value for protocol packets sent by the IPv6 SFTP clients.

Use the undo sftp client ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 SFTP clients is 8.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv6 SFTP clients.

<Sysname> system-view

[Sysname] sftp client ipv6 dscp 30

snmp-agent packet response dscp

Syntax

snmp-agent packet response dscp dscp-value

undo snmp-agent packet response dscp

View

System view

Default level

3: Manage level

Parameters

dscp-value: DSCP value in the SNMP response packets, which ranges from 0 to 63.

Description

Use the snmp-agent packet response dscp command to set the DSCP value for SNMP response packets.

Use the undo snmp-agent packet response dscp command to restore the default.

By default, the DSCP value in SNMP response packets is 0.

Examples

# Set the DSCP value to 45 for SNMP response packets.

<Sysname> system-view

[Sysname] snmp-agent packet response dscp 45

ssh client dscp

Syntax

ssh client dscp dscp-value

undo ssh client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ssh client dscp command to set the DSCP value for protocol packets sent by the IPv4 SSH clients.

Use the undo ssh client dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv4 SSH clients is 16.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv4 SSH clients.

<Sysname> system-view

[Sysname] ssh client dscp 30

ssh client ipv6 dscp

Syntax

ssh client ipv6 dscp dscp-value

undo ssh client ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ssh client ipv6 dscp command to set the DSCP value for protocol packets sent by the IPv6 SSH clients.

Use the undo ssh client ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 SSH clients is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv6 SSH clients.

<Sysname> system-view

[Sysname] ssh client ipv6 dscp 30

ssh server dscp

Syntax

ssh server dscp dscp-value

undo ssh server dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ssh server dscp command to set the DSCP value for protocol packets sent by the IPv4 SSH servers.

Use the undo ssh server dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv4 SSH servers is 16.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv4 SSH servers.

<Sysname> system-view

[Sysname] ssh server dscp 30

ssh server ipv6 dscp

Syntax

ssh server ipv6 dscp dscp-value

undo ssh server ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the ssh server ipv6 dscp command to set the DSCP value for protocol packets sent by the IPv6 SSH servers.

Use the undo ssh server ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 SSH servers is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv6 SSH servers.

<Sysname> system-view

[Sysname] ssh server ipv6 dscp 30

telnet client dscp

Syntax

telnet client dscp dscp-value

undo telnet client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the telnet client dscp command to set the DSCP value for protocol packets sent by the Telnet clients.

Use the undo telnet client dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the Telnet clients is 16.

Examples

# Set the DSCP value to 30 for protocol packets sent by the Telnet clients.

<Sysname> system-view

[Sysname] telnet client dscp 30

telnet client ipv6 dscp

Syntax

telnet client ipv6 dscp dscp-value

undo telnet client ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the telnet client ipv6 dscp command to set the DSCP value for protocol packets sent by the IPv6 Telnet clients.

Use the undo telnet client ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 Telnet clients is 0.

Examples

# Set the DSCP value to 0 for protocol packets sent by the IPv6 Telnet clients.

<Sysname> system-view

[Sysname] telnet client ipv6 dscp 30

telnet server dscp

Syntax

telnet server dscp dscp-value

undo telnet server dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the telnet server dscp command to set the DSCP value for protocol packets sent by the Telnet servers.

Use the undo telnet server dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the Telnet servers is 48.

Examples

# Set the DSCP value to 30 for protocol packets sent by the Telnet servers.

<Sysname> system-view

[Sysname] telnet server dscp 30

telnet server ipv6 dscp

Syntax

telnet server ipv6 dscp dscp-value

undo telnet server ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the telnet server ipv6 dscp command to set the DSCP value for protocol packets sent by the IPv6 Telnet servers.

Use the undo telnet server ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 Telnet servers is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv6 Telnet servers.

<Sysname> system-view

[Sysname] telnet server ipv6 dscp 30

tftp client dscp

Syntax

tftp client dscp dscp-value

undo tftp client dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the tftp client dscp command to set the DSCP value for protocol packets sent by the TFTP clients.

Use the undo tftp client dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the TFTP clients is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the TFTP clients.

<Sysname> system-view

[Sysname] tftp client dscp 30

tftp client ipv6 dscp

Syntax

tftp client ipv6 dscp dscp-value

undo tftp client ipv6 dscp

View

System view

Default level

2: System level

Parameters

dscp-value: DSCP value in the protocol packets, which ranges from 0 to 63.

Description

Use the tftp client ipv6 dscp command to set the DSCP value for protocol packets sent by the IPv6 TFTP clients.

Use the undo tftp client ipv6 dscp command to restore the default.

By default, the DSCP value in protocol packets sent by the IPv6 TFTP clients is 0.

Examples

# Set the DSCP value to 30 for protocol packets sent by the IPv6 TFTP clients.

<Sysname> system-view

[Sysname] tftp client ipv6 dscp 30

tos (DHCP operation type view)

Syntax

tos value

undo tos

View

DHCP operation type view

Default level

2: System level

Parameters

value: ToS value in the NQA probe packets, which ranges from 0 to 255.

Description

Use the tos command to set the ToS value for NQA probe packets.

Use the undo tos command to restore the default.

By default, the ToS value in NQA probe packets is 0.

Examples

# Set the ToS value to 1 for NQA probe packets.

<Sysname> system-view

[Sysname] nqa entry admin test

[Sysname-nqa-admin-test] type dhcp

[Sysname-nqa-admin-test-dhcp] tos 1

Modified commands

Modified command: info-center loghost

Old syntax

info-center loghost { host-ipv4-address | ipv6 host-ipv6-address } [ port port-number ] [ channel { channel-number | channel-name } | facility local-number ] *

New syntax

info-center loghost { host-ipv4-address | ipv6 host-ipv6-address } [ port port-number ] [ dscp dscp-value ] [ channel { channel-number | channel-name } | facility local-number ] *

Views

System view

Change description

The dscp dscp-value option is added.

dscp dscp-value: Sets the DSCP value in the packets sent to the log host, which ranges from 0 to 63 and defaults to 0.

Modified command: ping ipv6

Old syntax

ping ipv6 [ -a source-ipv6 | -c count | -m interval | -s packet-size | -t timeout ] * host [ -i interface-type interface-number ]

New Syntax

ping ipv6 [ -a source-ipv6 | -c count | -m interval | -s packet-size | -t timeout | -tos tos ] * host [ -i interface-type interface-number ]

Views

Any view

Change description

The -tos tos option is added.

-tos tos: Sets the Traffic Class field value in the ICMPv6 echo request. The tos argument ranges from 0 to 255 and defaults to 0.

Modified command: snmp-agent target-host

Old syntax

snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } [ udp-port port-number ] params securityname security-string [ v1 | v2c | v3 [ authentication | privacy ] ]

New Syntax

snmp-agent target-host trap address udp-domain { ip-address | ipv6 ipv6-address } [ udp-port port-number ] [ dscp dscp-value ] params securityname security-string [ v1 | v2c | v3 [ authentication | privacy ] ]

Views

System view

Change description

The dscp dscp-value option is added.

dscp dscp-value: Sets the DSCP value for the SNMP traps, which ranges from 0 to 63 and defaults to 0.

Modified command: tracert

Old syntax

tracert [ -a source-ip | -f first-ttl | -m max-ttl | -p port | -q packet-number | -w timeout ] * host

New Syntax

tracert [ -a source-ip | -f first-ttl | -m max-ttl | -p port | -q packet-number | -w timeout | -tos tos ] * host

Views

Any view

Change description

The -tos tos option is added.

-tos tos: Sets the ToS field value in the tracert request. The tos argument ranges from 0 to 255 and defaults to 0.

Modified command: tracert ipv6

Old syntax

tracert ipv6 [ -f first-ttl | -m max-ttl | -p port | -q packet-number | -w timeout ] * host

New Syntax

tracert ipv6 [ -f first-ttl | -m max-ttl | -p port | -q packet-number | -w timeout | -tos tos ] * host

Views

Any view

Change description

The -tos tos option is added.

-tos tos: Sets the Traffic Class field value in the tracert request. The tos argument ranges from 0 to 255 and defaults to 0.

Modified feature: Password configuration and display

Feature change description

Modified password setup and display for password-related security features.

|  | NOTE: To improve security, this release saves all plaintext and ciphertext passwords and keys in cipher text in the configuration file. |
| --- | --- |

Command changes

Modified command: bims-server

Old syntax

bims-server ip ip-address [ port port-number ] sharekey key

New syntax

bims-server ip ip-address [ port port-number ] sharekey [ cipher | simple ] key

View

DHCP address pool view

Parameters

ip ip-address: Specifies an IP address for the BIMS server.

port port-number: Specifies a port number for the BIMS server in the range of 1 to 65534.

cipher: Sets a ciphertext key.

simple: Sets a plaintext key.

key: Specifies the key string. This argument is case sensitive. If cipher is specified, it must be a ciphertext string of 1 to 53 characters. If simple is specified, it must be a string of 1 to 16 characters. If neither simple nor cipher is specified, you set a plaintext key.

Change description

Before modification: You can only set a plaintext shared key.

After modification: You can set a plaintext or a ciphertext shared key. A ciphertext shared key can comprise 1 to 53 characters.

Modified command: certificate request mode

Syntax

certificate request mode { auto [ key-length key-length | password { cipher | simple } password ] * | manual }

Views

PKI domain view

Parameters

auto: Requests a certificate in auto mode.

key-length: Length of the RSA keys in bits, in the range of 512 to 2048. It is 1024 bits by default.

cipher: Sets a ciphertext key for certificate revocation.

simple: Sets a plaintext key for certificate revocation.

password: Specifies the key. This argument is case sensitive. If simple is specified, it must be a plaintext string of 1 to 31 characters. If cipher is specified, it must be a ciphertext string of 1 to 73 characters.

manual: Requests a certificate in manual mode.

Change description

Before modification: A ciphertext key comprises 1 to 31 characters.

After modification: A ciphertext key comprises 1 to 73 characters.

Modified command: cluster-local-user

Syntax

cluster-local-user user-name [ password { cipher | simple } password ]

Views

Cluster view

Parameters

user-name: Specifies the username for logging onto the cluster member devices through Web. It is a string of 1 to 55 characters.

password: Specifies the password for logging onto the cluster member devices through Web. If this keyword is not specified, you can log in without a password.

cipher: Specifies a ciphertext password.

simple: Specifies a plaintext password.

password: Specifies the password string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 63 characters. If cipher is specified, it must be a ciphertext string of 1 to 117 characters.

Change description

Before modification: If cipher is specified, you can set a plaintext password of 1 to 63 characters, or a ciphertext password of 24 or 88 characters.

After modification: If cipher is specified, you must enter a ciphertext password of 1 to 117 characters.

Modified command: cluster-snmp-agent usm-user v3

Old syntax

cluster-snmp-agent usm-user v3 user-name group-name [ authentication-mode { md5 | sha } auth-password [ privacy-mode des56 priv-password ] ]

New syntax

cluster-snmp-agent usm-user v3 user-name group-name [ authentication-mode { md5 | sha } [ cipher | simple ] auth-password [ privacy-mode des56 [ cipher | simple ] priv-password ] ]

Views

Cluster view

Parameters

user-name: User name, a string of 1 to 32 characters.

group-name: Group name, a string of 1 to 32 characters.

authentication-mode: Specifies the security level to be authentication needed.

md5: Specifies the authentication protocol to be HMAC-MD5-96.

sha: Specifies the authentication protocol to be HMAC-SHA-96.

cipher: Specifies a ciphertext password.

simple: Specifies a plaintext password.

auth-password: Specifies the authentication password string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 16 characters. If cipher is specified, it must be a ciphertext string of 1 to 53 characters. If neither cipher nor simple is specified, you set a plaintext string.

privacy-mode: Specifies the security level to be encrypted.

des56: Specifies the encryption protocol to be DES (data encryption standard).

priv-password: Specifies the privacy password string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 16 characters. If cipher is specified, it must be a ciphertext string of 1 to 53 characters. If neither cipher nor simple is specified, you set a plaintext string.

Change description

Before modification: The cipher and simple keywords are not supported. You can directly enter a plaintext password of 1 to 16 characters or a ciphertext password of 24 characters.

After modification: You can use the cipher keyword to set a ciphertext password of 1 to 53 characters or use the simple keyword to set a plaintext password of 1 to 16 characters. If neither cipher nor simple is specified, you set a plaintext password.

Modified command: dldp authentication-mode

Old syntax

dldp authentication-mode { md5 md5-password | none | simple simple-password }

New syntax

dldp authentication-mode { none | { md5 | simple } password }

View

System view

Parameters

none: Specifies not to perform authentication.

md5: Specifies to perform MD5 authentication and sets the password in plain text or cipher text.

simple: Specifies to perform simple authentication and sets the password in plain text or cipher text.

password: Plain text password, a case-sensitive string of 1 to 16 characters; or a cipher text password, a case-sensitive string of 33 to 53 characters.

Change description

Before modification: You can set only a plaintext password for simple authentication, and a plaintext password or a 24-character ciphertext password for MD5 authentication.

After modification: You can set a plaintext password or a ciphertext password for both simple authentication and MD5 authentication. A ciphertext password comprises 33 to 53 characters.

Modified command: ftp-server

Syntax

ftp-server ip-address [ user-name username password { cipher | simple } password ]

Views

Cluster view

Parameters

ip-address: Specifies the IP address of the FTP server.

username: Specifies the username for logging onto the FTP server, a string of 1 to 32 characters.

cipher: Specifies a ciphertext password.

simple: Specifies a plaintext password.

password: Specifies the password string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 16 characters. If cipher is specified, it must be a ciphertext string of 1 to 53 characters.

Change description

Before modification: If cipher is specified, you can set a plaintext password of 1 to 16 characters, or a ciphertext password of 24 characters.

After modification: If cipher is specified, you must enter a ciphertext password of 1 to 53 characters.

Modified command: key (HWTACACS scheme view)

Syntax

key { accounting | authentication | authorization } [ cipher | simple ] key

Views

HWTACACS scheme view

Parameters

accounting: Sets the shared key for secure HWTACACS accounting communication.

authentication: Sets the shared key for secure HWTACACS authentication communication.

authorization: Sets the shared key for secure HWTACACS authorization communication.

cipher: Sets a ciphertext shared key.

simple: Sets a plaintext shared key.

key: Specifies the shared key string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 255 characters. If cipher is specified, it must be a ciphertext string of 1 to 373 characters. If neither cipher nor simple is specified, you set a plaintext shared key string.

Change description

Before modification: A ciphertext password comprises 1 to 352 characters.

After modification: A ciphertext password comprises 1 to 373 characters.

Modified command: key (RADIUS scheme view)

Syntax

key { accounting | authentication } [ cipher | simple ] key

Views

RADIUS scheme view

Parameters

accounting: Sets the shared key for secure RADIUS accounting communication.

authentication: Sets the shared key for secure RADIUS authentication/authorization communication.

cipher: Sets a ciphertext shared key.

simple: Sets a plaintext shared key.

key: Specifies the shared key string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 64 characters. If cipher is specified, it must be a ciphertext string of 1 to 117 characters. If neither cipher nor simple is specified, you set a plaintext shared key string.

Change description

Before modification: A ciphertext shared key must comprise 12, 24, 32, 44, 64, 76, 88, or 96 characters.

After modification: A ciphertext shared key comprises 1 to 117 characters.

Modified command: mac-authentication user-name-format

Syntax

mac-authentication user-name-format { fixed [ account name ] [ password { cipher | simple } password ] | mac-address [ { with-hyphen | without-hyphen } [ lowercase | uppercase ] ] }

Views

System view

Parameters

fixed: Uses a shared account for all MAC authentication users.

account name: Specifies the username for the shared account. The name takes a case-insensitive string of 1 to 55 characters. If no username is specified, the default name mac applies.

password: Specifies the password for the shared user account:

cipher: Sets a ciphertext password.

simple: Sets a plaintext password.

password: Specifies the password. This argument is case sensitive. If simple is specified, it must be a string of 1 to 63 characters. If cipher is specified, it must be a ciphertext string of 1 to 117 characters.

mac-address: Uses MAC-based user accounts for MAC authentication users. If this option is specified, you must create one user account for each user, and use the MAC address of the user as both the username and password for the account. You can also specify the format of username and password:

- with-hyphen—Hyphenates the MAC address, for example xx-xx-xx-xx-xx-xx.

- without-hyphen—Excludes hyphens from the MAC address, for example, xxxxxxxxxxxx.

- lowercase—Enters letters in lower case.

- uppercase—Capitalizes letters.

Change description

Before modification: If cipher is specified, you can enter a plaintext password of 1 to 63 characters, or a ciphertext password of 24 or 88 characters.

After modification: If cipher is specified, you must enter a ciphertext password of 1 to 117 characters.

Modified command: ntp-service authentication-keyid

Old syntax

ntp-service authentication-keyid keyid authentication-mode md5 value

New syntax

ntp-service authentication-keyid keyid authentication-mode md5 [ cipher | simple ] value

Views

System view

Parameters

keyid: Authentication key ID, which is in the range of 1 to 4294967295.

cipher: Sets a ciphertext key.

simple: Sets a plaintext key. This key will be saved in cipher text for secrecy.

value: Specifies the MD5 authentication key string. This argument is case sensitive. If simple is specified, it must be a plaintext string of 1 to 32 characters. If cipher is specified, it must be a ciphertext string of 1 to 73 characters. If neither cipher nor simple is specified, you set a plaintext key string.

Change description

Before modification: You can only set a plaintext key.

After modification: You can set a plaintext or ciphertext key. A ciphertext key comprises 1 to 73 characters.

Modified command: password (FTP operation type view)

Old syntax

password password

New syntax

password [ cipher | simple ] password

Views

FTP operation type view

Parameters

cipher: Sets a password in cipher text.

simple: Sets a password in plain text.

password: Specifies the password used to log in to the FTP server, a case-sensitive string of 1 to 32 characters in plain text, or 1 to 73 characters in cipher text. If the cipher or simple keyword is not specified, the password is in plain text.

Change description

Before modification: You can only set a plaintext password.

After modification: You can set a plaintext password or a ciphertext password. A ciphertext password comprises 1 to 73 characters.

Modified command: password (local user view)

Syntax

password [ { cipher | simple } password ]

Views

Local user view

Parameters

cipher: Sets a ciphertext password.

simple: Sets a plaintext password.

password: Specifies the password string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 63 characters. If cipher is specified, it must be a ciphertext string of 1 to 117 characters. If neither cipher nor simple is specified, you set a plaintext password string in interactive mode.

Change description

Before modification: If cipher is specified, you can set a plaintext password, or a 24-character or 88-character ciphertext password.

After modification: If cipher is specified, you must enter a ciphertext password of 1 to 117 characters.

Modified command: password (RADIUS-server user view)

Syntax

password [ cipher | simple ] password

Views

RADIUS-server user view

Parameters

cipher: Sets a ciphertext password.

simple: Sets a plaintext password.

password: Specifies the password string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 128 characters. If cipher is specified, it must be a ciphertext string of 1 to 201 characters. If neither cipher nor simple is specified, you set a plaintext password string.

Change description

Before modification: A ciphertext password must comprise 12, 24, 32, 44, 64, 76, 88, 96, 108, 120, 128, 140, 152, 160, 172, or 184 characters.

After modification: A ciphertext password comprises 1 to 201 characters.

Modified command: primary accounting (RADIUS scheme view)

Syntax

primary accounting { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key ] *

Views

RADIUS scheme view

Parameters

ipv4-address: Specifies the IPv4 address of the primary RADIUS accounting server.

ipv6 ipv6-address: Specifies the IPv6 address of the primary RADIUS accounting server, which must be a valid global unicast address.

port-number: Specifies the service port number of the primary RADIUS accounting server, which is a UDP port number in the range 1 to 65535 and defaults to 1813.

key [ cipher | simple ] key: Specifies the shared key for secure communication with the primary RADIUS accounting server.

- cipher key: Specifies a ciphertext shared key, a case-sensitive ciphertext string of 1 to 117 characters.

simple key: Specifies a plaintext shared key, a case-sensitive plaintext string of 1 to 64 characters.

Change description

Before modification: A ciphertext shared key must comprise 12, 24, 32, 44, 64, 76, 88, or 96 characters.

After modification: A ciphertext shared key comprises 1 to 117 characters.

Modified command: primary authentication (RADIUS scheme view)

Syntax

primary authentication { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key ] *

Views

RADIUS scheme view

Parameters

ipv4-address: Specifies the IPv4 address of the primary RADIUS authentication/authorization server.

ipv6 ipv6-address: Specifies the IPv6 address of the primary RADIUS authentication/authorization server, which must be a valid global unicast address.

port-number: Specifies the service port number of the primary RADIUS authentication/authorization server, which is a UDP port number in the range 1 to 65535 and defaults to 1812.

key [ cipher | simple ] key: Specifies the shared key for secure communication with the primary RADIUS authentication/authorization server.

- cipher key: Specifies a ciphertext shared key, a case-sensitive ciphertext string of 1 to 117 characters.

simple key: Specifies a plaintext shared key, a case-sensitive plaintext string of 1 to 64 characters.

Change description

Before modification: A ciphertext shared key must comprise 12, 24, 32, 44, 64, 76, 88, or 96 characters.

After modification: A ciphertext shared key comprises 1 to 117 characters.

Modified command: radius-server client-ip

Old syntax

radius-server client-ip ip-address [ key string ]

New syntax

radius-server client-ip ip-address [ key [ cipher | simple ] string ]

Views

System view

Parameters

ip-address: Specifies the IPv4 address of the RADIUS client.

key: Sets the shared key for secure communication with the RADIUS client.

cipher: Sets a ciphertext shared key.

simple: Sets a plaintext shared key.

string: Specifies the shared key string. This argument is case sensitive. If simple is specified, it must be a string of 1 to 64 characters. If cipher is specified, it must be a ciphertext string of 1 to 117 characters. If neither cipher nor simple is specified, you set a plaintext shared key string.

all: Specifies all RADIUS clients.

Change description

Before modification: You can only set a plaintext shared key.

After modification: You can set a plaintext or a ciphertext shared key. A ciphertext shared key comprises 1 to 117 characters.

Modified command: secondary accounting (RADIUS scheme view)

Syntax

secondary accounting { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key ] *

Views

RADIUS scheme view

Parameters

ipv4-address: Specifies the IPv4 address of the secondary RADIUS accounting server.

ipv6 ipv6-address: Specifies the IPv6 address of the secondary RADIUS accounting server, which must be a valid global unicast address.

port-number: Specifies the service port number of the secondary RADIUS accounting server, which is a UDP port number in the range 1 to 65535 and defaults to 1813.

key [ cipher | simple ] key: Specifies the shared key for secure communication with the secondary RADIUS accounting server.

- cipher key: Specifies a ciphertext shared key, a case-sensitive ciphertext string of 1 to 117 characters.

simple key: Specifies a plaintext shared key, a case-sensitive plaintext string of 1 to 64 characters.

Change description

Before modification: A ciphertext shared key must comprise12, 24, 32, 44, 64, 76, 88, or 96 characters.

After modification: A ciphertext shared key comprises 1 to 117 characters.

Modified command: secondary authentication (RADIUS scheme view)

Syntax

secondary authentication { ipv4-address | ipv6 ipv6-address } [ port-number | key [ cipher | simple ] key ] *

Views

RADIUS scheme view

Parameters

ipv4-address: Specifies the IPv4 address of the secondary RADIUS authentication/authorization server.

ipv6 ipv6-address: Specifies the IPv6 address of the secondary RADIUS authentication/authorization server, which is a valid global unicast address.

port-number: Specifies the service port number of the secondary RADIUS authentication/authorization server, which is a UDP port number in the range 1 to 65535 and defaults to 1812.

key [ cipher | simple ] key: Specifies the shared key for secure communication with the secondary RADIUS authentication/authorization server.

- cipher key: Specifies a ciphertext shared key, a case-sensitive ciphertext string of 1 to 117 characters.

simple key: Specifies a plaintext shared key, a case-sensitive plaintext string of 1 to 64 characters.

Change description

Before modification: A ciphertext shared key must comprise 12, 24, 32, 44, 64, 76, 88, or 96 characters.

After modification: A ciphertext shared key comprises 1 to 117 characters.

Modified command: set authentication password

Syntax

set authentication password { cipher | simple } password

Views

User interface view

Parameters

cipher: Sets a ciphertext password.

simple: Sets a plaintext password.

password: Specifies the password string. This argument is case sensitive. If simple is specified, it must be a plaintext string of 1 to 16 characters. If cipher is specified, it must be a ciphertext string of 1 to 53 characters.

Change description

Before modification: If cipher is specified, you can set a plaintext password of 1 to 16 characters or a 24-character ciphertext password.

After modification: If cipher is specified, you must set a ciphertext password of 1 to 53 characters.

Modified command: snmp-agent usm-user v3

Syntax

snmp-agent usm-user v3 user-name group-name [ cipher ] [ authentication-mode { md5 | sha } auth-password [ privacy-mode { 3des | aes128 | des56 } priv-password ] ] [ acl acl-number | acl ipv6 ipv6-acl-number ] *

Views

System view

Parameters

user-name: User name, a case-sensitive string of 1 to 32 characters.

group-name: Group name, a case-sensitive string of 1 to 32 characters.

cipher: Specifies that auth-password and priv-password are encrypted keys, which can be calculated to a hexadecimal string by using the snmp-agent calculate-password command. If this keyword is not specified, auth-password and priv-password are plaintext keys.

authentication-mode: Specifies an authentication algorithm. MD5 is faster but less secure than SHA.

- md5: Specifies the MD5 authentication algorithm.

- sha: Specifies the SHA-1 authentication algorithm.

auth-password: Specifies a case-sensitive plaintext or encrypted authentication key. A plaintext key is a string of 1 to 64 characters. If the cipher is specified, the encrypted authentication key length requirements differ by authentication algorithm and key string format, as shown in Table 1.

Encrypted authentication key length requirements

| Authentication algorithm | Hexadecimal string | Non-hexadecimal string |
| --- | --- | --- |
| MD5 | 32 characters | 53 characters |
| SHA | 40 characters | 57 characters |

privacy-mode: Specifies an encryption algorithm for privacy. The three encryption algorithms AES, 3DES, and DES are in descending order of security. Higher security means more complex implementation mechanism and lower speed. DES is enough to meet general requirements.

- 3des: Specifies the 3DES algorithm.

- des56: Specifies the DES algorithm.

- aes128: Specifies the AES algorithm.

priv-password: Specifies a case-sensitive plaintext or encrypted privacy key. A plaintext key is a string of 1 to 64 characters. If the cipher keyword is specified, the encrypted privacy key length requirements differ by authentication algorithm and key string format, as shown in Table 2.

Encrypted privacy key length requirements

| Authentication algorithm | Encryption algorithm | Hexadecimal string | Non-hexadecimal string |
| --- | --- | --- | --- |
| MD5 | 3DES | 64 characters | 73 characters |
| MD5 | AES128 or DES-56 | 32 characters | 53 characters |
| SHA | 3DES | 80 characters | 73 characters |
| SHA | AES128 or DES-56 | 40 characters | 53 characters |

acl acl-number: Specifies a basic ACL to filter NMSs by source IPv4 address. The acl-number argument represents a basic ACL number in the range of 2000 to 2999. Only the NMSs with the IPv4 addresses permitted in the ACL can use the specified username to access the SNMP agent.

acl ipv6 ipv6-acl-number: Specifies a basic ACL to filter NMSs by source IPv6 address. The ipv6-acl-number argument represents a basic ACL number in the range of 2000 to 2999. Only the NMSs with the IPv6 addresses permitted in the ACL can use the specified username to access the SNMP agent.

local: Represents a local SNMP entity user.

engineid engineid-string: Specifies an SNMP engine ID as a hexadecimal string. The engineid-string argument must comprise an even number of hexadecimal characters, in the range of 10 to 64. All-zero and all-F strings are invalid.

Change description

Before modification: You can only set keys in hexadecimal format.

After modification: You can set keys in either hexadecimal or non-hexadecimal format.

See Table 1 for the ciphertext authentication key length requirements.

See Table 2 for the ciphertext privacy key length requirements.

Modified command: super password

Syntax

super password [ level user-level ] { cipher | simple } password

Views

System view

Parameters

level user-level: User privilege level, which ranges from 1 to 3 and defaults to 3.

cipher: Sets a ciphertext password.

simple: Sets a plaintext password.

password: Specifies the password string. This argument is case sensitive. If simple is specified, it must be a plaintext string of 1 to 16 characters. If cipher is specified, it must be a ciphertext string of 1 to 53 characters.

Change description

Before modification: If cipher is specified, you can set a plaintext password of 1 to 16 characters or a 24-character ciphertext password.

After modification: If cipher is specified, you must set a ciphertext password of 1 to 53 characters.

Modified feature: Task ID for IPv6 socket display

Feature change description

Changed the task ID value range for IPv6 socket display.

Command changes

Modified command: display ipv6 socket

Syntax

display ipv6 socket [ socktype socket-type ] [ task-id socket-id ] [ slot slot-number ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: The task ID is in the range of 1 to 150.

After modification: The task ID is in the range of 1 to 255.

Removed feature: Local user password display

Feature change description

Deleted the feature used to set a display mode for all local user passwords.

Removed commands

local-user password-display-mode

Syntax

local-user password-display-mode { auto | cipher-force }

undo local-user password-display-mode

View

System view

# A5120EI-CMW520-F2211

This release has the following changes:

New feature: Critical VLAN

New feature: SCP

New feature: Specifying the source interface for DNS packets

New feature: MVRP

New feature: Enabling LLDP to automatically discover IP phones

New feature: Critical VLAN

  - Overview

The critical VLAN feature enables a port to assign a VLAN to 802.1X users or MAC authentication users when they fail authentication because all the RADIUS authentication servers in their ISP domains have been unreachable, for example, due to the loss of network connectivity. The critical VLAN feature takes effect when authentication is performed only through RADIUS servers. If an 802.1X or MAC authentication user fails local authentication after RADIUS authentication, the user is not assigned to the critical VLAN.

The critical VLANs for 802.1X users are called “802.1X critical VLANs” and the critical VLANs for MAC authentication users are called “MAC authentication critical VLANs.”

You can configure one 802.1X critical VLAN and one MAC authentication critical VLAN on a port.

Any of the following RADIUS authentication server change in the ISP domain for 802.1X or MAC authentication users can cause the users to be removed from the critical VLAN:

An authentication server is reconfigured, added, or removed.

The status of any RADIUS authentication server automatically changes to active or is administratively set to active.

The RADIUS server probing function detects that a RADIUS authentication server is reachable and sets its state to active.

      - 802.1X critical VLAN assignment

The way that the network access device handles VLANs on an 802.1X-enabled port differs by 802.1X access control mode.

            - On a port that performs port-based access control

| Authentication status | VLAN manipulation |
| --- | --- |
| A user that has not been assigned to any VLAN fails 802.1X authentication because all the RADIUS servers are unreachable. | Assigns the critical VLAN to the port as the PVID. The 802.1X user and all subsequent 802.1X users on this port can access only resources in the critical VLAN. |
| A user in the 802.1X critical VLAN fails authentication because all the RADIUS servers are unreachable. | The critical VLAN is still the PVID of the port, and all 802.1X users on this port are in this VLAN. |
| A user in the 802.1X critical VLAN fails authentication for any other reason than server unreachable. | If an Auth-Fail VLAN has been configured, the PVID of the port changes to Auth-Fail VLAN ID, and all 802.1X users on this port are moved to the Auth-Fail VLAN. |
| A user in the critical VLAN passes 802.1X authentication. | Assigns the VLAN specified for the user to the port as the PVID, and removes the port from the critical VLAN. After the user logs off, the default or user-configured PVID restores. If the authentication server assigns no VLAN, the default or user-configured PVID applies. The user and all subsequent 802.1X users are assigned to this port VLAN. After the user logs off, this PVID remains unchanged. |
| A user in the 802.1X guest VLAN or the Auth-Fail VLAN fails authentication because all the RADIUS servers is reachable. | The PVID of the port remains unchanged. All 802.1X users on this port can access only resources in the guest VLAN or the Auth-Fail VLAN. |

            - On a port that performs MAC-based access control

| Authentication status | VLAN manipulation |
| --- | --- |
| A user that has not been assigned to any VLAN fails 802.1X authentication because all the RADIUS servers are unreachable. | Maps the MAC address of the user to the critical VLAN. The user can access only resources in the critical VLAN. |
| A user in the 802.1X critical VLAN fails authentication because all the RADIUS servers are unreachable. | The user is still in the critical VLAN. |
| A user in the critical VLAN fails 802.1X authentication for any other reason than server unreachable. | If an Auth-Fail VLAN has been configured, re-maps the MAC address of the user to the Auth-Fail VLAN ID. |
| A user in the critical VLAN passes 802.1X authentication. | Re-maps the MAC address of the user to the server-assigned VLAN. If the authentication server assigns no VLAN, re-maps the MAC address of the user to the default or user-configured PVID on the port. |
| A user in the 802.1X guest VLAN or the Auth-Fail VLAN fails authentication because all the RADIUS server are unreachable. | The user remains in the 802.1X VLAN or the Auth-Fail VLAN. |
| A user in the MAC authentication guest VLAN fails 802.1X authentication because all the 802.1X authentication server are unreachable. | The user is removed from the MAC authentication VLAN and mapped to the 802.1X critical VLAN. |

|  | NOTE: - To perform the 802.1X critical VLAN function on a port that performs MAC-based access control, you must make sure that the port is a hybrid port, and enable MAC-based VLAN on the port. - The network device assigns a hybrid port to an 802.1X critical VLAN as an untagged member. - For more information about VLAN configuration and MAC-based VLAN, see Layer 2—LAN Switching Configuration Guide. |
| --- | --- |

      - MAC authentication critical VLAN assignment

The MAC authentication critical VLAN feature depends on the MAC-based VLAN feature and is available only on hybrid ports.

When a user fails MAC authentication because no RADIUS server is reachable, the switch maps the user’s MAC address to the MAC authentication critical VLAN. The MAC-to-critical VLAN mapping for the user is removed when any RADIUS server change in the ISP domain for the user occurs or after the user passes MAC authentication.

  - Configuring a 802.1X critical VLAN

    - Configuration guidelines

You can configure only one 802.1X critical VLAN on a port. The 802.1X critical VLANs on different ports can be different.

Assign different IDs for the voice VLAN, the port VLAN, and the 802.1X critical VLAN on a port, so the port can correctly process VLAN tagged incoming traffic.

You cannot specify a VLAN as both a super VLAN and an 802.1X critical VLAN. For information about super VLANs, see Layer 2—LAN Switching Configuration Guide.

- You can use the dot1x critical recovery-action reinitialize command to configure the port to trigger 802.1X re-authentication when the port or an 802.1X user on the port is removed from the critical VLAN.

  - If MAC-based access control is used, the port sends a unicast Identity EAP/Request to the 802.1X user to trigger authentication.

  - If port-based access control is used, the port sends a multicast Identity EAP/Request to the 802.1X users to trigger authentication.

- If no critical VLAN is configured, RADIUS server unreachable can cause an online user being re-authenticated to be logged off. If a critical VLAN is configured, the user remains online and in the original VLAN.

    - Configuration prerequisites

Create the VLAN to be specified as a critical VLAN.

If the 802.1X-enabled port performs port-based access control, enable 802.1X multicast trigger (dot1x multicast-trigger).

If the 802.1X-enabled port performs MAC-based access control, configure the port as a hybrid port, enable MAC-based VLAN on the port, and assign the port to the critical VLAN as an untagged member. For more information about the MAC-based VLAN function, see Layer 2—LAN Switching Configuration Guide.

    - Configuration procedure

To configure an 802.1X critical VLAN on a port:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| ##### Configure an 802.1X critical VLAN on the port. | dot1x critical vlan vlan-id | By default, no 802.1X critical VLAN is configured. |
| ##### (Optional) Configure the port to trigger 802.1X authentication on detection of a reachable authentication server for users in the critical VLAN. | dot1x critical recovery-action reinitialize | By default, when a reachable RADIUS server is detected, the system removes the port or 802.1X users from the critical VLAN without triggering authentication. |

  - Configuring a MAC authentication critical VLAN

You can configure only one MAC authentication critical VLAN on a port.

Follow the guidelines in Table 1 when you configure a MAC authentication critical VLAN on a port.

Relationships of the MAC authentication critical VLAN with other security features

| Feature | Relationship description | Reference |
| --- | --- | --- |
| Quiet function of MAC authentication | The MAC authentication critical VLAN function has higher priority. When a user fails MAC authentication because no RADIUS authentication server is reachable, the user can access the resources in the critical VLAN, and the user’s MAC address is not marked as a silent MAC address. | Security Configuration Guide |
| Super VLAN | You cannot specify a VLAN as both a super VLAN and a MAC authentication critical VLAN. | Layer 2—LAN Switching Configuration Guide |
| Port intrusion protection | The MAC authentication critical VLAN function has higher priority than the block MAC action but lower priority than the shut down port action of the port intrusion protection feature. | Security Configuration Guide |

    - Configuration prerequisites

Enable MAC authentication.

Enable MAC-based VLAN on the port.

Create the VLAN to be specified as the MAC authentication critical VLAN.

    - Configuration procedure

To configure a MAC authentication critical VLAN on a port:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter Layer 2 Ethernet interface view. | interface interface-type interface-number | N/A |
| ##### Configure a MAC authentication critical VLAN on the port. | mac-authentication critical vlan critical-vlan-id | By default, no MAC authentication critical VLAN is configured. |

  - Configuring a RADIUS server probe

When you add a primary or secondary RADIUS authentication server to a RADIUS scheme, you can configure a probe to regularly detect the availability (or reachability) of the server. If the server is unreachable, its state is block. If the server is reachable, its state is active.

When a server status change is detected, the RADIUS server probe advertises the change to the authentication modules, including the 802.1X module, so these modules can take prompt responsive action. For example, the 802.1X module removes users from an 802.1X critical VLAN and makes authentication triggering decision immediately after the RADIUS authentication server probing function detects that a server in the ISP domain for the 802.1X users has become reachable and changes the server state to active.

To configure RADIUS authentication server probes:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter RADIUS scheme view. | radius scheme radius-scheme-name | N/A |
| ##### Configure RADIUS authentication server probes. | - Configure the primary RADIUS authentication server probe: primary authentication { ipv4-address ¦ ipv6 ipv6-address } probe username name [ interval interval ] - Configure the secondary RADIUS server probe: secondary authentication { ipv4-address ¦ ipv6 ipv6-address } probe username name [ interval interval ] | By default, no RADIUS authentication server probes are configured. |

  - Command reference

    - dot1x critical vlan

Use the dot1x critical vlan command to configure an 802.1X critical VLAN on a port for users that fail 802.1X authentication because all the RADIUS servers in their ISP domains have been unreachable.

Syntax

dot1x critical vlan vlan-id

undo dot1x critical vlan

Default

No critical VLAN is configured on any port.

Views

Layer 2 Ethernet interface view

Default command level

2: System level

Parameters

vlan-id: Specifies a VLAN ID, in the range of 1 to 4094. Make sure the VLAN has been created.

Usage guidelines

You can configure only one critical VLAN on a port. The 802.1X critical VLANs on different ports can be different.

When you change the access control method from MAC-based to port-based on the port, the mappings between MAC addresses and the 802.1X critical VLAN are removed. You can use the display mac-vlan command to display MAC-to-VLAN mappings.

When you change the access control method from port-based to MAC-based on a port that is in a critical VLAN, the port is removed from the critical VLAN.

To delete a VLAN that has been configured as an 802.1X critical VLAN, you must perform the undo dot1x critical vlan command first.

Examples

# Specify VLAN 3 as the 802.1X critical VLAN on port GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x critical vlan 3

    - dot1x critical recovery-action

Use the dot1x critical recovery-action command to configure the action that a port takes when an active (reachable) RADIUS authentication server is detected for users in the 802.1X critical VLAN.

Use the undo dot1x critical recovery-action command to restore the default.

Syntax

dot1x critical recovery-action reinitialize

undo dot1x critical recovery-action

Default

When a reachable RADIUS server is detected, the system removes the port or 802.1X users from the critical VLAN without triggering authentication.

Views

Layer 2 Ethernet interface view

Default command level

2: System level

Parameters

reinitialize: Enables the port to trigger 802.1X re-authentication on detection of a reachable RADIUS authentication server for users in the critical VLAN.

Usage guidelines

The dot1x critical recovery-action command takes effect only for the 802.1X users in the critical VLAN on a port. It enables the port to take one of the following actions to trigger 802.1X authentication after removing 802.1X users from the critical VLAN on detection of a reachable RADIUS authentication server:

If MAC-based access control is used, the port sends a unicast Identity EAP/Request to each 802.1X user.

If port-based access control is used, the port sends a multicast Identity EAP/Request to all the 802.1X users attached to the port.

Examples

# Configure port GigabitEthernet 1/0/1 to trigger 802.1X re-authentication on detection of an active RADIUS authentication server for users in the critical VLAN.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dot1x critical recovery-action reinitialize

    - mac-authentication critical vlan

Use the mac-authentication critical vlan command to configure a MAC authentication critical VLAN on a port for MAC authentication users that have failed authentication because all the RADIUS authentication servers in their ISP domain are unreachable.

Use the undo mac-authentication critical vlan command to restore the default.

Syntax

mac-authentication critical vlan critical-vlan-id

undo mac-authentication critical vlan

Default

No MAC authentication critical VLAN is configured on a port.

Views

Layer 2 Ethernet interface view

Default command level

2: System level

Parameters

critical-vlan-id: Specifies a VLAN ID, in the range of 1 to 4094. Make sure the VLAN has been created.

Usage guidelines

You can configure only one MAC authentication critical VLAN on a port. The MAC authentication critical VLANs on different ports can be different.

You cannot specify a VLAN as both a super VLAN and a MAC authentication critical VLAN. For more information about super VLANs, see Layer 2—LAN Switching Configuration Guide.

To delete a VLAN that has been configured as a MAC authentication critical VLAN, you must perform the undo mac-authentication critical vlan command first.

Examples

# Specify VLAN 5 as the MAC authentication critical VLAN on port GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] mac-authentication critical vlan 5

Related commands

- mac-authentication

- mac-vlan enable

    - primary authentication probe

Use the primary authentication probe command to configure a probe to detect the availability of the primary authentication server in a RADIUS scheme.

Use the undo primary authentication command to remove the primary RADIUS authentication server in a RADIUS scheme.

Syntax

primary authentication { ipv4-address | ipv6 ipv6-address } probe username name [ interval interval ]

undo primary authentication

Default

No probe has been configured for any RADIUS authentication server.

Views

RADIUS scheme view

Default command level

2: System level

Parameters

ipv4-address: Specifies the IPv4 address of the primary RADIUS authentication server.

ipv6 ipv6-address: Specifies the IPv6 address of the primary RADIUS authentication server. The ipv6-address argument must be a valid IPv6 global unicast address.

probe: Configure a probe for the primary authentication server.

username name: Assigns a name to the probe. This name is used as the username in the authentication requests sent by the probe to the RADIUS server. The name argument is a string of 1 to 64 characters and can be a username that has not been created on the RADIUS server.

interval interval: Sets the probing interval in minutes, in the range of 1 to 3600. If no probing interval is specified, the probe performs probing every 60 minutes.

Usage guidelines

A primary RADIUS authentication server probe periodically sends authentication requests to the primary RADIUS authentication server. If no response has been received from the server before timer set by the timer response-timeout command expires, the probe re-transmits the request. If a response is received from the server before the maximum number of retries (set by using the retry command) is reached, the probe considers the server is reachable and sets the server in active state. If not, the probe considers the server is unreachable and sets the server in block state.

The state of a blocked RADIUS server changes to active when the server probe detects that the server is reachable or the server quiet timer (set by using the timer quiet command) times out.

Quiet timer timeouts cause block-to-active changes regardless of the real server availability state. A short server quiet timer setting can cause frequent server state changes when network connectivity is poor. To avoid frequent server state changes causing frequent critical VLAN assignments and removals, make sure the server quiet timer is long enough.

Examples

# Configure the probe test to detect the primary RADIUS authentication server in the RADIUS scheme radius1 every 120 minutes.

<Sysname> system-view

[Sysname] radius scheme radius1

[Sysname-radius-radius1] primary authentication 10.110.1.1 probe username test interval 120

    - secondary authentication probe

Use the secondary authentication probe command to configure a probe to detect the availability of the secondary authentication server in a RADIUS scheme.

Use the undo secondary authentication command to remove the secondary RADIUS authentication server in a RADIUS scheme.

Syntax

secondary authentication { ipv4-address | ipv6 ipv6-address } probe username name [ interval interval ]

undo secondary authentication [ ipv4-address | ipv6 ipv6-address ]

Default

No probe has been configured for any RADIUS authentication server.

Views

RADIUS scheme view

Default command level

2: System level

Parameters

ipv4-address: Specifies the IPv4 address of the secondary RADIUS authentication server.

ipv6 ipv6-address: Specifies the IPv6 address of the secondary RADIUS authentication server. The ipv6-address argument must be a valid IPv6 global unicast address.

probe: Configure a probe for the secondary authentication server.

username name: Assigns a name to the probe. This name is used as the username in the authentication requests sent by the probe to the RADIUS server. The name argument is a string of 1 to 64 characters and can be a username that has not been created on the RADIUS server.

interval interval: Sets the probing interval in minutes, in the range of 1 to 3600. If no probing interval is specified, the probe performs probing every 60 minutes.

Usage guidelines

A secondary RADIUS authentication server probe periodically sends authentication requests to the secondary RADIUS authentication server. If no response has been received from the server before timer set by the timer response-timeout command expires, the probe re-transmits the request. If a response is received from the server before the maximum number of retries (set by using the retry command) is reached, the probe considers the server is reachable and sets the server in active state. If not, the probe considers the server is unreachable and sets the server in block state.

The state of a blocked RADIUS server changes to active when the server probe detects that the server is reachable or the server quiet timer (set by using the timer quiet command) times out.

Quiet timer timeouts cause block-to-active changes regardless of the real server availability state. A short server quiet timer setting can cause frequent server state changes when network connectivity is poor. To avoid frequent server state changes causing frequent critical VLAN assignments and removals, make sure the server quiet timer is long enough.

Examples

# Configure the probe test to detect the secondary RADIUS authentication server in the RADIUS scheme radius1 every 120 minutes.

<Sysname> system-view

[Sysname] radius scheme radius1

[Sysname-radius-radius1]secondary authentication 10.110.1.1 probe username test interval 120

New feature: SCP

  - Overview

Secure copy (SCP) is based on SSH2.0 and offers a secure approach to copying files.

SCP uses SSH connections for copying files. The switch can act as the SCP server, allowing a user to log in to the switch for file upload and download. The switch can also act as an SCP client, enabling a user to log in from the switch to a remote server for secure file transfer.

|  | NOTE: When the switch acts as an SCP server, only one of all the FTP, SFTP and SCP users can access the switch. |
| --- | --- |

  - Configuring the switch as an SCP server

To configure the switch as an SCP server:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Configure the SSH server. | For more information, see the security guide for your switch. | N/A |
| ##### Create an SSH user for a SCP client, set the service type to all or scp, and specify the authentication method. | ssh user username service-type { all ¦ scp } authentication-type { password ¦ { any ¦ password-publickey ¦ publickey } assign publickey keyname work-directory directory-name } | N/A |
| ##### Create a user account and assign a working directory for the SSH user on the switch or a remote server if password authentication is used. | - On the remote server (details not shown) - On the switch: ##### local-user ##### password ##### service-type ssh ##### authorization-attribute work-directory directory-name | Skip this step if publickey authentication, whether with password authentication or not, is used. Make sure that the local user account has the name username as the username specified in the ssh user command. |

When you set the working directory for the user, follow these guidelines:

- If only password authentication is used, the working directory specified in the ssh user command does not take effect. You must set the working directory on the remote server or in the local user account for the SSH user.

- If publickey authentication, whether with password authentication or not, is used, you must set the working directory in the ssh user command.

  - Configuring the switch as the SCP client

To upload or download files to or from an SCP server, perform the following tasks in any view:

| Task | Command |
| --- | --- |
| Upload a file to an SCP server. | - Upload a file to the IPv4 SCP server: scp server [ port-number ] put source-file-path [ destination-file-path ] [ identity-key { dsa ¦ rsa } ¦ prefer-ctos-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-ctos-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ¦ prefer-kex { dh-group-exchange ¦ dh-group1 ¦ dh-group14 } ¦ prefer-stoc-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-stoc-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ] * - Upload a file to the IPv6 SCP server: scp ipv6 server [ port-number ] put source-file-path [ destination-file-path ] [ identity-key { dsa ¦ rsa } ¦ prefer-ctos-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-ctos-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ¦ prefer-kex { dh-group-exchange ¦ dh-group1 ¦ dh-group14 } ¦ prefer-stoc-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-stoc-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ] * |
| Download a file from an SCP server. | - Download a file from the remote IPv4 SCP server: scp server [ port-number ] get source-file-path [ destination-file-path ] [ identity-key { dsa ¦ rsa } ¦ prefer-ctos-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-ctos-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ¦ prefer-kex { dh-group-exchange ¦ dh-group1 ¦ dh-group14 } ¦ prefer-stoc-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-stoc-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ] * - Download a file from the remote IPv6 SCP server: scp ipv6 server [ port-number ] get source-file-path [ destination-file-path ] [ identity-key { dsa ¦ rsa } ¦ prefer-ctos-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-ctos-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ¦ prefer-kex { dh-group-exchange ¦ dh-group1 ¦ dh-group14 } ¦ prefer-stoc-cipher { 3des ¦ aes128 ¦ des } ¦ prefer-stoc-hmac { md5 ¦ md5-96 ¦ sha1 ¦ sha1-96 } ] * |

|  | NOTE: File transfer interruption during a downloading process can result in file fragments on the switch. You must manually delete them. |
| --- | --- |

    - SCP client configuration example

      - Network requirements

As shown in Figure 1, switch A acts as a client and download the file remote.bin from switch B. The user has the username test and uses the password authentication method.

Network diagram

      - Configuration procedure

# Create VLAN-interface 1 and assign an IP address to it.

<SwitchA> system-view

[SwitchA] interface vlan-interface 1

[SwitchA-Vlan-interface1] ip address 192.168.0.2 255.255.255.0

[SwitchA-Vlan-interface1] quit

# Download the file remote.bin from the SCP server, and save it with the file name local.bin.

<SwitchA> scp 192.168.0.1 get remote.bin local.bin

Username: test

Trying 192.168.0.1 ...

Press CTRL+K to abort

Connected to 192.168.0.1 ...

The Server is not authenticated. Continue? [Y/N]:y

Do you want to save the server public key? [Y/N]:n

Enter password:

18471 bytes transfered in 0.001 seconds.

    - SCP server configuration example

      - Network requirements

As shown in Figure 2, the switch acts as the SCP server, and the host acts as the SCP client. The host establishes an SSH connection to the switch. The user uses the username test and the password aabbcc. The username and password are saved on the switch for local authentication.

Network diagram

      - Configuration procedure

# Generate the RSA key pairs.

<Switch> system-view

[Switch] public-key local create rsa

The range of public key size is (512 ~ 2048).

NOTES: If the key modulus is greater than 512,

It will take a few minutes.

Press CTRL+C to abort.

Input the bits of the modulus[default = 1024]:

Generating Keys...

++++++++

++++++++++++++

+++++

++++++++

# Generate the DSA key pair.

[Switch] public-key local create dsa

The range of public key size is (512 ~ 2048).

NOTES: If the key modulus is greater than 512,

It will take a few minutes.

Press CTRL+C to abort.

Input the bits of the modulus[default = 1024]:

Generating Keys...

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

+++++++++++++++++++++++++++++++++++

# Enable the SSH server function.

[Switch] ssh server enable

# Configure an IP address for VLAN-interface 1, which the client will use as the destination for SSH connection.

[Switch] interface vlan-interface 1

[Switch-Vlan-interface1] ip address 192.168.1.45 255.255.255.0

[Switch-Vlan-interface1] quit

# Set the authentication mode of the user interfaces to AAA.

[Switch] user-interface vty 0 15

[Switch-ui-vty0-15] authentication-mode scheme

# Enable the user interfaces to support all protocols including SSH.

[Switch-ui-vty0-15] protocol inbound all

[Switch-ui-vty0-15] quit

# Create the local user test and specify a working directory for the user.

[Switch] local-user test

[Switch-luser-test] password simple aabbcc

[Switch-luser-test] service-type ssh

[Switch-luser-test] authorization-attribute work-directory flash:/

[Switch-luser-test] quit

# Configure the SSH user authentication method as password and service type as scp.

[Switch] ssh user test service-type scp authentication-type password

  - Command reference

    - scp

Use the scp command to transfer files with an SCP server.

Syntax

scp [ ipv6 ] server [ port-number ] { get | put } source-file-path [ destination-file-path ] [ identity-key { dsa | rsa } | prefer-ctos-cipher { 3des | aes128 | des } | prefer-ctos-hmac { md5 | md5-96 | sha1 | sha1-96 } | prefer-kex { dh-group-exchange | dh-group1 | dh-group14 } | prefer-stoc-cipher { 3des | aes128 | des } | prefer-stoc-hmac { md5 | md5-96 | sha1 | sha1-96 } ] *

Views

User view

Default command level

3: Manage level

Parameters

ipv6: Specifies the type of the server as IPv6. If this keyword is not specified, the server is an IPv4 server.

server: Specifies an IPv4 or IPv6 address or host name of the server. For an IPv4 server, it is a case-insensitive string of 1 to 20 characters. For an IPv6 server, it is a case-insensitive string of 1 to 46 characters.

port-number: Specifies the port number of the server, in the range of 0 to 65535. The default is 22.

identity-key: Specifies the algorithm for publickey authentication, either dsa or rsa. The default is dsa.

- dsa: Specifies the publickey algorithm dsa.

- rsa: Specifies the publickey algorithm rsa.

prefer-ctos-cipher: Preferred encryption algorithm from client to server, defaulted to aes128.

- 3des: Encryption algorithm 3des-cbc.

- aes128: Encryption algorithm aes128-cbc.

- des: Encryption algorithm des-cbc.

prefer-ctos-hmac: Preferred HMAC algorithm from client to server, defaulted to sha1-96.

- md5: HMAC algorithm hmac-md5.

- md5-96: HMAC algorithm hmac-md5-96.

- sha1: HMAC algorithm hmac-sha1.

- sha1-96: HMAC algorithm hmac-sha1-96.

prefer-kex: Preferred key exchange algorithm, defaulted to dh-group-exchange.

- dh-group-exchange: Key exchange algorithm diffie-hellman-group-exchange-sha1.

- dh-group1: Key exchange algorithm diffie-hellman-group1-sha1.

- dh-group14: Key exchange algorithm diffie-hellman-group14-sha1.

prefer-stoc-cipher: Preferred encryption algorithm from server to client, defaulted to aes128.

prefer-stoc-hmac: Preferred HMAC algorithm from server to client, defaulted to sha1-96.

Usage guidelines

When the client’s authentication method is publickey, the client needs to get the local private key for digital signature. As the publickey authentication includes RSA and DSA algorithms, you must specify an algorithm (by using the identity-key keyword) in order to get the correct data for the local private key. By default, the publickey algorithm is DSA.

Examples

# Download the file remote.bin from the SCP server, save it locally and change the file name to local.bin

<Sysname> scp 192.168.0.1 get remote.bin local.bin

    - ssh user

Use the ssh user command to create an SSH user and specify the service type and authentication method.

Use the undo ssh user command to delete an SSH user.

Syntax

ssh user username service-type stelnet authentication-type { password | { any | password-publickey | publickey } assign publickey keyname }

ssh user username service-type { all | scp | sftp } authentication-type { password | { any | password-publickey | publickey } assign publickey keyname work-directory directory-name }

undo ssh user username

Views

System view

Default command level

3: Manage level

Parameters

username: SSH username, a case-sensitive string of 1 to 80 characters.

service-type: Specifies the service type of an SSH user, which can be one of the following:

- all: Specifies Stelnet, SFTP, and SCP.

- scp: Specifies the service type as secure copy.

- sftp: Specifies the service type as secure FTP.

- stelnet: Specifies the service type of secure Telnet.

authentication-type: Specifies the authentication method of an SSH user, which can be one of the following:

- password: Performs password authentication. This authentication method features easy and fast encryption, but it is vulnerable. It can work with AAA to implement user authentication, authorization, and accounting.

- any: Performs either password authentication or publickey authentication.

- password-publickey: Performs both password authentication and publickey authentication (featuring higher security) if the client runs SSH2, and performs either type of authentication if the client runs SSH1.

- publickey: Performs publickey authentication. This authentication method has the downside of complicated and slow encryption, but it provides strong authentication that can defend against brute-force attacks. This authentication method is easy to use. Once it is configured, the authentication process completes automatically without the need of remembering or entering any password.

assign publickey keyname: Assigns an existing public key to an SSH user. The keyname argument indicates the name of the client public key and is a string of 1 to 64 characters.

work-directory directory-name: Specifies the working directory for an SCP or SFTP user. The directory-name argument indicates the name of the working directory and is a string of 1 to 135 characters.

Usage guidelines

For a publickey authentication user, you must configure the username and the public key on the switch. For a password authentication user, you can configure the account information on either the switch or the remote authentication server, such as a RADIUS server.

If you use the ssh user command to configure a public key for a user who has already had a public key, the new one overwrites the old one.

You can change the authentication method and public key of an SSH user when the user is communicating with the SSH server. However, your changes take effect only after the user logs out and logs in again.

If an SCP or SFTP user has been assigned a public key, it is necessary to set a working folder for the user.

The working folder of an SCP or SFTP user depends on the user authentication method. For a user using only password authentication, the working folder is the AAA authorized one. For a user using only publickey authentication or using both publickey authentication and password authentication, the working folder is the one set by using the ssh user command.

Examples

# Create an SSH user named user1, setting the service type as scp, the authentication method as publickey, the working directory of the SCP server as flash:/, and assigning a public key named key1 to the user.

<Sysname> system-view

[Sysname] ssh user user1 service-type scp authentication-type publickey assign publickey key1 work-directory flash:/

Related commands

display ssh user-information

New feature: Specifying the source interface for DNS packets

  - Specifying the source interface for DNS packets

By default, the device uses the primary IP address of the output interface of the matching route as the source IP address of a DNS request. Therefore, the source IP address of the DNS packets may vary with DNS servers. In some scenarios, the DNS server only responds to DNS requests sourced from a specific IP address. In such cases, you must specify the source interface for the DNS packets so that the device can always uses the primary IP address of the specified source interface as the source IP address of DNS packets.

To specify the source interface for DNS packets:

| Step | Command | Remarks |
| --- | --- | --- |
| Enter system view. | system-view | N/A |
| Specify the source interface for DNS packets. | dns source-interface interface-type interface-number | By default, no source interface for DNS packets is specified; the device looks up its routing table for an output interface for a DNS request destined for a DNS server and uses the primary IP address of the interface as the source IP address of the packet. |

Command reference

dns source-interface

Use dns source-interface to specify the source interface for DNS packets.

Use undo dns source-interface to restore the default.

Syntax

dns source-interface interface-type interface-number

undo dns source-interface

Default

No source interface for DNS packets is specified. The device uses the primary IP address of the output interface of the matching route as the source IP address of a DNS request.

Views

System view

Default command level

2. System level

Parameters

interface-type interface-number: Specifies the interface type and number.

Usage guidelines

The device uses the primary IP address of the specified source interface as the source IP address of a DNS request, which however is still forwarded through the output interface of the matching route.

Examples

# Specify VLAN-interface 2 as the source interface of DNS requests.

<Sysname> system-view

[Sysname] dns source-interface vlan-interface2

New feature: MVRP

  - Overview

Multiple Registration Protocol (MRP) is an attribute registration protocol and transmits attribute messages. Multiple VLAN Registration Protocol (MVRP) is a typical MRP application. MVRP propagates and learns VLAN configuration among devices. MVRP enables a device to propagates the local VLAN configuration to the other devices, receive VLAN configuration from other devices, and dynamically update the local VLAN configuration (including the active VLANs and the ports through which a VLAN can be reached). MVRP makes sure that all MVRP-enabled devices in a LAN maintain the same VLAN configuration, and reduces the VLAN configuration workload. When the network topology changes, MVRP can propagate and learn VLAN configuration information again according to the new topology, and real-time synchronize the network topology.

MRP is an enhanced version of Generic Attribute Registration Protocol (GARP) and improves the declaration efficiency. MVRP is an enhanced version of GARP VLAN Registration Protocol (GVRP). MVRP delivers the following benefits over GVRP:

- GVRP does not support the multiple spanning tree instance (MSTI). MVRP runs on a per-MSTI basis, and implements per-VLAN redundant link calculation and load sharing.

- MVRP decreases the number of packets transmitted for the same amount of VLAN configuration, and improves the declaration efficiency.

For more information about GVRP or MSTI, see " Layer 2—LAN Switching Configuration Guide."

    - MRP

MRP allows participants in the same LAN to declare, propagate, and register information (for example, VLAN information) on a per Multiple Spanning Tree Instance (MSTI) basis.

      - MRP implementation

Each port that participates in an MRP application (for example, MVRP) is called an "MRP participant". Similarly, a port that participates in an MVRP application is called an "MVRP participant."

As shown in Figure 3, an MRP participant registers and deregisters its attribute values on other MRP participants by sending declarations and withdrawals, and registers and deregisters the attribute values of other participants according to the received declarations and withdrawals. MRP rapidly propagates the configuration information of an MRP participant throughout the LAN.

MRP implementation

MVRP registers and deregisters VLAN attributes as follows:

- When a port receives the declaration of a VLAN attribute, the port registers the VLAN and joins the VLAN.

- When a port receives the withdrawal of a VLAN attribute, the port deregisters the VLAN and leaves the VLAN.

Figure 3 shows a simple MVRP implementation on an MSTI. In a network with multiple MSTIs, VLAN registration and deregistration are performed on a per-MSTI basis.

      - MRP messages

MRP exchanges information among MRP participants by advertising MRP messages, including Join, New, Leave, and LeaveAll. Join and New messages are declarations, and Leave and LeaveAll messages are withdrawals.

- Join message

  - An MRP participant sends Join messages when it wishes to declare the attribute values configured on it and receives Join messages from other MRP participants.

  - When receiving a Join message, an MRP participant sends a Join message to all participants except the sender.

Join messages fall into the following types:

  - JoinEmpty—An MRP participant sends JoinEmpty messages to declare attribute values that it has not registered. For example, when a static VLAN exists on a device, the attribute of the VLAN on the device is not changed even if the device learns the VLAN again through MRP. In this case, the Join message for the VLAN attribute is a JoinEmpty message, because the VLAN attribute is not registered.

  - JoinIn—An MRP participant sends JoinIn messages to declare attribute values that it has registered. For example, when the device learns a VLAN through MRP messages, and dynamically creates the VLAN, the Join message for the VLAN attribute is a JoinIn message.

- New message

Similar to a Join message, a New message enables MRP participants to register attributes.

  - When the Multiple Spanning Tree Protocol (MSTP) topology changes, an MRP participant sends New messages to declare the topology change.

  - On receiving a New message, an MRP participant sends a New message out of each port except the receiving port.

- Leave message

  - An MRP participant sends Leave messages when it wishes other participants to deregister the attributes that is has deregistered.

  - When receiving a Leave message, an MRP participant sends a Leave message to all participants except the sender.

- LeaveAll message

  - Each MRP participant is configured with an individual LeaveAll timer. When the timer expires, the MRP participant sends LeaveAll messages to the remote participants, so that the local participant and the remote participants deregister all attributes and re-register all attributes. This process periodically clears the useless attributes in the network.

  - On receiving a LeaveAll message, MRP determines whether to send a Join message to request the sender to re-register these attributes according to attribute status.

      - MRP timers

The implementation of MRP uses the following timers to control MRP message transmission.

- Periodic timer

On startup, an MRP participant starts its own Periodic timer to control MRP message transmission. The MRP participant collects the MRP messages to be sent before the Periodic timer expires, and sends the MRP messages in as few packets as possible when the Periodic timer expires and meanwhile restarts the Periodic timer. This mechanism reduces the number of MRP protocol packets periodically sent.

You can enable or disable the Periodic timer at the CLI. When you disable the Periodic timer, MRP will not periodically send MRP messages, and MRP messages are sent only when the LeaveAll timer expires or the local participant receives LeaveAll messages from a remote participant.

- Join timer

The Join timer controls the transmission of Join messages. To make sure that Join messages can be reliably transmitted to other participants, an MRP participant waits for a period of the Join timer after sending a Join message. If the participant receives JoinIn messages from other participants and the attributes in the JoinIn messages are the same as the sent Join messages before the Join timer expires, the participant does not re-send the Join message. When both the Join timer and the Periodic timer expire, the participant re-sends the Join message.

- Leave timer

The Leave timer controls the deregistration of attributes. When an MRP participant wishes other participants to deregister its attributes, it sends a Leave message. On receiving a Leave message, MRP starts the Leave timer, and deregisters the attributes if it does not receive any Join message for the attributes before the Leave timer expires. When an MRP participant sends or receives LeaveAll messages, it starts the Leave timer. MRP deregisters the attributes in the LeaveAll messages if it does not receive any Join message for the attributes before the Leave timer expires.

- LeaveAll timer

On startup, an MRP participant starts its own LeaveAll timer. When the LeaveAll timer expires, MRP sends out a LeaveAll message and restarts the LeaveAll timer. On receiving the LeaveAll message, other participants re-register all the attributes and re-start their LeaveAll timer.

When you configure the MRP timers, follow these guidelines:

- When the LeaveAll timer of an MRP participant expires, the MRP participant sends LeaveAll messages to the remote participants. On receiving a LeaveAll message, a remote participant restarts its LeaveAll timer, and stops sending out LeaveAll messages. This mechanism effectively reduces the number of LeaveAll messages in the network.

- To avoid the case that the LeaveAll timer of a fixed participant always first expires, the switch randomly changes the LeaveAll timer within a certain range when the MRP participant restarts its LeaveAll timer.

    - MVRP registration modes

The VLAN information propagated by MVRP includes not only locally, manually configured static VLAN information but also dynamic VLAN information from other devices.

VLANs created manually, locally are called "static VLANs", and VLANs learned through MVRP are called "dynamic VLANs". The following MVRP registration modes are available.

- Normal

An MVRP participant in normal registration mode performs dynamic VLAN registrations and deregistrations, and sends declarations and withdrawals for dynamic and static VLANs.

- Fixed

An MVRP participant in fixed registration mode disables deregistering dynamic VLANs, sends declarations for dynamic VLANs and static VLANs, and drops received MVRP protocol packets. As a result, an MVRP participant port in fixed registration mode does not deregister or register dynamic VLANs.

- Forbidden

An MVRP participant in forbidden registration mode disables registering dynamic VLANs, sends declarations for dynamic VLANs and static VLANs, and drops received MVRP protocol packets. As a result, an MVRP participant in forbidden registration mode does not register dynamic VLANs, and does not re-register a dynamic VLAN when the VLAN is deregistered.

    - Protocols and standards

IEEE 802.1ak IEEE Standard for Local and Metropolitan Area Networks: Virtual Bridged Local Area Networks – Amendment 07: Multiple Registration Protocol

  - MVRP configuration task list

| Task | Remarks |
| --- | --- |
| Enabling MVRP | Required. |
| Configuring the MVRP registration mode | Optional. |
| Configuring MRP timers | Optional. |
| Enabling GVRP compatibility | Optional. |

  - Configuration prerequisites

Before configuring MVRP, perform the following tasks:

- Make sure that all MSTIs in the network are effective and each MSTI is mapped to an existing VLAN on each device in the network, because MVRP runs on a per-MSTI basis.

- Configure the involved ports as trunk ports, because MVRP is available only on trunk ports.

  - Enabling MVRP

This section describes how to enable MVRP.

    - Configuration restrictions and guidelines

- MVRP can work with STP, RSTP, or MSTP, but not other link layer topology protocols, including PVST, RRPP, and Smart Link. Ports blocked by STP, RSTP, or MSTP can receive and send MVRP protocol packets. For more information about STP, RSTP, MSTP, and PVST, see " Layer 2—LAN Switching Configuration Guide." For more information about RRPP and Smart Link, see High Availability Configuration Guide.

- Do not enable both MVRP and remote port mirroring on a port. Otherwise, MVRP may register the remote probe VLAN to incorrect ports, which would cause the monitor port to receive undesired duplicates. For more information about port mirroring, see Network Management and Monitoring Configuration Guide.

- Enabling MVRP on a Layer 2 aggregate interface enables both the aggregate interface and all Selected member ports in the link aggregation group to participate in dynamic VLAN registration and deregistration.

    - Configuration procedure

To enable MVRP:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enable MVRP globally. | mvrp global enable | By default, MVRP is globally disabled. To enable MVRP on a port, first enable MVRP globally. |
| ##### Enter interface view. | - Enter Layer 2 Ethernet interface view or Layer 2 aggregate interface view: interface interface-type interface-number - Enter port group view: port-group manual port-group-name | Use one of the commands. |
| ##### Configure the port to permit the specified VLANs. | port trunk permit vlan { vlan-list ¦ all } | By default, a trunk port permits only VLAN 1. Make sure that the trunk port permits all registered VLANs. For more information about the port trunk permit vlan command, see Layer 2—LAN Switching Command Reference. |
| ##### Enable MVRP on the port. | mvrp enable | By default, MVRP is disabled on a port. |

  - Configuring the MVRP registration mode

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter interface view. | - Enter Layer 2 Ethernet interface view or Layer 2 aggregate interface view: interface interface-type interface-number - Enter port group view: port-group manual port-group-name | Use one of the commands. |
| ##### Configure the MVRP registration mode. | mvrp registration { fixed ¦ forbidden ¦ normal } | Optional. The default setting is normal registration mode. |

  - Configuring MRP timers

|  | CAUTION: The MRP timers apply to all MRP applications, for example, MVRP, on a port. To avoid frequent VLAN registrations and deregistrations, use the same MRP timers throughout the network. |
| --- | --- |

Each port maintains its own Periodic, Join, and LeaveAll timers, and each attribute of a port maintains a Leave timer.

To configure MRP timers:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view. | system-view | N/A |
| ##### Enter interface view. | - Enter Layer 2 Ethernet interface view or Layer 2 aggregate interface view: interface interface-type interface-number - Enter port group view: port-group manual port-group-name | Use one of the commands. |
| ##### Configure the LeaveAll timer. | mrp timer leaveall timer-value | Optional. The default setting is 1000 centiseconds. |
| ##### Configure the Join timer. | mrp timer join timer-value | Optional. The default setting is 20 centiseconds. |
| ##### Configure the Leave timer. | mrp timer leave timer-value | Optional. The default setting is 60 centiseconds. |
| ##### Configure the Periodic timer. | mrp timer periodic timer-value | Optional. The default setting is 100 centiseconds. |

Table 13 shows the value ranges for Join, Leave, and LeaveAll timers and their dependencies.

- If you set a timer to a value beyond the allowed value range, your configuration will fail. To do that, you can change the allowed value range by tuning the value of another related timer.

- To restore the default settings of the timers, restore the Join timer first, followed by the Leave and LeaveAll timers.

        - Dependencies of the Join, Leave, and LeaveAll timers

| Timer | Lower limit | Upper limit |
| --- | --- | --- |
| Join | 20 centiseconds | Half the Leave timer |
| Leave | Twice the Join timer | LeaveAll timer |
| LeaveAll | Leave timer on each port | 32760 centiseconds |

You can restore the Periodic timer to the default at any time.

  - Enabling GVRP compatibility

|  | CAUTION: - MVRP with GVRP compatibility enabled can work together with STP or RSTP, but cannot work together with MSTP. When MVRP with GVRP compatibility enabled works with MSTP, the network might operate improperly. - When GVRP compatibility is enabled for MVRP, HP recommends disabling the Period timer. Otherwise, the VLAN status might frequently change when the system is busy. |
| --- | --- |

MVRP can be compatible with GVRP. When the peer device supports GVRP, you can enable GVRP compatibility on the local end, so that the local end can receive and send MVRP and GVRP protocol packets at the same time.

To enable GVRP compatibility:

| Step | Command | Remarks |
| --- | --- | --- |
| - Enter system view | system-view | N/A |
| ##### Enable GVRP compatibility | mvrp gvrp-compliance enable | By default, GVRP compatibility is disabled. |

  - Displaying and maintaining MVRP

| Task | Command | Remarks |
| --- | --- | --- |
| Display the MVRP status of the specified port and each MVRP interface in the specified VLAN. | display mvrp state interface interface-type interface-number vlan vlan-id [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display the MVRP running status. | display mvrp running-status [ interface interface-list ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display the MVRP statistics. | display mvrp statistics [ interface interface-list ] [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Display the dynamic VLAN operation information of the specified port. | display mvrp vlan-operation interface interface-type interface-number [ ¦ { begin ¦ exclude ¦ include } regular-expression ] | Available in any view |
| Clear the MVRP statistics of the specified ports. | reset mvrp statistics [ interface interface-list ] | Available in user view |

  - Configuration example for MVRP in normal registration mode

    - Network requirements

As shown in Figure 4, configure MSTP, map VLAN 10 to MSTI 1, map VLAN 20 MST 2, and map the other VLANs to MSTI 0.

Configure MVRP and set the MVRP registration mode to normal, so that Device A, Device B, Device C, and Device D can register and deregister dynamic and static VLANs and keep identical VLAN configuration for each MSTI.

When the network is stable, set the MVRP registration mode to fixed on the port that connecting Device B to Device A, so that the dynamic VLANs on Device B are not de-registered.

Network diagram

    - Configuration procedure

      - Configuring Device A

# Enter MST region view.

<DeviceA> system-view

[DeviceA] stp region-configuration

# Configure the MST region name, VLAN-to-instance mappings, and revision level.

[DeviceA-mst-region] region-name example

[DeviceA-mst-region] instance 1 vlan 10

[DeviceA-mst-region] instance 2 vlan 20

[DeviceA-mst-region] revision-level 0

# Manually activate the MST region configuration.

[DeviceA-mst-region] active region-configuration

[DeviceA-mst-region] quit

# Configure Device A as the primary root bridge of MSTI 1.

[DeviceA] stp instance 1 root primary

# Globally enable the spanning tree feature.

[DeviceA] stp enable

# Globally enable MVRP.

[DeviceA] mvrp global enable

# Configure port GigabitEthernet 1/0/1 as a trunk port, and configure it to permit all VLANs.

[DeviceA] interface gigabitethernet 1/0/1

[DeviceA-GigabitEthernet1/0/1] port link-type trunk

[DeviceA-GigabitEthernet1/0/1] port trunk permit vlan all

# Enable MVRP on port GigabitEthernet 1/0/1.

[DeviceA-GigabitEthernet1/0/1] mvrp enable

[DeviceA-GigabitEthernet1/0/1] quit

# Configure port GigabitEthernet1/0/2 as a trunk port, and configure it to permit VLAN 40.

[DeviceA] interface gigabitethernet 1/0/2

[DeviceA-GigabitEthernet1/0/2] port link-type trunk

[DeviceA-GigabitEthernet1/0/2] port trunk permit vlan 40

# Enable MVRP on port GigabitEthernet1/0/2.

[DeviceA-GigabitEthernet1/0/2] mvrp enable

[DeviceA-GigabitEthernet1/0/2] quit

# Configure port GigabitEthernet 1/0/3 as a trunk port, and configure it to permit all VLANs.

[DeviceA] interface gigabitethernet 1/0/3

[DeviceA-GigabitEthernet1/0/3] port link-type trunk

[DeviceA-GigabitEthernet1/0/3] port trunk permit vlan all

# Enable MVRP on port GigabitEthernet 1/0/3.

[DeviceA-GigabitEthernet1/0/3] mvrp enable

[DeviceA-GigabitEthernet1/0/3] quit

# Create VLAN 10.

[DeviceA] vlan 10

[DeviceA-vlan10] quit

      - Configuring Device B

# Enter MST region view.

<DeviceB> system-view

[DeviceB] stp region-configuration

# Configure the MST region name, VLAN-to-instance mappings, and revision level.

[DeviceB-mst-region] region-name example

[DeviceB-mst-region] instance 1 vlan 10

[DeviceB-mst-region] instance 2 vlan 20

[DeviceB-mst-region] revision-level 0

# Manually activate the MST region configuration.

[DeviceB-mst-region] active region-configuration

[DeviceB-mst-region] quit

# Configure Device B as the primary root bridge of MSTI 2.

[DeviceB] stp instance 2 root primary

# Globally enable the spanning tree feature.

[DeviceB] stp enable

# Globally enable MVRP.

[DeviceB] mvrp global enable

# Configure port GigabitEthernet 1/0/1 as a trunk port, and configure it to permit VLANs 20 and 40.

[DeviceB] interface gigabitethernet 1/0/1

[DeviceB-GigabitEthernet1/0/1] port link-type trunk

[DeviceB-GigabitEthernet1/0/1] port trunk permit vlan 20 40

# Enable MVRP on port GigabitEthernet 1/0/1.

[DeviceB-GigabitEthernet1/0/1] mvrp enable

[DeviceB-GigabitEthernet1/0/1] quit

# Configure port GigabitEthernet1/0/2 as a trunk port, and configure it to permit all VLANs.

[DeviceB] interface gigabitethernet 1/0/2

[DeviceB-GigabitEthernet1/0/2] port link-type trunk

[DeviceB-GigabitEthernet1/0/2] port trunk permit vlan all

# Enable MVRP on port GigabitEthernet1/0/2.

[DeviceB-GigabitEthernet1/0/2] mvrp enable

[DeviceB-GigabitEthernet1/0/2] quit

# Configure port GigabitEthernet 1/0/3 as a trunk port, and configure it to permit all VLANs.

[DeviceB] interface gigabitethernet 1/0/3

[DeviceB-GigabitEthernet1/0/3] port link-type trunk

[DeviceB-GigabitEthernet1/0/3] port trunk permit vlan all

# Enable MVRP on port GigabitEthernet 1/0/3.

[DeviceB-GigabitEthernet1/0/3] mvrp enable

[DeviceB-GigabitEthernet1/0/3] quit

# Create VLAN 20.

[DeviceB] vlan 20

[DeviceB-vlan20] quit

      - Configuring Device C

# Enter MST region view.

<DeviceC> system-view

[DeviceC] stp region-configuration

# Configure the MST region name, VLAN-to-instance mappings, and revision level.

[DeviceC-mst-region] region-name example

[DeviceC-mst-region] instance 1 vlan 10

[DeviceC-mst-region] instance 2 vlan 20

[DeviceC-mst-region] revision-level 0

# Manually activate the MST region configuration.

[DeviceC-mst-region] active region-configuration

[DeviceC-mst-region] quit

# Configure Device C as the root bridge of MSTI 0.

[DeviceC] stp instance 0 root primary

# Globally enable the spanning tree feature.

[DeviceC] stp enable

# Globally enable MVRP.

[DeviceC] mvrp global enable

# Configure port GigabitEthernet 1/0/1 as a trunk port, and configure it to permit all VLANs.

[DeviceC] interface gigabitethernet 1/0/1

[DeviceC-GigabitEthernet1/0/1] port link-type trunk

[DeviceC-GigabitEthernet1/0/1] port trunk permit vlan all

# Enable MVRP on port GigabitEthernet 1/0/1.

[DeviceC-GigabitEthernet1/0/1] mvrp enable

[DeviceC-GigabitEthernet1/0/1] quit

# Configure port GigabitEthernet1/0/2 as a trunk port, and configure it to permit all VLANs.

[DeviceC] interface gigabitethernet 1/0/2

[DeviceC-GigabitEthernet1/0/2] port link-type trunk

[DeviceC-GigabitEthernet1/0/2] port trunk permit vlan all

# Enable MVRP on port GigabitEthernet1/0/2.

[DeviceC-GigabitEthernet1/0/2] mvrp enable

[DeviceC-GigabitEthernet1/0/2] quit

      - Configuring Device D

# Enter MST region view.

<DeviceD> system-view

[DeviceD] stp region-configuration

# Configure the MST region name, VLAN-to-instance mappings, and revision level.

[DeviceD-mst-region] region-name example

[DeviceD-mst-region] instance 1 vlan 10

[DeviceD-mst-region] instance 2 vlan 20

[DeviceD-mst-region] revision-level 0

# Manually activate the MST region configuration.

[DeviceD-mst-region] active region-configuration

[DeviceD-mst-region] quit

# Globally enable the spanning tree feature.

[DeviceD] stp enable

# Globally enable MVRP.

[DeviceD] mvrp global enable

# Configure port GigabitEthernet 1/0/1 as a trunk port, and configure it to permit VLANs 20 and 40.

[DeviceD] interface gigabitethernet 1/0/1

[DeviceD-GigabitEthernet1/0/1] port link-type trunk

[DeviceD-GigabitEthernet1/0/1] port trunk permit vlan 20 40

# Enable MVRP on port GigabitEthernet 1/0/1.

[DeviceD-GigabitEthernet1/0/1] mvrp enable

[DeviceD-GigabitEthernet1/0/1] quit

# Configure port GigabitEthernet1/0/2 as a trunk port, and configure it to permit VLAN 40.

[DeviceD] interface gigabitethernet 1/0/2

[DeviceD-GigabitEthernet1/0/2] port link-type trunk

[DeviceD-GigabitEthernet1/0/2] port trunk permit vlan 40

# Enable MVRP on port GigabitEthernet1/0/2.

[DeviceD-GigabitEthernet1/0/2] mvrp enable[DeviceD-GigabitEthernet1/0/2] quit

      - Verifying the configuration

            - Verify the normal registration mode configuration.

Use the display mvrp running-status command to display the local MVRP VLAN information to verify whether the configuration takes effect.

# Check the local VLAN information on Device A.

[DeviceA] display mvrp running-status

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/1]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default),

----[GigabitEthernet1/0/2]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default),

----[GigabitEthernet1/0/3]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 20,

The output shows that: ports GigabitEthernet 1/0/1 and GigabitEthernet1/0/2 have learned only VLAN 1 through MVRP; port GigabitEthernet 1/0/3 has learned VLAN 1 and dynamic VLAN 20 created on Device B through MVRP.

# Check the local VLAN information on Device B.

[DeviceB] display mvrp running-status

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/1]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default),

----[GigabitEthernet1/0/2]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 10,

----[GigabitEthernet1/0/3]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 10,

The output shows that: port GigabitEthernet 1/0/1 has learned only VLAN 1 through MVRP; ports GigabitEthernet 1/0/2 and GigabitEthernet 1/0/3 have learned VLAN 1 and dynamic VLAN 10 created on Device A through MVRP.

# Check the local VLAN information on Device C.

[DeviceC] display mvrp running-status

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/1]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 10, 20,

----[GigabitEthernet1/0/2]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 20,

The output shows that: port GigabitEthernet 1/0/1 has learned VLAN 1, dynamic VLAN 10 created on Device A, and dynamic VLAN 20 created on Device B through MVRP; port GigabitEthernet1/0/2 has learned VLAN 1 and dynamic VLAN 20 created on Device B through MVRP.

# Check the local VLAN information on Device D.

[DeviceD] display mvrp running-status

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/1]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 20,

----[GigabitEthernet1/0/2]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default),

The output shows that: port GigabitEthernet 1/0/1 has learned VLAN 1 and dynamic VLAN 20 created on Device B through MVRP; port GigabitEthernet1/0/2 has learned only VLAN 1 through MVRP.

            - Change the registration mode and verify the configuration.

Set the MVRP registration mode to fixed on GigabitEthernet 1/0/3 of Device B, so that the dynamic VLANs that Device B learns in VLAN 1 are not de-registered.

# Set the MVRP registration mode to fixed on GigabitEthernet 1/0/3.

[DeviceB] interface gigabitethernet 1/0/3

[DeviceB-GigabitEthernet1/0/3] mvrp registration fixed

[DeviceB-GigabitEthernet1/0/3] quit

# Display the local MVRP VLAN information on GigabitEthernet 1/0/3.

[DeviceB] display mvrp running-status interface gigabitethernet 1/0/3

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/3]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Fixed

Local VLANs :

1(default), 10,

The output shows that the VLAN information on GigabitEthernet 1/0/3 is not changed after you set the MVRP registration mode to fixed on GigabitEthernet 1/0/3.

# Delete VLAN 10 on Device A.

[DeviceA] undo vlan 10

# Display the local MVRP VLAN information on GigabitEthernet 1/0/3.

[DeviceB] display mvrp running-status interface gigabitethernet 1/0/3

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/3]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Fixed

Local VLANs :

1(default), 10,

The output shows that the dynamic VLAN information on GigabitEthernet 1/0/3 is not changed after you set the MVRP registration mode to fixed on GigabitEthernet 1/0/3.

Command reference

    - display mvrp running-status

Use display mvrp running-status to display the MVRP running status.

Syntax

display mvrp running-status [ interface interface-list ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

1: Monitor level

Parameters

interface interface-list: Specifies an Ethernet interface list in the form of interface-list = { interface-type interface-number1 [ to interface-type interface-number2 ] }&<1-10>, where interface-type interface-number specifies an interface by its type and number and &<1-10> indicates that you can specify up to 10 interface-type interface-number1 [ to interface-type interface-number2 ] parameters. If this option is not specified, this command displays MVRP running status of all MVRP-enabled trunk ports.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Examples

# Display the MVRP running status of all ports.

<Sysname> display mvrp running-status

-------[MVRP Global Info]-------

Global Status     : Enabled

Compliance-GVRP   : False

----[GigabitEthernet1/0/1]----

Config  Status                      : Enabled

Running Status                      : Enabled

Join Timer                          : 20 (centiseconds)

Leave Timer                         : 60 (centiseconds)

Periodic Timer                      : 100 (centiseconds)

LeaveAll Timer                      : 1000 (centiseconds)

Registration Type                   : Normal

Local VLANs :

1(default), 2-10,

        - Command output

| Field | Description |
| --- | --- |
| MVRP Global Info | Global MVRP information. |
| Global Status | Global MVRP status: - Enabled - Disabled |
| Compliance-GVRP | GVRP compatibility status: - True—Compatible - False—Incompatible |
| ----[GigabitEthernet1/0/1] ---- | Interface prompt. The information between the current interface prompt and the next interface prompt is information about the current interface. |
| Config Status | Whether MVRP is enabled on the port: - Enabled - Disabled |
| Running Status | Whether MVRP takes effect on the port (determined by the link state and MVRP enabling status of the port): - Enabled - Disabled |
| Join Timer | Join timer, in centiseconds. |
| Leave Timer | Leave timer, in centiseconds. |
| Periodic Timer | Periodic timer, in centiseconds. |
| LeaveAll Timer | LeaveAll timer, in centiseconds. |
| Registration Type | MVRP registration mode: - Fixed - Forbidden - Normal |
| Local VLANs | VLAN information in the local database, which displays the VLANs learned through MVRP. |

    - display mvrp state

Use display mvrp state to display the MVRP state of an interface in a VLAN.

Syntax

display mvrp state interface interface-type interface-number vlan vlan-id [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

0: Visit level

Parameters

interface interface-type interface-number: Displays the MVRP state of an interface specified by its type and number.

vlan vlan-id: Displays the MVRP state of an interface in an VLAN specified by its VLAN ID, which ranges from 1 to 4094.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Examples

# Display the MVRP state of port GigabitEthernet 1/0/1 in VLAN 2.

<Sysname> display mvrp state interface gigabitethernet 1/0/1 vlan 2

MVRP state of VLAN 2 on port GE1/0/1

Port      VLAN   App-state   Reg-state

----------- ------ ----------- -----------

GE1/0/1          2       VP          IN

        - Command output

| Field | Description |
| --- | --- |
| MVRP state of VLAN 2 on port GE1/0/1 | MVRP state of GigabitEthernet 1/0/1 in VLAN 2. |
| App-state | Declaration state, which indicates the state of the attribute that the local participant declares to the remote participant. The state can be VO, VP, VN, AN, AA, QA, LA, AO, QO, AP, QP, or LO. Each state consists of two letters. The first letter indicates the state: - V—Very anxious, which means that the local participant has not declared the attribute or has not received any Join message containing the attribute. - A—Anxious, which means that the local participant has declared the attribute once or has received one Join message containing the attribute. - Q—Quiet, which means that the local participant has declared the attribute two times, the local participant has declared the attribute once and has received one Join message containing the attribute, or the local participant has received two Join messages containing the attribute. - L—Leaving, which means that the local participant is deregistering the attribute. The second letter indicates the membership state: - A—Active member, which means that the local participant is declaring the attribute, has sent at least one Join message containing the attribute, and may receive Join messages. - P—Passive member, which means that the local participant is declaring the attribute, has received Join messages containing the attribute, but has not sent Join messages containing the attribute. - O—Observer, which means that the local participant is not declaring the attribute but is monitoring the attribute. - N—New, which means that the local participant is declaring the attribute, is receiving the Join message containing the attribute, but is not sending Join messages for the attribute. For example, VP indicates "Very anxious, Passive member". |
| Reg-state | Registration state of attributes declared by remote participants on the local end. The state can be IN, LV, or MT: - IN—Registered. - LV—Previously registered, but now being timed out. - MT—Not registered. |

    - display mvrp statistics

Use display mvrp statistics to display MVRP statistics.

Syntax

display mvrp statistics [ interface interface-list ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

1: Monitor level

Parameters

interface interface-list: Specifies an Ethernet interface list in the form of interface-list = { interface-type interface-number1 [ to interface-type interface-number2 ] }&<1-10>, where interface-type interface-number specifies an interface by its type and number and &<1-10> indicates that you can specify up to 10 interfaces or interface ranges. If this option is not specified, this command displays MVRP statistics of all MVRP-enabled trunk ports.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Examples

# Display MVRP statistics of all MVRP-enabled ports.

<Sysname> display mvrp statistics

----[GigabitEthernet1/0/1]----

Failed Registrations           : 1

Last PDU Origin                : 000f-e200-0010

Frames Received                : 201

New Event Received                 : 0

JoinIn Event Received              : 1167

In Event Received                  : 0

JoinMt Event Received              : 22387

Mt Event Received                  : 31

Leave Event Received               : 210

LeaveAll Event Received            : 63

Frames Transmitted             : 47

New Event Transmitted              : 0

JoinIn Event Transmitted           : 311

In Event Transmitted               : 0

JoinMt Event Transmitted           : 873

Mt Event Transmitted               : 11065

Leave Event Transmitted            : 167

LeaveAll Event Transmitted         : 4

Frames Discarded               : 0

----[GigabitEthernet1/0/2]----

Failed Registrations           : 0

Last PDU Origin                : 0000-0000-0000

Frames Received                : 0

New Event Received                 : 0

JoinIn Event Received              : 0

In Event Received                  : 0

JoinMt Event Received              : 0

Mt Event Received                  : 0

Leave Event Received               : 0

LeaveAll Event Received            : 0

Frames Transmitted             : 0

New Event Transmitted              : 0

JoinIn Event Transmitted           : 0

In Event Transmitted               : 0

JoinMt Event Transmitted           : 0

Mt Event Transmitted               : 0

Leave Event Transmitted            : 0

LeaveAll Event Transmitted         : 0

Frames Discarded               : 0

        - Command output

| Field | Description |
| --- | --- |
| ----[GigabitEthernet1/0/1]---- | Interface prompt. The statistics between the current interface prompt and the next interface prompt are statistics of the current interface. |
| Failed Registrations | Number of VLAN registration failures through MVRP on the local end. |
| Last PDU Origin | Source MAC address of the last MVRP PDU. |
| Frames Received | Number of MVRP protocol packets received |
| New Event Received | Number of New attribute events received. |
| JoinIn Event Received | Number of JoinIn attribute events received. |
| In Event Received | Number of In attribute events received. |
| JoinMt Event Received | Number of JoinMt attribute events received. |
| Mt Event Received | Number of Mt attribute events received. |
| Leave Event Received | Number of Leave attribute events received. |
| LeaveAll Event Received | Number of LeaveAll attribute events received. |
| Frames Transmitted | Number of MVRP protocol packets sent. |
| New Event Transmitted | Number of New attribute events sent. |
| JoinIn Event Transmitted | Number of JoinIn attribute events sent. |
| In Event Transmitted | Number of In attribute events sent. |
| JoinMt Event Transmitted | Number of JoinMt attribute events sent. |
| Mt Event Transmitted | Number of Mt attribute events sent. |
| Leave Event Transmitted | Number of Leave attribute events sent. |
| LeaveAll Event Transmitted | Number of LeaveAll attribute events sent. |
| Frames Discarded | Number of MVRP protocol packets dropped. |

    - display mvrp vlan-operation

Use display mvrp vlan-operation to display the dynamic VLAN operations of an interface.

Syntax

display mvrp vlan-operation interface interface-type interface-number [ | { begin | exclude | include } regular-expression ]

Views

Any view

Default command level

0: Visit level

Parameters

interface interface-type interface-number: Displays the dynamic VLAN operations of an interface specified its type and number.

|: Filters command output by specifying a regular expression. For more information about regular expressions, see Fundamentals Configuration Guide.

begin: Displays the first line that matches the specified regular expression and all lines that follow.

exclude: Displays all lines that do not match the specified regular expression.

include: Displays all lines that match the specified regular expression.

regular-expression: Specifies a regular expression, a case-sensitive string of 1 to 256 characters.

Usage guidelines

These dynamic VLANs refer to the VLANs that are dynamically learned through MVRP and have not taken effect on the local device.

If a dynamic VLAN learned through MVRP is an existing static VLAN on the device or a VLAN reserved for a protocol, the dynamic VLAN does not take effect on the local device.

Examples

# Display the dynamic VLAN operations of GigabitEthernet 1/0/1.

<Sysname> display mvrp vlan-operation interface gigabitethernet 1/0/1

Dynamic VLAN operations on port GigabitEthernet1/0/1

Operations of creating VLAN:  2-100

Operations of deleting VLAN:  none

Operations of adding VLAN to Trunk:  2-100

Operations of deleting VLAN from Trunk:  none

        - Command output

| Field | Description |
| --- | --- |
| Operations of adding VLAN to Trunk | Operations of adding VLANs to trunk ports |
| Operations of deleting VLAN from Trunk | Operations of removing VLAN from trunk ports |

    - mrp timer join

Use mrp timer join to set the Join timer.

Use undo mrp timer join to restore the default.

Syntax

mrp timer join timer-value

undo mrp timer join

Default

The Join timer is 20 centiseconds.

Views

Layer 2 Ethernet port view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Parameters

timer-value: Specifies the Join timer value (in centiseconds). The Join timer must be less than half the Leave timer, and must be a multiple of 20.

Usage guidelines

You will fail to restore the default Join timer if the default Join timer is not less than half the Leave timer.

Examples

# Set the Join timer to 40 centiseconds. (Suppose the Leave timer is 100 centiseconds)

<Sysname> system-view

[Sysname] interface ethernet 1/1

[Sysname-GigabitEthernet1/0/1] mrp timer join 40

Related commands

- display mvrp running-status

- mrp timer leave

    - mrp timer leave

Use mrp timer leave to set the Leave timer.

Use undo mrp timer leave to restore the default.

Syntax

mrp timer leave timer-value

undo mrp timer leave

Default

The Leave timer is 60 centiseconds.

Views

Layer 2 Ethernet port view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Parameters

timer-value: Specifies the Leave timer value (in centiseconds). The Leave timer must be greater than two times the Join timer, less than the LeaveAll timer, and a multiple of 20.

Usage guidelines

You will fail to restore the default Leave timer if the default Leave timer is not greater than two times the Join timer or not less than the LeaveAll timer.

Examples

# Set the Leave timer to 100 centiseconds. (Suppose the Join and LeaveAll timer use their default settings)

<Sysname> system-view

[Sysname] interface ethernet 1/1

[Sysname-GigabitEthernet1/0/1] mrp timer leave 100

Related commands

- display mvrp running-status

- mrp timer join

- mrp timer leaveall

    - mrp timer leaveall

Use mrp timer leaveall to set the LeaveAll timer.

Use undo mrp timer leaveall to restore the default.

Syntax

mrp timer leaveall timer-value

undo mrp timer leaveall

Default

The LeaveAll timer is 1000 centiseconds.

Views

Layer 2 Ethernet port view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Parameter

timer-value: Specifies the LeaveAll timer value (in centiseconds). The LeaveAll timer must be greater than any Leave timer on each port, no greater than 32760, and a multiple of 20.

Usage guidelines

You will fail to restore the default LeaveAll timer if the default LeaveAll timer is not greater than any Leave timer on each port.

Each time when the LeaveAll timer of a port expires, all attributes of the MSTIs on the port are deregistered throughout the network, and such a deregistration affects a large portion of the network. Do not set too small a value for the LeaveAll timer, and make sure that the LeaveAll timer is greater than any Leave timer on each port.

To keep the dynamic VLANs learned through MVRP stable, do not set the LeaveAll timer smaller than its default value (1000 centiseconds).

To avoid the case that the LeaveAll timer of a fixed participant always first expires, the switch randomly changes the LeaveAll timer within a certain range when the MRP participant restarts its LeaveAll timer.

Examples

# Set the LeaveAll timer to 1500 centiseconds. (Suppose the Leave timer is restored to the default)

<Sysname> system-view

[Sysname] interface ethernet 1/1

[Sysname-GigabitEthernet1/0/1] mrp timer leaveall 1500

Related commands

- display mvrp running-status

- mrp timer leave

    - mrp timer periodic

Use mrp timer periodic to set the Periodic timer.

Use undo mrp timer periodic to restore the default.

Syntax

mrp timer periodic timer-value

undo mrp timer periodic

Default

The Periodic timer is 100 centiseconds.

Views

Layer 2 Ethernet port view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Parameters

timer-value: Specifies the Periodic timer (in centiseconds), which can be 0 or 100.

Usage guidelines

Setting the Periodic timer to 0 centiseconds disables the Periodic timer.

Setting the Periodic timer to 100 centiseconds enables the Periodic timer.

Examples

# Set the Periodic timer to 0 centiseconds.

<Sysname> system-view

[Sysname] interface ethernet 1/1

[Sysname-GigabitEthernet1/0/1] mrp timer periodic 0

Related commands

display mvrp running-status

    - mvrp global enable

Use mvrp global enable to enable MVRP globally.

Use undo mvrp global enable to restore the default.

Syntax

mvrp global enable

undo mvrp global enable

Default

MVRP is disabled globally.

Views

System view

Default command level

2: System level

Usage guidelines

Disabling MVRP globally also disables MVRP on all ports.

Examples

# Enable MVRP globally.

<Sysname> system-view

[Sysname] mvrp global enable

Related commands

- display mvrp running-status

- mvrp enable

    - mvrp enable

Use mvrp enable to enable MVRP on a port.

Use undo mvrp enable to disable MVRP on a port.

Syntax

mvrp enable

undo mvrp enable

Default

MVRP is disabled on a port.

Views

Layer 2 Ethernet port view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Usage guidelines

To enable MVRP on a port, first enable MVRP globally.

Disabling MVRP globally also disables MVRP on each port.

This command is available only on trunk ports.

You cannot change the link type of MVRP-enabled trunk port.

Examples

# Configure GigabitEthernet 1/0/1 as a trunk port, and enable MVRP on it.

<Sysname> system-view

[Sysname] interface ethernet 1/1

[Sysname-GigabitEthernet1/0/1] port link-type trunk

[Sysname-GigabitEthernet1/0/1] port trunk permit vlan all

[Sysname-GigabitEthernet1/0/1] mvrp enable

Related commands

- display mvrp running-status

- mvrp global enable

    - mvrp gvrp-compliance

Use mvrp gvrp-compliance enable to enable GVRP compatibility, so that the device can process both MVRP protocol packets and GVRP protocol packets.

Use undo mvrp gvrp-compliance enable to restore the default.

Syntax

mvrp gvrp-compliance enable

undo mvrp gvrp-compliance enable

Default

GVRP compatibility is disabled.

Views

System view

Default command level

2: System level

Examples

# Enable GVRP compatibility.

<Sysname> system-view

[Sysname] mvrp gvrp-compliance enable

    - mvrp registration

Use mvrp registration to set the MVRP registration mode on the port.

Use undo mvrp registration to restore the default.

Syntax

mvrp registration { fixed | forbidden | normal }

undo mvrp registration

Default

The MVRP registration mode is normal.

Views

Layer 2 Ethernet port view, Layer 2 aggregate interface view, port group view

Default command level

2: System level

Parameters

fixed: Specifies the fixed registration mode.

forbidden: Specifies the forbidden registration mode.

normal: Specifies the normal registration mode.

Usage guidelines

This command is available only on trunk ports.

Examples

# Configure GigabitEthernet 1/0/1 as a trunk port, and set the MVRP registration mode to fixed on the port.

<Sysname> system-view

[Sysname] interface ethernet 1/1

[Sysname-GigabitEthernet1/0/1] port link-type trunk

[Sysname-GigabitEthernet1/0/1] port trunk permit vlan all

[Sysname-GigabitEthernet1/0/1] mvrp registration fixed

Related commands

display mvrp running-status

    - reset mvrp statistics

Use reset mvrp statistics to clear the MVRP statistics of ports.

Syntax

reset mvrp statistics [ interface interface-list ]

Views

User view

Default command level

2: System level

Parameters

interface interface-list: Specifies an Ethernet interface list in the form of interface-list = { interface-type interface-number1 [ to interface-type interface-number2 ] }&<1-10>, where interface-type interface-number specifies an interface by its type and number and &<1-10> indicates that you can specify up to 10 interfaces or interface ranges. If this option is not specified, the command clears MVRP statistics of all ports.

Examples

# Clear the MVRP statistics of all ports.

<Sysname> reset mvrp statistics

Related commands

display mvrp statistics

New feature: Enabling LLDP to automatically discover IP phones

  - Overview

In a traditional voice VLAN network, the switch maps the source MAC addresses of IP phones to a limited number of OUI addresses to allow them to access the network. This method restricts the types of IP phones on the network, if the IP phones with the source MAC addresses match the same OUI address are categorized as a type.

To break the restriction, you can enable the switch to automatically discover IP phones through LLDP. With this function, the switch can automatically discover the peer, and exchange LLDP TLVs with the peer. If the LLDP System Capabilities TLV received on a port shows that the peer is phone capable, the switch determines that the peer is an IP phone and sends an LLDP TLV carrying the voice VLAN configuration to the peer.

When the IP phone discovery process is complete, the port will automatically join the voice VLAN and improve the transmission priority of the voice traffic for the IP phone. To ensure that the IP phone can pass authentication, the switch will add the MAC address of the IP phone to the MAC address table.

|  | NOTE: This function is available only when your IP phone supports LLDP. Identify whether your IP phone supports LLDP by checking its usage guide. For more information about voice VLANs, see the chapter “Voice VLAN configuration.” |
| --- | --- |

  - Configuration prerequisites

Before you enable the switch to automatically discover IP phones through LLDP, complete the following tasks:

Enable LLDP globally and on ports.

Configure voice VLANs.

  - Configuration procedure

Follow these steps to enable LLDP to automatically discover IP phones:

| To do… | Use the command… | Remarks |
| --- | --- | --- |
| Enter system view | system-view | — |
| Enable LLDP to automatically discover IP phones | voice vlan track lldp | Required Disabled by default. |

|  | IMPORTANT: When the switch is enabled to automatically discover IP phones through LLDP, you can connect at most five IP phones to each port of the switch. You cannot use this function together with CDP compatibility. |
| --- | --- |

  - Command reference

    - voice vlan track lldp

Use the voice vlan track lldp command to enable LLDP to automatically discover IP phones.

Use the undo voice vlan track lldp command to disable LLDP from automatically discovering IP phones.

Syntax

voice vlan track lldp

undo voice vlan track lldp

Default

LLDP is disabled from automatically discovering IP phones.

Views

System view

Default command level

2: System level

Examples

# Enable the switch to automatically discover IP phones through LLDP.

<Sysname> system-view

[Sysname] voice vlan track lldp

# A5120EI-CMW520-R2210

This release has the following changes:

New feature: Displaying information about the patch package

New feature: Displaying alarm information

New feature: Configuring jumbo frame support on an Ethernet interface

New feature: Restoring the default settings for the interface

New feature: Enabling MAC address roaming

New feature: Assigning the port an aggregation priority

New feature: Setting the minimum number of Selected ports for an aggregation group

New feature: PVST

New feature: Configuring TC snooping

New feature: Setting the MTU for the VLAN interface

New feature: Restoring the default operating mode of CDP-compatible LLDP

New feature: PoE power negotiation through Power Via MDI TLV

New feature: Configuring multicast ARP

New feature: Specifying the IP address range for the DHCP clients of a specified vendor

New feature: Specifying a server’s IP address for the DHCP client

New feature: Configuring DHCP packet rate limit

New feature: Configuring DHCP snooping support for sub-option 9 in Option 82

New feature: Configuring TCP path MTU discovery

New feature: local ND proxy

New feature: Specifying the AFTR address

New feature: Displaying detailed information about neighbors

New feature: Configuring the interface as an uplink interface and disabling it from learning ND snooping entries

New feature: Configuring permanent static route

New feature: Enabling the IGMP snooping & MLD snooping host tracking function

New feature: PIM snooping & IPv6 PIM snooping

New feature: Configuring rule range remark

New feature: Configuring routing header type for an IPv6 ACL rule

New feature: Configuring byte-count or packet-based WFQ queuing

New feature: Configuring SP+WFQ queuing

New feature: Setting the validity time of the local user

New feature: Specifying the local user as a guest or guest manager

New feature: Setting the guest attribute for a user group

New feature: Authorizing a local user to use the Web service

New feature: Specifying ciphertext shared keys for RADIUS/HWTACACS servers

New feature: Specifying supported domain name delimiters

New feature: Enabling inactivity aging

New feature: Enabling the dynamic secure MAC function

New feature: Enabling SSL client weak authentication

New feature: Setting the maximum number of IPv4/IPv6 source guard binding entries

New feature: SAVI

New feature: Blacklist

New feature: Enabling Ethernet OAM remote loopback in user view and system view

New feature: Restoring the default Ethernet OAM connection mode

New feature: Configuring the collaboration between Smart Link and CC of CFD

Modified feature: Improving the isolate-user-VLAN usability

Modified feature: Installing patches in one step

Modified feature: Displaying file or directory information

Modified feature: Loopback interface numbering

Modified feature: Enabling address check

Modified feature: Enabling ND proxy

Modified feature: ND snooping

Modified feature: Displaying IPv6 FIB entries

Modified feature: Displaying the IPv6 information of an interface

Modified feature: Routing policy

Modified feature: CFD

Modified feature: Configuring the protected VLANs for the RRPP domain

Modified feature: Configuring the protected VLANs for a smart link group

Modified feature: Enabling traps globally

Modified feature: Configuring IP source guard

New feature: Displaying information about the patch package

Displaying information about the patch package

For more information about displaying information about the patch package, see "Software upgrade configuration" in HP A5120 EI Switch Series Fundamentals Configuration Guide-Release 2210.

Command reference

New commands: display patch.

For more information about this command, see "Software upgrade commands" in HP A5120 EI Switch Series Fundamentals Command Reference-Release 2210.

New feature: Displaying alarm information

Displaying alarm information

For more information about displaying alarm information, see "Device management configuration" in HP A5120 EI Switch Series Fundamentals Configuration Guide-Release 2210.

Command reference

New commands: display alarm.

For more information about this command, see "Device management commands" in HP A5120 EI Switch Series Fundamentals Command Reference-Release 2210.

New feature: Configuring jumbo frame support on an Ethernet interface

Configuring jumbo frame support on an Ethernet interface

For more information about jumbo frame support configuration, see "Ethernet interface configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

Modified command: The value argument was added to the jumboframe enable command.

For more information about this command, see "Ethernet interface configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Restoring the default settings for the interface

Restoring the default settings for the interface

For more information about restoring the default settings for the interface configuration, see "Ethernet interface configuration", "Loopback and null interface configuration", "Ethernet link aggregation configuration", and "VLAN configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: default.

For more information about this command, see "Ethernet interface configuration commands", "Loopback and null interface configuration commands", "Ethernet link aggregation configuration commands", and "VLAN configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Enabling MAC address roaming

Enabling MAC address roaming

For more information about MAC address roaming configuration, see "MAC address table configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: mac-address mac-roaming enable.

For more information about this command, see "MAC address table configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Assigning the port an aggregation priority

Assigning the port an aggregation priority

For more information about assigning the port an aggregation priority, see "Ethernet link aggregation configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: link-aggregation port-priority, which applies to dynamic and static aggregation groups.

Deleted command: lacp port-priority, which applies to only dynamic aggregation groups.

For more information about this command, see "Ethernet link aggregation configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Setting the minimum number of Selected ports for an aggregation group

Setting the minimum number of Selected ports for an aggregation group

For more information about configuring the minimum number of Selected ports for an aggregation group, see "Ethernet link aggregation configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: link-aggregation selected-port minimum.

For more information about this command, see "Ethernet link aggregation configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: PVST

Configuring PVST

For more information about PVST configuration, see "Spanning tree configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

Modified commands:

Keyword pvst was added to the stp mode command.

Option vlan vlan-list was added to the following commands:

display stp

display stp history

display stp tc

stp bridge-diameter

stp cost

stp enable (in system view)

stp port priority

stp port-log

stp priority

stp root primary

stp root secondary

stp timer forward-delay

stp timer hello

stp timer max-age.

For more information about PVST configuration commands, see "Spanning tree configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Configuring TC snooping

Configuring TC snooping

For more information about TC snooping configuration, see "Spanning tree configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: stp tc-snooping.

For more information about this command, see "Spanning tree configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Setting the MTU for the VLAN interface

Setting the MTU for the VLAN interface

For more information about VLAN interface MTU configuration, see "VLAN configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: mtu.

For more information about this command, see "VLAN configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: Restoring the default operating mode of CDP-compatible LLDP

Restoring the default operating mode of CDP-compatible LLDP

The undo lldp compliance admin-status cdp command was added to restore the default operating mode of CDP-compatible LLDP.

For more information about the operating mode of CDP-compatible LLDP, see "LLDP configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command reference

New command: undo lldp compliance admin-status cdp.

For more information about this command, see "LLDP configuration commands" in HP A5120 EI Switch Series Layer 2—LAN Switching Command Reference-Release 2210.

New feature: PoE power negotiation through Power Via MDI TLV

Configuring PoE power negotiation through Power Via MDI TLV

|  | NOTE: This feature is available on only PoE+ capable switches. |
| --- | --- |

The PSE devices that support this feature can autonegotiate the PoE power with the PD device connected to the port.

The feature does not need to be configured at the CLI. To implement the feature, you only need to enable PoE on the PSE device and PoE interface. After you use the poe max-power max-power command to configure the maximum power of a PoE interface, the configured value takes effect, and the PoE power autonegotiation feature is not supported.

For more information about "PoE configuration", see HP A5120 EI Switch Series Network Management and Monitoring Configuration Guide-Release 2210.

Command reference

You can use the display lldp local-information command and the display lldp neighbor-information command to view the power autonegotiation state.

    - Modified command: display lldp local-information

Syntax

display lldp local-information [ global | interface interface-type interface-number ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

The PoE power autonegotiation information was added to the output.

# Display all LLDP information to be sent. (The example describes only the newly added fields in the output.)

<Sysname> display lldp local-information

...

LLDP local-information of port 1[GigabitEthernet1/0/1]:

...

Power type                : Type 2 PSE

Power source              : Primary

Power priority            : High

PD requested power value  : 25.5(w)

PSE allocated power value : 25.5(w)

...

Command output

| Field | Description |
| --- | --- |
| Power type | Power type when the device supports PoE+. Type 2 PSE supplies power from 0 to 30 W, a voltage from 50 to 57 V, and a maximum current of 600 mA. |
| Power source | Power supply type of a PSE when the device supports PoE+: Unknown—Unknown power supply. Primary—Primary power supply. Backup—Backup power supply. |
| Power priority | Power supply priority on a PSE when the device supports PoE+: Unknown—Unknown priority. Critical—Priority 1. High—Priority 2. Low—Priority 3. |
| PD requested power value | Power (in watts) required by the PD that connects to the port. This field appears only on the devices that support PoE+. |
| PSE allocated power value | Power (in watts) supplied by the PSE to the connecting port. This field appears only on the devices that support PoE+. |

    - Modified command: display lldp neighbor-information

Syntax

display lldp neighbor-information [ brief | interface interface-type interface-number [ brief ] | list [ system-name system-name ] ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

The PoE power autonegotiation information was added to the output.

# Display the LLDP information sent from the neighboring devices received through all ports. (The example describes only the newly added fields in the output.)

<Sysname> display lldp neighbor-information

...

LLDP neighbor-information of port 1[GigabitEthernet1/0/1]:

...

Power type                : Type 2 PD

Power source              : PSE and local

Power priority            : High

PD requested power value  : 25.5(w)

PSE allocated power value : 25.5(w)

...

Command output

| Field | Description |
| --- | --- |
| Power type | This field appears only on the devices that support PoE+. PD type of an LLDP neighboring device which is a PD device: Type 1 PD—This type power from 0 to 15.4 W, a voltage from 44 to 57 V, and a maximum current of 350 mA. Type 2 PD—This type requires power from 0 to 30 W, a voltage from 50 to 57 V, and a maximum current of 600 mA. |
| Power source | This field appears only on the devices that support PoE+. Power source type of an LLDP neighboring device which is a PD device: Unknown—Unknown power supply. PSE—PSE power supply. Local—Local power supply. PSE and local—PSE and local power supply. |
| Power priority | This field appears only on the devices that support PoE+. Powered priority of ports on an LLDP neighboring device which is a PD device: Unknown—Unknown priority. Critical—Priority 1. High—Priority 2. Low—Priority 3. |
| PD requested power value | This field appears only on the devices that support PoE+. Power (in watts) requested by the LLDP neighboring device which is a PD device. |
| PSE allocated power value | This field appears only on the devices that support PoE+. Power (in watts) supplied by the PSE to the LLDP neighboring device which is a PD device. |

New feature: Configuring multicast ARP

Configuring multicast ARP

For more information about configuring multicast ARP, see "ARP configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

No new command. Use the existing arp static, mac-address multicast, and undo arp check enable commands to implement multicast ARP.

For more information about these commands, see "ARP configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Specifying the IP address range for the DHCP clients of a specified vendor

Specifying the IP address range for the DHCP clients of a specified vendor

For more information about specifying the IP address range for the DHCP clients of a specified vendor, see "DHCP server configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: vendor-class-identifier.

For more information about this command, see "DHCP server configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Specifying a server’s IP address for the DHCP client

Specifying a server’s IP address for the DHCP client

For more information about specifying a server’s IP address for the DHCP client, see "DHCP server configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: next-server.

For more information about this command, see "DHCP server configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Configuring DHCP packet rate limit

Configuring DHCP packet rate limit

For more information about configuring DHCP packet rate limit, see "DHCP snooping configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: dhcp-snooping rate-limit.

For more information about this command, see "DHCP snooping configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Configuring DHCP snooping support for sub-option 9 in Option 82

Configuring DHCP snooping support for sub-option 9 in Option 82

For more information about configuring DHCP snooping support for sub-option 9 in Option 82, see "DHCP snooping configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: dhcp-snooping information sub-option.

Option private private and keyword standard were added to the dhcp-snooping information format command.

Keyword append was added to the dhcp-snooping information strategy command.

For more information about these commands, see "DHCP snooping configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Configuring TCP path MTU discovery

Configuring TCP path MTU discovery

For more information about configuring TCP path MTU discovery, see "IP Performance Optimization configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: tcp path-mtu-discovery.

For more information about this command, see "IP performance optimization configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: local ND proxy

Configuring local ND proxy

For more information about configuring local ND proxy, see "IPv6 basics configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: local-proxy-nd enable.

For more information about this command, see "IPv6 basics configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Specifying the AFTR address

Specifying the AFTR address

For more information about specifying the AFTR address, see "DHCPv6 server configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: ds-lite address.

For more information about this command, see "DHCPv6 configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Displaying detailed information about neighbors

Displaying detailed information about neighbors

For more information about displaying detailed information about neighbors, see "IPv6 basics configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

Keyword verbose was added to the display ipv6 neighbors command to display detailed information about neighbors.

For more information about this command, see "IPv6 basics configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Configuring the interface as an uplink interface and disabling it from learning ND snooping entries

Configuring the interface as an uplink interface and disabling it from learning ND snooping entries

For more information about configuring the interface as an uplink interface and disabling it from learning ND snooping entries, see "IPv6 basics configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: ipv6 nd snooping uplink

For more information about this command, see "IPv6 basics configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

New feature: Configuring permanent static route

Configuring permanent static route

For more information about this feature, see "Static routing configuration" in HP A5120 EI Switch Series Layer 3—IP Routing Configuration Guide-Release 2210.

Command reference

Keyword permanent was added to the ip route-static command.

For more information about this command, see "Static routing configuration commands" in HP A5120 EI Switch Series Layer 3—IP Routing Command Reference-Release 2210.

New feature: Enabling the IGMP snooping & MLD snooping host tracking function

Enabling the IGMP snooping & MLD snooping host tracking function

For more information about enabling the IGMP snooping host tracking function and Enabling the MLD snooping host tracking function, see "IGMP snooping configuration" and "MLD snooping configuration" in HP A5120 EI Switch Series IP Multicast Configuration Guide-Release 2210.

Command reference

New commands:

display igmp-snooping host

host-tracking (IGMP-Snooping view)

igmp-snooping host-tracking

display mld-snooping host

host-tracking (MLD-Snooping view)

mld-snooping host-tracking

For more information about these commands, see "IGMP snooping configuration commands" and "MLD snooping configuration commands" in HP A5120 EI Switch Series IP Multicast Command Reference-Release 2210..

New feature: PIM snooping & IPv6 PIM snooping

Configuring PIM snooping & IPv6 PIM snooping

For more information about PIM snooping configuration and IPv6 PIM snooping configuration, see HP A5120 EI Switch Series IP Multicast Configuration Guide-Release 2210.

Command reference

For more information about PIM snooping configuration commands and IPv6 PIM snooping configuration commands, see HP A5120 EI Switch Series IP Multicast Command Reference-Release 2210.

New feature: Configuring rule range remark

Configuring rule range remark

For more information about configuring rule range remark, see "ACL configuration" in HP A5120 EI Switch Series ACL and QoS Configuration Guide-Release 2210.

Command reference

New command: rule remark.

For more information about this command, see "ACL configuration commands" in HP A5120 EI Switch Series ACL and QoS Command Reference-Release 2210.

New feature: Configuring routing header type for an IPv6 ACL rule

Configuring routing header type for an IPv6 ACL

For more information about configuring routing header type for an IPv6 rule, see "ACL configuration" in HP A5120 EI Switch Series ACL and QoS Configuration Guide-Release 2210.

Command reference

Modified commands:

Keyword routing was added to the rule command in IPv6 advanced view and IPv6 basic view.

For more information about the commands, see "ACL configuration commands" in HP A5120 EI Switch Series ACL and QoS Command Reference-Release 2210.

New feature: Configuring byte-count or packet-based WFQ queuing

Configuring byte-count or packet-based WFQ queuing

For more information about byte-count or packet-based WFQ queuing configuration, see "Congestion management configuration" in HP A5120 EI Switch Series ACL and QoS Configuration Guide-Release 2210.

Command reference

Keywords byte-count and weight were added to the qos wfq command.

For more information about this command, see "Congestion management configuration commands" in HP A5120 EI Switch Series ACL and QoS Command Reference-Release 2210.

New feature: Configuring SP+WFQ queuing

Configuring SP+WFQ queuing

For more information about SP+WFQ queuing configuration, see "Congestion management configuration" in HP A5120 EI Switch Series ACL and QoS Configuration Guide-Release 2210.

Command reference

New commands: qos wfq byte-count and qos wfq group sp.

For more information about these commands, see "Congestion management configuration commands" in HP A5120 EI Switch Series ACL and QoS Command Reference-Release 2210.

New feature: Setting the validity time of the local user

Setting the validity time of the local user

For more information about setting the validity time of the local user, see "AAA configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

New command: validity-date.

For more information about this command, see "AAA configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Specifying the local user as a guest or guest manager

Specifying the local user as a guest or guest manager

For more information about specifying the role user as a guest or guest manager, see "AAA configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

Keywords user-role-guest and user-role guest-manager are added to the authorization-attribute (local user view/user group view) command.

For more information about this command, see "AAA configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Setting the guest attribute for a user group

Setting the guest attribute for a user group

For more information about setting the guest attribute for a user group, see "AAA configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

New command: group-attribute allow-guest.

For more information about the command, see "AAA configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Authorizing a local user to use the Web service

Authorizing a local user to use the Web service

For more information about authorizing a local user to use the Web service, see "AAA configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

Modified commands:

The web keyword is added to the service-type command.

The service-type web keyword is added to display local-user and undo local-user commands.

For more information about the commands, see "AAA configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Specifying ciphertext shared keys for RADIUS/HWTACACS servers

Specifying ciphertext shared keys for RADIUS/HWTACACS servers

For more information about specifying ciphertext shared keys for RADIUS and HWTACACS servers, see "AAA configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

Keywords cipher and simple are added to the following commands:

key (HWTACACS scheme view)

key (RADIUS scheme view)

primary accounting (RADIUS scheme view)

primary authentication (RADIUS scheme view)

secondary accounting (RADIUS scheme view)

secondary authentication (RADIUS scheme view)

For more information about the commands, see "AAA configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Specifying supported domain name delimiters

Specifying supported domain name delimiters

For more information about specifying supported domain name delimiters configuration, see "802.1X configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

For more information about specifying supported domain name delimiters configuration commands, see "802.1X configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Enabling inactivity aging

Enabling inactivity aging

For more information about enabling inactivity aging configuration, see "Port security configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

New command: port-security mac-address aging-type inactivity.

For more information about this command, see "Port security configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Enabling the dynamic secure MAC function

Enabling the dynamic secure MAC function

For more information about enabling the dynamic secure MAC function configuration, see "Port security configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

New command: port-security mac-address dynamic.

For more information about this command, see "Port security configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Enabling SSL client weak authentication

Enabling SSL client weak authentication

For more information about enabling SSL client weak authentication, see "SSL configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

New command: client-verify weaken.

For more information about this command, see "SSL configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Setting the maximum number of IPv4/IPv6 source guard binding entries

Setting the maximum number of IPv4/IPv6 binding entries

For more information about setting the maximum number of IPv4/IPv6 binding entries, see "IP source guard configuration" in HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

New commands: ip verify source max-entries and ipv6 verify source max-entries.

For more information about these commands, see "IP source guard configuration commands" in HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: SAVI

Configuring SAVI

For more information about configuring SAVI, see HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

For more information about SAVI configuration commands, see HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Blacklist

Configuring blacklist

For more information about configuring blacklist, see HP A5120 EI Switch Series Security Configuration Guide-Release 2210.

Command reference

For more information about blacklist configuration commands, see HP A5120 EI Switch Series Security Command Reference-Release 2210.

New feature: Enabling Ethernet OAM remote loopback in user view and system view

Enabling Ethernet OAM remote loopback in user view and system view

For more information about enabling Ethernet OAM remote loopback in user view and system view, see "Ethernet OAM configuration" in HP A5120 EI Switch Series High Availability Configuration Guide-Release 2210.

Command reference

New command: oam loopback interface.

For more information about this command, see "Ethernet OAM configuration commands" in HP A5120 EI Switch Series High Availability Command Reference-Release 2210.

New feature: Restoring the default Ethernet OAM connection mode

Restoring the default Ethernet OAM connection mode

For more information about restoring the default Ethernet OAM connection mode, see "Ethernet OAM configuration" in HP A5120 EI Switch Series High Availability Configuration Guide-Release 2210.

Command reference

New command: undo oam mode.

For more information about this command, see "Ethernet OAM configuration commands" in HP A5120 EI Switch Series High Availability Command Reference-Release 2210.

New feature: Configuring the collaboration between Smart Link and CC of CFD

Configuring the collaboration between Smart Link and CC of CFD

For more information about configuring the collaboration between Smart Link and CC of CFD, see "Smart Link configuration" in HP A5120 EI Switch Series High Availability Configuration Guide-Release 2210.

Command reference

New command: port smart-link group track.

For more information about this command, see "Smart Link configuration commands" in HP A5120 EI Switch Series High Availability Command Reference-Release 2210.

Modified feature: Improving the isolate-user-VLAN usability

Feature change description

Compared with Release 2208, to make the isolate-user-VLAN easier to configure, the feature was modified as follows:

After an isolate-user-VLAN is associated with secondary VLANs, you can perform the following configurations:

Adding an access port to and deleting an access port from the isolate-user-VLAN and secondary VLANs.

Deleting the isolate-user-VLAN or secondary VLANs.

Isolating the ports in the secondary VLANs at Layer 2 by using the isolated-vlan enable command.

Modifying the promiscuous or host mode of ports.

When you use the port isolate-user-vlan vlan-id promiscuous command to configure an uplink port, the port is automatically assigned to the isolate-user-VLAN specified by the vlan-id argument and the secondary VLANs associated with the isolate-user-VLAN. You do not need to manually add the uplink port to the isolate-user-VLAN.

You can assign a trunk port to the isolate-user-VLAN or secondary VLANs.

For more information about configuring an isolate-user-VLAN after the feature is modified, see "Isolate-user-VLAN configuration" in HP A5120 EI Switch Series Layer 2—LAN Switching Configuration Guide-Release 2210.

Command changes

Modified command: port isolate-user-vlan promiscuous

Old Syntax

port isolate-user-vlan promiscuous

New syntax

port isolate-user-vlan vlan-id promiscuous

Views

Layer 2 Ethernet interface view, Layer 2 aggregate interface view

Change description

Before modification: You must manually add the uplink port to the isolate-user-VLAN.

After modification: When you use the port isolate-user-vlan vlan-id promiscuous command to configure an uplink port, the port is automatically assigned to the isolate-user-VLAN specified by the vlan-id argument and the secondary VLANs associated with the isolate-user-VLAN. You do not need to manually add the uplink port to the isolate-user-VLAN.

Modified feature: Installing patches in one step

Feature change description

The patch install command now supports specifying a patch package file name.

Command changes

Modified command: patch install

Old syntax

patch install patch-location

New syntax

patch install { patch-location | file filename }

Views

User view

Change description

Before modification: To install patches in one step, you must specify the patch file path.

After modification: To install patches in one step, you can choose to specify a patch package file name.

Modified feature: Displaying file or directory information

Feature change description

The dir command now can display files and folders in the root directories of all storage media on the device.

Command changes

Modified command: dir

Old syntax

dir [ /all ] [ file-url ]

New syntax

dir [ /all ] [ file-url | /all-filesystems ]

Views

User view

Change description

Before modification: The dir command displays a specific file or all files and folders in the current directory.

After modification: The dir command can also display files and folders in the root directories of all storage media on the device.

Modified feature: Loopback interface numbering

Feature change description

The maximum loopback interface number was modified into 1023.

Command changes

Modified command: interface loopback

Syntax

interface loopback interface-number

Views

System view

Change description

Before modification: The loopback interface number ranges from 0 to 127.

After modification: The loopback interface number ranges from 0 to 1023.

Modified feature: Enabling address check

Feature change description

The command for this feature changes.

Command changes

Modified command: dhcp relay address-check enable

Old syntax

dhcp relay address-check { disable | enable }

New syntax

dhcp relay address-check enable

undo dhcp relay address-check enable

Views

interface view

Change description

The command was changed from dhcp relay address-check { disable | enable } to dhcp relay address-check enable.

Modified feature: Enabling ND proxy

Feature change description

The command for this feature changes.

Command changes

Modified command: proxy-nd enable

Old syntax

ipv6 nd proxy enable

undo ipv6 nd proxy enable

New syntax

proxy-nd enable

undo proxy-nd enable

Views

interface view

Change description

The command was changed from ipv6 nd proxy enable to proxy-nd enable.

Modified feature: ND snooping

Feature change description

The device with the new feature can create ND snooping entries based on DAD NS messages that contain link local addresses or global unicast addresses. Configure at least one type of ND snooping.

Command changes

New command: ipv6 nd snooping enable global

Syntax

ipv6 nd snooping enable global

undo ipv6 nd snooping enable global

View

System view

Default level

2: System level

Parameters

None

Description

Use the ipv6 nd snooping enable global command to enable ND snooping based on global unicast addresses. The device uses DAD NS messages containing global unicast addresses to create ND snooping entries.

Use the undo nd snooping enable global command to restore the default.

By default, ND snooping based on global unicast addresses is disabled.

Examples

# Enable NS snooping based on global unicast addresses.

<Sysname> system-view

[Sysname] ipv6 nd snooping enable global

New command: ipv6 nd snooping enable link-local

Syntax

ipv6 nd snooping enable link-local

undo ipv6 nd snooping enable link-local

View

System view

Default level

2: System level

Parameters

None

Description

Use the ipv6 nd snooping enable link-local command to enable ND snooping based on link local addresses. The device uses DAD NS messages containing link local addresses to create ND snooping entries.

Use the undo nd snooping enable link-local command to restore the default.

By default, ND snooping based on link local addresses is disabled.

Examples

# Enable ND snooping based on link local addresses.

<Sysname> system-view

[Sysname] ipv6 nd snooping enable link-local

Modified feature: Displaying IPv6 FIB entries

Feature change description

The keyword in the display ipv6 fib command changes.

Command changes

Modified command: display ipv6 fib

Old syntax

display ipv6 fib [ slot slot-number ] [ ipv6-address ] [ | { begin | exclude | include } regular-expression ]

New syntax

display ipv6 fib [ acl6 acl6-number | ipv6-prefix ipv6-prefix-name ] [ | { begin | exclude | include } regular-expression ]

display ipv6 fib ipv6-address [ prefix-length ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: Option slot slot-number was supported in the command.

After modification: Option slot slot-number is deleted and options acl6 acl6-number, and ipv6-prefix ipv6-prefix-name are added to display the entries permitted by a specified ACL or matching a specified prefix list. Argument prefix-length is added to display the prefix length for the destination address.

Modified feature: Displaying the IPv6 information of an interface

Feature change description

The keyword in the display ipv6 interface command changes.

Command changes

Modified command: display ipv6 interface

Old syntax

display ipv6 interface [ interface-type [ interface-number ] ] [ verbose ] [ | { begin | exclude | include } regular-expression ]

New syntax

display ipv6 interface [ interface-type [ interface-number ] ] [ brief ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: Keyword verbose is used to display detailed information about the interface.

After modification: Keyword verbose is replaced with keyword brief to display brief information about the interface.

Modified feature: Routing policy

Feature change description

The value range of the route-policy-name argument changed to 1 to 63.

Command changes

Modified command: route-policy

Syntax

route-policy route-policy-name { deny | permit } node node-number

Views

System view

Change description

Before modification: The route-policy-name argument ranges from 1 to 19.

After modification: The route-policy-name argument ranges from 1 to 63.

Modified feature: CFD

Feature change description

The command levels or views of certain CFD commands have changed.

Command changes

Before modification:

Default command level of the commands cfd linktrace and cfd loopback was system, and the commands were available in system view.

Default command level of these commands was system: display cfd ais, display cfd dm one-way history, display cfd linktrace-reply, display cfd linktrace-reply auto-detection, display cfd ma, display cfd md, display cfd mep, display cfd remote-mep, display cfd service-instance, display cfd status, display cfd tst, reset cfd dm one-way history, and reset cfd tst.

After modification:

Default command level of the commands cfd linktrace and cfd loopback was visit, and the commands were available in any view.

Default command level of these commands was monitor: display cfd ais, display cfd dm one-way history, display cfd linktrace-reply, display cfd linktrace-reply auto-detection, display cfd ma, display cfd md, display cfd mep, display cfd remote-mep, display cfd service-instance, display cfd status, display cfd tst, reset cfd dm one-way history, and reset cfd tst.

For more information about these commands, see "CFD configuration commands" in HP A5120 EI Switch Series High Availability Command Reference-Release 2210.

Modified feature: Configuring the protected VLANs for the RRPP domain

Feature change description

The protected-vlan command configures the protected VLANs for the RRPP domain by referencing MSTIs. As PVST is introduced in this release, the value range of the instance-id-list argument changes accordingly.

Command changes

Modified command: protected-vlan

Syntax

protected-vlan reference-instance instance-id-list

undo protected-vlan [ reference-instance instance-id-list ]

Views

RRPP domain view

Change description

Before modification: The instance-id-list argument ranges from 0 to 16.

After modification: The instance-id-list argument ranges from 0 to 32.

Modified feature: Configuring the protected VLANs for a smart link group

Feature change description

The protected-vlan command configures the protected VLANs for a smart link group by referencing MSTIs. As PVST is introduced in this release, the value range of the instance-id-list argument changes accordingly.

Command changes

Modified command: protected-vlan

Syntax

protected-vlan reference-instance instance-id-list

undo protected-vlan [ reference-instance instance-id-list ]

Views

Smart link group view

Change description

Before modification: The instance-id-list argument ranges from 0 to 16.

After modification: The instance-id-list argument ranges from 0 to 32.

Modified feature: Enabling traps globally

Feature change description

The snmp-agent trap enable command has keyword changes.

Command changes

Modified command: snmp-agent trap enable

Old syntax

snmp-agent trap enable [ arp rate-limit | bfd | configuration | flash | standard [ authentication | coldstart | linkdown | linkup | warmstart ]* | system ]

New syntax

snmp-agent trap enable [ arp rate-limit | configuration | default-route | flash | standard [ authentication | coldstart | linkdown | linkup | warmstart ]* | system ]

Views

Any view

Change description

The default-route keyword was added, and the bfd keyword was deleted.

Modified feature: Configuring IP source guard

Feature change description

Changed the commands for configuring static IPv4/IPv6 source guard binding entries.

Changed the commands for enabling the IPv4/IPv6 source guard function.

Changed the commands for displaying IP source guard binding entries.

Removed the commands for configuring the exceptional ports for the global static IP source guard binding entries.

Command changes

      - Modified command: display ip source binding

Old syntax

display ip check source [ interface interface-type interface-number | ip-address ip-address | mac-address mac-address ] [ slot slot-number ] [ | { begin | exclude | include } regular-expression ]

New syntax

display ip source binding [ static ] [ interface interface-type interface-number | ip-address ip-address | mac-address mac-address ] [ slot slot-number ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: display ip check source

After modification: display ip source binding

      - Modified command: display ipv6 source binding

Old syntax

display ip check source ipv6 [ interface interface-type interface-number | ip-address ip-address | mac-address mac-address ] [ slot slot-number ] [ | { begin | exclude | include } regular-expression ]

New syntax

display ipv6 source binding [ static ] [ interface interface-type interface-number | ipv6-address ipv6-address | mac-address mac-address ] [ slot slot-number ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

Change description

Before modification: display ip check source ipv6

After modification: display ipv6 source binding

      - Modified command: ip source binding (system view)

Old syntax

user-bind ip-address ip-address mac-address mac-address

undo user-bind { all | ip-address ip-address mac-address mac-address }

New syntax

ip source binding ip-address ip-address mac-address mac-address

undo ip source binding { all | ip-address ip-address mac-address mac-address }

Views

System view

Change description

Before modification: user-bind (system view)

After modification: ip source binding(system view)

      - Modified command: ipv6 source binding (system view)

Old syntax

user-bind ipv6 ip-address ip-address mac-address mac-address

undo user-bind ipv6 { all | ip-address ip-address mac-address mac-address }

New syntax

ipv6 source binding ipv6-address ipv6-address mac-address mac-address

undo ipv6 source binding { all | ipv6-address ipv6-address mac-address mac-address }

Views

System view

Change description

Before modification: user-bind ipv6(system view)

After modification: ipv6 source binding(system view)

      - Modified command: ip source binding (interface view)

Old syntax

user-bind { ip-address ip-address | ip-address ip-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

undo user-bind { ip-address ip-address | ip-address ip-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

New syntax

ip source binding { ip-address ip-address | ip-address ip-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

undo ip source binding { ip-address ip-address | ip-address ip-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

Views

Ethernet interface view

Change description

Before modification: user-bind (interface view)

After modification: ip source binding(interface view)

      - Modified command: ipv6 source binding (interface view)

Old syntax

user-bind ipv6 { ip-address ip-address | ip-address ip-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

undo user-bind ipv6 { ip-address ip-address | ip-address ip-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

New syntax

ipv6 source binding { ipv6-address ipv6-address | ipv6-address ipv6-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

undo ipv6 source binding { ipv6-address ipv6-address | ipv6-address ipv6-address mac-address mac-address | mac-address mac-address } [ vlan vlan-id ]

Views

Ethernet interface view

Change description

Before modification: user-bind ipv6(interface view)

After modification: ipv6 source binding(interface view)

      - Modified command: ip verify source

Old syntax

ip check source { ip-address | ip-address mac-address | mac-address }

undo ip check source

New syntax

ip verify source { ip-address | ip-address mac-address | mac-address }

undo ip verify source

Views

Ethernet interface view, VLAN interface view, port group view

Change description

Before modification: ip check source

After modification: ip verify source

      - Modified command: ipv6 verify source

Old syntax

ip check source ipv6 { ip-address | ip-address mac-address | mac-address }

undo ip check source ipv6

New syntax

ipv6 verify source { ipv6-address | ipv6-address mac-address | mac-address }

undo ipv6 verify source

Views

Ethernet interface view, port group view

Change description

Before modification: ip check source ipv6

After modification: ipv6 verify source

      - Removed command: display user-bind

Syntax

display user-bind [ ipv6 ] [ interface interface-type interface-number | ip-address ip-address | mac-address mac-address ] [ slot slot-number ] [ | { begin | exclude | include } regular-expression ]

Views

Any view

      - Removed command: user-bind uplink

Syntax

user-bind uplink

undo user-bind uplink

Views

Ethernet interface view

# A5120EI-CMW520-R2208

This release has the following changes:

New feature: Setting the age timer for ND entries in stale state

New feature: Setting the age timer for ND entries in stale state

Setting the age timer for ND entries in stale state

For more information about setting the age timer for ND entries in stale state, see "IPv6 basics configuration" in HP A5120 EI Switch Series Layer 3—IP Services Configuration Guide-Release 2210.

Command reference

New command: ipv6 neighbor stale-aging.

For more information about this command, see "IPv6 basics configuration commands" in HP A5120 EI Switch Series Layer 3—IP Services Command Reference-Release 2210.

# S5120EI-CMW520-R2208

Related documentation:

H3C S5120-EI Series Ethernet Switches Configuration Guides-Release 2208

H3C S5120-EI Series Ethernet Switches Command References-Release 2208