HPE A5120EI-CMW520-R2222P11 Release Notes

The information in this document is subject to change without notice. © Copyright 2016, 2018 Hewlett Packard Enterprise Development LP

Contents

Introduction	1

Version information	1

Version number	1

Version history	1

Hardware and software compatibility matrix	7

Upgrade restrictions and guidelines	9

Hardware feature updates	10

Hardware feature updates in R2222P11	10

Hardware feature updates in R2222P09	10

Hardware feature updates in R2222P08	10

Hardware feature updates in R2222P07	10

Hardware feature updates in R2222P05	10

Hardware feature updates in R2222P02	10

Hardware feature updates in R2222P01	10

Hardware feature updates in R2222	10

Hardware feature updates in R2221P30	10

Hardware feature updates in R2221P29	10

Hardware feature updates in R2221P27	11

Hardware feature updates in R2221P25	11

Hardware feature updates in R2221P22	11

Hardware feature updates in R2221P20	11

Hardware feature updates in R2221P18	11

Hardware feature updates in R2221P15	11

Hardware feature updates in R2221P12	11

Hardware feature updates in R2221P10	11

Hardware feature updates in R2221P08	11

Hardware feature updates in R2221P07	11

Hardware feature updates in R2221P06	11

Hardware feature updates in R2221P05	12

Hardware feature updates in R2221P04	12

Hardware feature updates in R2221P02	12

Hardware feature updates in R2221P01	12

Hardware feature updates in R2221	12

Hardware feature updates in R2220P11	12

Hardware feature updates in R2220P09	12

Hardware feature updates in R2220P02	12

Hardware feature updates in R2220P01	12

Hardware feature updates in R2220	12

Hardware feature updates in F2218	13

Hardware feature updates in F2217	13

Hardware feature updates in R2215	13

Hardware feature updates in F2212P02	13

Hardware feature updates in F2212L01	13

Hardware feature updates in F2212	13

Hardware feature updates in F2211	13

Hardware feature updates in R2210	13

Hardware feature updates in R2208	13

S5120EI-CMW520-R2208	13

Software feature and command updates	13

MIB updates	14

Operation changes	18

Operation Changes in R2222P11	18

Operation Changes in R2222P09	18

Operation Changes in R2222P08	18

Operation Changes in R2222P07	18

Operation Changes in R2222P05	18

Operation Changes in R2222P02	18

Operation Changes in R2222P01	18

Operation Changes in R2222	18

Operation Changes in R2221P30	18

Operation Changes in R2221P29	18

Operation Changes in R2221P27	19

Operation Changes in R2221P25	19

Operation Changes in R2221P22	19

Operation Changes in R2221P20	19

Operation Changes in R2221P18	19

Operation Changes in R2221P15	19

Operation Changes in R2221P12	19

Operation Changes in R2221P10	20

Operation Changes in R2221P08	20

Operation Changes in R2221P07	20

Operation Changes in R2221P06	20

Operation Changes in R2221P05	20

Operation Changes in R2221P04	21

Operation Changes in R2221P02	21

Operation Changes in R2221P01	21

Operation Changes in R2221	21

Operation Changes in R2220P11	21

Operation Changes in R2220P09	21

Operation Changes in R2220P02	21

Operation Changes in R2220P01	21

Operation Changes in R2220	22

Operation Changes in F2218	22

Operation Changes in F2217	22

Operation Changes in R2215	23

Operation Changes in F2212P02	23

Operation Changes in F2212L01	23

Operation Changes in F2212	23

Operation Changes in F2211	23

Operation Changes in R2210	24

Operation Changes in R2208	24

Operation Changes in R2208	24

Restrictions and cautions	24

Open problems and workarounds	25

List of resolved problems	26

Resolved problems in R2222P11	26

Resolved problems in R2222P09	26

Resolved problems in R2222P08	26

Resolved problems in R2222P07	27

Resolved problems in R2222P05	28

Resolved problems in R2222P02	29

Resolved problems in R2222P01	29

Resolved problems in R2222	29

Resolved problems in R2221P30	30

Resolved problems in R2221P29	30

Resolved problems in R2221P27	31

Resolved problems in R2221P25	32

Resolved problems in R2221P22	34

Resolved problems in R2221P20	34

Resolved problems in R2221P18	35

Resolved problems in R2221P15	36

Resolved problems in R2221P12	37

Resolved problems in R2221P10	38

Resolved problems in R2221P08	40

Resolved problems in R2221P07	41

Resolved problems in R2221P06	42

Resolved problems in R2221P05	43

Resolved problems in R2221P04	44

Resolved problems in R2221P02	44

Resolved problems in R2221P01	46

Resolved problems in R2221	46

Resolved problems in R2220P11	47

Resolved problems in R2220P09	47

Resolved problems in R2220P02	50

Resolved problems in R2220P01	50

Resolved problems in R2220	51

Resolved problems in F2218	54

Resolved problems in F2217	54

Resolved problems in R2215	55

Resolved problems in F2212P02	56

Resolved problems in F2212L01	56

Resolved problems in F2212	57

Resolved problems in F2211	57

Resolved problems in R2210	57

Resolved problems in R2208	58

Resolved Problems in R2208	59

Support and other resources	59

Accessing Hewlett Packard Enterprise Support	59

Documents	60

Related documents	60

Documentation feedback	60

Appendix A Feature list	61

Hardware features	61

Software features	63

Appendix B Upgrading software	67

Upgrading software from the Boot menu	67

Accessing the Boot menu	67

XMODEM download through the console port	69

TFTP download through an Ethernet port	77

FTP download through an Ethernet port	79

Upgrading software from the CLI	81

FTP download from a server	81

TFTP download from a server	82

List of tables

Table 1 Version history	1

Table 2 HP A5120EI  product family matrix	8

Table 3 Hardware and software compatibility matrix	8

Table 4 MIB updates	14

Table 5 A5120 EI series hardware features (I)	61

Table 6 A5120 EI series hardware features (II)	62

Table 7 SFP-Ethernet port pairs forming Combo interfaces	63

Table 8 Software features of the A5120 EI Switch series	63

Table 9 Software upgrade methods	67

Table 10 Boot menu options	68

Table 11 Debugging menu options	69

Table 12 Description of the TFTP parameters	77

Table 13 Description of the FTP parameters	79

Introduction

This document describes the features, restrictions and guidelines, open problems, and workarounds for version HPE A5120EI-CMW520-R2222P11. Before you use this version on a live network, back up the configuration and test the version to avoid software upgrade affecting your live network.

Use this document in conjunction with HPE A5120EI-CMW520-R2222P11 Release Notes (Software Feature Changes) and the documents listed in "Related documents."

Version information

Version number

Comware software, Version 5.20.99, Release 2222P11

Note: You can see the version information with the command display version in any view. See Note①.

Version history

|  | IMPORTANT: The software feature changes listed in the version history table for each version are not complete. To obtain complete information about all software feature changes in each version, see the Software Feature Changes document for this release notes. |
| --- | --- |

Version history

| Version number | Last version | Release Date | Release type | Remarks |
| --- | --- | --- | --- | --- |
| A5120EI-CMW520-R2222P11 | A5120EI-CMW520-R2222P09 | 2018-10-31 | Release version | - Fixed bugs. |
| A5120EI-CMW520-R2222P09 | A5120EI-CMW520-R2222P08 | 2018-03-31 | Release version | This version fixed bugs and introduced feature changes. Modified feature include: RADIUS Calling-Station-ID attribute value for the login service Port security need to know feature |
| A5120EI-CMW520-R2222P08 | A5120EI-CMW520-R2222P07 | 2017-10-16 | Release version | - Fixed bugs. |
| A5120EI-CMW520-R2222P07 | A5120EI-CMW520-R2222P05 | 2017-08-03 | Release version | This version fixed bugs and introduced feature changes. New features include: Configuring the action a port takes after it receives an Ethernet OAM event from the remote end |
| A5120EI-CMW520-R2222P05 | A5120EI-CMW520-R2222P02 | 2017-03-31 | Release version | - Fixed bugs. |
| A5120EI-CMW520-R2222P02 | A5120EI-CMW520-R2222P01 | 2016-12-31 | Release version | Fixed bugs |
| A5120EI-CMW520-R2222P01 | A5120EI-CMW520-R2222 | 2016-11-30 | Release version | Fixed bugs: Removed feature: - 1. User profile |
| A5120EI-CMW520-R2222 | A5120EI-CMW520-R2221P30 | 2016-10-31 | Release version | New features: Modified feature: - 1. Support for forwarding PTP multicast packets of Layer 2 multicast - 2. Uploading IPv6 addresses for 802.1X and MAC authentication users Fixed bugs |
| A5120EI-CMW520-R2221P30 | A5120EI-CMW520-R2221P29 | 2016-08-31 | Release version | New features: Modified feature: - 1. Specifying file name for storing DHCP snooping entries on a remote server Fixed bugs |
| A5120EI-CMW520-R2221P29 | A5120EI-CMW520-R2221P27 | 2016-07-31 | Release version | New features: - 1. Including user IP addresses in realtime accounting packets for MAC authentication users with dynamic IP addresses - 2. Ignoring the ingress ports of ARP packets during user validity check Modified feature: Fixed bugs |
| A5120EI-CMW520-R2221P27 | A5120EI-CMW520-R2221P25 | 2016-05-31 | Release version | New features: - 1. Authorization VLAN auto-tagging for MAC authentication - 2. Confining RADIUS Vendor-Specific extended attributes to a specific vendor Modified feature: Fixed bugs |
| A5120EI-CMW520-R2221P25 | A5120EI-CMW520-R2221P22 | 2016-03-31 | Release version | New features: - 1. Configuring periodic MAC re-authentication Modified feature: Fixed bugs |
| A5120EI-CMW520-R2221P22 | A5120EI-CMW520-R2221P20 | 2016-01-31 | Release version | New features: - 1. Enabling sending of ICMPv6 redirect messages Modified feature: - 1. Disabling advertising prefix information in RA messages Fixed bugs |
| A5120EI-CMW520-R2221P20 | A5120EI-CMW520-R2221P18 | 2015-10-31 | Release version | New features: - 1.MAC authentication voice VLAN Fixed bugs |
| A5120EI-CMW520-R2221P18 | A5120EI-CMW520-R2221P15 | 2015-08-31 | Release version | New features: - 1.Sending EAP-Success packets to 802.1X users in critical VLAN Fixed bugs |
| A5120EI-CMW520-R2221P15 | A5120EI-CMW520-R2221P12 | 2015-07-09 | Release version | Fixed bugs |
| A5120EI-CMW520-R2221P12 | A5120EI-CMW520-R2221P10 | 2015-05-31 | Release version | New features: - 1.Login delay - 2. IPv6 address with a 127-bit prefix length |
| A5120EI-CMW520-R2221P10 | A5120EI-CMW520-R2221P08 | 2015-04-28 | Release version | New features: - 1.Support for NTP configuration in IPv6 networks |
| A5120EI-CMW520-R2221P08 | A5120EI-CMW520-R2221P07 | 2015-02-03 | Release version | New features: - 1.Applicable scope of packet filtering on a VLAN interface - 2.SNMP notifications for PVST topology changes - 3.Disabling SSL 3.0 |
| A5120EI-CMW520-R2221P07 | A5120EI-CMW520-R2221P06 | 2014-12-22 | Release version | New features: - 1.802.1X MAC address binding - 2.Automatic PI reset |
| A5120EI-CMW520-R2221P06 | A5120EI-CMW520-R2221P05 | 2014-10-31 | Release version | Fixed bugs |
| A5120EI-CMW520-R2221P05 | A5120EI-CMW520-R2221P04 | 2014-08-29 | Release version | New features: - Telnet/SSH user connection control Modified feature: - 1.Including time zone information in the timestamp of system information sent to a log host - 2. Configuring physical state change suppression on an Ethernet interface - 3.Configuring a tag and description for an IPv6 static route |
| A5120EI-CMW520-R2221P04 | A5120EI-CMW520-R2221P02 | 2014-07-15 | Release version | New features: - 1.802.1X voice VLAN - 2.Configuring the uplink port to permit multiple isolate-user-VLANs - 3.TCP fragment attack protection - 4.Portal roaming Modified feature: - 1.Username request timeout timer for 802.1X authentication |
| A5120EI-CMW520-R2221P02 | A5120EI-CMW520-R2221P01 | 2014-04-30 | Release version | New features: - 1.Support for BPDU guard configuration in interface or port group view; - 2.MAC re-authentication timer for users in guest VLAN; - 3.MAC and port uniqueness check by the DHCP snooping device; Modified feature: - 1. Auto status transition of dynamic secure MAC addresses; - 2.The maximum number of gateways supported in MFF automatic mode; |
| A5120EI-CMW520-R2221P01 | A5120EI-CMW520-R2221 | 2014-02-28 | Release version | New features: Discarding IPv6 packets that contain extension headers; |
| A5120EI-CMW520-R2221 | A5120EI-CMW520-R2220P11 | 2013-12-31 | Release version | New features: - 1. SSL server policy association with the FTP service; Modified features: - 1. Setting the device name; - 2. Displaying brief IP configuration for Layer 3 interfaces; - 3. Configuring static multicast MAC address entries; - 4. Specifying the username and password to log in to the SCP server; - 5. Disabling an untrusted port from recording clients' IP-to-MAC bindings; 6. Customizing DHCP options; |
| A5120EI-CMW520-R2220P11 | A5120EI-CMW520-R2220P09 | 2013-12-06 | Release version | New features: - 1. Specifying multiple public keys for an SSH user; Fixed bugs; |
| A5120EI-CMW520-R2220P09 | A5120EI-CMW520-R2220P02 | 2013-09-30 | Release version | New features: 1. 802.1X-based dynamic IPv4 source guard binding entries; 2. Multicast ND; 3.Enabling MAC authentication multi-VLAN mode; 4.Binding IP, MAC, and port on Web; 5. Configuring the ARP detection logging function Modified features: 1.Configuring system information for the SNMP agent; 2. Specifying multiple secondary HWTACACS servers; |
| A5120EI-CMW520-R2220P02 | A5120EI-CMW520-R2220P01 | 2013-04-10 | Release version | New features: None Modified features: - 1.Enabling/disabling FIPS mode - 2.Setting the IRF link down report delay - 3.Setting the minimum password length - 4.Switching the user privilege level - 5.Implementing ACL-based IPsec - 6.Cluster management Removed features: None |
| A5120EI-CMW520-R2220P01 | A5120EI-CMW520-R2220 | 2013-03-15 | Release version | New features: None Modified features: None Removed features: None |
| A5120EI-CMW520-R2220 | A5120EI-CMW520-F2218 | 2013-01-09 | Release version | New features: 1.Disabling password recovery capacity 2.Configuring a port to forward 802.1X EAPOL packets untagged 3.Enabling source IP conflict prompt 4.Delaying the MAC authentication 5. Disabling MAC entry aging timer refresh based on destination MAC address 6.Setting the deletion delay time for SAVI Modified features: 1.Default configuration; Removed features: None |
| A5120EI-CMW520-F2218 | A5120EI-CMW520-F2217 | 2012-09-29 | Feature version | New features: None; Modified features: 1. Changed the setting of the Telnet server in the default configuration file from Enabled to Disabled. 2. Changed the IP HTTP setting in the default configuration file from Enabled to Disabled. 3. Changed the default RADIUS scheme setting. 4. The time that the front panel interfaces use to open auto negotiation during device startup was changed from more than 30 seconds to 5 seconds. 5. Added attributes for CDP packets sent by the device. Removed features: NONE |
| A5120EI-CMW520-F2217 | A5120EI-CMW520-R2215 | 2012-08-31 | Feature version | New features: 1.Automatic configuration file backup for software downgrading; 2.FIPS; 3.Configuring ACL-based IPsec; 4.IKE; 5.Verifying the correctness and integrity of the file; Modified features: 1.Configuring a password for the local user; 2.Clearing all users from the password control blacklist; 3.802.1X critical VLAN; 4.MAC authentication critical VLAN; 5.Modifying CLI configuration commands executed in FIPS mode for CC evaluation; 6.Modifying login management commands executed in FIPS mode for CC evaluation; 7.Modifying software upgrade commands executed in FIPS mode for CC evaluation; 8.Modifying security commands executed in FIPS mode for CC evaluation; 9.Modifying SNMP commands executed in FIPS mode for CC evaluation; |
| A5120EI-CMW520-R2215 | A5120EI-CMW520-F2212P02 | 2012-04-28 | Release version | New features: 1. Interface range configuration; 2. NTPv4; 3. Changing the brand name; 4. Remaining POE power display by slot for IRF; 5. Set the maximum number of Selected ports for the aggregation group; |
| A5120EI-CMW520-F2212P02 | A5120EI-CMW520-F2212L01 | 2012-03-31 | Feature version | New features: 1. Configuring LLDP to advertise a specific voice VLAN; |
| A5120EI-CMW520-F2212L01 | A5120EI-CMW520-F2212 | 2012-03-08 | Feature version | Fix bugs |
| A5120EI-CMW520-F2212 | A5120EI-CMW520-F2211 | 2012-02-24 | Feature version | New features: 1.Setting the DSCP value for multiple types of protocol packets Modified features: 1. Modified password/key related configuration. For more information, see Feature and Command Change History for HP A5120EI-CMW520-F2212 |
| A5120EI-CMW520-F2211 | A5120EI-CMW520-R2210 | 2011-11-30 | Feature version | New features: 1.Critical VLAN 2.SCP(Secure copy) 3.Configuring a source interface for DNS packets 4.MVRP(Multiple VLAN Registration Protocol) 5.Enabling LLDP to automatically discover IP phones |
| A5120EI-CMW520-R2210 | A5120EI-CMW520-R2208 | 2011-9-20 | Release version | New features: 1.SAVI (Source Address Validation); 2.Global IP address binding; 3.PVST+; 4.Configuring secure addresses to age without being triggered by traffic; 5.Set Sticky MAC addresses as dynamic secure MAC addresses; 6.Dot1dTpFdbTable of RFC 1493; 7.Encryption of shared keys for HWTACACS packets; 8.DHCP snooping option 82 support for sub-option 9; 9.Obtaining receiving power of optical modules through MIB; 10.Obtaining utilization of H3C device ACL resources through MIB; 11.Local proxy ND; 12.Easy configuration for Isolate-user-VLAN; 13.STP TC-Snooping; 14.Configurable Jumbo frame size; 15.Restore port-based default settings; 16.WFQ support for SP/WRR/DRR; 17.WEB based Triple authentication configuration; 18.Collaboration between Smart Link and CFD CC detection; 19.PoE power negotiation through Power Via MDI TLV; 20.Port aggregation priority effective in static aggregation groups; 21.SSH2 IPv6; 22.SFTP IPv6; 23.MAC address roaming; 24.Configurable minimum number of ports in an aggregation group; 25.Restoring the default operating mode for CDP-compatible LLDP; 26.DHCP snooping support for packet rate limit; |
| A5120EI-CMW520-R2208 | S5120EI-CMW520-R2208 | 2011-6-17 | Release version | New features 1. ipv6 neighbor stale-aging; |

Hardware and software compatibility matrix

|  | CAUTION: To avoid an upgrade failure, use Table 3 to verify the hardware and software compatibility before performing an upgrade. |
| --- | --- |

Before HP A5120EI family, there was another 1 product family: H3C S5120EI, shipped to market. The product family has same hardware and software specification except brand. The product matrix is as following. In brief, the HP A5120EI will be the representation to all of them in subsequent document.

HP A5120EI  product family matrix

| HP A5120EI | H3C S5120EI | 3COM 4210G/4510G |
| --- | --- | --- |
| HP A5120-24G EI with 2 Interface Slots | H3C S5120-28C-EI | Switch 4210G 24-Port Switch 4210G 48-Por Switch 4210G 24-Port PWR Switch 4210G 48-Port PWR Switch 4210G NT 24-Port Switch 4210G NT 48-Port Switch 4510G 24-Port Switch 4510G 48-Por |
| HP A5120-48G EI with 2 Interface Slots | H3C S5120-52C-EI |  |
| HP A5120-24G-PoE+ EI | H3C S5120-28C-PWR-EI |  |
| HP A5120-48G-PoE+ EI | H3C S5120-52C-PWR-EI |  |
| HP A5120-24G EI TAA with 2 Interface Slots | H3C S5120-28C-EI |  |
| HP A5120-48G EI TAA with 2 Interface Slots | H3C S5120-52C-EI |  |
| HP A5120-24G-PoE+ EI TAA | H3C S5120-28C-PWR-EI |  |
| HP A5120-48G-PoE+ EI TAA | H3C S5120-52C-PWR-EI |  |
| HP A5120-24G EI | H3C S5120-24P-EI |  |
| HP A5120-48G EI | H3C S5120-48P-EI |  |

Hardware and software compatibility matrix

| Item | Specifications |
| --- | --- |
| Product family | A5120EI series H3C S5120EI series |
| Hardware platform | HP A5120-24G EI Switch HP A5120-48G EI Switch HP A5120-24G EI Switch with 2 Interface Slots HP A5120-24G EI TAA Switch with 2 Interface Slots HP A5120-48G EI Switch with 2 Interface Slots HP A5120-48G EI TAA Switch with 2 Interface Slots HP A5120-24G-PoE+ EI Switch with 2 Interface Slots HP A5120-24G-PoE+ EI TAA Switch with 2 Interface Slots HP A5120-48G-PoE+ EI Switch with 2 Interface Slots HP A5120-48G-PoE+ EI TAA Switch with 2 Interface Slots H3C S5120-28C-EI H3C S5120-52C-EI H3C S5120-28C-PWR-EI H3C S5120-52C-PWR-EI H3C S5120-24P-EI H3C S5120-48P-EI |
| Minimum memory requirements | 128 MB |
| Minimum Flash requirements | 16 MB |
| Boot ROM version | Version 621 or higher ( See Note②) |
| System software image | A5120EI-CMW520-R2222P11.bin |
| IMC | iMC EAD 7.3 (E0502) iMC TAM 7.3 (E0503) iMC UAM 7.3 (E0503) i iMC NTA 7.3 (E0502) iMC PLAT 7.3 (E0605) iMC QoSM 7.3 (E0502) iMC RAM 7.3 (E0501) iMC SHM 7.3 (E0502) |
| iNode | iNode PC 7.3 (E0504) |

Display the system software and Boot ROM version information of the A5120EI:

<HPE>display version

HPE Comware Platform Software

Comware Software, Version 5.20.99, Release 2222P11---------------------Note①

Copyright (c) 2010-2018 Hewlett Packard Enterprise Development LP

HP A5120-24G-PoE+ EI Switch with 2 Interface Slots uptime is 0 week, 0 day, 0 ho

ur, 1 minute

HP A5120-24G-PoE+ EI Switch with 2 Interface Slots with 1 Processor

128M bytes SDRAM

16384K bytes Flash Memory

Hardware Version is REV.B

CPLD Version is 007

Bootrom Version is 621----------------------------------------------Note②

[SubSlot 0] 24GE+4SFP+POE Hardware Version is REV.B

Upgrade restrictions and guidelines

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

If a save operation is performed on a switch where a software version of F2217 or later is running and the version number in the current startup configuration file is lower than F2212P02, the system first backs up the startup configuration file and then saves the current configuration. For example, suppose the startup configuration file is a.cfg. When a save operation is performed, the system first backs up a.cfg into _a_bak.cfg and then saves the current configuration into a.cfg.

Hardware feature updates

  - Hardware feature updates in R2222P11

None

  - Hardware feature updates in R2222P09

None

  - Hardware feature updates in R2222P08

None

  - Hardware feature updates in R2222P07

None

  - Hardware feature updates in R2222P05

None

  - Hardware feature updates in R2222P02

None

  - Hardware feature updates in R2222P01

None

  - Hardware feature updates in R2222

None

  - Hardware feature updates in R2221P30

None

  - Hardware feature updates in R2221P29

None

  - Hardware feature updates in R2221P27

None

  - Hardware feature updates in R2221P25

None

  - Hardware feature updates in R2221P22

None

  - Hardware feature updates in R2221P20

None

  - Hardware feature updates in R2221P18

None

  - Hardware feature updates in R2221P15

None

  - Hardware feature updates in R2221P12

None

  - Hardware feature updates in R2221P10

None

  - Hardware feature updates in R2221P08

None

  - Hardware feature updates in R2221P07

None

  - Hardware feature updates in R2221P06

None

  - Hardware feature updates in R2221P05

None

  - Hardware feature updates in R2221P04

None

  - Hardware feature updates in R2221P02

None

  - Hardware feature updates in R2221P01

New Features:

1. Support SFP 100M Optical Module (JF833A);

  - Hardware feature updates in R2221

None

  - Hardware feature updates in R2220P11

None

  - Hardware feature updates in R2220P09

None

  - Hardware feature updates in R2220P02

New Features:

1. Add 10G-BASE-T module;

  - Hardware feature updates in R2220P01

None

  - Hardware feature updates in R2220

None

  - Hardware feature updates in F2218

None

  - Hardware feature updates in F2217

None

  - Hardware feature updates in R2215

None

  - Hardware feature updates in F2212P02

None

  - Hardware feature updates in F2212L01

None

  - Hardware feature updates in F2212

None

  - Hardware feature updates in F2211

None

  - Hardware feature updates in R2210

None

  - Hardware feature updates in R2208

None

  - S5120EI-CMW520-R2208

None

Software feature and command updates

For more information about the software feature and command update history, see HPE A5120EI-CMW520-R2222P11 Release Notes (Software Feature Changes).

MIB updates

MIB updates

| Item | MIB file |  | Module | Description |
| --- | --- | --- | --- | --- |
| A5120EI-CMW520-R2222P11 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222P09 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222P08 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222P07 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222P05 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222P02 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222P01 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2222 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P30 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P29 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P27 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P25 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P22 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P20 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P18 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P15 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P12 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P10 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P08 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P07 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P06 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P05 |  |  |  |  |
| New | rfc2096-ip-forward.mib | IP-FORWARD-MIB |  | Added inetCidrRouteTable |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P04 |  |  |  |  |
| New | hh3c-ifqos2.mib | HH3C-IFQOS2-MIB |  | Added descriptions and support for the following MIBs: 1.hh3cIfQoSQSModeTable 2.hh3cIfQoSQSWeightTable 3.hh3cIfQoSPortPriorityTable 4.hh3cIfQoSPortPirorityTrustTable |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P02 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221P01 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2221 |  |  |  |  |
| New | None | None |  | None |
| Modified | None | None |  | None |
| A5120EI-CMW520-R2220P11 |  |  |  |  |
| New | None | None |  | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-R2220P09 |  |  |  |  |
| New | None | None |  | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-R2220P02 |  |  |  |  |
| New | New |  | New | New |
| Modified | Modified |  | Modified | Modified |
| A5120EI-CMW520-R2220P01 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-R2220 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-F2218 |  |  |  |  |
| New | None |  | None | None |
| Modified | hh3c-radius.mib |  | HH3C-RADIUS-MIB | Changed the value returned by the following MIBs from a plaintext or ciphertext password to empty or "******": (1)hh3cUserPassword (2)hh3cRdKey (3)hh3cRdSecKey (4)hh3cRdAccKey (5)hh3cRdSecAccKey (6)hh3cRadiusSchAuthPrimKey (7)hh3cRadiusSchAuthSecKey (8)hh3cRadiusSchAccPrimKey (9)hh3cRadiusSchAccSecKey (10)hh3cDot11SrvSecurityPskKeyString (11)hh3cSecureRalmAuthPassword (12)hh3cDot11SecurityPskKeyString |
| A5120EI-CMW520-F2217 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-R2215 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-F2212P02 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-F2212L01 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-F2212 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| A5120EI-CMW520-F2211 |  |  |  |  |
| New |  |  | hh3cCfgLogTable | HH3C-CONFIG-MAN-MIB |
|  |  |  | hh3cCfgOperateTable | HH3C-CONFIG-MAN-MIB |
| Modified | None |  | None | None |
| A5120EI-CMW520-R2210 |  |  |  |  |
| New |  |  | rfc1493-bridge.mib | BRIDGE-MIB |
|  |  |  | hh3c-acl.mib | ACL-MIB |
|  |  |  | hh3c-transceiver-info.mib | TRANSCEIVER-MIB |
|  |  |  | savi-mib.mib | SAVI-MID |
| Modified |  |  | rfc2011-ip-icmp.mib | IP-MIB |
|  |  |  | rfc2465-ipv6.mib | IPV6-MIB |
| A5120EI-CMW520-R2208 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |
| S5120EI-CMW520-R2208 |  |  |  |  |
| New | None |  | None | None |
| Modified | None |  | None | None |

Operation changes

  - Operation Changes in R2222P11

None

  - Operation Changes in R2222P09

None

  - Operation Changes in R2222P08

None

  - Operation Changes in R2222P07

None

  - Operation Changes in R2222P05

None

  - Operation Changes in R2222P02

None

  - Operation Changes in R2222P01

None

  - Operation Changes in R2222

None

  - Operation Changes in R2221P30

None

  - Operation Changes in R2221P29

None

  - Operation Changes in R2221P27

None

  - Operation Changes in R2221P25

1. The maximum number of DHCP snooping entries was changed from 2 K to 6 K for each interface.

  - Operation Changes in R2221P22

1. Modified the output destination of MemRatio log messages

Before modification, MemRatio log messages are not output to the log buffer when the switch fails to issue ACLs for lack of memory.

After modification, MemRatio log messages are output to the log buffer when the switch fails to issue ACLs for lack of memory.

  - Operation Changes in R2221P20

None

  - Operation Changes in R2221P18

1. Change to the count of IfInDiscards for an IRF physical interface:

Before modification, the counted dropped packets include the packets that fail to find the egress port.

After modification, the counted dropped packets do not include the packets that fail to find the egress port.

2. Change to VLAN assignment for voice users and data users when the server is unreachable:

Before modification: When the server is unreachable, both voice users and data users join the critical VLAN.

After modification: When the server is unreachable, voice users join the voice VLAN and data users join the critical VLAN.

  - Operation Changes in R2221P15

1. Added traps for master PoE DIMM module failures:

Before modification, no traps are generated when a PoE switch fails to supply power over PoE because its master PoE DIMM module becomes faulty.

After modification, power failure traps are generated for the preceding situation.

  - Operation Changes in R2221P12

1. Increased the number of supported syslog hosts from 4 to 20.

  - Operation Changes in R2221P10

1. Change to the logout threshold for offline detect of MAC authentication

Before modification: The switch logs out a user if it has not received traffic from the user within two offline detect intervals.

After modification: The switch logs out a user if it has not received traffic from the user within one offline detect interval.

2. Change to route learning after the ipv6 address dhcp-alloc command is configured

Before modification, the switch can learn only IPv6 addresses with the prefix as 128 and cannot generate network routes after the ipv6 address dhcp-alloc command is configured on an interface.

After modification, the switch can actively send RS messages and RA learning is enabled after the ipv6 address dhcp-alloc command is configured on an interface. Then, the following events occur on the switch:

  - The switch can request addresses from a DHCPv6 server and can learn IPv6 addresses with the prefix as 64.

  - The switch can learn the default gateway based on RA messages and add the default gateway to routes.

  - The switch can learn a prefix based on RA messages and add the prefix to routes.

  - Operation Changes in R2221P08

1. Change to the LED for a loop detection-enabled port

Before modification, when a loop is detected on a loop detection-enabled port, the LED status for the port does not change.

After modification, when a loop is detected on a loop detection-enabled port, the LED for the port is flashing green.

  - Operation Changes in R2221P07

None

  - Operation Changes in R2221P06

None

  - Operation Changes in R2221P05

1. Change to ACL limit for FP_RANGE_CHECK

Before modification, the FP_RANGE_CHECK register supports a maximum of 32 ACLs. The system prompts failure information when the maximum number is exceeded.

After modification, the FP_RANGE_CHECK register supports a maximum of more than 32 ACLs, which depends on the available ACL resources.

  - Operation Changes in R2221P04

None

  - Operation Changes in R2221P02

1. Changed the maximum number of gateways in a VLAN from 20 to 64 for auto-mode MFF.

  - Operation Changes in R2221P01

None

  - Operation Changes in R2221

1. Changed the maximum ARP rate on a port that is enabled with ARP detection from 50 pps to 400 pps.

2. Changed the maximum number for both ACLs and rules from 1000 to 2000.

  - Operation Changes in R2220P11

None

  - Operation Changes in R2220P09

1. Before modification, the CPE only supports using HTTP to upload/download files to/from the ACS.

After modification, if the CPE passes authentication on the ACS, the CPE can use either HTTP or HTTPS to upload/download files to/from the ACS.

2. Changed the flow control configuration policy:

Before modification, enabling or disabling flow control does not bring up or down the physical port.

After modification, enabling or disabling flow control brings down and up the physical port to apply the new configuration.

3. In this version and later versions, multicast MAC addresses starting with 01005e can be configured.

  - Operation Changes in R2220P02

None

  - Operation Changes in R2220P01

1.  Operation changes to the port security mode UserloginWithOui when multicast trigger is disabled and unicast trigger is enabled:

Before modification, packets with an unknown source MAC address that does not contain a permitted OUI are discarded without triggering authentication.

After modification, packets with an unknown source MAC address that does not contain a permitted OUI can trigger authentication.

2. Operation changes to the port security mode UserloginWithOui when multicast trigger is enabled:

Before modification, the device continues multicasting authentication requests after a PC passes authentication, resulting in re-authentication of the PC.

After modification, the device stops multicasting authentication packets after a PC passes authentication.

3. This release permits IP camera packets that have a Power consumption TLV longer than 6 bytes to pass.

  - Operation Changes in R2220

1. Added the following attributes for CDP packets sent by the device: Addresses, Capabilities, Software Version, Platform, Duplex, MTU and System Name.

2. The time that the front panel interfaces use to open auto negotiation during device startup was changed from more than 30 seconds to 5 seconds.

3. Default configuration changes: disable all the TCP/UDP port by default (For example: TCP ports including 23/80/7547,UDP ports  including 68/1812/3318/3799).

  - Operation Changes in F2218

1. The cluster management feature provides a simple method to manage multiple units using a single IP address, however it does use some protocols that are not considered totally secure. In this release, the cluster management protocols, including NDP, NTDP, and Cluster, are disabled by default to avoid any possible security risks.

2. If cluster management is required it is necessary to re-enable the required protocols with the following commands: ndp enable, ntdp enable, and cluster enable.  In addition, HPE recommends that a separate management VLAN for the cluster should be established. Only the access ports that are used to link the cluster members should belong to this VLAN so the inter-switch protocol will not be accessible to insecure devices, including PCs and other network devices.

3. The Winet feature is removed in this release as it is not considered totally secure.  The Winet functionality is available through other management methods.

4. Changed the maximum number of Free IP networks for 802.1X authentication from 4 to 16.Suffix requirement change for execute batch files

(1)Before modification, execute batch files must have a suffix of ".bat".

(2)After modification, execute batch files can have any suffix.

  - Operation Changes in F2217

1. Change the maximum online time from 65535 seconds to 2147483647 seconds for MAC authentication users in the RADIUS authentication approach.

2. Change the MAC authentication delay setting:

(1)Before: On a port where both 802.1X authentication and MAC authentication are enabled, MAC authentication starts after a delay of 30 seconds.

(2)After: By default, MAC authentication is not delayed. The following command is provided to enable MAC authentication delay and set the delay time.

mac-authentication timer auth-delay time

undo mac-authentication timer auth-delay

3. Change the maximum number of sub VLANs in an Isolate-user-VLAN from 64 to 192.

4. Patch operation change:

In earlier versions, when a patch is installed on a switch that has been installed with another patch, the switch replaces the existing patch with the new patch without any prompt. This creates risks. This version prompts a message "Another patch loaded, please uninstall it first."

5. Operation change for whether a port leaves the critical VLAN after the silent timer expires:

After a port is assigned to the critical VLAN, the RADIUS server state changes to "blocked", and the silent timer of the RADIUS server starts (this timer is configurable and  defaults to 5 minutes).

In earlier versions:

(1) If the port uses 802.1X authentication, it leaves the critical VLAN when the silent timer   expires. If the port is configured with the dot1x critical recovery-action command, its leaving triggers new 802.1X authentication.

(2)If the port uses MAC authentication, it leaves the critical VLAN when the silent timer expires.

In this version:

(1)If the port uses 802.1X authentication, it remains in the critical VLAN when the silent timer expires. If the port is configured with the dot1x critical recovery-action command, the silent timer expiration triggers new 802.1X authentication.

(2)If the port uses MAC authentication, it remains in the critical VLAN and triggers new MAC authentication when the silent timer expires.

6. If a save operation is performed on a switch where a software version of F2217 or later is running and the version number in the current startup configuration file is lower than F2212P02, the system first backs up the startup configuration file and then saves the current configuration. For example, suppose the startup configuration file is a.cfg. When a save operation is performed, the system first backs up a.cfg into _a_bak.cfg and then saves the current configuration into a.cfg.

  - Operation Changes in R2215

Forwarded ARP packets are not rate limited.

  - Operation Changes in F2212P02

None

  - Operation Changes in F2212L01

None

  - Operation Changes in F2212

None

  - Operation Changes in F2211

None

  - Operation Changes in R2210

1. In earlier versions, if you specify both the output interface and next hop for an IPv6 static route, the specified next hop must be a link-local address. This version removes the limitation and allows you to specify a global unicast address as the next hop.

2. Added Root protection on the edge port.

3. Changed the maximum number of VLAN interfaces from 8 to 32.

4. Changed the maximum number of IPv4 static routes from 32 to 128.

5. Changed the maximum number of IPv6 static routes from 32 to 64.

6. Changed the user-bind {ip-address X.X.X.X |mac-address H-H-H } [vlan INTEGER<1-4094> ] command in port view to :ip source binding { ip-address X.X.X.X | mac-address H-H-H } [ vlan INTEGER<1-4094>].

7. Changed the ip check source { ip-address | mac-address } command in port view to ip verify source { ip-address | mac-address }.

8. Change the command of {ipsec-policy } to {enable ipsec-policy } .

  - Operation Changes in R2208

None

  - Operation Changes in R2208

First Release

Restrictions and cautions

1. Due to implementation limitation, VLAN ACLs do not take effect on QinQ-enabled ports.

2. Port isolation group configuration takes precedence over traffic redirect configuration. For example, add GE1/0/1 and GE1/0/2 to a port isolation group, and configure GE1/0/1 to redirect specific traffic to GE1/0/2. The redirect configuration does not take effect because the two ports have been isolated.

3. Log in to the device and use the display diagnostic-information command to save running status statistics to a file. The system fails to save the file because the file exceeds the storage space of the flash.

4. Multi-port loopback detection is available on a single device only.

5. Multicast ARP is supported only on A5120-24G EI / A5120-24G-PoE+ EI. The output ports of a multicast ARP entry must reside on the same device.

6. Release F2212P02 or later adopts a new password encryption algorithm. The password saved in the configuration file has been processed by the new algorithm. If you roll back the software from Release F2212 or later to a version before F2212, the password cannot be restored, and login will fail.

7. Software auto-update function might fail to synchronize the software of subordinate devices from a version earlier than R2220P10 to R2220P10 or a later version. You must manually upgrade the software for the devices.

8. The feature of configuring the uplink port to permit multiple isolate-user-VLANs is not supported on A5120EI series.

9. MPLS label-based load sharing for link aggregation takes effect only on MPLS intermediate transit nodes.

Open problems and workarounds

LSD67923

Symptom: In this version of code, the password encryption within configuration files has been enhanced and cannot be interpreted by earlier revisions of the agent code.  This means that if a unit is downgraded to earlier code, it may no longer be possible to login and manage the device.

Condition: Condition: This symptom might occur after the software is downgraded to a version before F2212P02

Workarounds:

Before upgrading to the new code, it is necessary to ensure password control is disabled. Execute the “undo password-control enable” and then save this configuration file as a backup in case you need to downgrade the software again.  If it is later necessary to downgrade to earlier software, force the switch to use this backup configuration file by executing a ”startup saved-configuration (filename)” command before rebooting to the old code. Then, after the code has been downgraded, the device can be logged in from the console or by Telnet, but not SSH.  The SSH authentication details will need to be reset.

If no backup configuration has been saved but it is still possible to access the device management via some method while running the old code (e.g. Console, Telnet or SSH), then you can redefine all the device management passwords as required.

If after a downgrade it is impossible to login to the device via any method, then there are two ways to recover the switch:

From the BOOT menu, set the new code to run again and reboot the device.  Disable Telnet authentication:
    User-interface  vty 0 4
    Authentication mode none
Then save the configuration and downgrade the code again, login via Telnet and reset all the passwords as required.

From the BOOT menu.  On boot-up, use Ctrl+B to enter the Boot menu and then force the unit to use the factory default configuration (bypassing the user configuration).  The unit will then need to be fully reconfigured.

201508260388

Symptom: The boot-loader update command fails to upgrade the startup images on a subordinate switch because of high memory usage on the master and subordinate switches.

Condition: This symptom might occur if the boot-loader update command is used to upgrade the startup images for all IRF member switches after the master and subordinate switches start up.

Workaround: Use one of the following methods:

Copy the target startup images to the subordinate switch, and then use the boot-loader file command to specify these startup images for next startup on the master and subordinate switches separately. The CPU usage will be 100 % when the images are being copied, but copying is not affected.

Before the subordinate switch joins the IRF fabric, upgrade the startup images on the master switch and execute the irf auto-update enable command. The startup images on the subordinate switch are automatically upgraded after it joins the IRF fabric. During the upgrade process, the master switch will generate memory insufficiency alarms, but the upgrade is not affected.

201603300317

Symptom: The files in the flash might be corrupted if the DHCP snooping entry file is frequently updated to flash and can cause the switch image loading to fail after a reboot.

Condition: This symptom occurs if the DHCP snooping entry backup function is configured with a short DHCP snooping entry update interval, set by using the dhcp-snooping binding database update interval command.

Workaround: It is recommended to write the DHCP snooping entry file to a remote file server by using the command: dhcp snooping binding database filename { filename | url url [ username username [ password { cipher | simple } string ] ] }.

List of resolved problems

  - Resolved problems in R2222P11

201712050157

Symptom: An IRF fabric splits.

Condition: This symptom occurs if the following operations are performed:

Assign interfaces numbered 1 on the IRF fabric to an aggregation group.

Configure the qinq transparent-vlan command on the aggregate interface corresponding to the aggregation group to enable transparent transmission for a list of VLANs.

201808090430

Symptom: When the device is connected to a peer through an SFP-XG-LX220-MM1310 transceiver module, the switch generates the LLDP_DUPLEX_INCONSISTENT log frequently.

Condition: This symptom might occur if the device is connected to a peer through an SFP-XG-LX220-MM1310 transceiver module.

  - Resolved problems in R2222P09

201712190289

Symptom: CVE-2017-12190

Condition: Local attacker can exploit these issues to obtain sensitive information that may lead to further attacks.

  - Resolved problems in R2222P08

201706200187

Symptom: CVE-2010-3864

Condition: Successfully exploiting this issue may allow attackers to execute arbitrary code in the context of applications that use the affected library, but this has not been confirmed. Failed exploit attempts may crash applications, denying service to legitimate users.

201706200187

Symptom: CVE-2010-4252

Condition: A successful exploit may allow attackers to authenticate without the shared secret, aiding in further attacks.

201706200187

Symptom: CVE-2011-4109

Condition: An attacker may leverage these issues to obtain sensitive information, cause a denial-of-service condition and perform unauthorized actions.

201706200187

Symptom: CVE-2012-2110

Condition: Successfully exploiting this issue may allow an attacker to execute arbitrary code in the context of the application using the vulnerable library. Failed exploit attempts will result in a denial-of-service condition.

201708140543

Symptom: In the output from the display saved-configuration command, the PVST path cost setting for the only VLAN in the last line cannot be displayed.

Condition: This symptom might occur if the following operations are performed:

              - Configure the path cost for a PVST-enabled port in N×10+1 (n≥1) inconsecutive VLANs.

              - Save the configuration.

              - Execute the display saved-configuration command to view the path cost settings.

201707250789

Symptom: The RADIUS server has an IPv6 address inconsistent with the actual IPv6 address of a user.

Condition: This symptom might occur if the 802.1X authentication-enabled switch sends the RADIUS server an IPv6 RADIUS accounting request that contains an incorrect IPv6 attribute.

  - Resolved problems in R2222P07

201705040092

Symptom: CVE-2014-9298

Condition: An attacker could bypass source IP restrictions and send malicious control and configuration packets.

201705040049

Symptom: CVE-2017-6458

Condition: NTP are prone to a buffer-overflow vulnerability because it fails to properly bounds-check user-supplied data before copying it into an insufficiently sized buffer.

201706270522

Symptom: After a port on the master node in an RRPP ring is set to the secondary port, the RRPP ring transits to Disconnect state and the status of the secondary port changes from down to up.

Condition: This symptom might occur if the master node is an IRF subordinate device with member ID 1 and the port set to the secondary port is GigabitEthernet1/0/1.

201707040634

Symptom: Execution of the qinq transparent-vlan command fails if the switch runs software version R2221P20, R2221P22, R2221P25, R2221P27, R2221P29, R2221P30, R2222, R2222P02, or R2222P05.

Condition: This symptom might occur in one of the following conditions:

  - The switch runs software version R2221P20 or R2221P22, and the qinq transparent-vlan command is executed in interface view.

  - The switch runs software version R2221P20, R2221P22, R2221P25, R2221P27, R2221P29, R2221P30, R2222, R2222P02, or R2222P05, and the qinq transparent-vlan command is executed in Layer 2 aggregate interface view.

  - Resolved problems in R2222P05

201701060096

Symptom: An interface cannot learn the MAC address of a PC connected to an IP phone on an interface when the IP phone and the PC cannot reach the authentication server.

Condition: This symptom occurs if the following operations are performed:

  - Enable port security, and set the port security mode to userLoginSecure or userLoginSecureExt for the interface.

  - Configure a MAC authentication voice VLAN on the interface.

The MAC address of the IP phone is learned in the voice VLAN after the IP phone passes authentication.

  - Modify the MAC authentication voice VLAN ID, and configure the MAC authentication critical VLAN or 802.1X critical VLAN as the original MAC authentication voice VLAN ID.

201612080502

Symptom: An interface does not learn MAC addresses.

Condition: This symptom occurs if the following operations are performed:

  - a.    Set the maximum number of MAC addresses that can be learned and configure 802.1X authentication on the interface.

  - b.    Execute the default command on the interface after the maximum number of MAC addresses is reached.

201611220384

Symptom: A user logs in to a Comware 7 device or a third-party device from a Comware 5 switch. When the user presses Enter once, two new lines are created.

Condition: This symptom might occur if a user Telnets to a Comware 5 switch and then logs in to a Comware 7 device or a third-party device through SSH from the switch.

201612220015

Symptom:CVE-2016-8610

Condition: OpenSSL is prone to denial-of-service vulnerability.Successful exploitation of the issue will cause excessive memory or CPU resource consumption, resulting in a denial-of-service condition.

201612050252

Symptom: CVE-2016-7427

Condition: An attacker with access to the NTP broadcast domain can periodically inject specially crafted broadcast mode NTP packets into the broadcast domain which, while being logged by ntpd, can cause ntpd to reject broadcast mode packets from legitimate NTP broadcast servers

201612050252

Symptom: CVE-2016-7428

Condition: An attacker with access to the NTP broadcast domain can send specially crafted broadcast mode NTP packets to the broadcast domain which, while being logged by ntpd, will cause ntpd to reject broadcast mode packets from legitimate NTP broadcast servers.

  - Resolved problems in R2222P02

201609010374

Symptom: CVE-2013-0169

Condition: The TLS protocol and the DTLS protocol do not properly consider timing side-channel attacks on a MAC check requirement during the processing of malformed CBC padding, which allows remote attackers to conduct distinguishing attacks and plaintext-recovery attacks via statistical analysis of timing data for crafted packets, aka the "Lucky Thirteen" issue.

  - Resolved problems in R2222P01

201610260369

Symptom: Extra characters are displayed in the display saved-configuration command output.

Condition: This symptom occurs if the multiline text of a banner is pasted when the banner is configured, which results in line feeds but not carriage returns.

201610260340

Symptom: The device might fail to forward MPLS traffic.

Condition: This symptom occurs if route flapping occurs on the MPLS network.

201608160032

Symptom: An IRF fabric splits.

Condition: This symptom occurs if four devices form an IRF fabric and the remaining memory resources of the system are insufficient.

  - Resolved problems in R2222

201609210333

Symptom: CVE-2015-5219

Condition: NTP is prone to a denial-of-service vulnerability. A remote attacker may exploit this issue to cause an infinite loop, resulting in a denial-of-service condition.

201609010392

Symptom: CVE-2009-3238

Condition: The get_random_int function in the Linux kernel before 2.6.30 produces insufficiently random numbers, which allows attackers to predict the return value, and possibly defeat protection mechanisms.

201609010432

Symptom: CVE-2014-9751

Condition: The read_network_packet function in ntp_io.c in ntpd in NTP 4.x before 4.2.8p1 on Linux and OS X does not properly determine whether a source IP address is an IPv6 loopback address, which makes it easier for remote attackers to spoof restricted packets, and read or write to the runtime state, by leveraging the ability to reach the ntpd machine's network interface with a packet from the ::1 address.

201609190248

Symptom: The MAD IP addresses of members in an IRF fabric cannot be pinged, and ARP conflicts occur.

Condition: This symptom might occur if BFD MAD is configured on an IRF fabric, and the MAD IP addresses of multiple IRF members are pinged.

201609070163

Symptom: When certain conditions exist, the value of the EAP Request/Challenge Packets field is 0 in the output from the display dot1x command.

Condition: This symptom might occur if the dot1x authentication-method eap command is executed, and 802.1X authentication is successful.

  - Resolved problems in R2221P30

201608180290

Symptom: CVE-2015-7974

Condition: Fixed vulnerability in NTP 4.x before 4.2.8p6 and 4.3.x before 4.3.90 which might allow remote attackers to conduct impersonation attacks via an arbitrary trusted key.

201608180290

Symptom: CVE-2015-7973

Condition: Fixed vulnerability when NTP is configured in broadcast mode, a man-in-the-middle attacker or a malicious client could replay packets received from the broadcast server to all (other) clients, which cause the time on affected clients to become out of sync over a longer period of time.

201605170555

Symptom: CVE-2016-1550

Condition: Fixed vulnerability in ntpd function allow an attacker to conduct a timing attack to compute the value of the valid authentication digest causing forged packets to be accepted by ntpd.

201605170555

Symptom: CVE-2016-1551

Condition: Fixed vulnerability in ntpd allows unauthenticated network attackers to spoof refclock packets to ntpd processes on systems that do not implement bogon filtering.

201607050187

Symptom: CVE-2016-4954

Condition: Fixed vulnerability in ntpd in NTP 4.x before 4.2.8p8 allows remote attackers to cause a denial of service by sending spoofed packets from source IP addresses in a certain scenario.

  - Resolved problems in R2221P29

201606270469

Symptom: After an IRF master device is rebooted or powered off, BGP neighborship cannot be established.

Condition: This symptom occurs if the following operations are performed:

  - Configure BGP. BGP can establish neighborship properly.

  - Reboot or power off the IRF master device.

201606280333

Symptom: In the display transceiver interface command output for a transceiver module SFP-GE-LH70-SM1550, the Transfer Distance(km) and Ordering Name field are incorrectly displayed. The correct information is as follows:

Transfer Distance(km): 80(9um)

Ordering Name: SFP-GE-LH80-SM1550

Condition: This symptom occurs if the display transceiver interface command is used to display information about a transceiver module SFP-GE-LH70-SM1550.

201607150108

Symptom: The device reboots unexpectedly.

Condition: This symptom occurs if the following conditions exist:

  - The number of ECMP routes allowed is 1.

  - Multiple static routes with the same destination address, mask, and preference are configured.

201606130061

Symptom: After the master of a four-member IRF fabric is rebooted, traffic forwarding between downstream devices is interrupted for about one minute.

Condition: This symptom might occur if the master of a four-member IRF fabric is rebooted.

  - Resolved problems in R2221P27

201604260165

Symptom: When an aggregate interface is disconnected, traffic is interrupted for 9 seconds rather than 6 seconds.

Condition: This symptom occurs if the LACP timeout interval is set to the short timeout interval on an interface and the aggregate link is disconnected.

201604140359

Symptom: The IP source guard binding entries with IP address 255.255.255.255 cannot be deleted in the Web interface.

Condition: This symptom occurs if you continue to configure static IPv4 binding entries with only MAC addresses specified when the number of static IPv4 binding entries created on the switch has reached 200. These entries are displayed as static IPv4 binding entries with IP address 255.255.255.255 in the Web interface.

201603100046

Symptom: A walk on ifOutDiscards MIB returns a value of 0.

Condition: This symptom can be seen during a walk on ifOutDiscards MIB.

201603290559

Symptom: The switch reboots unexpectedly when private MIB nodes hh3cLswSlotPktBufFree and hh3cLswSlotPktBufInit are accessed.

Condition: This symptom might occur if private MIB nodes hh3cLswSlotPktBufFree and hh3cLswSlotPktBufInit are accessed.

201109130022

Symptom: The MIB does not have information of the dot3adAggPortSelectedAggID and dot3adAggPortAttachedAggID nodes.

Condition: This symptom might occur if link aggregation is configured on the switch.

201512250171

Symptom: The CLI for a Comware 7-based device is stuck.

Condition: This symptom occurs if you log in to a Comware 5-based device through a console port and then the device telnets to a Comware 7-based device.

201401190006

Symptom: The DHCP relay agent fails to assign an IP address to a client.

Condition: This symptom occurs if the DHCP relay agent receives an offer packet where the yiaddr is 0.0.0.0, and the Bflag is 0.

  - Resolved problems in R2221P25

201601080614

Symptom: BGP routes cannot be summarized when the labels of the routes change.

Condition: This symptom might occur if BGP route summarization is enabled and the labels of BGP routes change.

201602180247

Symptom: Execution of the qinq transparent-vlan command fails if the switch uses R2221P20 or R2221P22.

Condition: This symptom might occur if the switch uses R2221P20 or R2221P22, and the qinq transparent-vlan command is executed in interface view.

201602010047

Symptom: An IRF fabric cannot generate DHCP snooping entries for some interfaces.

Condition: This symptom might occur if the following conditions exist:

  - The IRF fabric contains three or more member switches, and DHCP snooping is enabled on the IRF fabric.

  - Two master/subordinate switchovers occur, and some interfaces of the subordinate switches are down before the second switchover.

201512010460

Symptom: An SSH client logs in to the switch that acts as an SSH server. When the SSH client tries to log out, the switch does not respond to the logout request. The SSH client must wait for the connection to time out.

Condition: This symptom might occur if the switch acts an SSH server.

201511190090

Symptom: On an IRF fabric, the static multicast MAC address entries on aggregate interfaces are lost after a master/subordinate switchover.

Condition: This symptom might occur if static multicast MAC address entries are configured on aggregate interfaces and a master/subordinate switchover occurs.

201601040443

Symptom: A PC connected to a Layer 3 interface cannot obtain an IPv6 address through stateless address autoconfiguration if the interface uses a static IPv6 address and constantly flaps.

Condition: This symptom might occur if the following conditions exist:

  - Stateless address autoconfiguration is enabled on a Layer 3 interface that is connected to a PC.

  - The Layer 3 interface uses a static IPv6 address and constantly flaps.

201603140431

Symptom: The switch keeps outputing the "System is busy with warm backup, please wait......" message.

Condition: This symptom might occur if routing loops occur.

201603140376

Symptom: MFF accesses invalid memory and the switch reboots unexpectedly if certain conditions exist.

Condition: This symptom might occur if the following conditions exist:

  - DHCP snooping and MFF are used together.

  - IP source guard generates IPSG bindings with invalid VLAN information based on DHCP snooping entries, and MFF uses these bindings.

LSD64848

Symptom: DHCP clients can obtain IP addresses only once because DHCP snooping entries are incorrect.

Condition: This symptom might occur if basic QinQ and DHCP snooping are used together.

201507090353

Symptom: When the display interface command is repeatedly executed, the CPU usage stays at 100% and the PVST topology changes after a period of time.

Condition: This symptom might occur if the display interface command is repeatedly executed.

201603150457

Symptom: The transfer distance and ordering name of an HP X125 1G SFP LC LH70 Transceiver module (JD063B) are incorrect in the output from the display transceiver interface command.

Condition: This symptom might occur if the display transceiver interface command is used to display information for an HP X125 1G SFP LC LH70 Transceiver module (JD063B).

201601190549

Symptom: The switch cannot establish an IPv6 BGP peer relationship with a neighbor if the primary IPv6 address of the output interface is a network address.

Condition: This symptom might occur if the primary IPv6 address of the output interface is a network address.

201511110159

Symptom: After the snmp-agent trap enable stp tc command is configured, the switch sometimes displays incorrect information for the configuration.

Condition: This symptom might occur if the snmp-agent trap enable stp tc command is executed and the configuration is saved.

  - Resolved problems in R2221P22

201512290216

Symptom: CVE-2015-3195.

Condition: When presented with a malformed X509_ATTRIBUTE structure OpenSSL will leak memory. This structure is used by the PKCS#7 and CMS routines so any application which reads PKCS#7 or CMS data from untrusted sources is affected.

201501060439

Symptom: ICMP error packets fail to be sent.

Condition: This symptom might be seen if an interface configured with NAT needs to forward packets that exceed the MTU of the interface and cannot be fragmented.

201601180342

Symptom: MAC address entries for online MAC authentication users age out before the offline detect timer (set by using mac-authentication timer offline-detect) expires.

Condition: This symptom might be seen if MAC authentication is enabled.

201512080086

Symptom: Dynamic MAC address entries do not age out when a large number of 802.1X or MAC authentication users come online and go offline repeatedly.

Condition: This symptom might be seen if a large number of 802.1X or MAC authentication users come online and go offline repeatedly.

201511100130

Symptom: An error occurs when the switch reboots to join an IRF fabric as a subordinate member.

Condition: This symptom might be seen if the switch reboots to join an IRF fabric as a subordinate member.

  - Resolved problems in R2221P20

201507280105

Symptom: All member switches in an IRF fabric reboot when the issu run switchover command is executed on a subordinate switch after the subordinate switch is upgraded successfully.

Condition: This symptom occurs if the following conditions exist:

  - The IRF fabric uses ISSU for upgrade.

  - The priority of the master switch is higher than that of the subordinate switch.

201510210123

Symptom: The switch fails to transparently transmit OSPF multicast protocol packets.

Condition: This symptom occurs if the OSPF multicast protocol packets are sent on an interface with QinQ enabled.

201510200101

Symptom: The switch prints a message indicating an IP address conflict when a newly online wireless client obtains the IP address of a wireless client that just went offline.

Condition: This symptom occurs if the following conditions exist:

  - Wireless clients obtain IP addresses through DHCP.

  - A wireless client comes online after another wireless client goes offline.

201510220542

Symptom: An IP phone cannot obtain an IP address after the IP phone passes 802.1X authentication.

Condition: This symptom occurs if the switch cannot advertise the voice VLAN ID specified by using the dot1x voice vlan command to the IP phone through LLDP or CDP.

201507170252

Symptom: The switch reboots unexpectedly when the FreeRADIUS server issues a command to force an 802.1X user offline.

Condition: Condition: This symptom might occur if the switch uses a FreeRADIUS server for 802.1X authentication.

201507160220

Symptom: CVE-2015-1788.

Condition: When processing an ECParameters structure OpenSSL enters an infinite loop. This can be used to perform denial of service against any system which processes public keys, certificate requests or certificates.

201507160220

Symptom: CVE-2015-1789.

Condition: X509_cmp_time does not properly check the length of the ASN1_TIME string and/or accepts an arbitrary number of fractional seconds in the time string. An attacker can use this to craft malformed certificates and CRLs of various sizes and potentially cause a segmentation fault, resulting in a DoS on applications that verify certificates or CRLs.

201507160220

Symptom: CVE-2015-1790.

Condition: The PKCS#7 parsing code does not handle missing inner EncryptedContent correctly. An attacker can craft malformed  PKCS#7 blobs with missing content and trigger a NULL pointer dereference on parsing.

201508180092

Symptom: Symptom: The switch cannot negotiate the power with a powered device through LLDP.

Condition: This symptom occurs if the powered device must negotiate the power twice with the switch.

  - Resolved problems in R2221P18

201506150192

Symptom: The DHCP server on the switch does not preferentially use the static address pool when processing DHCP-INFORM packets.

Condition: This symptom might occur if the following conditions exist:

The DHCP-INFORM packets are sent by a DHCP client bound to an IP address in the static address pool.

The address range of a dynamic address pool covers the static address pool.

201506250206

Symptom: The switch does not forward traffic based on PBR policies that have been configured.

Condition: This symptom might occur if PBR policies are configured on multiple VLAN interfaces, and a large number of PBR policies exist on the switch.

201505120391

Symptom: The server cannot assign the voice VLAN attribute to an IP phone.

Condition: This symptom might occur if the 802.1X authentication in EAP relay mode is used.

201505120286

Symptom: The switch cannot obtain the serial number of a transceiver module that is not certified.

Condition: This symptom occurs if a MIB browser is used to obtain the serial number.

201506180403

Symptom: The switch fails to cooperate with a specific authentication server.

Condition: This symptom might occur if the following conditions exist:

The switch is connected to a specific authentication server.

The NAS_PORT_ID field in the sent RADIUS packets contains the VLAN field, which cannot be processed by the authentication server.

201507100159

Symptom: An IP phone connected to a subordinate switch in an IRF fabric is removed from the voice VLAN after the subordinate switch reboots.

Condition: This symptom might occur if the following conditions exist:

The IP phone receives power through PoE.

The subordinate switch experienced a cold reboot.

201306280329

Symptom: The BIMS server fails to enable periodical notification for a switch that accesses the server for the first time. The BIMS server cannot manage the switch because the switch cannot actively access the BIMS server periodically.

Condition: This symptom might occur if the following conditions exist:

  - The switch starts up without a configuration file.

  - The switch accesses the BIMS server for the first time after the switch obtains CWMP settings through DHCP.

  - Resolved problems in R2221P15

201505140479

Symptom: The CPU usage of an IRF fabric is excessively high.

Condition: This symptom occurs if the following conditions exist:

A large number of GRE tunnels are configured on aggregate interfaces on the IRF fabric.

No member switch is specified by using the service slot slot-number command to forward traffic for the tunnel interfaces.

201505260034

Symptom: Each member switch in a split IRF fabric set to the Recovery state takes a long time to shut down its interfaces.

Condition: This symptom occurs if LACP MAD is used.

201506080236

Symptom: An IP phone connected to a subordinate switch in an IRF fabric is removed from the voice VLAN after the subordinate switch reboots.

Condition: This symptom might occur if the following conditions exist:

The IP phone receives power through PoE.

The subordinate switch is rebooted by using the reboot command.

201506270158

Symptom: The switch reboots unexpectedly when a match criterion for QoS is added, deleted, or modified.

Condition: This symptom occurs if a match criterion for QoS is added, deleted, or modified.

201506190301

Symptom: The TTL value in ICMP messages does not decease by hop.

Condition: This symptom occurs if the following conditions exist:

The switch is in a network that has routing loops.

The ICMP messages are triggered by using the ping –r command.

201505120295

Symptom: The switch cannot obtain the serial number of a transceiver module that is not certified by H3C.

Condition: This symptom occurs if a MIB browser is used to obtain the serial number.

201505180128

Symptom: A 24-port PoE+ switch cannot supply power over PoE after a sharp decrease in input voltage.

Condition: This symptom occurs if a sharp decrease in input voltage happens to the switch.

Resolved problems in R2221P12

201504170082

Symptom:  After being logged out, an authenticated user can access Internet resources without passing portal authentication in triple authentication.

Condition:   This symptom occurs if the cable is removed from and then installed into the interface connected to the user after the user passes the previous portal authentication.

201504170082

Symptom: MAC authentication succeeds after a delay of 20 to 30 seconds.

Condition: This symptom occurs if both portal authentication and MAC authentication are configured for triple authentication.

201504070107

Symptoms: CVE-2015-0209

Condition: A malformed EC private key file consumed via the d2i_ECPrivateKey function could cause a use after free condition. This could lead to a DoS attack or memory corruption for applications that receive EC private keys from untrusted sources.

201504070107

Symptoms: CVE-2015-0287

Condition: Reusing a structure in ASN.1 parsing may allow an attacker to cause memory corruption via an invalid write. Applications that parse structures containing CHOICE or ANY DEFINED BY components may be affected.

201504070107

Symptoms: CVE-2015-0288

Condition: The function X509_to_X509_REQ will crash with a NULL pointer dereference if the certificate key is invalid.

201504070107

Symptoms: CVE-2015-0289

Condition:  The PKCS#7 parsing code does not handle missing outer ContentInfo correctly. An attacker can craft malformed ASN.1-encoded PKCS#7 blobs with missing content and trigger a NULL pointer dereference on parsing.

201504070107

Symptoms: CVE-2015-0292

Condition: Vulnerability existed in previous versions of OpenSSL related to the processing of base64 encoded data.

201505110036

Symptom: The irf link-delay command configuration does not take effect.

Condition: This symptom occurs after the IRF fabric splits when the irf link-delay command is configured on an IRF fabric.

201504240250

Symptom: The switch fails to cooperate with a specific authentication server.

Condition: This symptom occurs when the following conditions exist:

The switch is connected to a specific authentication server.

The NAS_PORT_ID field in the sent RADIUS packets contains the VLAN field, which cannot be processed by the server.

201505180146

Symptom: A PoE+ switch is identified as a PoE switch.

Condition: This symptom occurs after the PoE+ switch is power cycled.

201504140243

Symptom: The values of the sysUptime and ifLastChange nodes are different.

Condition: This symptom occurs if the values are obtained by using a MIB tool.

201504130301

Symptom: The ARP detection log records IP addresses in the reverse order (for example, it records 10.4.35.199 as 199.35.4.10).

Condition: This symptom occurs if arp detection is configured and ARP detection logging is enabled by using the arp detection log enable command.

Resolved problems in R2221P10

201501200392

Symptom:  CVE-2015-0205

Condition:   An OpenSSL server will accept a DH certificate for client authentication without the certificate verify message. This effectively allows a client to authenticate without the use of a private key. This only affects servers which trust a client certificate authority which issues certificates containing DH keys.

201501200392

Symptom:  CVE-2014-3570

Condition:   Bignum squaring (BN_sqr) may produce incorrect results on some platforms, including x86_64. This bug occurs at random with a very low probability, and is not known to be exploitable in any way.

201501200392

Symptom:  CVE-2015-0204

Condition:   An OpenSSL client will accept the use of an RSA temporary key in a non-export RSA key exchange ciphersuite. A server could present a weak temporary key and downgrade the security of the session.

201501200392

Symptom:  CVE-2014-3572

Condition:   An OpenSSL client will accept a handshake using an ephemeral ECDH ciphersuite using an ECDSA certificate if the server key exchange message is omitted. This effectively removes forward secrecy from the ciphersuite.

201501200392

Symptom:  CVE-2014-8275

Condition:  By modifying the contents of the signature algorithm or the encoding of the signature, it is possible to change the certificate's fingerprint. Only custom applications that rely on the uniqueness of the fingerprint may be affected.

201501200392

Symptom:  CVE-2014-3569

Condition:   The ssl23_get_client_hello function in s23_srvr.c in OpenSSL 0.9.8zc, 1.0.0o, and 1.0.1j does not properly handle attempts to use unsupported protocols, which allows remote attackers to cause a denial of service (NULL pointer dereference and daemon crash) via an unexpected handshake, as demonstrated by an SSLv3 handshake to a no-ssl3 application with certain error handling.

201503230385

Symptom: A route does not take effect if its lower eight bits are 01111111 (127 in decimal format).

Condition: This symptom occurs if the lower eight bits of the route are 01111111 (127 in decimal format).

201502110105

Symptom: When a host moves between the local switch and a peer, the MAC address entry for the host does not update on the local switch.

Condition: This symptom might occur if the local switch is connected to its peer by an aggregate link, and the host moves between the lowest-numbered port of the local switch and a port of the peer.

201502270218

Symptom: iMC is disconnected from a managed switch and generates an ICMP no response alarm for the switch.

Condition: This symptom occurs if the switch suffers from attacks against the ipForwarding and ipDefaultTTL nodes.

201502160179

Symptom: After a switch obtains an IPv6 address from a DHCPv6 server, the switch cannot successfully ping the DHCPv6 server.

Condition: This symptom occurs when the following conditions exist:

  - A subordinate IRF member switch is connected to the DHCPv6 server through a VLAN interface.

  - The VLAN interface is configured to actively send RS messages and receive RA messages by using the ipv6 address dhcp-alloc command.

201501290196

Symptom: The switch reboots unexpectedly.

Condition: This symptom occurs after the following procedure is performed:

  - Configure a QoS policy. The QoS policy contains a traffic class and a traffic behavior with the same name as the QoS policy.

  - Apply the QoS policy to a control plane.

  - Remove the QoS policy from the control plane.

Resolved problems in R2221P08

201412310368

Symptom: CVE-2014-9295

Condition: Stack-based buffer overflows in ntpd in NTP before 4.2.8 allows remote attackers to execute arbitrary code via a crafted packet.

201410230229

Symptom: SSL 3.0 Fallback protection

Condition: OpenSSL has added support for TLS_FALLBACK_SCSV to allow applications to block the ability for a MITM attacker to force a protocol downgrade. Some client applications (such as browsers) will reconnect using a downgraded protocol to work around interoperability bugs in older servers. This could be exploited by an active man-in-the-middle to downgrade connections to SSL 3.0 even if both sides of the connection support higher protocols. SSL 3.0 contains a number of weaknesses including POODLE (CVE-2014-3566).

201501120301

Symptom: A routing policy contains a high-priority deny node and a low-priority permit node with the action of redirecting traffic to a next hop. Traffic matching both nodes is not forwarded based on the high-priority deny node. Instead, the traffic is forwarded based on the low-priority permit node and redirected to the next hop.

Condition: This symptom can be seen when a flow matches the following nodes of a routing policy at the same time:

A high-priority deny node.

A low-priority permit node with the action of redirecting traffic to a next hop.

201412190129

Symptom: The switch reboots unexpectedly.

Condition: This symptom can be seen when the switch receives NTP control packets with the Mode field being 6.

201412220343

Symptom: After a master/subordinate switchover in an IRF fabric, the dhcp-snooping check mac-port command configuration is lost.

Condition: This symptom can be seen after the following procedure is performed in the IRF fabric:

Execute the dhcp-snooping check mac-port command.

Save the configuration.

Perform a master/subordinate switchover.

201409240424

Symptom: The dhcp-snooping check mac-port command configuration does not take effect. After a client that has been assigned an IP address on port 1 is moved to port 2, the client can still be assigned an IP address.

Condition: This symptom can be seen after the following procedure is performed:

Execute the dhcp-snooping check mac-port command.

Move a client that has been assigned an IP address on port 1 to port 2.

201501160266

Symptom: The switch reboots unexpectedly.

Condition: This symptom can be seen when the following conditions exist in a tunneling network:

The switch receives an IP packet with a specific option. The option is not IPOPT_EOL (0) or IPOPT_NOP (1). The second byte of the option is 0.

The IP packet is too long and needs to be fragmented.

Resolved problems in R2221P07

201410110181

Symptom: IP broadcast packets cannot be relayed and forwarded.

Condition: This symptom occurs when the udp-helper server command is executed on a Layer 3 virtual interface to configure the IP address of the destination server for UDP helper as a subnet broadcast address.

201410110326

Symptom: The system displays an ARP conflict prompt for the MAD IP addresses.

Condition: This symptom occurs when the following procedure is performed:

Configure BFD MAD in the IRF fabric.

Configure the arp ip-conflict prompt command.

The switch receives TCN BPDUs.

201411190489

Symptom: The switch drops the packets sent by a user that comes online after passing MAC authentication.

Condition: This symptom occurs when the MAC address entries for the user that comes online after passing MAC authentication is deleted after the switch receives TCN BPDUs.

201411030489

Symptom: Subordinate IRF member switches reboot.

Condition: This symptom occurs when the following procedure is performed:

Configure the IRF fabric as the DHCP Server to allocate IP addresses in the extended address pool.

A master/subordinate switchover occurs in the IRF fabric.

A client releases its IP address. The IP address will exist in both the free IP list and the conflicting IP list.

The IP address is obtained by a client again.

201411180434

Symptom: The RADIUS protocol packets that the switch receives on the interface connected to the authentication server are dropped. As a result, a user fails to pass authentication.

Condition: This symptom occurs when the following conditions exist:

The switch is configured with RADIUS to authenticate and authorize users.

The interface connected to the user receives a large number of packets with unknown source MAC addresses.

Resolved problems in R2221P06

201408280078

Symptom: CVE-2008-5161

Description: Error handling in the SSH protocol in several SSH servers/clients, including OpenSSH 4.7p1 and possibly other versions, when using Cipher Block Chaining (CBC) mode, makes it easier for remote attackers to recover certain plaintext data.

201408140565

Symptom: CVE-2014-3508

Condition: A flaw in OBJ_obj2txt may cause pretty printing functions such as X509_name_oneline, X509_name_print_ex et al. to leak some information from the stack. Applications may be affected if they echo pretty printing output to the attacker.

201409150368

Symptom: The switch keeps generating log messages showing that the MAC learning limit has been reached on a port.

Condition: This symptom occurs if the mac-address max-mac-count value command is executed on two or more ports.

201409050021

Symptom: A user cannot pass the RADIUS authentication.

Condition: This symptom occurs when the attributes issued by the RADIUS server are as follows during the RADIUS authentication/authorization process:

The attribute 65 (Tunnel-Medium-Type) is set to 802.

The attribute 64 (Tunnel-Type) is set to VLAN.

No VLAN ID is configured in the attribute 81 (Tunnel-Private-Group-ID).

201408190596

Symptom: IPv6 portal users cannot be forcibly logged out by configuring the security policy server.

Condition: N/A.

Resolved problems in R2221P05

201407030446

Symptom: SPI conflicts occur during IKE SA establishment.

Condition: This symptom can be seen when the switch uses IKE autonegotiation to establish SAs with the peer.

201408050575

Symptom: Static routes might fail to take effect.

Condition: This symptom might be seen after an IRF master/subordinate switchover.

201407030392

Symptom: A software upgrade through IMC BIMS fails.

Condition: This symptom can be seen if you use IMC BIMS to upgrade software.

201407250142

Symptom: ND snooping fails to create entries on a port.

Condition: This symptom can be seen if the port is enabled with port security.

201408040236

Symptom: IMC fails to obtain MAC entries from a switch.

Condition: This symptom can be seen if the MAC entries are on an IRF subordinate switch and they are secure MAC entries.

201407280518

Symptom: The voice vlan qos command does not take effect on a port.

Condition: This symptom can be seen if the port is configured with lldp voice-vlan.

201407220494

Symptom: After commands are pasted in interface range view, some commands fail to be executed.

Condition: This symptom can be seen after commands are pasted in interface range view.

201407080366

Symptom: After an IRF split, the switches are forced to wait for three seconds to start up.

Condition: This symptom can be seen if an IRF fabric that is not configured with MAD splits.

201406200507

Symptom: A port does not learn MAC addresses.

Condition: This symptom can be seen if the following procedure is performed on the port:

Enable port security.

Configure port-based 802.1X authentication (userlogin).

Configure guest VLAN for 802.1X authentication.

Disable port security.

201408140267

Symptom: The switch generates log messages when MMU parity errors occur.

Condition: This symptom can be seen when MMU parity errors occur.

Resolved problems in R2221P04

201405190421

Symptom: A portal client fails to pass portal authentication.

Condition: This symptom can be seen if the following conditions exist:

The portal client, portal server, and RADIUS server belong to the same VPN instance.

A route that matches the IP address of the portal client exists in the public network or another VPN instance.

201406040506

Symptom: A client fails to ping the gateway address.

Condition: This symptom can be seen if the gateway address is in an 802.1X Free IP network.

201406130469

Symptom: On an IRF fabric, a port might fail to quit the 802.1X Guest VLAN after a user passes 802.1X authentication on the port.

Condition: This symptom can be seen after a user passes 802.1X authentication on a port in the 802.1X Guest VLAN.

201405130384

Symptom: The MAC addresses of authenticated users are aged out before the offline-detect timer expires.

Condition: This symptom can be seen when MAC authentication is enabled.

201404160027

Symptom: A DHCP client that moves from a port to another port of a DHCP snooping switch fails to re-obtain an IP address.

Condition: This symptom occurs if the dhcp-snooping no-user-binding command is configured on the downlink port of the switch that connects to the client.

201404240078

Symptom: The device provides no prompt information when the number of MAC entries exceeds the upper limit on a port.

Condition: This symptom occurs if the port is configured with voice VLAN.

201404160434

Symptom: A transient loop occurs in a smart link network.

Condition: This symptom occurs if the primary and secondary ports of the smart link group reside on different IRF member switches and the primary link recovers from a failure.

201406100280

Symptom: CVE-2014-0224

Condition: When Open SSL Server or Client is used.

Resolved problems in R2221P02

201402250517

Symptom: A user fails 802.1X authentication.

Condition: This symptom occurs if the server assigns the user an ACL name.

201402270224

Symptom: A user fails to log in to the switch.

Condition: This symptom occurs if the following conditions exist:

The user uses RADIUS authentication.

The RADIUS server assigns multiple login-service attributes for the user.

201402250220

Symptom: After an inactive combo interface is activated, the bpdu-drop any setting configured on the interface does not take effect.

Condition: This symptom can be seen after an inactive combo interface configured with bpdu-drop any is activated.

201403010118

Symptom: The server fails to assign an authorized VLAN to a user who has passed 802.1X, MAC, or portal authentication.

Condition: This symptom occurs if the authorized VLAN ID is a character string ended with null characters, such as 0x0032313900.

201404020445

Symptom: A DHCP client takes a long time to request an IP address.

Condition: This symptom occurs when the VLAN interface enabled with the DHCP server is not on the same subnet as the IP address requested by the DHCP client. The DHCP server does not respond with a NAK packet, so the client sends the request multiple times before sending a Discovery packet.

201404020414

Symptom: The switch unexpectedly reboots when the DHCP server receives a DHCP request.

Condition: This symptom occurs if the DHCP request contains Option 82 sub-option 5 that is longer than four bytes.

201404020474

Symptom: The CPU usage is 100% after a static route is configured.

Condition: This symptom occurs if the following conditions exist:

The static route has a nonexistent next hop that belongs to the static route's destination network.

There is a route destined to a super network that comprises the static route's destination network, or there is a default route.

201404040414

Symptom: Using SSH user accounts A and B on SecureCRT fails to log in to the switch through Stelnet or Telnet.

Condition: This symptom occurs if the following conditions exist:

Account A uses password authentication, and account B uses password-public key authentication.

Using account A fails to log into the switch and then use account B to log into the switch.

201403120056

Symptom: After a port goes down, dynamic secure MAC entries on the port cannot move to other ports.

Condition: This symptom occurs when a port configured with autolearn security mode and dynamic MAC learning goes down.

201404180344

Symptom: The default ARP rate-limit setting fails to be restored after the undo arp rate-limit command is configured.

Condition: This symptom can be seen if the following procedure is performed:

Enable ARP detection.

Execute the arp rate-limit rate command.

Execute the undo arp rate-limit command.

201404100060

Symptom: A portal-free rule configured with source ip any can be assigned for Layer 2 portal authentication.

Condition: This symptom can be seen when a portal-free rule is configured with source ip any.

201404020238

Symptom: A Web user can delete local users.

Condition: This symptom can be seen when the Web user has a level 2 privilege.

Resolved problems in R2221P01

201312230375

Symptom: The switch is attacked by IPv6 packets with a TTL of 1.

Condition: This symptom might be seen when the switch is disabled from sending ICMPv6 timeout packets.

201402190416

Symptom: The active physical port of a Combo interface automatically enables the STP BPDU drop function.

Condition: This symptom occurs if the inactive physical port of the Combo interface is configured with bpdu-drop any.

201401150506

Symptom: The DHCP server on the switch fails to assign IP addresses to clients.

Condition: This symptom occurs if PBR matching broadcast packets is configured on the switch.

Resolved problems in R2221

201311280368

Symptom: An interface on an LSPM2GP2P/LSPM2SP2P card cannot come up.

Condition: This symptom occurs after the following procedure is performed:

  - Install an LSPM2GP2P/LSPM2SP2P card on the switch.

  - Use the shutdown command to shut down an interface on the LSPM2GP2P/LSPM2SP2P card.

  - Save the configuration and reboot the switch.

  - After the switch is rebooted, execute the undo shutdown command on the interface.

201310160360

Symptom: The switch unexpectedly reboots when it repeatedly loads and unloads a hot patch for link aggregation.

Condition: This symptom might occur when the switch repeatedly loads and unloads a hot patch for link aggregation.

201308160251

Symptom: ARP Scan does not work on Layer 3 interfaces of IRF subordinate switches.

Condition: This symptom can be seen on Layer 3 interfaces of IRF subordinate switches.

201309220392

Symptom: The CDP neighbor continually ages out.

Condition: This symptom occurs if the CDP neighbor is a Cisco's LLDP-capable phone.

201311150133

Symptom: A DHCP response is discarded during inter-VLAN forwarding. The DHCP client thus fails to obtain an IP address.

Condition: This symptom can be seen when DHCP snooping is globally enabled and multiple VLANs are configured.

201310230091

Symptom: Modifying DHCP option 60 fails on the switch that acts as the DHCP server.

Condition: This symptom can be seen when you modify DHCP option 60 on the switch that acts as the DHCP server.

Resolved problems in R2220P11

201311280103

Symptom: Addressed SSRT101324.  A security bulletin for SSRT101324 should be published in January 2014.  Please see the security bulletin for additional details.

Condition: Addressed SSRT101324.  A security bulletin for SSRT101324 should be published in January 2014.  Please see the security bulletin for additional details.

Resolved problems in R2220P09

ZDD06392

Symptom: The switch unexpectedly reboots because of an SNMP agent anomaly.

Condition: This symptom occurs when the following conditions exist:

SNMPv2, SNMPv3, SSH and DHCP attacks exist.

The SMMP agent on the switch receives an SNMPv3 packet that is larger than the globPDUSize (the default is 1500) and the contextName field of the SNMPv3 packet is almost the globPDUSize.

LSD075609

Symptom: After a static route becomes invalid due to power-down of the master, the track entry bound to the static route is still in positive state.

Condition: This symptom occurs after a static route becomes invalid due to power-down of the master.

LSD075361

Symptom: sFlow MIBs such as sFlowRcvrTimeout fail to be set.

Condition: This symptom can be seen when sFlow is enabled.

LSD075443

Symptom: The switch unexpectedly reboots after two voice NQA operations that have the same source IP address but different destination IP addresses have been performed for a certain time.

Condition: This symptom occurs if two voice NQA operations that have the same source IP address but different destination IP addresses have been performed for a certain time.

201307310278

Symptom: The System Information Web page displays garbled characters under the Temperature option of the System Resource State field.

Condition: This symptom can be seen if you use IE10 to access the Web interface of the switch.

201308290108

Symptom: When SNMP Agent V3 is used to obtain the values of two MIB variables, the first value is Null.

Condition: This symptom can be seen if the second MIB variable is ifTableLastChange or ifStackLastChange.

201308300169

Symptom: When an ACS server acts as the RADIUS server, the switch fails to assign priorities for authenticated SSH users.

Condition: This symptom occurs when an ACS server is used as the RADIUS server to authenticate SSH users.

201308060203

Symptom: The hh3cSysImageName MIB cannot be read.

Condition: This symptom occurs if hh3cSysImageName MIB is read.

LSD074587

Symptom: Device could not handle invalid SNMP packet and resulted in an exception.

Condition: Device received an invalid SNMP packet with overlong OID.

LSD074729

Symptom: Device could not handle SSH packet which had many special characters and resulted in an exception.

Condition: Device received a SSH packet which had many 0x07 as control-character.

LSD075470

Symptom: Device could not handle invalid SNMP packet and resulted in an exception.

Condition: Device received an invalid SNMP packet which had an oversize ContextName field.

LSD075250

Symptom: A switch fails to communicate with a Cisco's 6509 device through STP.

Condition: This symptom can be seen when a switch tries to communicate with a Cisco's 6509 device through STP.

LSD074592

Symptom: The switch discards ARP packets with multiple VLAN tags received from a QinQ-enabled interface.

Condition: This symptom occurs if the QinQ-enabled interface is a trunk or hybrid interface that permits VLANs on which ARP detection or ARP snooping is enabled.

LSD075296

Symptom: A switch does not send traps to the NMS after it recovers from high-temperature alarm state.

Condition: This symptom can be seen after a switch recovers from high-temperature alarm state.

ZDTB00321

Symptom: The CPU usage on a PVST-enabled Switch is 100% when the switch receives a lot of TC packets.

Condition: This symptom occurs when the device enables PVST and receives a lot of TC packets.

ZDTB00323

Symptom: A switch unexpectedly reboots when it performs IKE negotiation with a TOPSEC device.

Condition: This symptom occurs when a switch performs IKE negotiation with a TOPSEC device.

ZDTB00324

Symptom: A 10-second traffic interruption occurs during an IRF split.

Condition: This symptom can be seen if the following conditions exist:

LACP MAD is enabled on the IRF fabric

The number of member ports in the link aggregation group that connect the intermediate device to the IRF fabric almost reach or has reached the upper limit.

An IRF split occurs.

LSD074994

Symptom: If a port that has learned an authenticated MAC address in multiple VLANs leaves a VLAN, the authenticated MAC address is removed from that VLAN and also from all other VLANs.

Condition: This symptom occurs if the following conditions exist:

The port is a trunk/hybrid port.

The port is enabled with MAC authentication multi-VLAN mode.

The port has learned an authenticated MAC address in multiple VLANs.

The port leaves a VLAN.

LSD074756

Symptom: The master device of the IRF fabric abnormally reboots.

Condition: The contact or location configured in SNMP contains more than 200 characters. A user logs into the Web NMS.

LSD074592

Symptom: The software drops the multi-tagged ARP packets received from a QinQ-enabled port.

Condition: Configure the link type of a port as trunk or hybrid, enable QinQ on the port, and assign the port to a VLAN with ARP detection or ARP snooping enabled.

LSD074248

Symptom: Directly connected devices cannot ping each other.

Condition: Configure a Layer 2 aggregation group in the Web interface. When you choose multiple member ports and add them to the aggregation group in batches, the member ports become Selected in the platform, but part of the ports are blocked in the driver.

LSD074348

Symptom: No DHCP snooping entries exist on the device.

Condition: As shown in the following figure, DHCP snooping and the DHCP relay agent are enabled on the device. The client obtains an IPv6 address from the DHCP server.

Resolved problems in R2220P02

None

Resolved problems in R2220P01

LSD074256

Symptom: When a DHCP relay device receives an Option 82-included packet in which the length value specified for the Agent Information Field is larger than the actual length of the Agent Information Field, the device reboots.

Condition: This symptom occurs when the DHCP relay device receives an Option 82-included packet in which the length value specified for the Agent Information Field is larger than the actual length of the Agent Information Field.

LSD074279

Symptom: After a user fails and then passes 802.1X authentication, the MAC address of the user in the 802.1X critical VLAN cannot be deleted.

Condition: This symptom occurs if the configured 802.1X critical VLAN ID is the same as the PVID on the port.

LSD074423

Symptom: After a PC passes MAC authentication on a port, the port still discards IGMP report packets from the PC.

Condition: This symptom occurs if the port works in userlogin-secure-or-mac-ext mode and is enabled with both MAC authentication and 802.1X authentication.

LSD074302

Symptom: After a switch reboots, the 10 GE fiber port on interface card 2 goes up but cannot forward packets and the STP state on the port is inactive.

Condition: This symptom might occur when the following conditions exist:

Interface card 1 is not in position.

Interface card 2 is inserted with a 10 GE transceiver module and the STP state on the fiber port is forwarding.

The switch is rebooted.

LSD073833

Symptom: After NTP traps are disabled with the “undo snmp-agent trap enable system” command, NTP traps are still generated.

Condition: This symptom can be seen although NTP traps have been disabled with the “undo snmp-agent trap enable system” command.

Resolved problems in R2220

LSD072929

Symptom: A switch in an IRF fabric might forward broadcast packets received from a member port of a link aggregation group to another member port in the same group.

Condition: This symptom might occur if the member ports of the link aggregation group reside on different switches in the IRF fabric, and some member ports go up and down.

LSD073000

Symptom: A port might fail to forward packets when its duplex state is continually changed.

Condition: This symptom might occur if the port continually changes its duplex state and meanwhile multiple ports are congested.

ZDTB00302

Symptom: During an IRF master/subordinate switchover, a user that is accessing a device connected to the IRF fabric has traffic interruption that lasts more than 1 minute.

Condition: This symptom might occur if an IRF master/subordinate switchover is performed when a user is accessing a device connected to the IRF fabric.

LSD072889

Symptom: When the switch acts as the SSH server, the first SSH login to the switch times out.

Condition: This symptom occurs if DSA, RSA, or hotkey is not configured, because the system needs time to create the key.

LSD072187

Symptom: When access the hh3cUserPassword node of hh3cUserInfoTable by SNMP, the device return the user's password.

Condition: Access the hh3cUserPassword node of hh3cUserInfoTable by SNMP.

ZDTB00298

Symptom: The switch might reboot if the portal authentication function on a VLAN interface is disabled when portal users are going online.

Condition: This symptom might occur if you disable portal authentication function on a VLAN interface when portal users are going online.

LSD071247

Symptom: After an 802.1X user on a port that uses MAC-based authentication moves to another port that uses port-based authentication, a long time service interruption occurs to the user.

Condition: This symptom occurs because the MAC entry for the user is not promptly updated after the user moves from a port that uses MAC-based authentication to a port that uses port-based authentication.

LSD071429

Symptom: After a successful configuration restoration through Web, the Web page prompts “Failed to backup the configuration file” and “Failed to write the file to the SMB.”

Condition: This symptom occurs after a successful configuration restoration for a single switch through Web.

LSD071198

Symptom: When a loopback occurs to a Combo interface, the interface is error blocked by loopback detection.

Condition: This symptom might occur when the following conditions exist:

Loopback detection is enabled for the Combo interface and has not configure the command of “loopback-detection action” in　the port view.

The Combo interface is a trunk interface.

A loopback occurs to the Combo interface.

LSTB005549

Symptom: The Web page of the switch displays incorrect information for a cluster member device.

Condition: This symptom occurs when the switch serves as the cluster management device.

LSD070368

Symptom: The irf link-delay changed for an IRF fabric through Web does not take effect on the IRF subordinate switch.

Condition: This symptom occurs after you change the irf link-delay for an IRF fabric through Web.

LSD070270

Symptom: After an IRF master/subordinate switchover, some routes might fail to be assigned.

Condition: This symptom might occur if routes learned on the IRF fabric have exceeded the upper limit before the IRF master/subordinate switchover.

LSD070096

Symptom: The configuration replace file command fails to roll back configuration for NTDP.

Condition: This symptom occurs when the configuration replace file command is executed to perform a configuration rollback.

LSD070367

Symptom: LLDP configuration cannot be displayed or made in the Web interface for a port where a sting of med-tlv location-id civic-address has been configured.

Condition: This symptom occurs for a port where a sting of med-tlv location-id civic-address has been configured.

LSD070273

Symptom: ND snooping entries that have been cleared on a port can still be displayed.

Condition: This symptom might occur if the port goes down when its ND snooping entries have reached the upper limit, and then the ND snooping entries on the port are cleared.

LSD070271

Symptom: If the MVRP registration mode is changed repeatedly for a long time, the memory might be exhausted. If the operations are performed on an IRF fabric, the IRF fabric might split.

Condition: This symptom might occur if the MVRP registration mode is changed repeatedly for a long time.

LSD070933

Symptom: Rebooting a cluster member device from the management device fails.

Condition: This symptom occurs if the port of the member device does not permit the VLAN identified by the PVID.

LSD070826

Symptom: After a cluster has been established, if you configure the IP address of the management VLAN interface, save the configuration, and then reboot the management device, the IP address of the management VLAN interface gets lost, and the cluster cannot be reestablished.

Condition: This symptom occurs if you configure the IP address of the management VLAN interface, save the configuration, and then reboot the management device after a cluster has been established.

LSD070822

Symptom: The VLAN information of a port displayed on the Network->VLAN->Port details Web page is not complete.

Condition: This symptom occurs if the port is added to many non-contiguous VLANs.

LSD070843

Symptom: The cluster management and delete options on the Cluster->Cluster Web page are not unusable.

Condition: This symptom exists on the Cluster->Cluster Web page.

LSD073378

Symptom: In a RADIUS AAA scenario, RADIUS accounting packets carry incorrect traffic statistics.

Condition: This symptom might occur if a dot1x user has been online for a long time in port security mode.

LSD074033

Symptom: The switch fails to learn new ARP entries when some ARP entries have errors.

Condition: This symptom might be seen when the following conditions exist:

Inter-VPN traffic exists.

Multiple ARP entries contain the same MAC address, and the egress port to the MAC address of one ARP entry is changed.

LSD073845

Symptom: ARP does not learn the addresses in an ARP reply in which the target MAC address in the message body is different from the destination MAC address in the message header.

Condition: This symptom can be seen when the switch receives an ARP reply in which the target MAC address in the message body is different from the destination MAC address in the message header.

LSD073575

Symptom: After a system reboot, the undo startup bootrom-access enable command does not take effect, and pressing Ctrl-B can access the BootROM menu.

Condition: This symptom can be seen if a switch configured with the undo startup bootrom-access enable command is rebooted.

Resolved problems in F2218

LSD072331

Symptom: After a reboot, the super password level x hash setting gets lost from the configuration file.

Condition: This symptom might occur after a reboot.

LSD072325

Symptom: A user that has passed MAC-based 802.1X authentication on a port cannot access the network.

Condition: This symptom might occur if some ports are configured with MAC-based 802.1X authentication and some other ports are configured with port-based 802.1X authentication and guest VLAN.

LSD072504

Symptom: If the mac-address station-move quick-notify enable command is configured on the RRPP master node, a link-down event on a transit node cannot be quickly reported.

Condition: This symptom occurs if the mac-address station-move quick-notify enable command is configured on the RRPP master node.

LSD072330

Symptom: After the switch starts up, Combo interface A cannot come up. After the transceiver module on Combo interface A is removed and inserted, Combo interface A comes up.

Condition: This symptom might occur if Combo interface A connects to a Phy-included 100M transceiver module such as 3COM3CSFP9-81/3CSFP9-82.

Resolved problems in F2217

ZDTB00288

Symptom: The IP address of a Null interface assigned through SNMP cannot be deleted.

Condition: This symptom might occur on a Null interface whose IP address is assigned through SNMP.

ZDTB00293

Symptom: A walk of lldpRemSysName MIB returns "No Such Instance currently exists at this OID".

Condition: This symptom might occur when the port has an LLDP neighbor and the TimeFilter is set to 0.

LSTB005612

Symptom: The etherStatsOversizePkts field has an exceptionally large value in the output of the display rmon statistics command.

Condition: This symptom might occur if the reset count interface command is executed on a port configured with RMON accounting when the port has traffic.

LSD071986

Symptom: This symptom might occur on a switch where MFF is enabled on some VLANs to which a combo port belongs but is disabled on other VLANs of the combo port , save configuration and the switch  cannot boot up.

Condition: None.

LSD071810

Symptom: The value obtained by an SNMP walk of probeCapabilities MIB is incorrect.

Condition: This symptom might occur during an SNMP walk of probeCapabilities MIB.

LSD071866

Symptom: A port connected to a client that fails and then passes 802.1X authentication cannot leave the guest VLAN. As a result, the client cannot access the network.

Condition: This symptom might occur if the both following conditions exist:

(1)802.1X is enabled on the port, and the guest VLAN and Auth-Fail VLAN are configured as the same VLAN.

(2)The port is assigned to the guest VLAN after the client fails authentication, and then the client passes authentication.

LSD071501

Symptom: A client that passes 802.1X authentication cannot access the network.

Condition: This symptom might occur when the both following conditions exist on the port connected to the client:

(1)MAC authentication and 802.1X authentication are both configured and the VLAN to which the port is assigned when the client passes MAC authentication is configured.

(2)802.1X authentication is performed after MAC authentication succeeds.

LSD071635

Symptom: A port cannot come up after the fiber or fiber transceiver connected to the port is inserted and removed multiple times.

Condition: This symptom might occur after the fiber or fiber transceiver connected to a port is inserted and removed multiple times.

LSD071876

Symptom: The received optical power of some fiber transceiver (for example, WTD modules) is incorrectly displayed.

Condition: This symptom exists in the output of the display transceiver diagnosis command.

Resolved problems in R2215

LSD69938

Symptom: After an IRF master/subordinate switchover, configuring PoE ports through a PoE profile fails.

Condition: This symptom might occur if you use a PoE profile to configure PoE ports after an IRF master/subordinate switchover.

ZDD04994

Symptom: The CLI does not respond to an NMS that uses SNMPv3 with 3DES to access the switch.

Condition: This symptom might occur when an NMS uses SNMPv3 with 3DES to access the switch.

LSD070340

Symptom: The switch reboots when an SNMPv3 client accesses it.

Condition: This symptom might occur if the SNMPv3 client matches an ACL rule that is configured with the logging keyword.

LSD070554

Symptom: Saving a large configuration file through Web fails.

Condition: This symptom might occur when you use Web to save a large configuration file.

LSD67940

Symptom: CDP packets have incorrect checksums.

Condition: This symptom might occur when the lldp compliance cdp command is configured in system view.

LSD070943

Symptom: IMC fails to walk the hh3cSysCurImageIndex MIB node and cannot get version information.

Condition: This symptom might occur when IMC walks the hh3cSysCurImageIndex MIB node to get version information.

LSD64524

Symptom: LLDP packets have incorrect auto-negotiation capability values in TLVs and relevant MIB values displayed are also incorrect.

Condition: This symptom might occur when the switch is enabled with LLDP.

Resolved problems in F2212P02

LSD66990

Symptom: Insert cables into all ports and all ports are up ,and the CPU usage is high.

Condition: None.

Resolved problems in F2212L01

LSD69433

Symptom:  Users matching the ACL cannot perform SNMP operations to the device.

Condition: Use an ACL to allow specific users to perform SNMP operation to the device.

Resolved problems in F2212

LSD66630

Symptom: The panels of HP TAA devices are wrongly displayed.

Condition:  Entering the web interface.

LSD67507

Symptom: Only 32 ARP entries are updated when more than 32 MAC addresses are moved to different ports.

Condition: This symptom might occur when more than 32 MAC addresses are moved to different ports.

LSD68504

Symptom: A QoS policy fails to be applied.

Condition: This symptom might occur when the traffic behaviors of the QoS policy include both Car and Remark actions.

LSD65758

Symptom: There is some checksum error information when insert old 3Com SFP transceivers.

Condition: None.

Resolved problems in F2211

LSD64553

Symptom: The switch cannot be rebooted from an NMS (for example, IMC) through SNMP.

Condition: This symptom might occur if the NMS uses SNMP to reboot the switch.

LSD65459

Symptom: Memory leaks occur on the HTTP module and the switch might reboot if the switch receives large amounts of HTTP packets with the same fields.

Condition: This symptom might occur if the switch receives large amounts of HTTP packets with the same fields.

LSD65492

Symptom: The switch drops IP fragments.

Condition: This symptom might occur when DHCP snooping is enabled globally.

LSD65749

Symptom: The returned value of dot3StatsDuplexStatus MIB on a down port is not "unknown" but is the same as the value (for example, full-duplex) when the port is up.

Condition: This symptom might occur during a walk of dot3StatsDuplexStatus MIB on a port that is down.

Resolved problems in R2210

LSD62084

Symptom: A MAC address that has passed authentication in a VLAN cannot be authenticated in other VLANs.

Condition: None.

LSD57794

Symptom: The port information in trap messages is incorrect when MAC addresses are added or removed on a port configured with mac-address information.

Condition: This symptom exists in trap messages generated when MAC addresses are added or removed on a port configured with mac-address information.

LSD62470

Symptom: The switch performs unknown unicast storm suppression according to all unicast packets, including known unicast packets and unknown unicast packets.

Condition: This symptom might occur if storm-constrain mode for unicast traffic is configured as PPS.

LSD62954

Symptom: EAP failure messages sent by the switch do not conform to RFC3748.

Condition: This symptom exists in EAP failure messages sent by the switch.

LSD58411

Symptom: During IRF split and merge, LACP MAD on an aggregate interface fails to work because some member ports cannot be selected.

Condition: This symptom might occur if the aggregate interface is configured as a reserved interface and IRF split and IRF merge occur.

TCD02667

Symptom: Some online users are logged off when many users get online through a port enabled with MAC-based VLAN, Voice VLAN, and 802.1X.

Condition: This symptom might occur if the port is enabled with MAC-based VLAN, Voice VLAN, and 802.1X and too many users get online through the port.

LSD60563

Symptom: When an SSH user passes TACACS+ authentication but fails authorization, a memory access anomaly occurs, resulting in protocol interruption or system reboot.

Condition: This symptom might occur when an SSH user passes TACACS+ authentication but fails authorization.

Resolved problems in R2208

LSD57597

Symptom: The Option field of DHCP packets received by the switch has incorrect information.

Condition: This symptom might occur when the switch is configured with the DHCP snooping option and the receiving port is configured with QinQ.

ZDD03868

Symptom: An NQA operation fails.

Condition: This symptom might occur when a next hop is specified for the NQA operation.

LSD55389

Symptom: The configured NAS IP address cannot be assigned.

Condition: This symptom might occur if the least significant octet of the NAS IP address is 255.

LSD58705

Symptom: The NTP peer on a VLAN interface cannot be removed.

Condition: This symptom might occur if you perform the following steps multiple times:

a. Configure the NTP broadcast-server or NTP broadcast-client on the VLAN interface.

b. Configure the NTP peer on the VLAN interface.

c. Remove the VLAN interface' IP address, the VLAN interface, and the NTP peer.

LSD60991

Symptom: The switch cannot establish LLDP neighbor relationship with a Cisco IP phone.

Condition: This symptom might occur when the switch connects to the Cisco IP phone through LLDP.

LSD59580

Symptom: The mac-address max-mac-count configuration cannot take effect when you make this configuration on two ports whose port number difference is 24 (for example, g1/0/1 and g1/0/25 or g1/0/3 and g1/0/27).

Condition: This symptom might occur if you configure mac-address max-mac-count on two ports whose port number difference is 24 on an HP A5120-48G EI/HP A5120-48G EI TAA /HP A5120-48G-PoE+ EI/HP A5120-48G-PoE+ EI TAA switch.

LSD60395

Symptom: An SNMP walk of dot1qVlanStaticUntaggedPorts MIB returns an incorrect value.

Condition: This symptom occurs during an SNMP walk of dot1qVlanStaticUntaggedPorts MIB.

ZDD03986

Symptom: The 64-byte memory is corrupted when a packet received from the RADIUS server contains a 63-byte callback number, resulting in command resolution failure or a system reboot.

Condition: This symptom might occur if a packet received from the RADIUS server contains a 63-byte callback number.

LSD59641

Symptom: Authentication and accounting servers configured on the IMC fail to be assigned.

Condition: This symptom might occur when you use IMC to configure and assign authentication and accounting servers.

LSD60392

Symptom: The bpdu-drop any configuration gets lost on some ports of a subcard.

Condition: This symptom might occur after you configure the bpdu-drop any command on the subcard and then remove and insert the subcard.

  - Resolved Problems in R2208

First release

Support and other resources

Accessing Hewlett Packard Enterprise Support

For live assistance, go to the Contact Hewlett Packard Enterprise Worldwide website:

www.hpe.com/assistance

To access documentation and support services, go to the Hewlett Packard Enterprise Support Center website:

www.hpe.com/support/hpesc

Information to collect:

Technical support registration number (if applicable).

Product name, model or version, and serial number.

Operating system name and version.

Firmware version.

Error messages.

Product-specific reports and logs.

Add-on products or components.

Third-party products or components.

Documents

To find related documents, see the Hewlett Packard Enterprise Support Center website at http://www.hpe.com/support/hpesc.

Enter your product name or number and click Go. If necessary, select your product from the resulting list.

For a complete list of acronyms and their definitions, see HPE FlexNetwork technology acronyms.

Related documents

The following documents provide related information:

HP 5120 EI & 5120 SI Switch Series Installation Guide

HP 5120 EI Switch Series Command References-Release 2220

HP 5120 EI Switch Series Configuration Guides-Release 2220

Documentation feedback

Hewlett Packard Enterprise is committed to providing documentation that meets your needs. To help us improve the documentation, send any errors, suggestions, or comments to Documentation Feedback (docsfeedback@hpe.com). When submitting your feedback, include the document title, part number, edition, and publication date located on the front cover of the document. For online help content, include the product name, product version, help edition, and publication date located on the legal notices page.

Feature list

Hardware features

        - A5120 EI series hardware features (I)

| Item |  | A5120-24G EI (2 slots)/A5120-24G EI TAA (2 slots) | A5120-24G EI | A5120-48G EI (2 slots)/A5120-48G EI TAA (2 slots) | A5120-48G EI |
| --- | --- | --- | --- | --- | --- |
| Dimensions (H × W × D) |  | 43.6 × 440 × 300 mm (1.72 × 17.32 × 11.81 in) |  | 43.6 × 440 × 300 mm (1.72 × 17.32 × 11.81 in) |  |
| Weight |  | < 4.5 kg (9.92 lb) |  | < 5 kg (11.02 lb) |  |
| Management port |  | One console port, on the front panel |  |  |  |
| Fixed network ports (on the front panel) |  | 24 × 10/100/1000Base-T auto-sensing Ethernet port 4 × 1000Base-X SFP port |  | 48 × 10/100/1000Base-T auto-sensing Ethernet port 4 × 1000Base-X SFP port |  |
|  |  | The last four 10/100/1000Base-T Ethernet ports and the four SFP ports comprise four combo interfaces. For each combo interface, either the SFP port or the corresponding Ethernet port can be used at a time. For the port pairs forming combo interfaces, see Table 8 . |  |  |  |
| Interface card slots |  | The A5120-24G EI and A5120-48G EI do not support interface cards. The other models can accommodate two interface cards. |  |  |  |
| Interface card models |  | LSPM2GP2P (JD367A) (not supporting IRF) LSPM1CX2P (JD360B) (supporting IRF) LSPM2SP2P (JD368B) (supporting IRF) LSPM1XP2P (JD359B) (supporting IRF) LSPM1XP1P (JD361B) (supporting IRF) |  |  |  |
| Power receptacle types and quantity |  | The switch models have two fixed power receptacles, one AC receptacle, and one RPS receptacle. The two power inputs can be simultaneously used, acting as backup for each other. Alternatively, you can use either power input. |  |  |  |
| Input voltage | AC | Rated voltage range: 100 VAC to 240 VAC, 50 Hz or 60 Hz Input voltage range: 90 VAC to 264 VAC, 47 Hz to 63 Hz |  |  |  |
|  | RPS | The rated voltage range is 10.8 VDC to 13.2 VDC. Use the external RPS power supply unit—A-RPS800 (JD183A)—recommended by HPE only. |  |  |  |
| Minimum power consumption |  | 36 W | 35 W | 55 W | 54 W |
| Maximum power consumption |  | 103 W | 62 W | 145 W | 110 W |
| Cooling system |  | 4 fans |  |  |  |
| Operating temperature |  | 0°C to 45°C (32°F to 113°F) |  |  |  |
| Relative humidity (noncondensing) |  | 10% to 90% |  |  |  |

        - A5120 EI series hardware features (II)

| Item |  |  | A5120-24G-PoE+ EI (2 slots) A5120-24G-PoE+ EI TAA (2 slots) | A5120-48G-PoE+ EI (2 slots) A5120-48G-PoE+ EI TAA (2 slots) |
| --- | --- | --- | --- | --- |
| Dimensions (H × W × D) |  |  | 43.6 × 440 × 420 mm (1.72 × 17.32 × 16.54 in) | 43.6 × 440 × 420 mm (1.72 × 17.32 × 16.54 in) |
| Weight |  |  | < 7.0 kg (15.43 lb) | < 7.5 kg (16.53 lb) |
| Management port |  |  | 1 console port, on the front panel |  |
| Fixed network ports (on the front panel) |  |  | 24 × 10/100/1000Base-T auto-sensing Ethernet port (support PoE+) 4 × 1000Base-X SFP port | 48 × 10/100/1000Base-T auto-sensing Ethernet port (support PoE+) 4 × 1000Base-X SFP port |
|  |  |  | The last four 10/100/1000Base-T Ethernet ports and the four SFP ports comprise four combo interfaces. For each combo interface, either the SFP port or the corresponding Ethernet port can be used at a time. For the port pairs forming combo interfaces, see Table 8 . |  |
| Interface card slots |  |  | Two on the rear panel |  |
| Interface card models |  |  | LSPM2GP2P (JD367A) (not supporting IRF) LSPM1CX2P (JD360B) (supporting IRF) LSPM2SP2P (JD368B) (supporting IRF) LSPM1XP2P (JD359B) (supporting IRF) LSPM1XP1P (JD361B) (supporting IRF) |  |
| Power supply system |  |  | The switch models have two fixed power receptacles, one AC receptacle, and one RPS receptacle. The two power inputs can be simultaneously used, acting as backup for each other. Alternatively, you can use either power input. |  |
| Input voltage | AC |  | Rated voltage range: 100 VAC to 240 VAC, 50 Hz or 60 Hz Input voltage range: 90 VAC to 264 VAC, 47 Hz to 63 Hz |  |
|  | RPS |  | The rated voltage range is –55 VDC to –52 VDC. Use the external RPS power supply unit—A-RPS1600 (JG136A)—recommended by HPE only. |  |
| Maximum PoE power per port |  |  | 30 W | 30 W |
| Total PoE power |  |  | 370 W | AC power supply: 370 W RPS power supply: 740 W (The total PoE power of ports numbered 1 through 24 is 370 W, and that of ports numbered 25 through 48 is 370 W.) |
| Minimum power consumption |  |  | 62 W | 90 W |
| Maximum power consumption (including PoE power) |  | AC | 215 W + 370W | 281 W + 370W |
|  |  | RPS | 121 W + 370W | 181 W + 740W |
| Cooling system |  |  | 6 fans |  |
| Operating temperature |  |  | 0°C to 45°C (32°F to 113°F) |  |
| Relative humidity (noncondensing) |  |  | 10% to 90% |  |

        - SFP-Ethernet port pairs forming Combo interfaces

| Model | SFP port | 10/100/1000Base-T Ethernet port |
| --- | --- | --- |
| A5120-24G EI (2 slots) A5120-24G EI TAA (2 slots) A5120-24G-PoE+ EI (2 slots) A5120-24G-PoE+ EI TAA (2 slots) A5120-24G EI | GigabitEthernet 1/0/25 | GigabitEthernet 1/0/22 |
|  | GigabitEthernet 1/0/26 | GigabitEthernet 1/0/24 |
|  | GigabitEthernet 1/0/27 | GigabitEthernet 1/0/21 |
|  | GigabitEthernet 1/0/28 | GigabitEthernet 1/0/23 |
| A5120-48G EI (2 slots) A5120-48G EI TAA (2 slots) A5120-48G-PoE+ EI (2 slots) A5120-48G-PoE+ EI TAA (2 slots) A5120-48G EI | GigabitEthernet 1/0/49 | GigabitEthernet 1/0/46 |
|  | GigabitEthernet 1/0/50 | GigabitEthernet 1/0/48 |
|  | GigabitEthernet 1/0/51 | GigabitEthernet 1/0/45 |
|  | GigabitEthernet 1/0/52 | GigabitEthernet 1/0/47 |

Software features

        - Software features of the A5120 EI Switch series

| Feature |  | A5120-24G EI (2 slots) A5120-24G EI TAA (2 slots) |  | A5120-48G EI (2 slots) A5120-48G EI TAA (2 slots) | A5120-24G-PoE+ EI (2 slots) A5120-24G-PoE+ EI TAA (2 slot) | A5120-48G-PoE+ EI (2 slots) A5120-48G-PoE+ EI TAA (2 slots) | A5120-24G EI |  | A5120-48G EI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wire speed L2 switching | Switching capacity (Full duplex) | 128 Gbps | 176 Gbps |  | 128 Gbps | 176 Gbps | 48 Gbps |  | 96 Gbps |
|  | Packet forwarding rate | 95.2 Mpps | 130.9 Mpps |  | 95.2 Mpps | 130.9 Mpps | 35.7 Mpps |  | 71.4 Mpps |
| Power over Ethernet |  | Not supported |  |  | Supported |  | Not supported |  |  |
| Link aggregation |  | Aggregation of GE ports Aggregation of 10GE ports (A5120-24G EI and A5120-48G EI do not support) Dynamic link aggregation Static link aggregation Supports up to 128 aggregation groups, each supporting up to eight GE ports or eight 10GE ports Supports local-first load sharing for link aggregation Supports configuring load sharing criteria for IRF links (A5120-24G EI and A5120-48G EI do not support this feature). |  |  |  |  |  |  |  |
| IRF |  | Supported |  |  |  |  |  | Not supported |  |
| IRF MAD Detection |  | LACP MAD ARP MAD |  |  |  |  |  | Not supported |  |
| Flow control |  | IEEE 802.3x flow control and back pressure |  |  |  |  |  |  |  |
| Jumbo Frame |  | Supports maximum frame size of 9 KB |  |  |  |  |  |  |  |
| MAC address table |  | 16K MAC addresses 1024 static MAC addresses Blackhole MAC addresses MAC address learning limit on a port |  |  |  |  |  |  |  |
| VLAN |  | Port-based VLANs (4,094 VLANs) QinQ and selective QinQ Voice VLAN Protocol-based VLANs GVRP MAC-based VLANs IP subnet-based VLANs |  |  |  |  |  |  |  |
| ARP |  | 1K entries 512 static entries Gratuitous ARP Proxy ARP and local proxy ARP ARP Detection(based on static IP source guard binding Entries/DHCP snooping entries/802.1X security entries/OUI MAC addresses) ARP filtering |  |  |  |  |  |  |  |
| ND |  | 512 entries 256 static entries ND detection ND snooping |  |  |  |  |  |  |  |
| VLAN virtual interface |  | 32 |  |  |  |  |  |  |  |
| IPV4 DHCP |  | DHCP client DHCP snooping DHCP relay agent DHCP server |  |  |  |  |  |  |  |
| IPV6 DHCP |  | IPv6 DHCP snooping IPv6 DHCP client IPv6 DHCP relay agent IPv6 DHCP server |  |  |  |  |  |  |  |
| UDP Helper |  | UDP helper |  |  |  |  |  |  |  |
| DNS |  | Dynamic domain name resolution Dynamic domain name resolution client IPv4/IPv6 addresses |  |  |  |  |  |  |  |
| IPv4 route |  | 128 static routes |  |  |  |  |  |  |  |
| IPv6 route |  | 64 static routes |  |  |  |  |  |  |  |
| IPv4 multicast |  | IGMP snooping v1/v2/v3 Multicast VLAN Multicast VLAN+ 1000 multicast groups Unknown multicast packets dropping |  |  |  |  |  |  |  |
| IPv6 multicast |  | MLD snooping v1/v2 IPv6 multicast VLAN 1000 multicast groups Unknown multicast packets dropping |  |  |  |  |  |  |  |
| Broadcast/multicast/unicast storm control |  | Storm control based on port rate percentage PPS-based storm control Kbps-based storm control |  |  |  |  |  |  |  |
| MSTP |  | MSTP protocol 16 instances STP root guard BPDU guard |  |  |  |  |  |  |  |
| RRPP |  | RRPP protocol Multi-instance RRPP |  |  |  |  |  |  |  |
| Smart link |  | Up to 26 groups supported Multi-instance Smart Link |  |  |  |  |  |  |  |
| Monitor link |  | Supported |  |  |  |  |  |  |  |
| QoS/ACL |  | Restriction of the rates at which a port sends and receives packets, with a granularity of 64 kbps. Packet redirection Committed access rate (CAR), with a granularity of 64 kbps. Eight output queues for each port Flexible queue scheduling algorithms based on port and queue, including strict priority (SP), weighted round robin (WRR), WFQ (Weighted Fair Queuing) and SP + WRR. Remarking of 802.1p and DSCP priorities Packet filtering at Layer 2 through Layer 4; flow classification based on source MAC address, destination MAC address, source IP (IPv4/IPv6) address, destination IP (IPv4/IPv6) address, port, protocol, and VLAN. Time range Traffic shaping Packet filtering Dynamically modifying QoS |  |  |  |  |  |  |  |
| Mirroring |  | Traffic mirroring Port mirroring |  |  |  |  |  |  |  |
| Remote mirroring |  | Remote port mirroring |  |  |  |  |  |  |  |
| Security |  | Hierarchical management and password protection of users AAA authentication RADIUS authentication HWTACACS+ SSH 2.0 Port isolation Port security Local security authentication based on Layer-2 Portal and RADIUS Triple authentication MAC address authentication IP-MAC-port binding(IPV4 and IPV6) IP Source Guard HTTPS SSL PKI EAD User profile |  |  |  |  |  |  |  |
| 802.1X |  | Up to 1024 users Port-based and MAC address–based authentication Guest VLAN Dynamic assignment of QoS/ACL/VLAN based on 802.1X 802.1X re-authentication |  |  |  |  |  |  |  |
| Download and upgrade |  | XModem FTP TFTP |  |  |  |  |  |  |  |
| Management |  | Configuration through CLI Remote configuration through Telnet Configuration through Console port Simple network management protocol (SNMP) Remote monitoring (RMON) alarm, event and history recording IMC Web-based network management System log Hierarchical alarms HGMP v2 NTP Power supply alarm function Fan and temperature alarms IRF (A5120-24G EI and A5120-48G EI do not support IRF) |  |  |  |  |  |  |  |
| Maintenance |  | Debugging information output Ping and Tracert NQA Track Virtual cable test CFD (IEEE 802.1ag and ITU-T Y.1731) Ethernet OAM (IEEE 802.3ah)DLDP sFlow |  |  |  |  |  |  |  |
| Energy saving |  | Port auto-power-down Configuring scheduled tasks Regulating fan speed according to temperature |  |  |  |  |  |  |  |

Upgrading software

You can access the Boot menu or the CLI to upgrade software images (.bin system software images and .btm Boot ROM images).

Software upgrade methods

| Method | Section |
| --- | --- |
| Upgrading software from the Boot menu | XMODEM download through the console port |
|  | TFTP download through an Ethernet port |
|  | FTP download through an Ethernet port |
| Upgrading software from the CLI | FTP download from a server |
|  | TFTP download from a server |

When upgrading software, make sure the versions of the Boot ROM and system software images are compatible.

The procedures for upgrading Boot ROM and system software from the Boot menu are the same except that you must choose different options from the Boot menu (1 for upgrading system software, and 6 for upgrading Boot ROM) to start the upgrade procedure. This appendix describes only the Boot ROM upgrade procedure.

Upgrading software from the Boot menu

Accessing the Boot menu

Power on the switch, for example, an HP A5120-24G EI Switch with 2 Interface Slots.

The following information appears:

Starting......

********************************************************************************

*                                                                              *

*      HP A5120-24G EI Switch with 2 Interface Slots BOOTROM, Version 618      *

*                                                                              *

********************************************************************************

Copyright (c) 2010-2015 Hewlett-Packard Development Company, L.P.

Creation date   : Dec 27 2012, 19:50:15

CPU Clock Speed : 264MHz

BUS Clock Speed : 33MHz

Memory Size     : 128MB

Mac Address     : 000fe0000011

Press Ctrl-B to enter Boot Menu... 1

Press Ctrl + B within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the "Press Ctrl-B to enter Boot Menu..." prompt message appears. If you fail to do this, the system starts decompressing the system software.

By default, the system starts up in fast mode.

BootRom password: Not required. Please press Enter to continue.

Press Enter at the prompt for password.

The password recovery capability is enabled." or "Password recovery capability is disabled." message appears, followed by the Boot menu. Availability of some menu options depends on the state of password recovery capability (see Table 11 ). For more information about password recovery capability, see HP 5120EI Switch Series Fundamentals Configuration Guide .

Password recovery capability is enabled.

BOOT  MENU

1. Download application file to flash

2. Select application file to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter bootrom upgrade menu

7. Skip current configuration file

8. Reserved

9. Set switch startup mode

0. Reboot

Ctrl+F: Format File System

Ctrl+D: Enter Debugging Mode

Ctrl+T: Enter Board Test Environment

Enter your choice(0-9):

Boot menu options

| Option | Tasks |
| --- | --- |
| 1. Download application file to flash | Download a .bin system software image to the flash. |
| 2. Select application file to boot | Specify the main and backup system software images for the next startup. Specify the main and backup configuration files for the next startup. This task can be performed only if password recovery capability is enabled. |
| 3. Display all files in flash | Display files on the flash. |
| 4. Delete file from flash | Delete files to free storage space. |
| 5. Restore to factory default configuration | Delete the current next-startup configuration files and restore the factory-default configuration. This option is available only if password recovery capability is disabled. |
| 6. Enter BootRom upgrade menu | Access the Boot ROM upgrade menu. If password recovery capability is enabled, you can upgrade the Boot ROM to any version. If password recovery capability is disabled, you can upgrade the Boot ROM to only Version 618 or higher. |
| 7. Skip current system configuration | Start the switch without loading any configuration file. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| 8. Reserved | Reserved option field. |
| 9. Set switch startup mode | Set the startup mode to fast startup mode or full startup mode. |
| 0. Reboot | Reboot the switch. |
| Ctrl+F: Format File System | Format the current storage medium. |
| Ctrl+D: Enter Debugging Mode | Access the debugging menu. For options in the debugging menu, see Table 12 . This option is available only if password recovery capability is enabled. |
| Ctrl+T: Enter Board Test Environment | This option is not supported. |

Debugging menu options

| Option | Task |
| --- | --- |
| 1. Load elf file | Download an ELF file and start the switch with the file. |
| 2. display cpld version | Display CPLD information. |
| 3. Load app file to sdram | Load and run a system software image in the SDRAM. |
| 0. Return to boot menu | Return to the Boot menu. |

XMODEM download through the console port

You can connect a PC or terminal to the console port to download files to the switch by using XMODEM. XMODEM supports 128-byte data packets and provides the reliability mechanisms including checksum, CRC, and retransmissions (up to 10).

Setting terminal parameters

Run a terminal emulator program on the console terminal, for example, a PC.

The following are the required terminal settings:

Bits per second—9,600

Data bits—8

Parity—None

Stop bits—1

Flow control—None

Emulation—VT100

Follow these steps to set terminal parameters, for example, on a Windows XP HyperTerminal:

Select Start > All Programs > Accessories > Communications > HyperTerminal, and in the Connection Description dialog box that appears, type the name of the new connection in the Name text box and click OK.

Connection description of the HyperTerminal

Select the serial port to be used from the Connect using drop-down list, and click OK.

Set the serial port used by the HyperTerminal connection

Set Bits per second to 9600, Data bits to 8, Parity to None, Stop bits to 1, and Flow control to None, and click OK.

Set the serial port parameters

Select File > Properties in the HyperTerminal window.

HyperTerminal window

Click the Settings tab, set the emulation to VT100, and click OK in the Switch Properties dialog box.

Set terminal emulation in Switch Properties dialog box

Upgrading Boot ROM

Perform the following tasks to upgrade Boot ROM by using XMODEM through the console port:

Enter 6 or press Ctrl + U at the Boot menu to access the Boot ROM update menu:

Bootrom update menu:

1. Set TFTP protocol parameter

2. Set FTP protocol parameter

3. Set XMODEM protocol parameter

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2. 19200

3. 38400

4. 57600

5. 115200

0. Return

Enter your choice (0-5):

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

|  | NOTE: Typically the size of a .bin file is over 10 MB. Even at a baud rate of 115200 bps, the download takes tens of minutes. |
| --- | --- |

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnect the terminal from the switch

Select File > Properties. In the Properties dialog box, click Configure (see Figure 7 ), and then select 115200 from the Bits per second drop-down list box (see Figure 8 ).

Properties dialog box

Modify the baud rate

Select Call > Call to reestablish the connection.

Reestablish the connection

|  | NOTE: The new settings take effect after you reestablish the connection. |
| --- | --- |

Upload the software package file from the terminal to the switch.

After establishing a connection between the terminal and the switch, press Enter in the HyperTerminal window.

CCCCCCCCCCCCCCCCCC

|  | NOTE: To abort the downloading, press Ctrl + X. |
| --- | --- |

Select Transfer > Send File in the HyperTerminal window (see Figure 10 ), and click Browse in the pop-up dialog box (see Figure 11 ) to select the source file (for example, bootrom.btm), and select Xmodem from the Protocol drop-down list.

Transfer menu

File transmission dialog box

Click Send. The following dialog box appears:

Send the application file using XMODEM

When the download is completed, the terminal displays the following information:

CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCdone!

Bootrom updating..........done!

Your baudrate should be set to 9600 bps again!

Press enter key when ready

If you are using a download rate other than 9600 bps, restore the baud rate of the serial port on the terminal to 9600 bps. If the baud rate is 9600 bps, skip this step.

Press Enter to return to the Boot menu and enter 0 to restart the switch so the updated image can take effect.

BOOT  MENU

1. Download application file to flash

2. Select application file to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter bootrom upgrade menu

7. Skip current configuration file

8. Reserved

9. Set switch startup mode

0. Reboot

Ctrl+F: Format File System

Ctrl+D: Enter Debugging Mode

Ctrl+T: Enter Board Test Environment

Enter your choice(0-9):

Upgrading system software

To upgrade system software, enter 1 at the Boot menu.

The following menu appears:

1. Set TFTP protocol parameter

2. Set FTP protocol parameter

3. Set XMODEM protocol parameter

0. Return to boot menu

Enter your choice(0-3):3

Enter 3 to set the XMODEM parameters for downloading the system software image.

The subsequent procedure is the same as loading Boot ROM images, except that you must set the attribute of the file as main, backup, or none to complete the file loading.

Please input a new file name           :update.bin

Free flash Space: 15598592 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.............................done!

Please input the file attribute (main/backup/none):main

done!

|  | NOTE: The switch always attempts to boot first with the main file. If the attempt fails, for example, because the main file is not available, the switch tries to boot with the backup file. A file with the none attribute is for backup only and cannot be used for startup. If a file with the same attribute as the file you are loading is already in the flash, the attribute of the old file changes to none after the new file becomes valid. |
| --- | --- |

TFTP download through an Ethernet port

The switch can work as a TFTP client to download files from a TFTP server.

Upgrading Boot ROM

Connect an Ethernet port (for example, GigabitEthernet 1/0/25) of the switch to the server and connect the console port of the switch to a PC (see Figure 13 ).

Load software using TFTP/FTP through Ethernet port

|  | NOTE: The PC and the TFTP/FTP server can be co-located. The HP A5120 EI switches do not come with any TFTP server program, and you must install one yourself. |
| --- | --- |

Run the TFTP server program on the server and specify the source file path.

Run a terminal emulator program on the PC, power on the switch, access the Boot menu, and enter 6 to access the Boot ROM update menu:

Bootrom update menu:

1. Set TFTP protocol parameter

2. Set FTP protocol parameter

3. Set XMODEM protocol parameter

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name     :update.btm

Switch IP address        :10.10.10.3

Server IP address        :10.10.10.2

Description of the TFTP parameters

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to be downloaded (for example, update.btm) |
| Switch IP address | IP address of the switch (for example, 10.10.10.3) |
| Server IP address | IP address of the TFTP server (for example, 10.10.10.2) |

|  | NOTE: The switch must be on the same subnet as the server. |
| --- | --- |

Enter all required parameters.

Are you sure you want to download file to flash? Yes or No(Y/N)

Enter Y at the prompt to upgrade Boot ROM.

Loading........................................done

Bootrom updating..........done!

BOOT  MENU

1. Download application file to flash

2. Select application file to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter bootrom upgrade menu

7. Skip current configuration file

8. Reserved

9. Set switch startup mode

0. Reboot

Ctrl+F: Format File System

Ctrl+D: Enter Debugging Mode

Ctrl+T: Enter Board Test Environment

Enter your choice(0-9):

Enter 0 to restart the switch from the Boot menu so the upgraded Boot ROM can take effect.

Upgrading system software

To upgrade system software, enter 1 at the Boot menu to access the following menu:

1. Set TFTP protocol parameter

2. Set FTP protocol parameter

3. Set XMODEM protocol parameter

0. Return to boot menu

Enter your choice(0-3):3

Enter 1 to set the TFTP parameters.

The subsequent procedure of is the same as upgrading Boot ROM, except that you must set the attribute of the file as main, backup, or none to complete the file loading.

Writing flash..................................................................

................Done!

Please input the file attribute (Main/Backup/None) Main

Done!

|  | NOTE: The switch always attempts to boot first with the main file. If the attempt fails, for example, because the main file is not available, the switch tries to boot with the backup file. A file with the none attribute is for backup only and cannot be used for startup. If a file with the same attribute as the file you are loading is already in the flash, the attribute of the old file changes to none after the new file becomes valid. |
| --- | --- |

FTP download through an Ethernet port

The switch can work as an FTP server or FTP client to download files through an Ethernet port. This section uses the switch as an FTP client to describe the procedure.

Upgrading Boot ROM

|  | NOTE: When upgrading Boot ROM, the switch can work only as an FTP client. |
| --- | --- |

Connect an Ethernet port (GigabitEthernet 1/0/25, for example) of the switch to the server and connect the console port of the switch to a PC (see Figure 13 ).

Run an FTP server program on the server, configure an FTP username and password, and specify the source file path.

Run a terminal emulator program on the PC, power on the switch, access the Boot menu, and enter 6 to access the Boot ROM update menu:

Bootrom update menu:

1. Set TFTP protocol parameter

2. Set FTP protocol parameter

3. Set XMODEM protocol parameter

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File name     :update.btm

Switch IP address   :10.10.10.3

Server IP address  :10.10.10.2

FTP User Name      :user

FTP User Password  :123

Description of the FTP parameters

| Item | Description |
| --- | --- |
| Load File name | Name of the file to be downloaded (for example, update.btm) |
| Switch IP address | IP address of the switch (for example, 10.10.10.3) |
| Server IP address | IP address of the FTP server (for example, 10.10.10.2) |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: The switch must be on the same subnet as the server. |
| --- | --- |

Enter all required parameters.

Are you sure you want to download file to flash? Yes or No(Y/N)

Enter Y at the prompt to upgrade Boot ROM.

Loading........................................done

Bootrom updating..........done!

BOOT  MENU

1. Download application file to flash

2. Select application file to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter bootrom upgrade menu

7. Skip current configuration file

8. Reserved

9. Set switch startup mode

0. Reboot

Ctrl+F: Format File System

Ctrl+D: Enter Debugging Mode

Ctrl+T: Enter Board Test Environment

Enter your choice(0-9):

Enter 0 to restart the switch from the Boot menu so the upgraded Boot ROM can take effect.

Upgrading system software

To upgrade system software, enter 1 at the Boot menu to access the following menu:

1. Set TFTP protocol parameter

2. Set FTP protocol parameter

3. Set XMODEM protocol parameter

0. Return to boot menu

Enter your choice(0-3):3

Enter 2 to set the FTP parameters.

The subsequent procedure is the same as upgrading Boot ROM, except that you must set the attribute of the file as main, backup, or none to complete the file loading.

Writing flash..................................................................

................Done!

Please input the file attribute (Main/Backup/None) M

Done!

|  | NOTE: The switch always attempts to boot first with the main file. If the attempt fails, for example, because the main file is not available, the switch tries to boot with the backup file. A file with the none attribute is for backup only and cannot be used for startup. If a file with the same attribute as the file you are loading is already in the flash, the attribute of the old file changes to none after the new file becomes valid. |
| --- | --- |

Upgrading software from the CLI

You can remotely download Boot ROM and system software images from the CLI.

FTP download from a server

This section uses the topology in Figure 14  as an example. Run FTP server on the management PC at 202.10.10.53, create an FTP username admin and password, specify the source file path, telnet to the switch, and get the system software image file update.bin and the Boot ROM image file update.btm from the server.

FTP download from a server

Get the files to the switch by using FTP.

<HP> ftp 202.10.10.53

Trying ...

Press CTRL+K to abort

Connected.

220 WFTPD 2.0 service (by Texas Imperial Software) ready for new user

User(none):user

331 Give me your password, please

Password:

230 Logged in successfully

[ftp] get update.bin

[ftp] get update.btm

[ftp] bye

Upgrade Boot ROM.

<HP> bootrom update file update.btm slot 1

This command will update bootrom file on the specified board(s), Continue? [Y/N]y

Now updating bootrom, please wait...

Succeeded to update bootrom of Board 1

Load the system software image and specify the file as the main file at the next reboot.

<HP> boot-loader file update.bin slot 1 main

Slot 1

This command will set the boot file. Continue? [Y/N]: y

The specified file will be used as the main boot file at the next reboot on slot 1!

<HP> display boot-loader

The current boot app is:  flash:/update.bin

The main boot app is:     flash:/update.bin

The backup boot app is:   flash:/update.bin

Reboot the switch with the reboot command to complete the upgrade.

|  | CAUTION: If you have made any configuration, save the configuration before the reboot to avoid data loss. Avoid power failure during the loading process. |
| --- | --- |

|  | TIP: If flash memory is insufficient, load the Boot ROM image first and delete unused files to free up flash memory before you load the system software image. |
| --- | --- |

TFTP download from a server

The switch can work as a TFTP client to download files from a TFTP server, and the downloading procedure is similar to downloading files through FTP. With these two protocols, the subsequent Boot ROM and system software image loading procedures are the same.