HPE 5120v3-CMW710-R6367 Release Notes

Software Feature Changes

The information in this document is subject to change without notice. © Copyright 2024 Hewlett Packard Enterprise Development LP

Contents

R6367	3

New Feature: Enabling port security unified secure MAC address control for access users	3

Enabling port security unified secure MAC address control for access users	3

Command reference	4

New command: port-security user-mac control enable	4

Modified command: display port-security mac-address security	5

New feature: Enabling port selection preemption on an aggregate interface	6

Enabling port selection preemption on an aggregate interface	6

Command reference	7

lacp preempt delay	7

lacp preempt enable	8

New feature: Configuring an interface as an uplink interface to disable it from learning ARP snooping entries	8

Configuring an interface as an uplink interface to disable it from learning ARP snooping entries	8

Command reference	9

arp snooping uplink	9

New feature: Configuring spanning tree blackhole detection	10

Configuring spanning tree blackhole detection	10

Command changes	12

display stp blackhole-detection blocked-port	12

stp blackhole-detection enable	13

stp global blackhole-detection enable	15

stp global blackhole-detection rx-bpdu timeout	16

stp global timer blackhole-detection-interval	17

stp global timer rx-blackhole-timeout	18

stp timer blackhole-detection-interval	19

stp timer rx-blackhole-timeout	20

New feature: LLDP back hole detection	22

Configuring LLDP black hole detection	22

Command reference	24

lldp blackhole-detection enable	24

lldp global blackhole-detection enable	26

lldp global blackhole-detection rx-lldpdu timeout	28

lldp global timer blackhole-detection-interval	29

lldp global timer rx-blackhole-timeout	30

lldp timer blackhole-detection-interval	31

lldp timer rx-blackhole-timeout	32

New feature: LLDP cross-domain detection	34

Configuring LLDP cross-domain detection	34

Command reference	36

lldp cross-domain-detection	36

lldp cross-domain-detection domain-id	37

lldp global cross-domain-detection enable	39

New feature: Using the subscriber ID as the client ID in all received DHCP requests	41

Using the subscriber ID as the client ID in all received DHCP requests	41

Command reference	42

dhcp server subscriber-id replace client-id	42

dhcp server subscriber-id replace client-id global	43

dhcp server subscriber-id interface-name	44

New feature: Configuring resource monitoring	45

Configuring resource monitoring	45

Command reference	46

display resource-monitor	46

resource-monitor minor resend enable	47

resource-monitor output	48

resource-monitor resource	48

New feature: Sending EAP-Success packets upon successful authorization in 802.1X	50

Sending EAP-Success packets upon successful authorization	50

Command reference	51

dot1x eap-success post-authorization	51

Modified feature: Configuring the padding mode and padding format for the Circuit ID sub-option	52

Feature change description	52

Command changes	52

Modified command: dhcp relay information circuit-id	52

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option	52

Feature change description	52

Command changes	53

Modified command: dhcp relay information remote-id	53

Modified feature: Support for specifying a custom hexadecimal string as the content of the Remote ID sub-option	53

Feature change description	53

Command changes	53

Modified command: dhcp snooping information remote-id	53

Modified command: dhcp relay information remote-id	54

Modified feature: Support for specifying a custom ASCII string as the client ID of a static binding	55

Feature change description	55

Modified command: static-bind	55

Release 6352P02	56

R6367

This release has the following changes:

New Feature: Enabling port security unified secure MAC address control for access users

New feature: Enabling port selection preemption on an aggregate interface

New feature: Configuring an interface as an uplink interface to disable it from learning ARP snooping entries

New feature: Configuring spanning tree blackhole detection

New feature: LLDP back hole detection

New feature: LLDP cross-domain detection

New feature: Using the subscriber ID as the client ID in all received DHCP requests

New feature: Configuring resource monitoring

New feature: Sending EAP-Success packets upon successful authorization in 802.1X

Modified feature: Configuring the padding mode and padding format for the Circuit ID sub-option

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option

Modified feature: Support for specifying a custom hexadecimal string as the content of the Remote ID sub-option

Modified feature: Support for specifying a custom ASCII string as the client ID of a static binding

  - New Feature: Enabling port security unified secure MAC address control for access users

  - Enabling port security unified secure MAC address control for access users

        - About this feature

By default, only the MAC addresses manually configured or automatically learned in the port security autoLearn mode are considered secure MAC addresses and are subject to related security features. To enhance network access security, the device now supports unified secure MAC management for various access users on a port.

With this feature enabled on a port, the MAC addresses of 802.1X authentication users, MAC authentication users, Web authentication users, and voice VLAN users who have successfully authenticated on the port will be added to the secure MAC table and controlled by related secure MAC functions. For example, the maximum number of secure MAC addresses on the port can be deleted using the undo port-security mac-address security sticky command, and support for intrusion detection on the port.

For successfully authenticated 802.1X, MAC authentication, and Web authentication users and voice VLAN users, their secure MAC addresses are sticky MAC addresses. You can convert them to dynamic secure MAC addresses by using the port-security mac-address dynamic command.

In contrast to sticky MAC entries generated in autoLearn mode, the sticky MAC entries for authenticated users and voice VLAN users do not age while they are online. These entries are only affected by timing and traffic aging mechanisms after the users go offline.

        - Restrictions and guidelines

Before configuring this feature, you must first set the maximum number of secure MAC addresses allowed on a port by executing the port-security max-mac-count command in interface view. After this feature is configured, you cannot change the limit on the number of secure MAC addresses on a port.

This feature and the autoLearn mode are mutually exclusive.

When the dot1x port-method portbased command or mac-authentication host-mode multi-vlan command is configured on a port, only the MAC address of the first authenticated user is added as a secure MAC address entry.

        - Procedure

Enter system view.

system-view

Enter interface view.

interface interface-type interface-number

Enable unified secure MAC address control for port security access users.

port-security user-mac control enable

By default, unified secure MAC address control for access users is not enabled.

  - Command reference

    - New command: port-security user-mac control enable

Use port-security user-mac control enable to enable the port security unified secure MAC address control for access users.

Use undo port-security user-mac control enable command to disable unified secure MAC control for access users.

Syntax

port-security user-mac control enable

undo port-security user-mac control enable

Default

Unified secure MAC control for access users is disabled.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Usage guidelines

Application scenarios

By default, only the MAC addresses manually configured or automatically learned in the port security autoLearn mode are considered secure MAC addresses and are subject to related security features.

To enhance network access security, the device now supports unified secure MAC management for various access users on a port.

About this feature

With this feature enabled on a port, the MAC addresses of 802.1X authentication users, MAC authentication users, Web authentication users, and voice VLAN users who have successfully authenticated on the port will be added to the secure MAC table and controlled by related secure MAC functions. For example, the maximum number of secure MAC addresses on the port can be deleted using the undo port-security mac-address security sticky command, and support for intrusion detection on the port.

For successfully authenticated 802.1X, MAC authentication, Web authentication, and voice VLAN users, their secure MAC addresses are sticky MAC addresses. You can convert them to dynamic secure MACs by using the port-security mac-address dynamic command.

In contrast to sticky MAC entries generated in autoLearn mode, the sticky MAC entries for authenticated users and voice VLAN users do not age while they are online. These entries are only affected by timing and traffic aging mechanisms after the users go offline.

Prerequisites

Before configuring this feature, you must first set the maximum number of secure MAC addresses allowed on a port by executing the port-security max-mac-count command in interface view. After this feature is configured, you cannot change the limit on the number of secure MAC addresses on a port.

Restrictions and guidelines

This feature and the autoLearn mode are mutually exclusive.

When the dot1x port-method portbased command or mac-authentication host-mode multi-vlan command is configured on a port, only the MAC address of the first authenticated user is added as a secure MAC address.

Examples

# Enable unified secure MAC address control for access users.

<Sysname> system-view

[Sysname] interface GigabitEthernet 1/0/1

[Sysname-GigabitEthernet 1/0/1] port-security max-mac-count 10

[Sysname-GigabitEthernet 1/0/1] port-security user-mac control enable

Related commands

display port-security mac-address security

    - Modified command: display port-security mac-address security

Syntax

display port-security mac-address security [ interface interface-type interface-number ] [ vlan vlan-id ] [ count ]

Views

Any view

Change description

The Type field was added to the command output to display the type (or origin) of the secure MAC address.

Example:

# Display information about all secure MAC addresses.

<Sysname> display port-security mac-address security

MAC addr         VLAN ID  State          Port index         Type            Aging time

0002-0002-0002  1         Secure         GE1/0/1             MAC-Auth       Not aged

--- Number of secure MAC addresses: 1 ---

# Display the number of secure MAC addresses.

<Sysname> display port-security mac-address security count

--- Number of secure MAC addresses: 1 ---

            - Command output

| Field | Description |
| --- | --- |
| MAC addr | Secure MAC address. |
| VLAN ID | VLAN to which the port belongs. |
| State | MAC address type. Secure—Indicates that this entry is a secure MAC address entry. |
| Port index | Port where the security MAC address entry resides. |
| Type | Type (or origin) of the secure MAC address. Autolearn—Automatically learned. Manual—Manually configured. MAC-Auth—MAC authentication user. 802.1X—802.1X authentication user. Web-Auth—Web authentication user. Voice VLAN—Voice VLAN user. |
| Aging time | Remaining lifetime of the secure MAC address. For static MAC addresses, this field displays Not aged. For sticky MAC addresses, this field displays the specific remaining lifetime. If the lifetime is less than 60 seconds, it is displayed in seconds. If the lifetime is 60 seconds or longer, it is displayed in minutes. Under the default setting, aging is not performed for sticky MAC addresses, and this field displays Not aged. |
| Number of secure MAC addresses | Current number of saved secure MAC addresses. |

New feature: Enabling port selection preemption on an aggregate interface

Enabling port selection preemption on an aggregate interface

About this task

When port A fails in a dynamic aggregation group, the device selects the highest priority member port, port B for example, as the new selected port. If port selection preemption is enabled and port A recovers from failure, it replaces port B. If port selection preemption is disabled, port B is still selected even though port A has higher priority, and traffic loss caused by selected port preemption is reduced.

If port selection preemption is enabled and port A recovers from failure, it immediately replaces port B. This might cause packet loss if port A's link status is unstable. To avoid this issue, configure a port selection preemption delay.

Procedure

Enter system view.

system-view

Enter aggregate interface view.

Enter Layer 2 aggregate interface view.

interface bridge-aggregation interface-number

Enter Layer 3 aggregate interface view.

interface route-aggregation interface-number

Enable port selection preemption on an aggregate interface.

lacp preempt enable

By default, port selection preemption is enabled on an aggregate interface.

Set the port selection preemption delay on an aggregate interface.

lacp preempt delay delay-time

By default, the port selection preemption delay is 0 seconds on an aggregate interface, which means port selection preemption is performed without delay.

Command reference

lacp preempt delay

Use lacp preempt delay to set the port selection preemption delay on an aggregate interface.

Use undo lacp preempt delay to restore the default.

Syntax

lacp preempt delay delay-time

undo lacp preempt delay

Default

On an aggregate interface, the port selection preemption delay is 0 seconds, which means port selection preemption is performed without delay.

Views

Layer 2 aggregate interface view

Layer 3 aggregate interface view

Predefined user roles

network-admin

Parameters

delay-time: Sets the port selection preemption delay in seconds. The value range for this argument is 10 to 180.

Usage guidelines

When port A fails in a dynamic aggregation group, the device selects the highest priority member port, port B for example, as the new selected port. If port selection preemption is enabled and port A recovers from failure, it immediately replaces port B. This might cause packet loss if port A's link status is unstable. To avoid this issue, configure a port selection preemption delay.

Examples

# Set the port selection preemption delay to 100 seconds on an aggregate interface.

<Sysname> system-view

[Sysname] interface bridge-aggregation 1

[Sysname-Bridge-Aggregation1] lacp preempt delay 100

lacp preempt enable

Use lacp preempt enable to enable port selection preemption.

Use undo lacp preempt enable to disable port selection preemption.

Syntax

lacp preempt enable

undo lacp preempt enable

Default

Port selection preemption is enabled on an aggregate interface.

Views

Layer 2 aggregate interface view

Layer 3 aggregate interface view

Predefined user roles

network-admin

Usage guidelines

When port A fails in a dynamic aggregation group, the device selects the highest priority member port, port B for example, as the new selected port. If port selection preemption is enabled and port A recovers from failure, it replaces port B. If port selection preemption is disabled, port B is still selected even though port A has higher priority, and traffic loss caused by selected port preemption is reduced.

Examples

# Disable port selection preemption on an aggregate interface.

<Sysname> system-view

[Sysname] interface bridge-aggregation 1

[Sysname-Bridge-Aggregation1] undo lacp preempt enable

New feature: Configuring an interface as an uplink interface to disable it from learning ARP snooping entries

Configuring an interface as an uplink interface to disable it from learning ARP snooping entries

About this task

After you enable ARP snooping on an access device by using the arp snooping enable command, the access device will generate ARP snooping entries by listening to ARP packets. In a network where the aggregate device acts as the gateway, if you enable local proxy ARP on the gateway by using the local-proxy-arp enable command, the uplink interface of the access device will also learn ARP snooping entries. As a result, the incoming interface of an ARP snooping entry flaps between the uplink and downlink interfaces. To avoid such an issue, you can configure this feature on the access device.

After you configure this feature on an access device enabled with ARP snooping, the interface no longer learns ARP snooping entries from incoming ARP packets.

Procedure

Enter system view.

system-view

Enter interface view.

interface interface-type interface-number

Supported interface types include Layer 2 Ethernet interface and Layer 2 aggregate interface.

Configure the interface as an uplink interface to disable it from learning ARP snooping entries.

arp snooping uplink

By default, an interface is not an uplink interface for ARP snooping. After you enable ARP snooping, the interface learns ARP snooping entries.

Command reference

arp snooping uplink

Use arp snooping uplink to configure an interface as an uplink interface to disable it from learning ARP snooping entries.

Use undo arp snooping uplink to restore the default.

Syntax

arp snooping uplink

undo arp snooping uplink

Default

An interface is not an uplink interface for ARP snooping. After you enable ARP snooping, the interface learns ARP snooping entries.

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

Usage guidelines

After you enable ARP snooping on an access device by using the arp snooping enable command, the access device will generate ARP snooping entries by listening to ARP packets. In a network where the aggregate device acts as the gateway, if you enable local proxy ARP on the gateway by using the local-proxy-arp enable command, the uplink interface of the access device will also learn ARP snooping entries. As a result, the incoming interface of an ARP snooping entry flaps between the uplink and downlink interfaces. To avoid such an issue, you can configure this feature on the access device.

After you configure this feature on an access device enabled with ARP snooping, the interface no longer learns ARP snooping entries from incoming ARP packets.

Examples

# Configure Layer 2 Ethernet interface GigabitEthernet1/0/1 as an uplink interface to disable it from learning ARP snooping entries.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] arp snooping uplink

# Configure Layer 2 aggregate interface Bridge-Aggregation 1 as an uplink interface to disable it from learning ARP snooping entries.

<Sysname> system-view

[Sysname] interface bridge-aggregation 1

[Sysname-Bridge-Aggregation1] arp snooping uplink

New feature: Configuring spanning tree blackhole detection

Configuring spanning tree blackhole detection

About this task

As shown in the following figure, Device A and Device B are connected via a HUB, which creates a loop in the network. Since BPDUs are point-to-point frames, once they are transmitted from a device and received by the next node, their transmission is terminated. The HUB acts as a blackhole for BPDUs. BPDUs cannot be transmitted between Device A and Device B through the HUB, and Device A and Device B cannot eliminate the loop through correct spanning tree topology calculation. Therefore, a method to detect BPDU blackholes is crucial for devices to block links to blackholes when detecting them, effectively eliminating potential loop risks.

          - Network diagram

Spanning tree blackhole detection feature can detect the presence of BPDU blackholes in the port links. Its working mechanism is as follows.

            - Blackhole verification phase.

A port enabled with spanning tree blackhole detection will start the BPDU reception timer of the blackhole detection feature. If the port receives a BPDU before the BPDU reception timer expires, the port resets the BPDU reception timer. If the BPDU reception timer expires, BPDU blackholes might exist on the network. As a result, the device fails to receive BPDUs for a long time, and the port starts to send spanning tree blackhole detection packets continuously.

Spanning tree blackhole detection packet is a type of Layer 2 packets defined by HPE. Different from the BPDU, the spanning tree blackhole detection packet can be transparently transmitted by the HUB, and will be sent continuously at a fixed time interval after being triggered to send.

Use the stp global blackhole-detection rx-bpdu timeout command to set the BPDU reception timer. Use the stp timer blackhole-detection-interval or stp global timer blackhole-detection-interval command to set the interval for sending spanning tree blackhole detection packets.

            - Blackhole confirmation phase.

If the port is triggered to send spanning tree blackhole detection packets and receives a BPDU, it proves that no BPDU blackhole exist between the devices. The port stops transmitting the spanning tree blackhole detection packets and resets the BPDU reception timer.

If a port with the spanning tree blackhole detection feature enabled receives a blackhole detection frame while its BPDU reception timer is in a timeout state, it indicates that the link between the local and remote devices is connected. Both devices are capable of sending BPDUs, but neither device receives BPDUs. The local device determines that a BPDU blackhole exists between the two ends of the link.

            - Blackhole handling phase.

After the device discovers a BPDU blackhole, it blocks the port that receives the spanning tree blackhole detection packet. Loops in the network are eliminated by preventing ports from sending and receiving any user data packets. You can use the stp timer rx-blackhole-timeout or stp global timer rx-blackhole-timeout command to specify the timeout period of the blocked port for spanning tree blackhole detection. If the port does not receive the spanning tree blackhole detection packet again within the timeout period, it means that the BPDU blackhole is eliminated, and the port returns to the normal forwarding state. Otherwise, the port continues to retain the blocking state.

The stp timer blackhole-detection-interval and stp global timer blackhole-detection-interval commands affect the frequency at which the specified port sends spanning tree blackhole detection packets. Set an appropriate packet sending interval as needed.

- When the sending interval is short, the port sends spanning tree blackhole packets more frequently. The advantage is that the network is more sensitive in detection and response to BPDU blackholes. The disadvantage is that it takes up more device CPU resources. Set a short sending interval in scenarios with strong device performance or a strong need to prevent loops.

- When the sending interval is long, the port sends spanning tree blackhole packets slowly. The advantage is that it occupies less CPU resources of the device. The disadvantage is that the network detects and responds more slowly to BPDU blackholes. Set a long sending interval in the scenario where the device performance is weak or tolerant of loops.

Restrictions and guidelines

The spanning tree blackhole detection feature takes effect when the following conditions are met:

- On the devices at both ends of a link, the spanning tree blackhole detection is enabled both globally and on ports.

- The link status of the port on which the spanning tree blackhole detection is enabled is up, and the spanning tree feature is enabled.

If the spanning tree blackhole detection is disabled on the port, or the BPDU reception timer on the port has not expired, the port does not perform any processing after receiving the spanning tree blackhole detection packet.

On a device with a long interval for sending BPDUs, set a longer value for the BPDU reception timer to prevent the device port from being blocked when no BPDU blackhole exists on the network. On the device with a short interval for sending BPDUs, set a shorter value for the BPDU reception timer to improve the network's detection speed of BPDU blackholes.

You can use the stp timer blackhole-detection-interval and stp global timer blackhole-detection-interval commands to modify the interval for sending spanning tree blackhole detection packets on a port. When both commands are configured at the same time, the configuration of the stp timer blackhole-detection-interval command takes effect. If no command is configured on the port, the port inherits the configuration of the stp global timer blackhole-detection-interval command.

You can use the stp timer rx-blackhole-timeout and stp global timer rx-blackhole-timeout commands to modify the timeout timer for receiving spanning tree blackhole detection packets on a port. When both commands are configured at the same time, the configuration of the stp timer rx-blackhole-timeout command takes effect. If no command is configured on the port, the port inherits the configuration of the stp global timer rx-blackhole-timeout command.

Procedure

            - Enter system view.

system-view

            - Enable spanning tree blackhole detection globally.

stp global blackhole-detection enable

By default, spanning tree blackhole detection is disabled globally.

            - (Optional.) Set the BPDU reception timer for the blackhole detection feature.

stp global blackhole-detection rx-bpdu timeout timeout

By default, the BPDU reception timer of the blackhole detection feature is 18 seconds.

            - (Optional.) Set the interval for sending spanning tree blackhole detection packets globally.

stp global timer blackhole-detection-interval interval

By default, the interval for sending spanning tree blackhole detection packets is 2 seconds.

            - (Optional.) Set the detection packet reception timer for spanning tree blackhole detection globally.

stp global timer rx-blackhole-timeout timeout

By default, the formula for calculating the timeout period (seconds) for receiving spanning tree blackhole packets is: Multiply the sending interval of spanning tree blackhole detection packets on a port by 3 and add 10.

            - Enter Layer 2 Ethernet interface view.

interface interface-type interface-number

            - (Optional.) Enable spanning tree blackhole detection on the port.

stp blackhole-detection enable

By default, spanning tree blackhole detection on the port is enabled.

            - (Optional.) Set the interval for sending spanning tree blackhole detection packets on the port.

stp timer blackhole-detection-interval interval

By default, the interval for sending spanning tree blackhole detection packets is 2 seconds.

            - (Optional.) Set the detection packet reception timer for spanning tree blackhole detection on the port.

stp timer rx-blackhole-timeout timeout

By default, the formula for calculating the timeout period (seconds) for receiving spanning tree blackhole packets on the port is: Multiply the sending interval of spanning tree blackhole detection packets on a port by 3 and add 10.

Command changes

display stp blackhole-detection blocked-port

Use display stp blackhole-detection blocked-port to display information about ports blocked by the spanning tree blackhole detection feature.

Syntax

display stp blackhole-detection blocked-port

Views

Any view

Predefined user roles

network-admin

network-operator

Examples

# Display the information about ports blocked by the spanning tree blackhole detection feature.

<Sysname> display stp blackhole-detection blocked-port

Blocked Port: Bridge-Aggregation1

Ten-GigabitEthernet3/0/1

Command output

| Field | Description |
| --- | --- |
| Blocked Port | Names of the port blocked by spanning tree blackhole detection. |

stp blackhole-detection enable

Use stp blackhole-detection enable to enable spanning tree blackhole detection on a port.

Use undo stp blackhole-detection enable to disable spanning tree blackhole detection on a port.

Syntax

stp blackhole-detection enable

undo stp blackhole-detection enable

Default

The spanning tree blackhole detection on a port is enabled.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Usage guidelines

Prerequisite

The spanning tree blackhole detection feature takes effect when the following conditions are met:

- On the devices at both ends of a link, the spanning tree blackhole detection is enabled both globally and on ports.

- The link status of the port on which the spanning tree blackhole detection is enabled is up, and the spanning tree feature is enabled.

Operating mechanism

            - Blackhole verification phase.

A port enabled with spanning tree blackhole detection will start the BPDU reception timer of the blackhole detection feature. If the port receives a BPDU before the BPDU reception timer expires, the port resets the BPDU reception timer. If the BPDU reception timer expires, BPDU blackholes might exist on the network. As a result, the device fails to receive BPDUs for a long time, and the port starts to send spanning tree blackhole detection packets continuously.

Spanning tree blackhole detection packet is a type of Layer 2 packets defined by HPE. Different from the BPDU, the spanning tree blackhole detection packet can be transparently transmitted by the HUB, and will be sent continuously at a fixed time interval after being triggered to send.

Use the stp global blackhole-detection rx-bpdu timeout command to set the BPDU reception timer. Use the stp timer blackhole-detection-interval or stp global timer blackhole-detection-interval command to set the interval for sending spanning tree blackhole detection packets.

            - Blackhole confirmation phase.

If the port is triggered to send spanning tree blackhole detection packets and receives a BPDU, it proves that no BPDU blackhole exist between the devices. The port stops transmitting the spanning tree blackhole detection packets and resets the BPDU reception timer.

If a port with the spanning tree blackhole detection feature enabled receives a blackhole detection frame while its BPDU reception timer is in a timeout state, it indicates that the link between the local and remote devices is connected. Both devices are capable of sending BPDUs, but neither device receives BPDUs. The local device determines that a BPDU blackhole exists between the two ends of the link.

            - Blackhole handling phase.

After the device discovers a BPDU blackhole, it blocks the port that receives the spanning tree blackhole detection packet. Loops in the network are eliminated by preventing ports from sending and receiving any user data packets. You can use the stp timer rx-blackhole-timeout or stp global timer rx-blackhole-timeout command to specify the timeout period of the blocked port for spanning tree blackhole detection. If the port does not receive the spanning tree blackhole detection packet again within the timeout period, it means that the BPDU blackhole is eliminated, and the port returns to the normal forwarding state. Otherwise, the port continues to retain the blocking state.

Restrictions and guidelines

If the spanning tree blackhole detection is disabled on the port, or the BPDU reception timer on the port has not expired, the port does not perform any processing after receiving the spanning tree blackhole detection packet.

Examples

# Enable spanning tree blackhole detection on Ten-GigabitEthernet3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] stp blackhole-detection enable

Related commands

stp timer rx-blackhole-timeout

stp global blackhole-detection enable

stp global blackhole-detection rx-bpdu timeout

stp global timer rx-blackhole-timeout

stp global timer blackhole-detection-interval

stp timer blackhole-detection-interval

stp global blackhole-detection enable

Use stp global blackhole-detection enable to enable spanning tree blackhole detection globally.

Use undo stp global blackhole-detection enable to disable spanning tree blackhole detection globally.

Syntax

stp global blackhole-detection enable

undo stp global blackhole-detection enable

Default

The spanning tree blackhole detection is disabled globally.

Views

System view

Predefined user roles

network-admin

Usage guidelines

Application scenarios

          - Network diagram

As shown in Figure 2, Device A and Device B are connected via a HUB, which creates a loop in the network. Since BPDUs are point-to-point frames, once they are transmitted from a device and received by the next node, their transmission is terminated. The HUB acts as a blackhole for BPDUs. BPDUs cannot be transmitted between Device A and Device B through the HUB, and Device A and Device B cannot eliminate the loop through correct spanning tree topology calculation. Therefore, a method to detect BPDU blackholes is crucial for devices to block links to blackholes when detecting them, effectively eliminating potential loop risks.

Prerequisite

The spanning tree blackhole detection feature takes effect when the following conditions are met:

- On the devices at both ends of a link, the spanning tree blackhole detection is enabled both globally and on ports.

- The link status of the port on which the spanning tree blackhole detection is enabled is up, and the spanning tree feature is enabled.

Operating mechanism

            - Blackhole verification phase.

A port enabled with spanning tree blackhole detection will start the BPDU reception timer of the blackhole detection feature. If the port receives a BPDU before the BPDU reception timer expires, the port resets the BPDU reception timer. If the BPDU reception timer expires, BPDU blackholes might exist on the network. As a result, the device fails to receive BPDUs for a long time, and the port starts to send spanning tree blackhole detection packets continuously.

Spanning tree blackhole detection packet is a type of Layer 2 packets defined by HPE. Different from the BPDU, the spanning tree blackhole detection packet can be transparently transmitted by the HUB, and will be sent continuously at a fixed time interval after being triggered to send.

Use the stp global blackhole-detection rx-bpdu timeout command to set the BPDU reception timer. Use the stp timer blackhole-detection-interval or stp global timer blackhole-detection-interval command to set the interval for sending spanning tree blackhole detection packets.

            - Blackhole confirmation phase.

If the port is triggered to send spanning tree blackhole detection packets and receives a BPDU, it proves that no BPDU blackhole exist between the devices. The port stops transmitting the spanning tree blackhole detection packets and resets the BPDU reception timer.

If a port with the spanning tree blackhole detection feature enabled receives a blackhole detection frame while its BPDU reception timer is in a timeout state, it indicates that the link between the local and remote devices is connected. Both devices are capable of sending BPDUs, but neither device receives BPDUs. The local device determines that a BPDU blackhole exists between the two ends of the link.

            - Blackhole handling phase.

After the device discovers a BPDU blackhole, it blocks the port that receives the spanning tree blackhole detection packet. Loops in the network are eliminated by preventing ports from sending and receiving any user data packets. You can use the stp timer rx-blackhole-timeout or stp global timer rx-blackhole-timeout command to specify the timeout period of the blocked port for spanning tree blackhole detection. If the port does not receive the spanning tree blackhole detection packet again within the timeout period, it means that the BPDU blackhole is eliminated, and the port returns to the normal forwarding state. Otherwise, the port continues to retain the blocking state.

Restrictions and guidelines

If the spanning tree blackhole detection is disabled on the port, or the BPDU reception timer on the port has not expired, the port does not perform any processing after receiving the spanning tree blackhole detection packet.

Examples

# Enable spanning tree blackhole detection globally.

<Sysname> system-view

[Sysname] stp global blackhole-detection enable

Related commands

stp blackhole-detection enable

stp timer rx-blackhole-timeout

stp global blackhole-detection rx-bpdu timeout

stp global timer rx-blackhole-timeout

stp global timer blackhole-detection-interval

stp timer blackhole-detection-interval

stp global blackhole-detection rx-bpdu timeout

Use stp global blackhole-detection rx-bpdu timeout to set the BPDU reception timer for the blackhole detection feature.

Use undo stp global blackhole-detection rx-bpdu timeout to restore the default.

Syntax

stp global blackhole-detection rx-bpdu timeout timeout

undo stp global blackhole-detection rx-bpdu timeout

Default

The BPDU reception timer of the blackhole detection feature is 18 seconds.

Views

System view

Predefined user roles

network-admin

Parameters

Timeout: Specifies the BPDU reception timer of the blackhole detection feature, in the range of 1 to 32768 seconds.

Usage guidelines

Operating mechanism

When the spanning tree blackhole detection is enabled both globally and on ports, a port enabled with spanning tree blackhole detection will start the BPDU reception timer of the blackhole detection feature. If the port receives a BPDU before the BPDU reception timer expires, the port resets the BPDU reception timer. If the BPDU reception timer expires, BPDU blackholes might exist on the network. As a result, the device fails to receive BPDUs for a long time, and the port starts to send spanning tree blackhole detection packets continuously.

Restrictions and guidelines

On a device with a long interval for sending BPDUs, set a longer value for the BPDU reception timer to prevent the device port from being blocked when no BPDU blackhole exists on the network. On the device with a short interval for sending BPDUs, set a shorter value for the BPDU reception timer to improve the network's detection speed of BPDU blackholes.

Only when the link status of the port is up and the spanning tree feature is enabled, the BPDU reception timer can be enabled on the port.

Examples

# Set the BPDU reception timer for the blackhole detection feature to 20 seconds.

<Sysname> system-view

[Sysname] stp global blackhole-detection rx-bpdu timeout 20s

Related commands

stp blackhole-detection enable

stp global blackhole-detection enable

stp global timer blackhole-detection-interval

Use stp global timer blackhole-detection-interval to set the interval for sending spanning tree blackhole detection packets globally.

Use undo global stp timer blackhole-detection-interval to restore the default.

Syntax

stp global timer blackhole-detection-interval interval

undo stp global timer blackhole-detection-interval

Default

The interval for sending spanning tree blackhole detection packets is 2 seconds

Views

System view

Predefined user roles

network-admin

Parameters

interval: Specifies the interval for sending spanning tree blackhole detection packets, in the range of 1 to 32768 seconds

Usage guidelines

Recommended configuration

This command affects the frequency at which the specified port sends spanning tree blackhole detection packets. Set an appropriate packet sending interval as needed.

- When the sending interval is short, the port sends spanning tree blackhole packets more frequently. The advantage is that the network is more sensitive in detection and response to BPDU blackholes. The disadvantage is that it takes up more device CPU resources. Set a short sending interval in scenarios with strong device performance or a strong need to prevent loops.

- When the sending interval is long, the port sends spanning tree blackhole packets slowly. The advantage is that it occupies less CPU resources of the device. The disadvantage is that the network detects and responds more slowly to BPDU blackholes. Set a long sending interval in the scenario where the device performance is weak or tolerant of loops.

Operating mechanism

You can use the stp timer blackhole-detection-interval and stp global timer blackhole-detection-interval commands to modify the interval for sending spanning tree blackhole detection packets on a port. When both commands are configured at the same time, the configuration of the stp timer blackhole-detection-interval command takes effect. If no command is configured on the port, the port inherits the configuration of the stp global timer blackhole-detection-interval command.

Examples

# Set the interval for sending spanning tree blackhole detection packets to 4 seconds globally.

<Sysname> system-view

[Sysname] stp global timer blackhole-detection-interval 4

Related commands

stp blackhole-detection enable

stp global blackhole-detection enable

stp timer blackhole-detection-interval

stp global timer rx-blackhole-timeout

Use stp global timer rx-blackhole-timeout to set the detection packet reception timer for spanning tree blackhole detection globally.

Use undo stp global timer rx-blackhole-timeout to restore the default.

Syntax

stp global timer rx-blackhole-timeout timeout

undo stp global timer rx-blackhole-timeout

Default

The formula for calculating the timeout period (seconds) for receiving spanning tree blackhole packets is: Multiply the sending interval of spanning tree blackhole detection packets on a port by 3 and add 10.

Views

System view

Predefined user roles

network-admin

Parameters

timeout: Specifies the detection packet reception timer for spanning tree blackhole detection, in the range of 10 to 65535 seconds.

Usage guidelines

Operating mechanism

After the port is blocked by the spanning tree blackhole detection, the timer for receiving spanning tree blackhole detection packets is started. Before the timer expires, the port will reset the timer when it receives a spanning tree blackhole detection packet, and remain in the blocking state. After the timer expires, the port returns to the normal forwarding state.

Restrictions and guidelines

You can use the stp timer rx-blackhole-timeout and stp global timer rx-blackhole-timeout commands to modify the interval for sending spanning tree blackhole detection packets on a port. When both commands are configured at the same time, the configuration of the stp timer rx-blackhole-timeout command takes effect. If no command is configured on the port, the port inherits the configuration of the stp global timer rx-blackhole-timeout command.

Examples

# Set the detection packet reception timer for spanning tree blackhole detection to 12 seconds globally.

<Sysname> system-view

[Sysname] stp global timer rx-blackhole-timeout 12

Related commands

stp blackhole-detection enable

stp global blackhole-detection enable

stp timer rx-blockhole-detection

stp timer blackhole-detection-interval

Use stp timer blackhole-detection-interval to set the interval for sending spanning tree blackhole detection packets on a port.

Use undo stp timer blackhole-detection-interval to restore the default.

Syntax

stp timer blackhole-detection-interval interval

undo stp timer blackhole-detection-interval

Default

The interval for sending spanning tree blackhole detection packets on a port is 2 seconds.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

interval: Specifies the interval for sending spanning tree blackhole detection packets, in the range of 10 to 32768 seconds.

Usage guidelines

Recommended configuration

This command affects the frequency at which the specified port sends spanning tree blackhole detection packets. Set an appropriate packet sending interval as needed.

- When the sending interval is short, the port sends spanning tree blackhole packets more frequently. The advantage is that the network is more sensitive in detection and response to BPDU blackholes. The disadvantage is that it takes up more device CPU resources. Set a short sending interval in scenarios with strong device performance or a strong need to prevent loops.

- When the sending interval is long, the port sends spanning tree blackhole packets slowly. The advantage is that it occupies less CPU resources of the device. The disadvantage is that the network detects and responds more slowly to BPDU blackholes. Set a long sending interval in the scenario where the device performance is weak or tolerant of loops.

Operating mechanism

You can use the stp timer blackhole-detection-interval and stp global timer blackhole-detection-interval commands to modify the interval for sending spanning tree blackhole detection packets on a port. When both commands are configured at the same time, the configuration of the stp timer blackhole-detection-interval command takes effect. If no command is configured on the port, the port inherits the configuration of the stp global timer blackhole-detection-interval command.

Examples

# Set the interval for sending spanning tree blackhole detection packets to 4 seconds on Ten-GigabitEthernet 3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] stp timer blackhole-detection-interval 4

Related commands

stp blackhole-detection enable

stp global blackhole-detection enable

stp global timer blackhole-detection-interval

stp timer rx-blackhole-timeout

Use stp timer rx-blackhole-timeout to set the detection packet reception timer for spanning tree blackhole detection on a port.

Use undo stp timer rx-blackhole-timeout to restore the default.

Syntax

stp timer rx-blackhole-timeout timeout

undo timer stp rx-blackhole-timeout

Default

The formula for calculating the timeout period (seconds) for receiving spanning tree blackhole packets on the port is: Multiply the sending interval of spanning tree blackhole detection packets on a port by 3 and add 10.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

timeout: Specifies the detection packet reception timer for spanning tree blackhole detection, in the range of 10 to 65535 seconds.

Usage guidelines

Operating mechanism

After the port is blocked by the spanning tree blackhole detection, the timer for receiving spanning tree blackhole detection packets is started. Before the timer expires, the port will reset the timer when it receives a spanning tree blackhole detection packet, and remain in the blocking state. After the timer expires, the port returns to the normal forwarding state.

Restrictions and guidelines

You can use the stp timer rx-blackhole-timeout and stp global timer rx-blackhole-timeout commands to modify the interval for sending spanning tree blackhole detection packets on a port. When both commands are configured at the same time, the configuration of the stp timer rx-blackhole-timeout command takes effect. If no command is configured on the port, the port inherits the configuration of the stp global timer rx-blackhole-timeout command.

Examples

# Set the detection packet reception timer for spanning tree blackhole detection to 12 seconds on Ten-GigabitEthernet 3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] stp timer rx-blackhole-timeout 12

Related commands

stp blackhole-detection enable

stp global blackhole-detection enable

stp global timer rx-blockhole-detection

New feature: LLDP back hole detection

Configuring LLDP black hole detection

About this task

As shown in Table 1Figure 1, Device A and Device B are connected through a hub, causing a loop in the network. However, since LLDP packets are point-to-point packets, they are terminated at the next node after being sent from the device. Therefore, the hub acts as a black hole for LLDP packets and LLDP packets cannot be transmitted between Device A and Device B through the HUB. As a result, Device A and Device B cannot use LLDP to learn about the physical link between them and the formed loop, and therefore the loop cannot be eliminated in time. In order to eliminate potential loop risks, it required to detect LLDP packet black holes and block the links leading to the black holes.

          - LLDP black hole detection application scenario

The LLDP black hole detection feature can detect the presence of LLDP black holes in links on ports. The operating mechanism is as follows:

            - Triggering the sending of black hole detection packets

The ports enabled with LLDP black hole detection will carry black hole detection TLVs in the transmitted LLDP packets. The black hole detection TLV is a type of TLV defined by HPE. If an LLDP packet carrying the black hole detection TLV is received, it indicates that there is no LLDP black hole between the device and its neighbor, and LLDP packets can be sent and received successfully. Only HPE devices can send and recognize the black hole detection TLV carried in LLDP packets.

After LLDP black hole detection is enabled on a port, the port will activate the LLDP packet receiving timer for black hole detection. If no LLDP packets are received on the port until the timer expires, it indicates a potential LLDP packet black hole in the network, causing the device to not receive LLDP packets. The port then starts sending LLDP black hole detection packets continuously. If the port receives an LLDP packet carrying the black hole detection TLV before the timer expires, the port terminates the timer and takes no further action.

After LLDP black hole detection is enabled, if a port receives an LLDP packet without the black hole detection TLV, the port will start sending LLDP black hole detection packets continuously.

LLDP black hole detection packet is an HPE proprietary Layer 2 packet type. Unlike LLDP packets, LLDP black hole detection packets can be transparently transmitted by a hub and are continuously sent at fixed intervals after being triggered.

The timeout for receiving LLDP packets is configured through the lldp global blackhole-detection rx-lldpdu timeout command. The interval for sending LLDP black hole detection packets is configured through the lldp timer blackhole-detection-interval command or the lldp global timer blackhole-detection-interval command.

            - Black hole confirmation

After a port starts to send LLDP black hole detection packets, if it receives an LLDP packet carrying the black hole detection TLV, it determines that no LLDP black hole exists between the port and its neighbor, stops sending LLDP black hole detection packets, and resets the LLDP packet receiving timer.

If a port enabled with LLDP black hole detection receives an LLDP black hole detection packet and the LLDP packet receiving timer on this port has timed out, it indicates that the local device and the peer device are connected with a link and both devices have the ability to send LLDP packets. However, neither of the devices has received LLDP packets, and therefore the local device concludes that there is an LLDP black hole between the two ends of the link.

            - Black hole processing

After detecting an LLDP black hole, the device blocks the port receiving LLDP black hole detection packets to eliminate network loops and prevent the transmission of user data packets. If the blocked port receives no LLDP black hole detection packets within the timeout period specified by the lldp timer rx-blackhole-timeout or lldp global timer rx-blackhole-timeout command, it means that the LLDP black hole is eliminated and the port resumes normal forwarding. Otherwise, it remains blocked.

The lldp timer blackhole-detection-interval and lldp global timer blackhole-detection-interval commands affect the interval at which the specified ports send LLDP black hole detection packets. Set a suitable packet sending interval based on actual requirements.

- When the sending interval is short, the port sends LLDP black hole detection packets frequently. The advantage is that the network detects and responds to LLDP black holes more sensitively. The disadvantage is that it occupies more device CPU resources. Configure a short sending interval in scenarios where the device performance is high or there is a strong demand for loop prevention.

- When the sending interval is long, the port sends LLDP black hole detection packets slowly. The advantage is that it occupies less CPU resources of the device; while the disadvantage is that the device cannot detect and respond to LLDP black holes quickly. Configure longer sending intervals in scenarios where device performance is weaker or loop tolerance is higher.

      - Restrictions and guidelines

The LLDP black hole detection feature takes effect only if the following conditions are met:

- On both ends of the LLDP connection, both global and port-level LLDP black hole detection functions are enabled.

- On the ports with LLDP black hole detection enabled, the link status is UP, LLDP is enabled, and the operating mode is TxRx.

If LLDP black hole detection is not enabled on a port or the LLDP packet receiving timer of the port has not timed out, the port will not process any LLDP black hole detection packets received.

On devices with a long LLDP packet sending interval,  configure a longer LLDP packet receiving timeout. Otherwise, it might result in device ports being blocked even if no LLDP black hole exists in the network. Configure a shorter LLDP receiving timeout on devices with a shorter LLDP packet sending interval for more efficient LLDP black hole detection.

The lldp timer blackhole-detection-interval and lldp global timer blackhole-detection-interval commands can both set the interval at which ports send LLDP black hole detection packets. When both commands are configured, the lldp timer blackhole-detection-interval command takes precedence. If the interval is not set on a port, the port uses the global interval set by the lldp global timer blackhole-detection-interval command.

The timeout for receiving LLDP black hole detection packets on a port can be configured by using the lldp timer rx-blackhole-timeout and lldp global timer rx-blackhole-timeout commands. When both commands are configured, the lldp timer rx-blackhole-timeout command takes precedence. If a port is not configured with a timeout, it uses the global timeout set by the lldp global timer rx-blackhole-timeout command.

The timeout for receiving LLDP black hole detection packets on the local device should not be less than the interval for sending LLDP black hole detection packets on the peer device. Otherwise, LLDP black hole detection will not take effect.

      - Procedure

            - Enter system view.

system-view

            - Enable global LLDP black hole detection.

lldp global blackhole-detection enable

By default, global LLDP black hole detection is disabled.

            - (Optional.) Configure the LLDP packet receiving timeout for black hole detection.

lldp global blackhole-detection rx-lldpdu timeout

By default, the LLDP packet receiving timeout for black hole detection is 120 seconds.

            - (Optional.) Set the global interval for sending LLDP black hole detection packets.

lldp global timer blackhole-detection-interval interval

By default, the interval for sending LLDP black hole detection packets is 2 seconds.

            - (Optional.) Set the global timeout for receiving LLDP black hole detection packets.

lldp global timer rx-blackhole-timeout timeout

By default, the timeout for receiving LLDP black hole detection packets is (the global interval for sending LLDP black hole detection packets × 3 + 10) seconds.

            - (Optional.) Enter Layer 2 Ethernet interface view.

interface interface-type interface-number

            - (Optional.) Enable LLDP black hole detection for a port.

lldp blackhole-detection enable

By default, LLDP black hole detection is enabled for a port.

            - (Optional.) Set the interval at which the port sends LLDP black hole detection packets.

lldp timer blackhole-detection-interval interval

By default, the interval for sending LLDP black hole detection packets is 2 seconds.

            - (Optional.) Set the timeout for receiving LLDP black hole detection packets on the port.

lldp timer rx-blackhole-timeout timeout

By default, the timeout for receiving LLDP black hole detection packets on a port is (the port's sending interval for LLDP black hole detection packets × 3 + 10) seconds.

  - Command reference

    - lldp blackhole-detection enable

Use lldp blackhole-detection enable to enable LLDP black hole detection for a port.

Use undo lldp blackhole-detection enable to disable LLDP black hole detection for a port.

Syntax

lldp blackhole-detection enable

undo lldp blackhole-detection enable

Default

LLDP black hole detection is enabled for a port.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Usage guidelines

Prerequisites

The LLDP black hole detection feature takes effect only if the following conditions are met:

- On both ends of the LLDP connection, both global and port-level LLDP black hole detection functions are enabled.

- On the ports with LLDP black hole detection enabled, the link status is UP, LLDP is enabled, and the operating mode is TxRx.

Operating mechanism

            - Triggering the sending of black hole detection packets

The ports enabled with LLDP black hole detection will carry black hole detection TLVs in the transmitted LLDP packets. The black hole detection TLV is a type of TLV defined by HPE. If an LLDP packet carrying the black hole detection TLV is received, it indicates that there is no LLDP black hole between the device and its neighbor, and LLDP packets can be sent and received smoothly. Only HPE devices can send and recognize the black hole detection TLV carried in LLDP packets.

After LLDP black hole detection is enabled on a port, the port will activate the LLDP packet receiving timer for black hole detection. If no LLDP packets are received on the port until the timer expires, it indicates a potential LLDP packet black hole in the network, causing the device to not receive LLDP packets. The port then starts sending continuous LLDP black hole detection packets. If the port receives an LLDP packet carrying the black hole detection TLV before the timer expires, the port terminates the timer and takes no further action.

After LLDP black hole detection is enabled, if a port receives an LLDP packet without the black hole detection TLV, the port will start sending LLDP black hole detection packets continuously.

LLDP black hole detection packet is an HPE proprietary Layer 2 packet type. Unlike LLDP packets, LLDP black hole detection packets can be transparently transmitted by a hub and are continuously sent at fixed intervals after being triggered.

The timeout period for receiving LLDP packets is configured by using the lldp global blackhole-detection rx-lldpdu timeout command. The interval for sending LLDP black hole detection packets is configured through the lldp timer blackhole-detection-interval command or the lldp global timer blackhole-detection-interval command.

            - Black hole confirmation

After a port starts to send LLDP black hole detection packets, if it receives an LLDP packet carrying the black hole detection TLV, it determines that no LLDP black hole exists between the port and its neighbor, stops sending LLDP black hole detection packets, and resets the LLDP packet receiving timer.

If a port enabled with LLDP black hole detection receives an LLDP black hole detection packet and the LLDP packet receiving timer on this port has timed out, it indicates that the local device and the peer device are connected with a link and both devices have the ability to send LLDP packets. However, neither of the devices has received LLDP packets, and therefore the local device concludes that there is an LLDP black hole between the two ends of the link.

            - Black hole processing

After detecting an LLDP black hole, the device blocks the ports receiving LLDP black hole detection packets to eliminate network loops and prevent the transmission of user data packets. If the blocked port receives no LLDP black hole detection packets within the timeout period specified by the lldp timer rx-blackhole-timeout or lldp global timer rx-blackhole-timeout command, it means that the LLDP black hole is eliminated and the port resumes normal forwarding. Otherwise, it remains blocked.

Restrictions and guidelines

If LLDP black hole detection is not enabled on a port or the LLDP packet receiving timer of the port has not timed out, the port will not process any LLDP black hole detection packets received.

For LLDP black hole detection to take effect on a port, you must enable LLDP black hole detection globally and for that port.

Examples

# Enable LLDP black hole detection on port Ten-GigabitEthernet3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] lldp blackhole-detection enable

Related commands

lldp blackhole-detection enable

lldp global timer rx-blackhole-timeout

lldp global blackhole-detection rx-lldpdu timeout

lldp global timer blackhole-detection-interval

lldp timer blackhole-detection-interval

lldp timer rx-blackhole-timeout

    - lldp global blackhole-detection enable

Use lldp global blackhole-detection enable to enable global LLDP black hole detection.

Use undo lldp global blackhole-detection enable to disable global LLDP black hole detection.

Syntax

lldp global blackhole-detection enable

undo lldp global blackhole-detection enable

Default

Global LLDP black hole detection is disabled.

Views

System view

Predefined user roles

network-admin

Usage guidelines

Application scenarios

          - LLDP black hole detection application scenario

As shown in Figure 2, Device A and Device B are connected through a hub, causing a loop in the network. However, since LLDP packets are point-to-point packets, they are terminated at the next node after being sent from the device. Therefore, the HUB acts as a black hole for LLDP packets. LLDP packets cannot be transmitted between Device A and Device B through the HUB. As a result, Device A and Device B cannot use LLDP to learn about the physical link between them and the formed loop, and therefore the loop cannot be eliminated in time. In order to eliminate potential loop risks, it required to detect LLDP black holes and block the links leading to the black holes.

Prerequisites

The LLDP black hole detection feature takes effect only if the following conditions are met:

- On both ends of the LLDP connection, both global and port-level LLDP black hole detection functions are enabled.

- On the ports with LLDP black hole detection enabled, the link status is UP, LLDP is enabled, and the operating mode is TxRx.

Operating mechanism

            - Black hole detection

The ports enabled with LLDP black hole detection will carry black hole detection TLVs in the transmitted LLDP packets. The black hole detection TLV is a type of TLV defined by HPE. If an LLDP packet carrying the black hole detection TLV is received, it indicates that there is no LLDP black hole between the device and its neighbor, and LLDP packets can be sent and received smoothly. Only HPE devices can send and recognize the black hole detection TLV carried in LLDP packets.

After LLDP black hole detection is enabled on a port, the port will activate the LLDP packet receiving timer for black hole detection. If no LLDP packets are received on the port until the timer expires, it indicates a potential LLDP packet black hole in the network, causing the device to not receive LLDP packets. The port then starts sending continuous LLDP black hole detection packets. If the port receives an LLDP packet carrying the black hole detection TLV before the timer expires, the port terminates the timer and takes no further action.

After LLDP black hole detection is enabled, if a port receives an LLDP packet without the black hole detection TLV, the port will start sending LLDP black hole detection packets continuously.

LLDP black hole detection packet is an HPE proprietary Layer 2 packet type. Unlike LLDP packets, LLDP black hole detection packets can be transparently transmitted by a hub and are continuously sent at fixed intervals after being triggered.

The timeout period for receiving LLDP packets is configured by using the lldp global blackhole-detection rx-lldpdu timeout command. The interval for sending LLDP black hole detection packets is configured through the lldp timer blackhole-detection-interval command or the lldp global timer blackhole-detection-interval command.

            - Black hole confirmation

After a port starts to send LLDP black hole detection packets, if it receives an LLDP packet carrying the black hole detection TLV, it determines that no LLDP black hole exists between the port and its neighbor, stops sending LLDP black hole detection packets, and resets the LLDP packet receiving timer.

If a port enabled with LLDP black hole detection receives an LLDP black hole detection packet and the LLDP packet receiving timer on this port has timed out, it indicates that the local device and the peer device are connected with a link and both devices have the ability to send LLDP packets. However, neither of the devices has received LLDP packets, and therefore the local device concludes that there is an LLDP black hole between the two ends of the link.

            - Black hole processing

After detecting an LLDP black hole, the device blocks the ports receiving LLDP black hole detection packets to eliminate network loops and prevent the transmission of user data packets. If the blocked port receives no LLDP black hole detection packets within the timeout period specified by the lldp timer rx-blackhole-timeout or lldp global timer rx-blackhole-timeout command, it means that the LLDP black hole is eliminated and the port resumes normal forwarding. Otherwise, it remains blocked.

Restrictions and guidelines

If LLDP black hole detection is not enabled on a port or the LLDP packet receiving timer of the port has not timed out, the port will not process any LLDP black hole detection packets received.

Examples

# Enable global LLDP black hole detection.

<Sysname> system-view

[Sysname] lldp global blackhole-detection enable

Related commands

lldp blackhole-detection enable

lldp global timer rx-blackhole-timeout

lldp global blackhole-detection rx-lldpdu timeout

lldp global timer blackhole-detection-interval

lldp timer blackhole-detection-interval

lldp timer rx-blackhole-timeout

    - lldp global blackhole-detection rx-lldpdu timeout

Use lldp global blackhole-detection rx-lldpdu timeout to configure the LLDP packet receiving timeout for black hole detection.

Use undo lldp global blackhole-detection rx-lldpdu timeout to restore the default.

Syntax

lldp global blackhole-detection rx-lldpdu timeout

undo lldp global blackhole-detection rx-lldpdu timeout

Default

The LLDP packet receiving timeout for black hole detection is 120 seconds.

Views

System view

Predefined user roles

network-admin

Parameters

timeout: LLDP packet receiving timeout for black hole detection, in the range of 1 to 32768, in seconds.

Usage guidelines

Operating mechanism

After LLDP black hole detection is enabled on a port, the port will activate the LLDP packet receiving timer for black hole detection. If no LLDP packets are received on the port until the timer expires, it indicates a potential LLDP packet black hole in the network, causing the device to not receive LLDP packets. The port then starts sending continuous LLDP black hole detection packets. If the port receives an LLDP packet carrying the black hole detection TLV before the timer expires, the port terminates the timer and takes no further action.

Restrictions and guidelines

On devices with a long LLDP packet sending interval, configure a longer LLDP packet receiving timeout. Otherwise, it might result in device ports being blocked even if no LLDP black hole exists in the network. Configure a shorter LLDP receiving timeout on devices with a shorter LLDP packet sending interval for more efficient LLDP black hole detection.

The LLDP packet receiving timeout can take effect on a port only when the link state of the port is UP, LLDP is enabled on the port, and the port operates in TxRx mode.

Examples

# Configure the LLDP packet receiving timeout for black hole detection as 140 seconds.

<Sysname> system-view

[Sysname] lldp global blackhole-detection rx-lldpdu timeout 140

Related commands

lldp admin-status

lldp blackhole-detection enable

lldp global blackhole-detection enable

    - lldp global timer blackhole-detection-interval

Use lldp global timer blackhole-detection-interval to configure the global interval for sending LLDP black hole detection packets.

Use undo lldp global timer blackhole-detection-interval to restore the default.

Syntax

lldp global timer blackhole-detection-interval interval

undo lldp global timer blackhole-detection-interval

Default

The interval for sending LLDP black hole detection packets is 2 seconds.

Views

System view

Predefined user roles

network-admin

Parameters

interval: Interval for sending LLDP black hole detection packets, in the range of 1 to 32768, in seconds.

Usage guidelines

Recommended configuration

This command takes effect on all ports that have enabled LLDP black hole detection. It controls the speed at which the ports send LLDP black hole detection packets globally. Set an appropriate packet sending interval based on the actual requirements.

- When the sending interval is short, the port sends LLDP black hole detection packets frequently. The advantage is that the network detects and responds to LLDP black holes more sensitively. The disadvantage is that it occupies more device CPU resources. Configure a short sending interval in scenarios where the device performance is high or there is a strong demand for loop prevention.

- When the sending interval is long, the port sends LLDP black hole detection packets slowly. The advantage is that it occupies less CPU resources of the device; while the disadvantage is that the device cannot detect and respond to LLDP black holes quickly. Configure longer sending intervals in scenarios where device performance is weaker or loop tolerance is higher.

Operating mechanism

The lldp timer blackhole-detection-interval and lldp global timer blackhole-detection-interval commands can both set the interval at which ports send LLDP black hole detection packets. When both commands are configured, the lldp timer blackhole-detection-interval command takes precedence. If the interval is not set on a port, the port uses the global interval set by the lldp global timer blackhole-detection-interval command.

Examples

# Set the global interval for sending LLDP black hole detection packets to 4 seconds.

<Sysname> system-view

[Sysname] lldp global timer blackhole-detection-interval 4

Related commands

lldp blackhole-detection enable

lldp global blackhole-detection enable

lldp timer blackhole-detection-interval

    - lldp global timer rx-blackhole-timeout

Use lldp global timer rx-blackhole-timeout to configure the global timeout for receiving LLDP black hole detection packets.

Use undo lldp global timer rx-blackhole-timeout to restore the default.

Syntax

lldp global timer rx-blackhole-timeout timeout

undo lldp global timer rx-blackhole-timeout

Default

The timeout for receiving LLDP black hole detection packets is (the global interval for sending LLDP black hole detection packets × 3 + 10) seconds.

Views

System view

Predefined user roles

network-admin

Parameters

timeout: Timeout for receiving LLDP black hole detection packets, in the range of 10 to 65535, in seconds.

Usage guidelines

Operating mechanism

When a port is blocked by LLDP black hole detection, the port starts the LLDP black hole detection packet receiving timer. Before the timer times out, the port resets the timer and remains in a blocked state when it receives an LLDP black hole detection packet. After the timer times out, the port resumes normal forwarding.

Restrictions and guidelines

The timeout for receiving LLDP black hole detection packets on a port can be configured by using the lldp timer rx-blackhole-timeout and lldp global timer rx-blackhole-timeout commands. When both commands are configured, the lldp timer rx-blackhole-timeout command takes precedence. If a port is not configured with a timeout, it uses the global timeout configured by the lldp global timer rx-blackhole-timeout command.

The timeout for receiving LLDP black hole detection packets on the local device should not be less than the interval for sending LLDP black hole detection packets on the peer device. Otherwise, LLDP black hole detection will not take effect.

Examples

# Set the global timeout for receiving LLDP black hole detection packets to 14 seconds.

<Sysname> system-view

[Sysname] lldp global timer rx-blackhole-timeout 14

Related commands

lldp blackhole-detection enable

lldp global blackhole-detection enable

lldp global timer blackhole-detection-interval

lldp timer rx-blackhole-timeout

lldp timer blackhole-detection-interval

    - lldp timer blackhole-detection-interval

Use lldp timer blackhole-detection-interval to configure the interval at which a port sends LLDP black hole detection packets.

Use undo lldp timer blackhole-detection-interval to restore the default.

Syntax

lldp timer blackhole-detection-interval interval

undo lldp timer blackhole-detection-interval

Default

A port sends LLDP black hole detection packets at intervals of 2 seconds.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

interval: Interval for sending LLDP black hole detection packets, in the range of 1 to 32768, in seconds.

Usage guidelines

Recommended configuration

This command takes effect on the port where the command is executed. Set a suitable packet sending  interval according to actual requirements.

- When the sending interval is short, the port sends LLDP black hole detection packets frequently. The advantage is that the network detects and responds to LLDP black holes more sensitively. The disadvantage is that it occupies more device CPU resources. Configure a short sending interval in scenarios where the device performance is high or there is a strong demand for loop prevention.

- When the sending interval is long, the port sends LLDP black hole detection packets slowly. The advantage is that it occupies less CPU resources of the device; while the disadvantage is that the device cannot detect and respond to LLDP black holes quickly. Configure longer sending intervals in scenarios where device performance is weaker or loop tolerance is higher.

Operating mechanism

The lldp timer blackhole-detection-interval and lldp global timer blackhole-detection-interval commands can both set the interval at which ports send LLDP black hole detection packets. When both commands are configured, the lldp timer blackhole-detection-interval command takes precedence. If the interval is not set on a port, the port uses the global interval set by the lldp global timer blackhole-detection-interval command.

Examples

# Configure port Ten-GigabitEthernet3/0/1 to send LLDP black hole detection packets at intervals of 4 seconds.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] lldp timer blackhole-detection-interval 4

Related commands

lldp blackhole-detection enable

lldp global blackhole-detection enable

lldp global timer blackhole-detection-interval

    - lldp timer rx-blackhole-timeout

Use lldp timer rx-blackhole-timeout to configure the timeout for receiving LLDP black hole detection packets on a port.

Use undo lldp timer rx-blackhole-timeout to restore the default.

Syntax

lldp timer rx-blackhole-timeout timeout

undo timer lldp rx-blackhole-timeout

Default

The timeout for a port to receive LLDP black hole detection packets is (the port’s sending interval for LLDP black hole detection packets × 3 + 10) seconds.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

timeout: Timeout for receiving LLDP black hole detection packets, in the range of 10 to 65535, in seconds.

Usage guidelines

Operating mechanism

When a port is blocked by LLDP black hole detection, the port starts the LLDP black hole detection packet receiving timer. Before the timer times out, the port resets the timer and remains in a blocked state when it receives an LLDP black hole detection packet. After the timer times out, the port resumes normal forwarding.

Restrictions and guidelines

The timeout for receiving LLDP black hole detection packets on a port can be configured by using the lldp timer rx-blackhole-timeout and lldp global timer rx-blackhole-timeout commands. When both commands are configured, the lldp timer rx-blackhole-timeout command takes precedence. If a port is not configured with a timeout, it uses the global timeout configured by the lldp global timer rx-blackhole-timeout command.

The timeout for receiving LLDP black hole detection packets on the local device should not be less than the interval for sending LLDP black hole detection packets on the peer device. Otherwise, LLDP black hole detection will not take effect.

Examples

# Configure the timeout for receiving LLDP black hole detection packets as 12 seconds on port Ten-GigabitEthernet3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] lldp timer rx-blackhole-timeout 12

Related commands

lldp blackhole-detection enable

lldp global blackhole-detection enable

lldp global timer rx-blackhole-timeout

lldp global timer blackhole-detection-interval

lldp timer blackhole-detection-interval

New feature: LLDP cross-domain detection

  - Configuring LLDP cross-domain detection

      - About this task

As shown in Figure 2, Device A and Device B belongs to different branch networks. If a link exists between Device A and Device B, a loop might occur in the network. After configuring LLDP cross-domain detection, you can manually assign the devices to different domains, and use LLDP to detect whether the specified LLDP neighbor is in the local domain. In addition, you can block the link to neighbors in other domains to eliminate the potential loop risk.

          - LLDP cross-domain detection application scenario

The LLDP cross-domain detection feature defines a private type of TLVs for LLDP, that is, the cross-domain detection TLVs. This TLV is used to advertise the local domain to the LLDP neighbor at the peer end. The LLDP cross-domain detection feature takes effect on a port only after you perform the following operations:

- Execute the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection.

- Execute the lldp cross-domain-detection domain-id command to configure the domain ID for the local device.

Then, LLDP packets sent by the device will carry cross-domain detection TLVs.

A cross-domain detection TLV contains the domain ID, device bridge ID, port cost, and port ID. The device bridge ID is the bridge ID of the device that sends the cross-domain detection TLV. The port cost is the cost of the port that sends the cross-domain detection TLV. The port ID is the ID of the port that sends the cross-domain detection TLV.

Upon receiving LLDP packets carrying cross-domain detection TLVs, if the receiving ports are enabled with cross-domain detection, the device compares the domain IDs in the packets with the local setting:

- If the domain IDs are the same, the devices at the two ends of the LLDP session belong to the same domain, and the local device does not process the packets.

- If the domain IDs are different, the devices at the two ends of the LLDP session do not belong to the same domain. Data traffic transmission between each other might result in loops. To prevent this issue, the device performs the following operations:

  - If only one port receives an LLDP packet carrying the cross-domain detection TLV, the device blocks the port. In addition, it disables the port from receiving or sending any user data packets to eliminate loops in the network.

  - If multiple ports receive LLDP packets carrying the cross-domain detection TLVs with the same domain ID, the the device compares the cross-domain detection TLVs received by the ports:

    - The cross-domain detection TLV received by the port with the smallest cost is the optimal TLV.

    - If the ports have the same cost, the cross-domain detection TLV received by the device with the smallest bridge ID is the optimal TLV.

    - If the devices have the same bridge ID, the cross-domain detection TLV received by the port with the smallest ID is the optimal TLV.

Except the port that receives the optimal cross-domain detection TLV, the device blocks all other ports that receive cross-domain detection TLVs. In addition, it disables the ports from receiving or sending any user data packets to eliminate loops in the network.

A blocked port can restore normal forwarding status when LLDP cross-domain detection is disabled or it receives a cross-domain detection TLV carrying the same domain ID as the local setting.

      - Restrictions and guidelines

If the domain ID is not configured for LLDP cross-domain detection, or the port is not enabled with the cross-domain detection TLV advertisement capability, the cross-domain detection feature cannot take effect on the port. In this case, the device cannot identify cross-domain detection TLVs carried in LLDP packets or add cross-domain detection TLVs to LLDP packets.

You can use the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection for a port. If the lldp cross-domain-detection command is not command for the port, the global setting (configured with the lldp global cross-domain-detection enable command) applies. If both commands are configured for the port, the port-specific setting (configured with the lldp cross-domain-detection command) applies.

      - Procedure

            - Enter system view.

system-view

            - Enable LLDP cross-domain detection globally.

lldp global cross-domain-detection enable

By default, LLDP cross-domain detection is disabled globally.

            - Enter Layer 2 Ethernet interface view.

interface interface-type interface-number

            - Configure the domain ID for LLDP cross-domain detection.

lldp cross-domain-detection domain-id domain-id

By default, the domain ID is not configured for LLDP cross-domain detection.

            - Enable or disable LLDP cross-domain detection for the port.

lldp cross-domain-detection { enable | disable }

By default, the global setting (configured with the lldp global cross-domain-detection enable command) applies.

  - Command reference

    - lldp cross-domain-detection

Use lldp cross-domain-detection to enable or disable LLDP cross-domain detection for the port.

Use undo cross-domain-detection to restore the default.

Syntax

lldp cross-domain-detection { enable | disable }

undo lldp cross-domain-detection

Default

The global setting (configured with the lldp global cross-domain-detection enable command) applies.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

enable: Enables LLDP cross-domain detection for the port.

disable: Disables LLDP cross-domain detection for the port.

Usage guidelines

Operating mechanism

The LLDP cross-domain detection feature defines a private type of TLVs for LLDP, that is, the cross-domain detection TLVs. This TLV is used to advertise the local domain to the LLDP neighbor at the peer end. The LLDP cross-domain detection feature takes effect on a port only after you perform the following operations:

- Execute the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection.

- Execute the lldp cross-domain-detection domain-id command to configure the domain ID for the local device.

Then, LLDP packets sent by the device will carry cross-domain detection TLVs.

A cross-domain detection TLV contains the domain ID, device bridge ID, port cost, and port ID. The device bridge ID is the bridge ID of the device that sends the cross-domain detection TLV. The port cost is the cost of the port that sends the cross-domain detection TLV. The port ID is the ID of the port that sends the cross-domain detection TLV.

Upon receiving LLDP packets carrying cross-domain detection TLVs, if the receiving ports are enabled with cross-domain detection, the device compares the domain IDs in the packets with the local setting:

- If the domain IDs are the same, the devices at the two ends of the LLDP session belong to the same domain, and the local device does not process the packets.

- If the domain IDs are different, the devices at the two ends of the LLDP session do not belong to the same domain. Data traffic transmission between each other might result in loops. To prevent this issue, the device performs the following operations:

  - If only one port receives an LLDP packet carrying the cross-domain detection TLV, the device blocks the port. In addition, it disables the port from receiving or sending any user data packets to eliminate loops in the network.

  - If multiple ports receive LLDP packets carrying the cross-domain detection TLVs with the same domain ID, the the device compares the cross-domain detection TLVs received by the ports:

    - The cross-domain detection TLV received by the port with the smallest cost is the optimal TLV.

    - If the ports have the same cost, the cross-domain detection TLV received by the device with the smallest bridge ID is the optimal TLV.

    - If the devices have the same bridge ID, the cross-domain detection TLV received by the port with the smallest ID is the optimal TLV.

Except the port that receives the optimal cross-domain detection TLV, the device blocks all other ports that receive cross-domain detection TLVs. In addition, it disables the ports from receiving or sending any user data packets to eliminate loops in the network.

A blocked port can restore normal forwarding status when LLDP cross-domain detection is disabled or it receives a cross-domain detection TLV carrying the same domain ID as the local setting.

Restrictions and guidelines

If the domain ID is not configured for LLDP cross-domain detection, or the port is not enabled with the cross-domain detection TLV advertisement capability, the cross-domain detection feature cannot take effect on the port. In this case, the device cannot identify cross-domain detection TLVs carried in LLDP packets or add cross-domain detection TLVs to LLDP packets.

You can use the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection for a port. If the lldp cross-domain-detection command is not command for the port, the global setting (configured with the lldp global cross-domain-detection enable command) applies. If both commands are configured for the port, the port-specific setting (configured with the lldp cross-domain-detection command) applies.

Examples

# Disable LLDP cross-domain detection for Ten-GigabitEthernet3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] lldp cross-domain-detection disable

Related commands

- lldp cross-domain-detection domain-id

- lldp global cross-domain-detection enable

    - lldp cross-domain-detection domain-id

Use lldp cross-domain-detection domain-id to configure the domain ID for LLDP cross-domain detection.

Use undo lldp cross-domain-detection domain-id to remove the configuration.

Syntax

lldp cross-domain-detection domain-id domain-id

undo lldp cross-domain-detection domain-id

Default

The domain ID is not configured for LLDP cross-domain detection.

Views

Layer 2 Ethernet interface view

Predefined user roles

network-admin

Parameters

domain-id: Specifies a domain ID in the range of 1 to 10.

Usage guidelines

Application scenarios

As shown in Figure 4, Device A and Device B belongs to different branch networks. If a link exists between Device A and Device B, a loop might occur in the network. After configuring LLDP cross-domain detection, you can manually assign the devices to different domains, and use LLDP to detect whether the specified LLDP neighbor is in the local domain. In addition, you can block the link to neighbors in other domains to eliminate the potential loop risk.

          - LLDP cross-domain detection application scenario

Operating mechanism

The LLDP cross-domain detection feature defines a private type of TLVs for LLDP, that is, the cross-domain detection TLVs. This TLV is used to advertise the local domain to the LLDP neighbor at the peer end. The LLDP cross-domain detection feature takes effect on a port only after you perform the following operations:

- Execute the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection.

- Execute the lldp cross-domain-detection domain-id command to configure the domain ID for the local device.

Then, LLDP packets sent by the device will carry cross-domain detection TLVs.

A cross-domain detection TLV contains the domain ID, device bridge ID, port cost, and port ID. The device bridge ID is the bridge ID of the device that sends the cross-domain detection TLV. The port cost is the cost of the port that sends the cross-domain detection TLV. The port ID is the ID of the port that sends the cross-domain detection TLV.

Upon receiving LLDP packets carrying cross-domain detection TLVs, if the receiving ports are enabled with cross-domain detection, the device compares the domain IDs in the packets with the local setting:

- If the domain IDs are the same, the devices at the two ends of the LLDP session belong to the same domain, and the local device does not process the packets.

- If the domain IDs are different, the devices at the two ends of the LLDP session do not belong to the same domain. Data traffic transmission between each other might result in loops. To prevent this issue, the device performs the following operations:

  - If only one port receives an LLDP packet carrying the cross-domain detection TLV, the device blocks the port. In addition, it disables the port from receiving or sending any user data packets to eliminate loops in the network.

  - If multiple ports receive LLDP packets carrying the cross-domain detection TLVs with the same domain ID, the the device compares the cross-domain detection TLVs received by the ports:

    - The cross-domain detection TLV received by the port with the smallest cost is the optimal TLV.

    - If the ports have the same cost, the cross-domain detection TLV received by the device with the smallest bridge ID is the optimal TLV.

    - If the devices have the same bridge ID, the cross-domain detection TLV received by the port with the smallest ID is the optimal TLV.

Except the port that receives the optimal cross-domain detection TLV, the device blocks all other ports that receive cross-domain detection TLVs. In addition, it disables the ports from receiving or sending any user data packets to eliminate loops in the network.

A blocked port can restore normal forwarding status when LLDP cross-domain detection is disabled or it receives a cross-domain detection TLV carrying the same domain ID as the local setting.

Restrictions and guidelines

If the domain ID is not configured for LLDP cross-domain detection, or the port is not enabled with the cross-domain detection TLV advertisement capability, the cross-domain detection feature cannot take effect on the port. In this case, the device cannot identify cross-domain detection TLVs carried in LLDP packets or add cross-domain detection TLVs to LLDP packets.

Examples

# Configure domain ID 2 for LLDP cross-domain detection on Ten-GigabitEthernet3/0/1.

<Sysname> system-view

[Sysname] interface ten-gigabitethernet 3/0/1

[Sysname-Ten-GigabitEthernet3/0/1] lldp cross-domain-detection domain-id 2

Related commands

- lldp cross-domain-detection

- lldp global cross-domain-detection enable

    - lldp global cross-domain-detection enable

Use lldp global cross-domain-detection enable to enable LLDP cross-domain detection globally.

Use undo lldp global cross-domain-detection enable to disable LLDP cross-domain detection globally.

Syntax

lldp global cross-domain-detection enable

undo lldp global cross-domain-detection enable

Default

LLDP cross-domain detection is disabled globally.

Views

System view

Predefined user roles

network-admin

Usage guidelines

Operating mechanism

The LLDP cross-domain detection feature defines a private type of TLVs for LLDP, that is, the cross-domain detection TLVs. This TLV is used to advertise the local domain to the LLDP neighbor at the peer end. The LLDP cross-domain detection feature takes effect on a port only after you perform the following operations:

- Execute the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection.

- Execute the lldp cross-domain-detection domain-id command to configure the domain ID for the local device.

Then, LLDP packets sent by the device will carry cross-domain detection TLVs.

A cross-domain detection TLV contains the domain ID, device bridge ID, port cost, and port ID. The device bridge ID is the bridge ID of the device that sends the cross-domain detection TLV. The port cost is the cost of the port that sends the cross-domain detection TLV. The port ID is the ID of the port that sends the cross-domain detection TLV.

Upon receiving LLDP packets carrying cross-domain detection TLVs, if the receiving ports are enabled with cross-domain detection, the device compares the domain IDs in the packets with the local setting:

- If the domain IDs are the same, the devices at the two ends of the LLDP session belong to the same domain, and the local device does not process the packets.

- If the domain IDs are different, the devices at the two ends of the LLDP session do not belong to the same domain. Data traffic transmission between each other might result in loops. To prevent this issue, the device performs the following operations:

  - If only one port receives an LLDP packet carrying the cross-domain detection TLV, the device blocks the port. In addition, it disables the port from receiving or sending any user data packets to eliminate loops in the network.

  - If multiple ports receive LLDP packets carrying the cross-domain detection TLVs with the same domain ID, the the device compares the cross-domain detection TLVs received by the ports:

    - The cross-domain detection TLV received by the port with the smallest cost is the optimal TLV.

    - If the ports have the same cost, the cross-domain detection TLV received by the device with the smallest bridge ID is the optimal TLV.

    - If the devices have the same bridge ID, the cross-domain detection TLV received by the port with the smallest ID is the optimal TLV.

Except the port that receives the optimal cross-domain detection TLV, the device blocks all other ports that receive cross-domain detection TLVs. In addition, it disables the ports from receiving or sending any user data packets to eliminate loops in the network.

A blocked port can restore normal forwarding status when LLDP cross-domain detection is disabled or it receives a cross-domain detection TLV carrying the same domain ID as the local setting.

Restrictions and guidelines

If the domain ID is not configured for LLDP cross-domain detection, or the port is not enabled with the cross-domain detection TLV advertisement capability, the cross-domain detection feature cannot take effect on the port. In this case, the device cannot identify cross-domain detection TLVs carried in LLDP packets or add cross-domain detection TLVs to LLDP packets.

You can use the lldp global cross-domain-detection enable or lldp cross-domain-detection command to enable LLDP cross-domain detection for a port. If the lldp cross-domain-detection command is not command for the port, the global setting (configured with the lldp global cross-domain-detection enable command) applies. If both commands are configured for the port, the port-specific setting (configured with the lldp cross-domain-detection command) applies.

Examples

# Enable LLDP cross-domain detection globally.

<Sysname> system-view

[Sysname] lldp global cross-domain-detection enable

Related commands

- lldp cross-domain-detection

- lldp cross-domain-detection domain-id

New feature: Using the subscriber ID as the client ID in all received DHCP requests

- Using the subscriber ID as the client ID in all received DHCP requests

- About this task

Typically, after receiving a DHCP request on a client-facing interface, the DHCP server identifies the client by the Client ID or chaddr field in the DHCP request, and assigns an IP address to the client. When a new client is attached to the interface and requests an IP address, the DHCP server assigns a new IP address to the client, because the Client ID or chaddr field changes. In certain scenarios, such as industrial production lines, when a device attached to an industrial switch is replaced by another device, the new device needs to participate in services with the same IP address.

To meet this requirement, enable the DHCP server to use the subscriber ID as the client ID in all received DHCP requests.

With this feature enabled on a DHCP server interface, the DHCP server performs interface-based address assignment as follows: 1. Identifies all client IDs in the DHCP requests received on that interface as the subscriber ID. 2. Assigns IP addresses based on the subscriber ID (interface name), ensuring that clients attached to this interface share the same IP address.

You can enable this feature globally by using the dhcp server subscriber-id replace client-id global command, or on a per-interface basis by using the dhcp server subscriber-id replace client-id command.

- This feature is enabled on an interface as long as it is enabled globally or on the interface.

- To enable this feature only on a single interface, you must disable this feature globally, and then enable the feature on the desired interface by using the dhcp server subscriber-id replace client-id command.

- Procedure

            - Enter system view.

system-view

            - Define the subscriber ID as interface name.

dhcp server subscriber-id interface-name

By default, content of the subscriber ID is not defined.

            - Enable the DHCP server to use the subscriber ID as the client ID on all interfaces.

dhcp server subscriber-id replace client-id global

By default, the DHCP server does not use the subscriber ID as the client ID.

            - Enable the DHCP server to use the subscriber ID as the client ID only on a single interface.

              - Globally disable the DHCP server from using the subscriber ID as the client ID.

undo dhcp server subscriber-id replace client-id global

By default, the DHCP server is globally disabled from using the subscriber ID as the client ID.

              - Enter interface view.

interface interface-type interface-number

              - Enable the DHCP server to use the subscriber ID as the client ID on the interface.

dhcp server subscriber-id replace client-id

By default, the DHCP server to use the subscriber ID as the client ID on an interface.

- Command reference

dhcp server subscriber-id replace client-id

Use dhcp server subscriber-id replace client-id to enable the DHCP server to use the subscriber ID as the client ID on an interface.

Use undo dhcp server subscriber-id replace client-id to disable the DHCP server from using the subscriber ID as the client ID on an interface.

Syntax

dhcp server subscriber-id replace client-id

undo dhcp server subscriber-id replace client-id

Default

The DHCP server does not use the subscriber ID as the client ID on any interface.

Views

Layer 2 Ethernet interface view/Layer 2 aggregate interface view

Predefined user roles

network-admin

Usage guidelines

Application scenarios

Typically, after receiving a DHCP request on a client-facing interface, the DHCP server identifies the client by the Client ID or chaddr field in the DHCP request, and assigns an IP address to the client. When a new client is attached to the interface and requests an IP address, the DHCP server assigns a new IP address to the client, because the Client ID or chaddr field changes. In certain scenarios, such as industrial production lines, when a device attached to an industrial switch is replaced by another device, the new device needs to participate in services with the same IP address.

To meet this requirement, enable the DHCP server to use the subscriber ID as the client ID in all received DHCP requests.

Operating mechanism

With this feature enabled on a DHCP server interface, the DHCP server performs interface-based address assignment as follows: 1. Identifies all client IDs in the DHCP requests received on that interface as the subscriber ID. 2. Assigns IP addresses based on the subscriber ID (interface name), ensuring that clients attached to this interface share the same IP address.

Prerequisites

To have this feature take effect, you must first define the subscriber ID as interface name by executing the dhcp server subscriber-id interface-name command.

Examples

# Enable the DHCP server to use the subscriber ID as the client ID on an interface.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] dhcp server subscriber-id replace client-id

dhcp server subscriber-id replace client-id global

Use dhcp server subscriber-id replace client-id global to enable the DHCP server to use the subscriber ID as the client ID on all interfaces.

Use undo dhcp server subscriber-id replace client-id to globally disable the DHCP server from using the subscriber ID as the client ID.

Syntax

dhcp server subscriber-id replace client-id global

undo dhcp server subscriber-id replace client-id global

Default

The DHCP server does not use the subscriber ID as the client ID on any interface.

Views

System view

Predefined user roles

network-admin

Usage guidelines

Application scenarios

Typically, after receiving a DHCP request on a client-facing interface, the DHCP server identifies the client by the Client ID or chaddr field in the DHCP request, and assigns an IP address to the client. When a new client is attached to the interface and requests an IP address, the DHCP server assigns a new IP address to the client, because the Client ID or chaddr field changes. In certain scenarios, such as industrial production lines, when a device attached to an industrial switch is replaced by another device, the new device needs to participate in services with the same IP address.

To meet this requirement, enable the DHCP server to use the subscriber ID as the client ID in all received DHCP requests.

Operating mechanism

With this feature enabled on a DHCP server interface, the DHCP server performs interface-based address assignment as follows: 1. Identifies all client IDs in the DHCP requests received on that interface as the subscriber ID. 2. Assigns IP addresses based on the subscriber ID (interface name), ensuring that clients attached to this interface share the same IP address.

You can enable this feature globally by using the dhcp server subscriber-id replace client-id global command, or on a per-interface basis by using the dhcp server subscriber-id replace client-id command.

- This feature is enabled on an interface as long as it is enabled globally or on the interface.

- To enable this feature only on a single interface, you must disable this feature globally, and then enable the feature on the desired interface by using the dhcp server subscriber-id replace client-id command.

Prerequisites

To have this feature take effect, you must first define the subscriber ID as interface name by executing the dhcp server subscriber-id interface-name command.

Examples

# Enable the DHCP server to use the subscriber ID as the client ID on all interfaces.

<Sysname> system-view

[Sysname] dhcp server subscriber-id replace client-id global

dhcp server subscriber-id interface-name

Use dhcp server subscriber-id interface-name to define the subscriber ID as interface name.

Use undo dhcp server subscriber-id replace client-id to remove the configuration.

Syntax

dhcp server subscriber-id interface-name

undo dhcp server subscriber-id interface-name

Default

Content of the subscriber ID is not defined.

Views

System view

Predefined user roles

network-admin

Usage guidelines

Before enabling the DHCP server to use the subscriber ID as the client ID, you must use this command to define the subscriber ID as interface name. If you fail to do so, the DHCP server cannot perform interface-based address assignment and thus it cannot assign the same IP address to clients attached to the same interface.

Examples

# Define the subscriber ID as interface name.

<Sysname> system-view

[Sysname] dhcp server subscriber-id interface-name

New feature: Configuring resource monitoring

Configuring resource monitoring

About this task

The resource monitoring feature enables the device to monitor the available amounts of types of resources, for example, the space for ARP entries. The device samples the available amounts periodically and compares the samples with resource depletion thresholds to identify the resource depletion status.

The device supports a minor resource depletion threshold and a severe resource depletion threshold for each supported resource type.

- If the available amount is equal to or less than the minor resource depletion threshold but greater than the severe resource depletion threshold, the resource type is in minor alarm state.

- If the available amount is equal to or less than the severe resource depletion threshold, the resource type is in severe alarm state.

- If the available amount increases above the minor resource depletion threshold, the resource type is in recovered state.

When a resource type enters severe alarm state, the device issues a severe alarm. If the resource type stays in severe alarm state, the device resends severe alarms periodically.

When a resource type enters minor alarm state, the device issues a minor alarm. If the resource type stays in minor alarm state or changes from severe alarm state to minor alarm state, the device identifies whether resending of minor resource depletion alarms is enabled. If the feature is disabled, the device does not issue additional minor alarms. If the feature is enabled, the device resends minor alarms periodically.

Resource depletion alarms can be sent to NETCONF, SNMP, and the information center to be encapsulated as NETCONF events, SNMP traps and informs, and log messages. For more information, see NETCONF, SNMP, and information center in Network Management and Monitoring Configuration Guide.

Resource depletion alarms and alarm-removed notifications

‌

Procedure

Enter system view.

system-view

Set resource depletion thresholds.

resource-monitor resource resource-name slot slot-number cpu cpu-number by-percent minor-threshold minor-threshold severe-threshold severe-threshold

The default settings vary by resource type. Use the display resource-monitor command to display the resource depletion thresholds.

Specify destinations for resource depletion alarms.

resource-monitor output { netconf-event | snmp-notification | syslog } *

By default, resource depletion alarms are sent to NETCONF, SNMP, and the information center.

Enable resending of minor resource depletion alarms.

resource-monitor minor resend enable

By default, resending of minor resource depletion alarms is enabled.

Command reference

display resource-monitor

Use display resource-monitor to display resource monitoring information.

Syntax

display resource-monitor [ resource resource-name ] [ slot slot-number [ cpu cpu-number ] ]

Views

Any view

Predefined user roles

network-admin

network-operator

Parameters

resource resource-name: Specifies a resource type by its name.

slot slot-number: Specifies an IRF member device by its member ID. If you do not specify a member device, this command displays resource monitoring information for all member devices.

cpu cpu-number: Specifies a CPU by its number.

Examples

# Display ARP resource monitoring information.

<Sysname> display resource-monitor resource arp

Minor alarms resending: Enabled

Slot 1:

Resource                         Minor Severe Free/Total

(%)   (%)    (absolute)

arp                              20    10     970/1019

Command output

| Field | Description |
| --- | --- |
| Minor alarms resending | Status of the minor resource depletion alarm resending feature, Enabled or Disabled. |
| Resource | Monitored resource type. |
| Minor (%) | Minor resource depletion threshold, in percentage. |
| Severe (%) | Severe resource depletion threshold, in percentage. |
| Free/Total (absolute) | Numbers of available resources and total resources, in absolute values. ‌ |

‌

Related commands

resource-monitor minor resend enable

resource-monitor resource

resource-monitor minor resend enable

Use resource-monitor minor resend enable to enable resending of minor resource depletion alarms.

Use undo resource-monitor minor resend enable to disable resending of minor resource depletion alarms.

Syntax

resource-monitor minor resend enable

undo resource-monitor minor resend enable

Default

Resending of minor resource depletion alarms is enabled.

Views

System view

Predefined user roles

network-admin

Usage guidelines

When a resource type enters minor alarm state, the device issues a minor alarm. If the resource type stays in minor alarm state or changes from severe alarm state to minor alarm state, the device identifies whether resending of minor resource depletion alarms is enabled. If the feature is disabled, the device does not issue additional minor alarms. If the feature is enabled, the device resends minor alarms periodically.

The resending period is fixed at 24 hours for a severe alarm and is fixed at 7 * 24 hours for a minor alarm.

Examples

# Enable resending of minor resource depletion alarms.

<Sysname> system-view

[Sysname] resource-monitor minor resend enable

Related commands

display resource-monitor

resource-monitor output

resource-monitor resource

resource-monitor output

Use resource-monitor output to specify destinations for resource depletion alarms.

Use undo resource-monitor output to remove destinations for resource depletion alarms.

Syntax

resource-monitor output { netconf-event | snmp-notification | syslog } *

undo resource-monitor output [ netconf-event | snmp-notification | syslog ] *

Default

Resource depletion alarms are sent to NETCONF, SNMP, and the information center.

Views

System view

Predefined user roles

network-admin

Parameters

netconf-event: Sends resource depletion alarms to the NETCONF feature to encapsulate the alarms in NETCONF events. For more information, see NETCONF in Network Management and Monitoring Configuration Guide.

snmp-notification: Sends resource depletion alarms to the SNMP feature to encapsulate the alarms in SNMP traps and informs. For more information, see SNMP in Network Management and Monitoring Configuration Guide.

syslog: Sends resource depletion alarms to the information center to encapsulate the alarms in log messages. For more information, see information center in Network Management and Monitoring Configuration Guide.

Usage guidelines

If you do not specify any keywords for the undo resource-monitor output command, the command disables resource depletion alarm output.

Examples

# Specify the information center module as the output destination for resource depletion alarms.

<Sysname> system-view

[Sysname] resource-monitor output syslog

Related commands

resource-monitor minor resend enable

resource-monitor resource

resource-monitor resource

Use resource-monitor resource to set resource depletion thresholds.

Use undo resource-monitor resource to disable resource depletion thresholds.

Syntax

resource-monitor resource resource-name slot slot-number cpu cpu-number  by-percent  minor-threshold minor-threshold severe-threshold severe-threshold

undo resource-monitor resource resource-name slot slot-number cpu cpu-number

Default

The default settings vary by resource type. Use the display resource-monitor command to display the resource depletion thresholds.

Views

System view

Predefined user roles

network-admin

Parameters

resource-name: Specifies a resource type by its name. The values for this argument are case insensitive and cannot be abbreviated. Table 3 shows the resource types that can be monitored.

Resource types that can be monitored

| Resource type | Description |
| --- | --- |
| arp | ARP resources. |
| ipv4host | IPv4 host route resources after the UNI mode is enabled. |
| ipv4route | Network routes and IPv4 host route resources not enabled with the UNI mode. |
| ipv6host | IPv6 host route resources after the UNI mode is enabled. |
| ipv6route | Network routes and IPv6 host route resources not enabled with the UNI mode. |
| nd | ND resources. |
| nexthoppool1 | Next-hop pool resources for the underlay network. |

‌

slot slot-number: Specifies an IRF member device by its member ID.

cpu cpu-number: Specifies a CPU by its number.

by-percent: Specifies resource depletion thresholds in percentage.

minor-threshold minor-threshold: Specifies the minor resource depletion threshold. To view the value range, enter a question mark (?) in the place of the minor-threshold argument.

severe-threshold severe-threshold: Specifies the severe resource depletion threshold. To view the value range, enter a question mark (?) in the place of the severe-threshold argument.

Usage guidelines

After you execute this command for a resource type, the device monitors the available amount of the type of resources. The device samples the available amount at intervals, compares the sample with the resource depletion thresholds to identify the resource depletion status, and sends alarms as configured.

Examples

# Set the minor resource depletion threshold to 30% and the severe resource depletion threshold to 10% for ARP entry resources on slot 1.

<Sysname> system-view

[Sysname] resource-monitor resource arp slot 1 cpu 0 by-percent minor-threshold 30 severe-threshold 10

Related commands

display resource-monitor

resource-monitor minor resend enable

resource-monitor output

  - New feature: Sending EAP-Success packets upon successful authorization in 802.1X

    - Sending EAP-Success packets upon successful authorization

      - About this task

The access device can send EAP-Success packets to 802.1X clients when it receives RADIUS Access-Accept packets from the RADIUS server upon successful authentication or authorization.

In the subnet authorization scenario, an 802.1X client must obtain an IP address through DHCP from the authorization subnet for network access after it receives an EAP-Success packet from the access device.

To make sure the 802.1X client can obtain an IP address from the authorization subnet, configure the access device to send EAP-Success packets upon successful authorization.

If the access device sends EAP-Success packets upon successful authentication, the client might send a DHCP request before it receives the authorization information. In this situation, the DHCP request is sent on the initial subnet to which the client is attached. The client will be unable to access the network with the IP address obtained from the initial subnet after the authorization subnet is issued.

      - Restrictions and guidelines

When you configure the device to send EAP-Success packets upon successful authorization, evaluate its impact on authentication service. When a large number of authentication sessions are present, this setting might result in authentication failure because the RADIUS server or access device fails to return EAP-Success packets before the authentication timeout time expires.

      - Procedure

            - Enter system view.

system-view

            - Configure the device to send EAP-Success packets to clients upon successful authorization.

dot1x eap-success post-authorization

By default, the device sends EAP-Success packets to clients upon successful authentication.

    - Command reference

      - dot1x eap-success post-authorization

Use dot1x eap-success post-authorization to configure the device to send EAP-Success packets to clients upon successful authorization.

Use undo dot1x eap-success post-authorization to configure the device to send EAP-Success packets to clients upon successful authentication.

Syntax

dot1x eap-success post-authorization

undo dot1x eap-success post-authorization

Default

The device sends EAP-Success packets to clients upon successful authentication.

Views

System view

Default command level

network-admin

Usage guidelines

Application scenarios

The access device can send EAP-Success packets to 802.1X clients when it receives RADIUS Access-Accept packets from the RADIUS server upon successful authentication or authorization.

In the subnet authorization scenario, an 802.1X client must obtain an IP address through DHCP from the authorization subnet for network access after it receives an EAP-Success packet from the access device.

To make sure the 802.1X client can obtain an IP address from the authorization subnet, configure the access device to send EAP-Success packets upon successful authorization.

If the access device sends EAP-Success packets upon successful authentication, the client might send a DHCP request before it receives the authorization information. In this situation, the DHCP request is sent on the initial subnet to which the client is attached. The client will be unable to access the network with the IP address obtained from the initial subnet after the authorization subnet is issued.

Restrictions and guidelines

When you configure the device to send EAP-Success packets upon successful authorization, evaluate its impact on authentication service. When a large number of authentication sessions are present, this setting might result in authentication failure because the RADIUS server or access device fails to return EAP-Success packets before the authentication timeout time expires.

Examples

# Configure the device to send EAP-Success packets to clients upon successful authorization.

<Sysname> system-view

[Sysname] dot1x eap-success post-authorization

Modified feature: Configuring the padding mode and padding format for the Circuit ID sub-option

Feature change description

As from this release, the dhcp relay information circuit-id command supports the sysname keyword. This keyword enables the device to insert the system name into the Circuit ID suboption.

Command changes

Modified command: dhcp relay information circuit-id

Old syntax

dhcp relay information circuit-id { bas | string circuit-id | { normal | verbose [ node-identifier { mac | sysname | user-defined node-identifier } ] [ interface ] } [ format { ascii | hex } ] }

undo dhcp relay information circuit-id

New syntax

dhcp relay information circuit-id { bas | string circuit-id | sysname | { normal | verbose [ node-identifier { mac | sysname | user-defined node-identifier } ] [ interface ] } [ format { ascii | hex } ] }

undo dhcp relay information circuit-id

Views

Interface view

Change description

Before modification: This command does not support the sysname keyword.

After modification: This command supports the sysname keyword. This keyword enables the device to insert the system name into the Circuit ID suboption. To configure the system name of a device, use the sysname command in system view.

Modified feature: Configuring the padding mode and padding format for the Remote ID sub-option

Feature change description

As from this release, the dhcp relay information remote-id command supports the interface and hex remote-id parameters. The interface keyword enables the device to insert the interface index of the interface that received the DHCP request into the Remote ID suboption. The hex remote-id option enables the device to insert the user-defined hexadecimal string into the Remote ID suboption.

Command changes

Modified command: dhcp relay information remote-id

Old syntax

dhcp relay information remote-id { normal [ format { ascii | hex } ] | string remote-id | sysname }

undo dhcp relay information remote-id

New syntax

dhcp relay information remote-id { interface | hex remote-id | normal [ format { ascii | hex } ] | string remote-id | sysname }

undo dhcp relay information remote-id

Views

Interface view

Change description

Before modification: The interface and hex remote-id parameters are not supported.

After modification: The interface and hex remote-id parameters are not supported. The interface keyword enables the device to insert the interface index of the interface that received the DHCP request into the Remote ID suboption. For example, if the interface that received the DHCP request is GE2/0/1, the device will insert interface index 1 into the Remote ID suboption. The hex remote-id option enables the device to insert the user-defined hexadecimal string into the Remote ID suboption. The user-defined hexadecimal string contains 2 to 256 characters and its length must be even.

Modified feature: Support for specifying a custom hexadecimal string as the content of the Remote ID sub-option

Feature change description

As from this release, you can configure a custom hexadecimal string as the content of the Remote ID sub-option on the DHCP snooping device or the DHCP relay agent.

Command changes

Modified command: dhcp snooping information remote-id

Old syntax

dhcp snooping information remote-id { normal [ format { ascii | hex } ] | [ vlan vlan-id ] { string remote-id | sysname } }

undo dhcp snooping information remote-id [ vlan vlan-id ]

New syntax

dhcp snooping information remote-id { normal [ format { ascii | hex } ] | [ vlan vlan-id ] { hex hex-string | string remote-id | sysname } }

undo dhcp snooping information remote-id [ vlan vlan-id

Views

Layer 2 Ethernet interface view/Layer 2 aggregate interface view

VLAN view

|  | NOTE: VLAN view is supported only in Release 6350 and later. |
| --- | --- |

Change description

Before modification: The device only supports using a custom string as the content of the Remote ID sub-option. The string cannot be hexadecimal.

After modification: The device supports using a custom hexadecimal string as the content of the Remote ID sub-option.

Parameters

hex hex-string: Specifies a hexadecimal string as the content of the Remote ID sub-option. The string length must be an even integer in the range of 2 to 256.

Modified command: dhcp relay information remote-id

Old syntax

dhcp relay information remote-id { normal [ format { ascii | hex } ] | string remote-id | sysname }

undo dhcp relay information remote-id

New syntax

dhcp relay information remote-id { hex remote-id | normal [ format { ascii | hex } ] | string remote-id | sysname }

undo dhcp relay information remote-id

Views

Interface view

Change description

Before modification: The device only supports using a custom string as the content of the Remote ID sub-option. The string cannot be hexadecimal.

After modification: The device supports using a custom hexadecimal string as the content of the Remote ID sub-option.

Parameters

hex hex-string: Specifies a hexadecimal string as the content of the Remote ID sub-option. The string length must be an even integer in the range of 2 to 256.

Modified feature: Support for specifying a custom ASCII string as the client ID of a static binding

Feature change description

As from this release, when you configure a static binding in a DHCP address pool, you can specify a custom ASCII string as the client ID of that static binding.

Modified command: static-bind

Old syntax

static-bind ip-address ip-address [ mask-length | mask mask ] { client-identifier client-identifier | hardware-address hardware-address [ ethernet | token-ring ] }

undo static-bind ip-address ip-address

New syntax

static-bind ip-address ip-address [ mask-length | mask mask ] { client-identifier { ascii ascii-string | hex hex-string } | hardware-address hardware-address [ ethernet | token-ring ] }

undo static-bind ip-address ip-address

Views

DHCP address pool view

Change description

Before modification: You can use the client-identifier client-identifier option to specify a client ID and the client ID is a hexadecimal string.

After modification: The client-identifier argument is replaced by the hex hex-string and ascii ascii-string options. The hex hex-string option specifies a hexadecimal client ID and the ascii ascii-string option specifies an ASCII client ID.

Parameters

hex hex-string: Specifies a hexadecimal string of 4 to 254 characters as the client ID. The string can contain only hexadecimal numbers and hyphen (-), in the format of H-H-H…. The last H can be a two-digit or four-digit hexadecimal number while the other Hs must be all four-digit hexadecimal numbers. For example, aabb-cccc-dd is valid, and aabb-c-dddd and aabb-cc-dddd are invalid.

ascii ascii-string: Specifies an ASCII string of 1 to 127 characters as the client ID.

Release 6352P02

This release has no feature changes.