HPE 5120v3-CMW710-R6367 Release Notes

The information in this document is subject to change without notice. © Copyright 2010, 2024 Hewlett Packard Enterprise Development LP

Contents

Introduction	1

Version information	1

Version number	1

Version history	1

Hardware and software compatibility matrix	1

Upgrade restrictions and guidelines	3

Hardware feature updates	3

5120v3-CMW710-R6367	3

5120v3-CMW710-R6352P02	3

Software feature and command updates	3

MIB updates	3

Operation changes	3

Operation changes in R6367	3

Operation changes in R6352P02	3

Restrictions and cautions	4

Restrictions	4

Cautions	4

Open problems and workarounds	4

List of resolved problems	4

Resolved problems in R6367	4

Resolved problems in R6352P02	7

Support and other resources	7

Accessing Hewlett Packard Enterprise Support	7

Documents	7

Related documents	8

Documentation feedback	8

Appendix A Feature list	9

Hardware features	9

Software features	10

Appendix B Upgrading software	13

System software file types	13

System startup process	13

Upgrade methods	14

Preparing for the upgrade	15

Verifying device status	15

Setting up the upgrade environment	15

Upgrading from the CLI	16

Preparing for the upgrade	16

Downloading software images to the master switch	17

Upgrading from the Boot menu	21

Prerequisites	21

Accessing the Boot menu	22

Accessing the extended Boot menu	23

Upgrading Comware images from the Boot menu	24

Upgrading Boot ROM from the Boot menu	32

Managing files from the Boot menu	39

Handling software upgrade failures	42

List of tables

Table 1 Version history	1

Table 2 Hardware and software compatibility matrix	1

Table 3 MIB updates	3

Table 4 5120v3 series hardware features	9

Table 5 Software features of the 5120v3 series	10

Table 6 Minimum free storage space requirements	21

Table 7 Shortcut keys	22

Table 8 Extended Boot ROM menu options	23

Table 9 EXTENDED ASSISTANT menu options	24

Table 10 TFTP parameter description	25

Table 11 FTP parameter description	27

Table 12 TFTP parameter description	33

Table 13 FTP parameter description	34

Introduction

This document describes the features, restrictions and guidelines, open problems, and workarounds for version HPE 5120v3-CMW710-R6367. Before you use this version on a live network, back up the configuration and test the version to avoid software upgrade affecting your live network.

Use this document in conjunction with HPE 5120v3-CMW710-R6367 Release Notes (Software Feature Changes) and the documents listed in "Related documents."

Version information

Version number

HPE Comware Software, Version 7.1.070, Release 6367

Note: You can see the version number with the command display version in any view. Please see Note.

Version history

|  | IMPORTANT: The software feature changes listed in the version history table for each version are not complete. To obtain complete information about all software feature changes in each version, see the Software Feature Changes document for this release notes. |
| --- | --- |

Version history

| Version number | Last version | Release date | Release type | Remarks |
| --- | --- | --- | --- | --- |
| 5120v3-CMW710-R6367 | 5120v3-CMW710-R6352P02 | 2024-09-12 | Release version | Fixed bugs |
| 5120v3-CMW710-R6352P02 | First release | 2023-05-18 | Release version | First release |

Hardware and software compatibility matrix

|  | CAUTION: To avoid an upgrade failure, use Table 2 to verify the hardware and software compatibility before performing an upgrade. |
| --- | --- |

Hardware and software compatibility matrix

| Item | Specifications |
| --- | --- |
| Product family | 5120v3 Series |
| Hardware platform | HPE NW CW 5120v3 8G PoE+ 2 SFP Sw S0F79A |
| Memory | 512M |
| Flash | 256M |
| Boot ROM version | Version 158 or higher (Note: Use the display version command in any view to view the version information. Please see Note②) |
| Software images and their MD5 checksums | 5120v3-CMW710-R6367.ipe(See the MD5 file) |
| IMC version | ADNET-FCAPS (E0709) AOM (E0706P01) iMC BIMS 7.3 (E0506H01) iMC EAD7.3 (E0611P10) iMC QoSM 7.3 (E0505P01) iMC EIA 7.3 (E0611P13) iMC PLAT 7.3 (E0705P12) iMC NTA 7.3 (E0707L06) iMC SHM 7.3 (E0707L06) |
| INode version | iNode PC 7.3 (E0585) |
| Remarks | N/A |

Display the system software and Boot ROM versions of 5120v3

<HPE>display version

HPE Comware Software, Version 7.1.070, Release 6367   ------Note

Copyright (c) 2010-2024 Hewlett Packard Enterprise Development LP

HPE NW CW 5120v3 8G PoE+ 2 SFP Sw uptime is 0 weeks, 0 days, 0 hours, 2 minutes

Last reboot reason : Cold reboot

Boot image: flash:/5120v3-cmw710-boot-r6367.bin

Boot image version: 7.1.070, Release 6367

Compiled Aug 08 2024 16:00:00

System image: flash:/5120v3-cmw710-system-r6367.bin

System image version: 7.1.070, Release 6367

Compiled Aug 08 2024 16:00:00

Slot 1:

Uptime is 0 weeks,0 days,0 hours,2 minutes

NW CW 5120v3 8G PoE+ 2 SFP Sw with 1 Processor

BOARD TYPE:         NW CW 5120v3 8G PoE+ 2 SFP Sw

DRAM:               512M bytes

FLASH:              256M bytes

PCB 1 Version:      VER.A

Bootrom Version:    158              ------Note

CPLD 1 Version:     001

Release Version:    HPE NW CW 5120v3 8G PoE+ 2 SFP Sw S0F79A-6367

Patch Version  :    None

Reboot Cause  :     ColdReboot

[SubSlot 0] 8GE+2SFP

Upgrade restrictions and guidelines

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

Hardware feature updates

5120v3-CMW710-R6367

None.

5120v3-CMW710-R6352P02

First release.

Software feature and command updates

For more information about the software feature and command update history, see HPE 5120v3-CMW710-R6367 Release Notes (Software Feature Changes).

MIB updates

MIB updates

| Item | MIB file | Module | Description |
| --- | --- | --- | --- |
| 5120v3-CMW710-R6367 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5120v3-CMW710-R6352P02 |  |  |  |
| New | First release | First release | First release |
| Modified | First release | First release | First release |

Operation changes

Operation changes in R6367

support ipv6 ready.

Operation changes in R6352P02

First release.

Restrictions and cautions

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

When you use this version of software, make sure you fully understand the restrictions and cautions described in this section.

Restrictions

Release 6367 must use BootROM 158 or a later version.

If data packets are assigned to queue 7 and the scheduling algorithm is SP, all packets sent from the CPU are affected.

To avoid false alarms, make sure the statistics collection and comparison interval for CRC error packets configured in the ifmonitor crc-error command is greater than 15 seconds.

Not Support SmartMC.

Cautions

None.

Open problems and workarounds

202409061705

- Symptom: On the Web interface, the month and the dropdown icon overlap.

Condition: This symptom occurs when you set the time on the Web interface.

Workaround: None.

202408141649

- Symptom: The value ranges for the secure MAC aging timer are different on the Web interface and at the CLI.

Condition: This symptom occurs if you set the secure MAC aging timer in seconds at the CLI.

Workaround: Set the secure MAC aging timer in minutes at the CLI.

List of resolved problems

Resolved problems in R6367

202408151048

Symptom: No MAC address entry is added and no trap about deleting a node is reported when a MAC address moves.

Condition: This symptom occurs if a MAC address moves from the first port to the second port.

202407160651

- Symptom: After the startup, a user cannot log in with the password, but can log in with an empty password and is required to change the password.

Condition: This symptom might occur if the device finishes the version upgrade, saves the configuration again, and restarts after the password control feature is enabled. The restart recovers the configuration through the configuration file by deleting the .mdb file or a version update. In addition, the lauthd process restarts, or the device restarts after the save operation.

202407090199

- Symptom: The poe max-power command cannot be configured after the device power cycles.

Condition: This symptom might occur if the device power cycles.

202403180789

- Symptom: PTP time synchronization fails if VLANs are inconsistent between the master and member clocks.

Condition: This symptom might occur if VLANs are inconsistent between the master and member clocks in PTP.

202408120285

- Symptom: The ifmonitor command fails to be configured on a multi-rate interface.

Condition: This symptom might occur if you configure the ifmonitor command on a multi-rate interface.

202404110742

Symptom: The device might report MIB node information about power supply failure if SNMP is enabled on the device.

Condition: This symptom might occur if SNMP is enabled on the device.

202307030467

Symptom: The VLAN deployed by iMC fails to be configured.

Condition: This symptom occurs if a VLAN with member ports already exists on the device and then the VLAN is deployed from iMC.

202312041375

Symptom: The firmware is lost after the device is power cycled.

Condition: This symptom occurs if the device is power cycled.

202311240127

Symptom: Temperature alarms are mistakenly reported.

Condition: None.

202311240114

Symptom: PTP packets cannot be transparently transmitted.

Condition: None.

202311101350

Symptom: The management port flaps.

Condition: This symptom occurs if the management port runs for a long time.

202311240102

Symptom: The subordinate device on an IRF fabric does not generate an alarm for the hh3cStackPortLinkStatusChange node when the master device is powered off.

Condition: This symptom occurs when the master device is powered off.

202307211617

Symptom: Failed to enable the HTTPS service by using the ip https enable command.

Condition: This symptom occurs when the device has configured with the restful http enable or restful https enable command.

202309071883

Symptom: When you conduct cable detection on certain Ethernet ports of the device, it prompts that this feature is not supported.

Condition: This symptom might occur if you use the virtual-cable-test command to conduct cable detection on Ethernet ports.

202309071086

Symptom: The device restarts unexpectedly.

Condition: This symptom might occur if the peer port continues to emit and extinguish light after the local fiber port is shut down.

202308161212

Symptom: The port_block ACL is not deleted, and ARP packets cannot be forwarded.

Condition: This symptom occurs if multiple Layer 2 protocols set an interface to the blocked state at the same time and then recover the interface to the forwarding state.

202307110941

Symptom: If you apply MQC with remark local-precedence behavior to multiple ports (on different forwarding chips) on a dual-chip device, undoing MQC on one port also invalidates the MQC applied to ports on the other chip.

Condition: This symptom might occur after you perform the following:

Apply MQC with remark local-precedence behavior to multiple ports (on different forwarding chips) on a dual-chip device.

Undo MQC on one port.

202308220092

Symptom: 100-Mbps transceiver modules cannot come up.

Condition: This symptom occurs if a 1000-Mbps transceiver module is installed in a fiber port on the front panel.

202307140367

Symptom: The display rules for IPv6 addresses are inconsistent. Some IPv6 addresses support abbreviation, while others don't.

Condition: This symptom occurs if the device is configured with IPv6 addresses.

202307051264

Symptom: The device does not display logs for adding MAC address entries and displays logs only for deleting MAC address entries.

Condition: This symptom occurs if you configure port security settings on a port and connect the port to the peer end.

202306160696

Symptom: The display qos-acl resource command shows that some ACL resources are not released after the many-to-one VLAN mapping configuration is deleted or the interface is shut down.

Condition: This symptom occurs if the following operations are performed:

Configure many-to-one VLAN mapping on an interface.

After traffic with the original VLAN tag is present on the interface for a period of time, delete the many-to-one VLAN mapping configuration or shut down the interface.

202306071059

Symptom: The configuration cannot be saved when factory defaults are restored.

Condition: This symptom occurs when you restore factory defaults.

202305112068

Symptom: When you execute the display mac-address command to display MAC address entries, the Aging field for a MAC address entry that can age out displays N instead of Y.

Condition: This symptom occurs if the user of the MAC address in the entry comes online through MAC authentication.

Resolved problems in R6352P02

First release.

Support and other resources

Accessing Hewlett Packard Enterprise Support

For live assistance, go to the Contact Hewlett Packard Enterprise Worldwide website:

www.hpe.com/assistance

To access documentation and support services, go to the Hewlett Packard Enterprise Support Center website:

www.hpe.com/support/hpesc

Information to collect:

Technical support registration number (if applicable).

Product name, model or version, and serial number.

Operating system name and version.

Firmware version.

Error messages.

Product-specific reports and logs.

Add-on products or components.

Third-party products or components.

Documents

To find related documents, see the Hewlett Packard Enterprise Support Center website at http://www.hpe.com/support/hpesc.

Enter your product name or number and click Go. If necessary, select your product from the resulting list.

For a complete list of acronyms and their definitions, see HPE FlexNetwork technology acronyms.

Related documents

The following documents provide related information:

HPE Networking Comware  5120v3 Switch Series Configuration Guides-R63xx

HPE Networking Comware  5120v3 Switch Series Command References-R63xx

HPE Networking Comware  5120v3 Switch Series Installation Guide

Documentation feedback

Hewlett Packard Enterprise is committed to providing documentation that meets your needs. To help us improve the documentation, send any errors, suggestions, or comments to Documentation Feedback (docsfeedback@hpe.com). When submitting your feedback, include the document title, part number, edition, and publication date located on the front cover of the document. For online help content, include the product name, product version, help edition, and publication date located on the legal notices page.

Feature list

Hardware features

5120v3 series hardware features

| Item | NW CW 5120v3 8G PoE+ 2 SFP Sw |
| --- | --- |
| Dimensions (H × W × D) | 43.6 × 330 × 230 mm (1.72 ×12.99 × 9.06 in) |
| Weight | ≤ 3 kg (6.61 lb) |
| Console port | 1 × serial console port |
| 10/100/1000BASE-T autosensing Ethernet port | 8 |
| SFP port | 2 |
| Input voltage | - Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz |
| Maximum PoE power per port | - 30 W |
| Total PoE power | - 125 W |
| Minimum power consumption | 10W |
| Maximum power consumption | 155W |
| Chassis leakage current compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |
| Melting current of power module fuse | - 6.3 A/250 V |
| Cooling system | Natural cooling without fan trays |
| Operating temperature | -5ºC～45ºC (23°F to 113°F) |
| Operating humidity | 5% to 95%, noncondensing |
| Fire resistance compliance | UL60950-1/EN60950-1/IEC60950-1/GB4943 |

Software features

Software features of the 5120v3 series

| Feature | 5120v3 series switch |
| --- | --- |
| IRF | - Ring topology - Daisy chain topology - LACP MAD - ARP MAD |
| Link aggregation | - Aggregation of 1-GE ports - Static link aggregation - Dynamic link aggregation - Inter-device aggregation - A maximum of 14 aggregation groups on a device - A maximum of 124 inter-device aggregation groups - A maximum of 8 ports for each aggregation group |
| Flow control | - IEEE 802.3x flow control |
| Jumbo Frame | - Supports maximum frame size of 10000 |
| MAC address table | - 16K MAC addresses - 1K static MAC addresses - Blackhole MAC addresses - MAC address learning limit on a port |
| VLAN | - Port-based VLANs (4094 VLANs) - QinQ - VLAN mapping |
| ARP | - 1K entries - 512 static entries - Gratuitous ARP - ARP black hole - ARP detection (based on DHCP snooping entries/802.1X security entries/static IP-to-MAC bindings) - ARP source suppression |
| ND | - 240 entries - 128 static entries |
| VLAN virtual interface | - 32 |
| DHCP | - DHCP client - DHCP snooping - DHCP relay - DHCP server - DHCPv6 Server - DHCPv6 relay - DHCPv6 snooping |
| UDP Helper | - UDP Helper |
| DNS | - Static DNS - Dynamic DNS - IPv4 and IPv6 DNS |
| unicast route | - IPv4 and IPv6 static routes - RIP/RIPng - OSPF/OSPFv3 - Routing policies - Policy-based routing - IPv6 policy-based routing |
| Multicast | - IGMP snooping - PIM Snooping - MLD snooping - IPv4 and IPv6 multicast VLAN - IPv6 PIM Snooping |
| Broadcast/multicast/unicast storm control | - Storm control based on port rate percentage - PPS-based storm control - Bps-based storm control |
| MSTP | - STP/RSTP/MSTP protocol - STP Root Guard - BPDU Guard - 128 PVST instances |
| QoS/ACL | - Remarking of 802.1p and DSCP priorities - Packet filtering at L2 (Layer 2) through L4 (Layer 4) - Eight output queues for each port - SP/WRR/SP+WRR queue scheduling algorithms - Port-based rate limiting - Flow-based redirection - Time range |
| Mirroring | - Stream mirroring - Port mirroring |
| Security | - Hierarchical management and password protection of users - AAA authentication - RADIUS authentication - HWTACACS - LDAP - SSH 2.0 - Port isolation - 802.1X - Portal - Port security - MAC-address-based authentication - IP Source Guard - HTTPS - PKI - IPsec - EAD - Public key management |
| 802.1X | - Up to 2K users - Port-based and MAC address-based authentication - Trunk port authentication - Dynamic 802.1X-based QoS/ACL/VLAN assignment |
| Loading and upgrading | - Loading and upgrading through XModem protocol - Loading and upgrading through FTP - Loading and upgrading through the trivial file transfer protocol (TFTP) |
| Management | - Configuration at the command line interface - Remote configuration through Telnet - Configuration through Console port - Simple network management protocol (SNMP) - Remote Monitoring(RMON) - IMC NMS - System log - Hierarchical alarms - NTP - Power supply alarm function - Fan and temperature alarms |
| Maintenance | - Debugging information output - Ping and Tracert - Remote maintenance through Telnet - NQA - 802.1ag - 802.3ah - DLDP - Virtual Cable Test |

Upgrading software

This chapter describes types of software used on the switch and how to upgrade software while the switch is operating normally or when the switch cannot correctly start up.

System software file types

Software required for starting up the switch includes:

Boot ROM image—A .bin file that comprises a basic section and an extended section. The basic section is the minimum code that bootstraps the system. The extended section enables hardware initialization and provides system management menus. You can use these menus to load software and the startup configuration file or manage files when the switch cannot correctly start up.

Software images—Includes boot images and system images.

Boot image—A .bin file that contains the operating system kernel. It provides process management, memory management, file system management, and the emergency shell.

System image—A .bin file that contains the minimum modules required for device operation and some basic features, including device management, interface management, configuration management, and routing management.

The software images that have been loaded are called “current software images.” The software images specified to load at next startup are called “startup software images.”

These images might be released separately or as a whole in one .ipe package file. If an .ipe file is used, the system automatically decompresses the file, loads the .bin boot and system images in the file and sets them as startup software images. Typically, the Boot ROM and software images for this switch series are released in an .ipe file named main.ipe.

|  | NOTE: Boot ROM images are not released along with the boot images and system images. To get a version of Boot ROM image, contact the H3C technical support. |
| --- | --- |

System startup process

Upon power-on, the Boot ROM image runs to initialize hardware and then the software images run to start up the entire system, as shown in Figure 1.

System startup process

Upgrade methods

You can upgrade system software by using one of the following methods:

| Upgrading method | Software types | Remarks |
| --- | --- | --- |
| Upgrading from the CLI | - Boot ROM image - Software images | - You must reboot the switch to complete the upgrade. - This method can interrupt ongoing network services. |
| Upgrading from the Boot menu | - Boot ROM image - Software images | Use this method when the switch cannot correctly start up. CAUTION: Upgrading an IRF fabric from the CLI instead of the Boot menu. The Boot menu method increases the service downtime, because it requires that you upgrade the member switches one by one. |

The output in this document is for illustration only and might vary with software releases. This document uses boot.bin and system.bin to represent boot and system image names. The actual software image name format is chassis-model_Comware-version_image-type_release, for example, 5120v3-CMW710-BOOT-R6367.bin and 5120v3-CMW710-SYSM-R6367.bin.

Preparing for the upgrade

Verifying device status

Verify that the system state, redundancy state, and state of each slot are stable.

<Sysname> display system stable state

System state     : Stable

Redundancy state : No redundance

Slot    CPU    Role       State

1       0      Active     Stable

If the device is unstable, use the following commands to troubleshoot the issue:

Use the display device command to verify that the device is operating correctly.

Use the display ha service-group command to verify that bulk backup has been finished for all modules.

Use the display system internal process state command in probe view to verify that services are running correctly.

If a slot persists in unstable state or there are other unrecoverable issues, contact the technical support.

Setting up the upgrade environment

Before you upgrade system software, complete the following tasks:

Set up the upgrade environment as shown in Figure 2.

Configure routes to make sure that the router and the file server can reach each other.

Run a TFTP or FTP server on the file server.

Log in to the CLI of the router through the console port.

Copy the upgrade file to the file server and correctly set the working directory on the TFTP or FTP server.

Make sure that the upgrade has minimal impact on the network services. During the upgrade, the router cannot provide any services.

Setting up the upgrade environment

Upgrading from the CLI

This section uses a two-member IRF fabric as an example to describe how to upgrade software from the CLI. If you have more than two subordinate switches, repeat the steps for the subordinate switch to upgrade their software. If you are upgrading a standalone switch, ignore the steps for upgrading the subordinate switch. For more information about setting up and configuring an IRF fabric, see the installation guide and IRF configuration guide for the HPE 5120v3 switch series.

Preparing for the upgrade

Before you upgrade software, complete the following tasks:

Log in to the IRF fabric through Telnet or the console port. (Details not shown.)

Identify the number of IRF members, each member switch's role, and IRF member ID.

<Sysname> display irf

MemberID   Role   Priority  CPU-Mac         Description

*+1      Master  5         0023-8927-afdc  ---

2      Standby 1         0023-8927-af43  ---

--------------------------------------------------

* indicates the device is the master.

+ indicates the device through which the user logs in.

The Bridge MAC of the IRF is: 0023-8927-afdb

Auto upgrade                : no

Mac persistent              : 6 min

Domain ID                   : 0

Verify that each IRF member switch has sufficient storage space for the upgrade images.

|  | IMPORTANT: Each IRF member switch must have free storage space that is at least two times the size of the upgrade image file. |
| --- | --- |

# Identify the free flash space of the master switch.

<Sysname> dir

Directory of flash:

0 drw-           - Jan 01 2013 00:17:27   diagfile

1 drw-           - Jan 01 2013 00:17:28   license

2 drw-           - Jan 01 2013 00:17:27   logfile

3 drw-           - Jan 01 2013 00:17:41   pki

4 -rw-     6161408 Jan 01 2013 00:17:27   boot.bin

5 -rw-    50729984 Jan 01 2013 00:17:27   system.bin

6 drw-           - Jan 01 2013 00:17:27   seclog

7 drw-           - Jan 01 2013 00:17:49   versionInfo

251904 KB total (192736 KB free)

# Identify the free flash space of each subordinate switch, for example, switch 2.

<Sysname> dir slot2#flash:/

Directory of slot2#flash:/

0 drw-           - Jan 01 2013 00:17:27   diagfile

1 drw-           - Jan 01 2013 00:17:28   license

2 drw-           - Jan 01 2013 00:17:27   logfile

3 drw-           - Jan 01 2013 00:17:41   pki

4 -rw-     6161408 Jan 01 2013 00:17:27   boot.bin

5 -rw-    50729984 Jan 01 2013 00:17:27   system.bin

6 drw-           - Jan 01 2013 00:17:27   seclog

7 drw-           - Jan 01 2013 00:17:49   versionInfo

251904 KB total (192736 KB free)

Compare the free flash space of each member switch with the size of the software file to load. If the space is sufficient, start the upgrade process. If not, go to the next step.

Delete unused files in the flash memory to free space:

|  | CAUTION: - To avoid data loss, do not delete the current configuration file. For information about the current configuration file, use the display startup command. - The delete /unreserved file-url command deletes a file permanently and the action cannot be undone. - The delete file-url command moves a file to the recycle bin and the file still occupies storage space. To free the storage space, first execute the undelete command to restore the file, and then execute the delete /unreserved file-url command. |
| --- | --- |

# Delete unused files from the flash memory of the master switch.

<Sysname> delete /unreserved flash:/backup.bin

The file cannot be restored. Delete flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file flash:/backup.bin...Done.

# Delete unused files from the flash memory of the subordinate switch.

<Sysname> delete /unreserved slot2#flash:/backup.bin

The file cannot be restored. Delete slot2#flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file slot2#flash:/backup.bin...Done.

Downloading software images to the master switch

Before you start upgrading software images packages, make sure you have downloaded the upgrading software files to the root directory in flash memory. This section describes downloading an .ipe software file as an example.

The following are ways to download, upload, or copy files to the master switch:

FTP download from a server

FTP upload from a client

TFTP download from a server

Prerequisites

If FTP or TFTP is used, the IRF fabric and the PC working as the FTP/TFTP server or FTP client can reach each other.

Prepare the FTP server or TFTP server program yourself for the PC. The switch series does not come with these software programs.

FTP download from a server

You can use the switch as an FTP client to download files from an FTP server.

To download a file from an FTP server, for example, the server at 10.10.110.1:

Run an FTP server program on the server, configure an FTP username and password, specify the working directory and copy the file, for example, newest.ipe, to the directory.

Execute the ftp command in user view on the IRF fabric to access the FTP server.

<Sysname> ftp 10.10.110.1

Press CTRL+C to abort

Connected to 10.10.110.1(10.10.110.1).

220 FTP service ready.

User (10.10.110.1:(none)):username

331 Password required for username.

Password:

230 User logged in.

Enable the binary transfer mode.

ftp> binary

200 Type is Image (Binary)

Execute the get command in FTP client view to download the file from the FTP server.

ftp> get newest.ipe

227 Entering Passive Mode (10,10,110,1,17,97).

125 BINARY mode data connection already open, transfer starting for /newest.ipe

226 Transfer complete.

32133120 bytes received in 35 seconds (896. 0 kbyte/s)

ftp> bye

221 Server closing.

FTP upload from a client

You can use the IRF fabric as an FTP server and upload files from a client to the IRF fabric.

To FTP upload a file from a client:

On the IRF fabric:

Enable FTP server.

<Sysname> system-view

[Sysname] ftp server enable

Configure a local FTP user account:

# Create the user account.

[Sysname] local-user abc

# Set its password and specify the FTP service.

[Sysname-luser-manage-abc] password simple pwd

[Sysname-luser-manage-abc] service-type ftp

# Assign the network-admin user role to the user account for uploading file to the working directory of the server.

[Sysname-luser-manage-abc] authorization-attribute user-role network-admin

[Sysname-luser-manage-abc] quit

[Sysname] quit

On the PC:

Log in to the IRF fabric (the FTP server) in FTP mode.

c:\> ftp 1.1.1.1

Connected to 1.1.1.1.

220 FTP service ready.

User(1.1.1.1:(none)):abc

331 Password required for abc.

Password:

230 User logged in.

Enable the binary file transfer mode.

ftp> binary

200 TYPE is now 8-bit binary.

Upload the file (for example, newest.ipe) to the root directory of the flash memory on the master switch.

ftp> put newest.ipe

200 PORT command successful

150 Connecting to port 10002

226 File successfully transferred

ftp: 32133120 bytes sent in 64.58 secs (497.60 Kbytes/sec).

TFTP download from a server

To download a file from a TFTP server, for example, the server at 10.10.110.1:

Run a TFTP server program on the server, specify the working directory, and copy the file, for example, newest.ipe, to the directory.

On the IRF fabric, execute the tftp command in user view to download the file to the root directory of the flash memory on the master switch.

<Sysname> tftp 10.10.110.1 get newest.ipe

Press CTRL+C to abort.

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

100 30.6M    0 30.6M    0     0   143k      0 --:--:--  0:03:38 --:--:--  142k

Upgrading the software images

To upgrade the software images:

Specify the upgrade image file (newest.ipe in this example) used at the next startup for the master switch, and assign the M attribute to the boot and system images in the file.

<Sysname> boot-loader file flash:/newest.ipe slot 1 main

Verifying the file flash:/newest.ipe on slot 1......Done.

Images in IPE:

boot.bin

system.bin

This command will set the main startup software images. Continue? [Y/N]:y

Add images to slot 1.

Decompressing file boot.bin to flash:/boot.bin....................Done.

Decompressing file system.bin to flash:/system.bin................Done.

Verifying the file flash:/boot.bin on slot 1...Done.

Verifying the file flash:/system.bin on slot 1............Done.

The images that have passed all examinations will be used as the main startup software images at the next reboot on slot 1.

Specify the upgrade image file as the main startup image file for each subordinate switch. This example uses IRF member 2. (The subordinate switches will automatically copy the file to the root directory of their flash memories.)

<Sysname> boot-loader file flash:/newest.ipe slot 2 main

Verifying the file flash:/newest.ipe on slot 2......Done.

Images in IPE:

boot.bin

system.bin

This command will set the main startup software images. Continue? [Y/N]:y

Add images to slot 2.

Decompressing file boot.bin to flash:/boot.bin....................Done.

Decompressing file system.bin to flash:/system.bin................Done.

Verifying the file flash:/boot.bin on slot 2...Done.

Verifying the file flash:/system.bin on slot 2............Done.

The images that have passed all examinations will be used as the main startup software images at the next reboot on slot 2.

Enable the software auto-update function.

<Sysname> system-view

[Sysname] irf auto-update enable

[Sysname] quit

This function checks the software versions of member switches for inconsistency with the master switch. If a subordinate switch is using a different software version than the master, the function propagates the current software images of the master to the subordinate as main startup images. The function prevents software version inconsistency from causing the IRF setup failure.

Save the current configuration in any view to prevent data loss.

<Sysname> save

The current configuration will be written to the device. Are you sure? [Y/N]:y

Please input the file name(*.cfg)[flash:/startup.cfg]

(To leave the existing filename unchanged, press the enter key):

flash:/startup.cfg exists, overwrite? [Y/N]:y

Validating file. Please wait.................

Saved the current configuration to mainboard device successfully.

Slot 2:

Save next configuration file successfully.

Reboot the IRF fabric to complete the upgrade.

<Sysname> reboot

Start to check configuration with next startup configuration file, please wait.

........DONE!

This command will reboot the device. Continue? [Y/N]:y

Now rebooting, please wait...

The system automatically loads the .bin boot and system images in the .ipe file and sets them as the startup software images.

Execute the display version command in any view to verify that the current main software images have been updated (details not shown).

|  | NOTE: The system automatically checks the compatibility of the Boot ROM image and the boot and system images during the reboot. If you are prompted that the Boot ROM image in the upgrade image file is different than the current Boot ROM image, upgrade both the basic and extended sections of the Boot ROM image for compatibility. If you choose to not upgrade the Boot ROM image, the system will ask for an upgrade at the next reboot performed by powering on the switch or rebooting from the CLI (promptly or as scheduled). If you fail to make any choice in the required time, the system upgrades the entire Boot ROM image. |
| --- | --- |

Upgrading from the Boot menu

In this approach, you must access the Boot menu of each member switch to upgrade their software one by one. If you are upgrading software images for an IRF fabric, using the CLI is a better choice.

|  | TIP: Upgrading through the Ethernet port is faster than through the console port. |
| --- | --- |

Prerequisites

Make sure the prerequisites are met before you start upgrading software from the Boot menu.

Setting up the upgrade environment

Use a console cable to connect the console terminal (for example, a PC) to the console port on the switch.

Connect the Ethernet port on the switch to the file server.

|  | NOTE: The file server and the configuration terminal can be co-located. |
| --- | --- |

Run a terminal emulator program on the console terminal and set the following terminal settings:

Bits per second—9,600

Data bits—8

Parity—None

Stop bits—1

Flow control—None

Emulation—VT100

Preparing for the TFTP or FTP transfer

To use TFTP or FTP:

Run a TFTP or FTP server program on the file server or the console terminal.

Copy the upgrade file to the file server.

Correctly set the working directory on the TFTP or FTP server.

Make sure the file server and the switch can reach each other.

Verifying that sufficient storage space is available

|  | IMPORTANT: For the switch to start up correctly, do not delete the main startup software images when you free storage space before upgrading Boot ROM. On the Boot menu, the main startup software images are marked with an asterisk (*). |
| --- | --- |

When you upgrade software, make sure each member switch has sufficient free storage space for the upgrade file, as shown in Table 6.

Minimum free storage space requirements

| Upgraded images | Minimum free storage space requirements |
| --- | --- |
| Comware images | Two times the size of the Comware upgrade package file. |
| Boot ROM | Same size as the Boot ROM upgrade image file. |

If no sufficient space is available, delete unused files as described in “Managing files from the Boot menu.”

Scheduling the upgrade time

During the upgrade, the switch cannot provide any services. You must make sure the upgrade has a minimal impact on the network services.

Accessing the Boot menu

Starting......

Press Ctrl+D to access BASIC BOOT MENU

Booting Normal Extend BootWare....

********************************************************************************

*                                                                              *

*           HPE NW CW 5120v3 8G PoE+ 2 SFP Sw BOOTROM, Version 158             *

*                                                                              *

********************************************************************************

Copyright (c) 2010-2024 Hewlett Packard Enterprise Development LP

Creation Date       : Mar 13 2023, 17:35:17

CPU Clock Speed     : 800MHz

Memory Size         : 512MB

Flash Size          : 256MB

CPLD Version        : 001

PCB Version         : Ver.A

Mac Address         : aa1122334455

Press Ctrl+B to access EXTENDED BOOT MENU...1

Press one of the shortcut key combinations at prompt.

Shortcut keys

| Shortcut keys | Prompt message | Function | Remarks |
| --- | --- | --- | --- |
| Ctrl+B | Press Ctrl+B to enter Extended Boot menu... | Accesses the extended Boot menu. | Press the keys within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the message appears. You can upgrade and manage system software and Boot ROM from this menu. |

Accessing the extended Boot menu

Press Ctrl+B within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the "Press Ctrl-B to enter Extended Boot menu..." prompt message appears. If you fail to do this, the system starts decompressing the system software.

Alternatively, you can enter 4 in the basic Boot menu to access the extended Boot menu.

The "Password recovery capability is enabled." or "Password recovery capability is disabled." message appears, followed by the extended Boot menu. Availability of some menu options depends on the state of password recovery capability (see Table 8). For more information about password recovery capability, see Fundamentals Configuration Guide in HPE Networking Comware  5120v3 Switch Series Configuration Guides-R63xx.

Password recovery capability is enabled.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8):

Extended Boot ROM menu options

| Option | Tasks |
| --- | --- |
| 1. Download image to flash | Download a software image file to the flash. |
| 2. Select image to boot | - Specify the main and backup software image file for the next startup. - Specify the main and backup configuration files for the next startup. This task can be performed only if password recovery capability is enabled. |
| 3. Display all files in flash | Display files on the flash. |
| 4. Delete file from flash | Delete files to free storage space. |
| 5. Restore to factory default configuration | Delete the current next-startup configuration files and restore the factory-default configuration. This option is available only if password recovery capability is disabled. |
| 6. Enter BootRom upgrade menu | Access the Boot ROM upgrade menu. |
| 7. Skip current system configuration | Start the switch without loading any configuration file. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| 8. Set switch startup mode | Set the startup mode to fast startup mode or full startup mode. |
| 0. Reboot | Reboot the switch. |
| Ctrl+F: Format file system | Format the current storage medium. |
| Ctrl+P: Change authentication for console login | Skip the authentication for console login. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| Ctrl+R: Download image to SDRAM and run | Download a system software image and start the switch with the image. This option is available only if password recovery capability is enabled. |
| Ctrl+Z: Access EXTENDED ASSISTANT MENU | Access the EXTENDED ASSISTANT MENU. For options in the menu, see Table 9. |
| Ctrl+Y: Change Work Mode | Change Work Mode. |
| Ctrl+C: Display Copyright | Display the copyright statement. |

EXTENDED ASSISTANT menu options

| Option | Task |
| --- | --- |
| 1. Display Memory | Display data in the memory. |
| 2. Search Memory | Search the memory for a specific data segment. |
| 0. Return to boot menu | Return to the extended Boot ROM menu. |

Upgrading Comware images from the Boot menu

You can use the following methods to upgrade Comware images:

Using TFTP to upgrade software images through the Ethernet port

Using FTP to upgrade software images through the Ethernet port

Using XMODEM to upgrade software through the console port

Using TFTP to upgrade software images through the Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu without downloading the upgrade file, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

|  | NOTE: - The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images are not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in flash memory for backup. To use it at reboot, you must change its attribute to main or backup. - If an image with the same attribute as the image you are loading is already in the flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 0

Using FTP to upgrade software images through the Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

FTP User Name       :switch

FTP User Password   :***

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu without downloading the upgrade file, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8):0

|  | NOTE: - The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in flash memory for backup. To use it at reboot, you must change its attribute to main or backup. - If an image with the same attribute as the image you are loading is already in the flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

Using XMODEM to upgrade software through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the Ethernet port. To save time, use the Ethernet port as long as possible.

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the file. (To return to the Boot menu, enter N.)

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) m

The boot.bin image is self-decompressing...

# At the Load File name prompt, enter a name for the boot image to be saved to flash memory.

Load File name  : default_file boot-update.bin       (At the prompt,

Free space: 470519808 bytes

Writing flash...................................................................

.............Done!

The system-update.bin image is self-decompressing...

# At the Load File name prompt, enter a name for the system image to be saved to flash memory.

Load File name  : default_file system-update.bin

Free space: 461522944 bytes

Writing flash...................................................................

.............Done!

Your baudrate should be set to 9600 bps again!

Press enter key when ready

|  | NOTE: - The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in the flash memory for backup. To use it at reboot, you must change its attribute to main or backup. - If an image with the same attribute as the image you are loading is already in flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps as described in step 5.a. If the baud rate is 9600 bps, skip this step.

|  | NOTE: The console port rate reverts to 9600 bps at a reboot. If you have changed the baud rate, you must perform this step so you can access the switch through the console port after a reboot. |
| --- | --- |

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 0

Enter 0 in the Boot menu to reboot the system with the new software images.

Upgrading Boot ROM from the Boot menu

You can use the following methods to upgrade the Boot ROM image:

Using TFTP to upgrade Boot ROM through the Ethernet port

Using FTP to upgrade Boot ROM through the Ethernet port

Using XMODEM to upgrade Boot ROM through the console port

Using TFTP to upgrade Boot ROM through the Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.btm

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using FTP to upgrade Boot ROM through the Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name     :update.btm

Server IP Address  :192.168.0.3

Local IP Address   :192.168.0.2

Subnet Mask        :255.255.255.0

Gateway IP Address :0.0.0.0

FTP User Name      :switch

FTP User Password  :123

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: - To use the default setting for a field, press Enter without entering any value. - If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using XMODEM to upgrade Boot ROM through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the Ethernet port. To save time, use the Ethernet port as long as possible.

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter to start downloading the file.

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter Y at the prompt to upgrade the basic Boot ROM section.

Loading ...CCCCCCCCCCCCCC  ...Done!

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps at the prompt, as described in step 4.a. If the baud rate is 9600 bps, skip this step.

Please change the terminal's baudrate to 9600 bps, press ENTER when ready.

|  | NOTE: The console port rate reverts to 9600 bps at a reboot. If you have changed the baud rate, you must perform this step so you can access the switch through the console port after a reboot. |
| --- | --- |

Press Enter to access the Boot ROM update menu.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Managing files from the Boot menu

From the Boot menu, you can display files in flash memory to check for obsolete files, incorrect files, or space insufficiency, delete files to release storage space, or change the attributes of software images.

Displaying all files

Enter 3 in the Boot menu to display all files in flash memory and identify the free space size.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 3

The following is a sample output:

Display all file(s) in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 464298848 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Deleting files

If storage space is insufficient, delete obsolete files to free up storage space.

To delete files:

Enter 4 in the Boot menu:

Deleting the file in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 464298848 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Enter the number of the file to delete. For example, enter 1 to select the file testbackup.cfg.

Please input the file number to change: 1

Enter Y at the confirmation prompt.

The file you selected is testbackup.cfg,Delete it? (Y/N):Y

Deleting....................................Done!

Changing the attribute of software images

Software image attributes include main (M), backup (B), and none (N). System software and boot software can each have multiple none-attribute images but only one main image and one backup image on the switch. You can assign both the M and B attributes to one image. If the M or B attribute you are assigning has been assigned to another image, the assignment removes the attribute from that image. If the removed attribute is the sole attribute of the image, its attribute changes to N.

For example, the system image system.bin has the M attribute and the system image system-update.bin has the B attribute. After you assign the M attribute to system-update.bin, the attribute of system-update.bin changes to M+B and the attribute of system.bin changes to N.

To change the attribute of a system or boot image:

Enter 2 in the Boot menu.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 2

1 or 2 at the prompt to set the attribute of a software image. (The following output is based on the option 2. To set the attribute of a configuration file, enter 3.)

1. Set image file

2. Set bin file

3. Set configuration file

0. Return to boot menu

Enter your choice(0-3): 2

File Number    File Size(bytes)     File Name

================================================================================

1(*)              53555200              flash:/system.bin

2(*)              9959424               flash:/boot.bin

3                 13105152              flash:/boot-update.bin

4                 91273216              flash:/system-update.bin

Free space: 417177920 bytes

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Note:Select .bin files. One but only one boot image and system image must be included.

Enter the number of the file you are working with. For example, enter 3 to select the boot image boot-update.bin. and enter 4 to select the system image system-update.bin.

Enter file No.(Allows multiple selection):3

Enter another file No.(0-Finish choice):4

Enter 0 to finish the selection.

Enter another file No.(0-Finish choice):0

You have selected:

flash:/boot-update.bin

flash:/system-update.bin

Enter M or B to change its attribute to main or backup. If you change its attribute to M, the attribute of boot.bin changes to none.

Please input the file attribute (Main/Backup) M

This operation may take several minutes. Please wait....

Next time, boot-update.bin will become default boot file!

Next time, system-update.bin will become default boot file!

Set the file attribute success!

Handling software upgrade failures

If a software upgrade fails, the system runs the old software version.

To handle a software upgrade failure:

Verify that the software release is compatible with the switch model and the correct file is used.

Verify that the software release and the Boot ROM release are compatible. For software and Boot ROM compatibility, see the hardware and software compatibility matrix in the correct release notes.

Check the physical ports for a loose or incorrect connection.

If you are using the console port for file transfer, check the HyperTerminal settings (including the baud rate and data bits) for any wrong setting.

Check the file transfer settings:

If XMODEM is used, you must set the same baud rate for the terminal as for the console port.

If TFTP is used, you must enter the same server IP addresses, file name, and working directory as set on the TFTP server.

If FTP is used, you must enter the same FTP server IP address, source file name, working directory, and FTP username and password as set on the FTP server.

Check the FTP or TFTP server for any incorrect setting.

Check that the storage device has sufficient space for the upgrade file.