# Contents

RFC1213-MIB	2

system Group {mib-2.1}	2

interfaces Group {mib-2.2}	3

at Group {mib-2.3}	3

atTable	3

ip Group {mib-2.4}	4

Scalar Objects	4

ipAddrTable	5

ipRouteTable	5

ipNetToMediaTable	6

ipv6InterfaceTable	6

ipSystemStatsTable	7

ipIfStatsTable	9

ipAddressPrefixTable	11

ipAddressTable	12

ipNetToPhysicalTable	12

ipDefaultRouterTable	13

ipv6RouterAdvertTable	13

icmp Group {mib-2.5}	14

Scalar objects	14

icmpStatsTable	15

icmpMsgStatsTable	15

tcp Group {mib-2.6}	16

Scalar objects	16

tcpConnTable	16

udp Group {mib-2.7}	17

Scalar objects	17

udpTable	17

snmp Group {mib-2.11}	17

RFC1213-MIB

system Group {mib-2.1}

This group is fully supported.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| sysDescr (1.3.6.1.2.1.1.1) | read-only | No | "H3C S5130S" for H3C S5130S series devices; "H3C S3100V3" for H3C S3100V3 series devices |
| sysObjectID (1.3.6.1.2.1.1.2) | read-only | No | H3C S5130S sysoid 1, 3, 6, 1, 4, 1, 25506, 1, 1585 (H3C S5130S-28S-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1586 (H3C S5130S-52S-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1589 (H3C S5130S-28S-HI) 1, 3, 6, 1, 4, 1, 25506, 1, 1590 (H3C S5130S-52S-HI) 1, 3, 6, 1, 4, 1, 25506, 1, 1569 (H3C S5130S-28S-PWR-HI) 1, 3, 6, 1, 4, 1, 25506, 1, 1574 (H3C S5130S-52S-PWR-HI) 1, 3, 6, 1, 4, 1, 25506, 1, 1650 (H3C S5130S-12TP-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1651 (H3C S5130S-20P-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1587 (H3C S5130S-28P-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1588 (H3C S5130S-52P-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1652 (H3C S5130S-12TP-HPWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1653 (H3C S5130S-20P-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1567 (H3C S5130S-28P-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1568 (H3C S5130S-28P-HPWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1572 (H3C S5130S-52P-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1570 (H3C S5130S-28S-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1571 (H3C S5130S-28S-HPWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1573 (H3C S5130S-52S-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1576 (H3C S5130S-52F-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1639 (H3C S5130S-10P-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1640 (H3C S5130S-10P-HPWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1637 (H3C S3100V3-10TP-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1645 (H3C S3100V3-18TP-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1646 (H3C S3100V3-28TP-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1647 (H3C S3100V3-52TP-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1638 (H3C S3100V3-10TP-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1648 (H3C S3100V3-20TP-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1649 (H3C S3100V3-28TP-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1575 (H3C S5130-54C-PWR-HI) 1, 3, 6, 1, 4, 1, 25506, 1, 1597 (H3C HS552X-S) 1, 3, 6, 1, 4, 1, 25506, 1, 1598 (H3C HS552X-E) 1, 3, 6, 1, 4, 1, 25506, 1, 1633 (H3C WP2024) 1, 3, 6, 1, 4, 1, 25506, 1, 1634 (H3C WP5024) 1, 3, 6, 1, 4, 1, 25506, 1, 1635 (H3C WP2008) 1, 3, 6, 1, 4, 1, 25506, 1, 1636 (H3C WP5008) 1, 3, 6, 1, 4, 1, 25506, 1, 1654 (H3C S5528S-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1655 (H3C S5552S-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1656 (H3C S5528P-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1657 (H3C S5552P-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1697 (H3C S5128P-PWR-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1698 (H3C S5152P-PWR-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1699 (H3C S5110P-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1700 (H3C S5110P-PWR-EI-D) 1, 3, 6, 1, 4, 1, 25506, 1, 1689 (H3C MS4320S-20P-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1685 (H3C MS4320S-28P-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1691 (H3C S5130S-28S-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1692 (H3C S5130S-52S-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1690 (H3C S5120V2-10P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1678 (H3C S5120V2-28P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1679 (H3C S5120V2-52P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1680 (H3C S5120V2-28P-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1681 (H3C S5120V2-28P-PWR-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1682 (H3C S5120V2-28P-HPWR-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1683 (H3C S5120V2-52P-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1684 (H3C S5120V2-52P-PWR-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1686 (H3C S5120V2-10P-PWR-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1687 (H3C S5120V2-20P-LI) 1, 3, 6, 1, 4, 1, 25506, 1, 1688 (H3C S5120V2-10P-LI) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 122 (H3C QX-S4108GT-2G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 123 (H3C QX-S4116GT-4G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 124 (H3C QX-S4124GT-4G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 125 (H3C QX-S4148GT-4G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 126 (H3C QX-S4108GT-2G-PW) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 127 (H3C QX-S4116GT-4G-PW) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 128 (H3C QX-S4124GT-4G-PW) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 129 (H3C QX-S4148GT-4G-PW) 1, 3, 6, 1, 4, 1, 25506, 1, 1718 (H3C MS4320S-52P) 1, 3, 6, 1, 4, 1, 25506, 1, 1715 (H3C E128C) 1, 3, 6, 1, 4, 1, 25506, 1, 1716 (H3C E152C) 1, 3, 6, 1, 4, 1, 25506, 1, 1717 (H3C MS4200-28TP) 1, 3, 6, 1, 4, 1, 25506, 1, 1722 (H3C MS4200-18TP) 1, 3, 6, 1, 4, 1, 25506, 1, 1723 (H3C MS4200-20TP-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1724 (H3C MS4200-28TP-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1719 (H3C S3100V3-10TP-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1720 (H3C S3100V3-18TP-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1721 (H3C S3100V3-28TP-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1725 (H3C S3100V3-52TP-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1726 (H3C S3100V3-28TP-PWR-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1727 (H3C S3100V3-10TP-PWR-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1728 (H3C S3100V3-20TP-PWR-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1787 (H3C WS5850-28X-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1788 (H3C WS5850-52X-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1789 (H3C WS5810-28P-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1790 (H3C WS5820-28P-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1791 (H3C WS5820-52X-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1792 (H3C WS5820-28X-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1793 (H3C WS5820-28X-POE-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1794 (H3C WS5820-28TP-POE-WiNet) 1, 3, 6, 1, 4, 1, 25506, 1, 1766 (H3C E528D-X-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1767 (H3C E552D-X-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1768 (H3C E528C-X) 1, 3, 6, 1, 4, 1, 25506, 1, 1769 (H3C E552C-X) 1, 3, 6, 1, 4, 1, 25506, 1, 1770 (H3C E528C-X-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1771 (H3C E552C-X-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1772 (H3C E528C) 1, 3, 6, 1, 4, 1, 25506, 1, 1773 (H3C E552C) 1, 3, 6, 1, 4, 1, 25506, 1, 1774 (H3C E528C-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1775 (H3C E552C-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1811 (H3C S5560S-28S-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1812 (H3C S5560S-52S-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1813 (H3C S5560S-28P-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1814 (H3C S5560S-52P-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1815 (H3C S5130S-28P-EI-M) 1, 3, 6, 1, 4, 1, 25506, 1, 1816 (H3C S5130S-52P-EI-M) 1, 3, 6, 1, 4, 1, 25506, 1, 1817 (H3C S5130S-28S-EI-M) 1, 3, 6, 1, 4, 1, 25506, 1, 1818 (H3C S5130S-52S-EI-M) 1, 3, 6, 1, 4, 1, 25506, 1, 1819 (H3C S5130S-28S-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1820 (H3C S5130S-52S-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1821 (H3C S5130S-16S-PWR-EI) 1,3,6,1,4,1,25506,1,2207 (H3C S5560S-28P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1822 (H3C S5560S-52P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1823 (H3C S5560S-28S-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1824 (H3C S5130S-10MS-UPWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1848 (H3C S5560S-52S-SI) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 131 (H3C QX-S3408FT-2G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 132 (H3C QX-S3416FT-2G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 133 (H3C QX-S3424FT-4G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 134 (H3C QX-S3448FT-4G) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 135 (H3C QX-S3408FT-2G-PW) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 136 (H3C QX-S3416FT-4G-PW) 1, 3, 6, 1, 4, 1, 119, 1,126,2, 137 (H3C QX-S3424FT-4G-PW) 1, 3, 6, 1, 4, 1, 25506, 1, 1749 (H3C IE4300-28P-M) 1, 3, 6, 1, 4, 1, 25506, 1, 1852 (H3C S5110V2-28P) 1, 3, 6, 1, 4, 1, 25506, 1, 1853 (H3C S5110V2-52P) 1, 3, 6, 1, 4, 1, 25506, 1, 1854 (H3C S5110V2-52P-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1855 (H3C S5110V2-28P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1856 (H3C S5110V2-52P-SI) 1, 3, 6, 1, 4, 1, 25506, 1, 1857 (H3C S5016PV3-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1858 (H3C S5024PV3-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1859 (H3C S5048PV3-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 1860 (H3C S5024PV3-EI-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1861 (H3C S5048PV3-EI-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 1865 (H3C S5552F-EI-D) H3C S3100V3 sysoid (H3C IE4300-12P-AC) 1, 3, 6, 1, 4, 1, 25506, 1, 2225 (H3C IE4300-12P-PWR) 1, 3, 6, 1, 4, 1, 25506, 1, 2226 (H3C S5130S-28ST-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 2318 (H3C S5130S-28ST-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 2320 (H3C S5130S-52ST-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 2319 (H3C S5130S-52ST-PWR-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 2316 (H3C S5130S-28PS-EI) 1, 3, 6, 1, 4, 1, 25506, 1, 2317 (H3C S5130S-28C-HI) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-52C-HI) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-28C-PWR-HI ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-52C-PWR-HI ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-28S-LI-MM) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-28S-LI-SM ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-28S-SI-MM) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S5130S-28S-SI-SM ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S3100V3-18TP-EI-DC ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S3100V3-20TP-PWR-EI-DC ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C S3100V3-28TP-EI-DC) 1, 3, 6, 1, 4, 1, 25506, 1, xxx (H3C WAS6108-HPWR ) 1, 3, 6, 1, 4, 1, 25506, 1, xxx |
| sysUpTime (1.3.6.1.2.1.1.3) | read-only | No | As per MIB |
| sysContact (1.3.6.1.2.1.1.4) | read-write | Current | "Hangzhou H3C Technologies Co., Ltd." for H3C devices; "" for HP devices The length of value is 1 to 255. |
| sysName (1.3.6.1.2.1.1.5) | read-write | Current | "H3C" for all H3C brand devices |
| sysLocation (1.3.6.1.2.1.1.6) | read-write | Current | "Hangzhou, China" for H3C devices; "" for HP devices The length of value is 1 to 255. |
| sysServices (1.3.6.1.2.1.1.7) | read-only | Current | 78 (2^ (2-1) + 2^ (3-1) + 2^ (4-1) + 2^ (7-1)). indicates that system can provide the following services: 2 datalink/subnetwork (e.g., bridges) 3 internet (e.g., supports the IP) 4 end-to-end (e.g., supports the TCP) 7 applications (e.g., supports the SMTP) |

interfaces Group {mib-2.2}

See ifTable in IF-MIB

at Group {mib-2.3}

at (Address Translation) group is fully supported.

atTable

OID of this table is: 1.3.6.1.2.1.3.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| atIfIndex (1.3.6.1.2.1.3.1.1.1) | read-write | Current | Only support read operation |
| atPhysAddress (1.3.6.1.2.1.3.1.1.2) | read-write | Current | Only support read operation |
| atNetAddress (1.3.6.1.2.1.3.1.1.3) | read-write | Current | Only support read operation |

ip Group {mib-2.4}

The following objects will be supported:

Scalar Objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipForwarding (1.3.6.1.2.1.4.1) | read-write | Current | Because the L3 switch can not be set to L2 mode via the MIB, so it is a read only object. |
| ipDefaultTTL (1.3.6.1.2.1.4.2) | read-write | No | Only support read operation |
| ipInReceives (1.3.6.1.2.1.4.3) | read-only | No | As per MIB |
| ipInHdrErrors (1.3.6.1.2.1.4.4) | read-only | No | As per MIB |
| ipInAddrErrors (1.3.6.1.2.1.4.5) | read-only | No | As per MIB |
| ipForwDatagrams (1.3.6.1.2.1.4.6) | read-only | No | As per MIB |
| ipInUnknownProtos (1.3.6.1.2.1.4.7) | read-only | No | As per MIB |
| ipInDiscards (1.3.6.1.2.1.4.8) | read-only | No | As per MIB |
| ipInDelivers (1.3.6.1.2.1.4.9) | read-only | No | As per MIB |
| ipOutRequests (1.3.6.1.2.1.4.10) | read-only | No | As per MIB |
| ipOutDiscards (1.3.6.1.2.1.4.11) | read-only | No | As per MIB |
| ipOutNoRoutes (1.3.6.1.2.1.4.12) | read-only | No | As per MIB |
| ipReasmTimeout (1.3.6.1.2.1.4.13) | read-only | No | As per MIB |
| ipReasmReqds (1.3.6.1.2.1.4.14) | read-only | No | As per MIB |
| ipReasmOKs (1.3.6.1.2.1.4.15) | read-only | No | As per MIB |
| ipReasmFails (1.3.6.1.2.1.4.16) | read-only | No | As per MIB |
| ipFragOKs (1.3.6.1.2.1.4.17) | read-only | No | As per MIB |
| ipFragFails (1.3.6.1.2.1.4.18) | read-only | No | As per MIB |
| ipFragCreates (1.3.6.1.2.1.4.19) | read-only | No | As per MIB |
| ipRoutingDiscards (1.3.6.1.2.1.4.23) | read-only | No | As per MIB |
| ipv6IpForwarding (1.3.6.1.2.1.4.25) | read-write | No | Only support read operation |
| ipv6IpDefaultHopLimit (1.3.6.1.2.1.4.26) | read-write | No | Only support read operation |
| ipv4InterfaceTableLastChange (1.3.6.1.2.1.4.27) | read-only | No | Not supported |
| ipv6InterfaceTableLastChange (1.3.6.1.2.1.4.29) | read-only | No | Not supported |
| ipIfStatsTableLastChange (1.3.6.1.2.1.4.31.2) | read-only | No | Not supported |
| ipAddressSpinLock (1.3.6.1.2.1.4.33) | read-write | No | Not supported |
| ipv6RouterAdvertSpinLock (1.3.6.1.2.1.4.38) | read-write | No | Not supported |

ipAddrTable

OID of this table is: 1.3.6.1.2.1.4.20

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAdEntAddr (1.3.6.1.2.1.4.20.1.1) | read-only | Current | As per MIB |
| ipAdEntIfIndex (1.3.6.1.2.1.4.20.1.2) | read-only | Current | As per MIB |
| ipAdEntNetMask (1.3.6.1.2.1.4.20.1.3) | read-only | Current | As per MIB |
| ipAdEntBcastAddr (1.3.6.1.2.1.4.20.1.4) | read-only | Current | As per MIB |
| ipAdEntReasmMaxSize (1.3.6.1.2.1.4.20.1.5) | read-only | Current | As per MIB |

ipRouteTable

OID of this table is: 1.3.6.1.2.1.4.21

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipRouteDest(1.3.6.1.2.1.4.21.1.1) | read-write | No | Only support read |
| ipRouteIfIndex(1.3.6.1.2.1.4.21.1.2) | read-write | No | Only support read |
| ipRouteMetric1(1.3.6.1.2.1.4.21.1.3) | read-write | No | Only support read |
| ipRouteMetric2(1.3.6.1.2.1.4.21.1.4) | read-write | No | Only support read |
| ipRouteMetric3(1.3.6.1.2.1.4.21.1.5) | read-write | No | Only support read |
| ipRouteMetric4(1.3.6.1.2.1.4.21.1.6) | read-write | No | Only support read |
| ipRouteNextHop(1.3.6.1.2.1.4.21.1.7) | read-write | No | Only support read |
| ipRouteType(1.3.6.1.2.1.4.21.1.8) | read-write | No | Only support read |
| ipRouteProto(1.3.6.1.2.1.4.21.1.9) | read-only | No | As per MIB |
| ipRouteAge(1.3.6.1.2.1.4.21.1.10) | read-write | No | Only support read |
| ipRouteMask(1.3.6.1.2.1.4.21.1.11) | read-write | No | Only support read |
| ipRouteMetric5(1.3.6.1.2.1.4.21.1.12) | read-write | No | Only support read |
| ipRouteInfo(1.3.6.1.2.1.4.21.1.13) | read-only | No | As per MIB |

ipNetToMediaTable

OID of this table is: 1.3.6.1.2.1.4.22

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipNetToMediaIfIndex (1.3.6.1.2.1.4.22.1.1) | read-write | Current | Only support read operation. |
| ipNetToMediaPhysAddress (1.3.6.1.2.1.4.22.1.2) | read-write | Current | Only support read operation |
| ipNetToMediaNetAddress (1.3.6.1.2.1.4.22.1.3) | read-write | Current | Only support read operation |
| ipNetToMediaType (1.3.6.1.2.1.4.22.1.4) | read-write | Current | Only support read operation |

ipv6InterfaceTable

OID of this table is: 1.3.6.1.2.1.4.30

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6InterfaceIfIndex (1.3.6.1.2.1.4.30.1.1) | not-accessible | No | As per MIB |
| ipv6InterfaceReasmMaxSize (1.3.6.1.2.1.4.30.1.2) | read-only | No | As per MIB |
| ipv6InterfaceIdentifier (1.3.6.1.2.1.4.30.1.3) | read-only | No | As per MIB |
| ipv6InterfaceEnableStatus (1.3.6.1.2.1.4.30.1.5) | read-write | No | Only support read operation |
| ipv6InterfaceReachableTime (1.3.6.1.2.1.4.30.1.6) | read-only | No | As per MIB |
| ipv6InterfaceRetransmitTime (1.3.6.1.2.1.4.30.1.7) | read-only | No | As per MIB |
| ipv6InterfaceForwarding (1.3.6.1.2.1.4.30.1.8) | read-write | No | Only support read operation |

ipSystemStatsTable

OID of this table is: 1.3.6.1.2.1.4.31.1

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipSystemStatsIPVersion (1.3.6.1.2.1.4.31.1.1.1) | not-accessible | No | Only support ipv6(2) |
| ipSystemStatsInReceives (1.3.6.1.2.1.4.31.1.1.3) | read-only | No | As per MIB |
| ipSystemStatsHCInReceives (1.3.6.1.2.1.4.31.1.1.4) | read-only | No | As per MIB |
| ipSystemStatsInOctets (1.3.6.1.2.1.4.31.1.1.5) | read-only | No | Not supported |
| ipSystemStatsHCInOctets (1.3.6.1.2.1.4.31.1.1.6) | read-only | No | Not supported |
| ipSystemStatsInHdrErrors (1.3.6.1.2.1.4.31.1.1.7) | read-only | No | As per MIB |
| ipSystemStatsInNoRoutes (1.3.6.1.2.1.4.31.1.1.8) | read-only | No | As per MIB |
| ipSystemStatsInAddrErrors (1.3.6.1.2.1.4.31.1.1.9) | read-only | No | As per MIB |
| ipSystemStatsInUnknownProtos (1.3.6.1.2.1.4.31.1.1.10) | read-only | No | As per MIB |
| ipSystemStatsInTruncatedPkts (1.3.6.1.2.1.4.31.1.1.11) | read-only | No | Not supported |
| ipSystemStatsInForwDatagrams (1.3.6.1.2.1.4.31.1.1.12) | read-only | No | As per MIB |
| ipSystemStatsHCInForwDatagrams (1.3.6.1.2.1.4.31.1.1.13) | read-only | No | As per MIB |
| ipSystemStatsReasmReqds (1.3.6.1.2.1.4.31.1.1.14) | read-only | No | As per MIB |
| ipSystemStatsReasmOKs (1.3.6.1.2.1.4.31.1.1.15) | read-only | No | As per MIB |
| ipSystemStatsReasmFails (1.3.6.1.2.1.4.31.1.1.16) | read-only | No | As per MIB |
| ipSystemStatsInDiscards (1.3.6.1.2.1.4.31.1.1.17) | read-only | No | As per MIB |
| ipSystemStatsInDelivers (1.3.6.1.2.1.4.31.1.1.18) | read-only | No | As per MIB |
| ipSystemStatsHCInDelivers (1.3.6.1.2.1.4.31.1.1.19) | read-only | No | Not supported |
| ipSystemStatsOutRequests (1.3.6.1.2.1.4.31.1.1.20) | read-only | No | As per MIB |
| ipSystemStatsHCOutRequests (1.3.6.1.2.1.4.31.1.1.21) | read-only | No | Not supported |
| ipSystemStatsOutNoRoutes (1.3.6.1.2.1.4.31.1.1.22) | read-only | No | As per MIB |
| ipSystemStatsOutForwDatagrams (1.3.6.1.2.1.4.31.1.1.23) | read-only | No | As per MIB |
| ipSystemStatsHCOutForwDatagrams (1.3.6.1.2.1.4.31.1.1.24) | read-only | No | As per MIB |
| ipSystemStatsOutDiscards (1.3.6.1.2.1.4.31.1.1.25) | read-only | No | As per MIB |
| ipSystemStatsOutFragReqds (1.3.6.1.2.1.4.31.1.1.26) | read-only | No | As per MIB |
| ipSystemStatsOutFragOKs (1.3.6.1.2.1.4.31.1.1.27) | read-only | No | As per MIB |
| ipSystemStatsOutFragFails (1.3.6.1.2.1.4.31.1.1.28) | read-only | No | As per MIB |
| ipSystemStatsOutFragCreates (1.3.6.1.2.1.4.31.1.1.29) | read-only | No | As per MIB |
| ipSystemStatsOutTransmits (1.3.6.1.2.1.4.31.1.1.30) | read-only | No | As per MIB |
| ipSystemStatsHCOutTransmits (1.3.6.1.2.1.4.31.1.1.31) | read-only | No | As per MIB |
| ipSystemStatsOutOctets (1.3.6.1.2.1.4.31.1.1.32) | read-only | No | Not supported |
| ipSystemStatsHCOutOctets (1.3.6.1.2.1.4.31.1.1.33) | read-only | No | Not supported |
| ipSystemStatsInMcastPkts (1.3.6.1.2.1.4.31.1.1.34) | read-only | No | Not supported |
| ipSystemStatsHCInMcastPkts (1.3.6.1.2.1.4.31.1.1.35) | read-only | No | Not supported |
| ipSystemStatsInMcastOctets (1.3.6.1.2.1.4.31.1.1.36) | read-only | No | Not supported |
| ipSystemStatsHCInMcastOctets (1.3.6.1.2.1.4.31.1.1.37) | read-only | No | Not supported |
| ipSystemStatsOutMcastPkts (1.3.6.1.2.1.4.31.1.1.38) | read-only | No | Not supported |
| ipSystemStatsHCOutMcastPkts (1.3.6.1.2.1.4.31.1.1.39) | read-only | No | Not supported |
| ipSystemStatsOutMcastOctets (1.3.6.1.2.1.4.31.1.1.40) | read-only | No | Not supported |
| ipSystemStatsHCOutMcastOctets (1.3.6.1.2.1.4.31.1.1.41) | read-only | No | Not supported |
| ipSystemStatsInBcastPkts (1.3.6.1.2.1.4.31.1.1.42) | read-only | No | Not supported |
| ipSystemStatsHCInBcastPkts (1.3.6.1.2.1.4.31.1.1.43) | read-only | No | Not supported |
| ipSystemStatsOutBcastPkts (1.3.6.1.2.1.4.31.1.1.44) | read-only | No | Not supported |
| ipSystemStatsHCOutBcastPkts (1.3.6.1.2.1.4.31.1.1.45) | read-only | No | Not supported |
| ipSystemStatsDiscontinuityTime (1.3.6.1.2.1.4.31.1.1.46) | read-only | No | Not supported |
| ipSystemStatsRefreshRate (1.3.6.1.2.1.4.31.1.1.47) | read-only | No | Not supported |

ipIfStatsTable

OID of this table is: 1.3.6.1.2.1.4.31.3

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipIfStatsIPVersion (1.3.6.1.2.1.4.31.3.1.1) | not-accessible | No | Only support ipv6(2) |
| ipIfStatsIfIndex (1.3.6.1.2.1.4.31.3.1.2) | not-accessible | No | As per MIB |
| ipIfStatsInReceives (1.3.6.1.2.1.4.31.3.1.3) | read-only | No | As per MIB |
| ipIfStatsHCInReceives (1.3.6.1.2.1.4.31.3.1.4) | read-only | No | Not supported |
| ipIfStatsInOctets (1.3.6.1.2.1.4.31.3.1.5) | read-only | No | Not supported |
| ipIfStatsHCInOctets (1.3.6.1.2.1.4.31.3.1.6) | read-only | No | Not supported |
| ipIfStatsInHdrErrors (1.3.6.1.2.1.4.31.3.1.7) | read-only | No | As per MIB |
| ipIfStatsInNoRoutes (1.3.6.1.2.1.4.31.3.1.8) | read-only | No | As per MIB |
| ipIfStatsInAddrErrors (1.3.6.1.2.1.4.31.3.1.9) | read-only | No | As per MIB |
| ipIfStatsInUnknownProtos (1.3.6.1.2.1.4.31.3.1.10) | read-only | No | As per MIB |
| ipIfStatsInTruncatedPkts (1.3.6.1.2.1.4.31.3.1.11) | read-only | No | As per MIB |
| ipIfStatsInForwDatagrams (1.3.6.1.2.1.4.31.3.1.12) | read-only | No | As per MIB |
| ipIfStatsHCInForwDatagrams (1.3.6.1.2.1.4.31.3.1.13) | read-only | No | Not supported |
| ipIfStatsReasmReqds (1.3.6.1.2.1.4.31.3.1.14) | read-only | No | As per MIB |
| ipIfStatsReasmOKs (1.3.6.1.2.1.4.31.3.1.15) | read-only | No | As per MIB |
| ipIfStatsReasmFails (1.3.6.1.2.1.4.31.3.1.16) | read-only | No | As per MIB |
| ipIfStatsInDiscards (1.3.6.1.2.1.4.31.3.1.17) | read-only | No | As per MIB |
| ipIfStatsInDelivers (1.3.6.1.2.1.4.31.3.1.18) | read-only | No | As per MIB |
| ipIfStatsHCInDelivers (1.3.6.1.2.1.4.31.3.1.19) | read-only | No | Not supported |
| ipIfStatsOutRequests (1.3.6.1.2.1.4.31.3.1.20) | read-only | No | As per MIB |
| ipIfStatsHCOutRequests (1.3.6.1.2.1.4.31.3.1.21) | read-only | No | Not supported |
| ipIfStatsOutForwDatagrams (1.3.6.1.2.1.4.31.3.1.23) | read-only | No | As per MIB |
| ipIfStatsHCOutForwDatagrams (1.3.6.1.2.1.4.31.3.1.24) | read-only | No | Not supported |
| ipIfStatsOutDiscards (1.3.6.1.2.1.4.31.3.1.25) | read-only | No | As per MIB |
| ipIfStatsOutFragReqds (1.3.6.1.2.1.4.31.3.1.26) | read-only | No | As per MIB |
| ipIfStatsOutFragOKs (1.3.6.1.2.1.4.31.3.1.27) | read-only | No | As per MIB |
| ipIfStatsOutFragFails (1.3.6.1.2.1.4.31.3.1.28) | read-only | No | As per MIB |
| ipIfStatsOutFragCreates (1.3.6.1.2.1.4.31.3.1.29) | read-only | No | As per MIB |
| ipIfStatsOutTransmits (1.3.6.1.2.1.4.31.3.1.30) | read-only | No | As per MIB |
| ipIfStatsHCOutTransmits (1.3.6.1.2.1.4.31.3.1.31) | read-only | No | Not supported |
| ipIfStatsOutOctets (1.3.6.1.2.1.4.31.3.1.32) | read-only | No | Not supported |
| ipIfStatsHCOutOctets (1.3.6.1.2.1.4.31.3.1.33) | read-only | No | Not supported |
| ipIfStatsInMcastPkts (1.3.6.1.2.1.4.31.3.1.34) | read-only | No | As per MIB |
| ipIfStatsHCInMcastPkts (1.3.6.1.2.1.4.31.3.1.35) | read-only | No | Not supported |
| ipIfStatsInMcastOctets (1.3.6.1.2.1.4.31.3.1.36) | read-only | No | Not supported |
| ipIfStatsHCInMcastOctets (1.3.6.1.2.1.4.31.3.1.37) | read-only | No | Not supported |
| ipIfStatsOutMcastPkts (1.3.6.1.2.1.4.31.3.1.38) | read-only | No | As per MIB |
| ipIfStatsHCOutMcastPkts (1.3.6.1.2.1.4.31.3.1.39) | read-only | No | Not supported |
| ipIfStatsOutMcastOctets (1.3.6.1.2.1.4.31.3.1.40) | read-only | No | Not supported |
| ipIfStatsHCOutMcastOctets (1.3.6.1.2.1.4.31.3.1.41) | read-only | No | Not supported |
| ipIfStatsInBcastPkts (1.3.6.1.2.1.4.31.3.1.42) | read-only | No | Not supported |
| ipIfStatsHCInBcastPkts (1.3.6.1.2.1.4.31.3.1.43) | read-only | No | Not supported |
| ipIfStatsOutBcastPkts (1.3.6.1.2.1.4.31.3.1.44) | read-only | No | Not supported |
| ipIfStatsHCOutBcastPkts (1.3.6.1.2.1.4.31.3.1.45) | read-only | No | Not supported |
| ipIfStatsDiscontinuityTime (1.3.6.1.2.1.4.31.3.1.46) | read-only | No | Not supported |
| ipIfStatsRefreshRate (1.3.6.1.2.1.4.31.3.1.47) | read-only | No | Not supported |

ipAddressPrefixTable

OID of this table is: 1.3.6.1.2.1.4.32

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAddressPrefixIfIndex (1.3.6.1.2.1.4.32.1.1) | not-accessible | No | As per MIB |
| ipAddressPrefixType (1.3.6.1.2.1.4.32.1.2) | not-accessible | No | Only support ipv6(2) and ipv6z(4) |
| ipAddressPrefixPrefix (1.3.6.1.2.1.4.32.1.3) | not-accessible | No | As per MIB |
| ipAddressPrefixLength (1.3.6.1.2.1.4.32.1.4) | not-accessible | No | As per MIB |
| ipAddressPrefixOrigin (1.3.6.1.2.1.4.32.1.5) | read-only | No | As per MIB |
| ipAddressPrefixOnLinkFlag (1.3.6.1.2.1.4.32.1.6) | read-only | No | As per MIB |
| ipAddressPrefixAutonomousFlag (1.3.6.1.2.1.4.32.1.7) | read-only | No | As per MIB |
| ipAddressPrefixAdvPreferredLifetime (1.3.6.1.2.1.4.32.1.8) | read-only | No | As per MIB |
| ipAddressPrefixAdvValidLifetime (1.3.6.1.2.1.4.32.1.9) | read-only | No | As per MIB |

ipAddressTable

OID of this table is: 1.3.6.1.2.1.4.34

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipAddressAddrType (1.3.6.1.2.1.4.34.1.1) | not-accessible | No | Only support ipv6(2) and ipv6z(4) |
| ipAddressAddr (1.3.6.1.2.1.4.34.1.2) | not-accessible | No | As per MIB |
| ipAddressIfIndex (1.3.6.1.2.1.4.34.1.3) | read-create | No | Only support read operation |
| ipAddressType (1.3.6.1.2.1.4.34.1.4) | read-create | No | Only support read operation |
| ipAddressPrefix (1.3.6.1.2.1.4.34.1.5) | read-only | No | As per MIB |
| ipAddressOrigin (1.3.6.1.2.1.4.34.1.6) | read-only | No | As per MIB |
| ipAddressStatus (1.3.6.1.2.1.4.34.1.7) | read-create | No | Only support read operation |
| ipAddressCreated (1.3.6.1.2.1.4.34.1.8) | read-only | No | As per MIB |
| ipAddressLastChanged (1.3.6.1.2.1.4.34.1.9) | read-only | No | As per MIB |
| ipAddressRowStatus (1.3.6.1.2.1.4.34.1.10) | read-create | No | Only support read operation Only support active(1). |
| ipAddressStorageType (1.3.6.1.2.1.4.34.1.11) | read-only | No | The value is always volatile(2). |

ipNetToPhysicalTable

OID of this table is: 1.3.6.1.2.1.4.35

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipNetToPhysicalIfIndex (1.3.6.1.2.1.4.35.1.1) | not-accessible | No | As per MIB |
| ipNetToPhysicalNetAddressType (1.3.6.1.2.1.4.35.1.2) | not-accessible | No | Only support ipv6(2),ipv6z(4) |
| ipNetToPhysicalNetAddress (1.3.6.1.2.1.4.35.1.3) | not-accessible | No | As per MIB |
| ipNetToPhysicalPhysAddress (1.3.6.1.2.1.4.35.1.4) | read-create | No | Only support read operation |
| ipNetToPhysicalLastUpdated (1.3.6.1.2.1.4.35.1.5) | read-only | No | As per MIB |
| ipNetToPhysicalType (1.3.6.1.2.1.4.35.1.6) | read-create | No | Only support read operation |
| ipNetToPhysicalState (1.3.6.1.2.1.4.35.1.7) | read-only | No | As per MIB |
| ipNetToPhysicalRowStatus (1.3.6.1.2.1.4.35.1.8) | read-create | No | Only support read operation |

ipDefaultRouterTable

OID of this table is: 1.3.6.1.2.1.4.37

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipDefaultRouterAddressType (1.3.6.1.2.1.4.37.1.1) | not-accessible | No | Only support ipv6(2) and ipv6z(4) |
| ipDefaultRouterAddress (1.3.6.1.2.1.4.37.1.2) | not-accessible | No | As per MIB |
| ipDefaultRouterIfIndex (1.3.6.1.2.1.4.37.1.3) | not-accessible | No | As per MIB |
| ipDefaultRouterLifetime (1.3.6.1.2.1.4.37.1.4) | read-only | No | As per MIB |
| ipDefaultRouterPreference (1.3.6.1.2.1.4.37.1.5) | read-only | No | As per MIB |

ipv6RouterAdvertTable

OID of this table is: 1.3.6.1.2.1.4.39

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| ipv6RouterAdvertIfIndex (1.3.6.1.2.1.4.39.1.1) | not-accessible | No | As per MIB |
| ipv6RouterAdvertSendAdverts (1.3.6.1.2.1.4.39.1.2) | read-create | No | Only support read operation |
| ipv6RouterAdvertMaxInterval (1.3.6.1.2.1.4.39.1.3) | read-create | No | Only support read operation |
| ipv6RouterAdvertMinInterval (1.3.6.1.2.1.4.39.1.4) | read-create | No | Only support read operation |
| ipv6RouterAdvertManagedFlag (1.3.6.1.2.1.4.39.1.5) | read-create | No | Only support read operation |
| ipv6RouterAdvertOtherConfigFlag (1.3.6.1.2.1.4.39.1.6) | read-create | No | Only support read operation |
| ipv6RouterAdvertLinkMTU (1.3.6.1.2.1.4.39.1.7) | read-create | No | Only support read operation |
| ipv6RouterAdvertReachableTime (1.3.6.1.2.1.4.39.1.8) | read-create | No | Only support read operation |
| ipv6RouterAdvertRetransmitTime (1.3.6.1.2.1.4.39.1.9) | read-create | No | Only support read operation |
| ipv6RouterAdvertCurHopLimit (1.3.6.1.2.1.4.39.1.10) | read-create | No | Only support read operation |
| ipv6RouterAdvertDefaultLifetime (1.3.6.1.2.1.4.39.1.11) | read-create | No | Only support read operation |
| ipv6RouterAdvertRowStatus (1.3.6.1.2.1.4.39.1.12) | read-create | No | Only support read operation |

icmp Group {mib-2.5}

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpInMsgs (1.3.6.1.2.1.5.1) | read-only | No | As per MIB |
| icmpInErrors (1.3.6.1.2.1.5.2) | read-only | No | As per MIB |
| icmpInDestUnreachs (1.3.6.1.2.1.5.3) | read-only | No | As per MIB |
| icmpInTimeExcds (1.3.6.1.2.1.5.4) | read-only | No | As per MIB |
| icmpInParmProbs (1.3.6.1.2.1.5.5) | read-only | No | As per MIB |
| icmpInSrcQuenchs (1.3.6.1.2.1.5.6) | read-only | No | As per MIB |
| icmpInRedirects (1.3.6.1.2.1.5.7) | read-only | No | As per MIB |
| icmpInEchos (1.3.6.1.2.1.5.8) | read-only | No | As per MIB |
| icmpInEchoReps (1.3.6.1.2.1.5.9) | read-only | No | As per MIB |
| icmpInTimestamps (1.3.6.1.2.1.5.10) | read-only | No | As per MIB |
| icmpInTimestampReps (1.3.6.1.2.1.5.11) | read-only | No | As per MIB |
| icmpInAddrMasks (1.3.6.1.2.1.5.12) | read-only | No | As per MIB |
| icmpInAddrMaskReps (1.3.6.1.2.1.5.13) | read-only | No | As per MIB |
| icmpOutMsgs (1.3.6.1.2.1.5.14) | read-only | No | As per MIB |
| icmpOutErrors (1.3.6.1.2.1.5.15) | read-only | No | As per MIB |
| icmpOutDestUnreachs (1.3.6.1.2.1.5.16) | read-only | No | As per MIB |
| icmpOutTimeExcds (1.3.6.1.2.1.5.17) | read-only | No | As per MIB |
| icmpOutParmProbs (1.3.6.1.2.1.5.18) | read-only | No | As per MIB |
| icmpOutSrcQuenchs (1.3.6.1.2.1.5.19) | read-only | No | Not supported |
| icmpOutRedirects (1.3.6.1.2.1.5.20) | read-only | No | As per MIB |
| icmpOutEchos (1.3.6.1.2.1.5.21) | read-only | No | As per MIB |
| icmpOutEchoReps (1.3.6.1.2.1.5.22) | read-only | No | As per MIB |
| icmpOutTimestamps (1.3.6.1.2.1.5.23) | read-only | No | Not supported |
| icmpOutTimestampReps (1.3.6.1.2.1.5.24) | read-only | No | As per MIB |
| icmpOutAddrMasks (1.3.6.1.2.1.5.25) | read-only | No | Not supported |
| icmpOutAddrMaskReps (1.3.6.1.2.1.5.26) | read-only | No | As per MIB |

icmpStatsTable

OID of this table is: 1.3.6.1.2.1.5.29

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpStatsIPVersion | not-accessible | No | Only support ipv6(2) |
| icmpStatsInMsgs | read-only | No | As per MIB |
| icmpStatsInErrors | read-only | No | As per MIB |
| icmpStatsOutMsgs | read-only | No | As per MIB |
| icmpStatsOutErrors | read-only | No | The value is always 0. |

icmpMsgStatsTable

OID of this table is: 1.3.6.1.2.1.5.30

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| icmpMsgStatsIPVersion | not-accessible | No | Only support ipv6(2) |
| icmpMsgStatsType | not-accessible | No | As per MIB |
| icmpMsgStatsInPkts | read-only | No | As per MIB |
| icmpMsgStatsOutPkts | read-only | No | As per MIB |

tcp Group {mib-2.6}

tcp Group in RFC1213-MIB is updated by RFC4022-MIB.

Scalar objects

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| tcpRtoAlgorithm (1.3.6.1.2.1.6.1) | read-only | No | As per MIB |
| tcpRtoMin (1.3.6.1.2.1.6.2) | read-only | No | As per MIB |
| tcpRtoMax (1.3.6.1.2.1.6.3) | read-only | No | As per MIB |
| tcpMaxConn (1.3.6.1.2.1.6.4) | read-only | No | As per MIB |
| tcpActiveOpens (1.3.6.1.2.1.6.5) | read-only | No | As per MIB |
| tcpPassiveOpens (1.3.6.1.2.1.6.6) | read-only | No | As per MIB |
| tcpAttemptFails (1.3.6.1.2.1.6.7) | read-only | No | As per MIB |
| tcpEstabResets (1.3.6.1.2.1.6.8) | read-only | No | As per MIB |
| tcpCurrEstab (1.3.6.1.2.1.6.9) | read-only | No | As per MIB |
| tcpInSegs (1.3.6.1.2.1.6.10) | read-only | No | As per MIB |
| tcpOutSegs (1.3.6.1.2.1.6.11) | read-only | No | As per MIB |
| tcpRetransSegs (1.3.6.1.2.1.6.12) | read-only | No | As per MIB |
| tcpInErrs (1.3.6.1.2.1.6.14) | read-only | No | As per MIB |
| tcpOutRsts (1.3.6.1.2.1.6.15) | read-only | No | As per MIB |

tcpConnTable

OID of this table is:  1.3.6.1.2.1.6.13

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| tcpConnState (1.3.6.1.2.1.6.13.1.1) | read-write | No | Only support read operation. |
| tcpConnLocalAddress (1.3.6.1.2.1.6.13.1.2) | read-only | No | As per MIB |
| tcpConnLocalPort (1.3.6.1.2.1.6.13.1.3) | read-only | No | As per MIB |
| tcpConnRemAddress (1.3.6.1.2.1.6.13.1.4) | read-only | No | As per MIB |
| tcpConnRemPort (1.3.6.1.2.1.6.13.1.5) | read-only | No | As per MIB |

udp Group {mib-2.7}

udp Group in RFC1213-MIB is updated by RFC4113-MIB.

Scalar objects

A statistics collection for UDP group.

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| udpInDatagrams (1.3.6.1.2.1.7.1) | read- only | No | As per MIB |
| udpNoPorts (1.3.6.1.2.1.7.2) | read-only | No | As per MIB |
| udpInErrors (1.3.6.1.2.1.7.3) | read-only | No | As per MIB |
| udpOutDatagrams (1.3.6.1.2.1.7.4) | read-only | No | As per MIB |

udpTable

OID of this table is:  1.3.6.1.2.1.7.5

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| udpLocalAddress (1.3.6.1.2.1.7.5.1.1) | read-only | Current | As per MIB |
| udpLocalPort (1.3.6.1.2.1.7.5.1.2) | read-only | Current | As per MIB |

snmp Group {mib-2.11}

| Name | Access | PDS | Description |
| --- | --- | --- | --- |
| snmpInPkts (1.3.6.1.2.1.11.1) | read-only | No | As per MIB |
| snmpOutPkts (1.3.6.1.2.1.11.2) | read-only | No | As per MIB |
| snmpInBadVersions (1.3.6.1.2.1.11.3) | read-only | No | As per MIB |
| snmpInBadCommunityNames (1.3.6.1.2.1.11.4) | read-only | No | As per MIB |
| snmpInBadCommunityUses (1.3.6.1.2.1.11.5) | read-only | No | As per MIB |
| snmpInASNParseErrs (1.3.6.1.2.1.11.6) | read-only | No | As per MIB |
| snmpInTooBigs (1.3.6.1.2.1.11.8) | read-only | No | The value is always 0. |
| snmpInNoSuchNames (1.3.6.1.2.1.11.9) | read-only | No | The value is always 0. |
| snmpInBadValues (1.3.6.1.2.1.11.10) | read-only | No | The value is always 0. |
| snmpInReadOnlys (1.3.6.1.2.1.11.11) | read-only | No | The value is always 0. |
| snmpInGenErrs (1.3.6.1.2.1.11.12) | read-only | No | The value is always 0. |
| snmpInTotalReqVars (1.3.6.1.2.1.11.13) | read-only | No | As per MIB |
| snmpInTotalSetVars (1.3.6.1.2.1.11.14) | read-only | No | As per MIB |
| snmpInGetRequests (1.3.6.1.2.1.11.15) | read-only | No | As per MIB |
| snmpInGetNexts (1.3.6.1.2.1.11.16) | read-only | No | As per MIB |
| snmpInSetRequests (1.3.6.1.2.1.11.17) | read-only | No | As per MIB |
| snmpInGetResponses (1.3.6.1.2.1.11.18) | read-only | No | Not supported |
| snmpInTraps (1.3.6.1.2.1.11.19) | read-only | No | Not supported |
| snmpOutTooBigs (1.3.6.1.2.1.11.20) | read-only | No | As per MIB |
| snmpOutNoSuchNames (1.3.6.1.2.1.11.21) | read-only | No | As per MIB |
| snmpOutBadValues (1.3.6.1.2.1.11.22) | read-only | No | As per MIB |
| snmpOutGenErrs (1.3.6.1.2.1.11.24) | read-only | No | As per MIB |
| snmpOutGetRequests (1.3.6.1.2.1.11.25) | read-only | No | As per MIB |
| snmpOutGetNexts (1.3.6.1.2.1.11.26) | read-only | No | As per MIB |
| snmpOutSetRequests (1.3.6.1.2.1.11.27) | read-only | No | As per MIB |
| snmpOutGetResponses (1.3.6.1.2.1.11.28) | read-only | No | As per MIB |
| snmpOutTraps (1.3.6.1.2.1.11.29) | read-only | No | As per MIB |
| snmpEnableAuthenTraps (1.3.6.1.2.1.11.30) | read-write | Current | The default value is enabled(1). |