HPE 5140HI-CMW710-R6823 Release Notes

Software Feature Changes

The information in this document is subject to change without notice. © Copyright 2025 Hewlett Packard Enterprise Development LP

Contents

R6823	1

New feature: Enabling MAC authentication user logging on an interface	1

Enabling MAC authentication user logging on an interface	1

Command reference	1

mac-authentication access-user log enable (interface-view)	1

New feature: Configuring the secure MAC aging timer in interface view	2

Configuring the secure MAC aging timer in interface view	2

Command reference	3

port-security timer autolearn aging (interface view)	3

R6820	5

R6817	6

R6816	7

New feature: Enabling the MAC address entry conversion feature	7

Enabling the MAC address entry conversion feature	7

Procedure	7

Command reference	7

mac-address convert-type	7

R6813	9

New feature: Configuring the tag community attribute	9

Configuring the tag community attribute	9

About this task	9

Procedure (IPv4 unicast)	10

Procedure (IPv6 unicast)	11

Command reference	11

extcommunity-type tag	11

peer advertise tag	12

New feature: Applying an ACL to a list of VLAN interfaces for packet filtering	14

Applying an ACL to a list of VLAN interfaces for packet filtering	14

Command reference	15

packet-filter vlan-interface	15

Modified feature: Configuring a static group member	16

Feature change description	16

Command changes	17

Modified command: igmp static-group	17

Modified command: mld static-group	17

Modified feature: Redistributing IGP routes into the BGP routing table	18

Feature change description	18

Command changes	18

Modified command: import-route	18

Modified feature: Setting the port security mode of a port	19

Feature change description	19

Command changes	20

Modified command: port-security port-mode	20

Modified feature: Configuring the system operating mode	20

Feature change description	20

Command changes	20

Modified command: switch-mode	20

Modified feature: Configuring an IPv4 advanced ACL rule	21

Feature change description	21

Command changes	21

Modified command: rule (IPv4 advanced ACL view)	21

Modified feature: Configuring an IPv6 advanced ACL rule	22

Feature change description	22

Command changes	23

Modified command: rule (IPv6 advanced ACL view)	23

Modified feature: Displaying BGP EVPN routes	24

Feature change description	24

Command changes	24

Modified command: display bgp l2vpn evpn	24

- R6823

This release has the following changes:

- New feature: Enabling MAC authentication user logging on an interface

- New feature: Configuring the secure MAC aging timer in interface view

  - New feature: Enabling MAC authentication user logging on an interface

    - Enabling MAC authentication user logging on an interface

        - About this task

This feature enables the device to generate logs about MAC authentication users and send the logs to the information center. For the logs to be output correctly, you must also configure the information center on the device.  For more information about configuring the information center, see Network Management and Monitoring Configuration Guide.

        - Restrictions and guidelines

To prevent excessive MAC authentication user log entries, use this feature only if you need to analyze abnormal MAC authentication user logins or logouts.

This feature takes effect on an interface only if you enable it both globally and on that interface.

        - Procedure

          - Enter system view.

system-view

          - Enable MAC authentication user logging globally.

mac-authentication access-user log enable [ failed-login | ip-change | logoff | successful-login ] *

By default, MAC authentication user logging is disabled globally.

          - Disable MAC authentication user logging on an interface.

                - Enter interface view.

interface interface-type interface-number

                - Disable MAC authentication user logging on the interface.

undo mac-authentication access-user log enable [ failed-login | ip-change | logoff | successful-login ] *

By default, MAC authentication user logging is enabled on an interface.

    - Command reference

      - mac-authentication access-user log enable (interface-view)

Use mac-authentication access-user log enable to enable MAC authentication user logging on an interface.

Use undo mac-authentication access-user log enable to disable MAC authentication user logging on an interface.

Syntax

mac-authentication access-user log enable [ failed-login | ip-change | logoff | successful-login ] *

undo mac-authentication access-user log enable [ failed-login | ip-change | logoff | successful-login ] *

Default

MAC authentication user logging is enabled on an interface.

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

Parameters

failed-login: Logs MAC authentication user login failures.

ip-change: Logs user IP address changes.

logoff: Logs MAC authentication user logoffs.

successful-login: Logs successful MAC authentication user logins.

Usage guidelines

If you do not specify any parameters, this command enables all types of MAC authentication user logs.

This feature takes effect on an interface only if you enable it both globally and on that interface.

Examples

# Disable all types of MAC authentication user logging on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] undo mac-authentication access-user log enable

Related commands

info-center source maca logfile deny (Network Management and Monitoring Command Reference)

mac-authentication access-user log enable (system view)

- New feature: Configuring the secure MAC aging timer in interface view

  - Configuring the secure MAC aging timer in interface view

            - Enter system view.

system-view

            - Enter interface view.

interface interface-type interface-number

            - Configure a secure MAC address.

port-security mac-address security [ sticky ] mac-address vlan vlan-id

By default, no secure MAC address is configured.

            - Configure the secure MAC aging timer.

port-security timer autolearn aging second time-value

By default, secure MAC addresses do not age out.

  - Command reference

    - port-security timer autolearn aging (interface view)

Use port-security timer autolearn aging to configure the secure MAC aging timer.

Use undo port-security timer autolearn aging to restore the default.

Syntax

port-security timer autolearn aging second time-value

undo port-security timer autolearn aging

Default

Secure MAC addresses do not age out.

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Predefined user roles

network-admin

Parameters

second time-value: Specifies the secure MAC aging timer. The value range for the time-value argument is 10 to 7776000 in seconds.

Usage guidelines

Application scenarios

By default, all secure MAC addresses do not age out. You can enable regular aging of secure MAC addresses for the following purposes:

- Prevent inactive or fast moving secure MAC addresses from occupying system resources for a long time.

- Ensure that secure MAC users can come online when they move to a new port.

This enhances port access security and improves port resource usage.

Operating mechanism

The secure MAC address aging timer configured by using this command in interface view applies to all sticky and dynamic secure MAC addresses, including those automatically learned and manually configured, on the interface.

The effective aging timer can be either of the following values:

- The nearest multiple of 30 seconds to the configured aging timer if the configured timer is not less than 60 seconds. The effective aging timer is not less than the configured aging timer. For example, if you set the aging timer to 80 seconds, the actual aging timer that takes effect is 90 seconds.

- The configured aging timer if the configured timer is less than 60 seconds.

Restrictions and guidelines

A short aging time improves port access security and port resource usage but affects online user stability. Set an appropriate secure MAC address aging timer according to your device performance and the network environment.

As a best practice, do not assign a large value to the maximum number of secure MAC addresses permitted by port security when a short aging timer (less than 60 seconds) works with inactivity aging. A large value in this case might affect device performance.

When you configure the secure MAC address aging timer in both interface view and system view, the configuration in interface view takes priority. The actual aging timer that takes effect uses the value set in interface view.

Examples

# Set the secure MAC address aging timer to 50 seconds on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] port-security timer autolearn aging second 50

Related commands

display port-security

port-security mac-address security

- R6820

This release has no feature changes.

- R6817

This release has no feature changes.

- R6816

This release has the following changes:

- New feature: Enabling the MAC address entry conversion feature

New feature: Enabling the MAC address entry conversion feature

  - Enabling the MAC address entry conversion feature

      - About this task

After you enable this feature, the system displays MAC address entries obtained through MAC or 802.1X authentication as dynamic entries when the device queries the MAC address table through MIB or NETCONF.

Procedure

          - Enter system view.

system-view

          - Enable the MAC address entry conversion feature.

mac-address convert-type

By default, the MAC address entry conversion feature is disabled.

    - Command reference

mac-address convert-type

Use mac-address convert-type to enable the MAC address entry conversion feature.

Use undo mac-address convert-type to disable the MAC address entry conversion feature.

Syntax

mac-address convert-type

undo mac-address convert-type

Default

The MAC address entry conversion feature is disabled.

Views

System view

Predefined user roles

network-admin

mdc-admin

Usage guidelines

After you execute this command, the system displays MAC address entries obtained through MAC or 802.1X authentication as dynamic entries when the device queries the MAC address table through MIB or NETCONF.

Examples

# Enable the MAC address entry conversion feature.

<Sysname> system-view

[Sysname] mac-address convert-type

- R6813

This software version has the following changes:

- New feature: Configuring the tag community attribute

- New feature: Applying an ACL to a list of VLAN interfaces for packet filtering

- Modified feature: Configuring a static group member

- Modified feature: Redistributing IGP routes into the BGP routing table

- Modified feature: Setting the port security mode of a port

- Modified feature: Configuring the system operating mode

- Modified feature: Configuring an IPv4 advanced ACL rule

- Modified feature: Configuring an IPv6 advanced ACL rule

- Modified feature: Displaying BGP EVPN routes

- New feature: Configuring the tag community attribute

  - Configuring the tag community attribute

    - About this task

When routes are redistributed from a routing protocol into an IGP, the IGP can add a tag for them. This tag can be used for route filtering methods such as routing policies. Routing information is distributed across ASs after it is redistributed between IGPs and BGP. As a result, different route filtering methods might be applied to an IGP route when it exists in different ASs. To remove the impact of route location on the selection of route filtering methods, you can enable BGP to transmit the tag of IGP routes. After IGP routes are distributed to other ASs, IGP routes with the same tag will match the same route filtering method, regardless of their locations.

The tag community attribute is a proprietary extended community attribute, which is designed for transmitting IGP route tags.

|  | NOTE: For brief introduction, a BGP route carrying the tag community attribute is called tagged route. When BGP tags a route, it adds the tag community attribute to that route. When BGP untags a route, it removes the tag community attribute from that route. |
| --- | --- |

The peer advertise tag command functions as follows:

- Before advertising a BGP route derived from a redistributed IGP route to the specified peer or peer group, BGP tags that route. The value of the tag community attribute depends on the configuration of the import-route command.

- After receiving a tagged BGP route, BGP forwards that route to the specified peer or peer group without untagging.

If the peer advertise tag commands is not configured or the undo peer advertise tag command is configured, BGP untags BGP routes (if tagged) before advertising them to peers. In this situation, BGP does not support the following operations:

- Tags BGP routes before route advertisement.

- Advertises BGP routes without untagging.

Route tag information is distributed across ASs as shown in Figure 1.

          - Distributing route tag information

In the source AS, the network administrator redistributes IGP routes to BGP and enables BGP to inherit IGP route tags. To have BGP inherit IGP route tags, use one of the following methods:

  - Execute the import-route command with the inherit-tag keyword specified. BGP will directly inherit the original tags of redistributed IGP routes.

  - Execute the import-route command with the inherit-tag keyword and the route-policy route-policy-name option specified, and configure an apply tag clause for the specified routing policy. BGP will tag the BGP routes derived from redistributed IGP routes according to the configuration of that apply tag clause.

The inherited tag information forms a tag community attribute, which is inserted into BGP routes and transmitted to another AS.

The network administrator of the receiver AS redistributes the tagged BGP routes into the desired IGP. The generated IGP routes can inherit those BGP routes' tags. You can use multiple methods to set the tag of IGP routes derived from redistributed BGP routes. For an IGP route, the priorities of tags configured by using those methods are as follows:

Tag configured in the apply tag clause of the specified routing policy.

Tag specified by using the tag keyword in the import-route command configured for the IGP.

Tag inherited from the tag community attribute of the redistributed BGP route.

Tag specified by using the tag keyword in the route-tag command.

    - Procedure (IPv4 unicast)

          - Enter System view.

system-view

Enter BGP instance view.

bgp as-number [ instance instance-name ]

(Optional.) Configure the type value for the tag community attribute.

extcommunity-type tag tag-type-value

By default, the type value is 83cf in hexadecimal notation.

Enter BGP IPv4 unicast address family view or BGP-VPN IPv4 unicast address family view.

  - Enter BGP IPv4 unicast address family view.

address-family ipv4 [ unicast ]

  - Execute the following commands in sequence to enter BGP-VPN IPv4 unicast address family view:

ip vpn-instance vpn-instance-name

address-family ipv4 [ unicast ]

Redistribute IGP routes into the BGP routing table.

import-route { isis | ospf | rip } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ] [ inherit-tag ]

By default, IGP routes are not redistributed into BGP.

Enable BGP to advertise the tag community attribute to a peer or peer group.

peer { group-name | ipv4-address [ mask-length ] | ipv6-address [ prefix-length ] } advertise tag

By default, BGP does not advertise the tag community attribute to a peer or peer group.

    - Procedure (IPv6 unicast)

          - Enter system view.

system-view

Enter BGP instance view.

bgp as-number [ instance instance-name ]

(Optional.) Configure the type value for the tag community attribute.

extcommunity-type tag tag-type-value

By default, the type value is 83cf in hexadecimal notation.

Enter BGP IPv6 unicast address family view or BGP-VPN IPv6 unicast address family view.

  - Enter BGP IPv6 unicast address family view.

address-family ipv6 [ unicast ]

  - Execute the following commands in sequence to enter BGP-VPN IPv6 unicast address family view:

ip vpn-instance vpn-instance-name

address-family ipv6 [ unicast ]

Redistribute IGP routes into the BGP routing table.

import-route { isisv6 | ospfv3 | ripng } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ] [ inherit-tag ]

By default, IGP routes are not redistributed into BGP.

Enable BGP to advertise the tag community attribute to a peer or peer group.

peer { group-name | ipv4-address [ mask-length ] | ipv6-address [ prefix-length ] } advertise tag

By default, BGP does not advertise the tag community attribute to a peer or peer group.

  - Command reference

    - extcommunity-type tag

Use extcommunity-type tag to set the type value for the tag community attribute.

Use undo extcommunity-type tag to restore the default.

Syntax

extcommunity-type tag tag-type-value

undo extcommunity-type tag

Default

The type value for the tag community attribute is 83cf in hexadecimal notation.

Views

BGP instance view

Predefined user roles

network-admin

Parameters

tag-type-value: Specifies a type value in the range of 0 to ffff, which is in hexadecimal notation.

Usage guidelines

The tag community attribute is a proprietary extended community attribute. It might be unrecognizable for devices from other vendors or have conflicts with other private extended community attributes. You can use this command to adjust the type value to a recognizable or non-conflicting one for the tag community attribute.

Examples

# Set the type value to 0x5688 for the tag community attribute.

<Sysname> system-view

[Sysname] bgp 100

[Sysname-bgp-default] extcommunity-type tag 5688。

Related commands

peer advertise tag

    - peer advertise tag

Use peer advertise tag to enable BGP to advertise the tag community attribute to a peer or peer group.

Use undo peer advertise tag to restore the default.

Syntax

peer { group-name | ipv4-address [ mask-length ] | ipv6-address [ prefix-length ] } advertise tag

undo peer { group-name | ipv4-address [ mask-length ] | ipv6-address [ prefix-length ] } advertise tag

Default

BGP does not advertise the tag community attribute to a peer or peer group.

Views

BGP IPv4 unicast address family view

BGP-VPN IPv4 unicast address family view

BGP IPv6 unicast address family view

BGP-VPN IPv6 unicast address family view

BGP VPNv4 address family view

BGP VPNv6 address family view

BGP EVPN address family view

Predefined user roles

network-admin

Parameters

group-name: Specifies a peer group by its name, a case-sensitive string of 1 to 47 characters. The peer group must have been created.

ipv4-address: Specifies a peer by its IPv4 address. The peer must have been created.

mask-length: Specifies a mask length in the range of 0 to 32. You can use the ipv4-address and mask-length arguments together to specify a subnet. If you specify a subnet, all dynamic peers in the subnet are specified.

ipv6-address: Specifies a peer by its IPv6 address. The peer must have been created.

prefix-length: Specifies a prefix length in the range of 0 to 128. You can use the ipv6-address and prefix-length arguments together to specify a subnet. If you specify a subnet, all dynamic peers in the subnet are specified.

Usage guidelines

Application scenarios

When routes are redistributed from a routing protocol into an IGP, the IGP can add a tag for them. This tag can be used for route filtering methods such as routing policies. Routing information is distributed across ASs after it is redistributed between IGPs and BGP. As a result, different route filtering methods might be applied to an IGP route when it exists in different ASs. To remove the impact of route location on the selection of route filtering methods, you can enable BGP to transmit the tag of IGP routes. After IGP routes are distributed to other ASs, IGP routes with the same tag will match the same route filtering method, regardless of their locations.

Operating mechanism

The tag community attribute is a proprietary extended community attribute, which is designed for transmitting IGP route tags.

|  | NOTE: For brief introduction, a BGP route carrying the tag community attribute is called tagged route. When BGP tags a route, it adds the tag community attribute to that route. When BGP untags a route, it removes the tag community attribute from that route. |
| --- | --- |

The peer advertise tag command functions as follows:

- Before advertising a BGP route derived from a redistributed IGP route to the specified peer or peer group, BGP tags that route. The value of the tag community attribute depends on the configuration of the import-route command.

- After receiving a tagged BGP route, BGP forwards that route to the specified peer or peer group without untagging.

If the peer advertise tag commands is not configured or the undo peer advertise tag command is configured, BGP untags BGP routes (if tagged) before advertising them to peers. In this situation, BGP does not support the following operations:

- Tags BGP routes before route advertisement.

- Advertises BGP routes without untagging.

Restrictions and guidelines

The tag community attribute might be unrecognizable for devices from other vendors or have conflicts with other private extended community attributes. You can use the extcommunity-type tag command to adjust the type value to a recognizable or non-conflicting one for the tag community attribute.

Examples

# In BGP IPv4 unicast address family view, enable BGP to advertise the tag community attribute to peer group test.

<Sysname> system-view

[Sysname] bgp 100

[Sysname-bgp-default] address-family ipv4 unicast

[Sysname-bgp-default-ipv4] peer test advertise tag

Related commands

extcommunity-type tag

- New feature: Applying an ACL to a list of VLAN interfaces for packet filtering

Applying an ACL to a list of VLAN interfaces for packet filtering

      - Restrictions and guidelines

You can apply only one ACL to the same direction of VLAN interfaces.

You can use the packet-filter command in VLAN interface view or use the packet-filter vlan-interface command in system view to configure packet filtering in one direction of a VLAN interface. You cannot configure both of them in the same direction of a VLAN interface.

If you specify a VPN instance for an ACL rule, the ACL does not take effect. If you do not specify a VPN instance for an ACL rule, the rule applies to both public network and private network packets.

This feature is supported only if the switch operates in PACKET-FILTER VLAN-INTERFACE mode. For more information about system operating modes, see device management in Fundamentals Configuration Guide.

Repeating this command with one ACL for the same direction adds new VLAN interfaces to the list of VLAN interfaces:

If you specify the hardware-count keyword the first time you configure this command, you must specify this keyword when repeating this command.

If you do not specify the hardware-count keyword the first time you configure this command, do not specify this keyword when repeating this command.

      - Procedure

            - Enter system view.

system-view

            - Apply an ACL to a list of VLAN interfaces to filter packets.

packet-filter [ ipv6 | mac ] { acl-number | name acl-name } vlan-interface vlan-interface-list { inbound | outbound } [ hardware-count ]

By default, the system does not filter packets on a VLAN interface.

Command reference

    - packet-filter vlan-interface

Use packet-filter vlan-interface to apply an ACL to a list of VLAN interfaces to filter packets.

Use undo packet-filter vlan-interface to remove an ACL from a list of VLAN interfaces.

Syntax

packet-filter [ ipv6 | mac ] { acl-number | name acl-name } vlan-interface vlan-interface-list { inbound | outbound } [ hardware-count ]

undo packet-filter [ ipv6 | mac ] { acl-number | name acl-name } vlan-interface vlan-interface-list { inbound | outbound }

Default

No ACL is applied to a list of VLAN interfaces to filter packets.

Views

System view

Predefined user roles

network-admin

Parameters

ipv6: Specifies the IPv6 ACL type.

mac: Specifies the Layer 2 ACL type.

acl-number: Specifies an ACL by its number. The following are available value ranges:

- 2000 to 2999 for basic ACLs.

- 3000 to 3999 for advanced ACLs.

- 4000 to 4999 for Layer 2 ACLs.

name acl-name: Specifies an ACL by its name. The acl-name argument is a case-insensitive string of 1 to 63 characters.

vlan-interface vlan-interface-list: Specifies a space-separated list of up to eight VLAN interface items. Each item specifies a VLAN interface or a range of VLAN interfaces in the form of start-vlan-interface to end-vlan-interface.

inbound: Filters incoming packets.

outbound: Filters outgoing packets.

hardware-count: Enables counting ACL rule matches performed in hardware. If you do not specify this keyword, rule matches for the ACL are not counted in hardware.

Usage guidelines

If you use the acl-number argument to specify an ACL, follow these guidelines:

- To specify an IPv4 ACL, use the acl-number argument directly.

- To specify an IPv6 ACL, specify the ipv6 keyword, and then the acl-number argument.

- To specify a Layer 2 ACL, the mac keyword is not a must. You can either specify the mac keyword and then the acl-number argument or specify only the acl-number argument.

If you use the name acl-name option to specify an ACL, follow these guidelines:

- To specify an IPv4 ACL, use the name acl-name option.

- To specify an IPv6, or Layer 2 ACL, specify the related keyword and then the name acl-name option.

You can use the packet-filter command in VLAN interface view or use the packet-filter vlan-interface command in system view to configure packet filtering in one direction of a VLAN interface. You cannot configure both of them in the same direction of a VLAN interface.

You can apply only one ACL to the same direction of VLAN interfaces.

If you specify a VPN instance for an ACL rule, the ACL does not take effect. If you do not specify a VPN instance for an ACL rule, the rule applies to both public network and private network packets.

This feature is supported only if the switch operates in PACKET-FILTER VLAN-INTERFACE mode. For more information about system operating modes, see device management in Fundamentals Configuration Guide.

The hardware-count keyword in this command enables match counting in hardware for all rules in an ACL, and the counting keyword in the rule command enables match counting specific to rules.

With the packet-filter vlan-interface command, you can configure only two filters: one for each direction. You can repeat the command to add VLAN interfaces to the packet filter in each direction. As a best practice to save resources, use the packet-filter vlan-interface command to configure packet filtering for VLAN interfaces that share the same packet filtering ACL.

For a packet filter, the use of the hardware-count keyword must be consistent across all its VLAN interfaces. You must specify the hardware-count keyword for all its VLAN interfaces or none of its VLAN interfaces.

A list of VLAN interfaces can contain up to eight VLAN interface items. Each item has at least one VLAN interface. Follow these restrictions and guidelines when you use the undo packet-filter vlan-interface command:

- You can specify the entire VLAN interface list to remove the ACL from all VLAN interfaces in the list.

- You can specify one or more VLAN interface items of the list to remove the ACL from the specified VLAN interfaces.

- For a VLAN interface item with multiple VLAN interfaces, you cannot remove the ACL from only some of the VLAN interfaces of the VLAN interface item.

To disable the ACL rule match counting in hardware when resources are insufficient, you must execute the undo packet-filter command and then reconfigure the packet-filter command without specifying the hardware-count keyword.

To disable the ACL rule match counting in hardware when resources are sufficient, you can directly reconfigure the packet-filter command without specifying the hardware-count keyword.

Examples

# Apply IPv4 basic ACL 2003 to filter incoming traffic on VLAN interfaces 3 through 10, and enable counting ACL rule matches performed in hardware.

<Sysname> system-view

[Sysname] packet-filter 2003 vlan-interface 3 to 10 inbound hardware-count

- Modified feature: Configuring a static group member

Feature change description

As from this release, you can configure a static group member for multiple multicast groups in bulk.

Command changes

Modified command: igmp static-group

Old syntax

igmp static-group group-address [ source source-address ]

undo igmp static-group { all | group-address [ source source-address ]

New syntax

igmp static-group group-address [ inc-step-mask { group-mask | group-mask-length } number group-number ] [ source source-address ]

undo igmp static-group { all | group-address [ inc-step-mask { group-mask | group-mask-length } number group-number ] [ source source-address ]

Views

Interface view

Change description

Before modification: The inc-step-mask { group-mask | group-mask-length } number group-number option is not supported.

After modification: The inc-step-mask { group-mask | group-mask-length } number group-number option is supported.

inc-step-mask group-mask: Specifies the mask step for multicast groups in batch configuration, the gap between two adjacent multicast group addresses. The step mask is in the dotted decimal notation format, in the range of 0.0.0.1 to 15.255.255.255.

inc-step-mask group-mask-length: Specifies the mask length step for multicast groups in batch configuration. The mask length step is an integer in the range of 5 to 32.

group-number: Specifies the number of multicast groups in batch configuration, an integer in the range of 2 to 512.

Modified command: mld static-group

Old syntax

mld static-group ipv6-group-address [ source ipv6-source-address ]

undo mld static-group { all | ipv6-group-address [ source ipv6-source-address ] }

New syntax

mld static-group ipv6-group-address [ inc-step-mask group-mask-length number group-number ] [ source ipv6-source-address ]

undo mld static-group { all | ipv6-group-address [ inc-step-mask  group-mask-length number group-number ] [ source ipv6-source-address ] }

Views

Interface view

Change description

Before modification: The inc-step-mask group-mask-length number group-number option is not supported.

After modification: The inc-step-mask group-mask-length number group-number option is supported.

inc-step-mask group-mask-length: Specifies the mask length step for IPv6 multicast groups in batch configuration. The mask length is an integer in the range of 9 to 128.

number group-number: Specifies the number of IPv6 multicast groups in batch configuration, an integer in the range of 2 to 512.

Modified feature: Redistributing IGP routes into the BGP routing table

Feature change description

As from this release, the import-route command supports the inherit-tag keyword. If you specify this keyword, BGP will add the tag community attribute for BGP routes derived from redistributed IGP routes, inheriting the original IGP route tags.

  - Command changes

    - Modified command: import-route

Old syntax

In BGP IPv4 unicast address family view/BGP-VPN IPv4 unicast address family view/BGP IPv4 multicast address family view:

import-route { isis | ospf | rip } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ]

import-route { direct | static } [ med med-value | route-policy route-policy-name ] *

undo import-route { direct | { isis | ospf | rip } [ process-id | all-processes ] | static }

In BGP IPv6 unicast address family view/BGP-VPN IPv6 unicast address family view/BGP IPv6 multicast address family view:

import-route { isisv6 | ospfv3 | ripng } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ]

import-route { direct | static } [ med med-value | route-policy route-policy-name ] *

undo import-route { direct | { isisv6 | ospfv3 | ripng } [ process-id | all-processes ] | static }

New syntax

In BGP IPv4 unicast address family view/BGP-VPN IPv4 unicast address family view:

import-route { isis | ospf | rip } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ] [ inherit-tag ]

import-route { direct | static } [ med med-value | route-policy route-policy-name ] *

undo import-route { direct | { isis | ospf | rip } [ process-id | all-processes ] | static }

In BGP IPv4 multicast address family view:

import-route { isis | ospf | rip } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ]

import-route { direct | static } [ med med-value | route-policy route-policy-name ] *

undo import-route { direct | { isis | ospf | rip } [ process-id | all-processes ] | static }

In BGP IPv4 unicast address family view/BGP-VPN IPv4 unicast address family view/BGP IPv4 multicast address family view:

import-route local-aggregate [ route-policy route-policy-name ]

undo import-route local-aggregate

In BGP IPv6 unicast address family view/BGP-VPN IPv6 unicast address family view:

import-route { isisv6 | ospfv3 | ripng } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ] [ inherit-tag ]

import-route { direct | static } [ med med-value | route-policy route-policy-name ] *

undo import-route { direct | { isisv6 | ospfv3 | ripng } [ process-id | all-processes ] | static }

In BGP IPv6 multicast address family view:

import-route { isisv6 | ospfv3 | ripng } [ { process-id | all-processes } [ allow-direct | med med-value | route-policy route-policy-name ] * ]

import-route { direct | static } [ med med-value | route-policy route-policy-name ] *

undo import-route { direct | { isisv6 | ospfv3 | ripng } [ process-id | all-processes ] | static }

Views

BGP IPv4 unicast address family view

BGP-VPN IPv4 unicast address family view

BGP IPv6 unicast address family view

BGP-VPN IPv6 unicast address family view

BGP IPv4 multicast address family view

BGP IPv6 multicast address family view

Change description

Before modification: The inherit-tag keyword is not supported.

After modification: The inherit-tag keyword is supported.

- Modified feature: Setting the port security mode of a port

  - Feature change description

As from this release, the userlogin-mab keyword is supported when you set the port security mode of a port.

  - Command changes

    - Modified command: port-security port-mode

Old syntax

port-security port-mode { autolearn | mac-and-userlogin-secure-ext | mac-authentication | mac-else-userlogin-secure | mac-else-userlogin-secure-ext | secure | userlogin | userlogin-secure | userlogin-secure-ext | userlogin-secure-or-mac | userlogin-secure-or-mac-ext | userlogin-withoui }

undo port-security port-mode

New syntax

port-security port-mode { autolearn | mac-and-userlogin-secure-ext | mac-authentication | mac-else-userlogin-secure | mac-else-userlogin-secure-ext | secure | userlogin | userlogin-mab | userlogin-secure | userlogin-secure-ext | userlogin-secure-or-mac | userlogin-secure-or-mac-ext | userlogin-withoui }

undo port-security port-mode

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Change description

Before modification: The userlogin-mab keyword is not supported.

After modification: The userlogin-mab keyword is supported.

- Modified feature: Configuring the system operating mode

  - Feature change description

As from this release, you can specify PACKET-FILTER VLAN-INTERFACE as the system operating mode for the device.

  - Command changes

    - Modified command: switch-mode

Old syntax

switch-mode { 0 | 1 | 3 | 4 | 5 | 6 }

New syntax

switch-mode { 0 | 1 | 3 | 4 | 5 | 6 | 7 }

Views

System view

Change description

Before modification: You cannot specify PACKET-FILTER VLAN-INTERFACE as the system operating mode for the device.

After modification: You can specify PACKET-FILTER VLAN-INTERFACE as the system operating mode for the device.

In this mode, the ACL resources are primarily used when you apply an ACL to a list of VLAN interfaces to filter packets. Microsegmentation is not supported. For more information about ACLs, see ACL and QoS Configuration Guide.

- Modified feature: Configuring an IPv4 advanced ACL rule

  - Feature change description

As from this release, you can configure an IPv4 advanced ACL rule to match the IPv4 packet length (the Total Length field in the packet header).

  - Command changes

    - Modified command: rule (IPv4 advanced ACL view)

Old syntax

rule [ rule-id ] { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { object-group address-group-name | dest-address dest-wildcard | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | { dscp dscp | { precedence precedence | tos tos } * } | fragment | icmp-type { icmp-type [ icmp-code ] | icmp-message } | [ flow-logging | logging ] | source { object-group address-group-name | source-address source-wildcard | any } | source-port { object-group port-group-name | operator port1 [ port2 ] } | time-range time-range-name | vpn-instance vpn-instance-name ] *

undo rule rule-id [ { { ack | fin | psh | rst | syn | urg } * | established } | counting | destination | destination-port | { dscp | { precedence | tos } * } | fragment | icmp-type | [ flow-logging | logging ] | source | source-port | time-range | vpn-instance  ] *

undo rule { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { object-group address-group-name | dest-address dest-wildcard | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | { dscp dscp | { precedence precedence | tos tos } * } | fragment | icmp-type { icmp-type [ icmp-code ] | icmp-message } | [ flow-logging | logging ] | source { object-group address-group-name | source-address source-wildcard | any } | source-port operator port1 [ port2 ] | time-range time-range-name | vpn-instance vpn-instance-name ] *

New syntax

rule [ rule-id ] { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { object-group address-group-name | dest-address dest-wildcard | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | { dscp dscp | { precedence precedence | tos tos } * } | fragment | icmp-type { icmp-type [ icmp-code ] | icmp-message } | [ flow-logging | logging ] | packet-length operator length-value1 [ length-value2 ] | source { object-group address-group-name | source-address source-wildcard | any } | source-port { object-group port-group-name | operator port1 [ port2 ] } | time-range time-range-name | vpn-instance vpn-instance-name ] *

undo rule rule-id [ { { ack | fin | psh | rst | syn | urg } * | established } | counting | destination | destination-port | { dscp | { precedence | tos } * } | fragment | icmp-type | [ flow-logging | logging ] | packet-length | source | source-port | time-range | vpn-instance  ] *

undo rule { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { object-group address-group-name | dest-address dest-wildcard | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | { dscp dscp | { precedence precedence | tos tos } * } | fragment | icmp-type { icmp-type [ icmp-code ] | icmp-message } | [ flow-logging | logging ] | packet-length operator length-value1 [ length-value2 ] | source { object-group address-group-name | source-address source-wildcard | any } | source-port operator port1 [ port2 ] | time-range time-range-name | vpn-instance vpn-instance-name ] *

Views

IPv4 advanced ACL view

Change description

Before modification: The packet-length operator length-value1 [ length-value2 ] parameter is not supported.

After modification: The packet-length operator length-value1 [ length-value2 ] parameter is supported.

packet-length operator length-value1 [ length-value2 ]: Specifies a packet length range used to match the Total Length field value in the IPv4 packet header. The operator argument can be lt (lower than), gt (greater than), eq (equal to), neq (not equal to), or range (inclusive range). The value ranges for the length-value1 and length-value2 arguments are both 0 to 16383. The length-value2 argument is needed only when the operator argument is range.

- Modified feature: Configuring an IPv6 advanced ACL rule

  - Feature change description

As from this release, you can configure an IPv6 advanced ACL rule to match the IPv6 packet length, including the basic header, extension headers, and payload.

  - Command changes

    - Modified command: rule (IPv6 advanced ACL view)

Old syntax

rule [ rule-id ] { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { dest-address dest-prefix | dest-address/dest-prefix | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | dscp dscp | flow-label flow-label-value | fragment | icmp6-type { icmp6-type icmp6-code | icmp6-message } | [ flow-logging | logging ] | routing [ type routing-type ] | hop-by-hop [ type hop-type ] | source { object-group address-group-name | source-address source-prefix | source-address/source-prefix | any } | source-port { object-group port-group-name | operator port1 [ port2 ] } | time-range time-range-name | vpn-instance vpn-instance-name ] *

undo rule rule-id [ { { ack | fin | psh | rst | syn | urg } * | established } | counting | destination | destination-port | dscp | flow-label | fragment | icmp6-type | [ flow-logging | logging ] | routing | hop-by-hop | source | source-port | time-range | vpn-instance ] *

undo rule { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { object-group address-group-name | dest-address dest-prefix | dest-address/dest-prefix | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | dscp dscp | flow-label flow-label-value | fragment | icmp6-type { icmp6-type icmp6-code | icmp6-message } | [ flow-logging | logging ] | routing [ type routing-type ] | hop-by-hop [ type hop-type ] | source { object-group address-group-name | source-address source-prefix | source-address/source-prefix | any } | source-port { object-group port-group-name | operator port1 [ port2 ] } | time-range time-range-name | vpn-instance vpn-instance-name ] *

New syntax

rule [ rule-id ] { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { dest-address dest-prefix | dest-address/dest-prefix | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | dscp dscp | flow-label flow-label-value | fragment | icmp6-type { icmp6-type icmp6-code | icmp6-message } | [ flow-logging | logging ] | packet-length operator length-value1 [ length-value2 ] | routing [ type routing-type ] | hop-by-hop [ type hop-type ] | source { object-group address-group-name | source-address source-prefix | source-address/source-prefix | any } | source-port { object-group port-group-name | operator port1 [ port2 ] } | time-range time-range-name | vpn-instance vpn-instance-name ] *

undo rule rule-id [ { { ack | fin | psh | rst | syn | urg } * | established } | counting | destination | destination-port | dscp | flow-label | fragment | icmp6-type | [ flow-logging | logging ] | packet-length | routing | hop-by-hop | source | source-port | time-range | vpn-instance ] *

undo rule { deny | permit } protocol [ { { ack ack-value | fin fin-value | psh psh-value | rst rst-value | syn syn-value | urg urg-value } * | established } | counting | destination { object-group address-group-name | dest-address dest-prefix | dest-address/dest-prefix | any } | destination-port { object-group port-group-name | operator port1 [ port2 ] } | dscp dscp | flow-label flow-label-value | fragment | icmp6-type { icmp6-type icmp6-code | icmp6-message } | [ flow-logging | logging ] | packet-length operator length-value1 [ length-value2 ] | routing [ type routing-type ] | hop-by-hop [ type hop-type ] | source { object-group address-group-name | source-address source-prefix | source-address/source-prefix | any } | source-port { object-group port-group-name | operator port1 [ port2 ] } | time-range time-range-name | vpn-instance vpn-instance-name ] *

Views

IPv6 advanced ACL view

Change description

Before modification: The packet-length operator length-value1 [ length-value2 ] parameter is not supported.

After modification: The packet-length operator length-value1 [ length-value2 ] parameter is supported.

packet-length operator length-value1 [ length-value2 ]: Specifies a packet length range used to match the IPv6 packet length, including the basic header, extension headers, and payload. The operator argument can be lt (lower than), gt (greater than), eq (equal to), neq (not equal to), or range (inclusive range). The value ranges for the length-value1 and length-value2 arguments are both 0 to 16383. The length-value2 argument is needed only when the operator argument is range.

- Modified feature: Displaying BGP EVPN routes

  - Feature change description

As from this release, the Ext-Community field in the output of the display bgp l2vpn evpn command supports the Tag and Unknown type values.

  - Command changes

    - Modified command: display bgp l2vpn evpn

Syntax

display bgp [ instance instance-name ] l2vpn evpn [ peer { ipv4-address | ipv6-address } { advertised-routes | received-routes } [ statistics ] | [ route-distinguisher route-distinguisher | route-type { auto-discovery | es | igmp-ls | igmp-js | imet | ip-prefix | mac-ip | s-pmsi | smet } ] * [ { evpn-route route-length | evpn-prefix } [ advertise-info | as-path | cluster-list | community | ext-community ] | { ipv4-address | ipv6-address | mac-address } [ verbose ] ] | statistics ]

display bgp [ instance instance-name ] l2vpn evpn [ route-distinguisher route-distinguisher ] [ statistics ] community [ community-number&<1-32> | aa:nn&<1-32> ] [ internet | no-advertise | no-export | no-export-subconfed ] [ whole-match ]

display bgp [ instance instance-name ] l2vpn evpn [ route-distinguisher route-distinguisher ] [ statistics ] community-list { basic-community-list-number | adv-community-list-number | comm-list-name } [ whole-match ]

display bgp [ instance instance-name ] l2vpn evpn [ route-distinguisher route-distinguisher ] [ statistics ] ext-community [ bandwidth link-bandwidth-value | rt route-target | soo site-of-origin]&<1-32> [ whole-match ]

Views

Any view

Change description

Before modification: The Ext-Community field does not support the Tag or Unknown type value.

After modification: The Ext-Community field supports the Tag and Unknown type values.

- Tag—Indicates the tag community attribute. The content includes the type value and the tag value.

- Unknown type—Indicates an unrecognizable extended community attribute.