HPE 5140HI-CMW710-R6652P05 Release Notes

Software Feature Changes

The information in this document is subject to change without notice. © Copyright 2024 Hewlett Packard Enterprise Development LP

Contents

R6652P05	1

Modified feature: Specifying the cipher suite for MACsec encryption	1

Feature change description	1

Command changes	1

Modified command: macsec cipher-suite	1

Modified feature: Displaying BGP dedicated routing information	1

Feature change description	1

R6652P03	1

New feature: Specifying the cipher suite for MACsec encryption	1

Specifying the cipher suite for MACsec encryption	1

Command reference	1

macsec cipher-suite	1

R6628P43	1

R6628P40	2

R6628P35	3

New feature: Enabling packet statistics for a Layer 3 aggregate subinterface	3

Enabling packet statistics for a Layer 3 aggregate subinterface	3

Command reference	4

traffic-statistic enable	4

New feature: Object groups	4

Configuring object groups	4

Configuring an IPv4 address object group	4

Configuring an IPv6 address object group	5

Configuring a port object group	5

Configuring a service object group	5

Display and maintenance commands for object groups	6

Command reference	6

description	6

display object-group	6

network (IPv4 address object group view)	8

network (IPv6 address object group view)	9

object-group	10

port (port object group view)	11

service (service object group view)	13

Modified feature: Applying a QoS policy to an interface	15

Feature change description	15

Command changes	15

Modified feature: Configuring the authentication load sharing mode for users attached to DRNI interfaces	15

Feature change description	15

Command changes	16

Modified command: port-security drni load-sharing-mode	16

Modified feature: Setting the maximum number of configuration archives for local archiving	16

Feature change description	16

Command changes	16

Modified command: archive configuration max	16

Modified feature: Displaying detailed next hop information in the RIB or for direct routes or static routes	16

Feature change description	16

Command changes	17

Modified command: display rib nib	17

Modified command: display route-direct nib	17

Modified command: display ipv6 rib nib	18

Modified command: display ipv6 route-direct nib	18

Modified command: display route-static nib	18

Modified command: display ipv6 route-static nib	19

Modified feature: Optimizing BMP server information display	19

Feature change description	19

Command changes	20

Modified command: display bgp bmp server	20

Modified feature: Setting the 802.1X periodic reauthentication timer	20

Feature change description	20

Command changes	21

Modified command: dot1x timer reauth-period (interface view)	21

Modified command: dot1x timer	21

Modified feature: Setting the periodic MAC reauthentication timer	22

Feature change description	22

Command changes	22

Modified command: mac-authentication timer (interface view)	22

Modified command: mac-authentication timer (system view)	22

Modified feature: Displaying local public keys	23

Feature change description	23

Command changes	23

Modified command: display public-key local public	23

Modified feature: Value range for the ID of an IPsec tunnel	23

Feature change description	23

Command changes	23

Modified command: display ipsec statistics	23

Modified command: display ipsec tunnel	23

Modified command: reset ipsec statistics	24

Modified feature: Configuring automatic email sending parameters for an EAA monitor policy	24

Feature change description	24

Command changes	24

Modified command: rtm email domain	24

Modified feature: Setting the usage alarm threshold of a log file	24

Feature change description	24

Command changes	25

Modified command: info-center logfile alarm-threshold	25

Modified feature: Displaying heap memory usage for a user process	25

Feature change description	25

Command changes	25

Modified command: display process memory heap	25

Modified feature: Displaying gRPC information	25

Feature change description	25

Command changes	26

R6615P07	27

Removed feature: Enabling the SSH server to support SSH1 clients	27

Feature change description	27

Deleted command	27

ssh server compatible-ssh1x enable	27

Removed feature: Setting the minimum interval for updating the RSA server key pair	27

Feature change description	27

Deleted command	27

ssh server rekey-interval	27

R6615P06	28

- R6652P05

This software version has the following changes:

- Modified feature: Specifying the cipher suite for MACsec encryption

- Modified feature: Displaying BGP dedicated routing information

- Modified feature: Specifying the cipher suite for MACsec encryption

- Feature change description

Added support for the standard GCM-AES-256 and GCM-AES-XPN-256 cipher suites.

- Command changes

- Modified command: macsec cipher-suite

Old syntax

macsec cipher-suite { gcm-aes-128 | gcm-aes-256 | gcm-aes-xpn-128 | gcm-aes-xpn-256 }

New syntax

macsec cipher-suite { gcm-aes-128 | gcm-aes-256 [ standard ] | gcm-aes-xpn-128 | gcm-aes-xpn-256 [ standard ] }

Views

Ethernet interface view

Change description

The standard keyword was added to the macsec cipher-suite command.

standard: Specifies the standard cipher suite. H3C's GCM-AES-256 and GCM-AES-XPN-256 cipher suites differ from the standard open source GCM-AES-256 and GCM-AES-XPN-256 cipher suites in implementation, respectively. To successfully establish MKA sessions with a peer using the standard open source GCM-AES-256 or GCM-AES-XPN-256 cipher suite for MACsec encryption, you must specify the standard keyword in the macsec cipher-suite command.

- Modified feature: Displaying BGP dedicated routing information

- Feature change description

The S – stale route state is added to the output from the display bgp routing-table dedicated command. This state indicates stale routes.

- R6652P03

This release has the following changes:

- New feature: Specifying the cipher suite for MACsec encryption

New feature: Specifying the cipher suite for MACsec encryption

  - Specifying the cipher suite for MACsec encryption

      - About this task

MACsec uses the GCM-AES-128 or GCM-AES-256 cipher suite to encrypt, validate, and decrypt protected data frames.

      - Restrictions and guidelines

Only the HPE FlexNetwork 5140/5520 10GBASE-T MACsec 2p Mod (R9L65A) modules support the GCM-AES-256 cipher suite.

This feature is supported only in device-oriented mode. Do not configure this feature on an 802.1X-enabled port.

Make sure the connected MACsec ports are configured with the same cipher suite. If the ports are configured with different cipher suites, they cannot successfully establish MKA sessions.

      - Procedure

            - Enter system view.

system-view

            - Enter interface view.

interface interface-type interface-number

            - Specify the cipher suite for MACsec encryption.

macsec cipher-suite { gcm-aes-128 | gcm-aes-256 }

By default, MACsec uses the GCM-AES-128 cipher suite for encryption.

  - Command reference

    - macsec cipher-suite

Use macsec cipher-suite to specify the cipher suite for MACsec encryption.

Use undo macsec cipher-suite to restore the default.

Syntax

macsec cipher-suite { gcm-aes-128 | gcm-aes-256 }

undo macsec cipher-suite

Default

MACsec uses the GCM-AES-128 cipher suite for encryption.

Views

Ethernet interface view

Predefined user roles

network-admin

Parameters

gcm-aes-128: Specifies the GCM-AES-128 cipher suite.

gcm-aes-256: Specifies the GCM-AES-256 cipher suite.

Usage guidelines

Only the HPE FlexNetwork 5140/5520 10GBASE-T MACsec 2p Mod (R9L65A) modules support the GCM-AES-256 cipher suite.

Do not use this command on an 802.1X-enabled port.

This command is supported only in device-oriented mode. Make sure the connected ports are configured with the same cipher suite. If the ports are configured with different cipher suites, they cannot successfully establish MKA sessions.

Examples

# Specify the GCM-AES-256 cipher suite for MACsec encryption on GigabitEthernet 1/0/1.

<Sysname> system-view

[Sysname] interface gigabitethernet 1/0/1

[Sysname-GigabitEthernet1/0/1] macsec cipher-suite gcm-aes-256

- R6628P43

This release has no feature changes.

- R6628P40

This release has no feature changes.

- R6628P35

This software version has the following changes:

- New feature: Enabling packet statistics for a Layer 3 aggregate subinterface

- New feature: Object groups

- Modified feature: Applying a QoS policy to an interface

- Modified feature: Configuring the authentication load sharing mode for users attached to DRNI interfaces

- Modified feature: Setting the maximum number of configuration archives for local archiving

- Modified feature: Displaying detailed next hop information in the RIB or for direct routes or static routes

- Modified feature: Optimizing BMP server information display

- Modified feature: Setting the 802.1X periodic reauthentication timer

- Modified feature: Setting the periodic MAC reauthentication timer

- Modified feature: Displaying local public keys

- Modified feature: Value range for the ID of an IPsec tunnel

- Modified feature: Configuring automatic email sending parameters for an EAA monitor policy

- Modified feature: Setting the usage alarm threshold of a log file

- Modified feature: Displaying heap memory usage for a user process

- Modified feature: Displaying gRPC information

  - New feature: Enabling packet statistics for a Layer 3 aggregate subinterface

    - Enabling packet statistics for a Layer 3 aggregate subinterface

      - Restrictions and guidelines

The packet statistics feature is CPU intensive. When you use this command for Layer 3 aggregate subinterfaces, make sure you fully understand its impact on system performance. You can use the flow-interval command to adjust the interval at which the statistics are polled. To conserve hardware resources, increase the polling interval.

      - Procedure

          - Enter system view.

system-view

          - Enter Layer 3 aggregate subinterface view.

interface route-aggregation interface-number.subnumber

          - Enable packet statistics for the Layer 3 aggregate subinterface.

traffic-statistic enable

By default, the packet statistics feature is disabled for a Layer 3 aggregate subinterface.

          - (Optional.) Display the packet statistics for the Layer 3 aggregate subinterface.

display interface

In the command output, the Input and Output fields display packet statistics.

- Command reference

    - traffic-statistic enable

Use traffic-statistic enable to enable packet statistics for a Layer 3 aggregate subinterface.

Use undo traffic-statistic enable to disable packet statistics for a Layer 3 aggregate subinterface.

Syntax

traffic-statistic enable

undo traffic-statistic enable

Default

The packet statistics feature is disabled for a Layer 3 aggregate subinterface.

Views

Layer 3 aggregate subinterface view

Predefined user roles

network-admin

Usage guidelines

The packet statistics feature is CPU intensive. When you use this command for Layer 3 aggregate subinterfaces, make sure you fully understand its impact on system performance. You can use the flow-interval command to adjust the interval at which the statistics are polled. To conserve hardware resources, increase the polling interval.

To view packet statistics of Layer 3 aggregate subinterfaces, use the display interface command.

Examples

# Enable packet statistics for Layer 3 aggregate subinterface Route-Aggregation 1.1.

<Sysname> system-view

[Sysname] interface route-aggregation 1.1

[Sysname-Route-Aggregation1.1] traffic-statistic enable

Related commands

display interface

flow-interval

- New feature: Object groups

- Configuring object groups

      - Configuring an IPv4 address object group

          - Enter system view.

system-view

          - Configure an IPv4 address object group and enter its view.

object-group ip address object-group-name

The system has one default IPv4 address object group named any.

          - (Optional.) Configure a description for the IPv4 address object group.

description text

By default, an object group does not have a description.

          - Configure an IPv4 address object.

[ object-id ] network { host { address ip-address | name host-name } | subnet ip-address { mask-length | mask } }

      - Configuring an IPv6 address object group

          - Enter system view.

system-view

          - Configure an IPv6 address object group and enter its view.

object-group ipv6 address object-group-name

The system has one default IPv6 address object group named any.

          - (Optional.) Configure a description for the IPv6 address object group.

description text

By default, an object group does not have a description.

          - Configure an IPv6 address object.

[ object-id ] network { host { address ipv6-address | name host-name } | subnet ipv6-address prefix-length }

    - Configuring a port object group

          - Enter system view.

system-view

          - Configure a port object group and enter its view.

object-group port object-group-name

The system has one default port object group named any.

          - (Optional.) Configure a description for the port object group.

description text

By default, an object group does not have a description.

          - Configure a port object.

[ object-id ] port { { eq | lt | gt } port | range port1 port2 }

    - Configuring a service object group

          - Enter system view.

system-view

          - Configure a service object group and enter its view.

object-group service object-group-name

The system has multiple default service object groups.

          - (Optional.) Configure a description for the service object group.

description text

By default, an object group does not have a description.

          - Configure a service object.

[ object-id ] service { protocol [ { source { { eq | lt | gt } port | range port1 port2 } | destination { { eq | lt | gt | } port | range port1 port2 } } * | icmp-type icmp-code | icmpv6-type icmpv6-code ] }

    - Display and maintenance commands for object groups

Execute display commands in any view.

| Task | Command |
| --- | --- |
| Display information about object groups. | display object-group [ { { ip ¦ ipv6 } address ¦ service ¦ port } [ default ] [ name object-group-name ] ¦ name object-group-name ] |

‌

    - Command reference

      - description

Use description to configure a description for an object group.

Use undo description to restore the default.

Syntax

description text

undo description

Default

No description is configured for an object group.

Views

Object group view

Predefined user roles

network-admin

Parameters

text: Specifies a description, a case-sensitive string of 1 to 127 characters.

Examples

# Configure the description as This is an IPv4 object-group for an IPv4 address object group.

<Sysname> system-view

[Sysname] object-group ip address ipgroup

[Sysname-obj-grp-ip-ipgroup] description This is an IPv4 object-group

      - display object-group

Use display object-group to display information about object groups.

Syntax

display object-group [ { { ip | ipv6 } address | service | port } [ default ] [ name object-group-name ] | name object-group-name ]

Views

Any view

Predefined user roles

network-admin

network-operator

Parameters

ip address: Specifies the IPv4 address object groups.

ipv6 address: Specifies the IPv6 address object groups.

port: Specifies the port object groups.

service: Specifies the service object groups.

default: Specifies the default object groups.

name object-group-name: Specifies an object group by its name, a case-insensitive string of 1 to 31 characters.

Examples

# Display information about all object groups.

<Sysname> display object-group

IP address object group obj1: 0 object(in use)

IP address object group obj2: 7 objects(out of use)

0 network host address 1.1.1.1

10 network host name host

20 network subnet 1.1.1.1 255.255.255.0

IPv6 address object-group obj3: 0 object(in use)

IPv6 address object-group obj4: 5 objects(out of use)

0 network host address 1::1:1

10 network host name host

20 network subnet 1::1:0 112

Service object-group obj5: 0 object(in use)

Service object-group obj6: 6 objects(out of use)

0 service 200

10 service tcp source lt 50 destination range 30 40

20 service udp source range 30 40 destination gt 30

30 service icmp 20 20

40 service icmpv6 20 20

Port object-group obj7: 0 object(in use)

Port object-group obj8: 3 objects(out of use)

0 port lt 20

10 port range 20 30

# Display information about object group obj2.

<Sysname> display object-group name obj2

IP address object-group obj2: 6 objects(out of use)

0 network host address 1.1.1.1

10 network host name host

20 network subnet 1.1.1.1 255.255.255.0

# Display information about all IPv4 address object groups.

<Sysname> display object-group ip address

IP address object-group obj1: 0 object(in use)

IP address object-group obj2: 6 objects(out of use)

0 network host address 1.1.1.1

10 network host name host

20 network subnet 1.1.1.1 255.255.255.0

# Display information about IPv6 address object group obj4.

<Sysname> display object-group ipv6 address name obj4

IPv6 address object-group obj4: 5 objects(out of use)

0 network host address 1::1:1

10 network host name host

20 network subnet 1::1:0 112

            - Command output

| Field | Description |
| --- | --- |
| in use | The object group is used by an ACL or object group. |
| out of use | The object group is not used. |

      - network (IPv4 address object group view)

Use network to configure an IPv4 address object.

Use undo network to delete an IPv4 address object.

Syntax

[ object-id ] network { host { address ip-address | name host-name } | subnet ip-address { mask-length | mask } }

undo network { host { address ip-address | name host-name } | subnet ip-address { mask-length | mask } }

undo object-id

Default

No IPv4 address objects exist.

Views

IPv4 address object group view

Predefined user roles

network-admin

Parameters

object-id: Specifies an object ID in the range of 0 to 4294967294. If you do not specify an object ID, the system automatically assigns the object a multiple of 10 next to the greatest ID being used. For example, if the greatest ID is 22, the system automatically assigns 30.

host: Configures an IPv4 address object with the host address or name.

address ip-address: Specifies an IPv4 host address.

name host-name: Specifies a host name, a case-insensitive string of 1 to 60 characters.

subnet ip-address { mask-length | mask }: Configures an IPv4 address object with the subnet address followed by a mask length in the range of 0 to 32 or a mask in dotted decimal notation.

Usage guidelines

This command fails if you use it to configure or change an IPv4 address object to be identical with an existing object.

This command creates an IPv4 address object if the specified object ID does not exist. Otherwise, the command overwrites the configuration of the specified object.

If you configure a subnet with the mask length of 32 or the mask of 255.255.255.255, the system configures the object with a host address.

Examples

# Configure an IPv4 address object with the host address of 192.168.0.1.

<Sysname> system-view

[Sysname] object-group ip address ipgroup

[Sysname-obj-grp-ip-ipgroup] network host address 192.168.0.1

# Configure an IPv4 address object with the host name of pc3.

<Sysname> system-view

[Sysname] object-group ip address ipgroup

[Sysname-obj-grp-ip-ipgroup] network host name pc3

# Configure an IPv4 address object with the IPv4 address of 192.167.0.0 and mask length of 24.

<Sysname> system-view

[Sysname] object-group ip address ipgroup

[Sysname-obj-grp-ip-ipgroup] network subnet 192.167.0.0 24

# Configure an IPv4 address object with the IPv4 address of 192.166.0.0 and mask of 255.255.0.0.

<Sysname> system-view

[Sysname] object-group ip address ipgroup

[Sysname-obj-grp-ip-ipgroup] network subnet 192.166.0.0 255.255.0.0

      - network (IPv6 address object group view)

Use network to configure an IPv6 address object.

Use undo network to delete an IPv6 address object.

Syntax

[ object-id ] network { host { address ipv6-address | name host-name } | subnet ipv6-address prefix-length }

undo network { host { address ipv6-address | name host-name } | subnet ipv6-address prefix-length }

undo object-id

Default

No IPv6 address objects exist.

Views

IPv6 address object group view

Predefined user roles

network-admin

Parameters

object-id: Specifies an object ID in the range of 0 to 4294967294. If you do not configure an object ID, the system automatically assigns the object a multiple of 10 next to the greatest ID being used. For example, if the greatest ID is 22, the system automatically assigns 30.

host: Configures an IPv6 address object with the host address or name.

address ipv6-address: Specifies an IPv6 host address.

name host-name: Specifies a host name, a case-insensitive string of 1 to 60 characters.

subnet ipv6-address prefix-length: Configures an IPv6 address object with the subnet address followed by the prefix length in the range of 1 to 128.

Usage guidelines

This command fails if you use it to configure or change an IPv6 address object to be identical with an existing object.

This command creates an IPv6 address object if the specified object ID does not exist. Otherwise, the command overwrites the configuration of the specified object.

If you configure a subnet address with the prefix length of 128, the system configures the object with a host address.

Examples

# Configure an IPv6 address object with the host address of 1::1.

<Sysname> system-view

[Sysname] object-group ipv6 address ipv6group

[Sysname-obj-grp-ipv6-ipv6group] network host address 1::1

# Configure an IPv6 address object with the host name of pc3.

<Sysname> system-view

[Sysname] object-group ipv6 address ipv6group

[Sysname-obj-grp-ipv6-ipv6group] network host name pc3

# Configure an IPv6 address object with the IPv6 address of 1:1:1::1 and prefix length of 24.

<Sysname> system-view

[Sysname] object-group ipv6 address ipv6group

[Sysname-obj-grp-ipv6-ip v6group] network subnet 1:1:1::1 24

      - object-group

Use object-group to configure an object group and enter its view, or enter the view of an existing object group.

Use undo object-group to delete an object group.

Syntax

object-group { { ip | ipv6 } address | port | service } object-group-name

undo object-group { { ip | ipv6 } address | port | service } object-group-name

Default

Default object groups exist.

Views

System view

Predefined user roles

network-admin

Parameters

ip address: Configures an IPv4 address object group.

ipv6 address: Configures an IPv6 address object group.

port: Configures a port object group.

service: Configures a service object group.

object-group-name: Specifies an object group name, a case-insensitive string of 1 to 31 characters.

Usage guidelines

The object-group command execution results vary with the specified object group.

- If the specified group does not exist, the system creates a new object group and enters the object group view.

- If the specified group exists but the group type is different from that in the command, the command fails.

The undo object-group command execution results vary with the specified object group.

- If the specified group does not exist, the system executes the command without any system prompt.

- If the specified group exists and the group type is the same as that in the command, the system deletes the group.

- If the specified group exists but the group type is different from that in the command, the command fails.

- If the specified object group is being used by an ACL, object policy, or object group, the command fails.

Default object groups cannot be deleted.

Examples

# Configure an IPv4 address object group named ipgroup.

<Sysname> system-view

[Sysname] object-group ip address ipgroup

# Configure an IPv6 address object group named ipv6group.

<Sysname> system-view

[Sysname] object-group ipv6 address ipv6group

# Configure a port object group named portgroup.

<Sysname> system-view

[Sysname] object-group port portgroup

# Configure a service object group named servicegroup.

<Sysname> system-view

[Sysname] object-group service servicegroup

      - port (port object group view)

Use port to configure a port object.

Use undo port to delete a port object.

Syntax

[ object-id ] port { { eq | lt | gt } port | range port1 port2 }

undo port { { eq | lt | gt } port | range port1 port2 }

undo object-id

Default

No port objects exist.

Views

Port object group view

Predefined user roles

network-admin

Parameters

object-id: Specifies an object ID in the range of 0 to 4294967294. If you do not specify an object ID, the system automatically assigns the object a multiple of 10 next to the greatest ID being used. For example, if the greatest ID is 22, the system automatically assigns 30.

eq: Configures a port object with a port number equal to the specified port.

lt: Configures a port object with a port number smaller than the specified port.

gt: Configures a port object with a port number greater than the specified port.

port: Specifies a port number in the range of 0 to 65535.

range port1 port2: Configures a port object with a port range. The value range for the port1 and port2 arguments is 0 to 65535.

Usage guidelines

This command fails if you use it to configure or change a port object to be identical with an existing object.

This command creates a port object if the specified object ID does not exist. Otherwise, the command overwrites the configuration of the specified object.

When you use the lt port option, follow these guidelines:

- The value of port cannot be 0.

- If the value of port is 1, the system configures the object with a port number of 0.

- If the value of port is in the range of 2 to 65535, the system configures the object with a port number range of [0, port–1].

When you use the gt port option, follow these guidelines:

- The value of port cannot be 65535.

- If the value of port is 65534, the system configures the object with a port number of 65535.

- If the value of port is in the range of 0 to 65533, the system configures the object with a port number range of [port+1, 65535].

When you use the range port1 port2 option, follow these guidelines:

- If port1 is equal to port2, the system configures the object with the port number port1.

- If port1 is smaller than port2, the system configures the object with the port number range.

- If port1 is greater than port2, the system changes the range to [port2, port1] and configures the object with the changed port number range.

- If port1 is 0, the range is displayed as lt port2+1.

- If port2 is 65535, the range is displayed as gt port1–1.

Examples

# Configure a port object with a port number of 100.

<Sysname> system-view

[Sysname] object-group port portgroup

[Sysname-obj-grp-port-portgroup] port eq 100

# Configure a port object with a port number smaller than 20.

<Sysname> system-view

[Sysname] object-group port portgroup

[Sysname-obj-grp-port-portgroup] port lt 20

# Configure a port object with a port number greater than 60000.

<Sysname> system-view

[Sysname] object-group port portgroup

[Sysname-obj-grp-port-portgroup] port gt 60000

# Configure a port object with a port number in the range of 1000 to 2000.

<Sysname> system-view

[Sysname] object-group port portgroup

[Sysname-obj-grp-port-portgroup] port range 1000 2000

      - service (service object group view)

Use service to configure a service object.

Use undo service to delete a service object.

Syntax

[ object-id ] service { protocol [ { source { { eq | lt | gt } port | range port1 port2 } | destination { { eq | lt | gt } port | range port1 port2 } } * | icmp-type icmp-code | icmpv6-type icmpv6-code ] }

undo service { protocol [ { source { { eq | lt | gt } port | range port1 port2 } | destination { { eq | lt | gt } port | range port1 port2 } } * | icmp-type icmp-code | icmpv6-type icmpv6-code ] }

undo object-id

Default

No service objects exist.

Views

Service object group view

Predefined user roles

network-admin

Parameters

object-id: Configures an object ID in the range of 0 to 4294967294. If you do not configure an ID for the object, the system automatically assigns the object a multiple of 10 next to the greatest ID being used. For example, if the greatest ID is 22, the automatically assigned ID is 30.

protocol: Configures the protocol number in the range of 0 to 255, or the protocol name such as TCP, UDP, ICMP, and ICMPv6.

source: Configures a service object with a source port when the protocol is TCP or UDP.

destination: Configures a service object with a destination port when the protocol is TCP or UDP.

eq: Configures a port equal to the specified port.

lt: Configures a port smaller than the specified port.

gt: Configures a port greater than the specified port.

port: Specifies a port number in the range of 0 to 65535.

range port1 port2: Configures a service object with a port range. The value range for the port1 and port2 arguments is 0 to 65535.

icmp-type: Configures the ICMP message type in the range of 0 to 255.

icmp-code: Configures the ICMP message code in the range of 0 to 255.

icmpv6-type: Configures the ICMPv6 message type in the range of 0 to 255.

icmpv6-code: Configures the ICMPv6 message code in the range of 0 to 255.

Usage guidelines

This command fails if you use it to configure or change a service object to be identical with an existing object.

This command creates a service object if the specified object ID does not exist. Otherwise, the command overwrites the configuration of the specified object.

When you use the lt port option, follow these guidelines:

- The value of port cannot be 0.

- If the value of port is 1, the system configures the object with a port number of 0.

- If the value of port is in the range of 2 to 65535, the system configures the object with a port number range of [0, port–1].

When you use the gt port option, follow these guidelines:

- The value of port cannot be 65535.

- If the value of port is 65534, the system configures the object with a port number of 65535.

- If the value of port is in the range of 0 to 65533, the system configures the object with a port number range of [port+1, 65535].

When you use the range port1 port2 option, follow these guidelines:

- If port1 is equal to port2, the system configures the object with the port number port1.

- If port1 is smaller than port2, the system configures the object with the port number range.

- If port1 is greater than port2, the system changes the range to [port2, port1] and configures the object with the changed port number range.

- If port1 is 0, the range is displayed as lt port2+1.

- If port2 is 65535, the range is displayed as gt port1–1.

Examples

# Configure a service object with a protocol number of 100.

<Sysname> system-view

[Sysname] object-group service servicegroup

[Sysname-obj-grp-service-servicegroup] service 100

# Configure a service object with the source and destination port numbers for the TCP service.

<Sysname> system-view

[Sysname] object-group service servicegroup

[Sysname-obj-grp-service-servicegroup] service tcp source eq 100 destination range 10 100

# Configure a service object with the message type and code for the ICMP service.

<Sysname> system-view

[Sysname] object-group service servicegroup

[Sysname-obj-grp-service-servicegroup] service icmp 100 150

- Modified feature: Applying a QoS policy to an interface

- Feature change description

As from this release, you can apply a QoS policy to a Layer 3 aggregate interface or Layer 3 aggregate subinterface.

- Command changes

Syntax

qos apply [ remarking | tcp-erspan ] policy policy-name { inbound | outbound }

undo qos apply [ remarking | tcp-erspan ] policy policy-name { inbound | outbound }

Views

Interface view

Change description

Before modification: You can apply a QoS policy to a Layer 2 Ethernet interface or Layer 3 Ethernet interface.

After modification: You can apply a QoS policy to a Layer 2 Ethernet interface, Layer 3 Ethernet interface, Layer 3 Ethernet subinterface, Layer 3 aggregate interface, or Layer 3 aggregate subinterface.

Usage guidelines

When the traffic class matches the VLAN IDs in outer VLAN tags, Layer 3 Ethernet interfaces and Layer 3 Ethernet subinterfaces do not support QoS policies.

  - Modified feature: Configuring the authentication load sharing mode for users attached to DRNI interfaces

    - Feature change description

As from this release, the default authentication load sharing mode for users attached to DRNI interfaces changes from distributed local mode to centralized mode.

    - Command changes

      - Modified command: port-security drni load-sharing-mode

Syntax

port-security drni load-sharing-mode { centralized | distributed { even-mac | local | odd-mac } }

undo port-security drni load-sharing-mode

Views

System view

Change description

Before modification: The default authentication load sharing mode for users attached to DRNI interfaces is distributed local.

After modification: The default authentication load sharing mode for users attached to DRNI interfaces is centralized.

  - Modified feature: Setting the maximum number of configuration archives for local archiving

    - Feature change description

As from this version, the value range for the maximum number of configuration archives that can be saved on the device changes from 1 to 10 to 1 to 1000.

    - Command changes

      - Modified command: archive configuration max

Syntax

archive configuration max file-number

Views

System view

Change description

Before modification: The value range for the file-number argument is 1 to 10.

After modification: The value range for the file-number argument is 1 to 1000.

  - Modified feature: Displaying detailed next hop information in the RIB or for direct routes or static routes

    - Feature change description

As from this version, the Age, ExtFlag, and Flags fields were added to the commands used to display detailed next hop information in the RIB or for direct routes or static routes.

- The Age field displays the elapsed time since a next hop was last updated.

- The ExtFlag field displays the extended flag of a next hop.

- The Flags field displays the flags of a next hop.

    - Command changes

      - Modified command: display rib nib

Use display rib nib to display next hop information in the RIB.

Syntax

display rib nib [ self-originated ] [ nib-id ] [ verbose ]

display rib nib protocol protocol [ verbose ]

Views

Any view

Change description

Before modification: The detailed next hop information in the RIB does not contain the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

After modification: The detailed next hop information in the RIB contains the Age, ExtFlag, and Flags fields to display the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

      - Modified command: display route-direct nib

Use display route-direct nib to display next hop information for direct routes.

Syntax

display route-direct nib [ nib-id ] [ verbose ]

Views

Any view

Change description

Before modification: The detailed next hop information for direct routes does not contain the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

After modification: The detailed next hop information for direct routes contains the Age, ExtFlag, and Flags fields to display the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

      - Modified command: display ipv6 rib nib

Use display ipv6 rib nib to display next hop information in the IPv6 RIB.

Syntax

display ipv6 rib nib [ self-originated ] [ nib-id ] [ verbose ]

display ipv6 rib nib protocol protocol [ verbose ]

Views

Any view

Change description

Before modification: The detailed next hop information in the IPv6 RIB does not contain the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

After modification: The detailed next hop information in the IPv6 RIB contains the Age, ExtFlag, and Flags fields to display the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

      - Modified command: display ipv6 route-direct nib

Use display ipv6 route-direct nib to display next hop information for IPv6 direct routes.

Syntax

display ipv6 route-direct nib [ nib-id ] [ verbose ]

Views

Any view

Change description

Before modification: The detailed next hop information for IPv6 direct routes does not contain the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

After modification: The detailed next hop information for IPv6 direct routes contains the Age, ExtFlag, and Flags fields to display the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

      - Modified command: display route-static nib

Use display route-static nib to display next hop information for static routes.

Syntax

display route-static nib [ nib-id ] [ verbose ]

Views

Any view

Change description

Before modification: The detailed next hop information for static routes does not contain the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

After modification: The detailed next hop information for static routes contains the Age, ExtFlag, and Flags fields to display the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

      - Modified command: display ipv6 route-static nib

Use display ipv6 route-static nib to display next hop information for IPv6 static routes.

Syntax

display ipv6 route-static nib [ nib-id ] [ verbose ]

Views

Any view

Change description

Before modification: The detailed next hop information for IPv6 static routes does not contain the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

After modification: The detailed next hop information for IPv6 static routes contains the Age, ExtFlag, and Flags fields to display the following information:

- The elapsed time since a next hop was last updated.

- The extended flag of a next hop.

- The flags of a next hop.

  - Modified feature: Optimizing BMP server information display

    - Feature change description

As from this release, the Statistics report interval, Reported route mode, Pu-monitor-mode, and Pd-monitor-mode fields are added to the output of the display bgp bmp server command.

    - Command changes

      - Modified command: display bgp bmp server

Examples

# Display information about BMP server 1.

<Sysname> display bgp bmp server 1

BMP server number: 1

Server VPN instance name: vpna

Server address: 100.1.1.1  Server port: 6895

Client address: 100.1.1.2  Client port: 21452

BMP server state: Connected  Up for 00h41m53s

TCP source interface has been configured

Statistics report interval: 5s

Reported route mode: adj-rib-in pre-policy

Pu-monitor-mode: Enabled

Pd-monitor-mode: Enabled

Message statistics:

Total messages sent: 15

INITIATION: 1

TERMINATION: 0

STATS-REPORT: 0

PEER-UP: 4

PEER-DOWN: 3

ROUTE-MON: 7

BGP peers monitored by BMP server:

10.1.1.1

Change description

As from this release, the following fields are added to the output of this command:

- Statistics report interval—Interval at which BGP sends statistics information to the BMP server.

- Reported route mode—Type of the routes that BGP reports to the BMP server.

- Pu-monitor-mode—Whether the BMP client sends peer up notifications with mode flags to the BMP server.

- Pd-monitor-mode—Whether the BMP client sends peer down notifications with mode flags to the BMP server.

  - Modified feature: Setting the 802.1X periodic reauthentication timer

    - Feature change description

As from this version, the value range for the 802.1X periodic reauthentication timer set in system view and interface view was changed from 60 to 7200 seconds to 60 to 86400 seconds.

    - Command changes

      - Modified command: dot1x timer reauth-period (interface view)

Syntax

dot1x timer reauth-period reauth-period-value

undo dot1x timer reauth-period

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Change description

Before modification: The value range for the reauth-period-value argument is 60 to 7200 seconds.

After modification: The value range for the reauth-period-value argument is 60 to 86400 seconds.

      - Modified command: dot1x timer

Syntax

dot1x timer { ead-timeout ead-timeout-value | handshake-period handshake-period-value | quiet-period quiet-period-value | reauth-period reauth-period-value | server-timeout server-timeout-value | supp-timeout supp-timeout-value | tx-period tx-period-value | unicast-trigger quiet-period quiet-period-value | user-aging { auth-fail-vlan | auth-fail-vsi | critical-microsegment | critical-vlan | critical-vsi | guest-vlan | guest-vsi } aging-time-value }

undo dot1x timer { ead-timeout | handshake-period | quiet-period | reauth-period | server-timeout | supp-timeout | tx-period | unicast-trigger quiet-period | user-aging { auth-fail-vlan | auth-fail-vsi | critical-microsegment | critical-vlan | critical-vsi | guest-vlan | guest-vsi } }

Views

System view

Change description

Before modification: The value range for the reauth-period-value argument is 60 to 7200 seconds.

After modification: The value range for the reauth-period-value argument is 60 to 86400 seconds.

  - Modified feature: Setting the periodic MAC reauthentication timer

    - Feature change description

As from this version, the value range for the periodic MAC reauthentication timer set in system view and interface view was changed from 60 to 7200 seconds to 60 to 86400 seconds.

    - Command changes

      - Modified command: mac-authentication timer (interface view)

Syntax

mac-authentication timer { auth-delay auth-delay-time | reauth-period reauth-period-value }

undo mac-authentication timer { auth-delay | reauth-period }

Views

Layer 2 Ethernet interface view

Layer 2 aggregate interface view

Change description

Before modification: The value range for the reauth-period-value argument is 60 to 7200 seconds.

After modification: The value range for the reauth-period-value argument is 60 to 86400 seconds.

      - Modified command: mac-authentication timer (system view)

Syntax

mac-authentication timer { offline-detect offline-detect-value | quiet quiet-value | reauth-period reauth-period-value | server-timeout server-timeout-value | temporary-user-aging aging-time-value | user-aging { critical-microsegment | critical-vlan | critical-vsi | guest-vlan | guest-vsi } aging-time-value }

undo mac-authentication timer { offline-detect | quiet | reauth-period | server-timeout | temporary-user-aging | user-aging { critical-microsegment | critical-vlan | critical-vsi | guest-vlan | guest-vsi } }

Views

System view

Change description

Before modification: The value range for the reauth-period-value argument is 60 to 7200 seconds.

After modification: The value range for the reauth-period-value argument is 60 to 86400 seconds.

  - Modified feature: Displaying local public keys

    - Feature change description

The Key length field was added to the output for the display public-key local public command to indicate the key length of public keys in local key pairs, in bits.

    - Command changes

      - Modified command: display public-key local public

Syntax

display public-key local { dsa | ecdsa | rsa } public [ name key-name ]

Views

Any view

Change description

The Key length field was added to the output for this command to indicate the key length of public keys in local key pairs, in bits.

  - Modified feature: Value range for the ID of an IPsec tunnel

    - Feature change description

The value range for the tunnel-id argument changed.

    - Command changes

      - Modified command: display ipsec statistics

Syntax

display ipsec statistics [ tunnel-id tunnel-id ]

Views

Any view

Change description

Before modification, the value range for the tunnel-id argument was 0 to 4294967295.

After modification, the value range for the tunnel-id argument is 0 to 4294967294.

      - Modified command: display ipsec tunnel

Syntax

display ipsec tunnel { brief | count | tunnel-id tunnel-id }

Views

Any view

Change description

Before modification, the value range for the tunnel-id argument was 0 to 4294967295.

After modification, the value range for the tunnel-id argument is 0 to 4294967294.

      - Modified command: reset ipsec statistics

Syntax

reset ipsec statistics [ tunnel-id tunnel-id ]

Views

Any view

Change description

Before modification, the value range for the tunnel-id argument was 0 to 4294967295.

After modification, the value range for the tunnel-id argument is 0 to 4294967294.

  - Modified feature: Configuring automatic email sending parameters for an EAA monitor policy

    - Feature change description

As from this release, the length of the email server domain name is changed to 1 to 253 characters.

    - Command changes

      - Modified command: rtm email domain

Syntax

rtm email domain domain-name

Views

System view

Change description

Before modification: The domain-name argument is a string of 1 to 255 characters.

After modification: The domain-name argument is a string of 1 to 253 characters.

  - Modified feature: Setting the usage alarm threshold of a log file

    - Feature change description

The value range for the usage alarm threshold of a log file was changed to 0 to 100, in percentage.

    - Command changes

      - Modified command: info-center logfile alarm-threshold

Syntax

info-center logfile alarm-threshold usage

Views

System view

Change description

Before modification, the value range for the usage alarm threshold of a log file was 1 to 100, in percentage.

After modification, the value range for the usage alarm threshold of a log file is 0 to 100, in percentage.

  - Modified feature: Displaying heap memory usage for a user process

  - Feature change description

As from this version, the Free physical memory ratio field is added to the output from the command that displays the heap memory usage for a user process.

  - Command changes

    - Modified command: display process memory heap

Syntax

display process memory heap job job-id [ verbose ] [ slot slot-number [ cpu cpu-number ] ]

Views

Any view

Change description

Before modification: The command does not support displaying the Free physical memory ratio field in the output.

After modification: The command supports displaying the Free physical memory ratio field in the output.

  - Modified feature: Displaying gRPC information

    - Feature change description

As from this release, the device supports displaying both detailed and brief information about both gRPC dial-in mode and gRPC dial-out mode. With previous releases, the device can display only brief gRPC dial-in mode information.

    - Command changes

Old syntax

display grpc

New syntax

display grpc [ verbose ]

Views

Any view

Change description

Before modification: The device supports displaying only brief information about only gRPC dial-in mode.

After modification: The device supports displaying both detailed and brief information about both gRPC dial-in mode and gRPC dial-out mode. To display detailed information, specify the verbose keyword. To display brief information, do not specify the verbose keyword.

- R6615P07

This release has the following changes:

- Removed feature: Enabling the SSH server to support SSH1 clients

- Removed feature: Setting the minimum interval for updating the RSA server key pair

Removed feature: Enabling the SSH server to support SSH1 clients

Feature change description

As from this version, this feature is not supported.

Deleted command

ssh server compatible-ssh1x enable

Syntax

ssh server compatible-ssh1x enable

undo ssh server compatible-ssh1x [ enable ]

Views

System view

Removed feature: Setting the minimum interval for updating the RSA server key pair

Feature change description

As from this version, this feature is not supported.

Deleted command

ssh server rekey-interval

Syntax

ssh server rekey-interval interval

undo ssh server rekey-interval

Views

System view

.

- R6615P06

First release.