5140HI-CMW710-R6652P05 Release Notes

The information in this document is subject to change without notice. © Copyright 2022, 2024 Hewlett Packard Enterprise Development LP

Contents

Introduction	1

Version information	1

Version number	1

Version history	1

Hardware and software compatibility matrix	2

ISSU upgrade type matrix	3

Upgrade advice	3

Upgrade restrictions and guidelines	3

Hardware feature updates	4

R6652P05~R6628P40	4

R6628P35	4

R6615P07	4

R6615P06	4

Software feature and command updates	4

MIB updates	4

Operation changes	4

Operation changes in R6652P05~R6628P40	4

Operation changes in R6628P35	5

Operation changes in R6615P07	5

Operation changes in R6615P06	5

Restrictions and cautions	5

Restrictions	5

Cautions	6

Open problems and workarounds	6

List of resolved problems	6

Resolved problems in R6652P05	6

Resolved problems in R6628P47	8

Resolved problems in R6628P43	10

Resolved problems in R6628P40	11

Resolved problems in R6628P35	16

Resolved problems in R6615P07	23

Resolved problems in R6615P06	26

Support and other resources	26

Accessing Hewlett Packard Enterprise Support	26

Documents	27

Related documents	27

Documentation feedback	28

Appendix A Feature list	29

Hardware features	29

Software features	32

Appendix B Fixed security vulnerabilities	36

Fixed security vulnerabilities in R6652P05	36

Fixed security vulnerabilities in R6628P35	42

Fixed security vulnerabilities in R6615P07	43

Appendix C Upgrading software	44

System software file types	44

System startup process	44

Upgrade methods	45

Upgrading from the CLI	46

Preparing for the upgrade	46

Downloading software images to the master switch	47

Upgrading the software images	49

Upgrading from the Boot menu	51

Prerequisites	51

Accessing the Boot menu	52

Accessing the basic Boot menu	53

Accessing the extended Boot menu	54

Upgrading Comware images from the Boot menu	55

Upgrading Boot ROM from the Boot menu	63

Managing files from the Boot menu	70

Handling software upgrade failures	73

List of tables

Table 1 Version history	1

Table 2 Hardware and software compatibility matrix	2

Table 3 ISSU version compatibility matrix	3

Table 4 MIB updates	4

Table 5 5140HI series hardware features for non-PoE switch models	29

Table 6 5140HI series hardware features for PoE switch models	30

Table 7 PoE power capacity of the HPE 5140 48G PoE+ 4SFP+ 1-slot HI Sw Swch switches	31

Table 8 Software features of the 5140HI series	32

Table 9 Minimum free storage space requirements	51

Table 10 Shortcut keys	52

Table 11 Basic Boot ROM menu options	53

Table 12 BASIC ASSISTANT menu options	53

Table 13 Extended Boot ROM menu options	54

Table 14 EXTENDED ASSISTANT menu options	55

Table 15 TFTP parameter description	56

Table 16 FTP parameter description	58

Table 17 TFTP parameter description	64

Table 18 FTP parameter description	65

Introduction

This document describes the features, restrictions and guidelines, open problems, and workarounds for version 5140HI-CMW710-R6652P05. Before you use this version on a live network, back up the configuration and test the version to avoid software upgrade affecting your live network.

Use this document in conjunction with the documents listed in "Related documents."

Version information

Version number

HPE Comware Software, Version 7.1.070, Release 6652P05.

|  | NOTE: To identify the version number (see Note①), execute the display version command in any view. |
| --- | --- |

Version history

|  | IMPORTANT: The software feature changes listed in the version history table for each version are not complete. To obtain complete information about all software feature changes in each version, see the Software Feature Changes document for this release notes. |
| --- | --- |

Version history

| Version number | Last version | Release Date | Release type | Remarks |
| --- | --- | --- | --- | --- |
| R6652P05 | R6628P47 | 2024-03-31 | Release version | None |
| R6628P47 | R6628P43 | 2023-12-29 | Release version | None |
| R6628P43 | R6628P40 | 2023-08-21 | Release version | None |
| R6628P40 | R6628P35 | 2023-07-15 | Release version | None |
| R6628P35 | R6615P07 | 2023-4-25 | Release version | None |
| R6615P07 | R6615P06 | 2022-06-09 | Release version | None |
| R6615P06 | First release | 2022-03-25 | Release version | First release |

Hardware and software compatibility matrix

|  | CAUTION: To avoid an upgrade failure, use Table 2 to verify the hardware and software compatibility before performing an upgrade. |
| --- | --- |

Hardware and software compatibility matrix

| Item | Specifications |
| --- | --- |
| Product family | 5140HI Series |
| Hardware platform | HPE 5140 24G 4SFP+ 1-slot HI Swch HPE 5140 48G 4SFP+ 1-slot HI Sw HPE 5140 24G PoE+ 4SFP+ 1-slot HI Sw HPE 5140 48G PoE+ 4SFP+ 1-slot HI Sw HPE 5140/5520 10GBASE-T MACsec 2p Mod |
| Minimum memory requirements | 2GB |
| Minimum Flash requirements | 512 M |
| Boot ROM version | Version 119 or higher (Note: Use the display version command in any view to view the version information. Please see Note) |
| Host software | 5140HI-CMW710-R6652P05.ipe (See the MD5 file) |
| iMC version | iMC ACLM 7.3 (E0705P12) iMC PLAT 7.3 (E0705P12) iMC QoSM 7.3(E0505P01) iMC EIA 7.3 (E0611P13) iMC NTA 7.3（E0707P06） iMC SHM 7.3 (E0707L06) iMC EAD 7.3 (E0611P10) iMC iCC 7.3 (E0705P12) iMC VLAN 7.3 (E0705P12) |
| iNode version | iNode 7.3 (E0585) |
| Remarks | None |

Display the system software and Boot ROM versions of 5140HI

<Sysname>display version

HPE Comware Software, Version 7.1.070, Release 6615P07     ------Note①

Copyright (c) 2010-2021 Hewlett Packard Enterprise Development LP

HPE 5520 24G 4SFP+ HI Swch R8M25A uptime is 0 weeks, 0 days, 0 hours, 8 minutes

Last reboot reason : User reboot

Boot image: flash:/5140HI-cmw710-boot-r6615P07.bin

Boot image version: 7.1.070, Release 6615P07

Compiled Jun 16 2021 11:00:00

System image: flash:/5140HI-cmw710-system-r6615P07.bin

System image version: 7.1.070, Release 6615P07

Compiled Jun 16 2021 11:00:00

Feature image(s) list:

flash:/5140HI-cmw710-freeradius-r6615P07.bin, version: 7.1.070, Release 6615P07

Compiled Jun 16 2021 11:00:00

flash:/5140HI-cmw710-escan-r6615P07.bin, version: 7.1.070, Release 6615P07

Compiled Jun 16 2021 11:00:00

Slot 1:

Uptime is 0 weeks,0 days,0 hours,8 minutes

5520 24G 4SFP+ HI Swch with 2 Processors

BOARD TYPE:         5520 24G 4SFP+ HI Swch

DRAM:               2048M bytes

FLASH:              512M bytes

PCB 1 Version:      VER.B

Bootrom Version:    119                  -------Note②

CPLD 1 Version:     001

Release Version:    HPE 5520 24G 4SFP+ HI Swch R8M26A-6615P07

Patch Version  :    None

Reboot Cause  :     UserReboot

[SubSlot 0] 48GE+4SFP Plus

ISSU upgrade type matrix

ISSU provides compatible upgrade and incompatible upgrade, depending on the compatibility between software versions. Table 3 provides the approved ISSU upgrade types only between the current version and the history versions within the past 18 months. This matrix does not include history versions that are 18 months earlier than the current version, for which, no ISSU upgrade verification was performed.

For more information about ISSU, see the fundamentals configuration guide for the device.

ISSU version compatibility matrix

| Current version | History version | ISSU upgrade method |
| --- | --- | --- |
| 5140HI-CMW710-R6652P05 | 5140HI-CMW710-R6628P47 | Incompatible |

Upgrade advice

Upgrade to this version as needed to gain benefits introduced in this version.

Upgrade restrictions and guidelines

Document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

Hardware feature updates

R6652P05~R6628P40

None

R6628P35

Supported HPE 5140/5520/5600 4P BASE-T/6P SFP Mod, HPE 5140/5520/5600 4P 1/10G SFP+ Mod and HPE 5140/5520/5600 2P SFP28 Mod.

R6615P07

None

R6615P06

First release

Software feature and command updates

For more information about the software feature and command update history, see HPE 5140HI-CMW710-R6652P05 Release Notes (Software Feature Changes).

MIB updates

MIB updates

| Item | MIB file | Module | Description |
| --- | --- | --- | --- |
| 5140HI-CMW710-R6652P05~5140HI-CMW710-R6615P07 |  |  |  |
| New | None | None | None |
| Modified | None | None | None |
| 5140HI-CMW710-R6615P06 |  |  |  |
| New | First release | First release | First release |
| Modified | First release | First release | First release |

Operation changes

Operation changes in R6652P05~R6628P40

None

Operation changes in R6628P35

Changed the default of the port-security m-lag load-sharing-mode command from distributed local to centralized.

For a PoE-capable device, the value for the max-power argument in the poe max-power command is changed from the range 1000-3000 to the range 1000-35000, in millwatts.

Operation changes in R6615P07

None

Operation changes in R6615P06

First release

Restrictions and cautions

Before performing a software upgrade, it is important to refer to the Software Feature Changes document for any feature changes in the new version. Also check the most recent version of the related documents (see "Related documents") available on the HPE website for more information about feature configuration and commands.

When you use this version of software, make sure you fully understand the restrictions and cautions described in this section.

Restrictions

Restrictions for VXLAN

Before you configure VXLAN settings, you must execute the switch-mode 1 command in system view to set the system operating mode to VXLAN.

Restrictions for 10-GE interfaces

To connect a 10-GE interface with a GE transceiver module installed to another interface by using a long optical fiber, you must perform the following tasks on the 10-GE interface:

Set the interface speed to 1000 Mbps by using the speed 1000 command.

Configure the interface to operate in full duplex mode by using the duplex full command.

Restrictions for 802.1X

When you enable 802.1X on a Layer 2 aggregate interface, make sure the 802.1X multicast trigger feature is disabled. To disabled the 802.1X multicast trigger feature, use the undo dot1x multicast-trigger command.

Restrictions for dynamic Ethernet service instances

A dynamic Ethernet service instance determines that a packet belongs to the matching VSI if the VLAN ID and source MAC address in the packet match the VLAN ID and MAC address in authorization information for a user.

Restrictions for the drop-unknown command in IGMP-snooping/MLD-snooping view

The drop-unknown command is not supported in IGMP-snooping/MLD-snooping view.

Restrictions for the convergence time after a master/subordinate switchover

The Layer 2 traffic convergence time after a master/subordinate switchover exceeds 2 seconds in an EVPN network.

Restrictions for the PSE Device

If a PSE cannot supply power correctly, execute the poe detection-mode simple command to enable the simple PD detection mode, and enable non-standard PD detection. To enable non-standard PD detection, execute the poe legacy enable command in system view or interface view.

Only PSEs that have a model name ending with character B support PD detection mode configuration. To obtain the model name of a PSE, execute the display poe pse command.

Cautions

None

Open problems and workarounds

202403270216

Symptom: A tunnel cannot forward traffic properly.

Condition: This symptom occurs if you configure a PBR policy, configure the outgoing interface as a tunnel, delete that tunnel, and then re-create that tunnel.

Workaround: Execute the shutdown and undo shutdown commands on the tunnel interface to restore traffic forwarding.

202403180668

Symptom: An M-LAG system records STP dispute logs, leading to traffic interruption. The symptom occurs because of a logic mistake in processing a specific situation.

Condition: This symptom occurs if the reliability settings are different between the old and the new software versions when the software is upgraded, especially in STP protocol processing.

Workaround: During the upgrade process, execute the undo stp dispute-protection command to avoid issues caused by version differences.

202311210102

Symptom: On an M-LAG network, STP convergence might be slow.

Condition: This symptom occurs if you use the m-lag standalone enable command to enable the M-LAG standalone mode and then power-cycle the primary M-LAG member device.

Workaround: Set the delay that the device must wait before changing to M-LAG standalone mode.

List of resolved problems

  - Resolved problems in R6652P05

202403040340

Symptom: Packet loss continues for more than 60 seconds after a master/subordinate switchover is performed on an IRF fabric.

Condition: This symptom occurs if multicast traffic is transmitted over the RPT path and the output interface of the optimal route is on the master device.

202401291764

Symptom: An authenticated user is online simultaneously on both M-LAG member devices.

Condition: This symptom occurs if the following operations are performed on an M-LAG network:
A user first comes online through authentication on a single-homing interface of  M-LAG 1.
The user comes online through authentication on a single-homing interface of M-LAG 2 and triggers a migration.

202403060066

Symptom: BFD flapping occurs.

Condition: This symptom occurs if the deny-mode ACL rule used to match IPv4 packets configured on VLAN-interface 200 matches BFD packets by mistake, resulting in BFD packet loss and BFD flapping.

202402291384

Symptom: Tunnel traffic received on a non-aggregate interface matches an ACL configured for an aggregate interface by mistake.

Condition: This symptom occurs if the ACL configured on an aggregate interface incorrectly matches the specific traffic characteristics. When an extension port is allocated to tunnel traffic, the lowest 8 bits coincide with the SRC TRUNK (source port aggregation) match criterion defined in the aggregate interface ACL.

202402040334

Symptom: The MAC address is not deleted after a static AC on an interface is deleted.

Condition: This symptom occurs if 802.1x authentication is performed before the static AC is deleted.

202401090475

Symptom: The keepalive link flaps due to timeout.

Condition: This symptom occurs if a large number of ARP packets are sent to the CPU.

202401230378

Symptom: On an IRF fabric, the number of available ARP resources is incorrect.

Condition: This symptom occurs if the ARP packets move between the member leaf devices repeatedly.

202311150214

Symptom: On an M-LAG network, the peer link cannot transmit traffic.

Condition: This symptom occurs if the following operations are performed on an M-LAG network:

              - Enable automatic setup of a VXLAN tunnel between M-LAG member devices.

              - Shut down the M-LAG interface on one member device.

202307181156

Symptom: In an EVPN M-LAG network, the member devices might not advertise BGP routes, and Layer 3 traffic cannot be forwarded.

Condition: This symptom occurs if BGP EVPN sessions are set up in an EVPN M-LAG network.

202310090404

Symptom: An OSPF route anomaly occurs.

Condition: This symptom occurs if you shut down the BFD MAD detection interfaces on IRF devices by shutting down the downlink interface and then the uplink interface.

202307051264

Symptom: The device does not display logs for adding MAC address entries and displays logs only for deleting MAC address entries.

Condition: This symptom occurs if you configure port security settings on a port and connect the port to the peer end.

202310240312

Symptom: On an EVPN DRNI system with a tunnel peer link, the peer-link tunnel goes up slowly or even cannot go up.

Condition: This symptom might occur if default VXLAN decapsulation is enabled for the IP address of loopback 0 and the IP address is the source IP addresses of non-peer-link VXLAN tunnels.

202310240098

Symptom: After patch installation and device restart, slow device startup and the EVENT_TIMEOUT log might occur.

Condition: This symptom might occur if DRNI and monitor link are configured together and the device reboots after installation of a patch with the drnid process included.

202308091835

Symptom: The xmlcfgd process has exceptions in the next installation of a patch after the patch is loaded, because a subprocess has residues.

Condition: This symptom might occur if a patch is loaded on the controller connected to the device.

202309260374

Symptom: The ovsdb-server process occasionally terminates abnormally on the device.

Condition: This symptom might occur when the controller deploys the configuration to the device.

202308291580

Symptom: Packet loss occurs during the bulk addition or deletion of M-LAG interfaces on an EVPN M-LAG system.

Condition: This symptom might occur if singlehoming AC-attached interfaces exist on the EVPN M-LAG system, and bulk addition or deletion of M-LAG interfaces is performed during traffic transmission between remote leaf devices and local ACs.

202309182105

Symptom: The switch sends 802.1X authentication packets and accounting packets to different RADIUS servers because the state of the port security process is incorrect.

Condition: This symptom occurs if a master/subordinate switchover is performed on an IRF fabric.

  - Resolved problems in R6628P47

202310090389

Symptom: The switch reboots after a QoS policy is applied to an aggregate interface.

Condition: This symptom occurs if the type protocol-type protocol-type-mask option in an ACL rule contained in the QoS traffic class is type 0806 ffff.

202311300105

Symptom: A valid user fails Web authentication on an interface.

Condition: This symptom occurs if the IP address of the remote Web server for Web authentication is the same as a Web authentication-free IP address and the following operations are performed:

  - Enable Web authentication on the interface.

  - Remove the Web authentication-free IP address.

  - Disable Web authentication on the interface.

  - Reenable Web authentication on the interface and reconfigure the Web authentication-free IP address.

202308160716

Symptom: A user obtains an authorization ACL after it passes authentication and comes online. However, the port ranges in the ACL rules cannot take effect.

Condition: This symptom occurs if the ACL number is not 2304 and the device is operating in switch mode.

202309121745

Symptom: On an IRF fabric, multicast forwarding is abnormal after a master/subordinate switchover.

Condition: This symptom occurs after a master/subordinate switchover if you have configured Layer 3 interfaces before setting up the IRF fabric.

202311141523

Symptom: In a VXLAN network, all VTEPs have a large number of unnecessary NS/NA packets on the tunnel side. As a result, the CPU usage is high.

Condition: This symptom occurs if the centralized gateway device is disabling from learning the ND entries on the tunnel side and configured with local ND proxy.

202311180396

Symptom: The system prompts that VSI resources are insufficient when no VSIs are created.

Condition: This symptom occurs if you change the link mode of an Ethernet interface to the Layer 3 mode in standard operating mode.

202311150193

Symptom: The device reboots because of deadloop.

Condition: This symptom occurs if the debug qacl show acl-resc slot 1 chip 0 all command is executed and the total number of ACLs exceeds 2000.

202309090594

Symptom: Residual dynamic ACL entries exist on subordinate devices in an IRF fabric.

Condition: This symptom occurs if the following conditions exist:

  - The IRF fabric has aggregate interfaces that contain member ports from multiple member devices.

  - MAC authentication users are assigned authorization VSIs after they pass MAC authentication and come online.

  - The MAC authentication users are frequently moving among the aggregate interfaces in different VLANs.

202308300886

Symptom: The device cannot learn ARP entries for endpoints.

Condition: The symptom might occur when the following conditions exist:

  - Two endpoints access the device from different VLANs and the device acts as the gateway for the two endpoints.

  - The two endpoints use only PTP service traffic for communication.

  - You execute the reset arp all command on the device.

202307130972

Symptom:  After the m-lag extra-vlan command is executed, the M-LAG member devices cannot synchronize ARP or ND entries for the extra VLANs through the peer-link interface.

Condition:  This symptom might occur if some M-LAG interfaces are not assigned to the extra VLANs, and the device is rebooted or the peer-link interface flaps.

202309182105

Symptom:  The switch sends 802.1X authentication packets and accounting packets to different RADIUS servers because the state of the port security process is incorrect.

Condition:  This symptom occurs if a master/subordinate switchover is performed on an IRF fabric.

202305080149

Symptom: On an EVPN M-LAG network, packet loss occurs when a device single-homed to a leaf device pings other devices.

Condition: This symptom occurs if a device single-homed to a leaf device broadcasts the received RARP packets on an EVPN+M-LAG network. As a result, ARP entries and ARP suppression entries become incorrect on other devices.

202307271094

Symptom: The queue priority of common Hello packets is not high enough on an RRPP network.

Condition: This symptom occurs on an RRPP network.

202306240066

Symptom: ARP entries are learned on the IPP incorrectly. As a result, remote IP addresses on the same subnet might fail to be accessed.

Condition: This symptom occurs if proxy ARP is configured for the DRNI dual-active VLAN gateways or VRRP and periodic automatic ARP scanning is enabled by using the arp scan auto enable command.

  - Resolved problems in R6628P43

202308161024

Symptom: Traffic forwarding fails because ARP entries fail to be learned.

Condition: This symptom occurs if the configured inbound rate limit is greater than 1 Gbps.

202308160701

Symptom: Protocol packets that pass through the device are dropped abnormally.

Condition: This symptom might occur when the rate of protocol packets sent to the CPU is so high that triggers deployment of attack defense ACLs.

202308160698

Symptom: An aggregate interface flaps and drops packets when the rate of an aggregation member port is edited.

Condition: This symptom occurs if the rate of an aggregation member port is edited.

202308160692

Symptom: When the VPN binding limit for Layer 3 aggregate subinterfaces is exceeded, the device does not output any message.

Condition: This symptom occurs if the VPN binding limit for Layer 3 aggregate subinterfaces is exceeded.

202308160716

Symptom: A user obtains an authorization ACL after it passes authentication and comes online. However, the port ranges in the ACL rules cannot take effect.

Condition: This symptom occurs if the ACL number is not 2304 and the device is operating in switch mode.

202308111636

Symptom: Service failure causes packet forwarding failure.

Condition: This symptom might occur when the device receives a large number of HTTP or HTTPS attack defense packets destined for the device.

202308101193

Symptom: A Layer 3 aggregate interface is deleted after VPNs are bound to its subinterfaces, and related ACL resources are not deleted completely.

Condition: This symptom occurs if a Layer 3 aggregate interface is deleted after VPNs are bound to its subinterfaces.

  - Resolved problems in R6628P40

202306270487

Symptom: A packet filter cannot drop the TCP packets with port 639.

Condition: This symptom occurs when you configure a packet filter to drop TCP packets with port 639.

202305251379

Symptom: The CPU usage is high on the leaf devices in an EVPN network.

Condition: This symptom occurs if ARP flood suppression works in response mode and the devices attached to the leaf devices migrate frequently, which causes IP address conflicts.

202305290972

Symptom: After a service card is restarted or removed, IKE negotiation fails, resulting in interruption of the IPsec service.

Condition: This symptom occurs if the device has multiple MPUs and a primary/backup switchover occurs.

202306301119

Symptom: On an RRPP ring, multicast traffic fails to be forwarded after a link switchover is performed.

Condition: This symptom occurs if you have enabled dropping unknown multicast data packets globally.

202305250569

Symptom: Residual IPv6SG bindings exist after you clear ND snooping entries in a VLAN.

Condition: This symptom might occur if you configure the device to create both ND snooping entries and IPv6SG bindings for a VLAN. An endpoint migrates frequently within the VLAN and sends NS packets to the device.

202306130155

Symptom: After a user goes offline, its IPv6 address binding entries are not deleted.

Condition: This symptom occurs if ND snooping is configured for a VSI and a large number of endpoints migrate between ACs of the VSI frequently and send ND packets.

202305251218

Symptom: The device outputs free memory early-warning notifications every hour if you edit the configured free-memory thresholds by adding the early-warning threshold and sufficient-memory threshold after a free-memory alarm has been triggered.

Condition: This symptom might occur if a free-memory alarm has been triggered without the early-warning threshold and the sufficient-memory threshold configured.

202306301160

Symptom: After a Layer 3 aggregate interface bound to a VPN instance is deleted, relevant ACL resources remain.

Condition: This symptom occurs if you bind a Layer 3 aggregate interface to a VPN instance, and then delete the Layer 3 aggregate interface directly.

202305251252

Symptom: A user fails HWTACACS authorization and accounting.

Condition: This symptom occurs if the following operations are performed:

              - Use the ip host or ipv6 host command to configure the host name of an HWTACACS server.

              - In HWTACACS scheme view, specify the HWTACACS server by its host name and use it as the authentication, authorization, and accounting servers.

202306130886

Symptom: The SNMP collected traffic statistics is not consistent with the actual statistics.

Condition: This symptom occurs if the inbound or outbound accumulated traffic statistics on the network management port exceeds 4294967295.

202306150365

Symptom: The device cannot ping the PCs attached to access ports, and the PCs attached to trunk ports can be pinged.

Condition: This symptom occurs if ports join an aggregation group, a VPN instance is bound to the related Layer 3 aggregate interface, and ACL configuration is issued to the ports.

202305251257

Symptom: An M-LAG peer-link interface cannot forward packets of 1859 bytes or larger.

Condition: This symptom occurs if an M-LAG peer-link interface forwards traffic.

202306250055

Symptom: A VRRP network cannot be established.

Condition: This symptom occurs if the intermediate device is not configured with VRRP and is enabled with dropping unknown multicast data packets for a VLAN.

202306250483

Symptom: You cannot use SSH or Telnet to log in to the local device from another directly connected device.

Condition: This symptom occurs if the following Web authentication-free subnets exist on the local device:

  - Web authentication-free subnet that contains the IP address for SSH or Telnet login.

  - Web authentication-free subnet that has a mask shorter than the mask of the IP address for SSH or Telnet login.

202306061815

Symptom: The switch generates a core file for the PIM module.

Condition: This symptom occurs if the next hop of the optimal route to the source in an SSM multicast forwarding entry is a secondary IP address and route flapping occurs.

202305181213

Symptom: The switch reboots due to KernelAbnormalReboot.

Condition: This symptom occurs when multicast settings are configured on an IRF fabric.

202305291037

Symptom: An ucmd exception occurs when you enter a command.

Condition: This symptom occurs when HWTACACS command accounting is configured and the server changes from unreachable to reachable.

202306122053

Symptom: When all online users that are assigned the same authorization ACL go offline, the device fails to delete the authorization ACL information. Residual authorization ACL information exists on the device. As a result, the ACL resources are insufficient.

Condition: This symptom occurs if the following operations are performed:

              - Assign the authorization ACL to multiple BYOD online users in the same VSI.

              - Log off all the users. The first online user assigned the authorization ACL is not the last one to go offline.

202305290786

Symptom: When conversational learning is enabled for forwarding entries of an AC, the device cannot issue AC forwarding entries to the hardware upon receiving traffic on the AC.

Condition: This symptom occurs if the AC is continuously receiving known unicast packets when you enable conversational learning for forwarding entries of the AC.

202306201965

Symptom: A MAC authenticated user cannot obtain an IP address after it is assigned to the BYOD authorization VSI.

Condition: This symptom occurs if the following conditions exist:

  - AD-Campus 6.3 solution.

  - IRF and EVPN VXLAN network.

  - MAC-based traffic match mode is disabled for dynamic Ethernet service instances on the interface on which the user is authenticated. This mode is configured by using the mac-based ac command.

  - The user must pass MAC portal authentication on the leaf device, and it has passed MAC authentication.

202305300747

Symptom: Known unicast traffic is not isolated between VXLAN tunnels of different VXLANs.

Condition: This symptom occurs if packets are forwarded between two VXLAN tunnel interfaces.

202305251496

Symptom: The undo telnet server enable command cannot take effect.

Condition: This symptom occurs if you execute this command when the device acts as the Telnet server and the third-party Telnet client does not support option negotiation.

202307010424

Symptom: If the physical interface on which a PW resides receives more than 500 DHCP Discover messages per second, services (such as OSPF) running on that interface will be interrupted.

Condition: This symptom occurs if the following conditions exist:

              - The device is on the MPLS L2VPN network and is enabled with the DHCP service.

              - The PW interface receives more than 500 DHCP Discover messages per second.

202305290977

Symptom: NAT port blocks for users run out easily, which causes insufficient port blocks and affects user services.

Condition: This symptom might occur when you configure DNS disabled with ALG in a NAT scenario. Five-tuple entries are generated and the aging timer for the entries is prolonged.

202305290958

Symptom: A user fails HWTACACS authentication and cannot log in to the Web interface of the device.

Condition: This symptom occurs if HWTACACS authentication is configured for login and the user attempts to log in to the Web interface of the device.

202305290894

Symptom: A SmartMC member repeatedly prints the following login failure log after it reboots: Feb 24 14:41:31:3042023 H3C NETCONF/6/SOAP_XML_LOGIN: admin from 127.0.0.1 loginfailed

Condition: This symptom occurs if you perform the following operations on the commander:

              - Modify the password for the default user (admin) on members.

              - Save the member configuration and reboot the command.

202305290914

Symptom: In a DR system, the outgoing interface for traffic is incorrect after ND entries migrate.

Condition: This symptom occurs if a DR member device reboots and synchronizes ND entries with the DR peer, ND entries migrate on the DR member device, and the outgoing interface for traffic changes.

202305260074

Symptom: The output from the display lldp neighbor-information list command is displayed in garbled characters when LLDP is enabled on the device.

Condition: This symptom occurs if LLDP is enabled on the device and you execute display lldp neighbor-information list to display brief LLDP information that all LLDP agents received from the neighboring devices in a list.

202305251384

Symptom: Command execution fails. The CLI gets stuck.

Condition: This symptom occurs if you execute the default command, MAC authentication commands, or port security commands on a port during optimized automated deployment of the AD-Campus 6.3 solution.

202305251188

Symptom: Some DDNS features are unavailable.

Condition: This symptom might occur when you send packets to the DDNS server in which the Host field is an IP address instead of its corresponding domain name.

202305251147

Symptom: The IKED process on the MPU experienced an exception, which triggered the device to reboot abnormally.

Condition: This symptom occurs if IPsec and DPD are deployed, the device acts as the headquarters device, has a large number of IKE packets to handle, and has run for a long time.

202305251580

Symptom: The output from the display resource-monitor command shows that the VSI resource specification is 2K, which does not match the specification list.

Condition: This symptom occurs if you execute the display resource-monitor command to view VSI resource information.

202305130310

Symptom: The following CAR-exceeded packet loss log is mistakenly reported:

%Apr 18 04:49:15:237 2023 zubojieru-sw DRVPLAT/4/SOFTCAR DROP: -Slot=2;

PktType=UNKNOWN_IPV4MCiptAKNOWN_IPV4MC , SrcMAC=642f-c7aa-d401, Dropped at Stage=0, StageCnt=0, TotalCnt=1.

Condition: This symptom occurs after the switch receives an unknown multicast packet and creates a drop-unknown entry.

202305200041

Symptom: The ACL resources are insufficient because the ACL resource occupation mode of voice VLAN is still the port mode after it is configured as the global mode.

Condition: This symptom occurs if IP phones are automatically discovered through LLDP.

202305230595

Symptom: A device cannot access the local device by using SSH through an aggregate interface. However, that device can ping the local device.

Condition: This symptom occurs if the following operations are performed on the local device:

              - Configure remote Web authentication.

              - Use the web-auth free-ip command to specify Web authentication-free subnets.

              - Enable Web authentication on the aggregate interface and an Ethernet interface.

              - Remove the Web authentication-free subnets.

              - Reconfigure the Web authentication-free subnets.

202306070855

Symptom: Packets carry incorrect source MAC addresses after being forwarded by an EVPN M-LAG system.

Condition: This symptom might occur if packets received on a tunnel interface are forwarded at Layer 3 over the peer-link to a singlehomed M-LAG interface on the M-LAG peer.

  - Resolved problems in R6628P35

202303280502

Symptom: The display interface brief command displays a nonexistent management port (MGE0/0/2) when it is executed on an IRF fabric.

Condition: This symptom might occur when you execute the display interface brief command on an IRF fabric.

202303130130

Symptom: Traffic coming into an AC interface is sent out of that AC interface, forming a loop.

Condition: This symptom occurs when the AC interface receives traffic whose source MAC address is the same as its destination MAC address.

202303101546

Symptom: After obtaining an IPv6 address through DHCPv6, the device fails to add the default route to its routing table.

Condition: This symptom might occur if a device uses DHCPv6 for IPv6 address acquisition.

202303280505

Symptom: The device cannot communicate with the directly connected peer device through IPv6, and the packet loss ratio approaches 100%.

Condition: This symptom occurs if a large number of unknown multicast packets exist between the devices and therefore ICMPv6 packets are abnormally dropped.

202303101686

Symptom: Enable the DHCP snooping entry auto backup feature, and back up the DHCP snooping entries for one time. When you use the dhcp snooping binding database update now command to manually save DHCP snooping entries to the backup file again, the backup fails. In this case, the Status field displays writing in the command output from the display dhcp snooping binding database command.

Condition: This symptom occurs if the TFTP server does not support the protocol length feature.

202303031164

Symptom: After a SmartMC member device restarts, the device keeps reporting log messages for local login failures. The log content is "Feb 24 14:41:31:3042023 H3C NETCONF/6/SOAP_XML_LOGIN: admin from 127.0.0.1 loginfailed."

Condition: This symptom might occur if you use smartmc tc password on the commander to edit the password of default user admin for members, save member configurations, and then restart members.

202303101381

Symptom: The IRF fabric reboots because the memory is exhausted.

Condition: This symptom occurs if a master/subordinate switchover is performed or a DHCP client requests multiple addresses from the IRF fabric acting as a DHCP relay.

202304101304

Symptom: An AP attached to an M-LAG system cannot obtain an IP address.

Condition: This symptom occurs if the M-LAG member devices act as management gateways and ARP snooping is enabled on them.

202303141487

Symptom: The DHCP process exits unexpectedly and then recovers after DHCP relay entries are aged out.

Condition: This symptom occurs if the following conditions exist:

  - The switch acts as a DHCP relay.

  - A DHCP client obtains two IP addresses on an interface and then obtained one of the two addresses on anther interface.

  - The DHCP relay entries are aged out.

202303240777

Symptom: When the device is automatically deployed, some ports fail to be assigned to an aggregation group.

Condition: This symptom occurs if the device is automatically deployed and multiple ports are assigned to the same aggregation group.

202303220706

Symptom: When the RADIUS authentication server for 802.1X authentication is unreachable, users cannot bypass authentication through the none authentication method.

Condition: This symptom occurs if the RADIUS authentication server is unreachable and the none authentication method is used.

Workaround: Execute the dot1x critical eapol command.

202303100258

Symptom: A server attached to an EVPN M-LAG system cannot ping an external network.

Condition: This symptom might occur if an M-LAG interface with the lacp edge-port setting configured flaps repeatedly.

202302270531

Symptom: After an IRF fabric splits, the subordinate device cannot detect loops.

Condition: This symptom might occur if an IRF fabric splits.

202303030899

Symptom: BGP sessions flap.

Condition: This symptom might occur if the device receives a large number of packets that do not match any routes.

202302201089

Symptom: The device does not support collecting packet statistics on Layer 3 aggregate subinterfaces.

Condition: This symptom might occur if the traffic-statistic enable command is executed on Layer 3 aggregate subinterfaces.

202302170758

Symptom: Track is associated with EAA. When the state of a track entry changes from negative to positive, the monitoring policy action is not executed.

Condition: This symptom might occur if a track monitoring event is associated with multiple track entries and one of the track entries changes from not ready state to positive state.

202303090021

Symptom: On an IRF fabric, traffic received on a Layer 3 aggregate interface cannot be forwarded between the IRF member devices.

Condition: This symptom might occur if a Layer 3 aggregation group is created before IRF physical interfaces are bound to IRF ports.

202302240358

Symptom: The device reboots unexpectedly because of a kernel exception.

Condition: This symptom might occur if the display diagnostic-information command is executed.

202301092178

Symptom: When a TFTP server is used to save auto backup DHCP snooping entries, only one entry can be stored.

Condition: This symptom occurs if a TFTP server is used to save auto backup DHCP snooping entries.

202302160003

Symptom: Static EVPN MAC address entries synchronized from the remote VTEP to the local VTEP are deleted.

Condition: This symptom occurs if only static EVPN MAC address entries are synchronized from the remote VTEP to the local VTEP. The synchronized EVPN MAC address entries are deleted after an aging period.

202302101133

Symptom: When a VXLAN tunnel is used as a peer link on an EVPN M-LAG network, the broadcast packets received on the peer link are incorrectly forwarded to the local M-LAG interface.

Condition: This symptom occurs if broadcast packets are received on the peer link.

202302101493

Symptom: On an IRF fabric, the SNMP server does not receive link-down alarms from IRF physical interfaces.

Condition: This symptom occurs if a service interface of the subordinate IRF member device is connected to the SNMP server and the IRF physical interfaces go down.

202212300039

Symptom: The device reboots unexpectedly.

Condition: This symptom occurs if MQC configuration matches both IPv4/IPv6 packets and packets with multiple outer VLAN tags.

202302021438

Symptom: The switch prints an error message when a DHCPv6 client requests an IPv6 prefix from the DHCPv6 server through the switch.

Condition: This symptom occurs if the switch acts as a DHCP snooping device and you have executed the ipv6 dhcp snooping pd binding record and ipv6 verify source ip-address mac-address commands on the switch.

202301110093

Symptom: On an M-LAG system, ARP entries and MAC address entries are incorrect, and the peer link cannot be used to forward traffic.

Condition: This symptom occurs if the M-LAG system is automatically deployed by using devices that start up with initial configuration.

202301111261

Symptom: On an EVPN VXLAN M-LAG system formed by two leaf devices, reboot of one M-LAG member device results in reboot of the other M-LAG member device. The M-LAG system resumes operation after multiple automatic reboots.

Condition: This symptom occurs if 1500 MAC authentication users access the network through ARP learning and the primary member device is rebooted.

202212300039

Symptom: The device reboots unexpectedly.

Condition: This symptom occurs if MQC configuration matches both IPv4/IPv6 packets and packets with multiple outer VLAN tags.

202301060304

Symptom: A delay exists when MAC authentication users access the network.

Condition: This symptom occurs if MAC authentication users go offline after successful authentication and MAC authentication is triggered again.

202212081055

Symptom: The device cannot come online because the ipv6 address dhcp-alloc command on VLAN interface 1 is lost.

Condition: This symptom occurs if the automatic configuration process ends or you manually terminate the automatic configuration process during an IPv6 automatic deployment.

202212080022

Symptom: The device reboots when a large number of MAC authentication users come online and go offline on an aggregate interface and ACLs and URLs are authorized to the users.

Condition: This symptom occurs might if a large number of MAC authentication users come online and go offline on an aggregate interface and ACLs and URLs are authorized to the users.

202208301357

Symptom: An endpoint cannot pass Web authentication in an M-LAG system.

Condition: This symptom occurs if the following conditions exist:

  - The distributed even-/odd-MAC mode is configured for authentication load sharing.

  - The MAC address of the endpoint is an odd MAC address.

  - The authentication packets are sent to the M-LAG member device in distributed even-MAC mode.

202211280698

Symptom: When a route server reflects an EBGP route, it mistakenly modifies the router MAC address in the route as its own router MAC address.

Condition: This symptom occurs if you have executed both the peer route-server-client and peer router-mac-local dci commands on the route server.

202211240773

Symptom: A clients reports two different XPATH messages, and another client reports no messages.

Condition: This symptom occurs if you configure gNMI subscriptions and concurrent sessions exist.

202301040134

Symptom: An error occurs during device startup.

Condition: This symptom occurs if the device starts up.

202301040841

Symptom: After you execute the display mad verbose command on an IRF member device, the command output displays both VLAN interfaces and excluded ports while only VLAN interfaces should be displayed.

Condition: This symptom occurs if an IRF fabric is split and then established.

202207050531

Symptom: After you delete an interface, the resources allocated to the interface cannot be released. As a result, the system cannot allocate these resources to other functions.

Condition: This symptom occurs if you delete the source interface specified for VXLAN default decapsulation.

202212280017

Symptom: In an EVPN multicast network, the multicast traffic is mistakenly forwarded.

Condition: This symptom occurs if the device forwards multicast traffic.

202212280016

Symptom: A QoS policy on an M-LAG member device fails to match with the packets sent from the peer.

Condition: This symptom occurs if an M-LAG member device configured with a QoS policy receives packets from the peer through the peer link in an M-LAG system.

202301051651

Symptom: Failed to restore the default settings for a Smartrate-Ethernet interface by executing the default command.

Condition: This symptom occurs if you have executed the stp instance 0 port priority 16 command on a Smartrate-Ethernet interface.

202301040139

Symptom: In an M-LAG system, the interfaces in M-LAG MAD DOWN state fail to restore to normal after an M-LAG member device restarts.

Condition: This symptom occurs if the member port rates of the peer link interface are inconsistent.

202212220265

Symptom: The device fails to issue the m-lag extra-vlan command through NETCONF for the first time.

Condition: This symptom occurs if the device issues the m-lag extra-vlan command through NETCONF for the first time after device startup.

202212281015

Symptom: In a VXLAN network configured with M-LAG, the device acting as a leaf node drops multicast packets from the spine.

Condition: This symptom occurs when the leaf node receives multicast packets from the spine in a VXLAN network configured with M-LAG.

202212260304

Symptom: The OSPF neighbors and PIM neighbors flap.

Condition: This symptom occurs if the device receives a large number of multicast packets with TTL 1.

202212191223

Symptom: On an MPLS network, the VSI TTI configuration is not cleared after you configure AC settings and then restore the device to empty configuration.

Condition: This symptom occurs if you configure AC on the device and then restore the empty configuration for the device.

202212060168

Symptom: No output is displayed upon execution of the display kernel reboot command.

Condition: This symptom might occur when you execute the display kernel reboot command to view information about device reboot events.

202211181050

Symptom: In an M-LAG network, online 802.1X user go offline and new users cannot come online after one member device (leaf device) is upgraded.

Condition: This symptom occurs if you form an M-LAG network by using two leaf devices and upgrade one leaf device

202207111528

Symptom: No commands can be entered after the dmesg command is executed.

Condition: This symptom occurs if you enter the dmesg command in kdb view after the device is power cycled.

202208051014

Symptom: In a VPLS network, the packets of a PW have inner encapsulation errors.

Condition: This symptom occurs if you flap the PW-side interface repeatedly.

202208040950

Symptom: VPLS packets fail to be forwarded in an MPLS network with P devices.

Condition: This symptom occurs if the outgoing label on the public network PE is the same as the incoming label on the P device.

202207220046

Symptom: Endpoints fail to be obtain IP addresses from the IRF fabric acting as a DHCP server.

Condition: This symptom occurs if the IRF fabric connects to the AC interface and experiences a master/subordinate switchover.

202206020061

Symptom: Cross-subnet packets cannot be forwarded in hardware.

Condition: This symptom occurs if the peer device is enabled with source MAC check.

202208041205

Symptom: The HardwareRev information about a subcard read through NETCONF is wrong.

Condition: This symptom occurs if you read the HardwareRev information about a subcard through NETCONF.

202206161204

Symptom: A user fails to obtain an IP address and fails to come online after the user.

Condition: This symptom occurs if the following conditions exist:

  - Policy check is enabled on the server.

  - The user comes online from the isolation security group and passes security checks.

  - The user is switched to the service security group.

202206210576

Symptom: The configuration fails to take effect because the free memory is insufficient.

Condition: This symptom occurs if a physical interface goes down and comes up frequently.

202206250439

Symptom: The VPN instance associated with interface does not take effect after the device reboots

Condition: This symptom occurs if you associate the same VPN instance with a Layer 3 Ethernet subinterface and a VLAN interface that have the same interface number.

202205231332

Symptom: PoE current is not displayed in the output from the display poe-power command.

Condition: This symptom might occur if the following conditions exist:

  - A GRE power supply and an LITEON power supply are installed on the device.

  - The GRE power supply supplies power and the LITEON power supply does not supply power

202112271474

Symptom: Member devices in a VXLAN DR system might reboot unexpectedly.

Condition: This symptom might occur if a certain script is executed.

202109060975

Symptom: PIM DM is disabled on a VLAN interface, Layer 2 multicast entries are not established on the subordinate IRF member device, and multicast traffic is broadcast within the VLAN.

Condition: This symptom occurs if both Layer 2 multicast and Layer 3 multicast are configured for the same VLAN, traffic is received on the subordinate IRF member device, and IGMP snooping is configured for the VLAN on an IRF fabric.

202205270372

Symptom: Outgoing packets carry an incorrect source MAC address.

Condition: This symptom occurs if the following operations have been performed:

  - Configure a MAC address on a VLAN interface.

  - Delete the VLAN interface and re-create it.

202205240571

Symptom: Threads of OSPFv3 access invalid pointers and are hanged, the core is abnormal, and routes are not updated.

Condition: This symptom occurs if the following operations are performed:

              - Configure a VPN instance that has no OSPFv3 instances.

              - Associate the VPN instance with an interface and execute the ipv6 address command on the interface.

              - Execute OSPFv3 preconfigured commands but not OSPFv3 enable commands. The ospfv3 1 area 0 command is an example of OSPFv3 enable commands. OSPFv3 preconfigured commands refer to commands other than enable commands, such as ospfv3 timer hello, ospfv3 network-type, and ospfv3 cost.

              - Remove the VPN instance-interface association or delete the VPN instance.

202204110848

Symptom: Source ports in a local mirroring group fail to be configured after the source ports in another local mirroring group are configured.

Condition: This symptom occurs if the following operations are performed:

  - Configure the monitor port as the same port for seven local mirroring groups.

  - Configure the source ports for the seventh local mirroring group.

  - Configure the source ports for another local mirroring group among the remaining local mirroring groups.

202110261296

Symptom: In an inter-VPN forwarding scenario, multicast traffic cannot be forwarded to the public network.

Condition: This symptom occurs if a member port is repeatedly added to and removed from the aggregate interface for the tunnel and the private route flaps.

202112310599

Symptom: The device issues Layer 3 IPv4 multicast entries successfully and might fail to issue some Layer 3 IPv6 multicast entries, which causes multicast forwarding errors.

Condition: This symptom might occur if the device issues 3000 IPv4 IPMC multicast entries and then 250 IPv6 IPMC multicast entries and the number of multicast entries reaches the upper limit.

Resolved problems in R6615P07

202205111296

Symptom: A VSI interface in down state can still act as a gateway interface to forward traffic.

Condition: This symptom occurs if the shutdown command is executed on a VSI interface configured as the VXLAN gateway interface.

202205111299

Symptom: When a PoE interface fails to supply power, the traps cannot correctly report the failure.

Condition: This symptom occurs if the maximum power configured on the PoE interface cannot meet the power requirements of the attached PDs.

202205111292

Symptom: Within 5 minutes after the VCF fabric is automatically deployed. the devices try to obtain the device list file.

Condition: This symptom occurs if legacy automated deployment is performed for the devices and the device list is not configured.

202205111301

Symptom: After the VCF fabric is automatically deployed, the original PVID settings of interfaces are lost.

Condition: This symptom occurs if a device is automatically deployed as an access device, the interfaces have original PVID settings, the interfaces are connected to APs, and then the APs are removed.

202203300334

Symptom: The device reboots unexpectedly.

Condition: This symptom occurs if an AC is associated with a VSI on the device.

202201200603

Symptom: When loop detection is configured on a VSI and ARP packets are injected to a blocked AC, the AC can still respond with ARP replies normally.

Condition: This symptom occurs if ARP proxy is configured on the VSI.

202108170529

Symptom: The MAC address entries for MAC authentication users and 802.1x users are not deleted after they go offline.

Condition: This symptom occurs if MAC authentication users and 802.1x users move between member devices on an IRF fabric.

202203211300

Symptom: After a transceiver module is installed into a port, the device reboots unexpectedly.

Condition: This symptom occurs if the following conditions exist:

              - A DR system has peer links.

              - Configure an AC on the DR interface (an aggregate interface).

              - On a single-homed interface, configure an AC with the same service instance.

202201050390

Symptom: A MAC address entry does not age out if the MAC address moves between two interfaces constantly.

Condition: This symptom might occur if two interfaces receive packets sourced from the same MAC address.

202112281596

Symptom: An EVPN DR system uses an Ethernet aggregate link as the IPL. After an AC is deleted and recreated, the AC does not take effect.

Condition: This symptom might occur if the following events occur:

              - The maximum number of ACs is reached.

              - A static AC is deleted and recreated on a non-DR interface or DR interface.

202112280428

Symptom: MAC address entries are not deleted completely, and the type of the MAC address entries is incorrect.

Condition: This symptom might occur if the following events occur on a DR system formed by two devices with different capabilities:

              - The traffic load reaches the limit of the device with higher capabilities.

              - The reset l2vpn mac command is executed.

202112270862

Symptom: An EVPN DR system does not forward the broadcast traffic received from DR interfaces over the IPL.

Condition: This symptom might occur if the configuration is rolled back after the b_evpn_drni_no_peer_link_1_3_3.tc script is executed.

202112301425

Symptom: On an EVPN DR system, synchronized MAC addresses are issued to incorrect ACs, and this issue cannot be recovered.

Condition: This symptom might occur if ACs match single-tagged packets and the following operations are performed:

              - ACs matching the same VLAN are mapped to different VSIs.

              - The ACs are deleted.

              - The ACs are recreated to match the same VLAN and mapped to the same VSI.

202201040231

Symptom: The device fails to forward some multiple packets.

Condition: This symptom might occur if BIDIR-PIM is enabled and RPs are configured in BIDIR-PIM domains.

202112280864

Symptom: MAC address learning is disabled globally when the device is receiving dense traffic, but dynamic MAC address entries are not deleted.

Condition: This symptom might occur if MAC address learning is disabled globally when the device is receiving dense traffic.

202111260029

Symptom: MAC address entries created for MAC authentication users are not deleted after MAC authentication is disabled on DR interfaces.

Condition: This symptom might occur if MAC authentication is disabled on DR interfaces of a DR system that uses an Ethernet aggregate link as the IPL.

202110191417

Symptom: Once removed from a monitoring group, an interface cannot be assigned to monitoring groups again.

Condition: This symptom might occur if traffic is mirrored to a monitoring group through local mirroring and flow mirroring.

202112081609

Symptom: On an EVPN DR system, a BGP task is abnormal and creates a core file.

Condition: This symptom might occur if the DR system receives ARP packets and 1000 attached hosts migrate from the DR system.

202112291428

Symptom: A non-existent VLAN is created on the primary DR device in type 2 configuration consistency check.

Condition: This symptom might occur if the following operations are performed:

              - Two devices are booted with initial configuration, and they are configured to set up a DR system.

              - The keepalive link comes up.

              - An IPP is configured on the primary and secondary devices in sequence.

202112291070

Symptom: Users fail authentication after the attached IRF fabric reboots.

Condition: This symptom might occur if an IRF master/subordinate switchover occurs when the interface used for authentication is down and users are online.

202112250446/202112081895

Symptom: EVPN and Layer 2 multicast are configured on the device, and the igmp-snooping drop-unknown setting does not take effect.

Condition: This symptom might occur if a VXLAN ID is deleted and recreated on a VSI.

202112081745

Symptom: The device generates blackhole MAC address entries and does not forward certain traffic.

Condition: This symptom might occur if incoming traffic matches a MAC-based VLAN and an IP subnet-based VLAN simultaneously on the same interface.

202112271474

Symptom: Member devices in a VXLAN DR system might reboot unexpectedly.

Condition: This symptom might occur if a certain script is executed.

202110210626

Symptom: AC resources for a VSI might not be deleted completely when an authentication user logs off and then logs on again.

Condition: This symptom might occur if user MAC addresses move between interfaces and a large number of authentication users exist.

202112131788

Symptom: EVPN is enabled to forward Layer 2 multicast traffic. After a VXLAN ID is deleted and then created again, the drop-unknown setting does not take effect.

Condition: This symptom might occur if a VXLAN ID is deleted and created again with the drop-unknown setting being intact.

202203181089

Symptom: When all types of MACsec debugging are enabled by using the debugging macsec all command, the system outputs an error message to indicate that it has received an oversized packet. However, the system does not receive an oversized packet.

Condition: This symptom occurs if MACsec is enabled on a port of the LSWM2XMGT2PM subcard and the local and peer MACsec-enabled ports forward traffic at wire speed.

202203211300

Symptom: A DR member device reboots unexpectedly after a transceiver module is installed.

Condition: This symptom might occur if the following conditions exist:

              - The DR member devices have a physical IPL between them.

              - An Ethernet service instance is configured both on a DR interface and an interface attached to a single-homed device.

Resolved problems in R6615P06

First release

Support and other resources

Accessing Hewlett Packard Enterprise Support

For live assistance, go to the Contact Hewlett Packard Enterprise Worldwide website:

www.hpe.com/assistance

To access documentation and support services, go to the Hewlett Packard Enterprise Support Center website:

www.hpe.com/support/hpesc

Information to collect:

Technical support registration number (if applicable).

Product name, model or version, and serial number.

Operating system name and version.

Firmware version.

Error messages.

Product-specific reports and logs.

Add-on products or components.

Third-party products or components.

Documents

To find related documents, see the Hewlett Packard Enterprise Support Center website at http://www.hpe.com/support/hpesc.

Enter your product name or number and click Go. If necessary, select your product from the resulting list.

For a complete list of acronyms and their definitions, see HPE FlexNetwork technology acronyms.

Related documents

The following documents provide related information:

HPE PSR150-A & PSR150-D Series Power Supplies User Guide

HPE PSR720-56A Power Supply User Guide

HPE PSR1110-56A Power Supply User Guide

HPE X721 Power to Port Fan Tray (5060-0175) & HPE X722 Port to Power Fan Tray (5060-0174) User Guide

HPE LSWM2XGT2PM Interface Card (JH156A) User Guide

HPE LSWM2SP2PM Interface Card (JH157A) User Guide

HPE LSWM2XMGT2PM Interface Card (R9L65A) User Guide

HPE FlexNetwork 5140 HI Switch Series Installation Guide

HPE FlexNetwork 5140 HI Switch Series Configuration Guides Index-R66xx

About the HPE FlexNetwork 5140 HI Switch Series Configuration Guides-R66xx

HPE FlexNetwork 5520 H  Switch Series Fundamentals Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Virtual Technologies Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Layer 2 - LAN Switching Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Layer 3 - IP Services Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Layer 3 - IP Routing Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series IP Multicast Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series MCE Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series ACL and QoS Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Security Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series High Availability Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Network Management and Monitoring Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series Telemetry Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series OpenFlow Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series VXLAN Configuration Guide-R66xx

HPE FlexNetwork 5140 HI Switch Series EVPN Configuration Guide-R66xx

About the HPE FlexNetwork 5140 HI Switch Series Command References-R66xx

HPE FlexNetwork 5140 HI Switch Series Fundamentals Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Virtual Technologies Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Layer 2 - LAN Switching Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Layer 3 - IP Services Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Layer 3 - IP Routing Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series IP Multicast Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series MCE Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series ACL and QoS Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Security Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series High Availability Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Network Management and Monitoring Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series OpenFlow Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series Telemetry Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series VXLAN Command Reference-R66xx

HPE FlexNetwork 5140 HI Switch Series EVPN Command Reference-R66xx

Documentation feedback

Hewlett Packard Enterprise is committed to providing documentation that meets your needs. To help us improve the documentation, send any errors, suggestions, or comments to Documentation Feedback (docsfeedback@hpe.com). When submitting your feedback, include the document title, part number, edition, and publication date located on the front cover of the document. For online help content, include the product name, product version, help edition, and publication date located on the legal notices page.

Feature list

Hardware features

5140HI series hardware features for non-PoE switch models

| Item | HPE 5140 24G 4SFP+ 1-slot HI Sw | HPE 5140 48G 4SFP+ 1-slot HI Sw |
| --- | --- | --- |
| Dimensions (H × W × D) | 43.6 × 440 × 360 mm (1.72 × 17.32 × 14.17 in) |  |
| Weight | ≤ 7 kg (15.43 lb) |  |
| Console ports | - 1 × Micro USB console port - 1 × serial console port Only the Micro USB console port is available when you connect both ports. |  |
| USB port | 1 |  |
| Management Ethernet port | 1 |  |
| SFP+ port | 4 |  |
| SFP port | 8 | - |
| 10/100/1000Base-T Ethernet ports | 24 | 48 |
| SFP+ ports | 4 |  |
| Expansion slot | 1, on the rear panel |  |
| Power module slot | 2, on the rear panel |  |
| Fan tray slot | 2, on the rear panel |  |
| Input voltage | AC power source Rated voltage: 100 VAC to 240 VAC @ 50 or 60 Hz Max voltage: 90 VAC to 264 VAC @ 47 to 63 Hz DC power source: –48 V DC power source in the equipment room or RPS (recommended HP RPS models: A-RPS800 or A-RPS1600) Rated voltage: –48 VDC to –60 VDC Max voltage: –36 VDC to –72 VDC |  |
| Minimum power consumption | Single AC input: 24 W Single DC input: 24W Dual AC inputs:29 W Dual DC inputs: 298W | Single AC input: 27 W Single DC input: 24W Dual AC inputs: 31 W - Dual DC inputs: 29 W |
| Maximum power consumption | - Single AC input: 87W - Single DC input: 88W - Dual AC inputs: 91 W - Dual DC inputs: 95W | - Single AC input: 88W - Single DC input: 89W - Dual AC inputs: 93 W - Dual DC inputs: 96 W |
| Chassis leakage current compliance | UL60950-1 EN60950-1 IEC60950-1 GB4943.1 |  |
| Melting current of power supply fuse | AC-input: 5 A/250 V DC-input: 8 A/250 V |  |
| Operating temperature | -5°C to 45°C (23°F to 113°F) |  |
| Operating humidity | 5% to 95%, noncondensing |  |
| Fire resistance compliance | UL60950-1 EN60950-1 IEC60950-1 GB4943.1 |  |

5140HI series hardware features for PoE switch models

| Item | HPE 5140 24G PoE+ 4SFP+ 1-slot HI Sw | HPE 5140 48G PoE+ 4SFP+ 1-slot HI Sw |
| --- | --- | --- |
| Dimensions (H × W × D) | 43.6 × 440 × 460 mm(1.72 × 17.32 × 18.11 in) |  |
| Weight | ≤ 9.6 kg (21.16 lb) |  |
| Console ports | - 1 × Micro USB console port - 1 × serial console port Only the Micro USB console port is available when you connect both ports. |  |
| USB port | 1 |  |
| Management Ethernet port | 1 |  |
| SFP+ port | 4 |  |
| 10/100/1000Base-T Ethernet ports | 24 | 48 |
| Expansion slot | 1, on the rear panel |  |
| Power module slot | 2, on the rear panel |  |
| Fan tray slot | 2, on the rear panel |  |
| Input voltage | AC input for the PSR360-56A/PSR720-56A power module: Rated voltage range: 100 VAC to 240 VAC @ 50 Hz or 60 Hz Max voltage range: 90 VAC to 264 VAC @ 47 Hz to 63 Hz AC input for the PSR1110-56A power module: Rated voltage range: 115 VAC to 240 VAC @ 50 Hz or 60 Hz Max voltage range: 102.5 VAC to 264 VAC @ 47 Hz to 63 Hz You can use a –48 VDC power source in the equipment room or an H3C RPS (RPS1600-A). |  |
| PoE power capacity | Depends on the power module configurations. For more information, see Table 7. |  |
| Minimum power consumption | 31.03W | 33.09W |
| Maximum power consumption (including PoE power consumption) | 927.2W | 1729W |
| Chassis leakage current compliance | UL60950-1 EN60950-1 IEC60950-1 GB4943.1 |  |
| Melting current of power supply fuse | 15 A/250 V |  |
| Operating temperature | -5°C to 45°C (23°F to 113°F) |  |
| Operating humidity | 5% to 95%, noncondensing |  |
| Fire resistance compliance | UL60950-1 EN60950-1 IEC60950-1 GB4943.1 |  |

PoE power capacity of the HPE 5140 48G PoE+ 4SFP+ 1-slot HI Sw Swch switches

| Power module configuration | HPE 5140 24G PoE+ 4SFP+ 1-slot HI Sw |  | HPE 5140 48G PoE+ 4SFP+ 1-slot HI Sw |  |
| --- | --- | --- | --- | --- |
|  | Total PoE power capacity | Max PoE power capacity per port | Total PoE power capacity | Max PoE power capacity per port |
| 2 × PSR1110-56A | 810W | 30W | 1680w | 30 W |
| 1 × PSR1110-56A and 1 × PSR720-56A | 810W | 30W | 1560 W | 30 W |
| 1 × PSR1110-56A and 1 ×PSR360-56A | 810W | 30W | 1200W | 30W |
| 2 × PSR720-56A | 810W | 30W | 1200W | 30W |
| 1 × PSR1110-56A | 810W | 30W | 810 W | 30 W |
| 1 × PSR720-56A and 1 × PSR360-56A | 810W | 30W | 810 W | 30 W |
| 1 × PSR720-56A | 450W | 30W | 450 W | 30 W |
| 2 × PSR360-56A | 450W | 30W | 450 W | 30 W |
| 1 × PSR360-56A | 180W | 30W | 180 W | 30 W |

Software features

Software features of the 5140HI series

| Feature | HPE 5140 24G 4SFP+ 1-slot HI Sw HPE 5140 48G 4SFP+ 1-slot HI Sw HPE 5140 24G PoE+ 4SFP+ 1-slot HI Sw HPE 5140 48G PoE+ 4SFP+ 1-slot HI Sw |
| --- | --- |
| Full duplex Wire speed L2 switching capacity | - Excluding subcards: 176 Gbps - Including subcards: - For frames (length=64 bytes): 240 Gbps - For frames (length≥128 bytes): 336 Gbps |
| Whole system Wire speed L2 switching Packet forwarding rate | 180Mpps |
| Link aggregation | - Aggregation of GE ports - Aggregation of 10-GE ports - Aggregation of 40-GE ports - Static link aggregation - Dynamic link aggregation - Inter-device aggregation - A maximum of 128 inter-device aggregation groups - A maximum of 32 ports for each aggregation group |
| Flow control | - IEEE 802.3x flow control |  |
| Jumbo Frame | - Supports maximum frame size of 10000 |
| MAC address table | - 64K MAC addresses - 1K static MAC addresses - Blackhole MAC addresses - MAC address learning limit on a port |
| VLAN | - A maximum of 4094 port-based VLANs - QinQ, selective QinQ, VLAN mapping - Voice VLANs - Protocol-based VLANs - MAC-based VLANs |
| ARP | - A maximum of 32K ARP entries (a maximum number of 32K ARP entries in later versions) - A maximum of 2K static ARP entries - Gratuitous ARP - ARP attack detection based on DHCP snooping entries, 802.1X entries, and static IPSG bindings - ARP rate limit |
| ND | - 16K entries - 2K static entries - ND Snooping |
| VLAN virtual interface | 1K |
| DHCP | - DHCP client - DHCP snooping - DHCP relay - DHCP server - DHCP Option82 |
| DHCPv6 | - DHCPv6 server - DHCPv6 relay - DHCPv6 client - DHCPv6 snooping |
| DNS | - Static DNS - Dynamic DNS - IPv4 and IPv6 DNS |
| unicast route | - IPv4 and IPv6 static routes - RIP/RIPng - OSPF/OSPFv3 - BGP/IPv6 BGP - ISIS/ISISv6 |
| Multicast | - IGMP Snooping - MLD Snooping - multicast VLAN - PIM SM - PIM DM - MSDP |
| Broadcast/multicast/unicast storm control | - Storm control based on port rate percentage - PPS-based storm control - Bps-based storm control |
| MSTP | - STP/RSTP/MSTP protocol - 64 Spanning Tree instances - STP Root Guard - BPDU Guard |
| SmartLink | - 32 |
| RRPP | - RRPP |
| QoS/ACL | - Remarking of 802.1p and DSCP priorities - Packet filtering at L2 (Layer 2) through L4 (Layer 4) - Eight output queues for each port - SP/WRR/SP+WRR queue scheduling algorithms - WRED - Port-based rate limiting - Flow-based redirection - Time range |
| Mirroring | - Local port mirroring - A maximum number of 7 mirroring groups - Layer 2 remote port mirroring - Flow mirroring |
| Security | - Hierarchical management and password protection of users - AAA authentication - RADIUS authentication - HWTACACS - SSH 2.0 - Port isolation - 802.1X - Port security - MAC-address-based authentication - IP Source Guard - HTTPS - PKI - EAD |
| 802.1X | - Up to 2K users - Port-based and MAC address-based authentication - Guest VLAN - Trunk port authentication - Dynamic 802.1X-based ACL/VLAN assignment |
| Open Flow | - 16 Instances - 1500 flow entries (issued by using ACL) |
| Loading and upgrading | - Loading and upgrading through XModem protocol - Loading and upgrading through FTP - Loading and upgrading through the trivial file transfer protocol (TFTP) |
| Management | - Configuration at the command line interface - Remote configuration through Telnet - Configuration through Console port - Simple network management protocol (SNMP) - Remote Monitoring(RMON) - IMC NMS - Web network management (later version) - System log - Hierarchical alarms - IRF - NTP - Power supply alarm function - Fan and temperature alarms |
| Maintenance | - Debugging information output - Ping and Tracert - Remote maintenance through Telnet - NQA - 802.1ag - 802.3ah - DLDP - Virtual Cable Test |

Fixed security vulnerabilities

Fixed security vulnerabilities in R6652P05

CVE-2023-2650

Issue summary: Processing some specially crafted ASN.1 object identifiers or data containing them may be very slow. Impact summary: Applications that use OBJ_obj2txt() directly, or use any of the OpenSSL subsystems OCSP, PKCS7/SMIME, CMS, CMP/CRMF or TS with no message size limit may experience notable to very long delays when processing those messages, which may lead to a Denial of Service. An OBJECT IDENTIFIER is composed of a series of numbers - sub-identifiers - most of which have no size limit. OBJ_obj2txt() may be used to translate an ASN.1 OBJECT IDENTIFIER given in DER encoding form (using the OpenSSL type ASN1_OBJECT) to its canonical numeric text form, which are the sub-identifiers of the OBJECT IDENTIFIER in decimal form, separated by periods. When one of the sub-identifiers in the OBJECT IDENTIFIER is very large (these are sizes that are seen as absurdly large, taking up tens or hundreds of KiBs), the translation to a decimal number in text may take a very long time. The time complexity is O(n^2) with 'n' being the size of the sub-identifiers in bytes (*). With OpenSSL 3.0, support to fetch cryptographic algorithms using names / identifiers in string form was introduced. This includes using OBJECT IDENTIFIERs in canonical numeric text form as identifiers for fetching algorithms. Such OBJECT IDENTIFIERs may be received through the ASN.1 structure AlgorithmIdentifier, which is commonly used in multiple protocols to specify what cryptographic algorithm should be used to sign or verify, encrypt or decrypt, or digest passed data. Applications that call OBJ_obj2txt() directly with untrusted data are affected, with any version of OpenSSL. If the use is for the mere purpose of display, the severity is considered low. In OpenSSL 3.0 and newer, this affects the subsystems OCSP, PKCS7/SMIME, CMS, CMP/CRMF or TS. It also impacts anything that processes X.509 certificates, including simple things like verifying its signature. The impact on TLS is relatively low, because all versions of OpenSSL have a 100KiB limit on the peer's certificate chain. Additionally, this only impacts clients, or servers that have explicitly enabled client authentication. In OpenSSL 1.1.1 and 1.0.2, this only affects displaying diverse objects, such as X.509 certificates. This is assumed to not happen in such a way that it would cause a Denial of Service, so these versions are considered not affected by this issue in such a way that it would be cause for concern, and the severity is therefore considered low.CVE-2022-32221

When doing HTTP(S) transfers, libcurl might erroneously use the read callback (`CURLOPT_READFUNCTION`) to ask for data to send, even when the `CURLOPT_POSTFIELDS` option has been set, if the same handle previously was used to issue a `PUT` request which used that callback. This flaw may surprise the application and cause it to misbehave and either send off the wrong data or use memory after free or similar in the subsequent `POST` request. The problem exists in the logic for a reused handle when it is changed from a PUT to a POST.

CVE-2023-2953

A vulnerability was found in openldap. This security flaw causes a null pointer dereference in ber_memalloc_x() function.

CVE-2023-0465

Applications that use a non-default option when verifying certificates may be vulnerable to an attack from a malicious CA to circumvent certain checks. Invalid certificate policies in leaf certificates are silently ignored by OpenSSL and other certificate policy checks are skipped for that certificate. A malicious CA could use this to deliberately assert invalid certificate policies in order to circumvent policy checking on the certificate altogether. Policy processing is disabled by default but can be enabled by passing the ‘-policy’ argument to the command line utilities or by calling the `X509_VERIFY_PARAM_set1_policies()' function.

CVE-2023-24329

An issue in the urllib.parse component of Python before 3.11.4 allows attackers to bypass blocklisting methods by supplying a URL that starts with blank characters.

CVE-2023-0286

There is a type confusion vulnerability relating to X.400 address processing inside an X.509 GeneralName. X.400 addresses were parsed as an ASN1_STRING but the public structure definition for GENERAL_NAME incorrectly specified the type of the x400Address field as ASN1_TYPE. This field is subsequently interpreted by the OpenSSL function GENERAL_NAME_cmp as an ASN1_TYPE rather than an ASN1_STRING. When CRL checking is enabled (i.e. the application sets the X509_V_FLAG_CRL_CHECK flag), this vulnerability may allow an attacker to pass arbitrary pointers to a memcmp call, enabling them to read memory contents or enact a denial of service. In most cases, the attack requires the attacker to provide both the certificate chain and CRL, neither of which need to have a valid signature. If the attacker only controls one of these inputs, the other input must already contain an X.400 address as a CRL distribution point, which is uncommon. As such, this vulnerability is most likely to only affect applications which have implemented their own functionality for retrieving CRLs over a network.

CVE-2023-0464

A security vulnerability has been identified in all supported versions of OpenSSL related to the verification of X.509 certificate chains that include policy constraints. Attackers may be able to exploit this vulnerability by creating a malicious certificate chain that triggers exponential use of computational resources, leading to a denial-of-service (DoS) attack on affected systems. Policy processing is disabled by default but can be enabled by passing the `-policy' argument to the command line utilities or by calling the `X509_VERIFY_PARAM_set1_policies()' function.

CVE-2023-0215

The public API function BIO_new_NDEF is a helper function used for streaming ASN.1 data via a BIO. It is primarily used internally to OpenSSL to support the SMIME, CMS and PKCS7 streaming capabilities, but may also be called directly by end user applications. The function receives a BIO from the caller, prepends a new BIO_f_asn1 filter BIO onto the front of it to form a BIO chain, and then returns the new head of the BIO chain to the caller. Under certain conditions, for example if a CMS recipient public key is invalid, the new filter BIO is freed and the function returns a NULL result indicating a failure. However, in this case, the BIO chain is not properly cleaned up and the BIO passed by the caller still retains internal pointers to the previously freed filter BIO. If the caller then goes on to call BIO_pop() on the BIO then a use-after-free will occur. This will most likely result in a crash. This scenario occurs directly in the internal function B64_write_ASN1() which may cause BIO_new_NDEF() to be called and will subsequently call BIO_pop() on the BIO. This internal function is in turn called by the public API functions PEM_write_bio_ASN1_stream, PEM_write_bio_CMS_stream, PEM_write_bio_PKCS7_stream, SMIME_write_ASN1, SMIME_write_CMS and SMIME_write_PKCS7. Other public API functions that may be impacted by this include i2d_ASN1_bio_stream, BIO_new_CMS, BIO_new_PKCS7, i2d_CMS_bio_stream and i2d_PKCS7_bio_stream. The OpenSSL cms and smime command line applications are similarly affected.

CVE-2022-4304

A timing based side channel exists in the OpenSSL RSA Decryption implementation which could be sufficient to recover a plaintext across a network in a Bleichenbacher style attack. To achieve a successful decryption an attacker would have to be able to send a very large number of trial messages for decryption. The vulnerability affects all RSA padding modes: PKCS#1 v1.5, RSA-OEAP and RSASVE. For example, in a TLS connection, RSA is commonly used by a client to send an encrypted pre-master secret to the server. An attacker that had observed a genuine connection between a client and a server could use this flaw to send trial messages to the server and record the time taken to process them. After a sufficiently large number of messages the attacker could recover the pre-master secret used for the original connection and thus be able to decrypt the application data sent over that connection.

CVE-2023-28321

An improper certificate validation vulnerability exists in curl <v8.1.0 in the way it supports matching of wildcard patterns when listed as "Subject Alternative Name" in TLS server certificates. curl can be built to use its own name matching function for TLS rather than one provided by a TLS library. This private wildcard matching function would match IDN (International Domain Name) hosts incorrectly and could as a result accept patterns that otherwise should mismatch. IDN hostnames are converted to puny code before used for certificate checks. Puny coded names always start with `xn--` and should not be allowed to pattern match, but the wildcard check in curl could still check for `x*`, which would match even though the IDN name most likely contained nothing even resembling an `x`.

CVE-2023-28322

An information disclosure vulnerability exists in curl <v8.1.0 when doing HTTP(S) transfers, libcurl might erroneously use the read callback (`CURLOPT_READFUNCTION`) to ask for data to send, even when the `CURLOPT_POSTFIELDS` option has been set, if the same handle previously wasused to issue a `PUT` request which used that callback. This flaw may surprise the application and cause it to misbehave and either send off the wrong data or use memory after free or similar in the second transfer. The problem exists in the logic for a reused handle when it is (expected to be) changed from a PUT to a POST.A vulnerability was found in curl.

CVE-2021-3753

A race problem was seen in the vt_k_ioctl in drivers/tty/vt/vt_ioctl.c in the Linux kernel, which may cause an out of bounds read in vt as the write access to vc_mode is not protected by lock-in vt_ioctl (KDSETMDE). The highest threat from this vulnerability is to data confidentiality.

CVE-2021-3739

A NULL pointer dereference flaw was found in the btrfs_rm_device function in fs/btrfs/volumes.c in the Linux Kernel, where triggering the bug requires ‘CAP_SYS_ADMIN’. This flaw allows a local attacker to crash the system or leak kernel internal information. The highest threat from this vulnerability is to system availability.

CVE-2021-45868

In the Linux kernel before 5.15.3, fs/quota/quota_tree.c does not validate the block number in the quota tree (on disk). This can, for example, lead to a kernel/locking/rwsem.c use-after-free if there is a corrupted quota file.

CVE-2022-1011

A flaw use after free in the Linux kernel FUSE filesystem was found in the way user triggers write(). A local user could use this flaw to get some unauthorized access to some data from the FUSE filesystem and as result potentially privilege escalation too.

CVE-2022-0854

A memory leak flaw was found in the Linux kernel’s DMA subsystem, in the way a user calls DMA_FROM_DEVICE. This flaw allows a local user to read random memory from the kernel space.

CVE-2022-0492

A vulnerability was found in the Linux kernel’s cgroup_release_agent_write in the kernel/cgroup/cgroup-v1.c function. This flaw, under certain circumstances, allows the use of the cgroups v1 release_agent feature to escalate privileges and bypass the namespace isolation unexpectedly.

CVE-2021-4002

A memory leak flaw in the Linux kernel's hugetlbfs memory usage was found in the way the user maps some regions of memory twice using shmget() which are aligned to PUD alignment with the fault of some of the memory pages. A local user could use this flaw to get unauthorized access to some data.

CVE-2022-25375

An issue was discovered in drivers/usb/gadget/function/rndis.c in the Linux kernel before 5.16.10. The RNDIS USB gadget lacks validation of the size of the RNDIS_MSG_SET command. Attackers can obtain sensitive information from kernel memory.

CVE-2020-7469

In FreeBSD 12.2-STABLE before r367402, 11.4-STABLE before r368202, 12.2-RELEASE before p1, 12.1-RELEASE before p11 and 11.4-RELEASE before p5 the handler for a routing option caches a pointer into the packet buffer holding the ICMPv6 message. However, when processing subsequent options the packet buffer may be freed, rendering the cached pointer invalid. The network stack may later dereference the pointer, potentially triggering a use-after-free.

CVE-2021-22924

libcurl keeps previously used connections in a connection pool for subsequenttransfers to reuse, if one of them matches the setup.Due to errors in the logic, the config matching function did not take 'issuercert' into account and it compared the involved paths *case insensitively*,which could lead to libcurl reusing wrong connections.File paths are, or can be, case sensitive on many systems but not all, and caneven vary depending on used file systems.The comparison also didn't include the 'issuer cert' which a transfer can setto qualify how to verify the server certificate.

CVE-2021-3753

A race problem was seen in the vt_k_ioctl in drivers/tty/vt/vt_ioctl.c in the Linux kernel, which may cause an out of bounds read in vt as the write access to vc_mode is not protected by lock-in vt_ioctl (KDSETMDE). The highest threat from this vulnerability is to data confidentiality.

CVE-2021-3739

A NULL pointer dereference flaw was found in the btrfs_rm_device function in fs/btrfs/volumes.c in the Linux Kernel, where triggering the bug requires ‘CAP_SYS_ADMIN’. This flaw allows a local attacker to crash the system or leak kernel internal information. The highest threat from this vulnerability is to system availability.

CVE-2021-45868

In the Linux kernel before 5.15.3, fs/quota/quota_tree.c does not validate the block number in the quota tree (on disk). This can, for example, lead to a kernel/locking/rwsem.c use-after-free if there is a corrupted quota file.

CVE-2022-1011

A flaw use after free in the Linux kernel FUSE filesystem was found in the way user triggers write(). A local user could use this flaw to get some unauthorized access to some data from the FUSE filesystem and as result potentially privilege escalation too.

CVE-2022-0854

A memory leak flaw was found in the Linux kernel’s DMA subsystem, in the way a user calls DMA_FROM_DEVICE. This flaw allows a local user to read random memory from the kernel space.

CVE-2022-0492

A vulnerability was found in the Linux kernel’s cgroup_release_agent_write in the kernel/cgroup/cgroup-v1.c function. This flaw, under certain circumstances, allows the use of the cgroups v1 release_agent feature to escalate privileges and bypass the namespace isolation unexpectedly.

CVE-2021-4002

A memory leak flaw in the Linux kernel's hugetlbfs memory usage was found in the way the user maps some regions of memory twice using shmget() which are aligned to PUD alignment with the fault of some of the memory pages. A local user could use this flaw to get unauthorized access to some data.

CVE-2022-25375

An issue was discovered in drivers/usb/gadget/function/rndis.c in the Linux kernel before 5.16.10. The RNDIS USB gadget lacks validation of the size of the RNDIS_MSG_SET command. Attackers can obtain sensitive information from kernel memory.

CVE-2020-7469

In FreeBSD 12.2-STABLE before r367402, 11.4-STABLE before r368202, 12.2-RELEASE before p1, 12.1-RELEASE before p11 and 11.4-RELEASE before p5 the handler for a routing option caches a pointer into the packet buffer holding the ICMPv6 message. However, when processing subsequent options the packet buffer may be freed, rendering the cached pointer invalid. The network stack may later dereference the pointer, potentially triggering a use-after-free.

CVE-2020-25577

In FreeBSD 12.2-STABLE before r368250, 11.4-STABLE before r368253, 12.2-RELEASE before p1, 12.1-RELEASE before p11 and 11.4-RELEASE before p5 rtsold(8) does not verify that the RDNSS option does not extend past the end of the received packet before processing its contents. While the kernel currently ignores such malformed packets, it passes them to userspace programs. Any programs expecting the kernel to do validation may be vulnerable to an overflow.

CVE-2020-8284

A malicious server can use the FTP PASV response to trick curl 7.73.0 and earlier into connecting back to a given IP address and port and this way potentially make curl extract information about services that are otherwise private and not disclosed for example doing port scanning and service banner extractions.

CVE-2020-8285

Curl 7.21.0 to and including 7.73.0 is vulnerable to uncontrolled recursion due to a stack overflow issue in FTP wildcard match parsing.

CVE-2021-22924

"libcurl keeps previously used connections in a connection pool for subsequenttransfers to reuse, if one of them matches the setup.Due to errors in the logic, the config matching function did not take 'issuercert' into account and it compared the involved paths *case insensitively*,which could lead to libcurl reusing wrong connections.File paths are, or can be, case sensitive on many systems but not all, and caneven vary depending on used file systems.The comparison also didn't include the 'issuer cert' which a transfer can setto qualify how to verify the server certificate."

CVE-2021-22925

curl supports the `-t` command line option, known as `CURLOPT_TELNETOPTIONS`in libcurl. This rarely used option is used to send variable=content pairs toTELNET servers.Due to flaw in the option parser for sending `NEW_ENV` variables, libcurlcould be made to pass on uninitialized data from a stack based buffer to theserver. Therefore potentially revealing sensitive internal information to theserver using a clear-text network protocol.This could happen because curl did not call and use sscanf() correctly whenparsing the string provided by the application.

CVE-2022-39028

Telnetd in GNU Inetutils through 2.3, MIT krb5-appl through 1.0.3, and derivative works has a NULL pointer dereference via 0xff 0xf7 or 0xff 0xf8. In a typical installation, the telnetd application would crash but the telnet service would remain available through inetd. However, if the telnetd application has many crashes within a short time interval, the telnet service would become unavailable after inetd logs a "telnet/tcp server failing (looping), service terminated" error. NOTE: MIT krb5-appl is not supported upstream but is shipped by a few Linux distributions. The affected code was removed from the supported MIT Kerberos 5 (aka krb5) product many years ago, at version 1.8.

CVE-2021-29629

In FreeBSD 13.0-STABLE before n245765-bec0d2c9c841, 12.2-STABLE before r369859, 11.4-STABLE before r369866, 13.0-RELEASE before p1, 12.2-RELEASE before p7, and 11.4-RELEASE before p10, missing message validation in libradius(3) could allow malicious clients or servers to trigger denial of service in vulnerable servers or clients respectively.

CVE-2021-29628

In FreeBSD 13.0-STABLE before n245764-876ffe28796c, 12.2-STABLE before r369857, 13.0-RELEASE before p1, and 12.2-RELEASE before p7, a system call triggering a fault could cause SMAP protections to be disabled for the duration of the system call. This weakness could be combined with other kernel bugs to craft an exploit.

CVE-2021-29626

In FreeBSD 13.0-STABLE before n245117, 12.2-STABLE before r369551, 11.4-STABLE before r369559, 13.0-RC5 before p1, 12.2-RELEASE before p6, and 11.4-RELEASE before p9, copy-on-write logic failed to invalidate shared memory page mappings between multiple processes allowing an unprivileged process to maintain a mapping after it is freed, allowing the process to read private data belonging to other processes or the kernel.	5.5 MEDIUM

CVE-2021-29627

In FreeBSD 13.0-STABLE before n245050, 12.2-STABLE before r369525, 13.0-RC4 before p0, and 12.2-RELEASE before p6, listening socket accept filters implementing the accf_create callback incorrectly freed a process supplied argument string. Additional operations on the socket can lead to a double free or use after free.

CVE-2020-25584

In FreeBSD 13.0-STABLE before n245118, 12.2-STABLE before r369552, 11.4-STABLE before r369560, 13.0-RC5 before p1, 12.2-RELEASE before p6, and 11.4-RELEASE before p9, a superuser inside a FreeBSD jail configured with the non-default allow.mount permission could cause a race condition between the lookup of ".." and remounting a filesystem, allowing access to filesystem hierarchy outside of the jail.

In FreeBSD 12.2-STABLE before r368250, 11.4-STABLE before r368253, 12.2-RELEASE before p1, 12.1-RELEASE before p11 and 11.4-RELEASE before p5 when processing a DNSSL option, rtsold(8) decodes domain name labels per an encoding specified in RFC 1035 in which the first octet of each label contains the label's length. rtsold(8) did not validate label lengths correctly and could overflow the destination buffer.

CVE-2020-7464

In FreeBSD 12.2-STABLE before r365730, 11.4-STABLE before r365738, 12.1-RELEASE before p10, 11.4-RELEASE before p4, and 11.3-RELEASE before p14, a programming error in the ure(4) device driver caused some Realtek USB Ethernet interfaces to incorrectly report packets with more than 2048 bytes in a single USB transfer as having a length of only 2048 bytes. An adversary can exploit this to cause the driver to misinterpret part of the payload of a large packet as a separate packet, and thereby inject packets across security boundaries such as VLANs.

CVE-2020-25578

In FreeBSD 12.2-STABLE before r368969, 11.4-STABLE before r369047, 12.2-RELEASE before p3, 12.1-RELEASE before p13 and 11.4-RELEASE before p7 several file systems were not properly initializing the d_off field of the dirent structures returned by VOP_READDIR. In particular, tmpfs(5), smbfs(5), autofs(5) and mqueuefs(5) were failing to do so. As a result, eight uninitialized kernel stack bytes may be leaked to userspace by these file systems.	5.3 MEDIUM

CVE-2020-25579

In FreeBSD 12.2-STABLE before r368969, 11.4-STABLE before r369047, 12.2-RELEASE before p3, 12.1-RELEASE before p13 and 11.4-RELEASE before p7 msdosfs(5) was failing to zero-fill a pair of padding fields in the dirent structure, resulting in a leak of three uninitialized bytes.

Fixed security vulnerabilities in R6628P35

CVE-1999-0524

ICMP information such as (1) netmask and (2) timestamp is allowed from arbitrary hosts.

CVE-2022-0778

A flaw was found in OpenSSL. It is possible to trigger an infinite loop by crafting a certificate that has invalid explicit curve parameters. Since certificate parsing happens before verification of the certificate signature, any process that parses an externally supplied certificate may be subject to a denial of service attack

CVE-2021-40490

A race condition was discovered in ext4_write_inline_data_end in fs/ext4/inline.c in the ext4 subsystem in the Linux kernel through 5.13.13.

CVE-2021-20317

A flaw was found in the Linux kernel. A corrupted timer tree caused the task wakeup to be missing in the timerqueue_add function in lib/timerqueue.c. This flaw allows a local attacker with special user privileges to cause a denial of service, slowing and eventually stopping the system while running OSP.

CVE-2021-3679

A lack of CPU resource in the Linux kernel tracing module functionality in versions prior to 5.14-rc3 was found in the way user uses trace ring buffer in a specific way. Only privileged local users (with CAP_SYS_ADMIN capability) could use this flaw to starve the resources causing denial of service.

CVE-2021-4160

There is a carry propagation bug in the MIPS32 and MIPS64 squaring procedure. Many EC algorithms are affected, including some of the TLS 1.3 default curves. Impact was not analyzed in detail, because the pre-requisites for attack are considered unlikely and include reusing private keys. Analysis suggests that attacks against RSA and DSA as a result of this defect would be very difficult to perform and are not believed likely. Attacks against DH are considered just feasible (although very difficult) because most of the work necessary to deduce information about a private key may be performed offline. The amount of resources required for such an attack would be significant. However, for an attack on TLS to be meaningful, the server would have to share the DH private key among multiple clients, which is no longer an option since CVE-2016-0701. This issue affects OpenSSL versions 1.0.2, 1.1.1 and 3.0.0. It was addressed in the releases of 1.1.1m and 3.0.1 on the 15th of December 2021. For the 1.0.2 release it is addressed in git commit 6fc1aaaf3 that is available to premium support customers only. It will be made available in 1.0.2zc when it is released. The issue only affects OpenSSL on MIPS platforms. Fixed in OpenSSL 3.0.1 (Affected 3.0.0). Fixed in OpenSSL 1.1.1m (Affected 1.1.1-1.1.1l). Fixed in OpenSSL 1.0.2zc-dev (Affected 1.0.2-1.0.2zb).

CNVD-2019-23102/CVE-2019-10638/HSVD-202103-0

In the Linux kernel before 5.1.7, a device can be tracked by an attacker using the IP ID values the kernel produces for connection-less protocols (e.g., UDP and ICMP). When such traffic is sent to multiple destination IP addresses, it is possible to obtain hash collisions (of indices to the counter array) and thereby obtain the hashing key (via enumeration). An attack may be conducted by hosting a crafted web page that uses WebRTC or gQUIC to force UDP traffic to attacker-controlled IP addresses.

CVE-2020-10188

utility.c in telnetd in netkit telnet through 0.17 allows remote attackers to execute arbitrary code via short writes or urgent data, because of a buffer overflow involving the netclear and nextitem functions.

CVE-1999-0511

IP forwarding is enabled on a machine which is not a router or firewall.

Fixed security vulnerabilities in R6615P07

None

Upgrading software

This chapter describes types of software used on the switch and how to upgrade software while the switch is operating normally or when the switch cannot correctly start up.

System software file types

Software required for starting up the switch includes:

Boot ROM image—A .bin file that comprises a basic section and an extended section. The basic section is the minimum code that bootstraps the system. The extended section enables hardware initialization and provides system management menus. You can use these menus to load software and the startup configuration file or manage files when the switch cannot correctly start up.

Software images—Includes boot images and system images.

Boot image—A .bin file that contains the operating system kernel. It provides process management, memory management, file system management, and the emergency shell.

System image—A .bin file that contains the minimum modules required for device operation and some basic features, including device management, interface management, configuration management, and routing management.

The software images that have been loaded are called “current software images.” The software images specified to load at next startup are called “startup software images.”

These images might be released separately or as a whole in one .ipe package file. If an .ipe file is used, the system automatically decompresses the file, loads the .bin boot and system images in the file and sets them as startup software images. Typically, the Boot ROM and software images for this switch series are released in an .ipe file named main.ipe.

System startup process

Upon power-on, the Boot ROM image runs to initialize hardware and then the software images run to start up the entire system, as shown in Figure 1.

System startup process

Upgrade methods

You can upgrade system software by using one of the following methods:

| Upgrading method | Software types | Remarks |
| --- | --- | --- |
| Upgrading from the CLI | Boot ROM image Software images | You must reboot the switch to complete the upgrade. This method can interrupt ongoing network services. |
| Upgrading from the Boot menu | Boot ROM image Software images | Use this method when the switch cannot correctly start up. CAUTION: Upgrading an IRF fabric from the CLI instead of the Boot menu. The Boot menu method increases the service downtime, because it requires that you upgrade the member switches one by one. |

The output in this document is for illustration only and might vary with software releases. This document uses boot.bin and system.bin to represent boot and system image names. The actual software image name format is chassis-model_Comware-version_image-type_release, for example, 5140HI-CMW710-BOOT-Rxxxx.bin and 5140HI-CMW710-SYSTEM-Rxxxx.bin.

Upgrading from the CLI

This section uses a two-member IRF fabric as an example to describe how to upgrade software from the CLI. If you have more than two subordinate switches, repeat the steps for the subordinate switch to upgrade their software. If you are upgrading a standalone switch, ignore the steps for upgrading the subordinate switch. For more information about setting up and configuring an IRF fabric, see the installation guide and IRF configuration guide for the HPE 5130 EI switch series.

Preparing for the upgrade

Before you upgrade software, complete the following tasks:

Log in to the IRF fabric through Telnet or the console port. (Details not shown.)

Identify the number of IRF members, each member switch's role, and IRF member ID.

<Sysname> display irf

MemberID   Role   Priority  CPU-Mac         Description

*+1      Master  5         0023-8927-afdc  ---

2      Standby 1         0023-8927-af43  ---

--------------------------------------------------

* indicates the device is the master.

+ indicates the device through which the user logs in.

The Bridge MAC of the IRF is: 0023-8927-afdb

Auto upgrade                : no

Mac persistent              : 6 min

Domain ID                   : 0

Verify that each IRF member switch has sufficient storage space for the upgrade images.

|  | IMPORTANT: Each IRF member switch must have free storage space that is at least two times the size of the upgrade image file. |
| --- | --- |

# Identify the free flash space of the master switch.

<Sysname> dir

Directory of flash:

0      -rw-       41424  Aug 23 2013 02:23:44     startup.mdb

1      -rw-        3792  Aug 23 2013 02:23:44     startup.cfg

2      -rw-    53555200  Aug 23 2013 09:53:48     system.bin

3      drw-           -  Aug 23 2013 00:00:07     seclog

4      drw-           -  Aug 23 2013 00:00:07     diagfile

5      drw-           -  Aug 23 2013 00:00:07     logfile

6      -rw-     9959424  Aug 23 2013 09:53:48     boot.bin

7      -rw-     9012224  Aug 23 2013 09:53:48     backup.bin

524288 KB total (453416 KB free)

# Identify the free flash space of each subordinate switch, for example, switch 2.

<Sysname> dir slot2#flash:/

Directory of slot2#flash:/

0      -rw-       41424  Jan 01 2011 02:23:44     startup.mdb

1      -rw-        3792  Jan 01 2011 02:23:44     startup.cfg

2      -rw-    93871104  Aug 23 2013 16:00:08     system.bin

3      drw-           -  Jan 01 2011 00:00:07     seclog

4      drw-           -  Jan 01 2011 00:00:07     diagfile

5      drw-           -  Jan 02 2011 00:00:07     logfile

6      -rw-     13611008  Aug 23 2013 15:59:00     boot.bin

7      -rw-     9012224  Nov 25 2011 09:53:48     backup.bin

524288 KB total (453416 KB free)

Compare the free flash space of each member switch with the size of the software file to load. If the space is sufficient, start the upgrade process. If not, go to the next step.

Delete unused files in the flash memory to free space:

|  | CAUTION: To avoid data loss, do not delete the current configuration file. For information about the current configuration file, use the display startup command. The delete /unreserved file-url command deletes a file permanently and the action cannot be undone. The delete file-url command moves a file to the recycle bin and the file still occupies storage space. To free the storage space, first execute the undelete command to restore the file, and then execute the delete /unreserved file-url command. |
| --- | --- |

# Delete unused files from the flash memory of the master switch.

<Sysname> delete /unreserved flash:/backup.bin

The file cannot be restored. Delete flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file flash:/backup.bin...Done.

# Delete unused files from the flash memory of the subordinate switch.

<Sysname> delete /unreserved slot2#flash:/backup.bin

The file cannot be restored. Delete slot2#flash:/backup.bin?[Y/N]:y

Deleting the file permanently will take a long time. Please wait...

Deleting file slot2#flash:/backup.bin...Done.

Downloading software images to the master switch

Before you start upgrading software images packages, make sure you have downloaded the upgrading software files to the root directory in flash memory. This section describes downloading an .ipe software file as an example.

The following are ways to download, upload, or copy files to the master switch:

FTP download from a server

FTP upload from a client

TFTP download from a server

Prerequisites

If FTP or TFTP is used, the IRF fabric and the PC working as the FTP/TFTP server or FTP client can reach each other.

Prepare the FTP server or TFTP server program yourself for the PC. The switch series does not come with these software programs.

FTP download from a server

You can use the switch as an FTP client to download files from an FTP server.

To download a file from an FTP server, for example, the server at 10.10.110.1:

Run an FTP server program on the server, configure an FTP username and password, specify the working directory and copy the file, for example, newest.ipe, to the directory.

Execute the ftp command in user view on the IRF fabric to access the FTP server.

<Sysname> ftp 10.10.110.1

Trying 10.10.110.1...

Press CTRL+C to abort

Connected to 10.10.110.1(10.10.110.1).

220 FTP service ready.

User (10.10.110.1:(none)):username

331 Password required for username.

Password:

230 User logged in.

Enable the binary transfer mode.

ftp> binary

200 Type set to I.

Execute the get command in FTP client view to download the file from the FTP server.

ftp> get newest.ipe

227 Entering Passive Mode (10,10,110,1,17,97).

125 BINARY mode data connection already open, transfer starting for /newest.ipe

226 Transfer complete.

32133120 bytes received in 35 seconds (896. 0 kbyte/s)

ftp> bye

221 Server closing.

FTP upload from a client

You can use the IRF fabric as an FTP server and upload files from a client to the IRF fabric.

To FTP upload a file from a client:

On the IRF fabric:

Enable FTP server.

<Sysname> system-view

[Sysname] ftp server enable

Configure a local FTP user account:

# Create the user account.

[Sysname] local-user abc

# Set its password and specify the FTP service.

[Sysname-luser-manage-abc] password simple pwd

[Sysname-luser-manage-abc] service-type ftp

# Assign the network-admin user role to the user account for uploading file to the working directory of the server.

[Sysname-luser-manage-abc] authorization-attribute user-role network-admin

[Sysname-luser-manage-abc] quit

[Sysname] quit

On the PC:

Log in to the IRF fabric (the FTP server) in FTP mode.

c:\> ftp 1.1.1.1

Connected to 1.1.1.1.

220 FTP service ready.

User(1.1.1.1:(none)):abc

331 Password required for abc.

Password:

230 User logged in.

Enable the binary file transfer mode.

ftp> binary

200 TYPE is now 8-bit binary.

Upload the file (for example, newest.ipe) to the root directory of the flash memory on the master switch.

ftp> put newest.ipe

200 PORT command successful

150 Connecting to port 10002

226 File successfully transferred

ftp: 32133120 bytes sent in 64.58 secs (497.60 Kbytes/sec).

TFTP download from a server

To download a file from a TFTP server, for example, the server at 10.10.110.1:

Run a TFTP server program on the server, specify the working directory, and copy the file, for example, newest.ipe, to the directory.

On the IRF fabric, execute the tftp command in user view to download the file to the root directory of the flash memory on the master switch.

<Sysname> tftp 10.10.110.1 get newest.ipe

Press CTRL+C to abort.

% Total    % Received % Xferd  Average Speed   Time    Time     Time  Current

Dload  Upload   Total   Spent    Left  Speed

100 30.6M    0 30.6M    0     0   143k      0 --:--:--  0:03:38 --:--:--  142k

Upgrading the software images

To upgrade the software images:

Specify the upgrade image file (newest.ipe in this example) used at the next startup for the master switch, and assign the M attribute to the boot and system images in the file.

<Sysname> boot-loader file flash:/newest.ipe slot 1 main

Verifying image file..........Done.

Images in IPE:

boot.bin

system.bin

This command will set the main startup software images. Continue? [Y/N]:y

Add images to target slot.

Decompressing file boot.bin to flash:/boot.bin....................Done.

Decompressing file system.bin to flash:/system.bin................Done.

The images that have passed all examinations will be used as the main startup so

ftware images at the next reboot on slot 1.

Specify the upgrade image file as the main startup image file for each subordinate switch. This example uses IRF member 2. (The subordinate switches will automatically copy the file to the root directory of their flash memories.)

<Sysname> boot-loader file flash:/newest.ipe slot 2 main

Verifying image file..........Done.

Images in IPE:

boot.bin

system.bin

This command will set the main startup software images. Continue? [Y/N]:y

Add images to target slot.

Decompressing file boot.bin to flash:/boot.bin....................Done.

Decompressing file system.bin to flash:/system.bin................Done.

The images that have passed all examinations will be used as the main startup so

ftware images at the next reboot on slot 2.

Enable the software auto-update function.

<Sysname> system-view

[Sysname] irf auto-update enable

[Sysname] quit

This function checks the software versions of member switches for inconsistency with the master switch. If a subordinate switch is using a different software version than the master, the function propagates the current software images of the master to the subordinate as main startup images. The function prevents software version inconsistency from causing the IRF setup failure.

Save the current configuration in any view to prevent data loss.

<Sysname> save

The current configuration will be written to the device. Are you sure? [Y/N]:y

Please input the file name(*.cfg)[flash:/startup.cfg]

(To leave the existing filename unchanged, press the enter key):

flash:/startup.cfg exists, overwrite? [Y/N]:y

Validating file. Please wait.................

Saved the current configuration to mainboard device successfully.

Slot 2:

Save next configuration file successfully.

Reboot the IRF fabric to complete the upgrade.

<Sysname> reboot

Start to check configuration with next startup configuration file, please wait.

........DONE!

This command will reboot the device. Continue? [Y/N]:y

Now rebooting, please wait...

The system automatically loads the .bin boot and system images in the .ipe file and sets them as the startup software images.

Execute the display version command in any view to verify that the current main software images have been updated (details not shown).

|  | NOTE: The system automatically checks the compatibility of the Boot ROM image and the boot and system images during the reboot. If you are prompted that the Boot ROM image in the upgrade image file is different than the current Boot ROM image, upgrade both the basic and extended sections of the Boot ROM image for compatibility. If you choose to not upgrade the Boot ROM image, the system will ask for an upgrade at the next reboot performed by powering on the switch or rebooting from the CLI (promptly or as scheduled). If you fail to make any choice in the required time, the system upgrades the entire Boot ROM image. |
| --- | --- |

Upgrading from the Boot menu

In this approach, you must access the Boot menu of each member switch to upgrade their software one by one. If you are upgrading software images for an IRF fabric, using the CLI is a better choice.

|  | TIP: Upgrading through the Ethernet port is faster than through the console port. |
| --- | --- |

Prerequisites

Make sure the prerequisites are met before you start upgrading software from the Boot menu.

Setting up the upgrade environment

Use a console cable to connect the console terminal (for example, a PC) to the console port on the switch.

Connect the Ethernet port on the switch to the file server.

|  | NOTE: The file server and the configuration terminal can be co-located. |
| --- | --- |

Run a terminal emulator program on the console terminal and set the following terminal settings:

Bits per second—9,600

Data bits—8

Parity—None

Stop bits—1

Flow control—None

Emulation—VT100

Preparing for the TFTP or FTP transfer

To use TFTP or FTP:

Run a TFTP or FTP server program on the file server or the console terminal.

Copy the upgrade file to the file server.

Correctly set the working directory on the TFTP or FTP server.

Make sure the file server and the switch can reach each other.

Verifying that sufficient storage space is available

|  | IMPORTANT: For the switch to start up correctly, do not delete the main startup software images when you free storage space before upgrading Boot ROM. On the Boot menu, the main startup software images are marked with an asterisk (*). |
| --- | --- |

When you upgrade software, make sure each member switch has sufficient free storage space for the upgrade file, as shown in Table 9.

Minimum free storage space requirements

| Upgraded images | Minimum free storage space requirements |
| --- | --- |
| Comware images | Two times the size of the Comware upgrade package file. |
| Boot ROM | Same size as the Boot ROM upgrade image file. |

If no sufficient space is available, delete unused files as described in “Managing files from the Boot menu.”

Scheduling the upgrade time

During the upgrade, the switch cannot provide any services. You must make sure the upgrade has a minimal impact on the network services.

Accessing the Boot menu

Starting......

Press Ctrl+D to access BASIC BOOT MENU

********************************************************************************

*                                                                              *

*             HPE 5140 48G 4SFP+ 1-slot HI Sw BOOTROM, Version 117             *

*                                                                              *

********************************************************************************

Copyright (c) 2010-2020 Hewlett Packard Enterprise Development LP

Creation Date       : Nov 10 2020, 10:01:13

CPU Clock Speed     : 800MHz

Memory Size         : 2048MB

Flash Size          : 512MB

CPLD Version        : 002

PCB Version         : Ver.B

Mac Address         : 000fe2699a00

Press Ctrl+B to access EXTENDED BOOT MENU...0

Press one of the shortcut key combinations at prompt.

Shortcut keys

| Shortcut keys | Prompt message | Function | Remarks |
| --- | --- | --- | --- |
| Ctrl+B | Press Ctrl+B to enter Extended Boot menu... | Accesses the extended Boot menu. | Press the keys within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the message appears. You can upgrade and manage system software and Boot ROM from this menu. |
| Ctrl+D | Press Ctrl+D to access BASIC BOOT MENU | Accesses the basic Boot menu. | Press the keys within 1 seconds after the message appears. You can upgrade Boot ROM or access the extended Boot ROM segment from this menu. |

Accessing the basic Boot menu

If the extended Boot ROM segment has corrupted, you can repair or upgrade it from the basic Boot menu.

Press Ctrl+D within 1 seconds after the "Press Ctrl+D to access BASIC BOOT MENU" prompt message appears. If you fail to do this within the time limit, the system starts to run the extended Boot ROM segment.

********************************************************************************

*                                                                              *

*                          BASIC BOOTROM, Version 117                          *

*                                                                              *

********************************************************************************

BASIC BOOT MENU

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

4. Boot extended BootRom

0. Reboot

Ctrl+U: Access BASIC ASSISTANT MENU

Enter your choice(0-4):

Basic Boot ROM menu options

| Option | Task |
| --- | --- |
| 1. Update full BootRom | Update the entire Boot ROM, including the basic segment and the extended segment. To do so, you must use XMODEM and the console port. For more information, see Using XMODEM to upgrade Boot ROM through the console port. |
| 2. Update extended BootRom | Update the extended Boot ROM segment. To do so, you must use XMODEM and the console port. For more information, see Using XMODEM to upgrade Boot ROM through the console port. |
| 3. Update basic BootRom | Update the basic Boot ROM segment. To do so, you must use XMODEM and the console port. For more information, see Using XMODEM to upgrade Boot ROM through the console port. |
| 4. Boot extended BootRom | Access the extended Boot ROM segment. For more information, see Accessing the extended Boot menu. |
| 0. Reboot | Reboot the switch. |
| Ctrl+U: Access BASIC ASSISTANT MENU | Press Ctrl + U to access the BASIC ASSISTANT menu (see Table 12). |

BASIC ASSISTANT menu options

| Option | Task |
| --- | --- |
| 1. RAM Test | Perform a RAM self-test. |
| 0. Return to boot menu | Return to the basic Boot menu. |

Accessing the extended Boot menu

Press Ctrl+B within 1 second (in fast startup mode) or 5 seconds (in full startup mode) after the "Press Ctrl-B to enter Extended Boot menu..." prompt message appears. If you fail to do this, the system starts decompressing the system software.

Alternatively, you can enter 4 in the basic Boot menu to access the extended Boot menu.

The "Password recovery capability is enabled." or "Password recovery capability is disabled." message appears, followed by the extended Boot menu. Availability of some menu options depends on the state of password recovery capability (see Table 13). For more information about password recovery capability, see Fundamentals Configuration Guide in HPE 5130 EI Switch Series Configuration Guides.

Password recovery capability is enabled.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8):

Extended Boot ROM menu options

| Option | Tasks |
| --- | --- |
| 1. Download image to flash | Download a software image file to the flash. |
| 2. Select image to boot | Specify the main and backup software image file for the next startup. Specify the main and backup configuration files for the next startup. This task can be performed only if password recovery capability is enabled. |
| 3. Display all files in flash | Display files on the flash. |
| 4. Delete file from flash | Delete files to free storage space. |
| 5. Restore to factory default configuration | Delete the current next-startup configuration files and restore the factory-default configuration. This option is available only if password recovery capability is disabled. |
| 6. Enter BootRom upgrade menu | Access the Boot ROM upgrade menu. |
| 7. Skip current system configuration | Start the switch without loading any configuration file. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| 8. Set switch startup mode | Set the startup mode to fast startup mode or full startup mode. |
| 0. Reboot | Reboot the switch. |
| Ctrl+F: Format file system | Format the current storage medium. |
| Ctrl+P: Change authentication for console login | Skip the authentication for console login. This is a one-time operation and takes effect only for the first system boot or reboot after you choose this option. This option is available only if password recovery capability is enabled. |
| Ctrl+R: Download image to SDRAM and run | Download a system software image and start the switch with the image. This option is available only if password recovery capability is enabled. |
| Ctrl+Z: Access EXTENDED ASSISTANT MENU | Access the EXTENDED ASSISTANT MENU. For options in the menu, see Table 14. |
| Ctrl+Y: Change Work Mode | Change Work Mode. |
| Ctrl+C: Display Copyright | Display the copyright statement. |

EXTENDED ASSISTANT menu options

| Option | Task |
| --- | --- |
| 1. Display Memory | Display data in the memory. |
| 2. Search Memory | Search the memory for a specific data segment. |
| 0. Return to boot menu | Return to the extended Boot ROM menu. |

Upgrading Comware images from the Boot menu

You can use the following methods to upgrade Comware images:

Using TFTP to upgrade software images through the Ethernet port

Using FTP to upgrade software images through the Ethernet port

Using XMODEM to upgrade software through the console port

Using TFTP to upgrade software images through the Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: To use the default setting for a field, press Enter without entering any value. If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu without downloading the upgrade file, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

|  | NOTE: The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images are not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in flash memory for backup. To use it at reboot, you must change its attribute to main or backup. If an image with the same attribute as the image you are loading is already in the flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 0

Using FTP to upgrade software images through the Ethernet port

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name      :update.ipe

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

FTP User Name       :switch

FTP User Password   :***

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.ipe). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: To use the default setting for a field, press Enter without entering any value. If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters, and enter Y to confirm the settings. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the image file. To return to the Boot menu without downloading the upgrade file, enter N.

Loading.........................................................................

................................................................................

................................................................................

................................................................Done!

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) M

Image file boot.bin is self-decompressing...

Free space: 534980608 bytes

Writing flash...................................................................

................................................................................

...................................................................Done!

Image file system.bin is self-decompressing...

Free space: 525981696 bytes

Writing flash...................................................................

................................................................................

................................................................................

................................................................................

................................................................................

................................................................................

.......................................................................Done!

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8):0

|  | NOTE: The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in flash memory for backup. To use it at reboot, you must change its attribute to main or backup. If an image with the same attribute as the image you are loading is already in the flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

Enter 0 in the Boot menu to reboot the switch with the new software images.

Using XMODEM to upgrade software through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the Ethernet port. To save time, use the Ethernet port as long as possible.

Enter 1 in the Boot menu to access the file transfer protocol submenu.

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter. The following prompt appears:

Are you sure to download file to flash? Yes or No (Y/N):Y

Enter Y to start downloading the file. (To return to the Boot menu, enter N.)

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter the M (main), B (backup), or N (none) attribute for the images. In this example, assign the main attribute to the images.

Please input the file attribute (Main/Backup/None) m

The boot.bin image is self-decompressing...

# At the Load File name prompt, enter a name for the boot image to be saved to flash memory.

Load File name  : default_file boot-update.bin       (At the prompt,

Free space: 470519808 bytes

Writing flash...................................................................

.............Done!

The system-update.bin image is self-decompressing...

# At the Load File name prompt, enter a name for the system image to be saved to flash memory.

Load File name  : default_file system-update.bin

Free space: 461522944 bytes

Writing flash...................................................................

.............Done!

Your baudrate should be set to 9600 bps again!

Press enter key when ready

|  | NOTE: The switch always attempts to boot with the main images first. If the attempt fails, for example, because the main images not available, the switch tries to boot with the backup images. An image with the none attribute is only stored in the flash memory for backup. To use it at reboot, you must change its attribute to main or backup. If an image with the same attribute as the image you are loading is already in flash memory, the attribute of the old image changes to none after the new image becomes valid. |
| --- | --- |

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps as described in step 5.a. If the baud rate is 9600 bps, skip this step.

|  | NOTE: The console port rate reverts to 9600 bps at a reboot. If you have changed the baud rate, you must perform this step so you can access the switch through the console port after a reboot. |
| --- | --- |

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 0

Enter 0 in the Boot menu to reboot the system with the new software images.

Upgrading Boot ROM from the Boot menu

You can use the following methods to upgrade the Boot ROM image:

Using TFTP to upgrade Boot ROM through the Ethernet port

Using FTP to upgrade Boot ROM through the Ethernet port

Using XMODEM to upgrade Boot ROM through the console port

Using TFTP to upgrade Boot ROM through the Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 1 to set the TFTP parameters.

Load File Name      :update.btm

Server IP Address   :192.168.0.3

Local IP Address    :192.168.0.2

Subnet Mask         :255.255.255.0

Gateway IP Address  :0.0.0.0

TFTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the TFTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |

|  | NOTE: To use the default setting for a field, press Enter without entering any value. If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using FTP to upgrade Boot ROM through the Ethernet port

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 2 to set the FTP parameters.

Load File Name     :update.btm

Server IP Address  :192.168.0.3

Local IP Address   :192.168.0.2

Subnet Mask        :255.255.255.0

Gateway IP Address :0.0.0.0

FTP User Name      :switch

FTP User Password  :123

FTP parameter description

| Item | Description |
| --- | --- |
| Load File Name | Name of the file to download (for example, update.btm). |
| Server IP Address | IP address of the FTP server (for example, 192.168.0.3). |
| Local IP Address | IP address of the switch (for example, 192.168.0.2). |
| Subnet Mask | Subnet mask of the switch (for example, 255.255.255.0). |
| Gateway IP Address | IP address of the gateway (in this example, no gateway is required because the server and the switch are on the same subnet). |
| FTP User Name | Username for accessing the FTP server, which must be the same as configured on the FTP server. |
| FTP User Password | Password for accessing the FTP server, which must be the same as configured on the FTP server. |

|  | NOTE: To use the default setting for a field, press Enter without entering any value. If the switch and the server are on different subnets, you must specify a gateway address for the switch. |
| --- | --- |

Enter all required parameters and press Enter to start downloading the file.

Loading.................................................Done!

Enter Y at the prompt to upgrade the basic Boot ROM section.

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Using XMODEM to upgrade Boot ROM through the console port

XMODEM download through the console port is slower than TFTP or FTP download through the Ethernet port. To save time, use the Ethernet port as long as possible.

Enter 6 in the Boot menu to access the Boot ROM update menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 1 in the Boot ROM update menu to upgrade the full Boot ROM.

The file transfer protocol submenu appears:

1. Set TFTP protocol parameters

2. Set FTP protocol parameters

3. Set XMODEM protocol parameters

0. Return to boot menu

Enter your choice(0-3):

Enter 3 to set the XMODEM download baud rate.

Please select your download baudrate:

1.* 9600

2.  19200

3.  38400

4.  57600

5.  115200

0.  Return to boot menu

Enter your choice(0-5):5

Select an appropriate download rate, for example, enter 5 to select 115200 bps.

Download baudrate is 115200 bps

Please change the terminal's baudrate to 115200 bps and select XMODEM protocol

Press enter key when ready

Set the serial port on the terminal to use the same baud rate and protocol as the console port. If you select 9600 bps as the download rate for the console port, skip this task.

Select Call > Disconnect in the HyperTerminal window to disconnect the terminal from the switch.

Disconnecting the terminal from the switch

Select File > Properties, and in the Properties dialog box, click Configure.

Properties dialog box

Select 115200 from the Bits per second list and click OK.

Modifying the baud rate

Select Call > Call to reestablish the connection.

Reestablishing the connection

Press Enter to start downloading the file.

Now please start transfer file with XMODEM protocol

If you want to exit, Press <Ctrl+X>

Loading ...CCCCCCCCCCCCCCCCCCCCCCCCC

Select Transfer > Send File in the HyperTerminal window.

Transfer menu

In the dialog box that appears, click Browse to select the source file, and select Xmodem from the Protocol list.

File transmission dialog box

Click Send. The following dialog box appears:

File transfer progress

Enter Y at the prompt to upgrade the basic Boot ROM section.

Loading ...CCCCCCCCCCCCCC  ...Done!

Will you Update Basic BootRom? (Y/N):Y

Updating Basic BootRom...........Done.

Enter Y at the prompt to upgrade the extended Boot ROM section.

Updating extended BootRom? (Y/N):Y

Updating extended BootRom.........Done.

If the baud rate of the HyperTerminal is not 9600 bps, restore it to 9600 bps at the prompt, as described in step 4.a. If the baud rate is 9600 bps, skip this step.

Please change the terminal's baudrate to 9600 bps, press ENTER when ready.

|  | NOTE: The console port rate reverts to 9600 bps at a reboot. If you have changed the baud rate, you must perform this step so you can access the switch through the console port after a reboot. |
| --- | --- |

Press Enter to access the Boot ROM update menu.

Enter 0 in the Boot ROM update menu to return to the Boot menu.

1. Update full BootRom

2. Update extended BootRom

3. Update basic BootRom

0. Return to boot menu

Enter your choice(0-3):

Enter 0 in the Boot menu to reboot the switch with the new Boot ROM image.

Managing files from the Boot menu

From the Boot menu, you can display files in flash memory to check for obsolete files, incorrect files, or space insufficiency, delete files to release storage space, or change the attributes of software images.

Displaying all files

Enter 3 in the Boot menu to display all files in flash memory and identify the free space size.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 3

The following is a sample output:

Display all file(s) in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 464298848 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Deleting files

If storage space is insufficient, delete obsolete files to free up storage space.

To delete files:

Enter 4 in the Boot menu:

Deleting the file in flash:

File Number    File Size(bytes)     File Name

================================================================================

1              8177                 flash:/testbackup.cfg

2(*)           53555200             flash:/system.bin

3(*)           9959424              flash:/boot.bin

4              3678                 flash:/startup.cfg_backup

5              30033                flash:/default.mdb

6              42424                flash:/startup.mdb

7              18                   flash:/.pathfile

8              232311               flash:/logfile/logfile.log

9              5981                 flash:/startup.cfg_back

10(*)          6098                 flash:/startup.cfg

11             20                   flash:/.snmpboots

Free space: 464298848 bytes

The current image is boot.bin

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Enter the number of the file to delete. For example, enter 1 to select the file testbackup.cfg.

Please input the file number to change: 1

Enter Y at the confirmation prompt.

The file you selected is testbackup.cfg,Delete it? (Y/N):Y

Deleting....................................Done!

Changing the attribute of software images

Software image attributes include main (M), backup (B), and none (N). System software and boot software can each have multiple none-attribute images but only one main image and one backup image on the switch. You can assign both the M and B attributes to one image. If the M or B attribute you are assigning has been assigned to another image, the assignment removes the attribute from that image. If the removed attribute is the sole attribute of the image, its attribute changes to N.

For example, the system image system.bin has the M attribute and the system image system-update.bin has the B attribute. After you assign the M attribute to system-update.bin, the attribute of system-update.bin changes to M+B and the attribute of system.bin changes to N.

To change the attribute of a system or boot image:

Enter 2 in the Boot menu.

EXTENDED BOOT MENU

1. Download image to flash

2. Select image to boot

3. Display all files in flash

4. Delete file from flash

5. Restore to factory default configuration

6. Enter BootRom upgrade menu

7. Skip current system configuration

8. Set switch startup mode

0. Reboot

Ctrl+Z: Access EXTENDED ASSISTANT MENU

Ctrl+F: Format file system

Ctrl+P: Change authentication for console login

Ctrl+R: Download image to SDRAM and run

Ctrl+Y: Change Work Mode

Ctrl+C: Display Copyright

Enter your choice(0-8): 2

1 or 2 at the prompt to set the attribute of a software image. (The following output is based on the option 2. To set the attribute of a configuration file, enter 3.)

1. Set image file

2. Set bin file

3. Set configuration file

0. Return to boot menu

Enter your choice(0-3): 2

File Number    File Size(bytes)     File Name

================================================================================

1(*)              53555200              flash:/system.bin

2(*)              9959424               flash:/boot.bin

3                 13105152              flash:/boot-update.bin

4                 91273216              flash:/system-update.bin

Free space: 417177920 bytes

(*)-with main attribute

(b)-with backup attribute

(*b)-with both main and backup attribute

Note:Select .bin files. One but only one boot image and system image must be included.

Enter the number of the file you are working with. For example, enter 3 to select the boot image boot-update.bin. and enter 4 to select the system image system-update.bin.

Enter file No.(Allows multiple selection):3

Enter another file No.(0-Finish choice):4

Enter 0 to finish the selection.

Enter another file No.(0-Finish choice):0

You have selected:

flash:/boot-update.bin

flash:/system-update.bin

Enter M or B to change its attribute to main or backup. If you change its attribute to M, the attribute of boot.bin changes to none.

Please input the file attribute (Main/Backup) M

This operation may take several minutes. Please wait....

Next time, boot-update.bin will become default boot file!

Next time, system-update.bin will become default boot file!

Set the file attribute success!

Handling software upgrade failures

If a software upgrade fails, the system runs the old software version.

To handle a software upgrade failure:

Verify that the software release is compatible with the switch model and the correct file is used.

Verify that the software release and the Boot ROM release are compatible. For software and Boot ROM compatibility, see the hardware and software compatibility matrix in the correct release notes.

Check the physical ports for a loose or incorrect connection.

If you are using the console port for file transfer, check the HyperTerminal settings (including the baud rate and data bits) for any wrong setting.

Check the file transfer settings:

If XMODEM is used, you must set the same baud rate for the terminal as for the console port.

If TFTP is used, you must enter the same server IP addresses, file name, and working directory as set on the TFTP server.

If FTP is used, you must enter the same FTP server IP address, source file name, working directory, and FTP username and password as set on the FTP server.

Check the FTP or TFTP server for any incorrect setting.

Check that the storage device has sufficient space for the upgrade file.